3GPP TSG-RAN WG5 Meeting #65	R5-145185

# Venice, Italy, 10th – 14th November 2014

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-1 | CR | 3677 | rev | - | Current version: | 11.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Section 7.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HSDPA_MFTX-UEConTest |  |  |  |  |  |  | Date: |  | 2014-10-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Missing title to Section 7.2.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Title to Section 7.2.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Specification will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

7.2.5	Multiflow RLC Tests

#### 7.2.5.1 to 7.2.5.3

#### 7.2.5.4	Erroneous Sequence Number Processing when two cell/Single frequency Multiflow configured on Single Band/Inter Node B

7.2.5.4.1	Definition and applicability

UE categories (Cat 21, Cat 22, Cat 23, Cat 24) which support combination of Multiflow HSDPA on Single Frequency/2 Cell on Single Band (SF-DC).

UE supports FDD.

7.2.5.4.2	Conformance requirement

A STATUS PDU or Piggybacked STATUS PDU including "erroneous Sequence Number" is a STATUS PDU or Piggybacked STATUS PDU that contains:

-	a LIST, BITMAP or RLIST SUFI in which the "Sequence Number" of at least one AMD PDU that is negatively acknowledged is outside the interval VT(A)"Sequence Number"< VT(S); or

-	an ACK SUFI in which "LSN" is outside the interval VT(A)"LSN" VT(S).

If an AM RLC entity receives a STATUS PDU or a Piggybacked STATUS PDU including "erroneous Sequence Number", it shall:

-	discard the STATUS PDU or the Piggybacked STATUS PDU;

-	if inter-Node B Multiflow operation is not configured by higher layers, initiate the RLC reset procedure (see subclause 11.4).

[…]

The Sender shall:

-	if one of the following triggers is detected:

…

3)	A STATUS PDU or a piggybacked STATUS PDU including "erroneous Sequence Number" is received (see clause 10);

-	stop transmitting any AMD PDU or STATUS PDU;

-	ignore any incoming AMD PDU, piggybacked STATUS PDU or STATUS PDU;

-	increment VT(RST) by 1;

-	if VT(RST) = MaxRST:

-	perform the actions specified in subclause 11.4.4a.

-	else (if VT(RST) < MaxRST):

-	submit a RESET PDU to the lower layer;

-	start the timer Timer_RST according to the description in subclause 9.5.

NOTE:	If the TFC selection exchange has been initiated by sending the RLC Entity Info parameter to MAC, the RLC entity may delay the RLC reset procedure until the end of the next TTI.

[…]

Reference

3GPP TS 25.322 clause 10.1, 11.4.2

7.2.5.4.3	Test purpose

To confirm that the UE does not initiate RLC PDU Reset Procedure/Inter-Node B

7.2.5.4.4	Method of test

Initial Condition

The generic procedure for Radio Bearer establishment (clause 7.1.3 of TS 34.108) is executed, with all the parameters as specified in the procedure, with the exception that the default Radio Access Bearer is replaced with the RAB defined for AM 7-bit "Length Indicator" tests in clause 7.2.3.1.

These settings apply to both the uplink and downlink DTCH.

The Radio Bearer is placed in UE test loop mode 1 with the UL SDU size set to (2 * AM_7_PayloadSize) – 1 bytes.

System Simulator:

SS configures cells as per Table 5.0aC of TS 25.101 [21].

2 cells –Cell 1/2 with Cell 1 (f1) (Anchor Carrier), and Cell 2 (f1) (Assisting Carrier) – One Anchor Carrier and One Assisting Carrier on Band A. (SF-DC). Configure with inter-Node-B multiflow operation. Tcell of Cell 2 is set to 3840 chips (1.5 slots).

UE: CELL_DCH (state 6-18) with condition A44 - SRBs mapped on DCH/DCH

Related ICS/IXIT statement(s)

-	UE supports FDD

-	UE category supports Multiflow HSDPA operation on 2 cells on single frequency (UE Category 21-24)

Test Procedure

The SS sends 2 RLC SDUs of size (2 * AM_7_PayloadSize) – 1 bytes. One SDU is sent to cell 1, and another SDU is sent to cell 2, where cell 1 and cell 2 are in inter-NodeB multiflow operation,

The SS checks the RLC PDUs received on the uplink and responds to all poll requests with a LIST SUFI negatively acknowledging the RLC PDU with sequence number that is outside the interval VT(A)"Sequence Number"< VT(S)

The SS waits for 1 second. The SS should not receive RESET PDU from UE.

Repeating the above steps, the SS sends 2 more RLC SDUs of size (2 * AM_7_PayloadSize) – 1 bytes. One SDU is sent to cell 1, and another SDU is sent to cell 2.

The SS checks the RLC PDUs received on the uplink and responds to all poll requests with an ACK SUFI positively acknowledging the RLC PDU with sequence number that is outside the interval VT(A)"Sequence Number"< VT(S)

The SS waits for 1 second. The SS should not receive RESET PDU from UE.

Expected sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | DOWNLINK RLC PDU | SDU 1 to cell 1 |
| 2 |  |  | DOWNLINK RLC PDU | SDU 1 to cell 1 |
| 3 |  |  | DOWNLINK RLC PDU | SDU 2 to cell 2 |
| 4 |  |  | DOWNLINK RLC PDU | SDU 2 to cell 2 |
| 5 |  |  | UPLINK RLC PDU | SDU 1 |
| 6 |  |  | … | SS continues to receive RLC PDUs |
| 7 |  |  | UPLINK RLC PDU | SDU, Poll The Poll may appear in returned PDU for either SDU 1 or 2 |
| 8 |  |  | STATUS PDU | LIST NAK SN, where SN is outside the interval VT(A)"Sequence Number"<VT(S). |
| 9 |  |  | … | SS should not receive RESET PDU from UE |
| 10 |  |  | DOWNLINK RLC PDU | SDU 3 to cell 1 |
| 11 |  |  | DOWNLINK RLC PDU | SDU 3 to cell 1 |
| 12 |  |  | DOWNLINK RLC PDU | SDU 4 to cell 2 |
| 13 |  |  | DOWNLINK RLC PDU | SDU 4 to cell 2 |
| 14 |  |  | UPLINK RLC PDU | SDU 3 |
| 15 |  |  | … | SS continues to receive RLC PDUs |
| 16 |  |  | UPLINK RLC PDU | SDU, Poll. The Poll may appear in returned PDU for either SDU 3 or 4 |
| 17 |  |  | STATUS PDU | ACK SUFI SN, where SN is outside the interval VT(A)"Sequence Number"< VT(S). |
| 18 |  |  | … | SS should not receive RESET PDU from UE |
| NOTE: The Expected Sequence shown is informative. The UPLINK and DOWNLINK PDU flows may overlap, but are shown separate for clarity. Information such as SDU, PDU or Sequence numbers given in the comments column shall be considered informative only, for test case development purposes. |  |  |  |  |

7.2.5.4.5	Test requirements

In steps 9 and 18, the SS should not receive RESET PDU from UE.