3GPP TSG-RAN WG4 Meeting #77	R4-158366

# Anaheim, US, 16 – 20 November 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | 1091 | rev | 1 | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Dual Carrier HSUPA enhancements for UTRAN CS in TS 25.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks, Ericsson, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_HSUPA_CS-Core |  |  |  |  |  |  | Date: |  | 2015-11-19 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Introduction of dual-carrier UL enhancements for UTRAN CS that allow for configuring simultaneously dual-carrier UL operation with the DPDCH channel, which is needed for CS UTRAN data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sub-clause 6.2.2A, the DPDCH channel is added to the list of UL channels in Table 6.1AA and 6.1AB. In sub-clauses 6.6.2.2.1A and 6.8.2.1A βd is included in the requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to configure simultaneously dual-carrier UL and DPDCH channel. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2A, 6.6.2.2.1A and 6.8.2.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS34.121-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ***Start of changes***

### 6.2.2A	UE maximum output, power for DC-HSUPA

The Maximum Power Reduction (MPR) for the nominal maximum output power defined in 6.2.1 is specified for the values of c, βd, hs, ec and ed defined in [8] fully or partially transmitted during a DPCCH timeslot, and defined through calculation of the Raw Cubic Metric (Raw CM) which is based on the UE transmit channel configuration and is given by

Raw CM = 20 * log10 ((v_norm 3) rms) - 20 * log10 ((v_norm_ref 3) rms)

where

-	v_norm is the normalized voltage waveform of the input signal

-	v_norm_ref is the normalized voltage waveform of the reference signal (12.2 kbps AMR Speech) and

-	20 * log10 ((v_norm_ref 3) rms) = 1.52 dB

For any DC-HSUPA signal not employing 16QAM modulation on any of the carriers, the MPR is specified in Table 6.1AA.

Table 6.1AA: UE maximum output power for DC-HSUPA signals not employing 16QAM modulation on any of the carriers

| UE transmit channel configuration | CM (dB) | MPR (dB) |
| --- | --- | --- |
| When DPCCH2 is not configured: For all combinations of; DPCCH, DPDCH, HS-DPCCH, E-DPDCH and E-DPCCH | 0.22 CM 3.72 | MAX (CM-0.72, 0) |
| When DPCCH2 is configured: For all combinations of; DPCCH, DPDCH, HS-DPCCH, E-DPDCH, E-DPCCH and DPCCH2 | 0.22 CM 3.72 | MAX (CM-0.72, 0) |

where Cubic Metric (CM) is based on the Raw CM and is given by

CM = CEIL { Raw CM / k, 0.22 }

where

-	CEIL { x, 0.22 } means rounding upwards to closest 0.22dB with 0.5 dB granularity, i.e. CM = [0.22, 0.72, 1.22, 1.72, 2.22, 2.72, 3.22, 3.72]

-	k is 1.66

For any DC-HSUPA signal employing 16QAM modulation on any of the carriers, the MPR is specified in Table 6.1AB..

Table 6.1AB: UE maximum output power for DC-HSUPA signals employing 16QAM modulation on any of the carriers

| UE transmit channel configuration | CM (dB) | MPR (dB) |
| --- | --- | --- |
| When DPCCH2 is not configured: For all combinations of; DPCCH, DPDCH, HS-DPCCH, E-DPDCH and E-DPCCH | [0.22 CM 3.72] | [CM+0.8] |
| When DPCCH2 is configured: For all combinations of DPCCH, DPDCH, HS-DPCCH, E-DPDCH and E-DPCCH and DPCCH2 | [0.22 CM 3.72] | [CM+0.8] |

where Cubic Metric (CM) is based on the Raw CM and is given by

[CM = CEIL { Raw CM / k, 0.2 }]

where

-	CEIL { x, 0.2 } means rounding upwards to closest 0.2dB with 0.5 dB granularity, i.e. CM = [0.2, 0.7, 1.2, 1.7, 2.2, 2.7, 3.2, 3.7]

-	k is 1.66.

The reference measurement channels for the requirements in subclause 6.2.2A are provided in subclause A.2.8.

### ***End of up changes***

### ***Start of changes***

##### 6.6.2.2.1A	Additional requirement for DC-HSUPA

If the adjacent channel power is greater than -50dBm then the ACLR shall be higher than the value specified in Table 6.11A. The requirements are applicable for all values of c, βd, hs, ec and ed as specified in [8]. The reference measurement channels for the requirements in subclause 6.6.2.2.1A are provided in subclause A.2.8.

# Table 6.11A: UE ACLR for DC-HSUPA

| Power Class | Adjacent channel frequency relative to the center of two assigned channel frequencies | ACLR limit |
| --- | --- | --- |
| 3 | + 7.5 MHz or – 7.5 MHz | 33 dB |
| 3 | + 12.5 MHz or – 12.5 MHz | 36 dB |
| 4 | + 7.5 MHz or – 7.5 MHz | 33 dB |
| 4 | + 12.5 MHz or -12.5 MHz | 36 dB |

NOTE 1:	The requirement shall still be met in the presence of switching transients.

NOTE 2:	The ACLR requirements reflect what can be achieved with present state of the art technology.

NOTE 3:	Requirement on the UE shall be reconsidered when the state of the art technology progresses.

### ***End of changes***

### ***Start of changes***

#### 6.8.2.1A	Additional requirement for DC-HSUPA

When 16QAM modulation is not used on any of the uplink code channels in a carrier, the Error Vector Magnitude in that carrier shall not exceed 17.5 % for the parameters specified in Table 6.15AA.

When 16QAM modulation is used on any of the uplink code channels in a carrier, the modulation accuracy requirement shall meet one or both of the following requirements:

1.	The Error Vector Magnitude does not exceed 14 % for the parameters specified in Table 6.15AA.

2.	The Relative Code Domain Error requirements specified in 6.8.3a are met.

The requirements are applicable for all values of βc, βd, βhs, βec and βed as specified in [8], when the total power in each of the assigned carriers is equal to each other. The reference measurement channels for the requirements in subclause 6.8.2.1A are provided in subclause A.2.6 and A.2.7.

# Table 6.15AA: Parameters for Error Vector Magnitude for DC-HSUPA

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power, no 16QAM | dBm | -20 |
| UE Output Power, 16QAM | dBm | -30 |
| Operating conditions |  | Normal conditions |
| Power control step size | dB | 1 |

### ***End of changes***