3GPP TSG-RAN Meeting #70	RP-152185

Sitges, Spain, December 7 - 10, 2015										update of RP-151680

Agenda Item:			16

Souce:	Samsung

Title:	UE category in fragmented spectrum or with lower baseband processing power

Document for:	Discussion

Introduction

Last RAN meeting and RAN2 meeting, 256QAM in the lower categories was proposed [1] with following two motivations.

- To meet the target data rate in the fragmented spectrum

- To allow more flexible UE implementation (e.g. lower baseband processing power than the max data rate of the radio configuration)

This contribution discusses the issue to facilitate RAN discussion with the full picture taken into account.

Discussion

Recap of the current specification w.r.t fragmented spectrum

UE category is defined by multiple parameters among which Maximum number of DL-SCH transport block bits received within a TTI (max bits per TTI hereafter) determines the maximum data rate of the category.

Max bits per TTI are a function of capabilities like carrier aggregation capability, MIMO capability and Modulation order that are basically per band combination parameters.

In general, a UE have multiple supportedBandCombination, each of which has different CA and MIMO capability hence different max bit per TTI. The current specification requires that at least one band combination shall meet the max bit per TTI as below.

<Table 1>

The carrier aggregation and MIMO capabilities indicated for at least one band combination together with modulation scheme shall meet the processing requirements defined by the physical layer parameter values in the UE category (i.e., maximum number of DL-SCH/UL-SCH transport block bits received/transmitted within a TTI, maximum number of bits of a DL-SCH/UL-SCH transport block received/transmitted within a TTI, and total number of soft channel bits for downlink).

Observation 1: According to the current specification, UE shall meet the target maximum data rate of its category in at least one supported band combination

Hereafter the supported band combination meeting the maximum data rate is denoted as reference band combination.

The max bit per TTI is calculated based on the assumption that the system bandwidth of all component carriers of the reference band combination is 20 MHz. It is however not always true due to different spectrum allocation of each operator For example, the reference band combination of certain early CA UE was (20 + 10) MHz. Then even if the MIMO capability and CA capability (in terms of the number of component carriers) are sufficient for category 6 the actual maximum data rate (225 Mbps) is lower than the target maximum data rate of the category (300 Mbps).

This issue has been discussed during RAN2#77 which made consensus on the principle that whether UE meet the max data rate or not is determined by the max data rate achieved when system bandwidths are 20 MHz. A CR is agreed [2] with adding the following note in 36.306. The intention of the CR is clearly indicated in the Reason for change of the CR.

<Table 2>

NOTE: For the purposes of determining whether the carrier aggregation and MIMO capabilities indicated for a band combination meets the processing requirements defined by the physical layer parameter values in the UE category as described above, the carrier aggregation and MIMO capabilities indicated for a band combination is considered to meet the processing requirements if the UE support the maximum processing requirements defined by the UE category assuming 20MHz channel bandwidth is supported on all bands.

Observation 2: According to the current specification, UE not meeting the target maximum data rate of its category due to fragmented spectrum problem (i.e. full bandwidth is not provided in the reference band combination) still declare the category if the maximum data rate were reached with 20 MHz per component carrier.

The definition of UE category

The principle relying on ‘20MHz per CC’ indicates that UE category is about what it can do when sufficient radio resource is provided. It is logical because UE category should not be about something affected by deployment scenarios but about UE implementation itself.

There are 3 different aspects related to UE category; baseband capability, radio capability and observable maximum data rate. Former two determines the target maximum data rate (i.e. what UE can do if sufficient radio resource is provided). Observable maximum data rate is varying depending on the deployment scenario as seen in table 3 and table 4.

E-UTRA carrier aggregation configuration and corresponding maximum aggregated bandwidth in the tables are captured from the Table 5.6A.1-1, Table 5.6A.1-2, Table 5.6A.1-2a, Table 5.6A.1-2b and Table 5.6A.1-3 [3].

<Table 3> Actual maximum data rates for 2 CC CA deployments

| Maximum aggregated Bandwidth | E-UTRA CA configuration | Observable maximum data rate (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Radio configuration 1: 2x[2, 64] ** | Radio configuration 2: 2x[2, 256] | Radio configuration 3: 2x[4, 64] | Radio configuration 4: [4,256]+[2,256] |
| 40 (20, 20)* | 1C, … Set 1 | 300 | 400 | 600 | 600 |
| 35 (20, 15) | 39C, …Set 2 | 262.5 | 350 | 525 | 550 |
| 30 (20, 10) | 1A-5A, … Set 3 | 225 | 300 | 450 | 500 |
| 30 (15, 15) |  | 225 | 300 | 450 | 450 |
| 25 (20, 5) | 3A-31A, … Set 4 | 187.5 | 250 | 375 | 450 |
| 25 (15, 10) |  | 187.5 | 250 | 375 | 400 |
| 20 (10, 10) | 7B, … Set 5 | 150 | 200 | 300 | 300 |
| 20 (15, 5) |  | 150 | 200 | 300 | 350 |
| 15 (10, 5) | 12B | 112.5 | 150 | 225 | 250 |
| 13 (10, 3) | 27B | 97.5 | 130 | 195 | 230 |
| UE category according to the current specification |  | Category 6/7 | Category 13 | Category 11/12 | Category 11/12 |

* (n, m) denotes the 2CC CA with each of bandwidth being n and m MHz

** k x [l, m] + o x [p, q] denotes the radio configuration where k component carriers with l layers and m QAM and o component carriers with p layers and q QAM are configured

Set 1: 1C, 2C, 3C, 7C, 7C,7C, 38C, 40C, 41C, 42C, 1A-3A, 1A-7A, 1A-20A, 1A-28A, 1A-40A, 1A-41A, 1A-42A, 2A-4A, 2A-28A, 3A-7A, 3A-20A, 3A-28A, 3A-38A, 3A-40A, 3A-42A, 4A-28A, 7A-20A, 7A-22A, 7A-28A, 7A-40A, 20A-42A, 25A-41A, 38A-40A, 39A-41A, 41A-42A

Set 2: 39C, 1A-18A, 1A-19A, 1A-21A, 1A-26A, 3A-19A, 3A-26A, 7A-28A, 19A-42A, 21A-42A, 25A-26A, 26A-41A

Set 3: 1A-5A, 1A-8A, 1A-11A, 2A-5A, 2A-12A, 2A-13A, 2A-29A, 2A-30A, 3A-5A, 3A-8A, 3A-20A, 3A-27A, 4A-5A, 4A-7A, 4A-12A, 4A-13A, 4A-27A, 4A-29A, 4A-30A, 5A-7A, 5A-25A, 5A-40A, 7A-8A, 7A-12A, 7A-20A, 8A-40A, 8A-41A, 12A-25A, 19A-21A, 20A-2A, 23A-29A

Set 4: 3A-31A, 11A-18A, 18A-28A, 19A-28A, 20A-31A

Set 5: 7B, 23B, 1A-5A, 1A-8A, 1A-18A, 1A-26A, 1A-28A, 2A-4A, 2A-5A, 2A-12A, 2A-13A, 2A-17A, 2A-29A, 3A-5A, 3A-8A, 3A-26A, 4A-5A, 4A-12A, 4A-12A, 4A-13A, 4A-17A, 4A-29A, 5A-12A, 5A-13A, 5A-17A, 5A-29A, 5A-30A, 8A-11A, 8A-20A, 12A-30A, 23A-29A, 25A-26A, 29A-30A

When all component carriers are 20 MHz (cases in the first row of the table), observable maximum data rate and target maximum data rate are same. Otherwise, they are different. Since UE category is not deployment scenario dependent, it is logical to separate the observable maximum data rate and the UE category. With the ‘20 MHz per CC’ principle, all the cases in the first column are category 6/7, in the second column category 13, in the third and fourth column category.

<Table 4> Actual maximum data rates for 3 CC CA deployments

| Maximum aggregated bandwidth | E-UTRA CA configuration | Observable maximum data rate (Mbps) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Radio configuration 5: 3x[2,64] | Radio configuration 6: 3x[2,256] | Radio configuration 7: [4,64]+2x[2, 64] | Radio configuration 8: 2x[4, 64]+[2, 64] |
| 60 (20,20,20) | Set 6 | 450 | 600 | 600 | 750 |
| 55 (20,20,15) | Set 7 | 412.5 | 550 | 562.5 | 712.5 |
| 50 (20,20,10) | Set 8 | 375 | 500 | 525 | 675 |
| 50 (20,15,15) |  | 375 | 500 | 525 | 637.5 |
| 45 (20,20,5) | 1A-18A-28A, 1A-19A-28A | 337.5 | 450 | 487.5 | 637.5 |
| 45 (20,15,10) |  | 337.5 | 450 | 487.5 | 600 |
| 45 (15,15,15) |  | 337.5 | 450 | 450 | 562.5 |
| 40 (20,10,10) | Set 9 | 300 | 400 | 450 | 525 |
| 40 (20,15,5) |  | 300 | 400 | 450 | 562.5 |
| 40 (15,15,10) |  | 300 | 400 | 412.5 | 525 |
| 35 (20,10,5) | 2A-12B, 4A-12B | 262.5 | 350 | 412.5 | 487.5 |
| 35 (15,15,5) |  | 262.5 | 350 | 375 | 487.5 |
| 35 (15,10,10) |  | 262.5 | 350 | 375 | 450 |
| UE category according to the current specification |  | UE category 9/10 | Category 11/12 | Category 11/12 | Categor77y 15 |

Set 6: 40D, 41D, 42D, 1A-3C, 1A-41C, 1A-42C, 2A-2A-4A, 2A-4A-4A, 3A-7C, 3C-7A, 3A-40C, 3A-42C, 7C-28A, 7A-40C, 20A-42A-42A, 25A-41C, 38A-40A-40A, 38A-40C, 39A-41C, 41A-42C, 1A-3A-20A, 1A-3A-28A, 1A-3A-42A, 3A-7A-20A, 3A-7A-28A

Set 7: 19A-42C, 21A-42C, 26A-41C, 39C-41A, 1A-3A-19A, 1A-7A-28A, 1A-19A-42A, 1A-21A-42A, 3A-19A-42A

Set 8: 2A-2A-5A, 2C-5A, 2A-2A-12A, 2C-12A, 2A-2A-13A, 2C-29A, 2C-30A, 3C-5A, 3A-3A-8A, 4A-4A-5A, 4A-4A-12A, 4A-4A-13A, 8A-41C, 1A-3A-5A, 1A-3A-8A, 1A-3A-26A, 1A-5A-7A, 1A-7A-20A, 1A-19A-21A, 2A-4A-5A, 2A-4A-12A, 2A-4A-13A, 2A-4A-29A, 2A-4A-30A, 3A-7A-8A, 3A-8A-40A, 19A-21A-42A

Set 9: 3A-7B, 3A-3A-8A, 4A-4A-7A, 7B-28A, 1A-3A-5A, 1A-3A-8A, 1A-5A-7A, 1A-8A-11A, 1A-18A-28A, 2A-5A-12A, 2A-5A-13A, 2A-5A-29A, 2A-5A-30A, 2A-12A-30A, 2A-29A-30A, 3A-7A-8A, 4A-5A-12A, 4A-5A-13A, 4A-5A-29A, 4A-5A-30A, 4A-7A-12A, 4A-12A-30A, 4A-29A-30A, 7A-8A-20A

The principle of relying on ‘20 MHz per CC’ unambiguously classifies that the first column belongs to cat 9/10, the second and the third column cat 11/12 and the last column cat 15. So there is no issue with the fragmented spectrum in the current UE category classification.

Observation 3: UE category is not determined by observable maximum data rate

Above observations do not mean that UE is allowed to declare a fake category. UE category is more about baseband capability and radio capability. If UE declares itself category 11, it shall be able to handle 600 Mbps even when its observable maximum data rate is e.g. 450 Mbps due to fragmented spectrum. When larger spectrum is provided in the future or in the visited PLMN, the observable maximum data rate of the UE shall reach to 600 Mbps.

Observation 4:  UE category is determined by baseband capability and radio capability

Figure 1

Observation 3 and 4 indicate that operators are not restricted in deploying certain UE category because of its fragmented spectrum. For example in the deployment scenario 2 where total bandwidth is not enough for target maximum data rate of category 11/12, operator can still use category 11/12 UE to utilize the enhanced spectral efficiency of 256QAM with the maximum data rate up to the extent the fragmented spectrum allows.

Observation 5: Even if the operators are unable to provide the target maximum data rate of a category due to insufficient spectrum, the operators are free to use the category and the associated radio technologies with the category.

Observation 6: There is no need to introduce category 10/11 with 256QAM to address the fragmented spectrum issue.

Discussion on the lower baseband processing power

One benefit of applying 256QAM for category 9/10 would be the cost reduction by implementing 450Mbps baseband instead of 600Mbps baseband.

<Table 5>

|  | 450 Mbps baseband | 600 Mbps baseband | Cost reduction |
| --- | --- | --- | --- |
| Maximum number of DL-SCH transport block bits received within a TTI | 452,256 bits | 603,008 bits | 25% |
| Total number of soft channel bits | 5,481,216 bits | 7,308,288 bits | 25% |

It should be noted that above two components are not dominant ones in the total modem chip cost. As a matter of fact, the cost per bit is decreasing with the manufacturing processing enhanced with shorter interval (e.g. from 14 ns processing to 10 ns processing).  RF related components like RF paths are more considerable. It is questionable whether 25% reduction from above two components would contribute the cost reduction for modem chip.

Observation 7: Allowing 256QAM for UE category 10/11 leads to cost reduction of baseband related components.

Observation 8: The modem chip cost is dominated more by RF components like RF paths than by baseband components like the number of soft channel bits

One implication of allowing 256QAM for category 10/11 is baseband capability and radio capability of a category could be different. So far, as least for CA categories (i.e. cat 6 ~ 16), radio capability (CA capability, MIMO capability and modulation order) is same as baseband capability as in table 7.

<Table 7>

| Category | Baseband | Radio |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Baseband processing power | Possible radio configurations |  |  | Maximum data rate of the radio configuration |
|  |  | Max. # of CCs | Max # of supported layers of each CC | Highest modulation order of each CC |  |
| 6/7 | 301504 bit per TTI | 1 | 4 | 64 | ≈ 300 Mbps |
|  |  | 2 | 2, 2 | 64, 64 |  |
| 9/10 | 452256 bit per TTI | 2 | 4, 2 | 64, 64 | ≈ 450 Mbps |
|  |  | 3 | 2, 2,2 | 64, 64, 64 |  |
| 11/12 | 603008 bit per TTI | 2 | 4, 4 | 64, 64 | ≈ 600 Mbps |
|  |  | 2 | 4, 2 | 256, 256 |  |
|  |  | 3 | 4, 2, 2 | 64, 64, 64 |  |
|  |  | 3 | 2, 2, 2 | 256, 256,256 |  |
|  |  | 4 | 2, 2, 2, 2 | 64, 64, 64, 64 |  |
| 13 | 391632 bit per TTI | 1 | 4 | 256 | ≈ 400 Mbps |
|  |  | 2 | 2 | 256 |  |

Observation 9: For CA UE categories, the baseband processing power and the maximum data rate of the radio configuration are same

By allowing 256QAM for category 9/10, radio configurations of category 9/10 are extended as below (one can put it as combination of category 9/10 baseband and category 11/12 radio).

<Table 7>

| Category | Baseband | Radio |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Baseband processing power | Possible radio configurations |  |  | Maximum data rate of the radio configuration |
|  |  | Max. # of CCs | Max # of supported layers of each CC | Highest modulation order of each CC |  |
| 9/10 | 452256 bit per TTI | 2 | 4, 2 | 64, 64 | ≈ 450 Mbps |
|  |  | 3 | 2, 2,2 | 64, 64, 64 |  |
|  |  | 2 | 4, 4 | 64, 64 | ≈ 600 Mbps |
|  |  | 2 | 4, 2 | 256, 256 |  |
|  |  | 3 | 4, 2, 2 | 64, 64, 64 |  |
|  |  | 3 | 2, 2, 2 | 256, 256,256 |  |
|  |  | 4 | 2, 2, 2, 2 | 64, 64, 64, 64 |  |

Observation 10:  If 256QAM is allowed for category 10 and lower categories, baseband processing power becomes lower than the maximum data rate of the radio configuration

Figure 2 summarize the difference between new category 9/10 supporting 256QAM and category 11/12.

Figure 2

Observation 11: Category 11/12 and Category 9/10 with 256QAM are same except the baseband processing power.

Conclusion

Regarding UE categories, following observations are made.

Observation 1: According to the current specification, UE shall meet the maximum data rate of its category in at least one supported band combination

Observation 2: According to the current specification, UE not meeting the maximum data rate of its category due to fragmented spectrum problem (i.e. full bandwidth is not provided in the reference band combination) still declare the category if the maximum data rate were reached with 20 MHz per component carrier.

Observation 3: UE category is not determined by observable maximum data rate

Observation 4:  UE category is about baseband capability and radio capability of the UE

Regarding UE categories and fragmented spectrums, following observations are made.

Observation 5: Even if the operators are unable to provide the target maximum data rate of a category due to insufficient spectrum, the operators are free to use the category and the associated radio technologies with the category.

Observation 6: There is no need to introduce category 10/11 with 256QAM to address the fragmented spectrum issue.

Regarding UE categories with lower baseband processing power, following observations are made.

Observation 7: Allowing 256QAM for UE category 10/11 leads to cost reduction of baseband related components.

Observation 8: The modem chip cost is dominated more by RF components like RF paths than by baseband components like number of soft channel bits

Observation 9: For CA UE categories, the baseband processing power and the maximum data rate of the radio configuration are same

Observation 10:  If 256QAM is allowed for category 10 and lower categories, baseband processing power becomes lower than the maximum data rate of the radio configuration

Observation 11: Category 11/12 and Category 9/10 with 256QAM are same except the baseband processing power.

Table below summarizes the pros and cons of such UE category (baseband processing power < maximum data rate of the radio configuration)

| Pros | Cons |
| --- | --- |
| Cheaper modem chip (by downgrading baseband processing power)* | Market fragmentation (the variants of UE implementations would increase) eNB scheduling complexity (ENB shall put scheduling constraints for such UEs to not schedule them with full bandwidth) |

Based on the observations, followings are proposed.

Proposal 1: RAN is requested to confirm that UE category is not determined by observable maximum data rate.

Proposal 2: RAN is requested to confirm that UE category is determined by baseband capability (e.g. Maximum number of DL-SCH transport block bits received within a TTI) and radio capability (e.g. CA capability, MIMO capability and the highest modulation order).

Proposal 3: RAN is requested to discuss whether UE implementation with lower baseband processing power than its radio capability should be allowed in 3GPP.

- Reference

[1]		R2-156199	Support of 256QAM for UE categories 10 and below		Intel

[2]	R2-121005	Clarification on physical layer parameter values requirement	Samsung, NTT DOCOMO, INC., Qualcomm Incorporated

[3]		TS 36.101	User Equipment (UE) radio transmission and reception