3GPP TSG-RAN4 Meeting #77 	R4-157842

# Anaheim, CA, US, 16 – 20 November 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.141 | CR | 0749 | rev | - | Current version: | 11.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on definition of f_offsetmax for BS operating in multiple bands or non-contiguous spectrum |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Ericsson, Nokia Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2015-11-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | With the current definition of ‘f_offsetmax is equal to the inter RF bandwidth / sub-block gap divided by two’, the UEM is only specified up to the middle of the inter RF bandwidth / sub-block gap from each RF bandwidth / sub-block edge. E.g. for an RF bandwidth gap of 15 MHz, the UEM is specified up to 7.5 MHz from each RF bandwidth edge, but the UEM 7.5 MHz away from each RF bandwidth edge is not specified. Hence the cumulative approach cannot be applied as there is no overlapping of the UEM within the inter RF bandwidth / sub-block gap according to the current definition of f_offsetmax. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the defintion of f_offsetmax to ‘f_offsetmax is equal to the inter RF bandwidth / sub-block gap minus half of the bandwidth of the measuring filter’. Also clarify where necessary the contribution from the far-end RF bandwidth / sub-block shall be scaled according to the measurement bandwidth of the near-end RF bandwidth / sub-block. Also clarify that the emission within the inter RF bandwidth / sub-block gap shall be measured using the specified measurement bandwidth from the closest RF bandwidth / sub block edge. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error remains, and the cumulative approach cannot be applied for the UEM requirements within the inter RF bandwidth / sub-block gap. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.1.4.2, 6.5.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 25.104 CR 0727 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

##### 6.5.2.1.4.2	Procedures

1)	For a BS declared to be capable of single carrier operation only, set the base station to transmit a signal according to TM1, clause 6.1.1.1, at manufacturer’s declared rated output power, Prated,c.

For a BS declared to be capable of multi-carrier operation, set the base station to transmit according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in sub-clause 4.12.

2)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth. For BS operating in multiple bands or non-contiguous spectrum, the emission within the Inter RF Bandwidth or sub-block gap shall be measured using the specified measurement bandwidth from the closest RF Bandwidth or sub block edge.

In addition, for a multi-band capable BS, the following step shall apply:

3)	For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band. For multi-band capable BS with separate antenna connector, the antenna connector not being under test in case of single-band or multi-band test shall be terminated.

##### 6.5.2.1.5	Test requirements

The measurement results in step 2 of 6.5.2.1.4.2 shall not exceed the test requirements specified in tables 6.18 to 6.21 for the appropriate BS maximum output power, where:

-	f is the separation between the carrier frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the carrier frequency and the centre of the measurement filter;

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in clause 3.4.1, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Inside any Inter RF Bandwidth gaps with Wgap < 20 MHz for BS operating in multiple bands, emissions shall not exceed the cumulative sum of the test requirements specified at the Base Station RF Bandwidth edges on each side of the Inter RF Bandwidth gap. The test requirement for Base Station RF Bandwidth edge is specified in Tables 6.18 to 6.21E below, where in this case:

-	f is equal to 2.5MHz plus the separation between the Base Station RF Bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the Base Station RF Bandwidth edge.

-	f_offset is equal to 2.5MHz plus the separation between the Base Station RF Bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is either 12.5 MHz or the offset to the UMTS Tx band edge as defined in section 5.2, whichever is the greater.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Inside any sub-block gap for a BS operating in non-contiguous spectrum, the measurement results shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The test requirement for each sub block is specified in Tables 6.18 to 6.21E below, where in this case:

-	f is equal to 2.5MHz plus the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is equal to 2.5MHz plus the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter plus 2.5MHz.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.18: Spectrum emission mask values, BS maximum output power P  43 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -12.5 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | -24.5 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | -11.5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -11.5 dBm | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.18A: Spectrum emission mask values, BS maximum output power P  43 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -12.2 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | -24.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | -11.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0 MHz f_offset < f_offsetmax | -11.2 dBm | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.19: Spectrum emission mask values, BS maximum output power 39  P < 43 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -12.5 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | -24.5 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | -11.5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | P - 54.5 dB | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in clause 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.19A: Spectrum emission mask values, BS maximum output power 39  P < 43 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -12.2 dBm | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | -24.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | -11.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | P - 54.2 dB | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in clause 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.20: Spectrum emission mask values, BS maximum output power 31  P < 39 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point,f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | P - 51.5 dB | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | P - 63.5 dB | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | P - 50.5 dB | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | P - 54.5 dB | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.20A: Spectrum emission mask values, BS maximum output power 31  P < 39 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point,f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | P - 51.2 dB | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | P - 63.2 dB | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | P - 50.2 dB | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | P - 54.2 dB | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.21: Spectrum emission mask values, BS maximum output power P < 31 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -20.5 dBm | 30 kHz |
| 2.7 f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | -32.5 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | -19.5 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | -23.5 dBm | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.21a: Spectrum emission mask values, BS maximum output power P < 31 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test Requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -20.2 dBm | 30 kHz |
| 2.7 f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | -32.2 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | -19.2 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | -23.2 dBm | 1 MHz |
| NOTE 1: For BS supporting non-contiguous spectrum operation the test requirement within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission test requirements in 6.5.3.7.1 and 6.5.3.7.2 shall be met. NOTE 2: For BS supporting multi-band operation with Inter RF Bandwidth gap < 20MHz the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

For operation in band II, IV, V, X, XII, XIII, XIV, XXV and XXVI, the applicable additional requirement in Tables 6.21A, 6.21B or 6.21C  apply in addition to the minimum requirements in Tables 6.18 to 6.21.

Table 6.21A: Additional spectrum emission limits for Bands II, IV, X, XXV

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 3.5 MHz | 2.515MHz f_offset < 3.515MHz | -15 dBm | 30 kHz |
| 3.5 MHz f fmax | 4.0MHz f_offset < f_offsetmax | -13 dBm | 1 MHz |

Table 6.21B: Additional spectrum emission limits for Bands V, XXVI

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 3.5 MHz | 2.515MHz f_offset < 3.515MHz | -15 dBm | 30 kHz |
| 3.5 MHz f fmax | 3.55MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |

Table 6.21C: Additional spectrum emission limits for Bands XII, XIII,  XIV

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.6 MHz | 2.515MHz f_offset < 2.615MHz | -13 dBm | 30 kHz |
| 2.6 MHz f fmax | 2.65MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |

For Home BS, the applicable additional requirements in Tables 6.21D or 6.21E apply in addition to the minimum requirements in Tables 6.18 to 6.21.

Table 6.21D: Additional spectrum emission limit for Home BS, BS maximum output power 6 ≤ P ≤ 20 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 12.5 MHz f fmax | 13MHz f_offset < f_offsetmax | P – 54.5dBm | 1 MHz |

Table 6.21Da: Additional spectrum emission limit for Home BS, BS maximum output power 6 ≤ P ≤ 20 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 12.5 MHz f fmax | 13MHz f_offset < f_offsetmax | P – 54.2dBm | 1 MHz |

Table 6.21E: Additional spectrum emission limit for Home BS, BS maximum output power P < 6 dBm for UTRA FDD bands ≤ 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 12.5 MHz f fmax | 13MHz f_offset < f_offsetmax | -48.5 dBm | 1 MHz |

Table 6.21Ea: Additional spectrum emission limit for Home BS, BS maximum output power P < 6 dBm for UTRA FDD bands > 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Additional requirement | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 12.5 MHz f fmax | 13MHz f_offset < f_offsetmax | -48.2 dBm | 1 MHz |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test is defined in clause 4.2 and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in Annex F.

In certain regions the following requirement may apply for protection of DTT. For UTRA BS operating in Band XX, the level of emissions in the band 470-790 MHz, measured in an 8MHz filter bandwidth on centre frequencies Ffilter according to Table 6.21F, shall not exceed the  maximum emission level PEM,N declared by the manufacturer.

Table 6.21F: Declared emissions levels for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission level [dBm] |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

NOTE: 	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the basestation needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in Annex D of [1].

Notes for the Tables in this subclause:

NOTE 3:	This frequency range ensures that the range of values of f_offset is continuous.

NOTE 4:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth can be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

<End of change>