3GPP TSG- RAN4 Meeting #77 	R4-157853

# Anaheim, CA, US, 16 – 20 November 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0436 | rev | - | Current version: | 11.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on definition of f_offsetmax for BS operating in multiple bands or non-contiguous spectrum |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Ericsson, Nokia Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Enh-Perf |  |  |  |  |  |  | Date: |  | 2015-11-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | With the current definition of ‘f_offsetmax is equal to the inter RF bandwidth / sub-block gap divided by two’, the UEM is only specified up to the middle of the inter RF bandwidth / sub-block gap from each RF bandwidth / sub-block edge. E.g. for an RF bandwidth gap of 15 MHz, the UEM is specified up to 7.5 MHz from each RF bandwidth edge, but the UEM 7.5 MHz away from each RF bandwidth edge is not specified. Hence the cumulative approach cannot be applied as there is no overlapping of the UEM within the inter RF bandwidth / sub-block gap according to the current definition of f_offsetmax. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the defintion of f_offsetmax to ‘f_offsetmax is equal to the inter RF bandwidth / sub-block gap minus half of the bandwidth of the measuring filter’. Also clarify where necessary the contribution from the far-end RF bandwidth / sub-block shall be scaled according to the measurement bandwidth of the near-end RF bandwidth / sub-block. Also clarify that the emission within the inter RF bandwidth / sub-block gap shall be measured using the specified measurement bandwidth from the closest RF bandwidth / sub block edge. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error remains, and the cumulative approach cannot be applied for the UEM requirements within the inter RF bandwidth / sub-block gap. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.4.2, 6.6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 37.104 CR 0275 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

##### 6.6.2.4.2	Procedure

1)	Set the Base Station to transmit at maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.9.2.

2)	Step the centre frequency of the measurement filter in contiguous steps and measure the emission within the specified frequency ranges with the specified measurement bandwidth. For BS operating in multiple bands or non-contiguous spectrum, the emission within the Inter RF Bandwidth or sub-block gap shall be measured using the specified measurement bandwidth from the closest RF Bandwidth or sub block edge.

3)	Repeat the test for the remaining test cases with channel set-up according to clause 5 and subclause 4.9.2.

In addition, for a multi-band capable BS, the following steps shall apply:

4)	For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

5)	For multi-band capable BS with separate antenna connector, the antenna connector not being under test in case of SBT or MBT shall be terminated.

#### 6.6.2.5	Test requirement

##### 6.6.2.5.1	Test requirements for Band Categories 1 and 3

For a Wide Area BS operating in Band Category 1 or Band Category 3, the requirement applies outside the RF bandwidth edges. In addition, for a Wide Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for a Wide Area BS operating in multiple bands, it applies inside any inter RF bandwidth gap.

For a Medium Range BS operating in Band Category 1 the requirement applies outside the RF bandwidth edges. In addition, for a Medium Range BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for a Medium Range BS operating in multiple bands, it applies inside any inter RF bandwidth gap.

For a Local Area BS operating in Band Category 1 the requirement applies outside the RF bandwidth edges. In addition, for a Local Area BS operating in non-contiguous spectrum, it applies inside any sub-block gap. In addition, for a Local Area BS operating in multiple bands, it applies inside any inter RF bandwidth gap.

Outside the RF bandwidth edges, emissions shall not exceed the maximum levels specified in Tables 6.6.2.5.1-1 to 6.6.2.5.1-4 below, where:

-	f is the separation between the RF bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the RF bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a BS operating in multiple bands, inside any inter-RF bandwidth gaps with Wgap < 20 MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the RF bandwidth edges on each side of the inter-RF bandwidth gap. The test requirement for RF bandwidth edge is specified in Table 6.6.2.5.1-1 to 6.6.2.5.1-4 below, where in this case:

-	f is the separation between the RF bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the RF bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the inter RF bandwidth gap minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Inside any sub-block gap for a BS operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the test requirements specified for the adjacent sub blocks on each side of the sub block gap. The test requirement for each sub block is specified in Tables 6.6.2.5.1-1 to 6.6.2.5.1-4 below, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge frequency.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.6.2.5.1-1: Wide Area BS operating band unwanted emission mask (UEM) for BC1 and BC3 bands ≤ 3GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015MHz f_offset < 0.215MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 3) | 1.015MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -11.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -15dBm/MHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.1-1a: Wide Area BS operating band unwanted emission mask (UEM) for BC1 and BC3 for bands > 3GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz | 0.015MHz f_offset < 0.215MHz | -12.2 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 3) | 1.015MHz f_offset < 1.5 MHz | -24.2 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -11.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 5) | 1 MHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -15dBm/MHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.1-2: Medium Range BS operating band unwanted emission mask (UEM) for BC1 for bands ≤ 3GHz, BS maximum output power 31 < Pmax,c  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 3) | 1.015MHz f_offset < 1.5 MHz | Pmax,c – 63.5 dB | 30 kHz |
| 1 MHz f 2.6 MHz | 1.5 MHz f_offset < 3.1 MHz | Pmax,c – 50.5 dB | 1 MHz |
| 2.6 MHz f 5 MHz | 3.1 MHz f_offset < 5.5 MHz | min(Pmax,c – 50.5 dB, -13.5dBm) | 1 MHz |
| 5 MHz f min(fmax, 10MHz) | 5.5 MHz f_offset < min (f_offsetmax, 10.5 MHz) | Pmax,c – 54.5 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Pmax,c-56dB (Note 5) | 1MHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be (Pmax,c – 56 dB)/MHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.1-2a: Medium Range BS operating band unwanted emission mask (UEM) for BC1 for bands > 3GHz, BS maximum output power 31 < Pmax,c  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 3) | 1.015MHz f_offset < 1.5 MHz | Pmax,c – 63.2 dB | 30 kHz |
| 1 MHz f 2.6 MHz | 1.5 MHz f_offset < 3.1 MHz | Pmax,c – 50.2 dB | 1 MHz |
| 2.6 MHz f 5 MHz | 3.1 MHz f_offset < 5.5 MHz | min(Pmax,c – 50.2 dB, -13.2dBm) | 1 MHz |
| 5 MHz f min(fmax, 10MHz) | 5.5 MHz f_offset < min(f_offsetmax ,10.5MHz) | Pmax,c – 54.2 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Pmax,c-56dB (Note 5) | 1MHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be (Pmax,c – 56 dB)/MHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.1-3: Medium Range BS operating band unwanted emission mask (UEM) for BC1 for bands≤ 3GHz, BS maximum output power Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 3) | 1.015MHz f_offset < 1.5 MHz | -32.5 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -19.5 dBm | 1 MHz |
| 5 MHz f min(fmax,10MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5MHz) | -23.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -25 dBm (Note 5) | 1MHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -25dBm/MHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.1-3a: Medium Range BS operating band unwanted emission mask (UEM) for BC1 for bands> 3GHz, BS maximum output power Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 3) | 1.015MHz f_offset < 1.5 MHz | -32.2 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -19.2 dBm | 1 MHz |
| 5 MHz f min(fmax,10MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5MHz) | -23.2 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -25 dBm (Note 5) | 1MHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -25dBm/MHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

# Table 6.6.2.5.1-4: Local Area operating band unwanted emission mask (UEM) for BC1 for bands  3GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -37dBm/100 kHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap. |  |  |  |

# Table 6.6.2.5.1-4a: Local Area operating band unwanted emission mask (UEM) for BC1 for bands > 3GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 5) | 100 kHz |
| NOTE 1: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -37dBm/100 kHz. NOTE2: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap. |  |  |  |

NOTE 3:	This frequency range ensures that the range of values of f_offset is continuous.

NOTE 4:	As a general rule for the requirements in the present subclause, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 5:	The requirement is not applicable when fmax < 10 MHz.

##### 6.6.2.5.2	Test requirements for Band Category 2

For a BS operating in Band Category 2 the requirement applies outside the RF bandwidth edges. In addition, for a BS operating in non-contiguous spectrum, it applies inside any sub-block gap.

Outside the RF bandwidth edges, emissions shall not exceed the maximum levels specified in Table 6.6.2.5.2-1 to 6.6.2.5.2-8 below, where:

-	f is the separation between the RF bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the RF bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is the offset to the frequency 10 MHz outside the downlink operating band.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

For a BS operating in multiple bands, inside any inter-RF bandwidth gaps with Wgap < 20 MHz, emissions shall not exceed the cumulative sum of the test requirements specified at the RF bandwidth edges on each side of the inter-RF bandwidth gap. The test requirement for RF bandwidth edge is specified in Table 6.6.2.5.2-1 to 6.6.2.5.2-8 below, where in this case:

-	f is the separation between the RF bandwidth edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the RF bandwidth edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the inter RF bandwidth gap minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Inside any sub-block gap for a BS operating in non-contiguous spectrum, emissions shall not exceed the cumulative sum of the test requirement specified for the adjacent sub blocks on each side of the sub block gap. The test requirement for each sub block is specified in Tables 6.6.2.5.2-1 to 6.6.2.5.2-8 below, where in this case:

-	f is the separation between the sub block edge frequency and the nominal -3 dB point of the measuring filter closest to the sub block edge.

-	f_offset is the separation between the sub block edge frequency and the centre of the measuring filter.

-	f_offsetmax is equal to the sub block gap bandwidth minus half of the bandwidth of the measuring filter.

-	fmax is equal to f_offsetmax minus half of the bandwidth of the measuring filter.

Table 6.6.2.5.2-1: Wide Area BS operating band unwanted emission mask (UEM) for BC2

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 2, 3) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 0.2 MHz (Note 1) | 0.015 MHz f_offset < 0.215 MHz | -12.5 dBm | 30 kHz |
| 0.2 MHz f < 1 MHz | 0.215 MHz f_offset < 1.015 MHz |  | 30 kHz |
| (Note 8) | 1.015 MHz f_offset < 1.5 MHz | -24.5 dBm | 30 kHz |
| 1 MHz f min(fmax, 10 MHz) | 1.5 MHz f_offset < min(f_offsetmax, 10.5 MHz) | -11.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -15 dBm (Note 10) | 1 MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge, the limits in Table 6.6.2.5-2 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -15dBm/MHz. NOTE3: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz operation the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.2-2: Wide Area BS operating band unwanted emission limits for operation in BC2 with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the RF bandwidth edge

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 5, 6, 7) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| NOTE 4: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge. NOTE 5: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 6: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap. NOTE 7: In case the carrier adjacent to the RF bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier – 43, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the RF bandwidth edge. In other cases, X = 0. |  |  |  |

Table 6.6.2.5.2-3: Medium Range BS operating band unwanted emission mask (UEM) for BC2, BS maximum output power 31 < Pmax,c  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 2, 3) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 8) | 1.015MHz f_offset < 1.5 MHz | Pmax,c – 63.5 dB | 30 kHz |
| 1 MHz f 2.8 MHz | 1.5 MHz f_offset < 3.3 MHz | Pmax,c – 50.5 dB | 1 MHz |
| 2.8 MHz f 5 MHz | 3.3 MHz f_offset < 5.5 MHz | min(Pmax,c – 50.5 dB, -13.5dBm) | 1 MHz |
| 5 MHz f min(fmax, 10 MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5MHz) | Pmax,c – 54.5 dB | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | Pmax,c-56dB (Note 10) | 1MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge, the limits in Table 6.6.2.5.2-5 apply for 0 MHz f < 0.15 MHz. NOTE 2: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be (Pmax,c – 56 dB)/MHz. NOTE 3: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.2-4: Medium Range BS operating band unwanted emission mask (UEM) for BC2, BS maximum output power Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 2, 3) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 0.6 MHz (Note 1) | 0.015MHz f_offset < 0.615MHz |  | 30 kHz |
| 0.6 MHz f < 1 MHz | 0.615MHz f_offset < 1.015MHz |  | 30 kHz |
| (Note 8) | 1.015MHz f_offset < 1.5 MHz | -32.5 dBm | 30 kHz |
| 1 MHz f 5 MHz | 1.5 MHz f_offset < 5.5 MHz | -19.5 dBm | 1 MHz |
| 5 MHz f min(fmax,10MHz) | 5.5 MHz f_offset < min(f_offsetmax,10.5MHz) | -23.5 dBm | 1 MHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -25dBm (Note 10) | 1MHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge, the limits in Table 6.6.2.5.2-6 apply for 0 MHz f < 0.15MHz. NOTE 2: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -25dBm/MHz. NOTE 3: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.6.2.5.2-5: Medium Range operating band unwanted emission limits for operation in BC2 with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the RF bandwidth edge, BS maximum output power 31 < Pmax,c  38 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 5, 6) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| NOTE 4: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge. NOTE 5: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 6: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap. |  |  |  |

Table 6.6.2.5.2-6: Medium Range operating band unwanted emission limits for operation in BC2 with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the RF bandwidth edge, BS maximum output power Pmax,c  31 dBm

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 5, 6, 7) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.15 MHz | 0.065 MHz f_offset < 0.165 MHz |  | 30 kHz |
| NOTE 4: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge. NOTE 5: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 6: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap. NOTE 7: In case the carrier adjacent to the RF bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier – 31, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the RF bandwidth edge. In other cases, X = 0. |  |  |  |

# Table 6.6.2.5.2-7: Local Area operating band unwanted emission mask (UEM) for BC2

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 2, 3) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz (Note 1) | 0.05 MHz f_offset < 5.05 MHz |  | 100 kHz |
| 5 MHz f < min(10 MHz, Δfmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -35.5 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -37 dBm (Note 7) | 100 kHz |
| NOTE 1: For operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge, the limits in Table 6.6.2.5.2-8 apply for 0 MHz f < 0.16 MHz. NOTE 2: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the test requirement within sub-block gaps shall be -37dBm/100 kHz. NOTE 3: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap. |  |  |  |

Table 6.6.2.5.2-8: Local Area operating band unwanted emission limits for operation in BC2 with GSM/EDGE or E-UTRA 1.4 or 3 MHz carriers adjacent to the RF bandwidth edge

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Test requirement (Note 5, 6, 7) | Measurement bandwidth (Note 9) |
| --- | --- | --- | --- |
| 0 MHz f < 0.05 MHz | 0.015 MHz f_offset < 0.065 MHz |  | 30 kHz |
| 0.05 MHz f < 0.16 MHz | 0.065 MHz f_offset < 0.175 MHz |  | 30 kHz |
| NOTE 4: The limits in this table only apply for operation with a GSM/EDGE or an E-UTRA 1.4 or 3 MHz carrier adjacent to the RF bandwidth edge. NOTE 5: For MSR BS supporting non-contiguous spectrum operation within any operating band the test requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. NOTE 6: For MSR BS supporting multi-band operation with inter RF bandwidth gap < 20MHz the test requirement within the inter RF bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the inter RF bandwidth gap. NOTE 7: In case the carrier adjacent to the RF bandwidth edge is a GSM/EDGE carrier, the value of X = PGSMcarrier – 24, where PGSMcarrier is the power level of the GSM/EDGE carrier adjacent to the RF bandwidth edge. In other cases, X = 0. |  |  |  |

NOTE 8:	This frequency range ensures that the range of values of f_offset is continuous.

NOTE 9:	As a general rule for the requirements in the present subclause, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 10:	The requirement is not applicable when fmax < 10 MHz.

<End of change>