# 3GPP TSG-RAN WG4 Meeting #77                                                           	R4-156972

# Anaheim, CA, US, 16 – 20 Nov, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 709 | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of intra-band NC CA_5A-5A to TS 36.104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_NC_B5-Core |  |  |  |  |  |  | Date: |  | 2015-11-16 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The requirements for the WI LTE_CA_NC_B5 are to be implemented in Rel-13 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Intra-band NC contiguous carrier aggregation for Band 5 is added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_5A-5A will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.141 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### ---Start of changes---

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA frequency bands

| E-UTRA Operating Band |  |  |  |  |  | Uplink (UL) operating band BS receive UE transmit |  |  |  |  |  |  |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  |  |  |  |  |  |  | Duplex Mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | FUL_low – FUL_high |  |  |  |  |  |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  | 1920 MHz |  |  | – |  | 1980 MHz |  |  |  |  | 2110 MHz |  |  |  | – |  |  | 2170 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 2 |  |  |  |  |  | 1850 MHz |  |  | – |  | 1910 MHz |  |  |  |  | 1930 MHz |  |  |  | – |  |  | 1990 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 3 |  |  |  |  |  | 1710 MHz |  |  | – |  | 1785 MHz |  |  |  |  | 1805 MHz |  |  |  | – |  |  | 1880 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 4 |  |  |  |  |  | 1710 MHz |  |  | – |  | 1755 MHz |  |  |  |  | 2110 MHz |  |  |  | – |  |  | 2155 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 5 |  |  |  |  |  | 824 MHz |  |  | – |  | 849 MHz |  |  |  |  | 869 MHz |  |  |  | – |  |  | 894MHz |  |  |  |  | FDD |  |  |  |  |  |
| 6 (NOTE 1) |  |  |  |  |  | 830 MHz |  |  | – |  | 840 MHz |  |  |  |  | 875 MHz |  |  |  | – |  |  | 885 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 7 |  |  |  |  |  | 2500 MHz |  |  | – |  | 2570 MHz |  |  |  |  | 2620 MHz |  |  |  | – |  |  | 2690 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 8 |  |  |  |  |  | 880 MHz |  |  | – |  | 915 MHz |  |  |  |  | 925 MHz |  |  |  | – |  |  | 960 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 9 |  |  |  |  |  | 1749.9 MHz |  |  | – |  | 1784.9 MHz |  |  |  |  | 1844.9 MHz |  |  |  | – |  |  | 1879.9 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 10 |  |  |  |  |  | 1710 MHz |  |  | – |  | 1770 MHz |  |  |  |  | 2110 MHz |  |  |  | – |  |  | 2170 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 11 |  |  |  |  |  | 1427.9 MHz |  |  | – |  | 1447.9 MHz |  |  |  |  | 1475.9 MHz |  |  |  | – |  |  | 1495.9 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 12 |  |  |  |  |  | 699 MHz |  |  | – |  | 716 MHz |  |  |  |  | 729 MHz |  |  |  | – |  |  | 746 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 13 |  |  |  |  |  | 777 MHz |  |  | – |  | 787 MHz |  |  |  |  | 746 MHz |  |  |  | – |  |  | 756 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 14 |  |  |  |  |  | 788 MHz |  |  | – |  | 798 MHz |  |  |  |  | 758 MHz |  |  |  | – |  |  | 768 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 15 |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  | FDD |  |  |  |  |  |
| 16 |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  | Reserved |  |  |  |  |  |  |  |  |  |  |  | FDD |  |  |  |  |  |
| 17 |  |  |  |  |  | 704 MHz |  |  | – |  | 716 MHz |  |  |  |  | 734 MHz |  |  |  | – |  |  | 746 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 18 |  |  |  |  |  | 815 MHz |  |  | – |  | 830 MHz |  |  |  |  | 860 MHz |  |  |  | – |  |  | 875 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 19 |  |  |  |  |  | 830 MHz |  |  | – |  | 845 MHz |  |  |  |  | 875 MHz |  |  |  | – |  |  | 890 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 20 |  |  |  |  |  | 832 MHz |  |  | – |  | 862 MHz |  |  |  |  | 791 MHz |  |  |  | – |  |  | 821 MHz |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  | 1447.9 MHz |  |  | – |  | 1462.9 MHz |  |  |  |  | 1495.9 MHz |  |  |  | – |  |  | 1510.9 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 22 |  |  |  |  |  | 3410 MHz |  |  | – |  | 3490 MHz |  |  |  |  | 3510 MHz |  |  |  | – |  |  | 3590 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 23 |  |  |  |  |  | 2000 MHz |  |  | – |  | 2020 MHz |  |  |  |  | 2180 MHz |  |  |  | – |  |  | 2200 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 24 |  |  |  |  |  | 1626.5 MHz |  |  | – |  | 1660.5 MHz |  |  |  |  | 1525 MHz |  |  |  | – |  |  | 1559 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 25 |  |  |  |  |  | 1850 MHz |  |  | – |  | 1915 MHz |  |  |  |  | 1930 MHz |  |  |  | – |  |  | 1995 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 26 |  |  |  |  |  | 814 MHz |  |  | – |  | 849 MHz |  |  |  |  | 859 MHz |  |  |  | – |  |  | 894 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 27 |  |  |  |  |  | 807 MHz |  |  | – |  | 824 MHz |  |  |  |  | 852 MHz |  |  |  | – |  |  | 869 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 28 |  |  |  |  |  | 703 MHz |  |  | – |  | 748 MHz |  |  |  |  | 758 MHz |  |  |  | – |  |  | 803 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 29 |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  | 717 MHz |  |  |  | – |  |  | 728 MHz |  |  |  |  | FDD (NOTE 2) |  |  |  |  |  |
| 30 |  |  |  |  |  | 2305 MHz |  |  | – |  | 2315 MHz |  |  |  |  | 2350 MHz |  |  |  | – |  |  | 2360 MHz |  |  |  |  | FDD |  |  |  |  |  |
| 31 |  |  |  |  |  | 452.5 MHz |  | – |  | 457.5 MHz |  |  |  |  |  | 462.5 MHz |  |  | – |  |  | 467.5 MHz |  |  |  |  |  | FDD |  |  |  |  |  |
| 32 |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  |  | 1452 MHz |  |  | – |  |  | 1496 MHz |  |  |  |  |  | FDD (NOTE 2) |  |  |  |  |  |
| 33 |  |  |  |  |  | 1900 MHz |  |  | – |  | 1920 MHz |  |  |  |  | 1900 MHz |  |  |  | – |  |  | 1920 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 34 |  |  |  |  |  | 2010 MHz |  |  | – |  | 2025 MHz |  |  |  |  | 2010 MHz |  |  |  | – |  |  | 2025 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 35 |  |  |  |  |  | 1850 MHz |  |  | – |  | 1910 MHz |  |  |  |  | 1850 MHz |  |  |  | – |  |  | 1910 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 36 |  |  |  |  |  | 1930 MHz |  |  | – |  | 1990 MHz |  |  |  |  | 1930 MHz |  |  |  | – |  |  | 1990 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 37 |  |  |  |  |  | 1910 MHz |  |  | – |  | 1930 MHz |  |  |  |  | 1910 MHz |  |  |  | – |  |  | 1930 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 38 |  |  |  |  |  | 2570 MHz |  |  | – |  | 2620 MHz |  |  |  |  | 2570 MHz |  |  |  | – |  |  | 2620 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 39 |  |  |  |  |  | 1880 MHz |  |  | – |  | 1920 MHz |  |  |  |  | 1880 MHz |  |  |  | – |  |  | 1920 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 40 |  |  |  |  |  | 2300 MHz |  |  | – |  | 2400 MHz |  |  |  |  | 2300 MHz |  |  |  | – |  |  | 2400 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 41 |  |  |  |  |  | 2496 MHz |  |  | – |  | 2690 MHz |  |  |  |  | 2496 MHz |  |  |  | – |  |  | 2690 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 42 |  |  |  |  |  | 3400 MHz |  |  | – |  | 3600 MHz |  |  |  |  | 3400 MHz |  |  |  | – |  |  | 3600 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 43 |  |  |  |  |  | 3600 MHz |  |  | – |  | 3800 MHz |  |  |  |  | 3600 MHz |  |  |  | – |  |  | 3800 MHz |  |  |  |  | TDD |  |  |  |  |  |
| 44 |  |  |  |  |  | 703 MHz |  |  | – |  | 803 MHz |  |  |  |  | 703 MHz |  |  |  | – |  |  | 803 MHz |  |  |  |  | TDD |  |  |  |  |  |
| NOTE 1: Band 6 is not applicable. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

E-UTRA is designed to operate for the carrier aggregation bands defined in Tables 5.5-2 to 5.5-4.

Table 5.5-2 Intra-band contiguous carrier aggregation bands

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_1 | 1 |
| CA_2 | 2 |
| CA_3 | 3 |
| CA_7 | 7 |
| CA_8 | 8 |
| CA_12 | 12 |
| CA_23 | 23 |
| CA_27 | 27 |
| CA_38 | 38 |
| CA_39 | 39 |
| CA_40 | 40 |
| CA_41 | 41 |
| CA_42 | 42 |

Table 5.5-3. Inter-band carrier aggregation bands (two bands)

| CA Band | E-UTRA operating bands |
| --- | --- |
| CA_1-3 | 1 |
|  | 3 |
| CA_1-5 | 1 |
|  | 5 |
| CA_1-7 | 1 |
|  | 7 |
| CA_1-8 | 1 |
|  | 8 |
| CA_1-11 | 1 |
|  | 11 |
| CA_1-18 | 1 |
|  | 18 |
| CA_1-19 | 1 |
|  | 19 |
| CA_1-20 | 1 |
|  | 20 |
| CA_1-21 | 1 |
|  | 21 |
| CA_1-26 | 1 |
|  | 26 |
| CA_1-28 | 1 |
|  | 28 |
| CA_1-40 | 1 |
|  | 40 |
| CA_1-41 | 1 |
|  | 41 |
| CA_1-42 | 1 |
|  | 42 |
| CA_2-4 | 2 |
|  | 4 |
| CA_2-2-4 | 2 |
|  | 4 |
| CA_2-2-4-4 | 2 |
|  | 4 |
| CA_2-4-4 | 2 |
|  | 4 |
| CA_2-5 | 2 |
|  | 5 |
| CA_2-2-5 | 2 |
|  | 5 |
| CA_2-12 | 2 |
|  | 12 |
| CA_2-2-12 | 2 |
|  | 12 |
| CA_2-13 | 2 |
|  | 13 |
| CA_2-2-13 | 2 |
|  | 13 |
| CA_2-17 | 2 |
|  | 17 |
| CA_2-28 | 2 |
|  | 28 |
| CA_2-29 | 2 |
|  | 29 |
| CA_2-30 | 2 |
|  | 30 |
| CA_3-5 | 3 |
|  | 5 |
| CA_3-7 | 3 |
|  | 7 |
| CA_3-8 | 3 |
|  | 8 |
| CA_3-3-8 | 3 |
|  | 8 |
| CA_3-19 | 3 |
|  | 19 |
| CA_3-20 | 3 |
|  | 20 |
| CA_3-26 | 3 |
|  | 26 |
| CA_3-27 | 3 |
|  | 27 |
| CA_3-28 | 3 |
|  | 28 |
| CA_3-31 | 3 |
|  | 31 |
| CA_3-38 | 3 |
|  | 38 |
| CA_3-40 | 3 |
|  | 40 |
| CA_3-42 | 3 |
|  | 42 |
| CA_4-5 | 4 |
|  | 5 |
| CA_4-4-5 | 4 |
|  | 5 |
| CA_4-7 | 4 |
|  | 7 |
| CA_4-4-7 | 4 |
|  | 7 |
| CA_4-12 | 4 |
|  | 12 |
| CA_4-4-12 | 4 |
|  | 12 |
| CA_4-13 | 4 |
|  | 13 |
| CA_4-4-13 | 4 |
|  | 13 |
| CA_4-17 | 4 |
|  | 17 |
| CA_4-27 | 4 |
|  | 27 |
| CA_4-28 | 4 |
|  | 28 |
| CA_4-29 | 4 |
|  | 29 |
| CA_4-30 | 4 |
|  | 30 |
| CA_5-7 | 5 |
|  | 7 |
| CA_5-12 | 5 |
|  | 12 |
| CA_5-13 | 5 |
|  | 13 |
| CA_5-17 | 5 |
|  | 17 |
| CA_5-25 | 5 |
|  | 25 |
| CA_5-29 | 5 |
|  | 29 |
| CA_5-30 | 5 |
|  | 30 |
| CA_5-40 | 5 |
|  | 40 |
| CA_7-8 | 7 |
|  | 8 |
| CA_7-12 | 7 |
|  | 12 |
| CA_7-20 | 7 |
|  | 20 |
| CA_7-22 | 7 |
|  | 22 |
| CA_7-28 | 7 |
|  | 28 |
| CA_7_40 | 7 |
|  | 40 |
| CA_8-11 | 8 |
|  | 11 |
| CA_8-20 | 8 |
|  | 20 |
| CA_8-40 | 8 |
|  | 40 |
| CA_8-41 | 8 |
|  | 41 |
| CA_11-18 | 11 |
|  | 18 |
| CA_12-25 | 12 |
|  | 25 |
| CA_12-30 | 12 |
|  | 30 |
| CA_18-28 | 18 |
|  | 28 |
| CA_19-21 | 19 |
|  | 21 |
| CA_19-28 | 19 |
|  | 28 |
| CA_19-42 | 19 |
|  | 42 |
| CA_20-31 | 20 |
|  | 31 |
| CA_20-32 | 20 |
|  | 32 |
| CA_20-42 | 20 |
|  | 42 |
| CA_20-42-42 | 20 |
|  | 42 |
| CA_21-42 | 21 |
|  | 42 |
| CA_23-29 | 23 |
|  | 29 |
| CA_25-26 | 25 |
|  | 26 |
| CA_25-41 | 25 |
|  | 41 |
| CA_26-41 | 26 |
|  | 41 |
| CA_29-30 | 29 |
|  | 30 |
| CA_38-40 | 38 |
|  | 40 |
| CA_38-40-40 | 38 |
|  | 40 |
| CA_39-41 | 39 |
|  | 41 |
| CA_41-42 | 41 |
|  | 42 |

Table 5.5-3A. Inter-band carrier aggregation bands (three bands)

| CA Band | E-UTRA operating bands |
| --- | --- |
| CA_1-3-5 | 1 |
|  | 3 |
|  | 5 |
| CA_1-3-8 | 1 |
|  | 3 |
|  | 8 |
| CA_1-3-19 | 1 |
|  | 3 |
|  | 19 |
| CA_1-3-20 | 1 |
|  | 3 |
|  | 20 |
| CA_1-3-26 | 1 |
|  | 3 |
|  | 26 |
| CA_1-3-28 | 1 |
|  | 3 |
|  | 28 |
| CA_1-3-42 | 1 |
|  | 3 |
| CA_1-5-7 | 1 |
|  | 5 |
|  | 7 |
| CA_1-7-20 | 1 |
|  | 7 |
|  | 20 |
| CA_1-7-28 | 1 |
|  | 7 |
|  | 28 |
| CA_1-8-11 | 1 |
|  | 8 |
|  | 11 |
| CA_1-18-28 | 1 |
|  | 18 |
|  | 28 |
| CA_1-19-21 | 1 |
|  | 19 |
|  | 21 |
| CA_1-19-28 | 1 |
|  | 19 |
|  | 28 |
| CA_1-19-42 | 1 |
|  | 19 |
|  | 42 |
| CA_1-21-42 | 1 |
|  | 21 |
|  | 42 |
| CA_2-4-5 | 2 |
|  | 4 |
|  | 5 |
| CA_2-4-12 | 2 |
|  | 4 |
|  | 12 |
| CA_2-2-4-12 | 2 |
|  | 4 |
|  | 12 |
| CA_2-4-4-12 | 2 |
|  | 4 |
|  | 12 |
| CA_2-4-13 | 2 |
|  | 4 |
|  | 13 |
| CA-2-4-29 | 2 |
|  | 4 |
|  | 29 |
| CA-2-4-30 | 2 |
|  | 4 |
|  | 30 |
| CA_2-5-12 | 2 |
|  | 5 |
|  | 12 |
| CA_2-5-13 | 2 |
|  | 5 |
|  | 13 |
| CA_2-5-29 | 2 |
|  | 5 |
|  | 29 |
| CA_2-5-30 | 2 |
|  | 5 |
|  | 30 |
| CA_2-12-30 | 2 |
|  | 12 |
|  | 30 |
| CA_2-29-30 | 2 |
|  | 29 |
|  | 30 |
| CA_3-7-8 | 3 |
|  | 7 |
|  | 8 |
| CA_3-7-20 | 3 |
|  | 7 |
|  | 20 |
| CA_3-7-28 | 3 |
|  | 7 |
|  | 28 |
| CA_3-8-40 | 3 |
|  | 8 |
|  | 40 |
| CA_3-19-42 | 3 |
|  | 19 |
|  | 42 |
| CA_4-5-12 | 4 |
|  | 5 |
|  | 12 |
| CA_4-5-13 | 4 |
|  | 5 |
|  | 13 |
| CA_4-5-29 | 4 |
|  | 5 |
|  | 29 |
| CA_4-5-30 | 4 |
|  | 5 |
|  | 30 |
| CA_4-7-12 | 4 |
|  | 7 |
|  | 12 |
| CA_4-12-30 | 4 |
|  | 12 |
|  | 30 |
| CA_4-29-30 | 4 |
|  | 29 |
|  | 30 |
| CA_7-8-20 | 7 |
|  | 8 |
|  | 20 |
| CA_19-21-42 | 19 |
|  | 21 |
|  | 42 |

Table 5.5-3B. Inter-band carrier aggregation bands (four bands)

| CA Band | E-UTRA operating bands |
| --- | --- |
| CA_1-3-19-42 | 1 |
|  | 3 |
|  | 19 |
|  | 42 |
| CA_1-19-21-42 | 1 |
|  | 19 |
|  | 21 |
|  | 42 |
| CA_2-4-5-29 | 2 |
|  | 4 |
|  | 5 |
|  | 29 |
| CA_2-4-5-30 | 2 |
|  | 4 |
|  | 5 |
|  | 30 |
| CA_2-4-12-30 | 2 |
|  | 4 |
|  | 12 |
|  | 30 |
| CA_2-4-29-30 | 2 |
|  | 4 |
|  | 29 |
|  | 30 |

Table 5.5-4. Intra-band non-contiguous carrier aggregation bands (with two sub-blocks)

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_2-2 | 2 |
| CA_3-3 | 3 |
| CA_4-4 | 4 |
| CA_5-5 | 5 |
| CA_7-7 | 7 |
| CA_23-23 | 23 |
| CA_25-25 | 25 |
| CA_40-40 | 40 |
| CA_41-41 | 41 |
| CA_42-42 | 42 |

##### ---End of changes---