3GPP TSG-RAN Meeting #71 	RP-160417

# Göteborg, Sweden, March 7 - 10, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.212 | CR | 0190 | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS36.212 for eMTC TDD TBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, CATR, CMCC, Xinwei, Potevio, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-03-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Inclusion of Rel-13 feature of eMTC for TDD UL supporting larger max TBS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MCS field is increased by 1 bit for Format 6-0A and 6-1A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eMTC UE will not satisfy the ~1Mbps peak data rate requirement for TDD |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.10, 5.3.3.1.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | 36.211, 36.213, 36.306 |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------------------------- Omitted --------------------------------------------------------------

##### 5.3.3.1.10	Format 6-0A

----------------------------------------------------------------------- Omitted --------------------------------------------------------------

- Modulation and coding scheme – 5 bits as defined in section 8.6 of [3].

----------------------------------------------------------------------- Omitted --------------------------------------------------------------

##### 5.3.3.1.12	Format 6-1A

----------------------------------------------------------------------- Omitted --------------------------------------------------------------

If the UE is not configured to decode MPDCCH with CRC scrambled by the C-RNTI, and the number of information bits in format 6-1A is less than that of format 6-0A, zeros shall be appended to format 6-1A until the payload size equals that of format 6-0A.

If the UE is configured to decode MPDCCH with CRC scrambled by the C-RNTI and the number of information bits in format 6-1A mapped onto a given search space is less than that of format 6-0A for scheduling the same serving cell and mapped onto the same search space, zeros shall be appended to format 6-1A until the payload size equals that of format 6-0A.

----------------------------------------------------------------------- Omitted --------------------------------------------------------------