3GPP TSG RAN Meeting #71	RP-160526

# Goteborg, Sweden, Mar 7th – 10th, 2016

# Source:	ZTE Corporation

# Title:	New WID: D2D communication for commercial use cases

# Document for:	Approval

Agenda Item:	10.1.2

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title:	D2D communication for commercial use cases

Acronym:	*

Unique identifier:	*

NOTE:	If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI. Please tick (X) the applicable box(es) in the table below:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

1	3GPP Work Area

| X | Radio Access |
| --- | --- |
|  | Core Network |
|  | Services |

2	Classification of WI and linked work items

2.0	Primary classification

This work item is a …

|  | Study Item (go to 2.1) |
| --- | --- |
| X | Feature (go to 2.2) |
|  | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

NOTE:	Core, Performance and Testing parts of RAN WIs are usually Building Blocks.
If you are in doubt, please contact MCC.

2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

2.2	Feature

| Related Study Item or Feature (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is …

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

2.3.1	Stage 1

| Source of external requirements (if any) |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

2.3.2	Stage 2

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify:

Go to §3.

2.3.3	Stage 3

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) or external document | Clause | Remarks |

If no identified source of stage 2 information, justify:

Go to §3.

2.3.4	Test spec

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

2.3.5	Other

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

2.4	Work task

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

3	Justification

In R12 and R13, the standardization of Proximity Services using LTE D2D communication has focused on the public safety use cases. D2D communication was designed to operate with few UEs transmitting VoIP messages, which is the most typical public safety use case. On the other hand, a number of commercial uses cases could benefit from the D2D operation (and specifically from the sidelink relay procedure):

- To minimize power consumption of devices in bad coverage

- To increase data rates of devices at cell edge, allowing a more uniform user experience

For instance, MTC devices in bad coverage conditions could benefit from the relay functionality provided by an anchor UE to exchange data with the network via a PC5 connection provided by the anchor point. This would significantly lower the MTC device power consumption, avoiding the need for a huge number of repetitions on the Uu interface. Similar benefits could apply to moving sensors, like wearable devices, that could connect via PC5 to a smartphone (relay UE) and then to the network.

Compared with the Rel-13 D2D communication for Public Safety use cases, commercial scenarios assume that all the devices are in coverage and require support for higher data rates, capacity and reliability. L2 improvements (like Acknowledged Mode sidelink transmission, adjustable MCS and transmission power control) are then needed for sidelink unicast communication.

Considering the various emerging services in commercial scenarios, it is essential to consider enhancements to improve user experience, such as access control and admission control (e.g. to support massive connections from MTC devices) for relay communication, service continuity and eNB controlled QoS support.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The goal of this work item is to specify support for sidelink (relay) communication for commercial use cases, with the following detailed objectives:

- Sidelink unicast communication with higher data rate and reliability:

  - Acknowledged Mode sidelink communication  / L2 feedback and re-transmission [RAN2]

  - Adjustable MCS and power control optimizations [RAN1, RAN2]

  - Sidelink communication RRM measurement and report based on e.g. PSSCH [RAN2, RAN1]

- Relay operation enhancements for sidelink communication:

  - Enhancements to PC5 signaling procedures, with AS layer access control and admission control for relay sidelink communication [RAN2]

  - Network controlled PC5 connection management and parameter configuration [RAN2]

  - Network controlled path selection and switch between Uu and PC5 for remote UEs, to provide service continuity based on IP preservation [RAN2, RAN3]

- QoS enhancements for sidelink communication:

  - Network controlled QoS support for sidelink communication [RAN2].

- Additional RRM core requirements (if required) [RAN4].

Only in-coverage operation under network control (and using licensed bands) is assumed.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Additional performance requirements for the UE (if required) [RAN4]

4.3	RAN time budget proposal

NOTE:	For WIs/SIs under RAN WG5 leadership this section is not filled out. Otherwise:
For a not yet approved WI/SI the rapporteur has to fill out the last row of the table(s) below up to the target date of the WI/SI (if necessary add further tables): Indicate the number of time units (1 TU ~ 2h), i.e. one value for each session/field. If no time unit is needed, leave the field empty.
For WI/SI already approved in the past, the tables below will no longer be updated in the WI/SI description (i.e. the tables reflect the status of the initial approval). But changes can be proposed in the status report of the WI/SI.

| RAN #71 Q2/2016 RAN #72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 84bis | 84bis | 93bis | 93bis | 89bis | 91bis | 78bis | 78bis | 78bis | 78bis | 85 | 85 | 94 | 94 | 90 | 92 | 79 | 79 | 79 | 79 |
| 1 |  | 1.5 |  |  |  |  |  |  |  | 1 |  | 1.5 |  |  |  | 0.5 | 0.5 |  |  |

| RAN #72 Q3/2016 RAN #73 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 86 | 86 | 95 | 95 | 91 | 93 | 80 | 80 | 80 | 80 |
| 1 |  | 1.5 |  |  | 0.5 | 1 | 1 |  |  |

| RAN #73 Q4/2016 RAN #74 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf | R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 86bis | 86bis | 95bis | 95bis | 91bis | 93bis | 80bis | 80bis | 80bis | 80bis | 87 | 87 | 96 | 96 | 92 | 94 | 81 | 81 | 81 | 81 |
| 1 |  | 1.5 |  |  | 0.5 | 1 | 1 |  |  | 1 |  | 1.5 |  |  | 0.5 | 1 | 1 |  |  |

| RAN #74 Q1/2017 RAN #75 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1L | R1U | R2L | R2U | R2J | R3 | R4RF Core | R4RD Core | R4RF Perf | R4RD Perf |
| 88 | 88 | 97 | 97 | 89 | 95 | 82 | 82 | 82 | 82 |
|  |  |  |  |  |  |  |  |  | 1.5 |

L: LTE, U: UMTS, J: Joint, RD: RRM/demodulation

NOTE:	In case further explanation of the time budget proposal is needed, then please explain this below.

additional comments to the time budget proposal:

5	Service Aspects

6	MMI-Aspects

7	Charging Aspects

8	Security Aspects

9	Impacts

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  | X |
| Don't know |  |  |  | X |  |

10	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |
|  |  | RAN2 |  |  |  |  |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Comments for each spec.
By default a new specs can only be new for one of both parts.

| Affected existing specifications [None in the case of Study Items] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |
| 36.211 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation | RAN #74 | Core part |
| 36.212 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding | RAN #74 | Core part |
| 36.213 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures | RAN #74 | Core part |
| 36.214 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements | RAN #74 | Core part |
| 36.300 |  | Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 | RAN #74 | Core part |
| 36.302 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer | RAN #74 | Core part |
| 36.304 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode | RAN #74 | Core part |
| 36.306 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities | RAN #74 | Core part |
| 36.321 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification | RAN #70 | Core part |
| 36.322 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification | RAN #70 | Core part |
| 36.323 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification | RAN #74 | Core part |
| 36.331 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification | RAN #74 | Core part |
| 36.423 |  | Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol (X2AP) | RAN #74 | Core part |
| 36.413 |  | Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP) | RAN #74 | Core part |
| 36.101 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception | RAN #74 | Core part |
| 36.133 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management | RAN #74 | Core part |
| 36.101 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception | RAN #76 | Performance part |
| 36.133 |  | Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management | RAN #76 | Performance part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Comments for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

11	Work item rapporteur(s)

Parolari, Sergio

Company:	ZTE

Email:	sergio.parolari@zte.com.cn

12	Work item leadership

Primary: RAN2

Secondary: RAN1, RAN3, RAN4

NOTE:	If this is a RAN WID including Core and Perf. part, then this WG specifies the WG leading the Core part.
RAN WG4 is by default leading the Perf. part.

13	Supporting Individual Members

Supporting IM name

ZTE Corporation

Nubia Technology Co., Ltd

Shanghai Tejet Com Technology

CATT

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"