3GPP TSG RAN									                   				             RP-160796

Busan, Korea, June 13 - 16, 2016

Agenda Item:		9.2.2

Source:	LG Electronics

Title:		pCR to reliability and KPI values for eV2X

Document for:		Approval

- Introduction

A KPI definition and tentative values related to reliability for eV2X is captured in TR 38.913 [1], and via email discussion [RAN#71-03] Part 6, it has been suggested to further discuss it.

This contribution proposes KPI definition for eV2X and related KPI values.

- Discussion

Different from general URLLC use cases, eV2X needs to handle high mobility, massive connection/density, and potentially different communication mechanisms (e.g., direct communication among vehicles, vehicle-to-vehicle communication via network relay, communication between vehicles and pedestrians, communication between vehicles and networks, etc). Furthermore, depending on the use case, the required KPI value can be quite different particularly in terms of latency and reliability requirements.

To capture this end-to-end requirement to user plane latency, high level communication mechanism needs to be clarified as it will determine budget for “one-hop” latency. For example, if V2V is accomplished via direct communication, one-hop latency can be comparable to end-to-end latency, whereas if V2V is accomplished via network relay, one-hop latency should be much smaller than end-to-end latency to allow at least V2N (uplink), core network delay, and N2V (downlink). In this sense, our proposal is to capture different latency requirement depending on the communication mechanisms on V2V communication.  In recent SA1 progress [2], a few use cases for eV2X have been identified which would require different latency and reliability requirement as shown in Table 1.

Table 1. Use case and relevant requirements

| Use case | Representative communication mechanism | Latency requirements | Reliability requirements | Packet size | Communication range |
| --- | --- | --- | --- | --- | --- |
| Vehicle platooning | Direct communication | [10]msec E2E | [TBD] | [50-1200] bytes, 30 messages/second | A few meter |
| Information exchange within platoon | Communication between vehicles and RSU, relay via vehicle | [TBD] | [TBD] | [TBD] | [TBD] |
| Sensor and state map sharing | V2V (direct or relay via RSU/network) | [10]msec E2E | 95% | [TBD] | [TBD] |
| Remote driving | V2N | [5]msec E2E between V2X application server and V2X UE | 99.999% or higher | [5] Mbps DL, [50] Mbps UL |  |
| Automated cooperative driving for short distance grouping | V2V | Phase I: [25]msec E2E Phase II: [10msec] E2E | Phase I: [99%] Phase II: over [99.99%] within [TBDm] | Phase I: 300-400B | Phase I: small distance [TBD] Phase II: smaller distance [TBD] |
| Collective perception of environment | V2V | [100msec] E2E | Phase I: [99%] in[1000m] Phase II:[99.999%] in [1000m] | Phase I: [1600B] Phase II:[50Mbps] in [1000m] Peak of [1Gbps] in [50m] | Phase 1:[1000m] Phase II:[50m] |

In terms of required reliability for eV2X, based on the above identified use cases (and potential other use cases), we propose 1-10-5 reliability within [10msec] end-to-end latency which can be captured differently for one-hop latency depending on communication mechanism as follows.

Proposed KPI value for eV2X:

Communication availability and resilience for eV2X can be evaluated by the success probability of transmitting [300 bytes] with user plane latency requirement (7.5) of [10] msec at a certain communication range (e.g., a few meters) when direct communication via sidelink is assumed and with user plane latency requirement (7.5) of [2] msec when message is relayed via BS.  Note that target communication range is dependent of deployment and operation scenario (e.g., the average inter-vehicle speed).

The target for communication availability and resilience for eV2X should be 1-10-5 within [10 msec] when direct communication via sidelink is assumed and  1-10-5 within [2 msec] when message is relayed via BS.

- Reference

[1] TR 38.913 v.0.2.1, “Study on Scenarios and Requirements for Next Generation Access Technologies”

[2] S1-161604, “TR Skeleton for eV2X”, LG Electronics

- Text Proposals

<< -------------------------------------- omitted ---------------------------------------------------------------------------------------->>

7.9	Reliability

Reliability can be evaluated by the success probability of transmitting X bytes NOTE1 within 1 ms, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface, at a certain channel quality (e.g., coverage-edge).

The target for reliability should be 1-10-5 within 1ms.

NOTE1: Specific value for X is FFS

Table 7.9-1: Reliability in each deployment scenario for each usage scenario

| Reliability | Indoor Hotspot | Dense Urban | Rural | Urban Macro | High Speed | Urban Grid | Highway |
| --- | --- | --- | --- | --- | --- | --- | --- |
| eMBB |  |  |  |  |  |  |  |
| mMTC |  |  |  |  |  |  |  |
| URLLC |  |  |  |  |  |  |  |
| eV2X |  |  |  |  |  | x | x |

[Editor’s notes: The relevant use cases (V2V, V2I, or any others), deployment scenarios and the traffic model should be clarified.]

Communication availability and resilience for eV2X can be evaluated by the success probability of transmitting [300 bytes] with user plane latency requirement (7.5) of [10] msec at a certain communication range (e.g., a few meters) when direct communication via sidelink is assumed and with user plane latency requirement (7.5) of [2] msec when message is relayed via BS.

The target for communication availability and resilience for eV2X should be 1-10-5 within [10 msec] when direct communication via sidelink is assumed and  1-10-5 within [2 msec] when message is relayed via BS.

Note that target communication range and reliability requirement is dependent of deployment and operation scenario (e.g., the average inter-vehicle speed).

<< -------------------------------------- omitted ---------------------------------------------------------------------------------------->>