3GPP TSG RAN Meeting #72	RP-161023

‎ Busan, Korea, 11th – 15th April 2016

Agenda Item:		9.2.3

Source:		MediaTek

Title:				NB-IoT for mMTC in NR

Document for:	Discussion

- Introduction

Following the target connection density for NR mMTC of 106 connections/km2, this document provides a capacity analysis of NB-IoT using assumptions documented in TR45.820 in order to assess whether or not this new technology specified in Rel-13 would be suitable for NR mMTC.

- Evaluation on NB-IoT for mMTC in NR

System level simulations including some numerical calculations were performed to evaluate the achievable connection density performance of Rel-13 NB-IoT. The detailed simulation assumptions and the evaluation methodologies are provided in Appendix.

The results are documented in the table below.

Table 1 NB-IoT Connection Density Performance (per 200kHz channel BW)

| Scenario | # of Connections per km2 (with 200kHz channel BW) |
| --- | --- |
| DL Standalone | 4.2×105 |
| DL Guard band | 3.7×105 |
| DL LTE In-band | 2.8×105 |
| UL | 4.8×105 |

According to the above results, the DL LTE in-band deployment scenario features the smallest capacity figure; this is caused by the overhead needed to avoid legacy LTE signal (e.g. CRS) and PDCCH region. However even in this scenario, a large number of connections well above 105 connections/km2 can be achieved. In order to achieve a million connections per square kilometer, a simple approach is to aggregate a few 200kHz channels, as shown in the table below, under the assumptions documented in Appendix.

Table 2. Minimum number of 200kHz channels to achieve 106 connections/km2 density

| Scenario | # of 200kHz channels |
| --- | --- |
| DL Standalone | 3 |
| DL Guard band | 3 |
| DL LTE In-band | 4 |
| UL | 3 |

Given the above, it is our view that NB-IoT is a feasible technology baseline for mMTC in NR.

For the study item on new radio (NR) access technology, the new waveform proposal targeting to satisfy mMTC requirement ought to use NB-IoT as the benchmark. Only solutions that could significantly (criteria tbd) outperform NB-IoT could be considered as valid candidates.

Proposal 1:  TSG RAN to agree NB-IoT as the baseline solution for mMTC in NR Study

- Other considerations

NB-IoT was developed in Rel-13 as a “new” narrowband LTE technology to address the growing LPWA market. It is expected that the global adoption and deployment of this technology will quickly follow suit i.e. major investments are anticipated. Furthermore, having in mind some of the targeted markets and scenarios initially laid out where for instance modules would be dispatched for several years at a time, we expect NB-IoT deployments will be long-lived. Hence the sustainability of NB-IoT as a technology is very important, not to forget the associated economies of scale that can be exploited as a result.

With this in mind we think it is essential to “leverage” NB-IoT in the NR mMTC landscape as opposed to redefining something new which would trigger fragmentation and could undermine NB-IoT.

- Conclusions

According to the above analysis, the following proposal is concluded:

Proposal 1  TSG RAN to agree NB-IoT as the baseline solution for mMTC in NR Study

Appendix

Assumptions for system level simulations

The setup for system level simulation follows the assumptions in Annex D of [1]. For standalone mode, the DL transmission power is 43dBm per carrier; for guard-band and in-band modes, the DL transmission power is 35dBm by assuming that 46dBm DL power spreads in LTE 10MHz bandwidth and with 6dB power boost in NB-IoT carrier. The maximum transmit power for UL is 23dBm for multi-tone and single-tone modes. NB-IoT UL power control is applied.

Table 3. Assumptions for system level simulations (Standalone mode)

| No | Parameter |  | Assumption |
| --- | --- | --- | --- |
| 1 | Cellular Layout |  | Hexagonal grid, 3 sectors per site |
| 2 | Frequency band |  | 900 MHz |
| 3 | Inter site distance |  | 1732 m |
| 4 | MS speed |  | 0 km/h as the baseline |
| 5 | User distribution |  | Users dropped uniformly in entire cell |
| 6 | BS transmit power per 200 KHz (at the antenna connector) |  | 43 dBm |
| 7 | MS Tx power (at the antenna connector) |  | Max. 23 dBm per uplink physical channel with open loop power control |
| 8 | Pathloss model |  | L=I + 37.6log10(.R), R in kilometers I=120.9 for the 900 MHz band |
| 9 | Shadowing standard deviation |  | 8 dB |
| 10 | Correlation distance of Shadowing |  | 110 m |
| 11 | Shadowing correlation | Between cell sites | 0.5 |
|  |  | Between sectors of the same cell site | 1.0 |
| 12 | Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) |  | See table 5-7, 3GPP TR 45.914, 65° H-plane. |
| 13 | BS antenna gain |  | 18 dBi |
| 14 | MS Antenna gain |  | -4 dBi |
| 15 | BS cable loss |  | 3 dB |
| 16 | Building Penetration Loss |  | Based on distributions derived from adapted COST 231 NLOS model. See clause D.1 of TR 45.820 and note 5 |
| 17 | Inter-site correlation coefficient |  | Two inter-site correlation coefficients will be used for simulations: 0.5 and 0.75 |

Traffic model

As suggested in [1], the capacity metric is evaluated by system level simulations with Mobile Autonomous Reporting (MAR) periodic traffic model and Network Command traffic model. The split of devices between MAR periodic and Network Command is MAR periodic (80%) and Network Command (20%). The application payload size for MAR follows Table 4. The typical size of Network Command is 20 bytes. The periodic inter-arrival time for MAR and NC is also shown in Table 4.  For the application payload, the header overhead above the equivalent of the SNDCP layer is considered. Here we assume header compression technique is applied so that extra 29 Bytes header is considered per application layer packet.

Table 4. MAR periodic UL reporting traffic model

| Characteristic |  |
| --- | --- |
| Application payload size distribution | Pareto distribution with shape parameter alpha = 2.5 and minimum application payload size = 20 bytes with a cut off of 200 bytes i.e. payloads higher than 200 bytes are assumed to be 200 bytes. |
| Periodic inter-arrival time | Split of inter-arrival time periodicity for MAR periodic is: 1 day (40%), 2 hours (40%), 1 hour (15%), and 30 minutes (5%) |

MAR traffic flow, the packet sizes, and the radio resource consumption

The MAR traffic flow for NB-IoT and the packet size of each step are shown in Figure 1, where it follows the agreed Control Plane (CP) solution, the UL data is carried in message 5.

Figure 1: MAR traffic flow and the packet sizes

Based on the radio link quality, the average radio resource consumption in PRB per MAR report per connection can be derived. The calculation methodology follows the agreement in [2]. In DL the Chase combining is used for repetition and in UL the incremental redundancy (RV=0, 2) and the Chase combining are used.

NC traffic flow, the packet sizes, and the radio resource consumption

The NC traffic flow for NB-IoT and the packet size of each step are shown in Figure 2.

Figure 2: NC traffic flow and the packet sizes

Connection Density Performance

Considering the inter-arrival time periodicity and the split weighting of MAR and NC, the overall radio resource consumption in PRBs per connection per day can be further derived.

In DL, in addition to control and data channels, the system overhead of synchronization, system information and paging should be considered. The minimum system overhead in DL is about 29.25%, including in average 2.625 PRBs allocated for PSS/SSS/MIB/SIB1 per radio frame according to the design of NB-IoT and the estimated 0.3 PRBs allocated for other SIBs and paging per radio frame.

Assume that one DL/UL pair dedicated NB-IoT carriers is allocated. The total available radio resource for DL and UL is 8.64*107 PRBs per day. Considering the radio resource consumption of each channel, the connection density is calculated and shown in Table 1.

References

- TR 45.820, Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)

- R1-163943, RAN1 agreements for Rel-13 NB-IoT