3GPP TSG RAN#72 							          RP-161254

Busan, South Korea, June 13 - 16, 2016

Source: 	Orange (Email discussion convenor Fatima Karim-Peters)

Title: 	Text proposal to TR 38.913 from email discussion: [RAN#71-07] Air-to-Ground Communications

Agenda Item:	9.2.2

Document for:	Approval

# Introduction

Following the email discussion that took place between RAN#71 and RAN#72 over 3GPP RAN reflector on  [RAN#71-07] Air-to-Ground Communications, we propose to approve the text proposal below to be included in TR38.913.

# Text Proposal for TR38.913

-------------------------------------------------- BEGIN TEXT PROPOSAL ----------------------------------------------------------

### 6.1.L	Commercial Air to Ground scenario

The commercial Air to Ground deployment scenario is defined to allow for the provision of services for commercial aircraft to enable both humans and machines aboard the aircraft to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput that are optimized for high altitude users that are travelling at very high speeds. The commercial airlines aircrafts are likely equipped with an aggregation point (e.g.Relay)

Some of the characteristics of this deployment scenario are listed below

Table 6.1.6-L: Attributes for commercial Air to Ground Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro + relay: for BS to relay: Below [4] GHz, for relay to UE: [TBD] GHz |
| System Bandwidth | [40] MHz (DL+UL) |
| Layout | Macro [layout including number of base stations is FFS]+ relay nodes (NOTE1) |
| Cell range | Macro cell: [100] km range to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation. Relay: up to [80] m |
| User density and UE speed | End user density per Macro: NOTE2 UE speed: Up to [1000] km/h Altitude: Up to [15] km |
| Traffic model | Average data throughput at busy hours/user: [TBD] kbps End User experienced data rate: [384kbps] DL. NOTE3 |

NOTE1: BS to relay link should be the priority for study compared to relay to UE link.

NOTE2: Evaluate how many users can be served per cell site when the range edge users are serviced with the target user experience data rate.

NOTE3: Target values for UL are lower than DL, 1/3 of DL is desirable.

### 6.1.M	Light aircraft scenario

The light aircraft scenario is defined to allow for the provision of services for general aviation aircrafts to enable both humans and machines aboard helicopters and small air plans to initiate and receive mobile services. It is not for the establishment of airborne based base stations.

The key characteristics of this scenario are upward pointed Macro cells with very large area coverage supporting basic data and voice services, with moderate user throughput and low user density that are optimized for moderate altitude users that might be traveling at high speeds. The general regime aviation aircrafts are not equipped with relays.

Some of the characteristics of this deployment scenario are listed below

Table 6.1.6-M: Attributes for Light aircraft Scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Macro only: Below [4 ]GHz |
| System Bandwidth | [40] MHz (DL+UL) |
| Layout | Single layer: Macro cell [layout including number of base stations is FFS] |
| Cell range | [100] km range to be evaluated through system level simulations. Feasibility of Higher Range shall be evaluated through Link level evaluation. |
| User density and UE speed | End user density per aircraft: up to [6] users UE speed: Up to [370] km/h Altitude: Up to [3] km |
| Traffic model | Average data throughput at busy hours/user: [TBD] kbps End user experienced data rate: [384kbps] DL. NOTE1 |

NOTE1: Target values for UL are lower than DL, 1/3 of DL is desirable.

-------------------------------------------------- END TEXT PROPOSAL ----------------------------------------------------------