3GPP TSG RAN WG1 Meeting #86		                                                R1-167185

Gothenburg, Sweden, August 22-26, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0699 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the relationship between IMCS and ITBS for DCI format 6-1A in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In the section 7.1.7.2 of TS 36.213, the relationship between IMCS and ITBS is not defined for DCI format 6-1A scrambled by RA-RNTI |  |  |  |  |  |  |  |  |
| Summary of change: |  | IMCS = ITBS for DCI format 6-1A scrambled by RA-RNTI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to derive ITBS for DCI format 6-1A scrambled by RA-RNTI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

#### 7.1.7.2	Transport block size determination

For BL/CE UEs configured with CEModeA,  is used in place of in the rest of this subclause

If the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI then

-	for DCI format 1A or DCI format 6-1A:

-	the UE shall set the TBS index () equal to  and determine its TBS by the procedure in subclause 7.1.7.2.1 for .

#### < Unchanged parts are omitted >