3GPP TSG RAN WG1 Meeting #86		                                                  R1-166184

Gothenburg, Sweden, August 22-26, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0690 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NPDCCH related procedure on TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - The following RAN1#84bis agreement regarding NPDCCH related procedure are not implemented in current specification: - Starting subframes: - eNB only configures values that result in T>=4 - DCI format definitions in TS 36.212, cross-reference section 16.6 of TS 36.213 for the definition of the “DCI subframe repetition number” field. However, this field is not defined in TS 36.213. - The dropping of NPDCCH candidates within 4 ms of the start of another search space needs to refer to k0, the first of the starting subframes according to the definition of . - The following RAN1#85 agreement regarding NPDCCH in UL gaps is not implemented in current specification: - If NPUSCH UL Gap is inserted at the end of NPUSCH transmission, the UE resumes monitoring of N-PDCCH from the end of the UL gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that for deriving subframe k0, T ≥ 4. - Add column or row to the search space tables to define the “DCI subframe repetition number” field. - Add the limitation “having starting subframe k0” to refer to the first starting subframe of a following search space. - Add a sentence stating that the UE is not required to monitor NPDCCH during an UL gap on NPUSCH. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for NPDCCH procedure is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

## 16.6	Narrowband physical downlink control channel related procedures

#### < Unchanged parts are omitted >

For NPDCCH UE-specific search space, the aggregation and repetition levels defining the search spaces and the corresponding NPDCCH candidates are listed in Table 16.6-1 by substituting the value of with the higher layer configured parameter al-Repetition-USS.

For Type1-NPDCCH common search space, the aggregation and repetition levels defining the search spaces are listed in Table 16.6-2 by substituting the value of with the higher layer configured parameter al-Repetition-CSS-Paging.

For Type2-NPDCCH common search space, the aggregation and repetition levels defining the search spaces and the corresponding monitored NPDCCH candidates are listed in Table 16.6-3 by substituting the value of with the higher layer configured parameter npdcch-MaxNumRepetitions-RA.The locations of starting subframe  are given by where is the th consecutive NB-IoT DL subframe from subframe , and , and , and where

-	subframe  is a subframe satisfying the condition , where ,

T≥4.

-	For NPDCCH UE-specific search space,

- is given by the higher layer parameter nPDCCH-startSF-UESS,

- is given by the higher layer parameter nPDCCH-startSFoffset-UESS,

-	For NPDCCH Type2-NPDCCH common search space,

- is given by the higher layer parameter nPDCCH-startSF-Type2CSS,

- is given by the higher layer parameter nPDCCH-startSFoffset-Type2CSS,

For Type1-NPDCCH common search space,and is determined from locations of NB-IoT paging opportunity subframes.

If the UE is configured by high layers with a PRB for monitoring of NPDCCH UE-specific search space,

-	the UE shall monitor the NPDCCH UE-specific search space on the higher layer configured PRB,

-	the UE is not expected to receive NPSS, NSSS, NPBCH on the higher layer configured PRB.

otherwise,

-	the UE shall monitor the NPDCCH UE-specific search space on the same PRB on which NPSS/NSSS/NPBCH are detected.

Table 16.6-1: NPDCCH UE- specific search space candidates

|  |  | DCI subframe repetition number | NCCE indices of monitored NPDCCH candidates |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L’=1 | L’=2 |  |
| 1 | 1 | 00 |  |  | {0},{1} |  | {0,1} |
| 2 | 1 | 00 |  |  | {0},{1} |  | {0,1} |
|  | 2 | 01 |  |  | - |  | {0,1} |
| 4 | 1 | 00 |  |  | - |  | {0,1} |
|  | 2 | 01 |  |  | - |  | {0,1} |
|  | 4 | 10 |  |  | - |  | {0,1} |
| >=8 |  | 00 |  |  | - |  | {0,1} |
|  |  | 01 |  |  | - |  | {0,1} |
|  |  | 10 |  |  | - |  | {0,1} |
|  |  | 11 |  |  | - |  | {0,1} |
| Note 1: {x}, {y} denotes NPDCCH Format 0 candidate with NCCE index ‘x’, and NPDCCH Format 0 candidate with NCCE index ‘y’ are monitored Note 2: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs ‘x’ and ‘y’ is monitored. |  |  |  |  |

Table 16.6-2: Type 1- NPDCCH common search space candidates

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NCCE indices of monitored NPDCCH candidates |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | L’=1 |  | L’=2 |  |
| 1 |  |  |  | 1 | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | {0,1} |  |  |
| 2 |  |  |  | 1 | 2 |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | {0,1} |  |  |
| 4 |  |  |  | 1 | 2 |  | 4 |  | - |  | - |  | - |  | - |  | - |  | - |  | {0,1} |  |  |
| 8 |  |  |  | 1 | 2 |  | 4 |  | 8 |  | - |  | - |  | - |  | - |  | - |  | {0,1} |  |  |
| 16 |  |  |  | 1 | 2 |  | 4 |  | 8 |  | 16 |  | - |  | - |  | - |  | -- |  | {0,1} |  |  |
| 32 |  |  |  | 1 | 2 |  | 4 |  | 8 |  | 16 |  | 32 |  | - |  | - |  | - |  | {0,1} |  |  |
| 64 |  |  |  | 1 | 2 |  | 4 |  | 8 |  | 16 |  | 32 |  | 64 |  | - |  | - |  | {0,1} |  |  |
| 128 |  |  |  | 1 | 2 |  | 4 |  | 8 |  | 16 |  | 32 |  | 64 |  | 128 |  | - |  | {0,1} |  |  |
| 256 |  |  |  | 1 | 4 |  | 8 |  | 16 |  | 32 |  | 64 |  | 128 |  | 256 |  | - |  | {0,1} |  |  |
| 512 |  |  |  | 1 | 4 |  | 16 |  | 32 |  | 64 |  | 128 |  | 256 |  | 512 |  | - |  | {0,1} |  |  |
| 1024 |  |  | 1 |  | 8 |  | 32 |  | 64 |  | 128 |  | 256 |  | 512 |  | 1024 |  | - |  | {0,1} |  |  |
| 2048 |  |  | 1 |  | 8 |  | 64 |  | 128 |  | 256 |  | 512 |  | 1024 |  | 2056 |  | - |  | {0,1} |  |  |
| DCI subframe repetition number | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |  |  |
| Note 1: {x}, {y} denotes NPDCCH Format 0 candidate with NCCE index ‘x’, and NPDCCH Format 0 candidate with NCCE index ‘y’ are monitored Note 2: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs ‘x’ and ‘y’ is monitored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 16.6-3: Type 2- NPDCCH common search space candidates

|  |  | DCI subframe repetition number | NCCE indices of monitored NPDCCH candidates |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | L’=1 |  | L’=2 |
| 1 | 1 | 00 | - |  | {0,1} |
| 2 | 1 | 00 | - |  | {0,1} |
|  | 2 | 01 | - |  | {0,1} |
| 4 | 1 | 00 | - |  | {0,1} |
|  | 2 | 01 | - |  | {0,1} |
|  | 4 | 10 | - |  | {0,1} |
| >=8 |  | 00 | - |  | {0,1} |
|  |  | 01 | - |  | {0,1} |
|  |  | 10 | - |  | {0,1} |
|  |  | 11 | - |  | {0,1} |
| Note 1: {x}, {y} denotes NPDCCH Format 0 candidate with NCCE index ‘x’, and NPDCCH Format 0 candidate with NCCE index ‘y’ are monitored Note 2: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs ‘x’ and ‘y’ is monitored. |  |  |  |  |

#### < Unchanged parts are omitted >

A NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap.

#### < Unchanged parts are omitted >