3GPP TSG RAN WG1 Meeting #86		                                                  R1-166196

Gothenburg, Sweden, August 22-26, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0691 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to RRC parameter names for NB-IoT in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The name of some higher layer parameters is inconsistent with the approved RRC specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the inconsistent parameter names according to TS 36.331 v13.2.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between physical layer and RRC specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.2, 16.4,16.4.1.3,16.4.1.5, 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 16.2.2	Downlink power allocation

#### < Unchanged parts are omitted >

If higher layer parameter operationModeInfo indicates ‘00’ for a cell, the ratio of NRS EPRE to CRS EPRE is given by the parameter nrs-CRS-PowerOffset if the parameter nrs-CRS-PowerOffsetis provided by higher layers, and the ratio of NRS EPRE to CRS EPRE may be assumed to be 0 dB if the parameter nrs-CRS-PowerOffsetis not provided by higher layers.

#### < Unchanged parts are omitted >

## 16.4	Narrowband physical downlink shared channel related procedures

A NB-IoT UE shall assume a subframe as a valid NB-IoT DL subframe if

- the UE determines that the subframe does not contain NPSS/NSSS/NPBCH/NB-SIB1 transmission, and

- the subframe is indicated as NB-IoT DL subframe by the higher layer parameter downlinkBitmap, if the UE is configured with the higher layer parameter downlinkBitmap.

#### < Unchanged parts are omitted >

#### 16.4.1.3	Resource allocation

#### < Unchanged parts are omitted >

The number of repetitions for the NPDSCH carrying SystemInformationBlockType1-NB is determined based on the parameter schedulingInfoSIB1 configured by higher-layers and according to Table 16.4.1.3-3.

Table 16.4.1.3-3: Number of repetitions for NPDSCH carrying SystemInformationBlockType1-NB.

| Value of schedulingInfoSIB1 | Number of NPDSCH repetitions |
| --- | --- |
| 0 | 4 |
| 1 | 8 |
| 2 | 16 |
| 3 | 4 |
| 4 | 8 |
| 5 | 16 |
| 6 | 4 |
| 7 | 8 |
| 8 | 16 |
| 9 | 4 |
| 10 | 8 |
| 11 | 16 |
| 12-15 | Reserved |

#### < Unchanged parts are omitted >

#### 16.4.1.5	Modulation order and transport block size determination

The UE shall use modulation order, = 2.

To determine the transport block size in the NPDSCH, the UE shall first,

- if NPDSCH carries SystemInformationBlockType1-NB

- set  to the value of the parameter schedulingInfoSIB1 configured by higher-layers

- otherwise

-	read the 4-bit "modulation and coding scheme" field () in the DCI and set .

#### < Unchanged parts are omitted >

## 16.6	Narrowband physical downlink control channel related procedures

#### < Unchanged parts are omitted >

For NPDCCH UE-specific search space, the aggregation and repetition levels defining the search spaces and the corresponding NPDCCH candidates are listed in Table 16.6-1 by substituting the value of with the higher layer configured parameternpdcch-NumRepetitions.

For Type1-NPDCCH common search space, the aggregation and repetition levels defining the search spaces are listed in Table 16.6-2 by substituting the value of with the higher layer configured parameternpdcch-NumRepetitionPaging.

For Type2-NPDCCH common search space, the aggregation and repetition levels defining the search spaces and the corresponding monitored NPDCCH candidates are listed in Table 16.6-3 by substituting the value of with the higher layer configured parameternpdcch-NumRepetitions-RA.

The locations of starting subframe  are given by where is the th consecutive NB-IoT DL subframe from subframe , and , and , and where

-	subframe  is a subframe satisfying the condition , where

-	For NPDCCH UE-specific search space,

- is given by the higher layer parameternpdcch-StartSF-USS,

- is given by the higher layer parameternpdcch-Offset-USS,

-	For NPDCCH Type2-NPDCCH common search space,

- is given by the higher layer parameternpdcch-StartSF-CSS-RA,

- is given by the higher layer parameternpdcch-Offset-RA,

#### < Unchanged parts are omitted >