3GPP TSG RAN WG1 Meeting #86		                                                  R1-167990

Gothenburg, Sweden, August 22-26, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 0692 | rev | 1 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NPDSCH related procedure in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - Correct interpretation of the specification in line with agreements requires the variable “k0” to have two different meanings in section 16.4.1. In the first paragraph of section 16.4.1, “k0” means the first valid DL subframe. While in Table 16.4.1-1 “k0” means the number of valid DL subframes starting in absolute DL subframe n+5 until the first valid DL subframe. It is necessary that each variable has only one meaning. The intention of the RAN1 agreement is that k0 should reflect kdelay, i.e. the second meaning: RAN1#84bis agreement: - 3 bits indicate number of valid DL subframes. The gap between end of NPDCCH and the start of the associated NPDSCH equals kdelay valid DL subframes + 4ms - For DCI format N2, the value of k0 should be such that paging NPDSCH begins in the first valid DL subframe which is >=4 ms later than the end of NPDCCH, i.e. the scheduling delay is zero valid subframes, but the specification indicates a scheduling delay of 5 valid subframes. - The RNTI used to initialize the scrambling for NPDSCH carrying system information is not defined in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use “ni” to replace “ki”. Add definition “k0 is the number of NB-IoT DL subframe(s) starting in DL subframe n+5 until DL subframe n0,” keeping the second meaning of “k0”. - Change “k0 = 0” for DCI format N2. - Add SI-RNTI for SIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Risk of misinterpretation of the specification other than in accordance with RAN1 agreements leading to different assumptions at UE and eNB. - The specification is inconsistent with RAN1 agreement. - The scrambling initialization for SI-message transmission on NPDSCH is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 16.4.1	UE procedure for receiving the narrowband physical downlink shared channel

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N1, N2 ending in subframe n intended for the UE, decode, starting in n+5 DL subframe, the corresponding NPDSCH transmission in N consecutive NB-IoT DL subframe(s) ni with i = 0, 1, …, N-1 according to the NPDCCH information, where

- subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

- subframe(s) ni with i=0,1,…,N-1 are N consecutive NB-IoT DL subframe(s) excluding subframes used for SI messages where, n0<n1<…,nN-1 ,

- , where the value of is determined by the repetition number field in the corresponding DCI (see subclause 16.4.1.3), and the value of is determined by the resource assignment field in the corresponding DCI (see subclause 16.4.1.3), and

- k0 is the number of NB-IoT DL subframe(s) starting in DL subframe n+5 until DL subframe n0, where k0 is determined by the scheduling delay field () for DCI format N1 according to Table 16.4.1-1, and k0 = 0 for DCI format N2. The value of is according to subclause 16.6 for the corresponding DCI format N1.

Table 16.4.1-1: for DCI format N1.

| 0 | 0 | 0 |
| --- | --- | --- |
| 1 | 4 | 16 |
| 2 | 8 | 32 |
| 3 | 12 | 64 |
| 4 | 16 | 128 |
| 5 | 32 | 256 |
| 6 | 64 | 512 |
| 7 | 128 | 1024 |

A UE is not expected to receive transmissions in 3 DL subframes following the end of a NPUSCH transmission by the UE.

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the P-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-2. 
The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by P-RNTI.

Table 16.4.1-2: NPDCCH and NPDSCH configured by P-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N2 | Type-1 Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 16.4.1.1), otherwise Transmit diversity (see subclause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the RA-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-3. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by RA-RNTI.

Table 16.4.1-3: NPDCCH and NPDSCH configured by RA-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | Type-2 Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 16.4.1.1), otherwise Transmit diversity (see subclause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the C-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-4. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by C-RNTI.

Table 16.4.1-4: NPDCCH and NPDSCH configured by C-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | UE specific by C-RNTI | If the number of NPBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 16.4.1.1), otherwise Transmit diversity (see subclause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the Temporary C-RNTI, and the UE is not configured to decode NPDCCH with CRC scrambled by the C-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to the combination defined in Table 16.4.1-5. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by Temporary C-RNTI.

Table 16.4.1-5: NPDCCH and NPDSCH configured by Temporary C-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | Type-2 Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 16.4.1.1), otherwise Transmit diversity (see subclause 16.4.1.2). |

For NPDSCH carrying SystemInformationBlockType1-NB and SI-messages, the UE shall decode NPDSCH according to the transmission scheme defined in Table 16.4.1-6. The scrambling initialization of NPDSCH is by SI-RNTI.

Table 16.4.1-6: NPDSCH configured by SI-RNTI

Transmission scheme of NPDSCH

If the number of NPBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 16.4.1.1), otherwise Transmit diversity (see subclause 16.4.1.2).

The transmission schemes for NPDSCH are defined in the following subclauses.

#### < Unchanged parts are omitted >