3GPP TSG-RAN1 Meeting #86	R1-166495

# Gothenburg, Sweden, 22nd Aug 2016 - 26th Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.214 | CR | 0749 | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on UL Discontinuous Transmission with DCH enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Europe Inc.(France) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | UTRA_DCHenh-Core |  |  |  |  |  | Date: |  |  | 2016-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | With DCH enhancements, UL TFC 0 cannot be transmitted continuously in 20 ms mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UL TFC 0 should be used to indicate 20 ms Mode as well as transmit continuously as per the 20 ms Mode requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without these changes there would be potential performance issues / call drops. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6C.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****************************************** TEXT START *****************************************

## 6C.0	Discontinuous transmission/reception of DCH

When DL_DCH_FET_Config is configured by higher layers, DL_DPCH_DYN_DRX and UL_DPCH_DYN_DTX control DTX and DRX operations for DPCH transmission and reception.  UE shall DRX DL DPDCH if DL_DPCH_DYN_DRX is TRUE. UE shall DTX UL DPDCH if UL_DPCH_DYN_DTX is TRUE.

In each slot, the values of the Boolean flags DL_DPCH_DYN_DRX and UL_DPCH_DYN_DTX are determined in Subclause 6C.0.1 and 6C.0.2.

### 6C.0.1	DL_DPCH_DYN_DRX and UL_DPCH_DYN_DTX

#### 6C.0.1.1	DL_DCH_FET_Config = 0

In each UL slot, the UL_DPCH_DYN_DTX is set to TRUE if all of the following conditions are satisfied:

-	This slot is in the second radio frame of 20ms CI.

-	UL_DPCH_10ms_Mode is configured by higher layers.

Or, if all of the following conditions are satisfied:

-	Only a single transport channel is configured with 40ms TTI by higher layers over UL DCH.

-	This slot is not in the first radio frame of 40ms CI.

-	This slot contains no UL DPDCH bits, i.e., no UL DCH transport blocks are sent in this radio frame.

Otherwise UL_DPCH_DYN_DTX is set to FALSE.

In each DL slot, the DL_DPCH_DYN_DRX is set to TRUE if all of the following conditions are satisfied:

-	This slot is in the second radio frame of 20ms CI.

-	DL DCH contains no SRBs in this radio frame.

-	UL_DPCH_10ms_Mode is configured by higher layers.

Or, if all of the following conditions are satisfied:

-	Only a single transport channel is configured with 40ms TTI by higher layers over DL DCH.

-	This slot is not in the first radio frame of 40ms CI.

-	DL DCH contains no SRBs in this radio frame.

Otherwise DL_DPCH_DYN_DRX is set to FALSE.

#### 6C.0.1.2	DL_DCH_FET_Config = 1

In each UL slot, the UL_DPCH_DYN_DTX is set to TRUE if all of the following conditions are satisfied, otherwise UL_DPCH_DYN_DTX is set to FALSE:

-	In this 20ms CI, excluding this slot, at least two UL DPCCH slots that contain DL FET ACK bits (Subclause 4.3A of [2]) have been transmitted.

-	UL_DPCH_10ms_Mode is configured by higher layers and this slot is in the second radio frame of 20ms CI.

In each DL slot, the DL_DPCH_DYN_DRX is set to TRUE if all of the following conditions are satisfied, otherwise DL_DPCH_DYN_DRX is set to FALSE

-	In this 20ms CI, at least two UL DPCCH slots that contain DL FET ACK bits (Subclause 4.3A of [2]) have been transmitted before the end of this DL slot.

-	UL_DPCH_DYN_DTX in the UL slot started during this DL slot is TRUE.

******************************************* TEXT END ******************************************