3GPP TSG-RAN4 Meeting #80	R4-166243

# Gothenburg, Sweden, 22nd Aug 2016 - 26th Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3928 | rev | - | Current version: | 12.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for correction to some parameters in D2D RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_D2D_Prox-Perf |  |  |  |  |  | Date: |  |  | 2016-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Some errors in OCNG and PDSCH RMC pairing in D2D RRM test + One clarification to resource pool information |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change 1: The Tx resource pool is currently using the IE that’s sent in SIB. Clarified the same Tx resource pool is appicable for dedicated signaling by adding the IE received in dedicated signaling (e.g., discTxPoolCommon/ discTxPoolDedicated). No change to the actual resource pool. - Change 2: OCNG for T1/T2/T3 duration clarified. Durign T1, UE is in IDLE. During T2, UE is in connected but without any PDSCH allocation. During T3, PDSCH is scheduled for the UE (R.5 RMC). Hence OCNG for T1/T2 is set as OP.16 (fully allocated), and during T3 is set as OP.15 (outer blocks). - Change 3: OCNG updated to be non-MBSFN version since the RMC used for the test is non-MBSFN with allcoation in all subframes. Further, during T1/T2, fully alloced OCNG is used (similar to Change 2), and during T3, outer block OCNG is used along with the PDSCH scheduling applicable with user data in all subframes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent OCNG and PDSCH RMC will be present in some D2D RRM tests. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.12.4, A.3.12.5, A.7.5.3, A.7.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | N |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | Y | Test specifications |  |  |  | 36.521-3 |  |  |
| (show related CRs) |  | N |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

## A.3.12	Proximity-based Services

### A.3.12.1	Introduction

This clause also defines the principle and the reference configurations that are applicable to test cases verifying RRM core requirements for ProSe Direct Discovery and ProSe Direct Communication.

### A.3.12.2	Reference DRX configurations for ProSe tests

Table A.3.12.2-1: Reference DRX Configurations

| Parameter | Value |
| --- | --- |
| Reference configuration | DRX_P1 |
| onDurationTimer | psf1 |
| drx-InactivityTimer | psf1 |
| drx-RetransmissionTimer | psf1 |
| longDRX-CycleStartOffset | sf320, 0 |
| shortDRX | Disabled |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |

### A.3.12.3	Test Cases with Different Channel Bandwidths

#### A.3.12.3.1	Introduction

This clause defines a principle which is applicable to test cases verifying ProSe RRM requirements with different channel bandwidths.

#### A.3.12.3.2	Principle of testing

Some ProSe test cases are defined for different channel bandwidths to verify the same RRM requirement.

If test cases with different channel bandwidth are defined to verify the same RRM requirement then the UE is required to pass the test cases only with one of the channel bandiwdths.

### A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery

Table A.3.12.4-1: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #1)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  |  | not present |
|  | rxParameters |  |  | not present |
| discTxPoolCommon/ discTxPoolDedicated | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 2 |
|  |  | prb-Start |  | 3 |
|  |  | prb-End |  | 5 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 10000000 00000000 00000000 00000000 00000000 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | ue-SelectedResourceConfig | poolSelection | random |
|  |  |  | txProbability | p100 |
|  | rxParameters |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 35 (155 mod 40) |
|  | slssid |  |  | 30 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | syncTxThreshIC |  | 0 (-infinity) |
|  | rxParamsNCell |  |  | not present |
| discInterFreqList |  |  |  | not present |

Table A.3.12.4-2: ProSe Direct Discovery configuration for E-UTRA FDD (Configuration #2)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 160 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 |
|  | txParameters |  |  | not present |
|  | rxParameters | tdd-Config |  | not present |
|  |  | syncConfigIndex |  | 0 |
| discTxPoolCommon/ discTxPoolDedicated |  |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 20 (140 mod 40) |
|  | slssid |  |  | 30 |
|  | txParameters |  |  | not present |
|  | rxParamsNCell | physCellId |  | 1 |
|  |  | discSyncWindow |  | w1 |
| discInterFreqLis |  |  |  | not present |

Table A.3.12.4-3: ProSe Direct Discovery configuration for E-UTRA TDD Config 0 (Configuration #3)

| Information Element |  |  |  | Value |
| --- | --- | --- | --- | --- |
| discRxPool | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 12 |
|  |  | prb-Start |  | 0 |
|  |  | prb-End |  | 23 |
|  |  | offsetIndicator |  | 163 |
|  |  | subframeBitmap |  | 11000000 00000000 00000000 00000000 00000000 00 |
|  | txParameters |  |  | not present |
|  | rxParameters |  |  | not present |
| discTxPoolCommon/ discTxPoolDedicated | cp-Len |  |  | Normal |
|  | discPeriod |  |  | rf32 |
|  | numRetx |  |  | 0 |
|  | numRepetition |  |  | 1 |
|  | tf-ResourceConfig | prb-Num |  | 2 |
|  |  | prb-Start |  | 3 |
|  |  | prb-End |  | 5 |
|  |  | offsetIndicator |  | 163 |
|  |  | subframeBitmap |  | 10000000 00000000 00000000 00000000 00000000 00 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | ue-SelectedResourceConfig | poolSelection | random |
|  |  |  | txProbability | p100 |
|  | rxParameters |  |  | not present |
| discTxPowerInfo | discMaxTxPower |  |  | 23 |
| SL-SyncConfig | syncCP-Len |  |  | Normal |
|  | syncOffsetIndicator |  |  | 38 (158 mod 40) |
|  | slssid |  |  | 30 |
|  | txParameters | txParametersGeneral | alpha | al0 |
|  |  |  | p0 | 31 |
|  |  | syncTxThreshIC |  | 0 (-infinity) |
|  | rxParamsNCell |  |  | not present |
| discInterFreqList |  |  |  | not present |

### A.3.12.5	Reference resource pool configurations for ProSe Direct Communication

Table A.3.12.5-1: ProSe Direct Communication configuration for E-UTRA FDD (Configuration #1)

| Information Element |  |  |  | Value (5MHz) | Value (10MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
| commRxPool | sc-CP-Len |  |  | Normal |  |  |
|  | sc-Period |  |  | sf40 |  |  |
|  | sc-TF-ResourceConfig | prb-Num |  | 12 | 25 |  |
|  |  | prb-Start |  | 0 | 0 |  |
|  |  | prb-End |  | 23 | 49 |  |
|  |  | offsetIndicator |  | 0 |  |  |
|  |  | subframeBitmap |  | 00011000 00000000 00000000 00000000 00000000 |  |  |
|  | data-CP-Len |  |  | Normal |  |  |
|  | dataHoppingConfig | hoppingParameter |  | 0 |  |  |
|  |  | numSubbands |  | ns1 |  |  |
|  |  | rb-Offset |  | 0 |  |  |
|  | ue-SelectedResourceConfig | data-TF-ResourceConfig | prb-Num | 12 |  | 25 |
|  |  |  | prb-Start | 0 |  | 0 |
|  |  |  | prb-End | 23 |  | 49 |
|  |  |  | offsetIndicator | 0 |  |  |
|  |  |  | subframeBitmap | 00000000 11111111 11111111 11111111 11111111 |  |  |
|  |  | trpt-Subset-r12 |  | 001 |  |  |
|  | rxParametersNCell |  |  | not present |  |  |
|  | txParameters |  |  | not present |  |  |
| commTxPoolNormalCommon/ commTxPoolNormalDedicated | sc-CP-Len |  |  | Normal |  |  |
|  | sc-Period |  |  | sf40 |  |  |
|  | sc-TF-ResourceConfig | prb-Num |  | 12 |  | 25 |
|  |  | prb-Start |  | 0 |  | 0 |
|  |  | prb-End |  | 24 |  | 49 |
|  |  | offsetIndicator |  | 0 |  |  |
|  |  | subframeBitmap |  | 00011000 00000000 00000000 00000000 00000000 |  |  |
|  | data-CP-Len |  |  | Normal |  |  |
|  | dataHoppingConfig | hoppingParameter |  | 0 |  |  |
|  |  | numSubbands |  | ns1 |  |  |
|  |  | rb-Offset |  | 0 |  |  |
|  | ue-SelectedResourceConfig | data-TF-ResourceConfig | prb-Num | 12 |  | 25 |
|  |  |  | prb-Start | 0 |  | 0 |
|  |  |  | prb-End | 23 |  | 49 |
|  |  |  | offsetIndicator | 0 |  |  |
|  |  |  | subframeBitmap | 00000000 11111111 11111111 11111111 11111111 |  |  |
|  |  | trpt-Subset-r12 |  | 001 |  |  |
|  | rxParametersNCell |  |  | not present |  |  |
|  | txParameters | sc-TxParameters | alpha | al0 |  |  |
|  |  |  | p0 | 31 |  |  |
|  |  | dataTxParameters | alpha | al0 |  |  |
|  |  |  | p0 | 31 |  |  |
| SL-SyncConfig | syncCP-Len |  |  | Normal |  |  |
|  | syncOffsetIndicator |  |  | 2 |  |  |
|  | slssid |  |  | 30 |  |  |
|  | txParameters | txParametersGeneral | alpha | al0 |  |  |
|  |  |  | p0 | 31 |  |  |
|  |  | syncTxThreshIC |  | 0 (-infinity) |  |  |
|  | rxParamsNCell |  |  | not present |  |  |

Table A.3.12.5-2: ProSe Direct Communication pre-configuration for E-UTRAN FDD for out-of-network coverage operation (Configuration #2)

| Information Element |  |  |  | Value (5MHz) | Value (10MHz) |  |
| --- | --- | --- | --- | --- | --- | --- |
| preconfigSync | syncCP-Len-r12 |  |  | Normal |  |  |
|  | syncOffsetIndicator1 |  |  | 2 |  |  |
|  | syncOffsetIndicator2 |  |  | 1 |  |  |
|  | syncTxParameters |  |  | 31 |  |  |
|  | syncTxThreshOoC |  |  | 0 (-110dBm / 15kHz) |  |  |
|  | filterCoefficient |  |  | fc0 |  |  |
|  | syncRefMinHyst |  |  | dB0 |  |  |
|  | syncRefDiffHyst |  |  | dB0 |  |  |
| preconfigComm | sc-CP-Len |  |  | Normal |  |  |
|  | sc-Period |  |  | sf40 |  |  |
|  | sc-TF-ResourceConfig | prb-Num |  | 12 | 25 |  |
|  |  | prb-Start |  | 0 | 0 |  |
|  |  | prb-End |  | 23 | 49 |  |
|  |  | offsetIndicator |  | 0 |  |  |
|  |  | subframeBitmap |  | 00011000 00000000 00000000 00000000 00000000 |  |  |
|  | data-CP-Len |  |  | Normal |  |  |
|  | dataHoppingConfig | hoppingParameter |  | 0 |  |  |
|  |  | numSubbands |  | ns1 |  |  |
|  |  | rb-Offset |  | 0 |  |  |
|  | ue-SelectedResourceConfig | data-TF-ResourceConfig | prb-Num | 12 |  | 25 |
|  |  |  | prb-Start | 0 |  | 0 |
|  |  |  | prb-End | 23 |  | 49 |
|  |  |  | offsetIndicator | 0 |  |  |
|  |  |  | subframeBitmap | 00000000 11111111 11111111 11111111 11111111 |  |  |
|  |  | trpt-Subset-r12 |  | 001 |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

### A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery

#### A.7.5.3.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the allowed PCell interruptions due to ProSe Direct Discovery defined in clause 7.16.3.1 and clause 7.16.3.3. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.3.1-1 and Table A.7.5.3.1-2 below. There is one active cell (PCell) in this test and 24 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe). Two tests (Test 1 and Test 2) are defined to verify interruptions due to synchronous (Test 1) and asynchronous (Test 2) ProSe Direct Discovery.

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Discovery announcements from other active Sidelink UEs on the ProSe Direct Discvoery resources.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating discRxInterest during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Discovery.

Table A.7.5.3.1-1: Test parameters for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| Active cell |  | Cell 1 |  | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |  |
| T1 | s | 5.12 |  |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |  |
| T3 | s | 10.24 |  |  |

Table A.7.5.3.1-2: ProSe Direct Discovery configuration for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-1 (Configuration #1) | As specified in Table A.3.12.4-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | PDP.2.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery (Test 1 and 2) and SLSS (for Test 2) |

Table A.7.5.3.1-3: Cell specific test parameters for interruption due to ProSe Direct Discovery tests

| Parameter |  |  | Unit |  | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T3 |  |
| E-UTRA RF Channel Number |  |  |  |  | 1 |  |  |  |  |
| BWchannel |  |  | MHz |  | 5 |  |  |  |  |
| UE RRC state |  |  |  |  | IDLE | CONNECTED |  |  |  |
| Paging configuration | defaultPagingCycle |  |  |  | rf256 | N/A |  |  |  |
|  | nB |  |  |  | T / 32 |  |  |  |  |
| DRX |  |  |  |  | N/A | OFF |  |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  |  |  |  | N/A | None |  | R.5 FDD |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1 |  |  |  |  | R.11 FDD |  |  |  |  |
| OCNG Pattern |  |  | OP.16 FDD |  | OP.15 FDD |
| PBCH_RA |  |  | dB |  | 0 |  |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz |  | -98 |  |  |  |  |
|  |  |  | dB |  | 16 |  |  |  |  |
| RSRP Note3 |  |  | dBm/15 kHz |  | -82 |  |  |  |  |
| SCH_RP Note 3 |  |  | dBm/15 kHz |  | -82 |  |  |  |  |
| Propagation Condition |  |  |  |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

#### A.7.5.3.2	Test Requirements

The UE shall be scheduled on PCell continuously during T3.

In Test 1, at least 98.75% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACKs can occur only on subframe ‘n’, if either n±1 subframe is a discovery subframe, or if n-3, or n-5 is a discovery subframe.

NOTE: For the test configuration in Table A.7.5.3.1-1 and Table A.7.5.3.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 159, 163, 162, 166, corresponding to allowed interruptions on subframe 159 and 162.

In Test 2, at least 97.5% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACK can occur only on subframe ‘n’, if either n±5 subframe is a discovery or SLSS subframe, or if n+1, or n-9 is a discovery or SLSS subframe.

NOTE: For the test configuration in Table A.7.5.3.1-1 and Table A.7.5.3.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 135, 139, 145, 149, 155, 159, 166, 170, corresponding to allowed interruptions on subframes 135, 145, 155 and 166.

< END OF CHANGE #2 >

< START OF CHANGE #3 >

### A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication

#### A.7.5.5.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to PCell interruptions due to ProSe Direct Communication defined in clause 7.16.3. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Communication.

The test parameters are given in Table A.7.5.5.1-1, Table A.7.5.5.1-2 and Table A.7.5.5.1-3 below. There is one active cell (PCell) in this test and 12 (5MHz) or 16 (10 MHz) active Sidelink UEs in this test transmitting ProSe Direct Communication.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Communication transmission from other active Sidelink UEs on the ProSe Direct Communication resoruces.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating commRxInterestedFreq during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to repond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Communication (no missed ACK/NACKs are allowed).

Table A.7.5.5.1-1: Test parameters for interruption due to ProSe Direct Communication tests

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| T1 | s | 5.12 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |
| T3 | s | 10.24 |  |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.5.1-2: ProSe Direct Communication specific configuration for interruption due to ProSe Direct Communication tests

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| ProSe Direct Communication configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs ConfigurationNote 1 |  | PCP.1.FDD As specified in Table A.3.12.8.1-1 | Transmitting ProSe Direct Communication (PSCCH + PSSCH) |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.5.1-2: Cell specific test parameters for interruption due to ProSe Direct Communication tests

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |
| BWchannel Note 4 |  | MHz | 5 or 10 |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |
| Paging configuration | defaultPagingCycle |  | rf256 | N/A |  |
|  | nB |  | T / 32 |  |  |
| DRX |  |  | N/A | OFF |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1, Note 4 |  |  | N/A | None | R.7 FDD (5MHz) or R.3 FDD (10MHz) (Note 5 applies) |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1, Note 4 |  |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD |  |  |
| OCNG PatternNote 4 |  |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD |  | OP.20 FDD (5MHz) or OP.10 FDD (10MHz) |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |
|  |  | dB | 16 |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: This test is according to the principle defined in section A.3.12.3. Note 5: The PDSCH scheduled subframes for R.7 FDD (5MHz) / R.3 FDD (10MHz) is changed as per the following bitmap that repeats every 40ms. PDSCH scheduled subframe bitmap: {01110111 11110111 11110111 11110111 11110110}. |  |  |  |  |  |

#### A.7.5.3.2	Test Requirements

The UE shall be scheduled on PCell continuously during T3. During T3, 100% of all expected ACK/NACKs shall be transmitted by the ProSe UE.

< END OF CHANGE #3 >