3GPP TSG-RAN WG4 Meeting #80	R4-166565

# Gothenburg, Sweden, 22 - 26 August, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS37.105 | CR | 0015 | rev |  | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 37.105: Readability improvements and corrections (section 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Core |  |  |  |  |  |  | Date: |  | 2016-08-12 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In order to improve the readability and consistency of the TS 37.105 specification text, number of improvements was identified in Section 3. Such corrections are considered especially important for readers unfamiliar with the AAS specification. NOTE: this is resubmission of the R4-164269, from RAN4#79. Removal of the RDN definition was withdrawn. Number of editorial correction were added to this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Despite of general readability improvements in Section 3, the following corrections were implemented: - Alignment of half-power beam vs. -3dB beam wording usage among definitions - Removal of the contiguous spectrum definition – not used in the specification - Alignment of the terminology (according to the definitions) - Application of the drafting rules and editorial corrections (e.g. use of the capital letter, italics, etc.) - Correction of the missing reference - Removal of symbols not used in the specification - Additional abbreviations added, as used in the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduced readability and inconsistencies among definitions will remain in the specification. Unnecessary symbols reducing readability. Missing abbreviations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 37.145-1, TS 37.145-2 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Start of change >

# Contents

Foreword	12

1	Scope	13

2	References	13

3	Definitions, symbols and abbreviations	14

3.1	Definitions	14

3.2	Symbols	17

3.3	Abbreviations	18

4	General	19

4.1	Relationship between the AAS BS specification and non-AAS BS single RAT & MSR specifications	19

4.2	Relationship between minimum requirements and test requirements	19

4.3	Conducted and radiated requirement reference points	19

4.4	Base station classes for AAS BS	20

4.5	Regional Requirements	21

4.6	Operating Bands and Band Categories	22

4.7	Channel arrangements	22

4.8	Requirements for contiguous and non-contiguous spectrum	22

4.9	Requirements for AAS BS capable of operation in multiple operating bands	22

5	Applicability of Requirements	23

5.1	General	23

5.2	Band category 1 (BC1)and band category 2 (BC2)	23

5.3	Band category 3 (BC3)	25

6	Conducted transmitter characteristics	26

6.1	General	26

6.2	Base station output power	27

6.2.1	General	27

6.2.2	Maximum output power	27

6.2.2.1	General	27

6.2.2.2	Minimum requirement for MSR operation	27

6.2.2.2.1	General	27

6.2.2.2.2	Additional requirements (regional)	27

6.2.2.3	Minimum requirement for single RAT UTRA operation	27

6.2.2.4	Minimum requirement for single RAT E-UTRA operation	27

6.2.2.4.1	General	27

6.2.2.4.2	Additional requirements (regional)	27

6.2.3	UTRA FDD primary CPICH power	28

6.2.3.1	General	28

6.2.3.2	Minimum requirement for MSR operation	28

6.2.3.3	Minimum requirement for single RAT UTRA operation	28

6.2.3.4	Minimum requirement for single RAT E-UTRA operation	28

6.2.4	UTRA TDD primary CCPCH power	28

6.2.4.1	General	28

6.2.4.2	Minimum requirement for MSR operation	28

6.2.4.3	Minimum requirement for single RAT UTRA operation	29

6.2.4.4	Minimum requirement for single RAT E-UTRA operation	29

6.2.5	UTRA FDD additional CPICH power for MIMO mode	29

6.2.5.1	General	29

6.2.5.2	Minimum requirement for MSR operation	30

6.2.5.3	Minimum requirement for single RAT UTRA operation	30

6.2.5.4	Minimum requirement for single RAT E-UTRA operation	31

6.2.6	E-UTRA DL RS power	31

6.2.6.1	General	31

6.2.6.2	Minimum requirement for MSR operation	31

6.2.6.3	Minimum requirement for single RAT UTRA operation	31

6.2.6.4	Minimum requirement for single RAT E-UTRA operation	31

6.3	Output power dynamics	32

6.3.1	General	32

6.3.2	UTRA Inner loop power control in the downlink	32

6.3.2.1	General	32

6.3.2.2	Minimum requirement for MSR operation	32

6.3.2.3	Minimum requirement for single RAT UTRA operation	32

6.3.2.4	Minimum requirement for single RAT E-UTRA operation	33

6.3.3	Power control dynamic range	33

6.3.3.1	General	33

6.3.3.2	Minimum requirement for MSR operation	33

6.3.3.3	Minimum requirement for single RAT UTRA operation	33

6.3.3.4	Minimum requirement for single RAT E-UTRA operation	33

6.3.4	Total power dynamic range	33

6.3.4.1	General	33

6.3.4.2	Minimum requirement for MSR operation	34

6.3.4.3	Minimum requirement for single RAT UTRA operation	34

6.3.4.4	Minimum requirement for single RAT E-UTRA operation	34

6.3.5	IPDL time mask	34

6.3.5.1	General	34

6.3.5.2	Minimum requirement for MSR operation	34

6.3.5.3	Minimum requirement for single RAT UTRA operation	34

6.3.5.4	Minimum requirement for single RAT E-UTRA operation	34

6.3.6	RE Power control dynamic range	34

6.3.6.1	General	34

6.3.6.2	Minimum requirement for MSR operation	35

6.3.6.3	Minimum requirement for single RAT UTRA operation	35

6.3.6.4	Minimum requirement for single RAT E-UTRA operation	35

6.4	Transmit ON/OFF power	35

6.4.1	General	35

6.4.2	Transmitter OFF power	35

6.4.2.1	General	35

6.4.2.2	Minimum requirement for MSR operation	35

6.4.2.3	Minimum requirement for single RAT UTRA operation	35

6.4.2.4	Minimum requirement for single RAT E-UTRA operation	35

6.4.3	Transmitter transient period	36

6.4.3.1	General	36

6.4.3.2	Minimum requirement for MSR operation	36

6.4.3.3	Minimum requirement for single RAT UTRA operation	36

6.4.3.4	Minimum requirement for single RAT E-UTRA operation	36

6.5	Transmitted signal quality	36

6.5.1	General	36

6.5.2	Frequency Error	36

6.5.2.1	General	36

6.5.2.2	Minimum requirement for MSR operation	37

6.5.2.3	Minimum requirement for single RAT UTRA operation	37

6.5.2.4	Minimum requirement for single RAT E-UTRA operation	37

6.5.3	Time alignment error	37

6.5.3.1	General	37

6.5.3.2	Minimum requirement for MSR operation	38

6.5.3.3	Minimum requirement for single RAT UTRA operation	38

6.5.3.4	Minimum requirement for single RAT E-UTRA operation	38

6.5.4	Modulation quality	38

6.5.4.1	General	38

6.5.4.2	Minimum requirement for MSR operation	38

6.5.4.3	Minimum requirement for single RAT UTRA operation	38

6.5.4.4	Minimum requirement for single RAT E-UTRA operation	39

6.5.5	Transmit pulse shape filter	39

6.5.5.1	General	39

6.5.5.2	Minimum requirement for MSR operation	39

6.5.5.3	Minimum requirement for single RAT UTRA operation	39

6.5.5.4	Minimum requirement for single RAT E-UTRA operation	39

6.6	Unwanted Emissions	39

6.6.1	General	39

6.6.2	Occupied bandwidth	40

6.6.2.1	General	40

6.6.2.2	Minimum requirement for MSR operation	40

6.6.2.3	Minimum requirement for single RAT UTRA operation	40

6.6.2.4	Minimum requirement for single RAT E-UTRA operation	40

6.6.3	Adjacent Channel Leakage power Ratio	41

6.6.3.1	General	41

6.6.3.2	Minimum requirement for MSR operation	41

6.6.3.3	Minimum requirement for single RAT UTRA operation	41

6.6.3.4	Minimum requirement for single RAT E-UTRA operation	41

6.6.4	Spectrum emission mask	42

6.6.4.1	General	42

6.6.4.2	Minimum requirement for MSR operation	42

6.6.4.3	Minimum requirement for single RAT UTRA operation	42

6.6.4.3.1	General	42

6.6.4.3.2	Basic limits for single RAT UTRA FDD operation	42

6.6.4.3.3 	Basic limits for single RAT UTRA TDD 1,28Mcps option operation	47

6.6.4.4	Minimum requirement for single RAT E-UTRA operation	49

6.6.5	Operating band unwanted emission	50

6.6.5.1	General	50

6.6.5.2	Minimum requirement for MSR operation	50

6.6.5.2.1	General	50

6.6.5.2.2	Basic limits for Band Categories 1 and 3	51

6.6.5.2.3	Basic limit for Band Category 2	54

6.6.5.2.4	Additional requirements	59

6.6.5.4	Minimum requirement for single RAT E-UTRA operation	59

6.6.5.4.1	General	59

6.6.5.4.2 	Basic limits for Wide Area BS (Category A)	61

6.6.5.4.3	Basic limits for Wide Area BS (Category B)	64

6.6.5.4.3.1	General	64

6.6.5.4.3.2	Category B requirements (Option 1)	64

6.6.5.4.3.3	Category B (Option 2)	68

6.6.5.4.4	Basic limits for Local Area BS (Category A and B)	70

6.6.5.4.5	Basic limits for Medium Range BS (Category A and B)	71

6.6.5.4.6	Basic limits for Local Area and Medium Range BS in Band 46 (Category A and B)	74

6.6.5.4.7	Additional requirements	74

6.6.6	Spurious emission	74

6.6.6.1	General	74

6.6.6.2	Minimum requirement for MSR operation	75

6.6.6.3	Minimum requirement for single RAT UTRA operation	75

6.6.6.4	Minimum requirement for single RAT E-UTRA operation	75

6.7	Transmitter intermodulation	76

6.7.1	General	76

6.7.2	Minimum requirement for MSR operation	76

6.7.2.1	General co-location minimum requirement	76

6.7.2.2	Additional co-location minimum requirement (BC1 and BC2)	77

6.7.2.3	Additional co-location minimum requirement (BC3)	78

6.7.2.4	Additional co-location minimum requirements	78

6.7.2.5	Intra-system minimum requirement	78

6.7.3	Minimum requirement for single RAT UTRA operation	79

6.7.3.1	General co-location minimum requirement for FDD UTRA	79

6.7.3.2	General co-location minimum requirement for 1,28 Mcps TDD UTRA	79

6.7.3.3	Intra-system minimum requirement	80

6.7.4	Minimum requirement for single RAT E-UTRA operation	80

6.7.4.1	General co-location minimum requirement	80

6.7.4.2	Additional requirement for Band 41	81

6.7.4.3	Intra-system minimum requirement	81

7	Conducted receiver characteristics	82

7.1	General	82

7.2	Reference sensitivity level	83

7.2.1	General	83

7.2.2	Minimum requirement for MSR operation	83

7.2.3	Minimum requirement for single RAT UTRA operation	83

7.2.4	Minimum requirement for single RAT E-UTRA operation	83

7.3	Dynamic range	83

7.3.1	General	83

7.3.2	Minimum requirement for MSR operation	83

7.3.3	Minimum requirement for single RAT UTRA operation	84

7.3.4	Minimum requirement for single RAT E-UTRA operation	84

7.4	Adjacent channel selectivity, general blocking, and narrowband blocking	84

7.4.1	General	84

7.4.2	Minimum requirement for MSR operation	84

7.4.2.1	General minimum requirement	84

7.4.2.2	General narrowband blocking minimum requirement	86

7.4.2.3	Additional BC3 blocking minimum requirement	86

7.4.3	Minimum requirement for single RAT UTRA operation	87

7.4.4	Minimum requirement for single RAT E-UTRA operation	87

7.5	Blocking	87

7.5.1	General	87

7.5.2	Minimum requirement for MSR operation	87

7.5.2.1	General minimum requirement	87

7.5.2.2	Co-location minimum requirement	88

7.5.3	Minimum requirement for single RAT UTRA operation	91

7.5.3.1	General minimum requirement	91

7.5.3.2	Co-location minimum requirement	91

7.5.4	Minimum requirement for single RAT E-UTRA operation	91

7.5.4.1	General minimum requirement	91

7.5.4.2	Co-location minimum requirement	91

7.6	Receiver spurious emissions	92

7.6.1	General	92

7.6.2	Minimum requirement for MSR operation	92

7.6.2.1	General minimum requirement	92

7.6.3	Minimum requirement for single RAT UTRA operation	93

7.6.4	Minimum requirement for single RAT E-UTRA operation	93

7.7	Receiver intermodulation	94

7.7.1	General	94

7.7.2	Minimum requirement for MSR operation	94

7.7.2.1	General intermodulation minimum requirement	94

7.7.2.2	General narrowband intermodulation minimum requirement	95

7.7.3	Minimum requirement for single RAT UTRA operation	96

7.7.4	Minimum requirement for single RAT E- UTRA operation	97

7.8	In-channel selectivity	97

7.8.1	General	97

7.8.2	Minimum requirement for MSR operation	97

7.8.3	Minimum requirement for single RAT UTRA operation	97

7.8.4	Minimum requirement for single RAT E- UTRA operation	97

8	Performance requirements	98

8.1	General	98

8.1.1	UTRA operation	98

8.1.2	E-UTRA operation	99

8.2	Minimum requirements for MSR operation	99

8.3	Minimum requirements for UTRA operation	99

8.4	Minimum requirements for E-UTRA operation	100

9	Radiated transmitter characteristics	100

9.1	General	100

9.2	Radiated transmit power	100

9.2.1	General	100

9.2.2	Minimum requirement for MSR operation	100

9.2.3	Minimum requirement for single RAT UTRA operation	100

9.2.4	Minimum requirement for single RAT E-UTRA operation	101

10	Radiated receiver characteristics	101

10.1	General	101

10.2	OTA sensitivity	101

10.2.1	General	101

10.2.2	Minimum requirement for MSR operation	102

10.2.3	Minimum requirement for single RAT UTRA operation	102

10.2.4	Minimum requirement for single RAT E-UTRA operation	102

Annex A (normative): Environmental requirements for the BS equipment	103

Annex B (informative): Change history	104

-

< Next modified section >

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

NOTE:	Multi-word definitions are treated as linguistic expressions and printed in italic font throughout this requirement specification. Linguistic expressions may not be split and are to be printed in their entirety.

AAS BS receiver: composite receiver function of an AAS BS receiving in an uplink operating band

active antenna system base station: BS system which combines an antenna array with an transceiver unit array and a radio distribution network

active transmitter unit: transmitter unit which is ON, and has the ability to send modulated data streams that are parallel and distinct to those sent from other transmitter units to one or more TAB connectors at the transceiver array boundary

band category: group of operating bands for which the same MSR scenarios apply

base station RF bandwidth: bandwidth in which a base station transmits and/or receives single or multiple carrier(s) and/or RATs simultaneously within a supported operating band

NOTE:	In single carrier operation, the base station RF bandwidth is equal to the channel bandwidth.

base station RF bandwidth edge: frequency of one of the edges of the base station RF bandwidth

basic limit: emissions limit taken from the non-AAS BS specifications that is converted into a per TAB connector TX min cell group AAS BS emissions limit, or into a per TAB connector RX min cell group AAS BS emissions limit by scaling, depending on the context

beam: main lobe of a radiation pattern from an AAS BS

NOTE:	For certain AAS BS antenna array, there may be more than one beam.

beam centre direction: direction equal to the geometric centre of the half-power EIRP contour of the beam

beam direction pair: data set consisting of  the beam centre direction and the related beam peak direction

beam peak direction: direction where the maximum EIRP is supposed to be found

beamwidth: angles describing the major and minor axes of an ellipsoid closest fit to an essentially elliptic half-power EIRP contour of the beam

carrier: modulated waveform conveying the E-UTRA or UTRA physical channels

carrier aggregation: aggregation of two or more E-UTRA component carriers in order to support wider transmission bandwidths

channel bandwidth: RF bandwidth supporting a single RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell

NOTE 1:	The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

NOTE 2:	For UTRA FDD, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.104 [2], For UTRA TDD 1,28 Mcps, the channel bandwidth is the nominal channel spacing specified in 3GPP TS 25.105 [3].

NOTE 3:	For E-UTRA, the channel bandwidths are specified in 3GPP TS 36.104 [4].

code domain power: part of the mean power which correlates with a particular (OVSF) code channel in a UTRA signal

NOTE:	The sum of all powers in the code domain equals the mean power in a bandwidth of (1+) times the chip rate of the radio access mode.

demodulation branch: single input of the AAS BS receiver to the demodulation algorithms.

NOTE 1:		For UTRA non-AAS BS a demodulation branch is referred to as a receive diversity branch or an UL MIMO branch. For E-UTRA non-AAS BS a demodulation branch is referred to as an RX antenna in the performance requirement tables.

NOTE 2:	The term "RX antenna" in chapter 8 (i.e. Performance requirements) of the E-UTRA specification 36.104 [4] does not refer to physical receiver antennas.

downlink operating band: part of the (FDD) operating band designated for downlink transmission

EIRP accuracy directions set: beam peak directions for which the EIRP accuracy requirement is intended to be met.

equivalent isotropic radiated power: equivalent power radiated from an isotropic directivity device producing the same field intensity at a point of observation as the field intensity radiated in the direction of the same point of observation by the discussed device

NOTE:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

equivalent isotropic sensitivity: sensitivity for an isotropic directivity device equivalent to the sensitivity of the discussed device exposed to an incoming wave from a defined AoA

NOTE 1:	The sensitivity is the minimum received power level at which a RAT specific requirement is met.

NOTE 2:	Isotropic directivity is equal in all directions (i.e. 0 dBi).

inter RF bandwidth gap: frequency gap between two consecutive base station RF bandwidths that are placed within two supported operating bands

maximum carrier output power per TAB connector: mean power level measured on a particular carrier at the TAB connector(s), during the transmitter ON period in a specified referencecondition

maximum throughput: maximum achievable throughput for a reference measurement channel

MSR operation: operation of AAS BS declared to be MSR in particular operating band(s)

multi-band requirements: requirements applying per one single operating band with exclusion bands or other multi-band provisions as defined for each requirement

multi-band TAB connector: TAB connector supporting operation in multiple operating bands through common active electronic component(s)

NOTE:	For common TX and RX TAB connectors, the definition applies where common active electronic components are in the transmit path and/or in the recieve path.

non-AAS BS: BS conforming to one of  the RF requirement specifications 3GPP TS 25.104 [2], 3GPP TS 25.105 [3], 3GPP TS 36.104 [4] or 3GPP TS 37.104 [5].

non-contiguous spectrum: spectrum consisting of two or more sub-blocks separated by sub-block gap(s)

OTA sensitivity directions declaration: set of manufacturer declarations comprising at least one  set of declared minimum EIS values (with related RAT and channel bandwidth), and related directions over which the EIS applies

output power at a TAB connector: mean power delivered to a load with resistance equal to the nominal load impedance of the TAB connector

polarization match: condition that exists when a plane wave, incident upon an antenna from a given direction, has a polarization that is the same as the receiving polarization of the antenna in that direction

radio bandwidth: frequency difference between the upper edge of the highest used carrier and the lower edge of the lowest used carrier

radio distribution network: passive network which distributes radio signals generated by the transceiver unit array to the antenna array, and/or distributes the radio signals collected by the antenna array to the transceiver unit array

NOTE:	In the case when the active transceiver units are physically integrated with the array elements of the antenna array, the radio distribution network is a one-to-one mapping.

rated beam EIRP: EIRP that is declared as being achieved in the beam peak direction associated with a particular beam direction pair

rated carrier output power per TAB connector: mean power level associated with a particular carrier the manufacturer has declared to be available at the TAB connector, during the transmitter ON period in a specified reference condition

rated total output power per TAB connector: mean power level associated with a particular operating band the manufacturer has declared to be available at the TAB connector, during the transmitter ON period in a specified reference condition

receive period: time during which the AAS BS is receiving data sub-frames (or UpPTS in case of E-UTRA frame Type2) on a (TDD) carrier

receiver target: aAoA in which reception is performed by AAS BS

receiver target redirection range: union of all the sensitivity RoAoA achievable through redirecting the receiver target related to particular OSDD

receiver target reference direction: direction inside the OTA sensitivity directions declaration declared by the manufacturer for conformance testing.

reference beam direction pair: declared beam direction pair, including reference beam centre direction and reference beam peak direction where the reference beam peak direction is the direction for the intended maximum EIRP within the EIRP accuracy directions set compliance

sensitivity RoAoA: RoAoA within the OTA sensitivity directions declaration, within which the declared EIS(s) of an OSDD is intended to be achieved at any instance of time for a specific AAS BS direction setting

single band requirements: requirements applying per one single operating band without exclusion bands or other multi-band provisions

single band TAB connector: TAB connector supporting operation either in a single operating band only, or in multiple operating bands but without any common active electronic component(s)

single RAT E-UTRA operation: operation of AAS BS declared to be Single RAT E-UTRA in the operating band

single RAT UTRA operation: operation of AAS BS declared to be Single RAT UTRA in the operating band

sub-block: one contiguous allocated block of spectrum for use by the same Base Station

NOTE:	There may be multiple instances of sub-blocks within an RF bandwidth.

sub-block gap: frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation

TAB connector: transceiver array boundary connector

TAB connector RX min cell group: operating band specific declared group of TAB connectors to which RX requirements are applied.

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for receiving a cell when the AAS BS setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

TAB connector TX min cell group: operating band specific declared group of TAB connectors to which TX requirements are applied.

NOTE:	Within this definition, the group corresponds to the group of TAB connectors which are responsible for transmitting a cell when the AAS BS setting corresponding to the declared minimum number of cells with transmission on all TAB connectors supporting an operating band, but its existence is not limited to that condition.

throughput: number of payload bits successfully received per second for a reference measurement channel in a specified reference condition

transceiver array boundary: conducted interface between the transceiver unit array and the composite antenna

transmission bandwidth: bandwidth of an instantaneous E-UTRA transmission from a UE or BS, measured in Resource Block units

transmitter OFF period: time period during which the transmitter is scheduled not to transmit

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter ON period: time period during which the transmitter is transmitting data and/or reference symbols

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

transmitter transient period: time period during which the transmitter unt belonging to the transceiver unit array is changing from the OFF period to the ON period or vice versa

NOTE:	For AAS BS, this definition applies per TAB connector and operating band.

uplink operating band: part of the (FDD) operating band designated for uplink transmission

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Ncells	A declared number corresponding to the minimum number of cells that can be transmitted by an AAS BS in a particular band with transmission on all TAB connectors supporting the operating band.

NRXU,active	The number of active receiver units. The same as the [number of receiver diversity branches] to which compliance is declared for chapter 8 performance requirements.

NRXU,counted	The number of active receiver units that are taken into account for unwanted emission scaling, as calculated in subclause 7.1.

NRXU,countedpercell	The number of active receiver units that are taken into account for unwanted emissions scaling per cell, as calculated in subclause 7.6. The number is defined in subclause 7.1.

NTXU, active	The number of active transmitter units.

NTXU,counted	The number of active transmitter units , as calculated in subclause 6.1, that are taken into account or conducted TX power limit in subclause 6.2, and for unwanted emissions scaling.

NTXU,countedpercell	The number of active transmitter units  that are taken into account for emissions scaling per cell, as calculated in subclause 6.6. The number is defined in subclause 6.1.

Pmax,c,TABC	The maximum carrier output power per TAB connector

PRated,c,TABC	The rated carrier output power per TAB connector

PRated,c,sys	The sum of PRated,c,TABC for all TAB connectors for a single carrier

Prated,t,group	The sum of PRated,t,TABC for all TAB connectors belonging to a specified group

PRated,t,TABC	The rated total output power per TAB connector

PREFSENS	Reference Sensitivity power level

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAS BS	Active Antenna System Base Station

ACLR	Adjacent Channel Leakage power Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

BC	Band Category

BER	Bit Error Rate

CACLR	Cumulative ACLR

CW	Continuous Wave (unmodulated signal)

D-CPICH	Demodulation Common Pilot Channel

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FCC	Federal Communications Commission

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

HARQ	Hybrid Automatic Repeat Request

HS-DSCH 	High Speed Downlink Shared Channel

ITU	International Telecommunication Union

ITU-R	Radio communication Sector of the ITU

MIMO	Multiple Inputs Multiple Outputs

MSR	Multi-Standard Radio

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

PCCPCH	Primary Common Control Physical CHannel

RAT	Radio Access Technology

RB	Resource Block (for E-UTRA)

RDN	Radio Distribution Network

RE	Resource Element

RF	Radio Frequency

RoAoA	Range of Angles of Arrival

TAB	Transceiver Array Boundary

TDD	Time Division Duplex

< Next modified section >

Table 6.6.4.3.2-3: Spectrum emission mask values, 31 dBm  Pmax,c,cell < 39 dBm for UTRA FDD bands

| Frequency offset of measurement filter -3 dB point,f | Frequency offset of measurement filter centre frequency, f_offset | basic limit (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | Pmax,c,cell – 10*log10(NTXU,countedpercell) - 53 dB | 30 kHz |
| 2.7 MHz f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | Pmax,c,cell – 10*log10(NTXU,countedpercell) - 65 dB | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | Pmax,c,cell – 10*log10(NTXU,countedpercell) - 52 dB | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | Pmax,c,cell – 10*log10(NTXU,countedpercell) - 56 dB | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clause 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. |  |  |  |

Table 6.6.4.3.2-4: Spectrum emission mask values, Pmax,c,cell < 31 dBm for UTRA FDD bands

| Frequency offset of measurement filter -3 dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic Limits (Note 1, 2) | Measurement bandwidth (Note 4) |
| --- | --- | --- | --- |
| 2.5 MHz f < 2.7 MHz | 2.515MHz f_offset < 2.715MHz | -22 dBm | 30 kHz |
| 2.7 f < 3.5 MHz | 2.715MHz f_offset < 3.515MHz |  | 30 kHz |
| (Note 3) | 3.515MHz f_offset < 4.0MHz | -34 dBm | 30 kHz |
| 3.5 MHz f < 7.5 MHz | 4.0 MHz f_offset < 8.0MHz | -21 dBm | 1 MHz |
| 7.5 MHz f fmax | 8.0MHz f_offset < f_offsetmax | -25 dBm | 1 MHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation the basic limit within sub-block gaps within any operating band is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 12.5MHz from both adjacent sub blocks on each side of the sub-block gap, where the spurious emission basic limits in clause 6.6.6.5.2.2 and 6.6.6.5.5.3 shall be met. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or Base Station RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or Base Station RF Bandwidth. |  |  |  |

< Next modified section >

##### 6.6.4.3.3 	Basic limits for single RAT UTRA TDD 1,28Mcps option operation

The basic limit is specified in Tables 6.6.4.3.3-1 to 6.6.4.3.3-3 for the appropriate PRated,c,sy, where:

The mask defined in Table 6.6.4.3.3-1 to 6.6.4.3.3-3 may be mandatory in certain regions. In other regions this mask may not be applied.

For regions where this clause applies, the basic limit is for a TAB connector transmitting on a single RF carrier configured in accordance with the manufacturer’s specification. Emissions shall use the basic limits specified in table 6.6.4.3.3-1 to 6.6.4.3.3-3 for the appropriate Pmax,c,cell , in the frequency range from f = 0.8 MHz to f max from the carrier frequency, where:

-	f is the separation between the carrier frequency and the nominal -3dB point of the measuring filter closest to the carrier frequency.

-	f_offset is the separation between the carrier frequency and the center frequency of the measuring filter.-	f_offsetmax is either 4 MHz or the offset to the UTRA TDD Tx band edge as defined in subclause 4.6, whichever is the greater.

-	f max is equal to f_offsetmax minus half of the bandwidth of the mesurement filter. Inside any Inter RF bandwidth gaps with Wgap < 8 MHz for multi-band TAB connector, emissions shall not exceed the cumulative sum of the basic limits specified at the Base Station RF bandwidth edges on each side of Inter RF bandwidth gap. The basic limit for Base Station RF bandwidth edge is specified in Tables 6.6.4.3.3-1 to 6.6.4.3.3-3 below, where in this case.

-	f equal to 0.8MHz plus the separation between the Base Station RF bandwidth edge frequency and the nominal -3dB point of the measuring filter closest to the Base Station RF bandwidth edge.

-	f_offset is equal to 0.8MHz plus the separation between the Base Station RF bandwidth edge frequency and the center frequency of the measuring filter.

-	f_offsetmax is either 4 MHz or the offset to the UTRA TDD Tx band edge as defined in subclause 4.6, whichever is the greater.

-	f max is equal to f_offsetmax minus half of the bandwidth of the mesurement filter.

For a multi-carrier TAB connetcor, the definitions above apply to the lower edge of the carrier transmitted at the lowest carrier frequency and the upper edge of the carrier transmitted at the highest carrier frequency within a specified frequency.

Figure 6.6.4.3.3-1

Table 6.6.4.5.2.2-1: Basic Limits for spectrum emission mask values, Pmax,c,cell   34 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | Basic Limits | Measurement bandwidth |
| --- | --- | --- |
| 0.8 MHz f < 1.0 MHz | -20 dBm | 30 kHz |
| 1.0 MHz f < 1.8 MHz |  | 30 kHz |
| See note | -28 dBm | 30 kHz |
| 1.8 MHz f fmax | -13 dBm | 1 MHz |
| NOTE 1: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap. |  |  |

Table 6.6.4.5.2.2-2: Basic Limits for spectrum emission mask values, 26 dBm  Pmax,c,cell < 34 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | Basic Limits | Measurement bandwidth |
| --- | --- | --- |
| 0.8 MHz f < 1.0 MHz | Pmax,c,cell – 10*log10(NTXU,countedpercell) -54 dB | 30 kHz |
| 1.0 MHz f < 1.8 MHz |  | 30 kHz |
| See note | Pmax,c,cell – 10*log10(NTXU,countedpercell) -62 dB | 30 kHz |
| 1.8 MHz f fmax | Pmax,c,cell – 10*log10(NTXU,countedpercell) - 47 dB | 1 MHz |
| NOTE 1: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap. |  |  |

Table 6.6.4.5.2.2-3: Basic Limits for spectrum emission mask values, Pmax,c,cell < 26 dBm for 1,28 Mcps TDD

| Frequency offset of measurement filter centre frequency, f_offset | Basic Limits | Measurement bandwidth |
| --- | --- | --- |
| 0.8 MHz f < 1.0 MHz | -28 dBm | 30 kHz |
| 1.0 MHz f < 1.8 MHz |  | 30 kHz |
| See note | -36 dBm | 30 kHz |
| 1.8 MHz f fmax | -21 dBm | 1 MHz |
| NOTE 1: For a multi-band TAB connector with Inter RF Bandwidth gap less than 8MHz, the basic limit within the Inter RF Bandwidth gap is calculated as a cumulative sum of emissions from the two adjacent carriers on each side of the Inter RF Bandwidth gap. |  |  |

#### NOTE:	This frequency range ensures that the range of values of f_offset is continuous.

#### 6.6.4.4	Minimum requirement for single RAT E-UTRA operation

There is no spectrum emission mask requirement for a single RAT E-UTRA AAS BS.

< Next modified section >

##### 6.6.5.2.4	Additional requirements

#### The MSR operating band unwanted emission basic limits for additional requirements are the same as the basic limits specified in 3GPP TS 37.104 [9], subclause 6.6.2.4.6.6.5.3	Minimum requirement for single RAT UTRA operation

There is no operating band unwanted emission requirement for a single RAT UTRA FDD or single RAT UTRA TDD AAS BS.

#### 6.6.5.4	Minimum requirement for single RAT E-UTRA operation

< Next modified section >

##### 6.6.5.4.2 	Basic limits for Wide Area BS (Category A)

For E-UTRA TAB connector operating in Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 26, 27, 28, 29, 31, 44, 68 the basic limits are  specified in Tables 6.6.5.4.2-1 to 6.6.5.4.2-3.

Table 6.6.5.4.2-1: Wide Area BS operating band unwanted emission limits for 1.4 MHz channel bandwidth (E-UTRA bands <1GHz) for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limit (Note 1, 2) | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- |
| 0 MHz f < 1.4 MHz | 0.05 MHz f_offset < 1.45 MHz |  | 100 kHz |
| 1.4 MHz f < 2.8 MHz | 1.45 MHz f_offset < 2.85 MHz | -11 dBm | 100 kHz |
| 2.8 MHz f fmax | 2.85 MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |
| NOTE 1: For a TAB connector supporting non-contiguous spectrum operation within any operating band, the basic limit within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap. Exception is f ≥ 10MHz from both adjacent sub blocks on each side of the sub-block gap, where the basic limit within sub-block gaps shall be -13dBm/100kHz. NOTE 2: For a multi-band TAB connector with Inter RF Bandwidth gap < 20MHz the basic limit within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or Base Station RF Bandwidth on each side of the Inter RF Bandwidth gap. |  |  |  |

< End of change >