3GPP TSG-RAN4 Meeting #81	R4-1609719

# Reno, Nevada, United States, 14th Nov 2016 - 18th Nov 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4050 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of completed R14 3DL band combinations to TS 36.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14_3DL1UL-Core |  |  |  |  |  | Date: |  |  | 2016-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The following 3DL band combinations are compleleted before the end of the meeting. CA_1A-3A-38A_BCS0 CA_1A-3A-11A_BCS0 CA_1A-7C_BCS1 CA_1A-46C_BCS1 CA_1C-3A_BCS0 CA_2A-2A-30A_BCS0 CA_2A-7C_BCS0 CA_2A-12A-12A_BCS0 CA_2A-30A-66A_BCS0 CA_3A-5A-40A_BCS1 CA_3A-8A-11A_BCS0 CA_3A-28A-41A_BCS0 CA_3A-40A-40A_BCS0 CA_3A-46C_ BCS1 CA_4A-7C_BCS0 CA_4A-12A-12A_BCS0 CA_5A-12A-12A_BCS0 CA_5A-12A-66A_BCS0 CA_5A-30A-66A_BCS0 CA_5A-40A-40A_BCS0 CA_5A-46C_BCS1 CA_7A-46C_BCS1 CA_8A-28A-41A_BCS0 CA_8A-46C_BCS0 CA_8B-46A_BCS0 CA_11A-46C_BCS0 CA_12A-30A-66A_BCS0 CA_12B-66A_BCS0 CA_13A-46C_BCS0 CA_30A-66A-66A_BCS0 CA_39A-46C_BCS0 CA_39C-46A_BCS0 CA_40A-42C_BCS0 CA_40C-42A_BCS0 CA_40C-46A_BCS0 CA_40A-46C_BCS0 This CR is to introduce the corresponding requirements to TS 36.101. The missing REFSENS requirements for CA_40A-46C_BCS0 CA_40C-46A_BCS0 are added. The missing CA_Some editorial correcitions are also included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are introduced. -5.5A: Operating bands for CA -5.6A.1: Channel bandwidths per operating band for CA -6.2.5: Configured transmitted power, Tib tables are updated -7.3.1: Reference sensitivity power level, Rib tables are updated. -7.3.1A: Minimum requirements (QPSK) for CA The two bands operating bands section is supposed to be updated in 2DL/1UL basket CA big CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above 3DL band combinations are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A, 5.6A.1, 6.2.5, 7.3.1, 7.3.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# <Start of Changes>

Table 5.5A-2: Inter-band CA operating bands (two bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_1-3 | 1, 3 |
| CA_1-5 | 1, 5 |
| CA_1-7 | 1, 7 |
| CA_1-7-7 | 1, 7 |
| CA_1-8 | 1, 8 |
| CA_1-11 | 1, 11 |
| CA_1-18 | 1, 18 |
| CA_1-19 | 1, 19 |
| CA_1-20 | 1, 20 |
| CA_1-21 | 1, 21 |
| CA_1-26 | 1, 26 |
| CA_1-28 | 1, 28 |
| CA_1-38 | 1, 38 |
| CA_1-40 | 1, 40 |
| CA_1-41 | 1, 41 |
| CA_1-42 | 1, 42 |
| CA_1-46 | 1, 46 |
| CA_2-4 | 2, 4 |
| CA_2-2-4 | 2, 4 |
| CA_2-2-4-4 | 2, 4 |
| CA_2-4-4 | 2, 4 |
| CA_2-5 | 2, 5 |
| CA_2-2-5 | 2, 5 |
| CA_2-7 | 2, 7 |
| CA_2-7-7 | 2, 7 |
| CA_2-12 | 2, 12 |
| CA_2-2-12 | 2, 12 |
| CA_2-12-12 | 2, 12 |
| CA_2-13 | 2, 13 |
| CA_2-2-13 | 2, 13 |
| CA_2-17 | 2, 17 |
| CA_2-28 | 2, 28 |
| CA_2-29 | 2, 29 |
| CA_2-30 | 2, 30 |
| CA_2-2-30 | 2, 30 |
| CA_2-46 | 2, 46 |
| CA_2-46-46 | 2, 46 |
| CA_2-66 | 2, 66 |
| CA_2-2-66-66 | 2, 66 |
| CA_2-2-66 | 2, 66 |
| CA_2-66-66 | 2, 66 |
| CA_3-5 | 3, 5 |
| CA_3-7 | 3, 7 |
| CA_3-3-7 | 3, 7 |
| CA_3-3-7-7 | 3, 7 |
| CA_3-7-7 | 3, 7 |
| CA_3-8 | 3, 8 |
| CA_3-3-8 | 3, 8 |
| CA_3-19 | 3, 19 |
| CA_3-20 | 3, 20 |
| CA_3-3-20 | 3, 20 |
| CA_3-21 | 3, 21 |
| CA_3-26 | 3, 26 |
| CA_3-27 | 3, 27 |
| CA_3-28 | 3, 28 |
| CA_3-31 | 3, 31 |
| CA_3-32 | 3, 32 |
| CA_3-38 | 3, 38 |
| CA_3-40 | 3, 40 |
| CA_3-40-40 | 3, 40 |
| CA_3-41 | 3, 41 |
| CA_3-42 | 3, 42 |
| CA_3-46 | 3, 46 |
| CA_3-69 | 3, 69 |
| CA_4-5 | 4, 5 |
| CA_4-4-5 | 4, 5 |
| CA_4-7 | 4, 7 |
| CA_4-4-7 | 4, 7 |
| CA_4-7-7 | 4, 7 |
| CA_4-12 | 4, 12 |
| CA_4-4-12 | 4, 12 |
| CA_4-12-12 | 4, 12 |
| CA_4-13 | 4, 13 |
| CA_4-4-13 | 4, 13 |
| CA_4-17 | 4, 17 |
| CA_4-27 | 4, 27 |
| CA_4-28 | 4, 28 |
| CA_4-29 | 4, 28 |
| CA_4-4-29 | 4, 29 |
| CA_4-30 | 4, 30 |
| CA_4-4-30 | 4, 30 |
| CA_4-46 | 4, 46 |
| CA_4-46-46 | 4, 46 |
| CA_5-7 | 5, 7 |
| CA_5-7-7 | 5, 7 |
| CA_5-12 | 5, 12 |
| CA_5-12-12 | 5, 12 |
| CA_5-13 | 5 ,13 |
| CA_5-17 | 5, 17 |
| CA_5-25 | 5, 25 |
| CA_5-29 | 5, 29 |
| CA_5-30 | 5, 30 |
| CA_5-38 | 5, 38 |
| CA_5-40 | 5, 40 |
| CA_5-40-40 | 5, 40 |
| CA_5-46 | 5, 46 |
| CA_5-66 | 5, 66 |
| CA_5-5-66 | 5, 66 |
| CA_5-66-66 | 5, 66 |
| CA_5-5-66-66 | 5, 66 |
| CA_7-8 | 7, 8 |
| CA_7-7-8 | 7, 8 |
| CA_7-12 | 7, 12 |
| CA_7-20 | 7, 20 |
| CA_7-22 | 7, 22 |
| CA_7-28 | 7, 28 |
| CA_7-40 | 7, 40 |
| CA_7-42 | 7, 42 |
| CA_7-42-42 | 7, 42 |
| CA_7-46 | 7, 46 |
| CA_7-66 | 7, 66 |
| CA_8-11 | 8, 11 |
| CA_8-20 | 8, 20 |
| CA_8-28 | 8, 28 |
| CA_8-39 | 8, 39 |
| CA_8-40 | 8, 40 |
| CA_8-41 | 8, 41 |
| CA_8-42 | 8, 42 |
| CA_8-46 | 8, 46 |
| CA_11-18 | 11, 18 |
| CA_11-41 | 11, 41 |
| CA_11-42 | 11, 42 |
| CA_12-25 | 12, 25 |
| CA_12-30 | 12, 30 |
| CA_12-66 | 12, 66 |
| CA_12-66-66 | 12, 66 |
| CA_13-46 | 13, 46 |
| CA_13-66 | 13, 66 |
| CA_13-66-66 | 13, 66 |
| CA_18-281 | 18, 28 |
| CA_19-21 | 19, 21 |
| CA_19-282 | 19, 28 |
| CA_19-42 | 19, 42 |
| CA_19-46 | 19, 46 |
| CA_20-281 | 20, 28 |
| CA_20-31 | 20, 31 |
| CA_20-32 | 20, 32 |
| CA_20-38 | 20, 38 |
| CA_20-40 | 20, 40 |
| CA_20-42 | 20, 42 |
| CA_20-42-42 | 20, 42 |
| CA_20-67 | 20, 67 |
| CA_21-28 | 21, 28 |
| CA_21-42 | 21, 42 |
| CA_21-46 | 21, 46 |
| CA_23-29 | 23, 29 |
| CA_25-26 | 25, 26 |
| CA_25-41 | 25, 41 |
| CA_26-41 | 26, 41 |
| CA_26-46 | 26, 46 |
| CA_28-40 | 28, 40 |
| CA_28-41 | 28, 41 |
| CA_28-42 | 28, 42 |
| CA_28-46 | 28, 46 |
| CA_29-30 | 29, 30 |
| CA_29-66 | 29, 66 |
| CA_29-66-66 | 29, 66 |
| CA_30-66-66 | 30, 66 |
| CA_38-40 | 38, 40 |
| CA_38-40-40 | 38, 40 |
| CA_39-41 | 39, 41 |
| CA_40-41 | 40, 41 |
| CA_40-46 | 40, 46 |
| CA_41-42 | 41, 42 |
| CA_41-46 | 41, 46 |
| CA_42-46 | 42, 46 |
| CA_46-66 | 46, 66 |
| CA_46-46-66 | 46, 66 |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 2: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |

Table 5.5A-2a: Inter-band CA operating bands (three bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_1-3-5 | 1, 3, 5 |
| CA_1-3-7 | 1, 3, 7 |
| CA_1-3-7-7 | 1, 3, 7 |
| CA_1-3-8 | 1, 3, 8 |
| CA_1-3-11 | 1, 3, 11 |
| CA_1-3-19 | 1, 3, 19 |
| CA_1-3-20 | 1, 3, 20 |
| CA_1-3-21 | 1, 3, 21 |
| CA_1-3-26 | 1, 3, 26 |
| CA_1-3-28 | 1, 3, 28 |
| CA_1-3-38 | 1, 3, 38 |
| CA_1-3-40 | 1, 3, 40 |
| CA_1-3-41 | 1, 3, 41 |
| CA_1-3-42 | 1, 3, 42 |
| CA_1-5-7 | 1, 5, 7 |
| CA_1-5-7-7 | 1, 5, 7 |
| CA_1-5-40 | 1, 5, 40 |
| CA_1-5-46 | 1, 5, 46 |
| CA_1-7-8 | 1, 7, 8 |
| CA_1-7-20 | 1, 7, 20 |
| CA_1-7-28 | 1, 7, 28 |
| CA_1-7-40 | 1, 7, 40 |
| CA_1-7-42 | 1, 7, 42 |
| CA_1-7-46 | 1, 7, 46 |
| CA_1-8-11 | 1, 8, 11 |
| CA_1-8-28 | 1, 8, 28 |
| CA_1-8-40 | 1, 8, 40 |
| CA_1-11-18 | 1, 11, 18 |
| CA_1-18-281 | 1, 18, 28 |
| CA_1-19-21 | 1, 19, 21 |
| CA_1-19-282 | 1, 19, 28 |
| CA_1-19-42 | 1, 19, 42 |
| CA_1-20-42 | 1, 20, 42 |
| CA_1-21-28 | 1, 21, 28 |
| CA_1-21-42 | 1, 21, 42 |
| CA_1-28-42 | 1, 28, 42 |
| CA_1-41-42 | 1, 41, 42 |
| CA_2-4-5 | 2, 4, 5 |
| CA_2-2-4-5 | 2, 4, 5 |
| CA_2-4-4-5 | 2, 4, 5 |
| CA_2-2-5-66 | 2, 5, 66 |
| CA_2-2-13-66 | 2,13, 66 |
| CA_2-4-7 | 2, 4, 7 |
| CA_2-4-7-7 | 2, 4, 7 |
| CA_2-4-12 | 2, 4, 12 |
| CA_2-2-4-12 | 2, 4, 12 |
| CA_2-4-4-12 | 2, 4, 12 |
| CA_2-4-13 | 2, 4, 13 |
| CA_2-4-29 | 2, 4, 29 |
| CA_2-4-30 | 2, 4, 30 |
| CA_2-5-12 | 2, 5, 12 |
| CA_2-2-5-12 | 2, 5, 12 |
| CA_2-5-13 | 2, 5, 13 |
| CA_2-5-29 | 2, 5, 29 |
| CA_2-5-30 | 2, 5, 30 |
| CA_2-5-66 | 2, 5, 66 |
| CA_2-7-12 | 2, 7, 12 |
| CA_2-7-66 | 2, 7, 66 |
| CA_2-12-30 | 2, 12, 30 |
| CA_2-12-66 | 2, 12, 66 |
| CA_2-13-66 | 2, 13, 66 |
| CA_2-13-66-66 | 2, 13, 66 |
| CA_2-29-30 | 2, 29, 30 |
| CA_2-30-66 | 2, 30, 66 |
| CA_3-5-7 | 3, 5, 7 |
| CA_3-5-7-7 | 3, 5, 7 |
| CA_3-5-40 | 3, 5, 40 |
| CA_3-7-7-8 | 3, 7, 8 |
| CA_3-7-8 | 3, 7, 8 |
| CA_3-7-20 | 3, 7, 20 |
| CA_3-7-28 | 3, 7, 28 |
| CA_3-7-40 | 3, 7, 40 |
| CA_3-7-42 | 3, 7, 42 |
| CA_3-8-11 | 3, 8, 11 |
| CA_3-8-28 | 3, 8, 28 |
| CA_3-8-40 | 3, 19, 40 |
| CA_3-7-38 | 3, 7, 38 |
| CA_3-19-21 | 3, 19, 21 |
| CA_3-19-42 | 3, 19, 42 |
| CA_3-20-32 | 3, 20, 32 |
| CA_3-20-42 | 3, 20, 42 |
| CA_3-21-28 | 3, 21, 28 |
| CA_3-21-42 | 3, 21, 42 |
| CA_3-28-40 | 3, 28, 40 |
| CA_3-28-41 | 3, 28, 41 |
| CA_3-28-42 | 3, 28, 42 |
| CA_3-41-42 | 3, 41, 42 |
| CA_4-5-12 | 4, 5, 12 |
| CA_4-4-5-12 | 4, 5, 12 |
| CA_4-5-13 | 4, 5, 13 |
| CA_4-5-29 | 4, 5, 29 |
| CA_4-5-30 | 4, 5, 30 |
| CA_4-4-5-30 | 4, 5, 30 |
| CA_4-7-12 | 4, 7, 12 |
| CA_4-12-30 | 4, 12, 30 |
| CA_4-4-12-30 | 4, 12, 30 |
| CA_4-29-30 | 4, 29, 30 |
| CA_4-4-29-30 | 4, 29, 30 |
| CA_5-7-46 | 5, 7, 46 |
| CA_5-12-66 | 5, 12, 66 |
| CA_5-30-66 | 5, 30, 66 |
| CA_7-8-20 | 7, 8, 20 |
| CA_7-20-38 | 7, 20, 38 |
| CA_7-20-42 | 7, 20, 42 |
| CA_8-28-41 | 8, 28, 41 |
| CA_12-30-66 | 12, 30, 66 |
| CA_19-21-42 | 19, 21, 42 |
| CA_21-28-42 | 21, 28, 42 |
| CA_28-41-42 | 28, 41, 42 |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 2: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |

# <Next change>

Table 5.6A.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (two bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-5A | CA_1A-5A6 | 1 |  |  |  | Yes |  |  | 20 | 0 |
|  |  | 5 |  |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-7A | CA_1A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-7A-7A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_1A-7C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-8A | CA_1A-8A6 | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
| CA_1A-11A | - | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-18A | CA_1A-18A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 18 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-19A | CA_1A-19A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-20A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-21A | CA_1A-21A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-26A | CA_1A-26A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-28A | CA_1A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-40A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-40C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-41A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-41C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-42A | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-42C | CA_1A-42A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-46A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_1A-46C | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-46D | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-46E | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1C-3A | - | 1 | See CA_1C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-4A | CA_2A-4A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 4 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 2 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-2A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-4A-4A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-2A-4A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-5A | CA_2A-5A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-5A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-5A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-5B | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2C-5B | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-7A | CA_2A-7A | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-7A-7A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-7C | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See the CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-12A | CA_2A-12A6 | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 12 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 2 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-12A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-12B | CA_2A-12A6 | 2 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-2A-12B | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2C-12A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-13A | CA_2A-13A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-2A-13A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-17A | - | 2 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-28A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-29A | - | 2 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-29A | - | 2 | See CA_2C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-46A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_2A-46A-46C | - | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-46C | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46D | - | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-46A-46A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-46A-46D | - | 2 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-66A | - | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 66 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 2 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-66B | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-66C | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-66D | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 66 | See CA_66D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-2A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-2A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-66A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-66A-66C |  | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-2A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-2A-66C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-5A | CA_3A-5A | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 3 |
|  |  | 5 |  | Yes | Yes | Yes |  |  |  |  |
| CA_3C-5A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-7A | CA_3A-7A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-3A-7A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 1 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-3A-7A-7A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_3A-7A-7A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_3A-7B | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-7C | CA_3A-7A CA_7C | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3C-7A | CA_3A-7A CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-7C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-8A | CA_3A-8A6 | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 3 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-3A-8A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_3C-8A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
| CA_3A-19A | CA_3A-19A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_3A-20A | CA_3A-20A | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-3A-20A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-20A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-21A | CA_3A-21A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_3A-26A | CA_3A-26A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 3 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-27A | - | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 27 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-28A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 3 |  | Yes | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-28A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-31A | - | 3 |  |  | Yes | Yes | Yes | Yes | 25 | 0 |
|  |  | 31 |  | Yes | Yes |  |  |  |  |  |
| CA_3A-32A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-38A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-40A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-40A-40A | - | 3 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-40C | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3C-40A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-40C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-41A | CA_3A-41A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-41C | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-41D | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3C-41A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3C-41C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3C-41D | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-42A | CA_3A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-42C | CA_3A-42A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-46A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_3A-46C | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | - | 3 |  | Yes | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-46D | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 3 |  | Yes | Yes | Yes | Yes | Yes | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-46E | - | 3 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-69A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 69 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-5A | CA_4A-5A | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-5A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-5B | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-4A-5B | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-7A | CA_4A-7A | 4 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-4A-7A | - | 4 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 4 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-7A-7A | - | 4 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_4A-7C | - | 4 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-12A | CA_4A-12A | 4 | Yes | Yes | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 | Yes | Yes | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 12 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 3 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 4 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes |  | 20 | 5 |
|  |  | 12 |  |  | Yes |  |  |  |  |  |
| CA_4A-4A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-12A-12A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_4A-4A-12B | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-12B | CA_4A-12A | 4 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-13A | CA_4A-13A | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_4A-4A-13A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_4A-17A | CA_4A-17A | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-27A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 27 |  | Yes | Yes | Yes |  |  |  |  |
| CA_4A-28A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-29A | - | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-29A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-30A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-46A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_4A-46A-46A | - | 4 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_4A-46A-46C | - | 4 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_4A-46C | - | 4 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-46D | - | 4 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_4A-46A-46D | - | 4 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 46 | See CA_46A-46D Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-7A | CA_5A-7A | 5 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  | 30 | 1 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_5A-7A-7A | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-7C | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-12A | CA_5A-12A | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-12A-12A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-12B | - | 5 |  |  | Yes | Yes |  |  | 25 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-13A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_5A-17A | CA_5A-17A | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-25A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5A-29A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-30A | - | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_5B-30A | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 30 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-38A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5A-40A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  | Yes | Yes | Yes |  |  | 30 | 1 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5A-40A-40A | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-40C | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 5 |  | Yes | Yes | Yes |  |  | 50 | 1 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-46A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_5A-46C | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | - | 5 |  |  | Yes | Yes |  |  | 50 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-46D | - | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-46E | - | 5 |  |  | Yes | Yes |  |  | 90 | 0 |
|  |  | 46 | See CA_46E of Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-66A | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5A-5A-66A | - | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5A-5A-66A-66A | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-5A-66B | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-5A-66C | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-66A-66A | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-66A-66C | - | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5A-66B | - | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-66C | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-66D |  | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5B-66A | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_5B-66A-66A |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_5B-66B |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5B-66C |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_7A-8A | - | 7 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes | 30 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_7A-7A-8A | - | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_7A-12A | - | 7 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_7A-20A | CA_7A-20A | 7 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes | 40 | 1 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-22A | - | 7 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 22 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-28A | CA_7A-28A | 7 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7B-28A | - | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7C-28A | CA_7A-28A CA_7C | 7 | See CA_7C bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-40A | - | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-40C | - | 7 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_7A-42A | - | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-42A-42A | - | 7 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_7A-46A | - | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_7A-46C | - | 7 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  | - | 7 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_7A-46D | - | 7 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_7A-46E | - | 7 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8A-11A | - | 8 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_8A-20A | - | 8 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  | 20 | 1 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| CA_8A-28A | - | 8 |  | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_8A-39A | - | 8 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 39 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_8A-39C | - | 8 | Yes | Yes | Yes | Yes |  |  | 45 | 0 |
|  |  | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8B-39A | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 39 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_8B-39C | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 55 | 0 |
|  |  | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8A-40A | - | 8 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
|  | - | 8 |  | Yes | Yes | Yes |  |  | 30 | 1 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_8A-41A | CA_8A-41A | 8 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  |
| CA_8A-41C | - | 8 | Yes | Yes | Yes | Yes |  |  | 50 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8A-41D | - | 8 | Yes | Yes | Yes | Yes |  |  | 70 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8B-41A | - | 8 | See CA_8B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_8B-41C | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8B-41D | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8A-42A | - | 8 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_8A-42C | - | 8 | Yes | Yes | Yes | Yes |  |  | 50 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8A-46A | - | 8 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_8A-46C | - | 8 | Yes | Yes | Yes | Yes |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_8B-46A | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_11A-18A | - | 11 |  |  | Yes | Yes |  |  | 25 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
| CA_11A-41A | - | 11 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_11A-41C | - | 11 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_11A-42A | - | 11 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_11A-42C | - | 11 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_11A-46C | - | 11 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_12A-25A | - | 12 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_12A-30A | - | 12 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_12A-66A | - | 12 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  | 30 | 1 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  | Yes | Yes | Yes |  |  | 30 | 2 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  | 20 | 3 |
|  |  | 66 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  | 30 | 4 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes |  |  |  | 20 | 5 |
|  |  | 66 |  |  | Yes | Yes | Yes |  |  |  |
| CA_12A-66A-66A | - | 12 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_12A-66C | - | 12 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_12B-66A | - | 12 | See CA_12B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  | 35 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_13A-46A | - | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_13A-46C | - | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_13A-66A | - | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_13A-66A-66A | - | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_13A-66A-66B | - | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_13A-66A-66C | - | 13 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_13A-66B | - | 13 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_13A-66C | - | 13 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_13A-66D | - | 13 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_18A-28A | CA_18A-28A | 18 |  |  | Yes | Yes | Yes |  | 25 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_19A-21A | CA_19A-21A | 19 |  |  | Yes | Yes | Yes |  | 30 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_19A-28A | - | 19 |  |  | Yes | Yes | Yes |  | 25 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_19A-42A | CA_19A-42A | 19 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_19A-42C | CA_19A-42A6 | 19 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_19A-46A | - | 19 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_19A-46C | - | 19 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_19A-46D | - | 19 |  |  | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_19A-46E | - | 19 |  |  | Yes | Yes | Yes |  | 95 | 0 |
|  |  | 46 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_20A-28A7 | - | 20 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_20A-31A | - | 20 |  |  | Yes | Yes | Yes | Yes | 25 | 0 |
|  |  | 31 |  | Yes | Yes |  |  |  |  |  |
| CA_20A-32A | - | 20 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_20A-38A | - | 20 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_20A-40A | - | 20 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |
| CA_20A-42A | - | 20 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_20A-42A-42A | - | 20 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_20A-67A | - | 20 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 67 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_21A-28A | - | 21 |  |  | Yes | Yes | Yes |  | 25 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_21A-42A | CA_21A-42A | 21 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_21A-42C | CA_21A-42A6 | 21 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_21A-46A | - | 21 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_21A-46C | - | 21 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 46 | See CA_46 Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_21A-46D | - | 21 |  |  | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_21A-46E | - | 21 |  |  | Yes | Yes | Yes |  | 95 | 0 |
|  |  | 46 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_23A-29A | - | 23 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 23 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
| CA_25A-26A | - | 25 |  | Yes | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | 25 |  | Yes | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 25 |  |  | Yes | Yes |  |  | 20 | 2 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_25A-41A | - | 25 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_25A-41C | CA_41C | 25 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_25A-41D | - | 25 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_26A-41A | - | 26 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_26A-41C | - | 26 |  |  | Yes | Yes | Yes |  | 55 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_26A-46A | - | 26 |  | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_28A-40A | - | 28 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_28A-40C | - | 28 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_28A-40D | - | 28 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_28A-41A | - | 28 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_28A-41C |  | 28 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_28A-42A | - | 28 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_28A-42C | - | 28 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_28A-46C | - | 28 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_29A-30A | - | 29 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_29A-66A | - | 29 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_29A-66C |  | 29 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_29A-66A-66A |  | 29 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_30A-66A-66A |  | 30 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_38A-40A | - | 38 |  |  |  | Yes |  | Yes | 40 | 0 |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  |
| CA_38A-40A-40A | - | 38 |  |  |  | Yes |  | Yes | 60 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_38A-40C | - | 38 |  |  |  | Yes |  | Yes | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_39A-41A | CA_39A-41A | 39 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_39A-41C | CA_41C CA_39A-41A CA_39A-41C | 39 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_39A-41D | CA_41C CA_39A-41A | 39 |  |  |  | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_39C-41A | CA_39C CA_39A-41A CA_39C-41A | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 55 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_39C-41C | CA_39C CA_41C CA_39A-41A | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 75 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| CA_39C-41D | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 95 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_39A-46C | - | 39 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_39C-46A | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 55 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_40A-41A | - | 40 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_40A-42C | - | 40 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_40A-46A | - | 40 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_40A-46C | - | 40 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_40A-46D | - | 40 |  |  |  | Yes | Yes | Yes | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_40C-42A | - | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_40C-46A | - | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_40C-46C | - | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_40D-46A | - | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_41A-42A | CA_41A-42A | 41 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_41A-42C | - | 41 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_41A-42D | - | 41 |  |  |  | Yes | Yes | Yes | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_41C-42A | - | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_41C-42C | - | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_41C-42D | - | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_41D-42A | - | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_41D-42C | - | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_41A-46A | - | 41 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_42A-46A | - | 42 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_46A-46A-66A | - | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46A-46C-66A | - | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46A-46D-66A | - | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46C-66A | - | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46A-66A | - | 46 |  |  |  |  |  | Yes | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_46D-66A | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 5: For TDD inter-band Carrier Aggregtion only non-simultaneous Rx/Tx uplink CA configurations can be supported by UE supporting corresponding DL CA configuration without simultaneous Rx/Tx. NOTE 6: If the UE supports uplink CA for corresponding downlink CA it shall support this uplink CA configuration. NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-2a: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (three bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A | CA_1A-3A CA_1A-5A6 CA_3A-5A | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 40 | 1 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-7A | CA_1A-3A CA_1A-7A CA_3A-7A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-3A-7A-7A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_1A-3A-7C | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-3A-8A | CA_1A-3A CA_1A-8A6 CA_3A-8A6 | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 40 | 1 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes |  | 40 | 2 |
|  |  | 3 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 3 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3C-8A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
| CA_1A-3A-11A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-19A | CA_1A-3A CA_1A-19A6 CA_3A-19A | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-26A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-3A-20A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-21A | - | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-3A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-38A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-40A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-40C | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-3C-40A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3C-40C | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-3A-41A9 | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-5A-40A | CA_1A-5A6 | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-5A-46A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_1A-5A-7A | CA_1A-5A6 CA_1A-7A CA_5A-7A6 | 1 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-5A-7A-7A | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_1A-5A-46C | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-5A-46D | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-7A-8A | CA_1A-7A, CA_1A-8A6 | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-7A-20A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-7A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-7C-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-7A-40A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-7A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-7A-46A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_1A-7A-46C | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-7A-46D | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-8A-11A | - | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-8A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-8A-40A | CA_1A-8A6 | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-11A-18A | - | 1 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 18 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-18A-28A | CA_1A-18A CA_1A-28A6 CA_18A-28A | 1 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 18 |  |  | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-19A-21A | CA_1A-19A6 CA_1A-21A CA_19A-21A6 | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-19A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-19A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-19A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-20A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-21A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-21A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-21A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-28A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-41A-42A10 | - | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-41A-42C10 | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_1A-41C-42A10 | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-41C-42C10 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-4A-5A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-4A-5A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 70 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-5B | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-4A-7A | . | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-4A-7A-7A | . | 2 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-4A-4A-5A | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-12A | CA_2A-4A CA_4A-12A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-12B | - | 2 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-2A-4A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 70 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-4A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-13A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-4A-29A | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-5A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-5A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-2A-5A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-2A-13A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 70 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-5A-12B | - | 2 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-5A-13A | CA_2A-13A6 | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-5A-29A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-5A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-5A-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-5B-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-5B-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 70 | 0 |
|  |  | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes |  |  | 30 |  |  |
| CA_2A-5A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-5A-66B |  | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-5A-66C |  | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-5B-66A |  | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-7A-12A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-7A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-12A-30A | CA_2A-12A6 | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-12A-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-12A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 40 | 1 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-13A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-13A-66A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_2A-13A-66B | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-13A-66C | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_2A-29A-30A | - | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2C-29A-30A | - | 2 | See CA_2C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-30A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-5A-7A | - | 3 |  |  |  | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3A-5A-7A-7A | - | 3 |  |  |  | Yes | Yes | Yes | 70 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
| CA_3A-5A-40A | CA_3A-5A | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes | Yes | Yes |  |  |
|  |  |  |  |  |  | 3 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  | 40 |  | 1 |  |
|  |  |  |  |  |  | 5 |  |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 40 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-7A-7A-8A | - | 3 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-7A-8A | - | 3 |  |  | Yes | Yes | Yes |  | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes |  |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 2 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-7A-20A | CA_3A-7A CA_3A-20A CA_7A-20A6 | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-7A-28A | CA_3A-7A CA_7A-28A | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-7C-28A | CA_3A-7A, CA_7C, CA_7A-28A | 3 |  |  |  | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3C-7A-28A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3C-7C-28A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3A-7A-38A7 | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-7A-40A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-7A-40C | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-7A-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-8A-11A | - | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-8A-28A | - | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-8A-40A | CA_3A-8A6 | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-19A-21A | - | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_3A-19A-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-19A-42C | - | 3 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-20A-32A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-20A-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-21A-28A | - | 3 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-21A-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-28A-40A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-28A-40C | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-28A-41A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-28A-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-41A-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3A-41A-42C | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_3A-41C-42A | - | 3 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3A-41C-42C | - | 3 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-5A-12A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-5A-12B | - | 4 |  |  | Yes | Yes | Yes | Yes | 45 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_4A-4A-5A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-5A-13A | CA_4A-13A6 | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_4A-5A-29A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-5A-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-5A-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-5B-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 70 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes |  |  | 30 |  |  |
| CA_4A-5B-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-7A-12A | - | 4 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 50 | 1 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-12A-30A | CA_4A-12A | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-12A-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-29A-30A | - | 4 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-4A-29A-30A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-7A-46A | - | 5 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |
| CA_5A-7A-46C | - | 5 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-7A-46D | - | 5 |  |  | Yes | Yes |  |  | 90 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_5A-30A-66A | - | 5 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-8A-20A | - | 7 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 8 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-12A-66A | - | 5 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  |  |  |  |  | 66 |  |  |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |
| CA_7A-20A-38A8 | - | 7 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 38 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-20A-42A | - | 7 |  |  |  | Yes | Yes | Yes | 60 | 0 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_8A-28A-41A | - | 8 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_12A-30A-66A | - | 12 |  |  | Yes | Yes |  |  | 40 | 0 |
|  |  | 30 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_19A-21A-42A | - | 19 |  |  | Yes | Yes | Yes |  | 50 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_19A-21A-42C | - | 19 |  |  | Yes | Yes | Yes |  | 70 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_21A-28A-42A | - | 21 |  |  | Yes | Yes | Yes |  | 45 | 0 |
|  |  | 28 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_28A-41A-42A |  | 28 |  |  | Yes | Yes |  |  | 50 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_28A-41A-42C | - | 28 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 41 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_28A-41C-42A | - | 28 |  |  | Yes | Yes |  |  | 70 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes | Yes | Yes |  |  |
| CA_28A-41C-42C | - | 28 |  |  | Yes | Yes |  |  | 90 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination. NOTE 5: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 6: If the UE supports uplink CA for corresponding downlink CA it shall support this uplink CA configuration. NOTE 7: UL carrier shall be supported in Band 3 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 8: UL carrier shall be supported in Band 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 9: UL carrier is only supported on Band 1 or Band 3 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 10: UL carrier is only supported on Band 1 or Band 42 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. |  |  |  |  |  |  |  |  |  |  |

# <Next change>

Table 6.2.5-2: ΔTIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1A-3C | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1C-3A | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1A-5A | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-7A | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-7A-7A | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-7C | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-8A | 1 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-11A | 1 | 0.3 |
|  | 11 | 0.3 |
| CA_1A-18A | 1 | 0.3 |
|  | 18 | 0.3 |
| CA_1A-19A | 1 | 0.3 |
|  | 19 | 0.3 |
| CA_1A-20A | 1 | 0.3 |
|  | 20 | 0.3 |
| CA_1A-21A | 1 | 0.3 |
|  | 21 | 0.3 |
| CA_1A-26A | 1 | 0.3 |
|  | 26 | 0.3 |
| CA_1A-28A | 1 | 0.3 |
|  | 28 | 0.6 |
| CA_1A-38A | 1 | 0.5 |
|  | 38 | 0.5 |
| CA_1A-40A | 1 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-40C | 1 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-41A8 | 1 | 0.5 |
|  | 41 | 0.5 |
| CA_1A-41C8 | 1 | 0.5 |
|  | 41 | 0.5 |
| CA_1A-42A | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-42C | 1 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-46A | 1 | 0 |
| CA_1A-46C | 1 | 0 |
| CA_1A-46D | 1 | 0 |
| CA_1A-46E | 1 | 0 |
| CA_2A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-2A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-4A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-2A-4A-4A | 2 | 0.5 |
|  | 4 | 0.5 |
| CA_2A-5A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-2A-5A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2C-5A | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-5B | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2C-5B | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2A-7A | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-7C | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-7A-7A | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-2A-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-12A-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-2A-12B | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-12B | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2C-12A | 2 | 0.3 |
|  | 12 | 0.3 |
| CA_2A-13A | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-2A-13A | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2A-17A | 2 | 0.3 |
|  | 17 | 0.8 |
| CA_2A-28A | 2 | 0.3 |
|  | 28 | 0.3 |
| CA_2A-29A | 2 | 0.3 |
| CA_2C-29A | 2 | 0.3 |
| CA_2A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-2A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2C-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-46A | 2 | 0 |
| CA_2A-46A-46C | 2 | 0 |
| CA_2A-46C | 2 | 0 |
| CA_2A-46D | 2 | 0 |
| CA_2A-46A-46A | 2 | 0 |
| CA_2A-46A-46D | 2 | 0 |
| CA_2A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66B | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66C | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66D | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66A-66A | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-66A-66C | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66B | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-2A-66C | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_3A-5A | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_3C-5A | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_3A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-3A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-3A-7A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-7A-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-7B | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-7C | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3C-7A | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3C-7C | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3A-8A | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3A-3A-8A | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3C-8A | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_3A-19A | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_3A-20A | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3C-20A | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3A-3A-20A | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_3A-21A | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_3A-26A | 3 | 0.3 |
|  | 26 | 0.3 |
| CA_3A-27A | 3 | 0.3 |
|  | 27 | 0.3 |
| CA_3A-28A | 3 | 0.3 |
|  | 28 | 0.3 |
| CA_3C-28A | 3 | 0.3 |
|  | 28 | 0.3 |
| CA_3A-31A | 3 | 0.3 |
|  | 31 | 0.6 |
| CA_3A-32A | 3 | 0.5 |
| CA_3A-38A | 3 | 0,5 |
|  | 38 | 0,5 |
| CA_3A-40A | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-40A-40A | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-40C | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3C-40A | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3C-40C | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_3A-41A | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3A-41C | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3A-41D | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3C-41A | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3C-41C | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3C-41D | 3 | 0.5 |
|  | 41 | 0.310 |
|  |  | 0.811 |
| CA_3A-42A | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-42C | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-46A | 3 | 0 |
| CA_3A-46C | 3 | 0 |
| CA_3A-46D | 3 | 0 |
| CA_3A-46E | 3 | 0 |
| CA_3A-69A | 3 | 0.5 |
| CA_4A-5A | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-4A-5A | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-5B | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-4A-5B | 4 | 0.3 |
|  | 5 | 0.3 |
| CA_4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-7A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-7C | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-12A | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-12A-12A | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-4A-12A | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-4A-12B | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-12B | 4 | 0.3 |
|  | 12 | 0.8 |
| CA_4A-13A | 4 | 0.3 |
|  | 13 | 0.3 |
| CA_4A-4A-13A | 4 | 0.3 |
|  | 13 | 0.3 |
| CA_4A-17A | 4 | 0.3 |
|  | 17 | 0.8 |
| CA_4A-27A | 4 | 0.3 |
|  | 27 | 0.3 |
| CA_4A-28A | 4 | 0.3 |
|  | 28 | 0.6 |
| CA_4A-29A | 4 | 0.3 |
| CA_4A-4A-29A | 4 | 0.3 |
| CA_4A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_4A-4A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_4A-46A | 4 | 0 |
| CA_4A-46A-46C | 4 | 0 |
| CA_4A-46C | 4 | 0 |
| CA_4A-46D | 4 | 0 |
| CA_4A-46A-46A | 4 | 0 |
| CA_4A-46A-46D | 4 | 0 |
| CA_5A-7A | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7A-7A | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7C | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-12A | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5A-12A-12A | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5A-12B | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5A-13A | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_5A-17A | 5 | 0.8 |
|  | 17 | 0.4 |
| CA_5A-25A | 5 | 0.3 |
|  | 25 | 0.3 |
| CA_5A-29A | 5 | 0.5 |
| CA_5A-30A | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_5B-30A | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_5A-38A | 5 | 0.3 |
|  | 38 | 0.3 |
| CA_5A-40A | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5A-40A-40A | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5A-40C | 5 | 0.3 |
|  | 40 | 0.3 |
| CA_5A-46A | 5 | 0 |
| CA_5A-46C | 5 | 0 |
| CA_5A-46D | 5 | 0 |
| CA_5A-46E | 5 | 0 |
| CA_5A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-5A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5A-66D | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66A-66A | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66B | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5B-66C | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_7A-8A | 7 | 0.3 |
|  | 8 | 0.6 |
| CA_7A-7A-8A | 7 | 0.3 |
|  | 8 | 0.6 |
| CA_7A-12A | 7 | 0.3 |
|  | 12 | 0.3 |
| CA_7A-20A | 7 | 0.3 |
|  | 20 | 0.3 |
| CA_7A-22A | 7 | 0.5 |
|  | 22 | 0.8 |
| CA_7A-28A | 7 | 0.3 |
|  | 28 | 0.3 |
| CA_7B-28A | 7 | 0.3 |
|  | 28 | 0.3 |
| CA_7C-28A | 7 | 0.3 |
|  | 28 | 0.3 |
| CA_7A-40A | 7 | 0.5 |
|  | 40 | [0.6] |
| CA_7A-40C | 7 | 0.5 |
|  | 40 | [0.6] |
| CA_7A-42A | 7 | 0.5 |
|  | 42 | 0.8 |
| CA_7A-42A-42A | 7 | 0.5 |
|  | 42 | 0.8 |
| CA_7A-46A | 7 | 0 |
| CA_7A-46C | 7 | 0 |
| CA_7A-46D | 7 | 0 |
| CA_7A-46E | 7 | 0 |
| CA_7A-66A | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_8A-11A | 8 | 0.3 |
|  | 11 | 0.4 |
| CA_8A-20A | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_8A-28A | 8 | 0.6 |
|  | 28 | 0.5 |
| CA_8A-39A | 8 | 0,3 |
|  | 39 | 0,3 |
| CA_8A-39C | 8 | 0.3 |
|  | 39 | 0.3 |
| CA_8B-39A | 8 | 0.3 |
|  | 39 | 0.3 |
| CA_8B-39C | 8 | 0.3 |
|  | 39 | 0.3 |
| CA_8A-40A | 8 | 0.3 |
|  | 40 | 0.3 |
| CA_8A-41A | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8A-41C | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8A-41D | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8B-41A | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8B-41C | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8B-41D | 8 | 0.3 |
|  | 41 | 0.3 |
| CA_8A-42A | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_8A-42C | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_8A-46A | 8 | 0 |
| CA_8A-46C | 8 | 0 |
| CA_8B-46A | 8 | 0 |
| CA_11A-18A | 11 | 0.3 |
|  | 18 | 0.3 |
| CA_11A-41A | 11 | 0.3 |
|  | 41 | 0.3 |
| CA_11A-41C | 11 | 0.3 |
|  | 41 | 0.3 |
| CA_11A-42A | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_11A-42C | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_11A-46A | 11 | 0 |
| CA_12A-25A | 12 | 0.3 |
|  | 25 | 0.3 |
| CA_12A-30A | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_12A-66A | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12A-66A-66A | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12A-66C | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_12B-66A | 12 | 0.8 |
|  | 66 | 0.3 |
| CA_13A-46A | 13 | 0 |
| CA_13A-46C | 13 | 0 |
| CA_13A-66A | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66A-66A | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66A-66C | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66A-66D | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66B | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66C | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13A-66D | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_18A-28A9 | 18 | 0.5 |
|  | 28 | 0.5 |
| CA_19A-21A | 19 | 0.3 |
|  | 21 | 0.4 |
| CA_19A-28A9 | 19 | 0.5 |
|  | 28 | 0.5 |
| CA_19A-42A | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_19A-42C | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_19A-46A | 19 | 0 |
| CA_19A-46C | 19 | 0 |
| CA_19A-46D | 19 | 0 |
| CA_19A-46E | 19 | 0 |
| CA_20A-28A | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_20A-31A | 20 | 0.5 |
|  | 31 | 0.5 |
| CA_20A-32A | 20 | 0.3 |
| CA_20A-38A | 20 | 0.3 |
|  | 38 | 0.3 |
| CA_20A-40A | 20 | 0.3 |
|  | 40 | 0.3 |
| CA_20A-42A | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_20A-42A-42A | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_20A-67A | 20 | 0.5 |
| CA_21A-28A | 21 | 0.4 |
|  | 28 | 0.3 |
| CA_21A-42A | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_21A-42C | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_21A-46A | 21 | 0 |
| CA_21A-46C | 21 | 0 |
| CA_21A-46D | 21 | 0 |
| CA_21A-46E | 21 | 0 |
| CA_23A-29A | 23 | 0.3 |
| CA_25A-26A | 25 | 0.3 |
|  | 26 | 0.3 |
| CA_25A-41A8 | 25 | 0.5 |
|  | 41 | 0.5 |
| CA_25A-41C8 | 25 | 0.5 |
|  | 41 | 0.5 |
| CA_25A-41D8 | 25 | 0.5 |
|  | 41 | 0.5 |
| CA_26A-41A | 26 | 0.3 |
|  | 41 | 0.3 |
| CA_26A-41C | 26 | 0.3 |
|  | 41 | 0.3 |
| CA_26A-46A | 26 | 0 |
| CA_28A-40A | 28 | 0.3 |
|  | 40 | 0.3 |
| CA_28A-40C | 28 | 0.3 |
|  | 40 | 0.3 |
| CA_28A-40D | 28 | 0.3 |
|  | 40 | 0.3 |
| CA_28A-41A | 28 | 0.3 |
|  | 41 | 0.3 |
| CA_28A-41C | 28 | 0.3 |
|  | 41 | 0.3 |
| CA_28A-42A | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_28A-42C | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_29A-30A | 30 | 0.3 |
| CA_29A-66A | 66 | 0.3 |
| CA_29A-66A-66A | 66 | 0.3 |
| CA_29A-66C | 66 | 0.3 |
| CA_30A-66A-66A | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_38A-40A | 38 | 04 |
|  | 40 | 04 |
| CA_38A-40A-40A | 38 | 04 |
|  | 40 | 04 |
| CA_38A-40C | 38 | 04 |
|  | 40 | 04 |
| CA_39A-41A | 39 | 04 |
|  | 41 | 04 |
| CA_39A-41A | 39 | 0.57 |
|  | 41 | 0.57 |
| CA_39A-41C | 39 | 04 |
|  | 41 | 04 |
| CA_39A-41C | 39 | 0.57 |
|  | 41 | 0.57 |
| CA_39A-41D | 39 | 04 |
|  | 41 | 04 |
| CA_39C-41A | 39 | 04 |
|  | 41 | 04 |
| CA_39C-41A | 39 | 0.57 |
|  | 41 | 0.57 |
| CA_39C-41C | 39 | 04 |
|  | 41 | 04 |
| CA_39C-41D | 39 | 04 |
|  | 41 | 04 |
| CA_39A-46C | 39 | 0 |
| CA_39C-46A | 39 | 0 |
| CA_40A-41A | 40 | 0.54 |
|  | 41 | 0.54 |
| CA_40A-42C | 40 | 04 |
|  | 42 | 0.54 |
| CA_40C-42A | 40 | 04 |
|  | 42 | 0.54 |
| CA_40A-46A | 40 | 0 |
| CA_40A-46C | 40 | 0 |
| CA_40A-46D | 40 | 0 |
| CA_40C-46A | 40 | 0 |
| CA_40C-46C | 40 | 0 |
| CA_40D-46A | 40 | 0 |
| CA_41A-42A | 41 | 04 |
|  | 42 | 0.54 |
| CA_41A-42C | 41 | 04 |
|  | 42 | 0.54 |
| CA_41A-42D | 41 | 04 |
|  | 42 | 0.54 |
| CA_41C-42A | 41 | 04 |
|  | 42 | 0.54 |
| CA_41C-42C | 41 | 04 |
|  | 42 | 0.54 |
| CA_41D-42A | 41 | 04 |
|  | 42 | 0.54 |
| CA_41C-42D | 41 | 04 |
|  | 42 | 0.54 |
| CA_41D-42C | 41 | 04 |
|  | 42 | 0.54 |
| CA_41A-46A | 41 | 0 |
| CA_42A-46A | 42 | [0.5] |
| CA_46A-46C-66A | 66 | 0 |
| CA_46A-46D-66A | 66 | 0 |
| CA_46A-66A | 66 | 0 |
| CA_46C-66A | 66 | 0 |
| CA_46D-66A | 66 | 0 |
| CA_46A-46A-66A | 66 | 0 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances above, truncated to one decimal place for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 2DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) |  |  |

NOTE:	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and another band is >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

NOTE:	To meet the TIB,c requirements for CA_3A-7A with state-of-the-art technology, an increase in power consumption of the UE may be required. It is also expected that as the state-of-the-art technology evolves in the future, this possible power consumption increase can be reduced or eliminated.

Table 6.2.5-3: ΔTIB,c (three bands)

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-5A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-3A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3A-7A-7A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3A-7C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1A-3A-8A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-3C-8A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_1A-3A-11A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
| CA_1A-3A-19A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_1A-3A-20A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_1A-3A-21A | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_1A-3A-26A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 26 | 0.3 |
| CA_1A-3A-28A | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
| CA_1A-3A-38A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 38 | 0.5 |
| CA_1A-3A-40A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3A-40C | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3C-40A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3C-40C | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1A-3A-41A | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.35/0.86 |
| CA_1A-3A-42A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-3A-42C | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-5A-7A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-5A-7A-7A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1A-5A-40A | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_1A-5A-46A | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-5A-46C | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-5A-46D | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1A-7A-8A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
| CA_1A-7A-20A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
| CA_1A-7A-28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-7C-28A | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-7A-40A | 1 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_1A-7A-42A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-7A-46A | 1 | 0.5 |
|  | 7 | 0.8 |
| CA_1A-7A-46C | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-7A-46D | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1A-8A-11A | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.4 |
| CA_1A-8A-28A11 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
| CA_1A-8A-40A | 1 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_1A-11A-18A | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
| CA_1A-18A-28A | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
| CA_1A-19A-21A | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
| CA_1A-19A-28A | 1 | 0.3 |
|  | 19 | 0.5 |
|  | 28 | 0.5 |
| CA_1A-19A-42A | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-19A-42C | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-20A-42A | 1 | 0.3 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_1A-21A-28A | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
| CA_1A-21A-42A | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1A-21A-42C | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1A-28A-42A | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1A-41A-42A8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1A-41A-42C8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1A-41C-42C8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1A-41C-42A8, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_2A-2A-4A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-5A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-2A-4A-5A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-4A-4A-5A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-4A-5B | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2A-4A-7A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-7A-7A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-12B | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-4A-12A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2A-4A-13A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 13 | 0.3 |
| CA_2A-4A-29A | 2 | [0.5] |
|  | 4 | 0.5 |
| CA_2A-4A-30A | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-5A-12A | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2A-2A-5A-12A | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2A-2A-5A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-2A-13A-66A | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-12B | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2A-5A-13A | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_2A-5A-29A | 2 | 0.3 |
|  | 5 | 0.5 |
| CA_2A-5A-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2C-5B-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2C-5A-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-5B-30A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-5A-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-66B | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5A-66C | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-5B-66A | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-7A-12A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-7A-66A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-12A-30A | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_2C-12A-30A | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_2A-12A-66A | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2A-13A-66A | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66A-66A | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66B | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-13A-66C | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2A-29A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2C-29A-30A | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2A-30A-66A | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_3A-5A-7A | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
| CA_3A-5A-7A-7A | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
| CA_3A-5A-40A | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-7A-7A-8A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3A-7A-8A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3A-7A-20A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.3 |
| CA_3A-7A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3A-7C-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3C-7A-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3C-7C-28A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3A-7A-38A | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 38 | 0.5 |
| CA_3A-7A-40A | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_3A-7A-40C | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_3A-7A-42A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 42 | 0.8 |
| CA_3A-8A-11A | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |
| CA_3A-8A-28A12 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.5 |
| CA_3A-8A-40A | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-19A-21A | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| CA_3A-19A-42A | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_3A-19A-42C | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_3A-20A-32A | 3 | 0.5 |
|  | 20 | 0.3 |
| CA_3A-20A-42A | 3 | 0.6 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_3A-21A-28A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 28 | 0.3 |
| CA_3A-21A-42A | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_3A-28A-40A | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-28A-40C | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
| CA_3A-28A-41A | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 41 | 0.35/0.86 |
| CA_3A-28A-42A | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_3A-41A-42A14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_3A-41A-42C14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_3A-41C-42A14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_3A-41C-42C14 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_4A-5A-12A | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4A-5A-12B | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4A-4A-5A-12A | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4A-5A-13A | 4 | 0.3 |
|  | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_4A-5A-29A | 4 | 0.3 |
|  | 5 | 0.5 |
| CA_4A-5A-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-4A-5A-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-5B-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-4A-5B-30A | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4A-7A-12A | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
| CA_4A-12A-30A | 4 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
| CA_4A-4A-12A-30A | 4 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
| CA_4A-29A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_4A-4A-29A-30A | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_5A-7A-46A | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7A-46C | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-7A-46D | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5A-12A-66A | 5 | 0.3 |
|  | 12 | 0.8 |
|  | 66 | 0.8 |
| CA_5A-30A-66A | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_7A-8A-20A | 7 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | [0.6] |
| CA_7A-20A-38A | 7 | 0.3 |
|  | 20 | 0.3 |
|  | 38 | 0.3 |
| CA_7A-20A-42A | 7 | 0.3 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_8A-28A-41A15 | 8 | 0.6 |
|  | 28 | 0.5 |
|  | 41 | 0.3 |
| CA_12A-30A-66A | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_19A-21A-42A | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_19A-21A-42C | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_21A-28A-42A | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_28A-41A-42A10 | 28 | 0.5 |
|  | 41 | 0.31 |
|  | 42 | 0.81 |
| CA_28A-41C-42C | 28 | 0.5 |
|  | 41 | 0.39 |
|  | 42 | 0.89 |
| CA_28A-41A-42C | 28 | 0.5 |
|  | 41 | 0.39 |
|  | 42 | 0.89 |
| CA_28A-41C-42A | 28 | 0.5 |
|  | 41 | 0.39 |
|  | 42 | 0.89 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. NOTE 7: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 42. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx on Band 41 and Band 42. NOTE 10: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8 NOTE 12: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3 or Band 8. NOTE 13: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 14: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 15: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41. |  |  |

# <Next change>

Table 7.3.1-1A: ΔRIB,c (two bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A | 1 | 0 |
|  | 3 | 0 |
| CA_1A-3C | 1 | 0 |
|  | 3 | 0 |
| CA_1C-3A | 1 | 0 |
|  | 3 | 0 |
| CA_1A-5A | 1 | 0 |
|  | 5 | 0 |
| CA_1A-7A | 1 | 0 |
|  | 7 | 0 |
| CA_1A-7A-7A | 1 | 0 |
|  | 7 | 0 |
| CA_1A-7C | 1 | 0 |
|  | 7 | 0 |
| CA_1A-8A | 1 | 0 |
|  | 8 | 0 |
| CA_1A-11A | 1 | 0 |
|  | 11 | 0 |
| CA_1A-18A | 1 | 0 |
|  | 18 | 0 |
| CA_1A-19A | 1 | 0 |
|  | 19 | 0 |
| CA_1A-20A | 1 | 0 |
|  | 20 | 0 |
| CA_1A-21A | 1 | 0 |
|  | 21 | 0 |
| CA_1A-26A | 1 | 0 |
|  | 26 | 0 |
| CA_1A-28A | 1 | 0 |
|  | 28 | 0.2 |
| CA_1A-38A | 1 | 0 |
|  | 38 | 0 |
| CA_1A-40A | 1 | 0 |
|  | 40 | 0 |
| CA_1A-40C | 1 | 0 |
|  | 40 | 0 |
| CA_1A-41A8 | 1 | 0 |
|  | 41 | 0 |
| CA_1A-41C8 | 1 | 0 |
|  | 41 | 0 |
| CA_1A-42A | 1 | 0 |
|  | 42 | 0.5 |
| CA_1A-42C | 1 | 0 |
|  | 42 | 0.5 |
| CA_2A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-2A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-4A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-2A-4A-4A | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2A-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2A-2A-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2A-5B | 2 | 0 |
|  | 5 | 0 |
| CA_2C-5A | 2 | 0 |
|  | 5 | 0 |
| CA_2C-5B | 2 | 0 |
|  | 5 | 0 |
| CA_2A-7A | 2 | 0 |
|  | 7 | 0 |
| CA_2A-7C | 2 | 0 |
|  | 7 | 0 |
| CA_2A-7A-7A | 2 | 0 |
|  | 7 | 0 |
| CA_2A-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-12A-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-2A-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-2A-12B | 2 | 0 |
|  | 12 | 0 |
| CA_2A-12B | 2 | 0 |
|  | 12 | 0 |
| CA_2C-12A | 2 | 0 |
|  | 12 | 0 |
| CA_2A-13A | 2 | 0 |
|  | 13 | 0 |
| CA_2A-2A-13A | 2 | 0 |
|  | 13 | 0 |
| CA_2A-17A | 2 | 0 |
|  | 17 | 0.5 |
| CA_2A-28A | 2 | 0 |
|  | 28 | 0 |
| CA_2A-30A | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_2A-2A-30A | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_2C-30A | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_2A-66A | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-66B | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-66C | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-66D | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-2A-66A | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-2A-66A-66A | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-66A-66A | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-66A-66C | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-2A-66B | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2A-2A-66C | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_3A-5A | 3 | 0 |
|  | 5 | 0 |
| CA_3C-5A | 3 | 0 |
|  | 5 | 0 |
| CA_3A-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3A-3A-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3A-3A-7A-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3A-7A-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3A-7B | 3 | 0 |
|  | 7 | 0 |
| CA_3A-7C | 3 | 0 |
|  | 7 | 0 |
| CA_3C-7A | 3 | 0 |
|  | 7 | 0 |
| CA_3C-7C | 3 | 0 |
|  | 7 | 0 |
| CA_3A-8A | 3 | 0 |
|  | 8 | 0 |
| CA_3A-3A-8A | 3 | 0 |
|  | 8 | 0 |
| CA_3C-8A | 3 | 0 |
|  | 8 | 0 |
| CA_3A-19A | 3 | 0 |
|  | 19 | 0 |
| CA_3A-20A | 3 | 0 |
|  | 20 | 0 |
| CA_3C-20A | 3 | 0 |
|  | 20 | 0 |
| CA_3A-3A-20A | 3 | 0 |
|  | 20 | 0 |
| CA_3A-21A | 3 | 0.3 |
|  | 21 | 0.5 |
| CA_3A-26A | 3 | 0 |
|  | 26 | 0 |
| CA_3A-27A | 3 | 0 |
|  | 27 | 0 |
| CA_3A-28A | 3 | 0 |
|  | 28 | 0 |
| CA_3C-28A | 3 | 0 |
|  | 28 | 0 |
| CA_3A-31A | 3 | 0 |
|  | 31 | 0.2 |
| CA_3A-32A | 3 | 0 |
|  | 32 | 0 |
| CA_3A-38A | 3 | 0 |
|  | 38 | 0 |
| CA_3A-40A | 3 | 0 |
|  | 40 | 0 |
| CA_3A-40A-40A | 3 | 0 |
|  | 40 | 0 |
| CA_3A-40C | 3 | 0 |
|  | 40 | 0 |
| CA_3C-40A | 3 | 0 |
|  | 40 | 0 |
| CA_3C-40C | 3 | 0 |
|  | 40 | 0 |
| CA_3A-41A | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3A-41C | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3A-41D | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3C-41A | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3C-41C | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3C-41D | 3 | 0 |
|  | 41 | 010 |
|  |  | 0.511 |
| CA_3A-42A | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_3A-42C | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_4A-5A | 4 | 0 |
|  | 5 | 0 |
| CA_4A-4A-5A | 4 | 0 |
|  | 5 | 0 |
| CA_4A-5B | 4 | 0 |
|  | 5 | 0 |
| CA_4A-4A-5B | 4 | 0 |
|  | 5 | 0 |
| CA_4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-4A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-7A-7A | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-7C | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_4A-12A | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-12B | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-4A-12A | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-12A-12A | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-4A-12B | 4 | 0 |
|  | 12 | 0.5 |
| CA_4A-13A | 4 | 0 |
|  | 13 | 0 |
| CA_4A-4A-13A | 4 | 0 |
|  | 13 | 0 |
| CA_4A-17A | 4 | 0 |
|  | 17 | 0.5 |
| CA_4A-27A | 4 | 0 |
|  | 27 | 0 |
| CA_4A-28A | 4 | 0 |
|  | 28 | 0.2 |
| CA_4A-30A | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_4A-4A-30A | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_5A-7A | 5 | 0 |
|  | 7 | 0 |
| CA_5A-7A-7A | 5 | 0 |
|  | 7 | 0 |
| CA_5A-7C | 5 | 0 |
|  | 7 | 0 |
| CA_5A-12A | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5A-12A-12A | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5A-12B | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5A-13A | 5 | 0 |
|  | 13 | 0 |
| CA_5A-17A | 5 | 0.5 |
|  | 17 | 0.3 |
| CA_5A-25A | 5 | 0 |
|  | 25 | 0 |
| CA_5A-30A | 5 | 0 |
|  | 30 | 0 |
| CA_5B-30A | 5 | 0 |
|  | 30 | 0 |
| CA_5A-38A | 5 | 0 |
|  | 38 | 0 |
| CA_5A-40A | 5 | 0 |
|  | 40 | 0 |
| CA_5A-40A-40A | 5 | 0 |
|  | 40 | 0 |
| CA_5A-40C | 5 | 0 |
|  | 40 | 0 |
| CA_5A-66A | 5 | 0 |
|  | 66 | 0 |
| CA_5A-5A-66A | 5 | 0 |
|  | 66 | 0 |
| CA_5A-5A-66A-66A | 5 | 0 |
|  | 66 | 0 |
| CA_5A-5A-66B | 5 | 0 |
|  | 66 | 0 |
| CA_5A-5A-66C | 5 | 0 |
|  | 66 | 0 |
| CA_5A-66A-66A | 5 | 0 |
|  | 66 | 0 |
| CA_5A-66A-66C | 5 | 0 |
|  | 66 | 0 |
| CA_5A-66B | 5 | 0 |
|  | 66 | 0 |
| CA_5A-66C | 5 | 0 |
|  | 66 | 0 |
| CA_5A-66D | 5 | 0 |
|  | 66 | 0 |
| CA_5B-66A | 5 | 0 |
|  | 66 | 0 |
| CA_5B-66A-66A | 5 | 0 |
|  | 66 | 0 |
| CA_5B-66B | 5 | 0 |
|  | 66 | 0 |
| CA_5B-66C | 5 | 0 |
|  | 66 | 0 |
| CA_7A-8A | 7 | 0 |
|  | 8 | 0.2 |
| CA_7A-7A-8A | 7 | 0 |
|  | 8 | 0.2 |
| CA_7A-12A | 7 | 0 |
|  | 12 | 0 |
| CA_7A-20A | 7 | 0 |
|  | 20 | 0 |
| CA_7A-22A | 7 | 0 |
|  | 22 | 0.5 |
| CA_7A-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7B-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7C-28A | 7 | 0 |
|  | 28 | 0 |
| CA_7A-40A | 7 | 0 |
|  | 40 | 0.5 |
| CA_7A-40C | 7 | 0 |
|  | 40 | 0.5 |
| CA_7A-42A | 7 | 0 |
|  | 42 | 0.5 |
| CA_7A-42A-42A | 7 | 0 |
|  | 42 | 0.5 |
| CA_7A-46C | 7 | 0 |
| CA_7A-66A | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_8A-11A | 8 | 0 |
|  | 11 | 0 |
| CA_8A-20A | 8 | 0 |
|  | 20 | 0 |
| CA_8A-28A | 8 | 0.2 |
|  | 28 | 0.1 |
| CA_8A-39A | 8 | 0 |
|  | 39 | 0 |
| CA_8A-39C | 8 | 0 |
|  | 39 | 0 |
| CA_8B-39A | 8 | 0 |
|  | 39 | 0 |
| CA_8B-39C | 8 | 0 |
|  | 39 | 0 |
| CA_8A-40A | 8 | 0 |
|  | 40 | 0 |
| CA_8A-41A | 8 | 0 |
|  | 41 | 0 |
| CA_8A-41C | 8 | 0 |
|  | 41 | 0 |
| CA_8A-41D | 8 | 0 |
|  | 41 | 0 |
| CA_8B-41A | 8 | 0 |
|  | 41 | 0 |
| CA_8B-41C | 8 | 0 |
|  | 41 | 0 |
| CA_8B-41D | 8 | 0 |
|  | 41 | 0 |
| CA_8A-42A | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_8A-42C | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_8A-46A | 8 | 0 |
| CA_8A-46C | 8 | 0 |
| CA_8B-46A | 8 | 0 |
| CA_11A-18A | 11 | 0 |
|  | 18 | 0 |
| CA_11A-41A | 11 | 0 |
|  | 41 | 0 |
| CA_11A-41C | 11 | 0 |
|  | 41 | 0 |
| CA_11A-42A | 11 | 0 |
|  | 42 | 0.5 |
| CA_11A-42C | 11 | 0 |
|  | 42 | 0.5 |
| CA_11A-46C | 11 | 0 |
| CA_12A-25A | 12 | 0 |
|  | 25 | 0 |
| CA_12A-30A | 12 | 0 |
|  | 30 | 0 |
| CA_12A-66A | 12 | 0.5 |
|  | 66 | 0 |
| CA_12A-66A-66A | 12 | 0.5 |
|  | 66 | 0 |
| CA_12A-66C | 12 | 0.5 |
|  | 66 | 0 |
| CA_12B-66A | 12 | 0.5 |
|  | 66 | 0 |
| CA_13A-46C | 13 | 0 |
| CA_13A-66A | 13 | 0 |
|  | 66 | 0 |
| CA_13A-66A-66A | 13 | 0 |
|  | 66 | 0 |
| CA_13A-66A-66B | 13 | 0 |
|  | 66 | 0 |
| CA_13A-66A-66C | 13 | 0 |
|  | 66 | 0 |
| CA_13A-66B | 13 | 0 |
|  | 66 | 0 |
| CA_13A-66C | 13 | 0 |
|  | 66 | 0 |
| CA_13A-66D | 13 | 0 |
|  | 66 | 0 |
| CA_18A-28A9 | 18 | 0 |
|  | 28 | 0 |
| CA_19A-21A | 19 | 0 |
|  | 21 | 0 |
| CA_19A-28A9 | 19 | 0 |
|  | 28 | 0 |
| CA_19A-42A | 19 | 0 |
|  | 42 | 0.5 |
| CA_19A-42C | 19 | 0 |
|  | 42 | 0.5 |
| CA_19A-46A | 19 | 0 |
| CA_19A-46C | 19 | 0 |
| CA_19A-46D | 19 | 0 |
| CA_20A-28A | 20 | 0 |
|  | 28 | 0 |
| CA_20A-31A | 20 | 0 |
|  | 31 | 0 |
| CA_20A-38A | 20 | 0 |
|  | 38 | 0 |
| CA_20A-40A | 20 | 0 |
|  | 40 | 0 |
| CA_20A-42A | 20 | 0 |
|  | 42 | 0.5 |
| CA_20A-42A-42A | 20 | 0 |
|  | 42 | 0.5 |
| CA_21A-28A | 21 | 0 |
|  | 28 | 0 |
| CA_21A-42A | 21 | 0 |
|  | 42 | 0.5 |
| CA_21A-42C | 21 | 0 |
|  | 42 | 0.5 |
| CA_21A-46A | 21 | 0 |
| CA_21A-46C | 21 | 0 |
| CA_21A-46D | 21 | 0 |
| CA_25A-26A | 25 | 0 |
|  | 26 | 0 |
| CA_25A-41A8 | 25 | 0 |
|  | 41 | 0 |
| CA_25A-41C8 | 25 | 0 |
|  | 41 | 0 |
| CA_25A-41D8 | 25 | 0 |
|  | 41 | 0 |
| CA_26A-41A | 26 | 0 |
|  | 41 | 0 |
| CA_26A-41C | 26 | 0 |
|  | 41 | 0 |
| CA_28A-40A | 28 | 0 |
|  | 40 | 0 |
| CA_28A-40C | 28 | 0 |
|  | 40 | 0 |
| CA_28A-40D | 28 | 0 |
|  | 40 | 0 |
| CA_28A-41A | 28 | 0 |
|  | 41 | 0 |
| CA_28A-41C | 28 | 0 |
|  | 41 | 0 |
| CA_28A-42A | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_28A-42C | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_29A-66A | 66 | 0 |
| CA_29A-66A-66A | 29 | 0 |
|  | 66 | 0 |
| CA_29A-66C | 29 | 0 |
|  | 66 | 0 |
| CA_30A-66A-66A | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_38A-40A | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_38A-40A-40A | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_38A-40C | 38 | 0.54 |
|  | 40 | 0.54 |
| CA_39A-41A | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39A-41A | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39A-41C | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39A-41C | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39A-41D | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39C-41A | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39C-41A | 39 | 0.27 |
|  | 41 | 0.27 |
| CA_39C-41C | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_39C-41D | 39 | 0.24 |
|  | 41 | 0.24 |
| CA_40A-41A | 40 | 04 |
|  | 41 | 04 |
| CA_40A-42C | 40 | 0.44 |
|  | 42 | 0.54 |
| CA_40C-42A | 40 | 0.44 |
|  | 42 | 0.54 |
| CA_40A-46A | 40 | 0 |
| CA_40A-46C | 40 | 0 |
| CA_40A-46D | 40 | 0 |
| CA_40C-46A | 40 | 0 |
| CA_40C-46C | 40 | 0 |
| CA_40D-46A | 40 | 0 |
| CA_41A-42A | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41A-42C | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41A-42D | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41C-42A | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41C-42C | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41D-42A | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41C-42D | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_41D-42C | 41 | 0.44 |
|  | 42 | 0.54 |
| CA_42A-46A | 42 | [0] |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances in Table 7.3.1-1A, truncated to one decimal place that would apply for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum 2DL tolerance in Table 7.3.1-1A that would apply for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

NOTE:	To meet the RIB,c requirements for CA_20A-28A state-of-the-art filter combiner technology is needed.

Table 7.3.1-1B: ΔRIB,c (three bands)

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-3A-5A | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
| CA_1A-3A-7A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
| CA_1A-3A-7A-7A | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
| CA_1A-3A-7C | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
| CA_1A-3A-8A | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
| CA_1A-3C-8A | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
| CA_1A-3A-11A | 1 | 0 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
| CA_1A-3A-19A | 1 | 0 |
|  | 3 | 0 |
|  | 19 | 0 |
| CA_1A-3A-20A | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
| CA_1A-3A-21A | 1 | 0 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
| CA_1A-3A-26A | 1 | 0 |
|  | 3 | 0 |
|  | 26 | 0 |
| CA_1A-3A-28A | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
| CA_1A-3A-38A | 1 | 0 |
|  | 3 | 0 |
|  | 38 | 0 |
| CA_1A-3A-40A | 1 | 0 |
|  | 3 | 0 |
|  | 40 | 0 |
| CA_1A-3A-40C | 1 | 0 |
|  | 3 | 0 |
|  | 40 | 0 |
| CA_1A-3C-40A | 1 | 0 |
|  | 3 | 0 |
|  | 40 | 0 |
| CA_1A-3C-40C | 1 | 0 |
|  | 3 | 0 |
|  | 40 | 0 |
| CA_1A-3A-41A | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 05/0.56 |
| CA_1A-3A-42A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-3A-42C | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-5A-7A | 1 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_1A-5A-7A-7A | 1 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_1A-5A-40A | 1 | 0 |
|  | 5 | 0 |
|  | 40 | 0 |
| CA_1A-5A-46A | 1 | 0 |
|  | 5 | 0 |
| CA_1A-5A-46C | 1 | 0 |
|  | 5 | 0 |
| CA_1A-7A-8A | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_1A-7A-20A | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
| CA_1A-7A-28A | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1A-7C-28A | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1A-7A-40A | 1 | 0 |
|  | 7 | 0.3 |
|  | 40 | 0.8 |
| CA_1A-7A-42A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-7A-46A | 1 | 0 |
|  | 7 | 0 |
| CA_1A-7A-46C | 1 | 0 |
|  | 7 | 0 |
| CA_1A-8A-11A | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0 |
| CA_1A-8A-28A10 | 1 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |
| CA_1A-8A-40A | 1 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
| CA_1A-11A-18A | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
| CA_1A-18A-28A | 1 | 0 |
|  | 18 | 0 |
|  | 28 | 0 |
| CA_1A-19A-21A | 1 | 0 |
|  | 19 | 0 |
|  | 21 | 0 |
| CA_1A-19A-28A | 1 | 0 |
|  | 19 | 0 |
|  | 28 | 0 |
| CA_1A-19A-42A | 1 | 0 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_1A-19A-42C | 1 | 0 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_1A-20A-42A | 1 | 0 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_1A-21A-28A | 1 | 0 |
|  | 21 | 0 |
|  | 28 | 0.2 |
| CA_1A-21A-42A | 1 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
| CA_1A-28A-42A | 1 | 0 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_1A-21A-42C | 1 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
| CA_1A-41A-42A7, 12 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
| CA_1A-41A-42C7, 12 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
| CA_1A-41C-42A7, 12 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
| CA_1A-41C-42C7, 12 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
| CA_2A-2A-4A-12A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2A-4A-5A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2A-2A-4A-5A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2A-4A-4A-5A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2A-4A-5B | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2A-4A-7A | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-7A-7A | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2A-4A-12A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2A-4A-12B | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2A-4A-4A-12A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2A-4A-13A | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 13 | 0 |
| CA_2A-4A-30A | 2 | 0.4 |
|  | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_2A-5A-12A | 2 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-2A-5A-12A | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-2A-5A-66A | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
| CA_2A-2A-13A-66A | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
| CA_2A-5A-12B | 2 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_2A-5A-13A | 2 | 0 |
|  | 5 | 0 |
|  | 13 | 0 |
| CA_2A-5A-30A | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2C-5A-30A | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2A-5B-30A | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2A-5A-66A | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
| CA_2A-5A-66B | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
| CA_2A-5A-66C | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
| CA_2A-5B-66A | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
| CA_2C-5B-30A | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2A-7A-12A | 2 | 0 |
|  | 7 | 0 |
|  | 12 | 0 |
| CA_2A-7A-66A | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_2A-12A-30A | 2 | 0.4 |
|  | 12 | 0 |
|  | 30 | 0.5 |
| CA_2C-12A-30A | 2 | 0.4 |
|  | 12 | 0 |
|  | 30 | 0.5 |
| CA_2A-12A-66A | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
| CA_2A-13A-66A | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
| CA_2A-13A-66A-66A | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
| CA_2A-13A-66B | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
| CA_2A-13A-66C | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
| CA_2A-30A-66A | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_3A-5A-7A | 3 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_3A-5A-7A-7A | 3 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_3A-5A-40A | 3 | 0 |
|  | 5 | 0 |
|  | 40 | 0 |
| CA_3A-7A-7A-8A | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_3A-7A-8A | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_3A-7A-20A | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
| CA_3A-7A-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3A-7C-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3C-7A-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3C-7C-28A | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3A-7A-38A | 3 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
| CA_3A-7A-40A | 3 | 0 |
|  | 7 | 0.3 |
|  | 40 | 0.8 |
| CA_3A-7A-40C | 3 | 0 |
|  | 7 | 0.3 |
|  | 40 | 0.8 |
| CA_3A-7A-42A | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 42 | 0.5 |
| CA_3A-8A-11A | 3 | 0.3 |
|  | 8 | 0 |
|  | 11 | 0.5 |
| CA_3A-8A-28A11 | 3 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.1 |
| CA_3A-8A-40A | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
| CA_3A-19A-21A | 3 | 0.3 |
|  | 19 | 0 |
|  | 21 | 0.5 |
| CA_3A-19A-42A | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_3A-19A-42C | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_3A-20A-32A | 3 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_3A-20A-42A | 3 | 0.2 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_3A-21A-28A | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 28 | 0 |
| CA_3A-21A-42A | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| CA_3A-28A-40A | 3 | 0 |
|  | 28 | 0 |
|  | 40 | 0 |
| CA_3A-28A-40C | 3 | 0 |
|  | 28 | 0 |
|  | 40 | 0 |
| CA_3A-28A-41A | 3 | 0 |
|  | 28 | 0 |
|  | 41 | 05/0.56 |
| CA_3A-28A-42A | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_3A-41A-42A13 | 3 | 0.5 |
|  | 41 | 05/0.56 |
|  | 42 | 0.5 |
| CA_3A-41A-42C13 | 3 | 0.5 |
|  | 41 | 05/0.56 |
|  | 42 | 0.5 |
| CA_3A-41C-42A13 | 3 | 0.5 |
|  | 41 | 05/0.56 |
|  | 42 | 0.5 |
| CA_3A-41C-42C13 | 3 | 0.5 |
|  | 41 | 05/0.56 |
|  | 42 | 0.5 |
| CA_4A-5A-12A | 4 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
| CA_4A-5A-12B | 4 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
| CA_4A-4A-5A-12A | 4 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
| CA_4A-5A-13A | 4 | 0 |
|  | 5 | 0 |
|  | 13 | 0 |
| CA_4A-5A-30A | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_4A-4A-5A-30A | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_4A-4A-5B-30A | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_4A-5B-30A | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_4A-7A-12A | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.5 |
| CA_4A-12A-30A | 4 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
| CA_4A-4A-12A-30A | 4 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
| CA_5A-7A-46A | 5 | 0 |
|  | 7 | 0 |
| CA_5A-7A-46C | 5 | 0 |
|  | 7 | 0 |
| CA_5A-12A-66A | 5 | 0 |
|  | 12 | 0.5 |
|  | 66 | 0.5 |
| CA_5A-30A-66A | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_7A-8A-20A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | [0.2] |
| CA_7A-20A-38A | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |
| CA_7A-20A-42A | 7 | 0 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_8A-28A-41A14 | 7 | 0 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_12A-30A-66A | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_19A-21A-42A | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
| CA_19A-21A-42C | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
| CA_21A-28A-42A | 21 | 0 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_28A-41A-42A9 | 28 | 0.2 |
|  | 41 | 0.41 |
|  | 42 | 0.51 |
| CA_28A-41A-42C | 28 | 0.2 |
|  | 41 | 0.48 |
|  | 42 | 0.58 |
| CA_28A-41C-42A | 28 | 0.2 |
|  | 41 | 0.48 |
|  | 42 | 0.58 |
| CA_28A-41C-42C | 28 | 0.2 |
|  | 41 | 0.48 |
|  | 42 | 0.58 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. NOTE 7: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 42. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx on Band 41 and Band 42. NOTE 9: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8. NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3 or Band 8. NOTE 12: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 13: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41. |  |  |

# <Next change>

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) |  | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-8A4,5,6 | 1 |  |  | -100 | -97 | -95.2 |  | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 |  | -96.720 |  |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_1A-3A-7A-28A5,6 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 3 |  |  |  | -94 | -92.2 |  | -91 |  |
|  |  |  |  |  | [-96.7]20 | -[94.9]20 |  | -[93.7]20 |  |
|  | 7 |  |  |  | -95 | -93.2 |  | -92 |  |
|  | 28 |  |  |  | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-3C-8A4 | 1 |  |  | -100 | -97 | -95.2 |  | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 |  | -96.720 |  |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
| CA_1A-3A-8A-40A4 | 1 |  |  | -100 | -97 | -95.2 |  | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 |  | -96.720 |  |
|  | 3 |  |  | N/A | N/A | N/A |  | N/A |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_1A-3A-19A-42A9,10 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-19A-42A11 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-3A-19A-42C9,10 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-19A-42C11 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-3A-28A | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 |  | -91 |  |
|  |  |  |  | -99.720 | -96.720 | -94.920 |  | -93.720 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-3A-42A9,10 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_1A-3A-42A11 | 1 |  |  | -99.8 | -96.8 | -95 |  | -93.8 | FDD |
|  |  |  |  | -102.520 | -99.520 | -97.720 |  | -96.520 |  |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 |  |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_1A-7A-8A5,6 | 1 |  |  | -100 | -97 | -95.2 |  | -94 | FDD |
|  |  |  |  | -102.720 | -99.720 | -97.920 |  | -96.720 |  |
|  | 7 |  |  |  | -87.4 | -87 |  | -86.7 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_1A-7A-28A5,6 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 7 |  |  |  | -95 | -93.2 |  | -92 |  |
|  |  |  |  |  | -97.720 | -95.920 |  | -94.720 |  |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-19A-28A14 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 19 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_1A-21A-28A4,21 | 1 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 21 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_1A-28A5,6,14 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_1A-28A-42A5,6,17,18 | 1 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |
|  | 42 |  |  | -85.7 | -85.4 | -85.1 |  | -84.9 | TDD |
| CA_2A-2A-4A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  |  |  |  | -100.420 | -97.420 | -95.620 |  | -94.420 |  |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-4A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-12B5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-5A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 5 |  |  | -97.5 | -94.5 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-7A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 7 |  |  | -97.5 | -94.5 | -92.7 |  | -91.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_2A-4A-12A-30A5,6 | 2 |  |  | -97.6 | -94.6 | -92.8 |  | -91.6 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |  |
| CA_2A-12A-66A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 |  | -91.7 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_3A-7A-8A4,5,6 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_3A-7A-7A-8A4,5,6 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 |  |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_3A-7A-20A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-7A-20A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-7A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-7A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 7 |  |  | -97.8 | -94.8 | -93 |  | -91.8 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-8A4 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 8 |  | N/A | N/A | N/A |  |  |  |  |
| CA_3C-8A4 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 8 |  | N/A | N/A | N/A |  |  |  |  |
| CA_3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 8 |  |  | N/A | N/A |  |  |  |  |
|  | 11 |  |  | N/A | N/A |  |  |  |  |
| CA_3A-19A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-19A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 19 |  |  | -100 | -97 | -95.2 |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-20A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-20A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | [-99.5]20 | [-96.5]20 | [-94.7]20 |  | [-93.5]20 |  |
|  | 20 |  |  | -97 | -94 | -91.2 |  | -90 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-21A-28A4,21 | 3 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 21 |  |  | N/A | N/A | N/A |  |  |  |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_3A-21A-42A1,2 | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 | FDD |
|  |  |  |  | [-99.4]4 | [-96.4]4 | [-94.6]4 |  | [-93.4]4 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-21A-42A3 | 3 |  |  | -96.7 | -93.7 | -91.9 |  | -90.7 | FDD |
|  |  |  |  | [-99.4]4 | [-96.4]4 | [-94.6]4 |  | [-93.4]4 |  |
|  | 21 |  |  | -99.5 | -96.5 | -94.7 |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_3A-28A-40A15,16 | 3 |  |  | -97 | -94 | -92.2 |  | -91 | FDD |
|  |  |  |  | -99.720 | -96.720 | -94.920 |  | -93.720 |  |
|  | 28 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 |  |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_3A-28A-40C15,16 | 3 |  |  | -97 | -94 | -92.2 |  | -91 | FDD |
|  |  |  |  | -99.720 | -96.720 | -94.920 |  | -93.720 |  |
|  | 28 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 |  |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_3A-28A-42A9,10 | 3 |  |  | -96.8 | -93.8 |  | -92 | -90.8 | FDD |  |
|  |  |  |  | [-99.5]6 | [-96.5]6 |  | [-94.7]6 | [-93.5]6 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 42 |  |  | -71.7 | -71.7 |  | -71.7 | -71.7 | TDD |  |
| CA_3A-28A-42A11 | 3 |  |  | -96.8 | -93.8 |  | -92 | -90.8 | FDD |  |
|  |  |  |  | [-99.5]6 | [-96.56] 6 |  | [-94.7]6 | [-93.5]6 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 42 |  |  | -97.1 | -94.7 |  | -93.2 | -92.5 | TDD |  |
| CA_3A-28A-42A17,18 | 3 |  |  | -96.8 | -93.8 |  | -92 | -90.8 | FDD |  |
|  |  |  |  | [-99.5]6 | [-96.5]6 |  | [-94.7]6 | [-93.5]6 |  |  |
|  | 28 |  |  | -98.3 | -95.3 |  |  |  |  |  |
|  | 42 |  |  | -85.7 | -85.4 |  | -85.1 | -84.9 | TDD |  |
| CA_3A-31A12,13 | 3 |  |  | -86.9 | -86.4 | -86 |  | -85.6 | FDD |
|  | 31 |  | -95.5 | -93.3 |  |  |  |  |  |
| CA_3A-42A9,10 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 |  | -71.7 | TDD |
| CA_3A-42A11 | 3 |  |  | -96.8 | -93.8 | -92 |  | -90.8 | FDD |
|  |  |  |  | -99.520 | -96.520 | -94.720 |  | -93.520 |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 |  | -92.5 | TDD |
| CA_4A-4A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-4A-12B5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-5A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 5 |  |  | -97.5 | -94.5 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-4A-5A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 5 |  |  | -97.5 | -94.5 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-5A-12B5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 5 |  |  | -97.5 | -94.5 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-7A-12A5,6 | 4 |  |  | [-90] | [-89.5] | [-89] |  | [-88.5] | FDD |
|  | 7 |  |  | -97.5 | -94.5 |  |  |  |  |
|  |  |  |  | -100.220 | -97.220 |  |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-12A5,6 | 4 | -89.2 | -89.2 | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  | -98.2 | -96.5 | -93.5 |  |  |  |  |
| CA_4A-12A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-12A-30A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |  |
| CA_4A-4A-12A-30A5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |  |
| CA_4A-12B5,6 | 4 |  |  | -90 | -89.5 | -89 |  | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-17A5,6 | 4 |  |  | -90 | -89.5 |  |  |  | FDD |
|  | 17 |  |  | -96.5 | -93.5 |  |  |  |  |
| CA_4A-28A5,6 | 4 |  |  | -89.8 | -89.4 | -89 |  | -88.7 | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -90.8 |  |
| CA_5A-12A-66A5,6 | 5 |  |  | -97.5 | -94.5 |  |  |  | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |  |
|  | 66 |  |  | -90 | -89.5 | -89 |  | -88.5 |  |
| CA_5A-38A19 | 5 |  |  | N/A | N/A |  |  |  | FDD |
|  | 38 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_7A-8A5,6 | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 | FDD |
|  | 8 |  | -99 | -96.8 | -93.8 |  |  |  |  |
| CA_7A-7A-8A5,6 | 7 |  |  | -88 | -87.4 | -87 |  | -86.7 | FDD |
|  | 8 |  |  | -96.8 | -93.8 |  |  |  |  |
| CA_7A-8A-20A5,6 | 7 |  |  |  | -87.4 | -87 |  | -86.7 | FDD |
|  | 8 |  | -99 | -96.8 | -93.8 |  |  |  |  |
|  | 20 |  |  | [-96.8] | [-93.8] |  |  |  |  |
|  |  |  |  | [-99.5]20 | [-96.5]20 |  |  |  |  |
| CA_7A-20A-38A19 | 7 |  |  |  | N/A | N/A |  | N/A | FDD |
|  | 20 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 38 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_8A-28A-41A22 | 8 |  |  | N/A | N/A |  |  |  | FDD |
|  | 28 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 41 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_8A-41A8 | 8 | N/A | N/A | N/A | N/A |  |  |  | FDD |
|  | 41 |  |  |  | N/A | N/A |  | N/A | TDD |
| CA_8A-42A12,13 | 8 | -102 | -99 | -96.8 | -93.8 |  |  |  | FDD |
|  | 42 |  |  | -84.8 | -84.7 | -84.6 |  | -84.5 | TDD |
| CA_8A-42C12,13 | 8 | -102 | -99 | -96.8 | -93.8 |  |  |  | FDD |
|  | 42 |  |  | -84.8 | -84.7 | -84.6 |  | -84.5 | TDD |
| CA_11A-42A | 11 |  |  | -100 | -97 |  |  |  | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 |  | -92.5 | TDD |
| CA_12A-30A-66A5,6 | 12 |  |  | -96.5 | -93.5 |  |  |  | FDD |
|  |  | 30 |  |  |  |  |  | -98.5 |  | -95.5 |  |  |  |  |  |  |  |  |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_12A-66A5,6 | 12 |  | -98.2 | -96.5 | -93.5 |  |  |  | FDD |
|  | 66 | -88.7 | -88.7 | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_12A-66C5,6 | 12 |  |  | -96.5 | -93.5 |  |  |  | FDD |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_12B-66A5,6 | 12 |  |  | -96.5 | -93.5 |  |  |  | FDD |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_12A-66A-66A5,6 | 12 |  |  | -96.5 | -93.5 |  |  |  | FDD |
|  | 66 |  |  | -89.5 | -89 | -88.5 |  | -88 |  |
| CA_20A-38A19 | 20 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 38 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_20A-40A15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 | FDD |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  |  | FDD |
|  | 28 |  |  | N/A | N/A |  |  |  |  |
| CA_21A-28A-42A4,21 | 21 |  |  | N/A | N/A | N/A |  | N/A | FDD |
|  | 28 |  |  | N/A | N/A | N/A |  |  |  |
|  | 42 |  |  | N/A | N/A |  |  |  |  |
| CA_26A-41A8 | 26 |  |  | N/A | N/A | N/A |  |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A |  | N/A | TDD |
| CA_28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 | FDD |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_28A-40C15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 | FDD |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_28A-40D15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 |  | -60.7 | FDD |
|  | 40 |  |  | -100 | -97 | -95.2 |  | -94 | TDD |
| CA_28A-41A-42A17,18 | 28 |  |  | -98.3 | -95.3 |  |  |  | FDD |
|  | 41 |  |  |  | -94.6 | -92.8 |  | -91.6 |  |
|  | 42 |  |  |  | -85.4 | -85.1 |  | -84.9 | TDD |
| CA_28A-41A-42C17, 18 | 28 |  |  | -98.3 | -95.3 |  |  |  | FDD |
|  | 41 |  |  |  | -94.6 | -92.8 |  | -91.6 | FDD |
|  | 42 |  |  |  | -85.4 | -85.1 |  | -84.9 | TDD |
| CA_28A-41C-42A 17, 18 | 28 |  |  | -98.3 | -95.3 |  |  |  | FDD |
|  | 41 |  |  |  | -94.6 | -92.8 |  | -91.6 | FDD |
|  | 42 |  |  |  | -85.4 | -85.1 |  | -84.9 | TDD |
| CA_28A-42A17,18 | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -92.3 | FDD |
|  | 42 |  |  | -85.7 | -85.4 | -85.1 |  | -84.9 | TDD |
| CA_28A-42C17,18 | 28 |  |  | -98.3 | -95.3 | -93.5 |  | -92.3 | FDD |
|  | 42 |  |  | -85.7 | -85.4 | -85.1 |  | -84.9 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: Void. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_3A-19A-42A, CA_3A-19A-42C, CA_1A-3A-19A-42A and CA_1A-3A-19A-42C. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 14: For the UE that supports CA_1A-19A-28A, no requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 17: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 18: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 19: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 21: No requirements apply when there is at least one individual RE on band 28 uplink outside frequencies 728 – 738 MHz. The reference sensitivity is only verified when all configured RE’s are confined within frequencies 728 – 738 MHz (the requirements specified in clause 7.3.1 of [6] apply). NOTE 22: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-7A-8A | 8 |  |  |  | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-28A | 28 |  |  |  | 16 | 25 | 25 | FDD |
| CA_1A-3C-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-19A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-19A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-7A-8A | 8 |  |  |  | 16 | 25 | 25 | FDD |
| CA_1A-7A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-28A-42A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_2A-2A-4A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-4A-4A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-12B | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-5A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-7A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-12A-30A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_3A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-7A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-7A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-7A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-19A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |  |
| CA_3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_4A-4A-5A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-4A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-12A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-4A-12B | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-4A-12A-30A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-5A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-5A-12B | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-7A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-12A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_4A-12A-30A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-12B | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-17A | 17 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-28A | 28 |  |  | [8] | [16] | [25] | [25] | FDD |
| CA_5A-12A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_7A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_7A-8A-20A | 8 |  |  |  | 16 | 25 | 25 | FDD |
| CA_8A-42A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_8A-42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_12A-30A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_12A-66A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_12A-66A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_12A-66C | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_12B-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_20A-40A3 | 40 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-40C | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-41A-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-41A-42C | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-41C-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-42A | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_28A-42C | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0bA: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for two bands)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.78 | -99.78 | -97.98 | -96.78 |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1A-3A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.78 | -99.78 | -97.98 | -96.78 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.78 | -96.78 | -94.98 | -93.78 |  |
| CA_1A-3C4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.78 | -99.78 | -97.98 | -96.78 |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1A-3C5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.78 | -99.78 | -97.98 | -96.78 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.78 | -96.78 | -94.98 | -93.78 |  |
| CA_1C-3A4 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.78 | -99.78 | -97.98 | -96.78 |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
| CA_1C-3A5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.78 | -99.78 | -97.98 | -96.78 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.78 | -96.78 | -94.98 | -93.78 |  |
| CA_18A-28A6 | 18 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 28 |  |  | -94 | -92.5 |  |  |  |
| CA_19A-28A7 | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 28 |  |  | -94 | -92 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 8: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bB: Uplink configuration for the uplink band (exceptions for two bands)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1C-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1C-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_18A-28A4 | 18 |  |  | 18 | 18 | 18 |  | FDD |
| CA_19A-28A4 | 19 |  |  | 18 | 18 | 18 |  | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 4: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 28 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0bC: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for three bands)

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) |  |  | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 5 |  |  | -98 |  |  | -95 |  |  |  |
| CA_1A-3A-5A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 5 |  |  | -98 |  |  | -95 |  |  |  |
| CA_1A-3A-7A9 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-7A10 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
|  |  |  |  |  |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-7A-7A | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  | 39 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 310 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-7C9 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  |  |  |  | -91.5 | -90 | -89 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-7C10 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  |  |  |  | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  |  |  | -95 | -93.2 | -92 |  |
| CA_1A-3A-8A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3A-8A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3C-8A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3C-8A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  | 8 |  | -99.2 | -97 |  |  | -94 |  |  |  |
| CA_1A-3A-11A4 | 1 |  |  |  | -100 | -97 |  | -95.2 | -94 | FDD |
|  |  |  |  |  | -102.711 | -99.711 |  | -97.911 | -96.711 |  |
|  | 3 |  |  |  | -94 | -91.5 |  | -90 | -89 |  |
|  | 11 |  |  |  | -100 | -97 |  |  |  |  |
| CA_1A-3A-11A5 | 1 |  |  |  | -100 | -97 |  | -95.2 | -94 | FDD |
|  |  |  |  |  | -102.711 | -99.711 |  | -97.911 | -96.711 |  |
|  | 3 |  |  |  | -97 | -94 |  | -92.2 | -91 |  |
|  |  |  |  |  | -99.711 | -96.711 |  | -94.911 | -93.711 |  |
|  | 11 |  |  |  | -100 | -97 |  |  |  |  |
| CA_1A-3A-19A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 19 |  |  | -100 |  |  | -97 | -95.2 |  |  |
| CA_1A-3A-19A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 19 |  |  | -100 |  |  | -97 | -95.2 |  |  |
| CA_1A-3A-20A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 20 |  |  | -97 |  |  | -94 | -91.2 | -90 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -93.911 | -92.711 |  |
| CA_1A-3A-20A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 20 |  |  | -97 |  |  | -94 | -91.2 | -90 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -93.911 | -92.711 |  |
| CA_1A-3A-21A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 21 |  |  | -100 |  |  | -97 | -95.2 |  |  |
| CA_1A-3A-21A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | [-99.7]11 |  |  | [-96.7]11 | [-94.9]11 | [-93.7]11 |  |
|  | 21 |  |  | -100 |  |  | -97 | -95.2 |  |  |
| CA_1A-3A-26A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 26 |  |  | -97.57 |  |  | -94.57 |  |  |  |
| CA_1A-3A-26A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 26 |  |  | -97.57 |  |  | -94.57 |  |  |  |
| CA_1A-3A-28A4 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -94 |  |  | -91.5 | -90 | -89 |  |
|  | 28 |  |  | -98.3 |  |  | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-28A5 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 3 |  |  | -97 |  |  | -94 | -92.2 | -91 |  |
|  |  |  |  | -99.711 |  |  | -96.711 | -94.911 | -93.711 |  |
|  | 28 |  |  | -98.3 |  |  | -95.3 | -93.5 | -90.8 |  |
| CA_1A-3A-42A4 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -93.8 |  |  | -91.3 | -89.8 | -88.8 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-42A5 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -96.8 |  |  | -93.8 | -92 | -90.8 |  |
|  |  |  |  | -99.511 |  |  | -96.511 | -94.711 | -93.511 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-42C4 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -93.8 |  |  | -91.3 | -89.8 | -88.8 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-3A-42C5 | 1 |  |  | -99.8 |  |  | -96.8 | -95 | -93.8 | FDD |
|  |  |  |  | -102.511 |  |  | -99.511 | -97.711 | -96.511 |  |
|  | 3 |  |  | -96.8 |  |  | -93.8 | -92 | -90.8 |  |
|  |  |  |  | -99.511 |  |  | -96.511 | -94.711 | -93.511 |  |
|  | 42 |  |  | -98.5 |  |  | -95.5 | -93.7 | -92.5 | TDD |
|  |  |  |  | -100.711 |  |  | -97.711 | -95.911 | -94.711 |  |
| CA_1A-18A-28A6 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 18 |  |  | -100 |  |  | -97 | -95.2 |  |  |
|  | 28 |  |  | -94 |  |  | -92.5 |  |  |  |
| CA_1A-19A-28A8 | 1 |  |  | -100 |  |  | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.711 |  |  | -99.711 | -97.911 | -96.711 |  |
|  | 19 |  |  | -100 |  |  | -97 | -95.2 |  |  |
|  | 28 |  |  | -94 |  |  | -92 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bD: Uplink configuration for the uplink band (exceptions for three bands)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-5A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-5A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-7A-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-7A-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-7C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-7C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-8A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-8A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3C-8A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C-8A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-19A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-19A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-20A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-20A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-21A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-21A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-26A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-26A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-28A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-28A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-18A-28A4 | 18 |  |  | 18 | 18 | 18 |  | FDD |
| CA_1A-19A-28A4 | 19 |  |  | 18 | 18 | 18 |  | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 4: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 28 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

# <Next change>

# Table 7.3.1A-0bE: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |  |
| CA_1A-3A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-5A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-7A-40A15 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A16 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_1A-3A-7A-40A | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_1A-3A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-8A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-38A12 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 3 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
|  | 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-38A13 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 1 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 3 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
|  | 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-3C-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |  |
| CA_1A-3A-41A12,14 | 3 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-41A13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 41 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3C-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 3 |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |
|  | 40 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD |  |
| CA_1A-3C-40C | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |
| CA_1A-3C-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |  |
| CA_1A-5A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-5A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |  |
|  | 20 |  |  | -96.8 | -93.8 | -91 | -89.8 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
| CA_1A-7A-20A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 20 |  |  | -96.8 | -93.8 | -91 | -89.8 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-96.9]11 | [-94.7]11 |  |  |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_1A-7A-40A | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_1A-7A-42A | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 1 |  |  | -99.8 | -96.8 | -95 | -93.8 | FDD | 42 |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-96.9]11 | [-94.7]11 |  |  |  |
| CA_1A-8A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-8A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-40A | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_1A-40C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |  |
|  |  |  |  | -102.711 | -99.711 | -97.911 | -96.711 |  |  |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |  |  |
| CA_1A-40C | 1 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-5A-40A | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  | -101.411 | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 5 |  | -100.2 | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-5A-40A | 3 |  | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 5 |  | -100.2 | -98 | -95 |  |  |  |  |  |
|  | 40 |  |  |  | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-7A-38A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  |  |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_3A-7A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 7 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |  |
|  | 7 |  |  | -97.7 | -94.7 | -92.9 | -91.7 |  |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |  |
| CA_3A-7A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 7 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |  |
| CA_3A-7A-42A | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD | 7 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 3 |  |  | [-96.6] | [-93.6] | [-91.8] | [-90.6] | FDD | 42 |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-96.9]11 | [-94.7]11 |  |  |  |
| CA_3A-8A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-8A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 8 |  | -99.2 | -97 | -94 |  |  |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-20A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |  |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-28A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-28A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |  |
|  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 28 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_3A-28A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-28A-41A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 |  |  |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
| CA_3A-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-40A-40A | 3 |  |  | -97 | -94 |  |  | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 |  |  |  |  |  |
|  | 40 |  |  |  | -92.9 |  | -90.2 | TDD |  |  |
| CA_3A-40A-40A | 3 |  |  | -94.2 | -91.2 |  |  | FDD | 40 |  |
|  | 40 |  |  |  | -97 |  | -94 | TDD |  |  |
| CA_3A-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3A-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3C-40A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3C-40A | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3C-40C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 40 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD |  |  |
| CA_3C-40C | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_3A-41A5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3A-41C5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3A-41D5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41A5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41C5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |  |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
| CA_3C-41D5 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 3 |
|  |  |  |  | -99.711 | -96.711 | -94.911 | -93.711 |  |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
| CA_3A-41A-42A5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42A5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42A5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_3A-41A-42A5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  |  |  |  | -100.211 | -97.211 | -95.411 | -94.211 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42C5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41C-42A5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |  |
|  | 41 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD |  |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |  |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 |  |  |  |
| CA_3A-41C-42C5,6,7,8 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |
| CA_3A-41C-42C5,6,9 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 3 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD |  |
|  | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |
| CA_3A-41C-42C5,6,10 | 3 |  |  | -96.5 | -93.5 | -91.7 | -90.5 | FDD | 42 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |
| CA_3A-41C-42C5,6,10 | 3 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |
|  | 41 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |  |
| CA_7A-20A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-95.9]11 | [-94.7]11 |  |  |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |  |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | [-100.7]11 | [-97.7]11 | [-96.9]11 | [-94.7]11 |  |  |  |
| CA_7A-40A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 40 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |  |
|  | 7 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |  |
| CA_7A-40C | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 40 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD |  |  |
|  | 7 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
|  | 40 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | TDD |  |  |
| CA_7A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_7A-42A-42A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | 7 |  |
|  |  |  |  | -100.711 | -97.711 | -95.911 | -94.711 |  |  |  |
|  | 42 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD |  |  |
|  | 7 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
|  |  |  |  | -100.711 | -97.711 | -96.911 | -94.711 |  |  |  |
| CA_28A-40A | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_28A-40A | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_28A-40C | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD |  |  |
| CA_28A-40C | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |  |  |
| CA_28A-40D | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD | 28 |  |
|  | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 |  |  |  |
| CA_28A-40D | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
|  | 40 |  |  | -100 | -97 | -95.2 | -94 |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply regardless of the channel bandwidth and the location of UL band. NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 6: The antenna isolation for MSD calculation is assumed as 10 dB. For conducted mode REFSENS test such antenna isolation is not observed as the antennas are disconnected. Additionally antenna isolation assumption is under discussion depending on the frequency range NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_3A-19A-42A, CA_3A-19A-42C and CA_1A-3A-19A-42A, CA 3A-41A-42A. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 11: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 15: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1 NOTE 16: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. |  |  |  |  |  |  |  |  |  |  |

# Table 7.3.1A-0bF: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-7A-40A | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-38A | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 38 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3C-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-41A | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-5A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_1A-8A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 8 |  | 15 | 25 | 251 |  |  | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-20A-42A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 20 |  |  | 25 | 201 | 205 | 205 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-42A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-7A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-40C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-8A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 8 |  | 15 | 25 | 251 |  |  | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-40A CA_3A-40C CA_3A-5A-40A CA_3A-28A-40A CA_3A-28A-40C CA_1A-3A-5A-40A CA_1A-3A-8A-40A | 3 |  | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-40A-40A | 3 |  |  | 25 | 50 |  |  | FDD |
|  | 40 |  |  |  | 50 |  | 100 | TDD |
| CA_3A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41C CA_3C-41A CA_3C-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3C-41C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41A-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-20A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 20 |  |  | 25 | 50 | 75 | 100 |  |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-40A, CA_7A-40C | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-42A, CA_7A-42A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-40A, CA_28A-40C | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 5: 5 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 |  |  |  |  |  |  |  |  |

# <Next change>

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-5A-7A-46C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-5A-46A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-5A-46D | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-7A-46A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-7A-46D | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-46A CA_1A-46D CA_1A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.74 | -99.74 | -97.94 | -96.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-46C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  |  |  |  | -102.74 | -99.74 | -97.94 | -96.74 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_1A-5A-46C5 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-5A-7A-46A1 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-7A-46C1 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-46A CA_2A-46A-46A CA_2A-46A-46C CA_2A-46D CA_2A-46A-46D | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-46A CA_3A-46D CA_3A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_3A-46C | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_4A-46A CA_4A-46A-46A CA_4A-46A-46C CA_4A-46D CA_4A-46A-46D | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_4A-46C | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-7A-46A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-7A-46C1 | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-7A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46C | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  | -93 |  |  | -90 | TDD |
| CA_5A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46E | 5 |  |  | -98 | -95 |  |  | FDD |  |
|  | 46 |  |  |  |  |  | -90 | TDD |  |
| CA_7A-46A CA_7A-46C CA_7A-46D CA_7A-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_8A-46A | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8A-46C | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8B-46A | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46C | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46C | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_19A-46A | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_19A-46C | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_19A-46D | 19 |  |  | -100 | -97 | -95.2 |  | FDD |  |
|  | 46 |  |  |  |  |  | -90 | TDD |  |
| CA_19A-46E | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46A | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46C | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46D | 21 |  |  | -100 | -97 | -95.2 |  | FDD |  |
|  | 46 |  |  |  |  |  | -90 | TDD |  |
| CA_21A-46E | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_26A-46A | 26 |  | -99.7 | -97.57 | -94.57 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_28A-46C | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_39A-46C | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39C-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40A-46A | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40A-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40C-46A | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_41A-46A | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
|  |  |  |  | -100.74 | -97.74 | -95.94 | -94.74 |  |
|  | 46 |  |  |  |  |  | -83 | TDD |
| CA_46A-66A CA_46A-46A-66A CA_46A-46C-66A CA_46A-46D-66A CA_46D-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_46C-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The signal power is specified per port. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 5: The requirements do not apply when there is at least one individual RE within the uplink transmission bandwidth of the lower band for which the transmitter harmonic is within the downlink transmission bandwidth of the higher band and a range FHD from lower and higher frequency of the uplink transmission bandwidth multiplied by the harmonic order. Negative FHD reduces the REFSENS exclusion range while positive FHD extends the range. FHD = [30] MHz for the E-UTRA configuration which has 2nd harmonic issue. FHD = 15 MHz for the E-UTRA configuration which has 3rd harmonic issue. FHD = [10] MHz for the E-UTRA configuration which has 4th harmonic issue. FHD = [-20] MHz for the E-UTRA configuration which has 6th harmonic issue. FHD = [-30] MHz for the E-UTRA configuration which has 7th harmonic issue. FHD = [-40] MHz for the E-UTRA configuration which has for 8th harmonic issue. For band combinations including more than one licensed band with band 46, the value FHD depends on which licensed band is used for uplink transmission. NOTE 6: The value FHD in NOTE 5 shall be scaled as FHD *(NRB/100) according to the uplink configuration defined in Table 7.3.1A-0eB. NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0eB: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-5A-7A-46C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 5 |  |  | 25 | 25 | 25 | 25 |  |
|  | 7 |  |  | 25 | 50 | 75 | 75 |  |
| CA_1A-5A-46A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 5 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-5A-46D | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 5 |  |  | 25 | 25 | 25 | 25 |  |
| CA_1A-7A-46A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-7A-46D | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 75 |  |
| CA_1A-46A CA_1A-46D CA_1A-46E | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_1A-46C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_1A-5A-46C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 5 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-5A-7A-46A1 | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 5 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 75 | FDD |
| CA_1A-7A-46C1 | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 75 | FDD |
| CA_2A-46A CA_2A-46A-46A CA_2A-46A-46C CA_2A-46D CA_2A-46A-46D | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-46C | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-46A CA_3A-46D CA_3A-46E | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-46C | 3 |  | 15 | 25 | 50 | 501 | 501 | FDD |
| CA_4A-46A CA_4A-46A-46A CA_4A-46A-46C CA_4A-46A-46D CA_4A-46D | 4 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_4A-46C | 4 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_5A-7A-46C1 | 5 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 75 | FDD |
| CA_5A-7A-46D | 5 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 75 |  |
| CA_5A-46A | 5 |  |  | 25 | 251 |  |  | FDD |
| CA_5A-46C | 5 |  |  | 25 | 251 | 251 | 251 | FDD |
| CA_5A-46D | 5 |  |  | 25 | 251 | 251 | 251 | FDD |
| CA_5A-46E | 5 |  |  | 25 | 251 | 251 | 251 | FDD |  |
| CA_5A-7A-46A | 5 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 75 | FDD |
| CA_7A-46A CA_7A-46D CA_7A-46E | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
| CA_8A-46A | 8 | 6 | 15 | 25 | 251 |  |  | FDD |
| CA_8A-46C | 8 | 6 | 15 | 25 | 251 |  |  | FDD |
| CA_8B-46A | 8 | 6 | 15 | 25 | 251 |  |  | FDD |
| CA_11A-46C | 11 |  |  | 25 | 50 |  |  | FDD |
| CA_13A-46A | 13 |  |  | 25 | 50 |  |  | FDD |
| CA_13A-46C | 13 |  |  | 25 | 50 |  |  | FDD |
| CA_19A-46A | 19 |  |  | 25 | 251 | 251 |  | FDD |
| CA_19A-46C | 19 |  |  | 25 | 251 | 251 |  | FDD |
| CA_19A-46D | 19 |  |  | 25 | 251 | 251 |  | FDD |
| CA_19A-46E | 19 |  |  | 25 | 25 1 | 251 |  | FDD |
| CA_21A-46A | 21 |  |  | 25 | 251 | 251 |  | FDD |
| CA_21A-46C | 21 |  |  | 25 | 251 | 251 |  | FDD |
| CA_21A-46D | 21 |  |  | 25 | 251 | 251 |  | FDD |
| CA_21A-46E | 21 |  |  | 25 | 25 1 | 251 |  | FDD |
| CA_26A-46A | 26 |  |  | 25 | 25 |  |  | FDD |
| CA_40A-46A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_40A-46C | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_40C-46A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_41A-46A | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_42A-46A | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_46A-66A CA_46A-46A-66A CA_46A-46C-66A CA_46A-46D-66A CA_46D-66A | 66 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_46C-66A | 66 |  |  | 25 | 50 | 75 | 100 | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

# <End of Changes>