3GPP TSG RAN#75	 R1-170511

Dubrovnik, Croatia

6th – 9th March, 2017

Title:  	On eMTC 5G mMTC requirement fulfilment

Source: 	Ericsson

Agenda Item:	10.1.1

Document for:	Information

# Introduction

At RAN#74 it was agreed that “3GPP’s IMT-2020 self-evaluations towards mMTC requirements will assess NB-IoT and/or LTE eMTC” [1]. To prepare for these self-evaluations it is proposed to assess the eMTC performance in a short study phase during the beginning of the efeMTC Release 15 work item [3]. The intention of this assessment is to identify potential gaps between eMTC performance and the 5G mMTC requirements. The normative phase of the efeMTC work item can then be used to bridge these gaps.

Technical Report (TR) 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies [2] identifies four key performance indicators for mMTC that should be addressed during this work:

- Support coverage of 164 dB Maximum Coupling Loss (MCL).

- Support latency of at least 10 seconds.

- Support UE battery life beyond 10 years.

- Support connection density of 1 000 000 devices per square km.

To use the time efficiently in Release 15 it is important to make this study phase as short as possible. To facilitate this Ericsson has already in a set of contributions presented and proposed mMTC performance evaluation methodologies [4], [5], [6]. In this paper we use these methodologies to evaluate and present Release-14 eMTC performance.

# eMTC performance

### Coverage

Coverage has been identified as a key enabler for mMTC applications. For the 5G evaluations a target of 164 dB maximum coupling loss (MCL) has been agreed. At this coverage a data rate of 160 bps is to be achieved. Besides the agreements captured  in TR 38.913 the sourcing company proposes to reuse the methods captured in TR 45.820 [7] when evaluating coverage.

Table 1 summarizes a collection of results from earlier published material [8][9][10]. The data rates are taking the 10% BLER into account. The results clearly indicate the potential of eMTC to meet the 5G MCL requirements.

Table 1 eMTC Release 14 MCL.

| # | Physical channel name | PUCCH | PRACH | PUSCH | PDSCH | MPDCCH | PBCH, MIB | PDSCH, SIB1-BR | PSS/ SSS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | BLER target [%] | 10% | 10% | 10% | 10% | 10% | 90th perc. | 90th perc | 90th perc |  |
| 2 | TBS [bits] | - | - | 392 | 936 | - | - | 152 | - |  |
| 3 | Repetitions | 32 | 128 | 2048 | 512 | 64 | - | 16 |  |  |
| 4 | Data rate [bps], Acquisition time [ms] | - | - | 172 bps | 1645 bps | - | 520 ms | 1040 ms | 460 ms |  |
| Transmitter |  |  |  |  |  |  |  |  |  |
| 5 | Total Tx power [dBm] | 23 | 23 | 23 | 46 | 46 | 46 | 46 | 46 |  |
|  | Power boosting [dB] | - | - | - | 3 | 3 | 3 | 3 | 3 |  |
| 6 | Actual Tx power [dBm] | 23 | 23 | 23 | 39.8 | 39.8 | 39.8 | 39.8 | 39.8 |  |
| Receiver |  |  |  |  |  |  |  |  |  |
| 7 | Thermal noise [dBm/Hz] | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |  |
| 8 | Receiver noise figure [dB] | 3 | 3 | 3 | 5 | 5 | 5 | 5 | 5 |  |
| 9 | Interference margin [dB] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| 10 | Channel bandwidth [kHz] | 180 | 1080 | 180 | 1080 | 1080 | 1080 | 1080 | 1080 |  |
| 11 | Effective noise power [dBm] = (7)+(8)+(9)+10log10((10)) | -118.5 | -110.7 | -118.5 | -108.7 | -108.7 | -108.7 | -108.7 | -108.7 |  |
| 12 | Required SINR [dB] | -24 | -31.2 | -23.6 | -18.5 | -18.5 | -15 | -15 | -16.2 |  |
| 13 | Dual antenna receiver sensitivity [dBm] = (11)+(12) | -142.5 | -141.9 | -142.1 | -127.2 | -127.2 | -123.7 | -123.7 | -124.9 |  |
| 14 | MCL [dB] = (3)-(10) | 165.5 | 164.9 | 165.1 | 167 | 167 | 163.5 | 163.5 | 164.7 |  |

It should be noted that these are preliminary results. Some of the assumptions used in these evaluations differs compared to the assumptions used in TR 45.820:

- Initial synchronization has been evaluate assuming a 1 kHz initial frequency offset. Following the TR 45.820 assumption of 18 kHz initial frequency offset is expected to increase the initial synchronization time.

- EPA 1 Hz propagation conditions was used in the PDSCH and MPDCCH evaluations. The difference between EPA 1 Hz and TU 1 Hz is however expected to be small [11].

In TR 38.913 noise figures of 5 dB and 9 dB have been agreed for the eNB and UE, respectively. The herein presented evaluations follows the Release 13 noise figure assumptions of 3 dB and 5 dB for the eNB and UE, respectively.

### Latency

The latency of eMTC to deliver 85 bytes using the RRC Resume procedure is captured in Table 2. The methodology followed is presented in [5]. At 164 dB MCL the performance presented in Table 1 was used when deriving the results.

In these initial evaluations it is seen that 8.6 second latency is estimated to be required to deliver 85 bytes at the MCL of 164 dB. This indicates the potential of eMTC to meet the mMTC KPIs.

Table 2 eMTC Release 14 latency.

| Coupling loss [dB] | Latency |
| --- | --- |
| 144 | 0.2 s |
| 154 | 0.6 s |
| 164 | 8.6 s |

### Battery life

The battery life of eMTC for small infrequent data transmission is captured in Table 3. The methodology followed is presented in [4]. At 164 dB the performance presented in Table 1 was used when deriving the results with one exception. The assumption in [4] of using the PUSCH and PDSCH performance at 10% BLER is quite pessimistic. More realistic is to use HARQ in combination with the average number of transmissions needed to deliver a data packet. At 164 dB MCL this alternative approach was investigated for the PUSCH which is believe to more accurately reflect the achievable battery life.

This preliminary investigation indicates that eMTC falls somewhat short compared to 10 years KPI for the case of 200 bytes delivered once every 24 hours. The improvements proposed in the scope of the Release 15 efeMTC WI are expected to improve this performance and bridge this potential gap to the mMTC requirement.

Table 3 eMTC Release 14 battery life.

| Reporting interval | DL Packet size | UL Packet size | Battery life [Years] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 144 dB MCL | 154 dB MCL | 164 dB MCL |
| 2 hours | 65 bytes | 50 bytes | 23.7 y | 14.0 y | 2.1 y |
|  |  | 200 bytes | 22.4 y | 8.8 y | 0.9 y |
| 24 hours |  | 50 bytes | 36.5 y | 33.5 y | 15.7 y |
|  |  | 200 bytes | 36.2 y | 30.0 y | 8.8 y |

### Connection density

The connection density of eMTC derived from system simulations presented in [6] are shown in Table 4. The service latency recorded at this extreme load is summarized in Table 5. These findings indicate that the target of 1000 000 devices per square kilometre can be served by three narrowband, each six PRB wide.

Table 4 eMTC Release 14 connection density.

| Case | eMTC narrowband |
| --- | --- |
| Connection density @ 1% outage | 361 000 devices/km2 |

Table 5 eMTC service latency at extreme load.

| Percentile | Latency |
| --- | --- |
| 50th percentile | 0.17 s |
| 90th percentile | 1.2 s |
| 99th percentile | 45 s |

# Conclusions

In this paper we present an initial assessment of eMTC performance in terms of coverage, latency, battery life and connection density. The evaluations are based on a set of methodologies intended to be used in the Release 15 efeMTC and feNB-IoT work items for mMTC performance evaluations. The results indicate a potential to meet the 5G mMTC requirements, but also indicates that there is room for improvement if the mMTC requirements are to be met for the more conservative UE and eNB noise figures agreed in TR 38.913.

# References

RP-162560, “Way forward on mMTC in NR Rel-15”, source Ericsson et. al, RAN#74

TR 38.913 v14.0.0 “Study on Scenarios and Requirements for Next Generation Access Technologies” v14.0.0

RP-170465, “New WI: Even further enhanced MTC for LTE”, source Ericsson, RAN#75

R1-1703112, “On mMTC, NB-IoT and eMTC battery life evaluation”, source Ericsson, RAN1#88

R1-1703113, “On mMTC, NB-IoT and eMTC latency evaluation”, source Ericsson, RAN1#88

R1-1703865, “On 5G mMTC requirement fulfilment, NB-IoT and eMTC connection density”, source Ericsson, RAN1#88

TR 45.820 “Cellular system support for ultra-low complexity and low throughput Internet of Things” v13.0.0

White paper: “Coverage Analysis of LTE-M Category M-1”, source Sierra-Wireless, Ericsson et al., December 2016

“On the achievable coverage and uplink capacity of Machine-Type Communications (MTC) in LTE Release 13”, source Ericsson

R4-1609696, “Simulation result for eMTC system information acquisition time”, source Ericsson, RAN4#81

GP-140566, “Performance comparison between TU and EPA”, source Huawei, GERAN#63