3GPP TSG RAN Meeting #80	RP-181018

S. Diego, US; 11 – 14 June 2018

Agenda item:	9.5.2

Source:	Ericsson

Title:	TP to TR 37.890 EU Regulatory Aspects section

Document for:	Approval

# Introduction

This document is a Text Proposal to provide corrections to Table 4.1.1.3-1 of EU Unlicensed Operations section of 3GPP Techincal Report TR 37.890 “Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations”.

# 2	Text proposal

<<< Start of TP>>>

#### 4.1.1.3	Unlicensed operations

In December 2017, EC Radio Spectrum Committee finalized the Mandate to CEPT [21] to study and identify harmonised compatibility and sharing technical conditions for a sustainable and efficient use on a shared basis of the frequency range 5.925-6.425 GHz for Wireless Access Systems including Radio Local Area Networks (WAS/RLANs).

CEPT has created two new project teams, SE 45 and FM 57. The Project Team (PT) SE 45 has been tasked to undertake compatibility and sharing studies in the 6 GHz frequency range to support FM 57, and the latter will define the regulatory rules for the band. The scope of the work items in these groups is the introduction of low power wireless access systems (including RLAN) in the frequency range 5.925-6.425 GHz under a non-protected and non-interference basis, ensuring certainty of continued operation, development and protection of existing services (Fixed Services (FS), Fixed Satellite Service (FSS)) considering RR 5.440 and 5.458.

ETSI has developed the draft TR 103 524 [22] providing information on the technical parameters for WAS/RLANs to support the CEPT Work Items activities covering the frequency range 5.925 GHz to 6.425 GHz. In addition, it contains a request for considering additional frequencies up to 6.725 GHz.

ETSI assumptions/proposals on technical RLAN parameters to be used for compatibility and sharing studies are reported in the following:

- BS maximum TX output power: 30 dBm EIRP;

- Band plan: 5.925-6.725 GHz (guard band towards ITS below 5.925 GHz may be accommodated at the lower edge);

- Channel BW: 20, 40, 80, 160 MHz for the IEEE802 standard; wider channel bandwidth can be supported by 3GPP standards via carrier aggregation, up to 32 carriers (including at least one anchor carrier in a licensed band) for eLAA. (Similar configurations for NR-U mentioned but without details on bandwidths and aggregated carriers).

- TX unwanted emissions in the 6 GHz bands: same transmitter spectrum mask as used in 5 GHz (since same modulation assumed) shown in Figure 4.1.1.3-1;

Figure 4.1.1.3-1: Transmit spectral power mask for RLAN equipment

- TX unwanted emissions outside the 6 GHz bands: as specified in ERC Recommendation 74.01 [23] and reported in Table 4.1.1.3-1.

Table 4.1.1.3-1: Out of band emissions limits

| Frequency range | Limits | Reference Bandwidth |
| --- | --- | --- |
| 30-47 MHz | -36 dBm | 100 kHz |
| 47-74 MHz | -54 dBm | 100 kHz |
| 74-87.5 MHz | -36 dBm | 100 kHz |
| 87.5-118 MHz | -54 dBm | 100 kHz |
| 118-174 MHz | -36 dBm | 100 kHz |
| 174-230 MHz | -54 dBm | 100 kHz |
| 230-470 MHz | -36 dBm | 100 kHz |
| 470-862 MHz | -54 dBm | 100 kHz |
| 0.862-1 GHz | -36 dBm | 100 kHz |
| 1-5.925GHz | -30 dBm | 1 MHz |
| 6.725-26 GHz | -30 dBm | 1 MHz |

The Draft ECC Report on compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency range 5.925 – 6.425 GHz [24], developed by SE45, will use the technical characteristics specified in the ETSI TR 103 524 [22] for RLAN as starting point, and the technical parameters provided from SE19 and SE40 for FS and FSS respectively. It is to be noted that some of the technical parameters (for example max EIRP, TX unwanted emissions, etc.) for RLAN in the ETSI TR 103 524 may differ from the final rules for the band, depending on the results of the compatibility studies done by CEPT.

<<< End of TP>>>