##### RAN1

- RAN1 #92bis (April 2018)

General

R1-1805141 is endorsed. Send an LS to RAN2 – R1-1805558, which is approved by fixing the title with final LS in R1-1805765 R1-1803865 Draft CR to 38.211 Ericsson The change in 7.4.1.1.1 in R1-1803865 is endorsed (to be included in the next editor CR update) R1-1805175 Draft CR to 38.211 Ericsson R1-1805751 The draft LS is approved with final LS in R1-1805766

Initial access and mobility

Agreements: - Agree the text proposal below For a half frame with SS/PBCH blocks, the first symbol indexes for candidate SS/PBCH blocks are determined according to the subcarrier spacing of SS/PBCH blocks as follows, where index 0 is the first symbol of the first slot in a half-frame. Conclusion: - For the discrepancy in carrier definition in 38.211 and 38.331, 38.211 editor will align the definition with 38.331 Agreements: - To adopt the following TP to 38.213, Section 4.1: For a half frame with SS/PBCH blocks, the first symbol indexes for candidate SS/PBCH blocks are determined according to the subcarrier spacing of SS/PBCH blocks as follows. - Case A - 15 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes of {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3. - Case B - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies smaller than or equal to 3 GHz, n=0. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1. - Case C - 30 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {2, 8} + 14*n. For carrier frequencies smaller than or equal to 3 GHz, n=0, 1. For carrier frequencies larger than 3 GHz and smaller than or equal to 6 GHz, n=0, 1, 2, 3. - Case D - 120 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8, 16, 20} + 28*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18. - Case E - 240 kHz subcarrier spacing: the first symbols of the candidate SS/PBCH blocks have indexes {8, 12, 16, 20, 32, 36, 40, 44} + 56*n. For carrier frequencies larger than 6 GHz, n=0, 1, 2, 3, 5, 6, 7, 8. Which case(s) of Case A through E to be used on a carrier depends on frequency band(s) which is given by section of 5.4.3.3 of [8-1, TS 38.101-1] and [8-2, TS 38.101-2], and is the same case for all SS/PBCH blocks on a carrier. Agreements: - To adopt the following TP to 38.212, Section 7.1.1: Generate the following additional timing related PBCH payload bits , where: - are the 4th, 3rd, 2nd, and 1st LSB of SFN, respectively; - is the half radio frame bit ; - if are the 6th, 5th, and 4th bits of SS/PBCH block index, respectively. else is the MSB of as defined in Subclause 7.4.3.1 of [4, TS 38.211]. are reserved. end if Agreements: - The working assumption made in RAN1 AdHoc#1 on OFDM signal generation is confirmed Agreements: - In initial cell selection and idle mode, the UE may assume that the EPRE offset between PDCCH DMRS and SSS in the QCL’ed SSB is in the range [X, Y]dB when PDDCH with CRC scrambled by SI-RNTI, P-RNTI and RA-RNTI - FFS: value of X and Y and dependence on numerology - ¦X¦ & Y are no more than 12 Conclusion: - It is RAN1’s understanding that the f0 does not have to be on the channel raster nor the center of the RF filter Agreements: (RRC update) - From RAN1 perspective, the signalling of M in RMSI with parameter frequencyOffsetSSB is not necessary in Rel-15 Agreements: - Confirm working assumption on signalling tables for indication of cell-defining SSB Agreements: - Confirm working assumption on step size X=1 (for cell-defining SSB) Agreements: - To adopt the following TP to 38.213, Section 13: If a UE detects a first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for for FR1 or for for FR2, the UE determines the nearest (in the corresponding frequency direction) global synchronization channel number (GSCN) of a second SS/PBCH block having a control resource set for an associated Type0-PDCCH common search space as . is the GSCN of the first SS/PBCH block and is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. Agreements: - To adopt the following TP for 7.4.1.4.1 of 38.211 The UE shall assume the reference-signal sequence for an SS/PBCH block is defined by where is given by clause 5.2. The scrambling sequence generator shall be initialized at the start of each SS/PBCH block occasion with where - for, , is the number of the half-frame in which the PBCH is transmitted in a frame with for the first half-frame in the frame and for the second half-frame in the frame, and is the two least significant bits of the SS/PBCH block index as defined in [5, TS 38.213] - for or , and is the three least significant bits of the SS/PBCH block index as defined in [5, TS 38.213] with being the maximum number of SS/PBCH beams in an SS/PBCH period for a particular band as given by [38.104]. Agreements: - To adopt the following TP of 38.213, Section 13: Table 13-1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz ... Table 13-2: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz ... Table 13-3: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz ... Table 13-4: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz ... Table 13-5: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 15} kHz for frequency bands with minimum channel bandwidth 40MHz ... Table 13-6: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {30, 30} kHz for frequency bands with minimum channel bandwidth 40MHz Conclusion: - Editor to make updates regarding the wording of “subcarrier k” in 38.211 when necessary and applicable. Agreements: - For {SSB SCS, PDCCH SCS} = {240, 120} kHz, remove the configurations with =2 for Pattern 2 in Table 13-10 in TS 38.213. - ================== Start of Test Proposal (TS 38.213) ================= Table 13-10: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {240, 120} kHz Index; SS/PBCH block and control resource set multiplexing pattern; Number of RBs; Number of Symbols; Offset (RBs). 0; 1; 48; 1; 0. 1; 1; 48; 1; 8. 2; 1; 48; 2; 0. 3; 1; 48; 2; 8. 4; Reserved. 5; Reserved. 6; Reserved. 7; Reserved. 8; Reserved. 9; Reserved. 10; Reserved. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved. - ================== END of Test Proposal (TS 38.213) ================= R1-1805676 is approved. Final LS is in R1-1805712 Agreements: - Regarding the questions in LS R1-1803586: - 1st question: whether NR can support approximately 1700 bits RMSI in one TB in all cases. - Answer: - Yes, NR can support RMSI size of approximately 1700 bits in one TB for both FR1 and FR2 with an appropriate RMSI configuration. RAN1 agrees that max TBS of 2976 bits is supported for PDSCH by SI-RNTI from physical layer perspective. - 2nd question: If necessary, whether L1 parameters size in RMSI can be reduced. - Answer: Since the answer to the 1st question is yes, RAN1 sees no urgency to reduce L1 parameters size in RMSI. However, RAN1 is considering the possibility of doing so and will update RAN2 upon new progress. - To send the reply LS in R1-1805617, which is approved and final LS in R1-1805653 Agreements: - Update the WA (remains as WA) on FR1 configuration Table 6.3.3.2-3 in 38.211 section 6.3.3.2, with the following update for configuration index 166 for format B4: Table 6.3.3.2-3: Random access configurations for FR1 and unpaired spectrum. PRACH Configuration Index; Preamble format; Subframe number; Starting symbol; Number of PRACH slots within a subframe; , number of time-domain PRACH occasions within a RACH slot; , PRACH duration. 166; B4; 1; 0; 0,1,2,3,4,5,6,7,8,9 3,4,8,9; 0; 2; 1; 12. Agreements: - Replace the previous working assumption (remains WA) for FR2 RACH configuration table, with the following table. - The ones in yellow background are not final - Note: RACH configuration indexing will be up to editor to update. =============== Text proposal for 38.211, section 6.3.3 ============================= Table 6.3.3.2-4: Random access configurations for FR2 and unpaired spectrum. PRACH Config. Index; Preamble format; Slot number; Starting symbol; Number of PRACH slots within a 60 kHz slot; , number of time-domain PRACH occasions within a RACH slot; , PRACH duration. 0; A1; 1; 0; 4,9,14,19,24,29,34,39; 0; 1; 6; 2. 1; A1; 1; 0; 4,9,14,19,24,29,34,39; 7; 1; 3; 2. 2; A1; 1; 0; 9,19,29,39; 0; 2; 6; 2. 3; A1; 1; 0; 24,29,34,39; 7; 1; 3; 2. 4; A1; 1; 0; 17,19,37,39; 0; 1; 6; 2. 5; A1; 1; 0; 19,39; 7; 1; 3; 2. 6; A1; 1; 0; 9,19,29,39; 7; 2; 3; 2. 7; A1; 1; 0; 0,1,2,…,39; 7; 1; 3; 2. 8; A1; 1; 0; 23,27,31,35,39; 0; 1; 6; 2. 9; A1; 1; 0; 23,27,31,35,39; 7; 1; 3; 2. 10; A1; 1; 0; 3,7,11,15,19,23,27,31,35,39; 0; 1; 6; 2. 11; A1; 1; 0; 3,7,11,15,19,23,27,31,35,39; 7; 1; 3; 2. 12; A1; 1; 0; 1,3,5,7,…,37,39; 0; 1; 6; 2. 13; A1; 1; 0; 3,5,7; 0; 1; 6; 2. 14; A1; 1; 0; 3,5,7,9,11,13; 7; 1; 3; 2. 15; A1; 16; 1; 4,9,14,19,24,29,34,39; 0; 2; 6; 2. 16; A1; 16; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 6; 2. 17; A1; 8; 1; 4,9,14,19,24,29,34,39; 0; 2; 6; 2. 18; A1; 8; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 6; 2. 19; A1; 4; 1; 4,9,14,19,24,29,34,39; 0; 2; 6; 2. 20; A1; 4; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 6; 2. 21; A1; 2; 1; 4,9,14,19,24,29,34,39; 0; 2; 6; 2. 22; A1; 2; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 6; 2. 23; A1; 1; 0; 7,15,23,31,39; 0; 1; 6; 2. 24; A1; 4; 1; 4,9,14,19,24,29,34,39; 0; 1; 6; 2. 25; A1; 2; 1; 4,9,14,19,24,29,34,39; 0; 1; 6; 2. 26; A1; 1; 0; 13,14,15, 29,30,31,37,38,39; 7; 2; 3; 2. 27; A1; 8; 1,2; 9,19,29,39; 0; 2; 6; 2. 28; A1; 2; 1; 7,15,23,31,39; 0; 2; 6; 2. 0; A2; 1; 0; 4,9,14,19,24,29,34,39; 0; 1; 3; 4. 1; A2; 1; 0; 4,9,14,19,24,29,34,39; 5; 1; 2; 4. 2; A2; 1; 0; 9,19,29,39; 0; 2; 3; 4. 3; A2; 1; 0; 24,29,34,39; 5; 1; 2; 4. 4; A2; 1; 0; 17,19,37,39; 0; 1; 3; 4. 5; A2; 1; 0; 19,39; 5; 1; 2; 4. 6; A2; 1; 0; 9,19,29,39; 5; 2; 2; 4. 7; A2; 1; 0; 0,1,2,…,39; 5; 1; 2; 4. 8; A2; 1; 0; 23,27,31,35,39; 0; 1; 3; 4. 9; A2; 1; 0; 23,27,31,35,39; 5; 1; 2; 4. 10; A2; 1; 0; 3,7,11,15,19,23,27,31,35,39; 0; 1; 3; 4. 11; A2; 1; 0; 3,7,11,15,19,23,27,31,35,39; 5; 1; 2; 4. 12; A2; 1; 0; 1,3,5,7,…,37,39; 0; 1; 3; 4. 13; A2; 1; 0; 3,5,7; 0; 1; 3; 4. 14; A2; 1; 0; 3,5,7,9,11,13; 5; 1; 2; 4. 15; A2; 16; 1; 4,9,14,19,24,29,34,39; 0; 2; 3; 4. 16; A2; 16; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 3; 4. 17; A2; 8; 1; 4,9,14,19,24,29,34,39; 0; 2; 3; 4. 18; A2; 8; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 3; 4. 19; A2; 4; 1; 4,9,14,19,24,29,34,39; 0; 2; 3; 4. 20; A2; 4; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 3; 4. 21; A2; 2; 1; 4,9,14,19,24,29,34,39; 0; 2; 3; 4. 22; A2; 2; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 3; 4. 23; A2; 1; 0; 7,15,23,31,39; 0; 1; 3; 4. 24; A2; 4; 1; 4,9,14,19,24,29,34,39; 0; 1; 3; 4. 25; A2; 2; 1; 4,9,14,19,24,29,34,39; 0; 1; 3; 4. 26; A2; 1; 0; 13,14,15, 29,30,31,37,38,39; 5; 2; 2; 4. 27; A2; 8; 1,2; 9,19,29,39; 0; 2; 3; 4. 28; A2; 2; 1; 7,15,23,31,39; 0; 2; 3; 4. 0; A3; 1; 0; 4,9,14,19,24,29,34,39; 0; 1; 2; 6. 1; A3; 1; 0; 4,9,14,19,24,29,34,39; 7; 1; 1; 6. 2; A3; 1; 0; 9,19,29,39; 0; 2; 2; 6. 3; A3; 1; 0; 24,29,34,39; 7; 1; 1; 6. 4; A3; 1; 0; 17,19,37,39; 0; 1; 2; 6. 5; A3; 1; 0; 19,39; 7; 1; 1; 6. 6; A3; 1; 0; 9,19,29,39; 7; 2; 1; 6. 7; A3; 1; 0; 0,1,2,…,39; 7; 1; 1; 6. 8; A3; 1; 0; 23,27,31,35,39; 0; 1; 2; 6. 9; A3; 1; 0; 23,27,31,35,39; 7; 1; 1; 6. 10; A3; 1; 0; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 11; A3; 1; 0; 3,7,11,15,19,23,27,31,35,39; 7; 1; 1; 6. 12; A3; 1; 0; 1,3,5,7,…,37,39; 0; 1; 2; 6. 13; A3; 1; 0; 3,5,7; 0; 1; 2; 6. 14; A3; 1; 0; 3,5,7,9,11,13; 7; 1; 1; 6. 15; A3; 16; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 16; A3; 16; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 17; A3; 8; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 18; A3; 8; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 19; A3; 4; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 20; A3; 4; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 21; A3; 2; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 22; A3; 2; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 23; A3; 1; 0; 7,15,23,31,39; 0; 1; 2; 6. 24; A3; 4; 1; 4,9,14,19,24,29,34,39; 0; 1; 2; 6. 25; A3; 2; 1; 4,9,14,19,24,29,34,39; 0; 1; 2; 6. 26; A3; 1; 0; 13,14,15, 29,30,31,37,38,39; 7; 2; 1; 6. 27; A3; 8; 1,2; 9,19,29,39; 0; 2; 2; 6. CMCC; A3; 1; 0; 7,15,23,31,39 or 9,11,13; 2; 1; 2; 6. 0; B1; 1; 0; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 1; B1; 1; 0; 4,9,14,19,24,29,34,39; 8; 1; 3; 2. 2; B1; 1; 0; 9,19,29,39; 2; 2; 6; 2. 3; B1; 1; 0; 24,29,34,39; 8; 1; 3; 2. 4; B1; 1; 0; 17,19,37,39; 2; 1; 6; 2. 5; B1; 1; 0; 19,39; 8; 1; 3; 2. 6; B1; 1; 0; 9,19,29,39; 8; 2; 3; 2. 7; B1; 1; 0; 0,1,2,…,39; 8; 1; 3; 2. 8; B1; 1; 0; 23,27,31,35,39; 2; 1; 6; 2. 9; B1; 1; 0; 23,27,31,35,39; 8; 1; 3; 2. 10; B1; 1; 0; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 11; B1; 1; 0; 3,7,11,15,19,23,27,31,35,39; 8; 1; 3; 2. 12; B1; 1; 0; 1,3,5,7,…,37,39; 2; 1; 6; 2. 13; B1; 1; 0; 3,5,7; 2; 1; 6; 2. 14; B1; 1; 0; 3,5,7,9,11,13; 8; 1; 3; 2. 15; B1; 16; 1; 4,9,14,19,24,29,34,39; 2; 2; 6; 2. 16; B1; 16; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 17; B1; 8; 1; 4,9,14,19,24,29,34,39; 2; 2; 6; 2. 18; B1; 8; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 19; B1; 4; 1; 4,9,14,19,24,29,34,39; 2; 2; 6; 2. 20; B1; 4; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 21; B1; 2; 1; 4,9,14,19,24,29,34,39; 2; 2; 6; 2. 22; B1; 2; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 23; B1; 1; 0; 7,15,23,31,39; 2; 1; 6; 2. 24; B1; 4; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 25; B1; 2; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 26; B1; 1; 0; 13,14,15, 29,30,31,37,38,39; 8; 2; 3; 2. 27; B1; 8; 1,2; 9,19,29,39; 2; 2; 6; 2. 0; B4; 1; 0; 4,9,14,19,24,29,34,39; 0; 1; 1; 12. 1; B4; 1; 0; 4,9,14,19,24,29,34,39; 2; 1 or 2; 1; 12. 2; B4; 1; 0; 9,19,29,39; 0; 2; 1; 12. 3; B4; 1; 0; 24,29,34,39; 2; 1; 1; 12. 4; B4; 1; 0; 17,19,37,39; 0; 1; 1; 12. 5; B4; 1; 0; 19,39; 2; 1 or 2; 1; 12. 6; B4; 1; 0; 9,19,29,39; 2; 2; 1; 12. 7; B4; 1; 0; 0,1,2,…,39; 2; 1; 1; 12. 8; B4; 1; 0; 23,27,31,35,39; 0; 1; 1; 12. 9; B4; 1; 0; 23,27,31,35,39; 2; 1 or 2; 1; 12. 10; B4; 1; 0; 3,7,11,15,19,23,27,31,35,39; 0; 1; 1; 12. 11; B4; 1; 0; 3,7,11,15,19,23,27,31,35,39; 2; 1 or 2; 1; 12. 12; B4; 1; 0; 1,3,5,7,…,37,39; 0; 1; 1; 12. 13; B4; 1; 0; 3,5,7; 0; 1; 1; 12. 14; B4; 1; 0; 3,5,7,9,11,13; 2; 1; 1; 12. 15; B4; 16; 1,2; 4,9,14,19,24,29,34,39; 0; 2; 1; 12. FO; B4; 16; 1,2; 3,7,11,15,19,23,27,31,35,39; 0; 1; 1; 12. 17; B4; 8; 1,2; 4,9,14,19,24,29,34,39; 0; 2; 1; 12. 18; B4; 8; 1,2; 3,7,11,15,19,23,27,31,35,39; 0; 1; 1; 12. 19; B4; 4; 1; 4,9,14,19,24,29,34,39; 0; 2; 1; 12. 20; B4; 4; 1,2; 3,7,11,15,19,23,27,31,35,39; 0; 1; 1; 12. 21; B4; 2; 1; 4,9,14,19,24,29,34,39; 0; 2; 1; 12. 22; B4; 2; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 1; 12. 23; B4; 1; 0; 7,15,23,31,39; 0; 1 or 2; 1; 12. 24; B4; 4; 1; 4,9,14,19,24,29,34,39; 0; 1; 1; 12. 25; B4; 2; 1; 4,9,14,19,24,29,34,39; 0; 1; 1; 12. 26; B4; 1; 0; 13,14,15, 29,30,31,37,38,39; 2; 2; 1; 12. 27; B4; 8; 1,2; 9,19,29,39; 0; 2; 1; 12. 28; B4; 2; 1; 7,15,23,31,39; 2; 2; 1; 12. 0; C0; 1; 0; 4,9,14,19,24,29,34,39; 0; 1; 7; 2. 1; C0; 1; 0; 4,9,14,19,24,29,34,39; 8; 1; 3; 2. 2; C0; 1; 0; 9,19,29,39; 0; 2; 7; 2. 3; C0; 1; 0; 24,29,34,39; 8; 1; 3; 2. 4; C0; 1; 0; 17,19,37,39; 0; 1; 7; 2. 5; C0; 1; 0; 19,39; 8; 1; 3; 2. 6; C0; 1; 0; 9,19,29,39; 8; 2; 3; 2. 7; C0; 1; 0; 0,1,2,…,39; 8; 1; 3; 2. 8; C0; 1; 0; 23,27,31,35,39; 0; 1; 7; 2. 9; C0; 1; 0; 23,27,31,35,39; 8; 1; 3; 2. 10; C0; 1; 0; 3,7,11,15,19,23,27,31,35,39; 0; 1; 7; 2. 11; C0; 1; 0; 3,7,11,15,19,23,27,31,35,39; 8; 1; 3; 2. 12; C0; 1; 0; 1,3,5,7,…,37,39; 0; 1; 7; 2. 13; C0; 1; 0; 3,5,7; 0; 1; 7; 2. 14; C0; 1; 0; 3,5,7,9,11,13; 8; 1; 3; 2. 15; C0; 16; 1; 4,9,14,19,24,29,34,39; 0; 1 or 2; 7; 2. EF; C0; 16; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 7; 2. 17; C0; 8; 1; 4,9,14,19,24,29,34,39; 0; 1 or 2; 7; 2. 18; C0; 8; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 7; 2. 19; C0; 4; 1; 4,9,14,19,24,29,34,39; 0; 2; 7; 2. 20; C0; 4; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 7; 2. 21; C0; 2; 1; 4,9,14,19,24,29,34,39; 0; 2; 7; 2. 22; C0; 2; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 7; 2. 23; C0; 1; 0; 7,15,23,31,39; 0; 1; 7; 2. 24; C0; 4; 1; 4,9,14,19,24,29,34,39; 0; 1; 7; 2. 25; C0; 2; 1; 4,9,14,19,24,29,34,39; 0; 1; 7; 2. 26; C0; 1; 0; 13,14,15, 29,30,31,37,38,39; 8; 2; 3; 2. 27; C0; 8; 1,2; 9,19,29,39; 0; 2; 7; 2. 28; C0; 2; 1; 7,15,23,31,39; 0; 2; 7; 2. 0; C2; 1; 0; 4,9,14,19,24,29,34,39; 2; 1; 2; 6. 1; C2; 1; 0; 4,9,14,19,24,29,34,39; 7; 1 or 2; 1; 6. 2; C2; 1; 0; 9,19,29,39; 2; 2; 2; 6. 3; C2; 1; 0; 24,29,34,39; 7; 1; 1; 6. 4; C2; 1; 0; 17,19,37,39; 0; 1; 2; 6. 5; C2; 1; 0; 19,39; 2; 1; 2; 6. 6; C2; 1; 0; 9,19,29,39; 7; 2; 1; 6. 7; C2; 1; 0; 0,1,2,…,39; 7; 1; 1; 6. JF; C2; 1; 0; 23,27,31,35,39; 0; 1; 2; 6. 9; C2; 1; 0; 23,27,31,35,39; 7; 1 or 2; 1; 6. 10; C2; 1; 0; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 11; C2; 1; 0; 3,7,11,15,19,23,27,31,35,39; 7; 1 or 2; 1; 6. 12; C2; 1; 0; 1,3,5,7,…,37,39; 0; 1; 2; 6. 13; C2; 1; 0; 3,5,7; 0; 1; 2; 6. 14; C2; 1; 0; 3,5,7,9,11,13; 7; 1; 1; 6. 15; C2; 16; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 16; C2; 16; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 17; C2; 8; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 18; C2; 8; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 19; C2; 4; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 20; C2; 4; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 21; C2; 2; 1; 4,9,14,19,24,29,34,39; 0; 2; 2; 6. 22; C2; 2; 1; 3,7,11,15,19,23,27,31,35,39; 0; 1; 2; 6. 23; C2; 1; 0; 7,15,23,31,39; 2; 1 or 2; 2; 6. 24; C2; 4; 1; 4,9,14,19,24,29,34,39; 0; 1; 2; 6. 25; C2; 2; 1; 4,9,14,19,24,29,34,39; 0; 1; 2; 6. 26; C2; 1; 0; 13,14,15, 29,30,31,37,38,39; 7; 2; 1; 6. 27; C2; 8; 1,2; 9,19,29,39; 0; 2; 2; 6. 28; C2; 2; 1; 7,15,23,31,39; 2; 2; 2; 6. 0; A1/B1; 1; 0; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. HK; A1/B1; 1; 0; 4,9,14,19,24,29,34,39; 8; 1; 3; 2. 2; A1/B1; 1; 0; 9,19,29,39; 2; 2; 6; 2. 3; A1/B1; 1; 0; 9,19,29,39; 8; 1; 3; 2. 4; A1/B1; 1; 0; 17,19,37,39; 2; 1; 6; 2. 5; A1/B1; 1; 0; 19,39; 8; 1; 3; 2. 6; A1/B1; 1; 0; 23,27,31,35,39; 2; 1; 6; 2. 7; A1/B1; 1; 0; 23,27,31,35,39; 8; 1; 3; 2. 8; A1/B1; 1; 0; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 9; A1/B1; 1; 0; 1,3,5,7,…,37,39; 2; 1; 6; 2. 10; A1/B1; 1; 0; 7,15,23,31,39; 2; 1; 6; 2. 11; A1/B1; 2; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 12; A1/B1; 4; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 13; A1/B1; 4; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 14; A1/B1; 8; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 15; A1/B1; 8; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 16; A1/B1; 16; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 17; A1/B1; 16; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 0; A2/B2; 1; 0; 4,9,14,19,24,29,34,39; 2; 1; 3; 4. 1; A2/B2; 1; 0; 4,9,14,19,24,29,34,39; 6; 1; 2; 4. 2; A2/B2; 1; 0; 9,19,29,39; 2; 2; 3; 4. 3; A2/B2; 1; 0; 9,19,29,39; 6; 1; 2; 4. 4; A2/B2; 1; 0; 17,19,37,39; 2; 1; 3; 4. 5; A2/B2; 1; 0; 19,39; 6; 1; 2; 4. 6; A2/B2; 1; 0; 23,27,31,35,39; 2; 1; 3; 4. 7; A2/B2; 1; 0; 23,27,31,35,39; 6; 1; 2; 4. 8; A2/B2; 1; 0; 3,7,11,15,19,23,27,31,35,39; 2; 1; 3; 4. 9; A2/B2; 1; 0; 1,3,5,7,…,37,39; 2; 1; 3; 4. 10; A2/B2; 1; 0; 7,15,23,31,39; 2; 1; 3; 4. 11; A2/B2; 2; 1; 4,9,14,19,24,29,34,39; 2; 1; 3; 4. 12; A2/B2; 4; 1; 4,9,14,19,24,29,34,39; 2; 1; 3; 4. 13; A2/B2; 4; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 3; 4. 14; A2/B2; 8; 1; 4,9,14,19,24,29,34,39; 2; 1; 3; 4. 15; A2/B2; 8; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 3; 4. 16; A2/B2; 16; 1; 4,9,14,19,24,29,34,39; 2; 1; 3; 4. 17; A2/B2; 16; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 3; 4. 0; A3/B3; 1; 0; 4,9,14,19,24,29,34,39; 2; 1; 2; 6. 1; A3/B3; 1; 0; 4,9,14,19,24,29,34,39; 2; 2; 2; 6. 2; A3/B3; 1; 0; 9,19,29,39; 2; 2; 2; 6. 3; A3/B3; 1; 0; 9,19,29,39; 2; 1; 2; 6. 4; A3/B3; 1; 0; 17,19,37,39; 2; 1; 2; 6. 5; A3/B3; 1; 0; 19,39; 2; 1; 2; 6. 6; A3/B3; 1; 0; 23,27,31,35,39; 2; 1; 2; 6. 7; A3/B3; 1; 0; 23,27,31,35,39; 2; 2; 2; 6. 8; A3/B3; 1; 0; 3,7,11,15,19,23,27,31,35,39; 2; 1; 2; 6. 9; A3/B3; 1; 0; 1,3,5,7,…,37,39; 2; 1; 2; 6. 10; A3/B3; 1; 0; 7,15,23,31,39; 2; 1; 2; 6. 11; A3/B3; 2; 1; 4,9,14,19,24,29,34,39; 2; 1; 2; 6. 12; A3/B3; 4; 1; 4,9,14,19,24,29,34,39; 2; 1; 2; 6. 13; A3/B3; 4; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 2; 6. 14; A3/B3; 8; 1; 4,9,14,19,24,29,34,39; 2; 1; 2; 6. 15; A3/B3; 8; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 2; 6. 16; A3/B3; 16; 1; 4,9,14,19,24,29,34,39; 2; 1; 2; 6. 17; A3/B3; 16; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 2; 6. =============== End Text proposal for 38.211, section 6.3.3 ============================= Agreements: - To adopt the following TP to 6.3.3.1 of 38.211 There are 64 preambles defined in each time-frequency PRACH occasion, enumerated in increasing order of first increasing cyclic shift of a logical root sequence, and then in increasing order of the logical root sequence index, starting with the index obtained from the higher-layer parameter PRACHRootSequenceIndex. Additional preamble sequences, in case 64 preambles cannot be generated from a single root Zadoff-Chu sequence, are obtained from the root sequences with the consecutive logical indexes until all the 64 sequences are found. The logical root sequence order is cyclic: the logical index 0 is consecutive to 837 when and is consecutive to 137 when . The sequence number is obtained from the logical root sequence index according to Tables 6.3.3.1-3 and 6.3.3.1-4. Agreements: - For both FR1 and FR2, the previous working assumption is replaced with the following working assumption - Note: The value with yellow background is not final - For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is more than at least N symbols after DL part and/or the last symbol of an SSB. - N = 0 for Msg1 SCS = 1.25kHz, 5kHz - N = 2 for Msg1 SCS = 15kHz - N = 2 for Msg1 SCS = 30kHz - N = 2 for Msg1 SCS = 60kHz - N = 2 or 3 for Msg1 SCS = 120kHz - Note for format B4, N = 0 irrespectively of Msg1 SCS - UE is not expected to receive DL signals during any valid RACH occasion - The SSB to RO association pattern period shall repeat at most every 160 ms - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid. - Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured - FFS how to handle the gap necessary in between SS/PBCH block or DL part and PRACH tx Agreements: - Confirm the following working assumption: - Minimum time gap between the end of PDSCH for Msg4 and the start of HARQ-ACK equals to the duration of N1 + L2. - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - L2=500 usec refers to the MAC layer processing time - Note: the reason for working assumption is due to potential concern over satisfying ITU requirement (to be checked) Agreements: - For the parameter new, new =250us Agreements: Modify the previous agreement with following updates: - For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is - N2+BWPSwitching+ Delay, - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4 - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - N2 is dependent on the SCS of Msg1 - Working assumption: For Msg1 with 1.25 kHz or 5 kHz SCS, calculation of N2 uses 15 kHz SCS. - Delay includes at least MAC layer delay in initializing PRACH - Value of Delay is FFS (to be decided in RAN1) Delay = 250 us for FR2, Delay = 500 us for FR1 Agreements: UE does not simultaneously transmit PRACH and PUSCH/PUCCH/SRS in, at least, single CC and in intra-band CA, during any of the following scenarios: - Same slot - When the gap between the end of PRACH (PUSCH/PUCCH/SRS) and the start of PUSCH/PUCCH/SRS (PRACH) is less than N symbols - N = 2 for 15 kHz and 30 kHz SCS. - N = 4 for 60 kHz and 120 kHz SCS. - Working assumption: Reference SCS for determining N is the SCS for UL BWP. - FFS: inter-band CA. - Transmission of PRACH or SR (on PUCCH) if any, is up to UE implementation. To draft reply LS based on the above agreements in R1-1805713, which is approved, final LS in R1-1805759 Agreements: - To adopt the following TP to 8.1 of 38.213: […] The UE is provided a number of contention based preambles per SS/PBCH block per PRACH occasion by the value of higher layer parameter cb-preamblePerSSB and the UE determines a total number of preambles per SSB per PRACH occasion as the multiple of the value of SSB-perRACH-Occasion and the value of cb-preamblePerSSB. Agreements: - For the case that N SSBs are associated with one RACH occasion, where N >= 1, the subset of cb-preamblePerSSB consecutive CBRA preambles associated with SSB i (i=0, …, N-1) starts from preamble index i*(64/N). Working assumption: For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where - N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 - N2 also refers to the value determined in control session with UE capability #1 - L2 refers to the MAC processing latency and is equal to 0.5 msec. - TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS - Note: Spec captures the value k for different SCS - UE is not expected that UL transmissions in the same slot shall apply different TA values. - If this happens, it is up to UE’s implementation. • FFS: the reference SCS of slot n+k and the time duration of (k-1) slots when UL and DL have different SCSs. Agreements: - The parameter cb-preamblePerSSB indicates the number of CBRA preambles per SSB per RACH occasion. Agreements: Change the previous working assumption: - Time period for SSB to RO association is multiple of PRACH configuration periods - The number of multiple PRACH configuration periods in the time period is the smallest value in the set {1, 2, 4} such that the number is ≥ Actually transmitted SSBs / SSBs that can be mapped to one PRACH config period - The time period starts from frame 0 And replace it with the following: - Time period for SSB to RO association is multiple of PRACH configuration periods - The number of multiple PRACH configuration periods in the time period is the smallest value in the set determined by the RACH configuration period from following table such that the number is ≥ Actually transmitted SSBs / SSBs that can be mapped to one PRACH config period such that all actually transmitted SSBs can be mapped, at least once, to the ROs within the time period. - The time period starts from frame 0 RACH configuration period (ms); Mapping period set (# of RACH configuration period ). 10; {1，2，4，8，16}. 20; {1，2，4，8}. 40; {1，2，4}. 80; {1，2}. 160; {1}. Agreements: Update the previous agreement on Msg1 retransmission as follows: - If a received Msg2 does not contain a response to the transmitted preamble sequence, or no RAR is received by the end of the RAR window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence after no later than the duration of N1 + new + L2. - N1 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - new = 250 us - L2=500 usec refers to the MAC layer processing time - Note: UE is not mandated to measure each SSB prior to every Msg1 retransmission Agreements: Update the previous agreement in the following way: - From UE perspective, minimum time gap between Msg2 over-the-air reception and Msg3 transmission if Msg2 and Msg3 have the same SCS - Duration of N1 + duration of N2 + L2 + N_TA + TA - N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 - N2 also refers to the value determined in control session with UE capability #1 - TA is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the SCS of Msg3 - L2 refers to the MAC processing latency and it does not depend on subcarrier spacing - L2=500us as a working assumption - Note: If Msg2 and Msg3 have different SCS, value of N1 and N2 will refer to the ones determined in control session. - Note: If Msg2 and Msg3 have different SCS, value of N1 and N2 – in terms of absolute time - are determined by lower SCS between Msg2 and Msg3 - Note: Network schedules Msg3 while ensuring the minimum time gap and considering the TA command conveyed in Msg2 LS R1-1805657 is agreed Agreements: - PDCCH order for RACH procedure includes the following fields: - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure - For CFRA only: - FFS BWP index. Indicating which BWP to transmit the Random access preamble on - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions - RACH occasion index - 3 bits are used to indicate the relative RACH occasion index that corresponds to the indicated SSB index identified group of RACH occasions Agreements: - To adopt the following TP for 38.215 Definition; CSI reference signal received power (CSI-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry CSI reference signals configured for RSRP measurements within the considered measurement frequency bandwidth in the configured CSI-RS occasions. For CSI-RSRP determination CSI reference signals transmitted on antenna port 3000 or antenna ports 3000, 3001 according to 3GPP TS 38.211 [4] shall be used. If CSI-RSRP is used for L1-RSRP, CSI reference signals transmitted on antenna ports 3000, 3001 can be used for CSI-RSRP determination. For frequency range 1, the reference point for the CSI-RSRP shall be the antenna connector of the UE. For frequency range 2, CSI-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported CSI-RSRP value shall not be lower than the corresponding CSI-RSRP of any of the individual receiver branches.. Applicable for; If CSI-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency. Agreements: Send an LS to RAN4 – R1-1805649, which is approved with final LS in R1-1805760 - RAN1 has discussed following proposals regarding Rx beam selection for RRM measurements in the context of requiring the UE to provide more stable measurements and request that RAN4 take the discussion into account in their work. - Measurement to be reported is the best among the measurements based on each RX beam in the selected set - Measurement to be reported shall be greater than average of measurements based on each RX beam in the selected set - The selection of Rx beam set to perform measurement on carrier is left to the UE implementation with the limitation that the same Rx beam set is used to measure the same carrier - RAN1 believes the selection and/or definition of the Rx beam for measurement should be determined by RAN4, and have not concluded on a specific proposal. - The following are related agreement from RAN1 2017 Adhoc #3: Agreements: - For a SS-RSRQ measurement, the same RX beam shall be applied between RSSI measurement and RSRP measurement - For a CSI-RSRQ measurement, the same RX beam shall be applied between RSSI measurement and RSRP measurement - It is up to UE implementation how to select a set of RX beams to perform RRM measurement on a carrier - Different sets of RX beams can be used in measurements based on different measurement objects - Same set of RX beams shall be used in measurement of each TX beam based on a measurement object - FFS - Alt.1: Measurement to be reported shall be greater than average of measurements based on each RX beam in the selected set - Alt.2: Measurement to be reported shall be the best among measurements based on each RX beam in the selected set - Other alternatives are not precluded. Agreements: - NR does not support different measurement BWs for SS-RSRP and SS-RSSI for Rel-15. Agreements: - For reception of SSB for the target PSCell, frequency indication will be required for SSB not on the synchronization signal raster. - Send LS to RAN2 (R1-1805651, which is approved and final LS in R1-1805717) and ask RAN2 to provide feedback if any Conclusion: - It is up to RAN4 to define associated UE behavior in context of reception of PUSCH/PDSCH during measurements of CSI-RS resource with same/different numerology with PUSCH/PDSCH. Agreements: - For intra-frequency measurement and if measurement gap is not configured, UE is not expected to measure the CSI-RS resources outside the active DL BWP. Conclusion: - It is RAN1’s understandings that UE, which does not support simultaneous reception of multiple numerologies, requires measurement gap configuration in order to perform measurements of the CSI-RS for L3 mobility for intra-frequency measurements when the numerology of CSI-RS resource is different from the active DL BWP numerology. Agreements: - For FR1: support alt 1 with total maximum number of CSI-RS for L3 mobility as 64. - For FR2: Support alt 2. - Alt 1 & Alt 2 - Alt 1) NR supports configuration of mixture of CSI-RS resources with associated SSB and without associated SSB in the same measurement object (e.g. same carrier frequency) - Alt 2) NR does not supports configuration of mixture of CSI-RS resources with associated SSB and without associated SSB in the same measurement object (e.g. same carrier frequency) Agreements: - For intra-frequency measurements, SS-RSSI configured by higher layers is measured with timing reference from serving cell in the frequency layer. - For inter-frequency measurements, SS-RSSI configured by higher layers is measured with timing reference from any one of the cells in the target frequency layer Agreements: - Send Reply LS to RAN2 and inform that agreement 5 of R1-1805264 is not aligned with RAN1 understanding and therefore should be reconsidered. - It is RAN1 understanding that useServingCellTimingForSync parameter is only applicable for SSB based RRM measurements and should not be coupled with configuration of CSI-RS based RRM measurements. - Additionally, the configuration of SSB based RRM measurement should not be a condition for configuring CSI-RS based RRM measurement with associated SSB. - In case assocatedSSB is configured for CSI-RS based RRM measurement, frequency information of the SSB (to be associated with CSI-RS) is required. For example, ssbFrequency in MO. The draft LS is approved with the updates of fixing the typo “assocated”, & “agreement 5 in R2-1804103”, final LS in R1-1805762 Agreements: - To reply the LS from RAN2 in R1-1803577 Answer 1: - The maximum number of BFD-RS(s) is 2 per BWP. - The maximum number of RLM-RS(s) and BFD-RS(s) should depend on whether same RS(s) is shared between RLM and BFD. The maximum number of unique RS(s), each RS using different set of resources, for both RLM-RS(s) and BFD-RS(s) are: - X RS(s) per BWP for below 3 GHz, - X=2(working assumption) - Y RS(s) per BWP for above 3 GHz and below 6 GHz, - Y=6 (working assumption) - Z RS(s) per BWP for above 6 GHz, - Z=8(working assumption) - where maximum number of BFD-RS(s) is 2 RSs per BWP and maximum number of RLM-RS(s) is 2 RSs per BWP for below 3 GHz, 4 RSs per BWP for above 3 GHz and below 6 GHz, 8 RSs per BWP for above 6 GHz. - Please note that support of 8 RLM-RSs and 2 BFD-RS for above 6 GHz is feasible if the 2 BFD-RS are a subset of 8 RLM-RSs. Answer 2: - Yes. They can be completely orthogonal, depending on NW configuration. Draft reply LS to x3577 in R1-1805718, which is approved with final LS in R1-1805761 Agreements: - Further clarification of RAN2 agreement (will require additional physical layer text proposals): - Working assumption: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM - FFS on the UE behavior when TCI-states indicate a combination of SSB, CSI-RS, and CSI-RS for tracking Conclusion: - It is RAN1 understanding that each CSI-RS resource can be configured with QCL-info independently, and in general UE may not assume that different reference signals have QCL relationship unless configured. No further clarification is needed on the QCL assumption for RLM-RSs. Agreements: - For TDD+FDD CA, in a TAG without PCell or PSCell, UE should adjust its uplink transmission timing of all SCells in the TAG using the largest TA offset value within the TAG

MIMO

Agreement Text proposal for TS38.211 section 6.3.1.1 - equals the higher-layer parameter Data-scrambling-Identity if configured and the RNTI equals the C-RNTI or CS-RNTI, Agreement Text proposal for TS38.211 section 7.3.1.1 - equals the higher-layer parameter Data-scrambling-Identity if configured and the RNTI equals the C-RNTI or CS-RNTI Agreement Text proposal for Section 7.3.1.2.2 of 38.212 ------------------------------------------- Start of Text Proposal ------------------------------------------- 7.3.1.2.2 Format 1_1 < Unchanged parts are omitted > - Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4. < Unchanged parts are omitted > Agreement For TB to CW mapping: - 2-TB: TB1 CW0, TB2 CW1 - 1-TB: always mapped to CW0 Section 5.1.3.2 of TS38.214 In case the higher layer parameter Number-MCS-HARQ-DL-DCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS=26 and if rvid=1 for the corresponding transport block, otherwise the transport block is enabled. For the PDSCH assigned by a PDCCH with DCI format 1_0/1_1 with CRC scrambled by C-RNTI, if the higher layer parameter MCS-Table-PDSCH is set to '256QAM' is configured and , or the higher layer parameter MCS-Table-PDSCH is not set to '256QAM' configured and , the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below: Agreement Adopt the following TP. /************************* Start of Text Proposal for 38.212 **************************/ 7.3.1.1.2 Format 0_1 - SRS resource indicator – or bits, where is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'CodeBook' or 'NonCodeBook', and is the maximum number of supported layers for the PUSCH. - bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter ulTxConfig = NonCodeBook, where is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'NonCodeBook'; - bits according to Tables 7.3.1.1.2-32 if the higher layer parameter ulTxConfig = CodeBook, where is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter SRS-SetUse of value 'CodeBook'. - Precoding information and number of layers – number of bits determined by the following: - 0 bits if the higher layer parameter ulTxConfig = NonCodeBook; - 0 bits for 1 antenna port and if the higher layer parameter ulTxConfig = Codebook; - 4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters PUSCH-tp, ULmaxRank, and ULCodebookSubset; - 2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters PUSCH-tp, ULmaxRank, and ULCodebookSubset; - 2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters ULmaxRank and ULCodebookSubset; - 1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if ulTxConfig = Codebook, and according to the values of higher layer parameters ULmaxRank and ULCodebookSubset. - Antenna ports – number of bits determined by the following - 2 bits as defined by Tables 7.3.1.1.2-6, if PUSCH-tp=Enabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=1; - 4 bits as defined by Tables 7.3.1.1.2-7, if PUSCH-tp=Enabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=2; - 3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if PUSCH-tp=Disabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook; - 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if PUSCH-tp=Disabled, UL-DMRS-config-type=1, and UL-DMRS-max-len=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook; - 4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if PUSCH-tp=Disabled, UL-DMRS-config-type=2, and UL-DMRS-max-len=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook; - 5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if PUSCH-tp=Disabled, UL-DMRS-config-type=2, and UL-DMRS-max-len=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter ulTxConfig = NonCodeBook and according to the Precoding information and number of layers field if the higher layer parameter ulTxConfig = CodeBook /************************* End of Text Proposal for 38.212 **************************/ Agreement Text alignment between section 6.1.2.3 of 38214 and the UL MIMO parts in section 6.1.1 in terminology. Remove references to TPMI, TRI, etc, and use the DCI field names from 38.212 as section 6.1.2.3 does now. 38.214 editor to make necessary changes. Agreement Text proposal for TS38.214 Section 6.1.1.2 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI field from the DCI. The UE shall use one or multiple SRS resources for SRS transmission, where the number of SRS resources which can be configured to the UE for simultaneously transmission in the same RBs is being part of UE capability signalling Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter srsSetUse set to ‘nonCodebook’. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n. For non-codebook based transmission, the UE can measure NZP CSI-RS resource to calculate the precoder used for the transmission of precoded SRS. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set. - If aperiodic SRS resource is configured, the [CSI-RS information in the same slot TBD] for UL channel measurement is indicated via DCI, where the association among aperiodic SRS triggering state, triggered SRS resource(s) SRS-ResourceConfigId, CSI-RS resource ID NZP-CSI-RS-ResourceConfigID are higher layer configured by AperiodicSRS-ResourceTrigger. A UE may receive the dynamic SRS transmission request for aperiodic SRS transmission in the same slot as the reception of the DL CSI-RS resource. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the AP-CSI-RS resource and the first symbol of the AP-SRS transmission is less than 42 OFDM symbols. A UE may receive the SRI which shall be associated with the most recent SRS transmission. - If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceConfigID for measurement is indicated via higher layer parameter SRS-AssocCSIRS per set. Agreement When the UE is configured with AP-SRS and AP-CSI-RS associated with the AP-SRS for non-codebook based UL transmission, - Any of the TCI states in the scheduled CC should not be configured with QCL Type D. - AP-CSI-RS is located in the same slot as the AP-SRS trigger/request. - The presence of AP-CSI-RS is (implicitly) indicated via the SRS request field in the DCI. - The QCL assumption for receiving the associated AP-CSI-RS for non-codebook-based transmission is up to UE implementation - Note: This does not preclude the use of the CSI request field to trigger this AP CSI-RS for DL CSI acquisition Agreement In case UE is scheduled by DCI 1_0, the higher layer parameter prb-BundlingType is set to 'static', and when receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, - the value of is fixed as 2 PRBs. Agreement Text Proposal for TS 38.214 5.1.2.3 Physical resource block (PRB) bundling < Unchanged parts are omitted > A UE may assume that precoding granularity is consecutive resource blocks in the frequency domain. can be equal to one of the values among {2, 4, wideband}. If is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the carrier bandwidth part i with consecutive PRBs can be equal to one of the values among {2, 4, wideband}. Actual number of consecutive PRBs in each PRG could be one or more. for each carrier bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that is equal to 2 PRBs. If is determined as "wideband", the UE is not expected to be scheduled with non-contiguous resource allocation and the UE may assume that the same precoding is applied to the allocated resource. When is determined as one of the values among {2, 4}, the first PRG size is given by and the last PRG size given by if , and the last PRG size is if . The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above. For PDSCH carrying SIB1 scheduled by PDCCH with CRC scrambled by SI-RNTI, PRG is partitioned from the lowest numbered resource block of the CORESET signalled in PBCH. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, for each carrier bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that is equal to 2 PRBs. In case UE is scheduled by DCI 1_0, the higher layer parameter prb-BundlingType is set to 'static', and when receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, the value of is fixed as 2 PRBs. If the higher layer parameter prb-BundlingType is set to 'dynamic', and the UE is scheduled a PDSCH with DCI format 1_0 scrambled with C-RNTI and CS-RNTI, then the UE shall assume that is equal to 2 PRBs. If the higher layer parameter prb-BundlingType is set to 'dynamic', the higher layer parameter bundleSizeSet1 and bundleSizeSet2 configures two sets of values, the first set includes one or two values among {2, 4, wideband}, and the second set includes one value. The UE is not expected to be configured with (2, 4) in the first set. If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] - is set to '0', the UE shall use the value from the second set of PRG values when receiving PDSCH scheduled by the same DCI. - is set to '1' and one value is configured for the first set of values, the UE shall use this value when receiving PDSCH scheduled by the same DCI - is set to '1' and two values are configured for the first set of values as ‘n2-wideband’ (corresponding to two values 2 and wideband) or ‘n4-wideband’ (corresponding to two values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise is set to the remaining configured value of 2 or 4, respectively. If the higher layer parameter prb-BundlingType is set to 'static', the value is configured with the single value indicated by the higher layer parameter bundleSize. When a UE is configured with RBG = 2 for bandwidth part i according to Section 5.1.2.2.1, or when a UE is configured with resource block bundle size of 2 for VRB to PRB mapping provided by the higher layer parameter VRB-to-PRB-interleaver for bandwidth part i, the UE is not expected to be configured with = 4. < Unchanged parts are omitted > Agreement The following text proposal for TS38.212 section 7.3.1.2.2 Format 1_1 < Unchanged parts are omitted > - PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType PRB_bundling is not configured or is set to ‘static’, or 1 bit if the higher layer parameter prb-BundlingType PRB_bundling is set to ‘dynamic’, according to Subclause 5.1.2.3 of [6, TS38.214]. < Unchanged parts are omitted > Agreement: Support 3ms as MAC CE latency for activation/deactivation of SP CSI on PUCCH, SP CSI-RS and trigger states of AP-CSI/CSI-RS. Note that MAC CE latency is the time interval from the slot UE transmits the ACK corresponding to PDSCH carrying the MAC-CE message to the time that the MAC-CE message is applied. Agreement: Periodicities/slot offsets for P/SP CMR and IMR are configured independently Agreement: From a UE’s perspective, the same CSI-RS/CSI-IM resource ID cannot be configured in multiple CSI Resource settings with different time-domain behaviors. The LS is endorsed in R1-1805621 Conclusion: 1 slot periodicity for CSI-IM is not supported in NR Rel-15 Agreement Aperiodic triggering offset of CSI-IM follows triggering offset value of the associated NZP CSI-RS for channel measurement. Agreement What is the time offset of the CSI reference resource of P / SP CSI report? - A fixed time offset, slots, is used for a given numerology - N=4 for 15kHz, N=8 for 30kHz, N=16 for 60kHz, N=32 for 120kHz in case of CSI report without CRI - N=5 for 15kHz, N=10 for 30kHz, N=20 for 60kHz, N=40 for 120kHz in case of CSI report with CRI - FFS: Applicability of smaller value of N depending on UE capability Agreement For CSI latency requirement when an A-CSI trigger state triggers multiple CSI reports, up to X CSI reports are assumed to be calculated in a given time period. - FFS: A UE is not expected to update a CSI report if the PUSCH scheduling offset does not give enough CSI calculation time according to the Z’ values for that report - UE ignores a DCI scheduling a PUSCH with scheduling offset smaller than the any of the Z values in different reports - FFS: Details in case A-CSI is triggered for different numerologies - Value of X depends on UE capability and takes into account all the CCs - The above applies for the case of CSI only transmission Agreement: - For PUSCH-based SP-CSI reporting triggered by DCI, the timing offset of the initial report after the trigger should meet the PUSCH preparation procedure time (Z) for A-CSI reporting Agreement: - A UE capable of X simultaneous CSI calculations (according to capability 2-35) is said to have X CSI processing units. For aperiodic CSI report using AP CSI-RS (with a single CSI-RS resource in the resource set for channel measurement). - the CSI processing unit remains occupied from the first OFDM symbol after the PDCCH trigger until the last OFDM symbol of the PUSCH carrying the CSI report - If N AP CSI reports (each with a single CSI-RS resource in the resource set for channel measurement) are triggered in a slot, but the UE only has M un-occupied CSI processing units, UE is only required to update M of the N CSI reports - FFS if a rule is needed which CSI reports are required to be update or if it’s up to the UE - FFS if a CSI report linked with Ks > 1 CSI-RS resources for channel measurement occupies Ks CSI processing units or one CSI processing units Agreement - Introduce a new UE capability on support of either “Type A CSI processing capability” or “Type B CSI processing capability” with regard to the number of simultaneous CSI calculations X - For CSI latency requirement when an A-CSI trigger state triggers N CSI reports (where each report n is associated with (Zn, Z’n)) and have M un-occupied CSI processing units: - For Type A CSI processing capability: - A UE is not expected to update any of the triggered CSI reports if the time gap between the first symbol of PUSCH and the last symbol of the associated ap-CSI-RS / ap-CSI-IM does not give enough CSI calculation time according to Z'TOT=n=1MZn' - FFS how to index the M reports in this case to form Z'TOT - UE may ignore a DCI scheduling a PUSCH with scheduling offset smaller than ZTOT=n=1MZn - This applied to CSI only case, FFS for CSI+UL-SCH case - For Type B CSI processing capability: - A UE is not expected to update a CSI report if the PUSCH scheduling offset does not give enough CSI calculation time according to the Z’ values for that report - UE may ignore a DCI scheduling a PUSCH with scheduling offset smaller than the any of the Z values in different reports - This applied to CSI only case, FFS for CSI+UL-SCH case - CSI reports based on P / SP CSI-RS are assigned to CSI processing units as follows: - Type A: - For P / SP CSI reports, the CSI processing unit is occupied from the first symbol of the CSI reference resource of the P / SP report until the first symbol of the physical channel carrying the report - For A CSI reports, the CSI processing unit is occupied from the first symbol after the PDCCH triggering the report until the [first or last] symbol of the PUSCH carrying the report - Type B: - A periodic or aperiodic CSI reporting setting associated with P/SP CSI-RS is assigned one (or Ks) CSI processing unit and always occupy them - An activated SP-CSI report setting is assigned one (or Ks) CSI processing unit and occupies them until deactivated - Once the SP-CSI report is deactivated, the CSI processing unit can be used for other CSI report - Note: Type A assumes serial CSI processing implementation while Type B assume parallel CSI processing implementation. Note that this will not be captured in specification. Agreement: - For Type A CSI processing capability, a UE is not expected to update P/SP CSI reports if the number of CSI processing units occupied by P/SP CSI reports exceeds the number of simultaneous CSI calculations X according to the UE capability Agreement - When a CSI report is linked with Ks > 1 CSI-RS resources for channel measurement is triggered - For Type A CSI processing capability: - The CSI report occupies a single CSI processing unit and the latency requirement is calculated based on (Zn, Z’n) according to the High latency CSI class - For Type B CSI processing capability, downselect between the following in RAN1#93: - Alt1: The CSI report occupies a single CSI processing unit and the latency requirement is calculated based on (Zn, Z’n) according to the High latency CSI class - Alt2: The CSI report occupies Ks CSI processing units and the latency requirement for each unit is calculated based on (Zn, Z’n) according to the High latency CSI class - Companies are encouraged to provide their preference on either of the above alternatives and relevant (Zn, Z’n) values - Note: If more than one CSI-RS resource for CM is configured for a CSI report, the report belongs to the High Latency CSI class - Assuming Type A CSI processing capability or Type B if Alt 1 above is agreed: - If a CSI report is linked with Ks > 1 CSI-RS resources for channel measurement - For Ks=2, at most 16 CSI-RS ports per CSI-RS resource can be configured - For 8>=Ks>2, at most 8 CSI-RS ports per CSI-RS resource can be configured Agreement - Adopt the following (Z,Z’) values for Low Latency CSI class for normal UE capability for the Type A and Type B UE CSI processing capabilities: For normal UE capability (assuming single CSI report, single CMR/IMR) For Type A CSI processing capability; For Type B CSI processing capability. CSI only; CSI+UL-SCH; CSI only; CSI+UL-SCH. Low latency CSI; 15 kHZ; Z; 5; 6; 22; 32. Z'; 3; 4; 15; 25. 30 kHZ; Z; 9; 10; 25; 37. Z'; 6; 7; 16; 28. 60 kHZ; Z; 17; 18; 34; 57. Z'; 12; 13; 25; 48. 120 kHZ; Z; 27; 30; 49; 85. Z'; 22; 25; 37; 73. High latency CSI; 15 kHZ; Z. Z'. 30 kHZ; Z. Z'. 60 kHZ; Z. Z'. 120 kHZ; Z. Z'. Agreement - When DL BWP is switched, PUCCH-based SP-CSI report setting in activated state stays in its activated state. SP-CSI reporting is simply suspended until the DL BWP is switched back, whereon it resumes Agreement: The following text change is agreed: - In Section 5.2.1 of TS 38.214, add SSBRI (SSB Resource Indicator) to the contents of CSI reporting. [6] >>> Text Proposal for 38.214 Section 5.2.1 >>> 5.2.1 Channel state information framework The time and frequency resources that can be used by the UE to report CSI are controlled by the gNB. CSI may consist of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), SS/PBCH Block Resource indicator (SSBRI), layer indication (LI), rank indication (RI) and/or and L1-RSRP. For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings, and a list of trigger states ReportTriggerList containing a list of associated CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference. >>> End text proposal >>> Agreement For the case of single CC case, to determine the “lowest CORESET-ID” for determining default spatial QCL assumption for PDSCH, only consider CORESETs in active BWP Agreement: - For PUSCH scheduled by DCI format 0_0, the UE shall use a default spatial relation corresponding to the spatial relation, if applicable, used by the PUCCH resource with the lowest ID configured in the active UL BWP - Above applies for a cell configured with PUCCH - Note: the UE is configured with a list of spatial relations in PUCCH-SpatialRelationInfo. MAC-CE indicates a single selected spatial relation from the list on a per-PUCCH resource basis if the list has more than one element. Agreement - Support SP-SRS as mandatory with UE capability signalling Agreement The latency for MAC CE activation is 12 slots for 60kHz SCS and 24 slots for 120kHz SCS. The latency is calculated from the ending symbol of UE’s ACK transmission for PDSCH carrying MAC-CE activation message to when the MAC CE message is applied. Agreement: - Support uplink cross-carrier beam indication for PUCCH and SRS - Add Cell index and BWP information in SpatialRelation configuration LS is endorsed in R1-1805627 with the following additional sentences: “RAN1 has agreed on cross carrier indication of spatial relations with SRS and PUCCH. For the case of SRS, changes are needed to the MAC CE message used for activating semi-persistent SRS to enable cross carrier indication. RAN1 suggests that RAN2 take this into their work.” Agreement: BFI indication interval is lower bounded by 2ms Agreement: Agree to the following text change for TS38.213 section 6 A UE is configured with one control resource set by higher layer parameter Beam-failure-Recovery-Response-CORESET and with an associated search space provided by higher layer parameter search-space-config, as described in subcaluse 10.1, for monitoring PDCCH in the control resource set. The UE may receive from higher layers, by parameter Beam-failure-recovery-request-RACH-Resource, a configuration for a PRACH transmission as described in Subclause 8.1. For PRACH transmission in slot and according to antenna port quasi co-location parameters associated with periodic CSI-RS configuration or SS/PBCH block with index provided by higher layer [5, TS 38.321], the UE monitors PDCCH for detection of a DCI format with CRC scrambled by C-RNTI starting from slot within a window configured by higher layer parameter Beam-failure-recovery-request-window. For PDSCH reception, the UE assumes the same antenna port quasi-collocation parameters as for monitoring PDCCH until the UE receives by higher layers an activation for a TCI state or a parameter TCI-StatesPDCCH. Agreement: - Send an LS to RAN2 with following information - RAN1 confirms the need for beamFailureRecoveryTimer and keep its uses as current RAN1 agreement - Note: unsuccessful CFRA based BFR upon expiry of beamFailureRecoveryTimer means that UE shall not use CFRA for BFR after beamFailureRecoveryTimer expired and no indication to higher layer is required - The following use case will be captured in the LS - Candidate beam selection for contention-free PRACH-based BFR is based on L1-RSRP. For qualified candidate beam but poor SINR, the timer enables UE to try contention-based PRACH resources after timer expires - Note: CBRA resource can be used when there is no candidate beam identified from candidate-beam-RS-list, as described by TS 38.321 section 5.1.2. - beamFailureRecoveryTimer does not apply to the use of CBRA resources. The LS is endorsed in R1-1805622 Agreement: Text proposal for TS38.213 section 6 ------------------------------Start Text Proposal--------------------------------- The physical layer in the UE shall, in slots where the radio link quality according to the set is assessed, provide an indication to higher layers when the radio link quality for all corresponding resource configurations in the set that the UE uses to assess the radio link quality is worse than the threshold Qout,LR. The physical layer informs the higher layers when the radio link quality is worse than the threshold Qout,LR with a periodicity determined by the maximum between the shortest periodicity of periodic CSI-RS configurations or SS/PBCH blocks in the set and X. ------------------------------End Text Proposal--------------------------------- LS is endorsed in R1-1805623 Agreement Text proposals for TS 38.214 v15.0.0 Section 6.1.4.1 < Unchanged parts are omitted > - for MCS index 0,1 and 28, q=1 if UE has reported to support pi/2 BPSK modulation; and q=2 in other cases else - the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. end Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM MCS Index IMCS; Modulation Order Qm; Target code Rate x 1024 R; Spectral efficiency. 0; q; 240/q; 0.2344. 1; q; 314/q; 0.3066. 2; 2; 193; 0.3770. 3; 2; 251; 0.4902. 4; 2; 308; 0.6016. 5; 2; 379; 0.7402. 6; 2; 449; 0.8770. 7; 2; 526; 1.0273. 8; 2; 602; 1.1758. 9; 2; 679; 1.3262. 10; 4; 340; 1.3281. 11; 4; 378; 1.4766. 12; 4; 434; 1.6953. 13; 4; 490; 1.9141. 14; 4; 553; 2.1602. 15; 4; 616; 2.4063. 16; 4; 658; 2.5703. 17; 6; 466; 2.7305. 18; 6; 517; 3.0293. 19; 6; 567; 3.3223. 20; 6; 616; 3.6094. 21; 6; 666; 3.9023. 22; 6; 719; 4.2129. 23; 6; 772; 4.5234. 24; 6; 822; 4.8164. 25; 6; 873; 5.1152. 26; 6; 910; 5.3320. 27; 6; 948; 5.5547. 28; 1q; reserved. 29; 2; reserved. 30; 4; reserved. 31; 6; reserved. < Unchanged parts are omitted > Agreement Text proposals for TS 38.214 v15.0.0 Section 5.2.2.1.1 The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Subclause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,...,1000 1008+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…,3000+P-1], as given by where is a vector of PDSCH symbols from the layer mapping defined in Subclause 7.3.1.4 of [4, TS 38.211], is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1, otherwise W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 4.1. Agreement: - For PDSCH scheduled by DCI format 1_0, UE assumes the default 64QAM table regardless of RRC configuration - For PUSCH scheduled by DCI format 0_0, UE assumes the default 64QAM table regardless of RRC configuration Agreement: - For PDSCH scheduled with other RNTIs than C-RNTI & CS-RNTI, UE assumes 64 QAM MCS table - For PUSCH scheduled with other RNTIs than C-RNTI & CS-RNTI, SP-CSI C-RNTI, UE assumes 64 QAM MCS table Agreement Text proposals for TS 38.214 v15.0.0 Section 5.2.2.1 < Unchanged parts are omitted > If a UE is not configured with higher layer parameter MeasRestrictionConfig-time-channel, the UE shall derive the channel measurements for computing CQI CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. If a UE is configured with higher layer parameter MeasRestrictionConfig-time-channel, the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. If a UE is not configured with higher layer parameter MeasRestrictionConfig-time-interference, the UE shall derive the interference measurements for computing CQI CSI value reported in uplink slot n based on only the CSI-IM and/or NZP CSI-RS for interference measurement no later than the CSI reference resource associated with the CSI resource setting. If a UE is configured with higher layer parameter MeasRestrictionConfig-time-interference the UE shall derive the interference measurements for computing the CQI CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting. < Unchanged parts are omitted > Agreement Text proposals for TS 38.214 v15.0.0 Section 5.2.2.1 ------------------------------------------------------Start text proposal----------------------------------------------------------- 5.2.2.1.1 CSI reference resource definition - the PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Subclause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000 3000+P-1], as given by where is a vector of PDSCH symbols from the layer mapping defined in Subclause 7.3.1.4 of [4, TS 38.211], is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1,. otherwise If the higher layer parameter reportQuantity for the CSI-ReportConfig for which the CQI is reported is set to either ‘CRI/RI/PMI/CQI’ or ‘CRI/RI/LI/PMI/CQI’, W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity for the CSI-ReportConfig for which the CQI is reported is set to ‘CRI/RI/CQI’, W(i) is the precoding matrix corresponding to the procedure described in Subclause 5.2.1.4.2. If the higher layer parameter reportQuantity for the CSI-ReportConfig for which the CQI is reported is set to ‘CRI/RI/i1/CQI’, W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Subclause 5.2.1.4. The corresponding PDSCH signals transmitted on antenna ports [3000,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Subclause 4.1. Agreement Text proposal in Section 5.1.6.1 of TS 38.214 The CSI-RS defined in Subclause 7.4.1.5 of [4, TS 38.211], may be used for time/frequency tracking, CSI computation, L1-RSRP computation and mobility. If the UE is configured with a CSI-RS resource and a CORESET in the same OFDM symbol(s), the UE may assume that the CSI-RS and a PDCCH DM-RS transmitted in the CORESET are quasi co-located with ‘QCL-TypeD’, if ‘QCL-TypeD’ is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET. Agreement Text proposal in Section 6.2.1 of TS38.214 A UE shall not transmit simultaneously SRS resource(s) and PRACH. If a UE is configured with an SRS resource and PRACH in the same OFDM symbols in a slot, SRS is not transmitted in the overlapping symbols. If SRS and PRACH would occur on overlapping OFDM symbol(s), SRS is not transmitted on the overlapping symbol(s). Agreement In the case of collision of aperiodic/semi-persistent/periodic SRS and short PUCCH carrying HARQ-ACK, drop SRS. And related Text Proposal: TP for Section 6.2.1 in TS 38.214 (treatment of short PUCCH with A/N against SRS) ------Unchanged parts are omitted------ For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS happens is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only. ------Unchanged parts are omitted------ Agreement: Confirm the WA on CSI-RS sequence mapping for the case with 0.5 density Agreement: A UE may assume all CSI-RS ports within one CSI-RS resource are quasi co-located with respect to ‘QCL type A’ and ‘QCL type D’ when applicable. Agreement: Text proposal for Section 5.2.2.3.1 in 38.214 ---------------------------------------------------------------------------------------------------------------------------------- - CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211] , and the minimum bandwidth is the minimum between 24 and resource blocks. ---------------------------------------------------------------------------------------------------------------------------------- Agreement Text proposal for Section 5.1.6.1.2 in 38.214 ---------------------------------------------------------------------------------------------------------------------------------- If the UE is configured with a CSI-ReportConfig with reportQuantity set to “CRI/RSRP”, or “No Report” and if the Resource Setting for channel measurement contains a CSI-RS Resource Set that is configured with the higher layer parameter CSI-RS-ResourceRep and configured without the higher layer parameter TRS-Info, the UE can only be configured with 1 or 2 ports with the higher layer parameter NrofPorts for all CSI-RS resources within the set, and the UE may be configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, and the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with ‘QCL-TypeD’ if ‘QCL-TypeD‘ is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the SS/PBCH block, and the UE shall expect that the same subcarrier spacing is used for both the CSI-RS and the SS/PBCH block. Agreement The number of ports in a resource set for L1-RSRP - Always have the same number of ports (1 or 2) across all resources ---------------------------------------------------------------------------------------------------------------------------------- Agreement For density ½, odd/even PRB allocations is respect to the common resource block grid Include in LS to RAN2 for CSI-RS Xi (Huawei) Agreement Text proposal for Section 5.1.4.2 in 38.214 ---------------------------------------------------------------------------------------------------------------------------------- - ZP-CSI-RS-ResourceConfigId determines ZP CSI-RS resource configuration identity. - NrofPorts defines the number of CSI-RS ports, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. - ZP-CSI-RS-Density defines the zero-power CSI-RS frequency density, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. - CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. ---------------------------------------------------------------------------------------------------------------------------------- Agreement Text proposal for Section 5.2.2.3.1 in 38.214 - CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211]. If the configured bandwidth is larger than the corresponding BWP, UE shall assume that the actual CSI-RS bandwidth is equal to the BWP size. Agreement - The UE expects that all the CSI resources of a resource set are configured with the same starting RB and number of RBs and the same CDM-type. - FFS: Other parameters Agreement Text proposal for Section 5.2.2.3.1 in 38.214 -------------------------------------------------------------------------------------------------------------------------- - CSI-RS-Density defines CSI-RS frequency density of each CSI-RS port per PRB and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. For density ½, odd/even PRB allocations is respect to the common resource block grid. - CDMType defines CDM values and pattern, where the allowable values are given in Subclause 7.4.1.5 of [4, TS 38.211]. - CSI-RS-FreqBand parameters configure the bandwidth and the initial RB index in the frequency domain, both in units of 4RBs, of a CSI-RS resource within a BWP as defined in Subclause 7.4.1.5 of [4, TS 38.211]. - Pc: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size. - Pc_SS: which is the assumed ratio of SS/PBCH block EPRE to NZP CSI-RS EPRE. - ScramblingID: defines scrambling ID of CSI-RS with length of 10 bits. - CC_Info defines which component carrier the configured CSI-RS is located in. - BWP_Info defines which bandwidth part the configured CSI-RS is located in. -------------------------------------------------------------------------------------------------------------------------- Agreement Text proposal for Section 5.1.4.2 in 38.214 ---------------------------------------------------------------------------------------------------------------------------------- - ZP-CSI-RS-ResourceMapping defines the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Subclause 7.4.1.5 of [4, TS 38.211]. ---------------------------------------------------------------------------------------------------------------------------------- LS is endorsed in R1-1805626 Agreement: Text proposals for TS 38.211 Section 6.4.1.1.1.2 ------Unchanged parts are omitted------ 6.4.1.1.1.2 Sequence generation when transform precoding is enabled If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where is given by clause 5.2.2 with and for a PUSCH transmission dynamically scheduled by DCI. ------Unchanged parts are omitted------ Agreement Text proposals for TS 38.211 Section 7.4.1.1.1 ------Unchanged parts are omitted------ 7.4.1.1 Demodulation reference signals for PDSCH 7.4.1.1.1 Sequence generation The UE shall assume the sequence is defined by . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within a frame, and - and is given by the higher-layer parameter DL-DMRS-Scrambling-ID if provided and the PDSCH is scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI - and otherwise ------Unchanged parts are omitted------ Agree with the above TP and change 14 to Agreement <<<< Text proposal for section 6.4.1.1.3 in TS 38.211 <<<< 6.4.1.1.3 Precoding and mapping to physical resources […] Table 6.4.1.1.3-6: PUSCH DM-RS positions for single-symbol DM-RS and frequency hopping enabled. Duration in symbols for each hop; DM-RS positions. PUSCH mapping type A; PUSCH mapping type B. UL-DMRS-add-pos; UL-DMRS-add-pos. 0; 1; 0; 1. First hop; Second hop; First hop; Second hop; First hop; Second hop; First hop; Second hop. ≤3; -; -; -; -; 0; l0; 0. 4; 0; l0; 0; 0; l0; 0. 5, 6; 0; l0; 0, 4; 0; ,; 0, 4. 7; 0; ,; 0, 4; 0; ,; 0, 4. […] Agreement: UE DM-RS transmission procedure for before RRC configuration in case of frequency hopping enabled should be specified in TS38.214 as following: - If frequency hopping is enabled, the UE shall assume dmrs-AdditionalPosition = 1, and one additional DM-RS can be transmitted according to PUSCH duration. Text Proposal for this correction shown below: 6.2.2 UE DM-RS transmission procedure When transmitted PUSCH is not scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining Res not used for DM-RS in the symbols are not used for any PUSCH transmission, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled, - For PUSCH with allocation duration of more than 7 symbols, the UE shall assume dmrs-AdditionalPosition=2 and up to two additional DM-RS can be transmitted according to PUSCH duration, or - For PUSCH with allocation duration of 7 symbols with mapping type B, the UE shall assume dmrs-AdditionalPosition=1 and one additional DM-RS can be transmitted - For PUSCH with allocation duration of less than 7 symbols with mapping type B, the UE shall assume dmrs-AdditionalPosition=0 and not transmit additional DM-RS. If frequency hopping is enabled, - The UE shall assume dmrs-AdditionalPosition=1 and up to one additional DM-RS can be transmitted according to PUSCH duration. […] Agreement For PUSCH not scheduled by PDCCH format 0_1: - Support FDM for PUSCH and DMRS when PUSCH duration is N or less OFDM symbols - N=2 for CP-OFDM. TDM for other cases. Note: For DFT-S-OFDM, always TDM. R1-1805587 WF on group/sequence hopping for DFT-S-OFDM PUSCH DMRS CMCC, ZTE, Sanechips, Intel, Nokia, Nokia Shanghai Bell, OPPO, NEC, LG Electronics, NTT DOCOMO, MediaTek, vivo, Qualcomm Agreement - For DFT-s-OFDM based PUSCH transmission, if group hopping or sequence hopping is used, the sequence of the second DMRS symbol follows the first symbol for each pair of adjacent DMRS symbols. Agreement The following text proposal for section 7.4.1.1.2 of TS38.211 is agreed. --------------------- For PDSCH mapping type B - if the PDSCH duration is 2, 4, or 7 OFDM symbols for normal CP, or 2, 4, 6 OFDM symbols for extended CP, and the PDSCH allocation collides with resources reserved for a CORESET, shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and - if the PDSCH duration is 4 symbols, the UE is not expected to receive a DM-RS symbol beyond the third symbol, - if the PDSCH duration is 7 symbols for normal CP, or 6 OFDM symbols for extended CP, - the UE is not expected to receive the first DM-RS beyond the fourth symbol, and - if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted. Table 7.4.1.1.2-3: PDSCH DM-RS positions for single-symbol DM-RS. Duration in symbols; DM-RS positions. PDSCH mapping type A; PDSCH mapping type B. DL-DMRS-add-pos; DL-DMRS-add-pos. 0; 1; 2; 3; 0; 1; 2; 3. 2; -; -; -; -. 3; -; -. 4. 5; -; -. 6. 7. 8; , [7]; , [7]; , [7]; -; -. 9; ,7; ,7; ,7; -; -. 10; ,9; , 6, 9; , 6, 9; -; -. 11; ,9; , 6, 9; , 6, 9; -; -. 12; 9; , 6, 9; , 5, 8, 11; -; -. 13; ,11; , 7, 11; , 5, 8, 11; -; -. 14; ,11; , 7, 11; , 5, 8, 11; -; -. Table 7.4.1.1.2-4: PDSCH DM-RS positions for double-symbol DM-RS Duration in symbols; DM-RS positions. PDSCH mapping type A; PDSCH mapping type B. DL-DMRS-add-pos; DL-DMRS-add-pos. 0; 1; 2; 0; 1; 2. <4; -; -. 4; -; -. 5; -; -. 6. 7. 8; -; -. 9; -; -. 10; , 8; -; -. 11; , 8; -; -. 12; , 8; -; -. 13; , 10; -; -. 14; , 10; -; -. Agreement In Section 5.1.6.2 of TS38.214 When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, DL-DMRS-max-len and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in subclause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition - For PDSCH with mapping type A, the UE shall assume dmrs-AdditionalPosition=2 and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Subclause 7.4.1.1 of [4, TS 38.211], and - For PDSCH with allocation duration of 7 symbols for normal CP or 6 symbols for extended CP with mapping type B, the UE shall assume one additional single-symbol DM-RS present in the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol respectively of the PDSCH allocation duration, otherwise the UE shall assume that the additional DM-RS symbol is not present, and - For PDSCH with allocation duration of 4 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and - For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and the UE shall assume that the PDSCH is present in the symbol carrying DM-RS Agreement Add the following text in Section 7.4.1.1.2 of 38.211 to clarify the above: ------Unchanged parts are omitted------ The position(s) of the DM-RS symbols is given by and - for PDSCH mapping type A, the duration is between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot - for PDSCH mapping type B, the duration is the number of OFDM symbols of the scheduled PDSCH resources as signalled and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4. The case DL-DMRS-add-pos equal to 3 is only supported when DL-DMRS-typeA-pos is equal to 2. For PDSCH mapping type A, duration of 3 and 4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable for DL-DMRS-typeA-pos equals 2. ------Unchanged parts are omitted------ Agreement Add the following text in Section 6.4.1.1.3 of 38.211 to clarify the above: ------Unchanged parts are omitted------ The position(s) of the DM-RS symbols is given by and - the signalled duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PUSCH resources in the slot for PUSCH mapping type A according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if frequency hopping is not used, or - the signalled duration of scheduled PUSCH resources for PUSCH mapping type B according to Tables 6.4.1.1.3-3 and 6.4.1.1.3-4 if frequency hopping is not used, or - the signalled duration per hop according to Table 6.4.1.1.3-6 if frequency hopping is used. For PUSCH mapping type A, the case UL-DMRS-add-pos equal to 3 is only supported when DL-DMRS-typeA-pos is equal to 2. For PUSCH mapping type A, duration of 4 symbols in Table 6.4.1.1.3-4 is only applicable for DL-DMRS-typeA-pos equals 2. ------Unchanged parts are omitted------ Agreement Text proposal for Section 5.1.6.2, 38.214 ------Unchanged parts are omitted------ When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH scheduling DCI format 1_0 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, DL-DMRS-max-len and dmrs-Type, ------Unchanged parts are omitted------ When receiving PDSCH scheduled by DCI format 1_1 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, - the UE may be configured with the higher layer parameter dmrs-Type, and the configured DM-RS configuration type is used for receiving PDSCH in as defined in Subclause 7.4.1.1 of [4, TS 38.211]. - the UE may be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter DL-DMRS-max-len. - if DL-DMRS-max-len is equal to 1, single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be 0, 1, 2 or 3. - if DL-DMRS-max-len equal to 2, both single-symbol DM-RS and double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be 0 or 1. - and the UE shall assume to receive additional DM-RS as specified in Table 7.4.1.1.2-3 and Table 7.4.1.1.2-4 as described in Subclause 7.4.1.1.2 of [4, TS 38.211]. ------Unchanged parts are omitted------ Agreement Text proposal for TS38.211 6.4.1.1.3 Precoding and mapping to physical resources […] The time-domain index and the supported antenna ports are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6: - if the higher-layer parameter UL-DMRS-max-len is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS - if the higher-layer parameter UL-DMRS-max-len is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used - if higher-layer parameter UL-DMRS-add-pos is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [5, TS 38.212], Table 6.4.1.1.3-6 shall be used assuming UL-DMRS-add-pos is 1 for each hop. If , only UL-DMRS-add-pos equal to 0 is supported is assumed to be 0. […] Agreement For DMRS scrambling ID for PDSCH scheduled by PDCCH format 1_0 with CRC scrambled by C-RNTI, or PUSCH scheduled by PDCCH format 0_0 with CRC scrambled by C-RNTI - If a PUSCH/PDSCH of a UE is scheduled by DCI format 0_0 or DCI format 1_0, the UE would assume the first DMRS scrambling ID if configured, otherwise is used. Working assumption: Support variable time density LPT-RS for UCI-only PUSCH where information (e.g. MCS) in DCI is used to determine LPT-RS. Agreement - For the following case, the field of PTRS-DMRS association is 0 bit for both codebook based UL transmission and non-codebook based UL transmission - High layer parameter maxRank is configured as 1. - Update the following Table 7.3.1.1.2-26 for PTRS-DMRS association for UL PTRS ports 0 and 1, UL-PTRS-ports = 2 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port transmitting layers corresponding to SRS port 0 and 2, the 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port transmitting layers corresponding to SRS port 1 and 3, 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port transmitting layers corresponding to SRS port 0 and 2, 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port transmitting layers corresponding to SRS port 1 and 3, 2nd DMRS port which shares PTRS port 1. Agreement Agree on the following text proposal TS 38.211 v15.0.0 Section 6.4.1.2.1.2 to implement the previous agreement by changing m’ in the exponential to m: < Unchanged parts are omitted > If transform precoding is enabled, the phase-tracking reference signal to be mapped in position before transform precoding, where depends on the number of PT-RS groups , the number of samples per PT-RS group , and according to Table 6.4.1.2.2.2-1, shall be generated according to < Unchanged parts are omitted > Agreement The following working assumption is confirmed. - NR supports the case that different ports of one 4-port SRS resource have different comb offsets for comb 2 and 4. Agreement Text proposal for section 6.2.1.3, 38.214): For an aperiodic SRS triggered in DCI format 2_3 and UE configured with higher layer parameter typeA-SRS-TPC-PDCCH-Group without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers. For an aperiodic SRS triggered in DCI format 2_3 and UE configured with higher layer parameter typeB-SRS-TPC-PDCCH-Group without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI. Agreement Text proposal for section 7.3.1.3.4, 38.212: If the UE is configured with higher layer parameter typeA-SRS-TPC-PDCCH-Group for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block is configured for the UE by higher layers, with the following fields defined for the block: - SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause x.x of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-5. - TPC command number –2 bits- TPC command number 1, TPC command number 2, …, TPC command number n where each TPC command applies to a respective serving cell index provided by higher layer parameter cc-IndexInOneCC-Set. If the UE is configured with higher layer parameter typeB-SRS-TPC-PDCCH-Group for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one or more blocks is configured for the UE by higher layers where each block applies to a serving cell, with the following fields defined for the block: - SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause x.x of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-5. - TPC command number –2 bits Agreement: If TS38.214 does not capture that slot-level offset is for aperiodic SRS resource set, the spec editor should capture this Agreement The configuration of SRS starting position offset and symbol number should satisfy . The following text proposal for TS38.211 section 7.1.2.4 captures the above agreement. < Unchanged parts are omitted > An SRS resource consists of - antenna ports , , given by the higher layer parameter NrofSRS-Ports - consecutive OFDM symbols contained in the higher layer parameter SRS-ResourceMapping - , the starting position in the time domain given by where the offset counts symbols backwards from the end of the slot and is contained in the higher layer parameter SRS-ResourceMapping, where - , the frequency-domain starting position of the sounding reference signal < Unchanged parts are omitted > Agreement Inform RAN2 that aperiodicSRS-ResourceTrigger should start from 1 not 0. LS to be prepared by Steve (Ericsson) Agreement >>> Text proposal for 38.212 Section 7.3.1.1.2 >>> Table 7.3.1.1.2-24: SRS request Value of SRS request field; Triggered aperiodic SRS resource set(s). 00; No aperiodic SRS resource set triggered. 01; SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1. 10; SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 2. 11; SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 3. >>> End text proposal >>> Agreement To avoid duplication between 38.212 and 38.214 of the table mapping the codepoints of the SRS request field in DCI to the triggered aperiodic SRS resource set(s), remove Table 6.2.1-1 from 38.214. Text proposal is provided below. >>> Text proposal for 38.214 Section 6.2.1 >>> For aperiodic SRS, at least one state of the DCI field is used to select at least one out of the configured SRS resource set. The 2-bit SRS request field [5, TS38.212] in DCI format 0_1, 1_1 indicates the triggered SRS resource set given in Table 6.2.1-1 7.3.1.1.2-24 of [5, TS38.212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Subclause 11.4 of [6, TS 38.213]. Table 6.2.1-1: SRS request value for aperiodic SRS Value of SRS request field; Description. '00'; No aperiodic SRS trigger. '01'; The 1st SRS resource set(s) configured by higher layers. '10'; The 2nd SRS resource set(s) configured by higher layers. '11'; The 3rd SRS resource set(s) configured by higher layers. If a UE is configured with the higher layer parameter SRS-AssocCSIRS and with the higher layer parameter ulTxConfig set to 'NonCodebook', the UE may be configured with a NZP CSI-RS resource where a NZP-CSI-RS-ResourceConfigId is associated with an SRS resource set. >>> End text proposal >>> Agreement - 1T4R antenna switching for SRS transmission is supported by 2 aperiodic SRS resource sets - two aperiodic SRS resource sets with total four SRS resources transmitted in different symbols of two different slots, each SRS resource consisting of a single SRS port, and where the SRS port of each resource is associated with a different UE antenna port - each of the two SRS resource sets consists of two SRS resources, or one SRS resource set consists of a single SRS resource and the other SRS resource set consists of three SRS resources - the UE shall expect the same value for the higher layer parameters alpha-srs, p0-srs, srs-pathlossReference-rs-config and srs-pcadjustment-state-config in the two SRS resource sets - FFS: On UE ambiguity if two DCI triggers separately triggers two SRS resource sets - FFS: The gap before or after any SRS resource set for antenna switching - FFS: The number of SRS resource sets for other cases LS is endorsed in R1-1805625 Agreement >>> Text proposal for 38.214 Section 6.2.1.2 >>> 6.2.1.2 UE antenna switching When UE antenna switching is enabled by the higher layer parameter SRS-SetUse set as 'antenna switching' for a UE that supports transmit antenna switching, a UE may be configured with one of the following configurations depending on the UE capability: - SRS resource set with two SRS resources transmitted in different symbols, each SRS resource consisting of a single SRS port, being associated with different UE antenna ports, or - SRS resource set with two SRS resources transmitted in different symbols, each SRS resource consisting of two SRS ports where the port pair of the second resource is associated with a different UE antenna pair than the port pair of the first resource, or - SRS resource set with four SRS resources transmitted in different symbols, each SRS resource consisting of a single SRS port being associated with different UE antenna port, or - SRS resource set with one SRS resource, where the number of SRS ports is equal to 1, 2, or 4 and a guard period where UE does not transmit any other signal of Y symbols in-between the SRS resources of the set, is used in case the SRS resources are transmitted in the same slot. The value of Y is defined by Table 6.2.1.2-1. >>> End text proposal >>> Agreement The following text proposals are agreed. Text proposal 5.1.6.1.2 38.214 A UE does not expect to be configured with a Reporting Setting, which is linked to a CSI-RS resource set for tracking, where the higher layer parameter MeasRestrictionConfig-time-channel for the Reporting Setting set to ‘ON’. Text proposal 5.1.6.1.1 38.214 The periodic CSI-RS resources in the CSI-RS resource set configured with higher layer parameter TRS-Info have the same periodicity, bandwidth and subcarrier location. A UE does not expect to be configured with a reporting setting, which is linked to a CSI-RS resource set for tracking, where the higher layer parameter MeasRestrictionConfig-time-channel for the reporting setting set to ‘ON’. Agreement A CSI-RS resource set should not be configured with TRS-info and CSI-RS-ResourceRep simultaneously. Agreement: - Rel-15 supports aperiodic TRS for FR2 as an optional UE feature with UE capability signalling - (Working assumption) A UE does not expect to be triggered to aperiodic TRS unless it has been configured with an associated periodic TRS with the same burst length X slots on the same BWP - The aperiodic TRS and the associated periodic TRS has the same BW, [same symbol position, same subcarrier location] and the aperiodic TRS will be QCL type A+D (if applicable) with this periodic TRS - FFS on DCI trigger timing for SCell activation and BWP switching - The UE does not expect the scheduling offset between DCI to the first symbol of TRS be smaller than the threshold UE reported - The aperiodic TRS is triggered by UL DCI (same as CSI-RS) - No new/additional RRC parameters are introduced for this feature except UE capability Send an LS to RAN2 inform the above agreements and ask RAN2 to take into account in their work – R1-1805758 (Xiaoyi, ATT), which is approved with final LS in R1-1805763 The above optional UE feature is to be included in the current UE feature spreadsheet for RAN1#93 Note: (from Huawei) Aperiodic TRS is a new feature in Rel-15 but Huawei is willing to accept this for progress Note: AT&T has concerns to make this an UE optional feature since this is a forward compatibility issue Agreement Text proposal for TS38.211 ----------------------------begin text change------------------------ 4.4.1 Antenna ports An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. - For DM-RS associated with PDSCH the channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same scheduled PDSCH, same slot and in the same PRG as described in 3GPP TS 38.214 Section 5.1.2.3. ---------------------------- end text change------------------------ Note that the above text change does not imply the support of time domain PRB bundling. Working assumption on ‘same’ slot in Rel 15. If there is no consensus to revisit the working assumption in RAN1#93, it is automatically confirmed.

Scheduling/HARQ aspects

Agreements: - NW and UE maintain the same understanding on SSB/CORESET#0/SS#0 in connected_mode at least for non-broadcast PDCCH - Solutions FFS - For the broadcast PDCCH, it is up to UE which common search space to monitor based on which SSB in both connected, in-active, and idle modes - Unicast PDSCH can be scheduled by a DCI associated with the CORESET #0 Agreements: - The size of DCI 1_1 is the same (by zero-padding when necessary) across CORESETs of a BWP of a serving cell Agreements: - For WB RS CORESET, the UE is expected to be configured with the CORESET only with the following possibilities: - Up to 4 clusters, each cluster consisting of contiguous RBs. - It is clarified that the counting rule of CCE for channel estimation is the same between NB RS CORESET and WB RS CORESET, which implies following: - The number of CCEs for channel estimation in case of WB RS CORESET is counted as the union of the sets of CCEs for PDCCH candidates to be monitored and hence BW of the WB RS CORESET is not restricted by CCE limit for channel estimation. - Note: it is up to RAN4 whether UE may perform channel estimation within a BW smaller than the maximum number of contiguous RBs within the WB RS CORESET Agreements: - For Rel.15 December 2017 version of Case 2, number of CCEs for channel estimation per slot is {56, 56, 48, 32} CCEs for SCS {15kHz, 30kHz, 60kHz, 120kHz} Agreements: - UE is not expected to be configured with PDCCH monitoring in CSS(s) for more than what UE can monitor in terms of numbers of BDs/CCEs Agreements: - Following working assumption is made: - For a UE supporting CA with up to X DL-CCs with the same numerology with X <= 4, maximum number of PDCCH blind decodes per slot the UE shall support is X*M, and; - For a UE supporting CA with up to Y DL-CCs with the same numerology Y > 4, maximum number of PDCCH blind decodes per slot the UE shall support is y*M, where; - M = {44, 36, 22, 20} for SCS = {15kHz, 30kHz, 60kHz, 120kHz} - y is an integer from {4, …, 16} and is reported as UE capability to the network. - For a UE supporting CA with up to X DL-CCs with the same numerology with X <= 4, maximum number of CCEs for channel estimation per slot the UE shall support is X*N, and; - For a UE supporting CA with up to Y DL-CCs with the same numerology with Y > 4, maximum number of CCEs for channel estimation per slot the UE shall support is y*N, where; - N = {56, 56, 48, 32} for SCS = {15kHz, 30kHz, 60kHz, 120kHz} - y is an integer from {4, …, 16} and is reported as UE capability to the network. Agreements: - The working assumption as part of the agreements below is confirmed: - Change Y_{p, kp} to Y_{p, ns,f } in the search space hashing function in subclause 10.1 of 38.213, where the index ns,f is the slot number. - (Working assumption) The reset of the update is per radio frame Agreements: - A UE is not expected to be configured to monitor PDCCH across a slot boundary. Working assumption: - For a common search space configured with RMSI-PDCCH-Config, osi-searchSpace, paging-searchSpace, and ra-searchSpace, DCI format 0_0/1_0 with C-RNTI is monitored in non-DRX occasions after C-RNTI is available. - For a common search space configured with RMSI-PDCCH-Config, osi-searchSpace, paging-searchSpace, and ra-searchSpace, DCI format 0_0/1_0 with CS-RNTI is monitored in non-DRX occasions after CS-RNTI is available. Agreements: - Following is to be clarified in 213: - Type0-CSS is configured by RMSI-PDCCH-Config provided by MIB or is configured by searchSpace-SIB1 provided by PDCCH-ConfigCommon - Type0A-CSS is configured by searchSpace-OSI provided by PDCCH-ConfigCommon - Type1-CSS is configured by ra-SearchSpace provided by PDCCH-ConfigCommon - Type2-CSS is configured by pagingSearchSpace provided by PDCCH-ConfigCommon - Type3-CSS is configured by a SearchSpace with the searchSpaceType “common” provided by PDCCH-Config Working assumption: - At least for Case 1-1 and Case 1-2, map all candidates of USS search-space-set with lower SS set ID before candidates of USS with higher ID - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped) - Case 2 FFS Agreements: Adopt the TP in R1-1805569 for 38.213 section 11.1 - Note that it reflects the agreements reached before RAN1 #92bis related to TDD UL/UL configurations and SFI Agreements: UE does not expect the reference SCS in TDD UL/DL configuration common and common2 to be different UE does not expect the reference SCS in cell-specific UL/DL configuration in a cell to be larger than the SCS of any BWP configured for the cell Agreements: - Limit the size of the UE-specific SFI table to a max total of 512 values across all entries in Rel 15. Agreements: - On slot format indication for ECP, reuse the slot format indication for NCP, and determine the direction of an ECP symbol using the indicated direction of (partially) overlapping NCP symbols - If all (partially) overlapping NCP symbols are D, the ECP symbol is D - If all (partially) overlapping NCP symbols are U, the ECP symbol is U - If one of (partially) overlapping NCP symbols is X, the ECP symbol is X Agreements: Regarding cancellation of RRC configured DL reception with a DCI granted UL transmission, or the cancellation of RRC configured UL transmission with a DCI granted DL reception, the cancellation is subject to a minimum time constraint, which follows N2 timeline Agreements: After active BWP switching, the SFI received before the BWP switching is still applicable to the new active BWP after switching Agreements: UE is not expected to monitor GC-PDCCH for SFI for a first cell in another cell with larger SCS than the first cell in Rel-15 Agreements: In UE-specific SFI table configuration, it is possible for the length of an entry to be longer than the configured monitoring period of the SFI For a slot covered by multiple SFIs transmitted at different slots, the UE does not expect to receive different slot format indicated by different SFIs. - If UE receives different slot formats for the same slot from different SFI, the UE behaviour is not defined. Agreements - In the slot format table in TS38.211, the entry 255 is defined such that when a slot format for a slot is indicated as 255, the UE does not use this information in deciding the cancellation of UE-specific RRC configured DL receptions or UE-specific RRC configured UL transmissions Agreements: When an RRC configured UL transmission is cancelled by SFI or DCI, the UE is not expected to cancel the part of RRC configured UL transmission that is to be transmitted over the OFDM symbols within N2 OFDM symbols after the end of the OFDM symbol carrying the SFI or DCI from UE perspective. Agreements: For cancellation of RRC configured transmission or reception by SFI, the cancellation is for a unit of transmission/reception if any OFDM symbol within the unit is cancelled by SFI. - For RRC configured CSI-RS resource set, the cancellation unit is the CSI-RS resource set - For RRC configured PDSCH and PUSCH with slot aggregation, the cancellation unit is the whole PDSCH or PUSCH within a slot - For RRC configured PDSCH, PUCCH, and PUSCH without slot aggregation, the cancellation unit is the whole PDSCH, PUCCH, and PUSCH - For RRC configured SRS transmission, the cancellation unit is OFDM symbol Agreements: For a grant based PDSCH, rate matching around RRC configured CSI-RS, if the CSI-RS is cancelled by setting SFI to “flexible” or the UE does not detect the SFI for the slot, the PDSCH still rate match around the CSI-RS RE locations. - This may not have spec impact Agreements: In 38.213 section 11.1, when PDSCH granted by DCI with CRC scrambles by C-RNTI is mentioned, add CS-RNTI, P-RNTI, SI-RNTI, RA-RNTI, and TC-RNTI to the list of RNTIs as well Agreements: For a UE-specific RRC configured UL transmission, if one OFDM symbol of the configured transmission falls on the semi-static DL symbol or a symbol SSB is transmitted as indicated, the UE shall cancel the transmission. For a UE-specific RRC configured DL reception, if one OFDM symbol of the configured reception falls on the semi-static UL symbol or a symbol PRACH is configured, the UE shall cancel the reception. Agreements: - To confirm the following working assumption: - (working assumption) At most 4 different DCI sizes are monitored by the UE per slot - At most 3 different DCI sizes are monitored per C-RNTI per slot Conclusion: - It is understood that DCI sizes to monitor do not vary dynamically from slot-to-slot (other than impact due to BWP switching) but are based on RRC configuration, CSS vs. USS, and/or active BWP. Agreements: - To confirm the following working assumption with update Working assumption: - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWP - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - FFS: Otherwise, for format 0-0/1-0, the size is given by the initial DL BWP Agreements: - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI: - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWP. - The case of TC-RNTI is a working assumption - The case of C-RNTI/CS-RNTI is at least applicable to non-CA case and Pcell in CA - FFS other cases Agreements: Confirm the following working assumption: - (working assumption) Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP Agreements: When a valid DCI with P-RNTI is detected, the following fields apply: Field; Bits; Comment. Short Messages Indicator; 1; Already captured in 38.212. Need to add interpretation. 0 = short message ([8] bits defined by RAN2 + padding to meet the DCI size) 1 = paging message scheduled (following fields). In case of short message service. Short message; [8]; Defined by RAN2. The number of bits to be checked with RAN2.. Reserved bits; To meet overall DCI size. In case of paging message. Frequency domain resource assignment. Time domain resource assignment; 4; Note: applicability of the currently agreed default table to paging or whether an additional default table or SIB1 signaled table is FFS.. VRB-to-PRB mapping; 1. Modulation and coding scheme; 5; Same MCS table as for “normal” transmission without 256QAM. TB scaling; 2; To scale TB size. Reserved bits; (fixed value); To meet overall DCI size. - Note: If initial access session defines the possibility for simultaneous short message service and paging, the DCI design may need to be updated. From a DCI payload perspective, concurrent transmission of short messages and paging is possible. - Note: The TB size and RB allocation are dependent and there may be a need to apply TB size scaling to obtain low code rates for small payloads. Agreements: - For PDSCH scheduled by P-RNTI (working assumption), and RA-RNTI, scaling factor is applied in the calculation of Ninfo of TBS determination - (Working assumption) values of scaling factor: 1, ½, ¼ - FFS the possibility of adding another value, e.g., 1/8 or 1/16 Agreements: When a valid DCI with SI-RNTI is detected in either CSS type 0 or CSS type 0a, the following fields apply: Field; Bits; Comment. Frequency domain resource assignment. Time domain resource assignment; 4; Index into default table. VRB-to-PRB mapping; 1; FFS whether this field is needed or not. Modulation and coding scheme; 5; Same MCS table as for “normal” transmission without 256QAM, only lowest part used.. Redundancy version; 2; FFS whether this field is needed or not. Reserved bits; (fixed value); To meet fallback DCI size. - Note: Need to check whether interleaved VRB-to-PRB mapping is possible for SI-RNTI as the UE does not know the CRB grid. - Note: Need to check with initial access session whether RV field is needed or not. From a DCI perspective, there is no problem of providing 2 bits for signaling the redundancy version. Agreements: - 4 bits are used for time-domain allocation field for RA-RNTI - RV0 is used for transmissions using RA-RNTI - 4 bits are used for time-domain resource allocation field for TC-RNTI Working assumption: interpret RIV in RAR for initial msg3 according to the same method as in LTE Agreements: Time domain allocation field in RAR for initial msg3 interpreted according to - use table provided by RMSI (if provided) - otherwise use default PUSCH table Agreements: SUL indicator for DCI format 0-0 is located last in the DCI (including padding) FH bit is optional in DCI format 0-1 Conclusion: - Remove UL VRB-to-PRB interleaver from Rel-15 Agreements: - To adopt the following TP for 38.213, Section 9.2.3: A UE may transmit one or more two PUCCHs on a serving cell in different symbols within a slot of symbols as defined in [4, TS 38.211]. In case of two PUCCHs in the slot, at least one of them is based on PUCCH format 0 or 2. Agreements: - TS38.212 to catpure that in case of multiplexing CSI with SR in a same PUCCH format, bit sequence order is that SR bit(s) are mapped to the UCI bit sequence first and CSI bits are mapped to the UCI bit sequence second. - The corresponding description is to be removed from 38.213 Agreements: - When HARQ-ACK PUCCH resource with PUCCH F1 overlaps with SR PUCCH resource with PUCCH F0 with the same starting symbol in a slot, - The UE transmits only HARQ-ACK in the HARQ-ACK PUCCH resource ( i.e the UE does not transmit SR if it is positive). Agreements: - When AN/SR and configured CSI PUCCH resources based on PF3 or 4 have the same starting symbols in a slot, when the UE determines the PUCCH resource set is based on the total number of AN/SR and CSI and the PUCCH resource based on ARI, if the PUCCH resource based on ARI for transmitting HARQ-ACK/SR and CSI is determined to PF2, - Drop CSI part 2 (if present). Transmit jointly encoded AN/SR and CSI part 1 on the determined PUCCH format 2 resource indicated by ARI using the mapping rules for PF2. Agreements: - When AN/SR and CSI PUCCH resources have the same starting symbols in a slot, for a wideband semi-persistent/periodic CSI reporting based on a PUCCH format 2, if the PUCCH resource based on ARI for transmitting HARQ-ACK/SR and CSI is based on format 3: - CSI (only CSI part 1) is jointly encoded with HARQ-ACK/SR, following the existing procedure of encoding and mapping on resources for PUCCH format 3 or 4 to transmit the UCI. Agreements: - When a configured CSI PUCCH resource overlaps with one or more configured SR PUCCH resources in a slot, - The UE multiplexes all UCIs on the CSI PUCCH resource using the corresponding UCI multiplexing rules for the overlapping PUCCH resources with the same starting symbol. - FFS: When one SR PUCCH resource overlaps with two non-overlapping CSI PUCCH resources Agreements: - To adopt the following TP to 9.2.5.2 of 38.213 - else, the UE selects CSI report(s) for transmission together with HARQ-ACK/SR in ascending order of , where is determined according to [6, TS 38.214]; the value of satisfies OACK+OSR+n=1NCSIreportedOCSI,n+OCRCOCRC,N≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH∙Qm∙r and OACK+OSR+n=1NCSIreported+1OCSI,n+OCRCOCRC,N+1>MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH∙Qm∙r, where is the number of CSI report bits for the CSI report in ascending order of , OCRC,N is the number of CRC bits corresponding to OACK+OSR+n=1NCSIreportedOCSI,n and OCRC,N+1 is the number of CRC bits corresponding to OACK+OSR+n=1NCSIreported+1OCSI,n. Agreements: When AN/SR and SP-CSI PUCCH resources have the same starting symbol, multiplexing AN/SR and SP-CSI on a PUCCH resource. The reference payload for CSI part 2 is based on assuming by rank 1. Agreements: - For SP-CSI on PUSCH or A-CSI-only on PUSCH colliding with SR/HARQ-ACK with the same starting symbol within the same CC, adopt the proposals in the table below SP-CSI on PUSCH without UL-SCH; A-CSI-only on PUSCH without UL-SCH; HARQ-ACK; SR; Proposal. *; Drop A-CSI-only on PUSCH, transmit SR on PUCCH. (follow LTE approach). *; *; *; Drop A-CSI-only on PUSCH, transmit SR and HARQ-ACK on PUCCH. (follow LTE approach). *; *; Drop SP-CSI on PUSCH, transmit SR on PUCCH (follow LTE approach). *; *; *; Drop SP-CSI on PUSCH, transmit HARQ-ACK and SR on PUCCH (follow LTE approach). - The symbol “*” in SR means when SR is positive - FFS when the starting symbols among UL transmissions are different in the above scenarios. Agreements: - For SP-CSI on configured PUSCH without UL-SCH colliding with a PUSCH with UL-SCH with overlapped symbols in time - When starting symbols between the two channels are aligned, drop SP-CSI (following MIMO agreements made in last meeting) - When starting symbols between the two channels are not aligned - FFS - Note: this replaces the MIMO agreements made earlier Agreements: - Adopt the following TP in Section 6.2.7 of 38.212: for to ; ; ; ; end for Conclusion: - It is clarified that based on this sentence of “A UE is not expected to receive more than one aperiodic CSI report request for a given slot” in Section 5.2.1.5.1 in 38.214 V15.0.1, for a given slot, UE is not expected to transmit CSI report triggered by more than one CSI requests received in multiple DCIs, in both CA and non-CA scenarios, unless for a given CC where it is possible that two or more DCIs may trigger the same CSI report - No spec update is necessary. Working assumption: - When single-slot PUCCH overlaps with single-slot PUCCH or single-slot PUSCH in slot n for a PUCCH group, - The UE multiplex all UCIs on either one PUCCH or one PUSCH, using the existing UCI multiplexing rule, if both following conditions are satisfied: - If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than symbol N1+X after the last symbol of PDSCH(s) - If the first symbol of the earliest PUCCH(s)/PUSCH(s) among all the overlapping channels starts no earlier than N2+Y after the last symbol of PDCCHs scheduling UL transmissions including HARQ-ACK and PUSCH (if applicable) for slot n - If at least one pair of overlapping channels does not meet the above timeline requirements, UE consider it is an error case for all UL channels in the group of overlapping channels. UE behavior is not specified. - The definition of N1 and N2 follows the same definition in current NR spec. - X and Y are non-negative integer values. - FFS on values of X and Y - FFS on timeline requirement for multiplexing UCIs on PUSCH with A-CSI. - FFS how to handle one PUCCH overlap with multiple PUSCHs which satisfy timeline requirement. - FFS: how to handle HARQ-ACK for semi-static PDSCH. - FFS multiplexing rule when AN PUCCH resource with F1 overlaps with SR PUCCH resource with F0. - FFS: how to handle semi-statically configured PUCCH overlap with semi-statically configured PUCCH or PUSCH. - Note: The above proposal does not override the dropping rules defined for ACK/SR colliding with A-CSI-only on PUSCH without UL-SCH, or ACK/SR colliding with SP-CSI on PUSCH without UL-SCH. - Note: Consider how to handle PUCCH colliding with other UL channels in NR Rel. 15 June drop when URLLC is taking into account. Agreements: For HARQ-ACK, CSI part 1, and CSI part 2 (if exists) transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1, and CSI part 2 (exists), are determined as follows: - SE is the spectrum efficiency which is code rate * modulation order - FFS: UE is not expected to transmit A-CSI on PUSCH without UL-SCH, if the actual coding rate for CSI-part 1 is smaller than T_m for a modulation order m. - When A-CSI is dropped, HARQ-ACK if exists is transmitted on PUCCH resource. - FFS the value of T_m for each modulation order m Agreements: - For PUCCH resource allocation with NUCI≤2 after a UE has a dedicated PUCCH configuration, when >8 resources are configured, - The resource index r within the resource set is derived by: r=i=0mRi-Rm+ CCEstartRmNCCE,p - m is the ARI value - NCCE,p is the total number of CCEs in CORESET p of the corresponding PDCCH - R is the resource set - ¦Ri¦ is given by: Ri=R8, if i<ModR, 8R8, if i≥ModR, 8 - CCEstart is the starting CCE index of the last received DCI within the ACK bundling window - In case of CA, CCEstart is the starting CCE index of the DCI received on the CC with the smallest CC index in the last slot where a DCI is received within the ACK bundling window - FFS the case of different SCS of CCs in CA - Note: for m>0, the summation in the equation is equivalent to having the upper bound as m-1 without substracting ¦Rm¦ - up to editor how to capture Agreements: Confirm the following working assumption: For HARQ-ACK resource allocation before a UE has a dedicated PUCCH configuration, - Frequeny hopping is always enabled for PUCCH transmission for FR1 and FR2. Agreements: For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, - One RMSI value is used for indicating cell-specific PRB offset = 0 for PUCCH duration = 2, 4, 10,14 symbols respectively. - (working assumption) One RMSI value is used for indicating cell-specific PRB offset = Floor(NBWP/4) for PUCCH duration = 14 symbols. - NBWP is the size of initial UL BWP Agreements: For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, - Up to two sets of initial CS shift indices can be specified for different RMSI values for a specific PUCCH format Agreements: For PUCCH resource allocation with NUCI≤2 after a UE has a dedicated PUCCH configuration, when >8 resources are configured, - Adopt the following equation for 38.213 subclause 9.2.3 for resource index r within the resource set: r= m*RPUCCH8+CCEstartRPUCCH8NCCE,pif m<RPUCCH mod 8m*RPUCCH8+RPUCCH mod 8+CCEstartRPUCCH/8NCCE,pif m ≥RPUCCH mod 8 Where, m is the PUCCH resource indicator field value, RPUCCH is the number of the PUCCH resources given by higher layer parameter maxNrofPUCCH-ResourcesPerSet. CCEstart is the starting CCE index of the last received DCI within the ACK bundling window, In case of CA, CCEstart is the starting CCE index of the DCI received on the CC with the largest CC index in the last slot where a DCI is received within the ACK bundling window . And NCCE,p is the number of CCEs in control resource set p. r is the PUCCH resource index within the PUCCH resource set. r=0… RPUCCH-1. Agreements: For PUCCH resource determination within a resource set before a UE has a dedicated PUCCH configuration, besides 3-bit ARI, the additional 1-bit indication is based on - CCE-index-based implicit mapping same as that after UE has a dedicated PUCCH configuration Agreements: - Adopt the tables below for RMSI time domain resource allocation - The previously agreed 4-bit indicator in DCI is not changed Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 1 L; DL-DMRS-typeA-pos; PDSCH mapping type; K0; S; L. 0; 2; Type A; 0; 2; 12. 3; Type A; 0; 3; 11. 1; 2; Type A; 0; 2; 10. 3; Type A; 0; 3; 9. 2; 2; Type A; 0; 2; 9. 3; Type A; 0; 3; 8. 3; 2; Type A; 0; 2; 7. 3; Type A; 0; 3; 6. 4; 2; Type A; 0; 2; 5. 3; Type A; 0; 3; 4. 5; 2; Type B; 0; 9; 4. 3; Type B; 0; 10; 4. 6; 2; Type B; 0; 4; 4. 3; Type B; 0; 6; 4. 7; x; Type B; 0; 5; 7. 8; x; Type B; 0; 5; 2. 9; x; Type B; 0; 9; 2. 10; x; Type B; 0; 12; 2. 11; x; Type A; 0; 1; 13. 12; x; Type A; 0; 1; 6. 13; x; Type A; 0; 2; 4. 14; x; Type B; 0; 4; 7. 15; x; Type B; 0; 8; 4. Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 2 I; PDSCH mapping type; K0; S; L. 0; Type B; 0; 2; 2. 1; Type B; 0; 4; 2. 2; Type B; 0; 6; 2. 3; Type B; 0; 8; 2. 4; Type B; 0; 10; 2. 5; Type B; 1; 2; 2. 6; Type B; 1; 4; 2. Note: From a table size perspective, it is possible to add 7 more rows with same K0 and S as above but with L=4 as well as 2 more rows S=0 and L=4. Table 5.1.2.1-x: Resource allocation for PDSCH scheduled using CORESET #0 and multiplexing pattern 3 i; PDSCH mapping type; K0; S; L. 0; Type B; 0; 4; 2. 1; Type B; 0; 6; 2. 2; Type B; 0; 8; 2. 3; Type B; 0; 10; 2. Note: From a table size perspective, additional entries are possible. Working assumption: - For PUSCH tx, 4-bit to indicate the resource allocation, as shown in Table below Table 6.1.2.1-x: Default resource allocation for PUSCH i; PUSCH mapping type; K2; S; L. 0; Type A; j; 0; 14. 1; Type A; j; 0; 12. 2; Type A; j; 0; 10. 3; Type B; j; 2; 10. 4; Type B; j; 4; 10. 5; Type B; j; 4; 8. 6; Type B; j; 4; 6. 7; Type A; j+1; 0; 14. 8; Type A; j+1; 0; 12. 9; Type A; j+1; 0; 10. 10; Type B; j+2; 0; 14. 11; Type A; j+2; 0; 12. 12; Type A; j+2; 0; 10. 13; Type B; j; 8; 6. 14; Type A; j +3; 0; 14. 15; Type A; j +3; 0; 10. Table 6.1.2.1-x: Definition of value j in determination for K2 µ; j for msg 3 initial tx; j for other cases. 0; 3; 1. 1; 3; 1. 2; 6; 2. 3; 9; 3. Agreements: For time-domain signaling for PUSCH scheduled by DCI in USS - if dedicated RRC signaling has provided a table - Use this table for PUSCH transmission scheduled by DCI in USS - else if RMSI signaling has provided a table - Use this table for PUSCH transmission scheduled by DCI in USS - else - Use the default PUSCH table For time-domain signaling for PDSCH scheduled by DCI in USS - if dedicated RRC signaling has provided a table - Use this table for PDSCH transmission scheduled by DCI in USS - else if RMSI signaling has provided a table - Use this table for PDSCH transmission scheduled by DCI in USS - else - Use pattern 1 from the agreed default table Agreements: The following text proposal is adopted to clarify that N1 processing time at least follows the semi-static configuration for DMRS, and not that resulting from the allocation. ------ Text Proposal for TS 38.214 Subclause 5.3 ------ - N1 is based on µ of table 5.3-1 that corresponds to the min (µDL, µUL) where the µDL corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted. If the higher layer parameter dmrs-AdditionalPosition is configured with a value greater than 0, or if the high layer parameter is not configured, then N1 value follows from the second column of table 5.3.1; otherwise, the N1 value follows from the first column of table 5.3-1. Agreements: The following text proposal is adopted to clarify the start position of the physical channel carrying HARQ-ACK. ------ Text Proposal for TS 38.214 Subclause 5.3 ------ If the first uplink symbol of the physical channel which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1 then the UE shall provide a valid HARQ-ACK message, where L1 is defined as the next uplink symbol with its CP starting after after the end of the last symbol of the PDSCH carrying the TB being acknowledged. <text omitted which is unchanged> - If the PDSCH is mapping type A as given in subclause 7.4.1.1 of [4, TS 38.211], and the last symbol of PDSCH is on the ith symbol of the slot where i<7, then , otherwise d1,3 = 0 ------ End of Text Proposal ------ Agreements: - To confirm the following working assumption: - For broadcast PDSCH, MCS is limited to QPSK, rank is limited to 1. Agreements: - Adopt the corresponding text proposals: ----- Text proposals for 38.214, 5.1.1.1 ----- For transmission scheme 1 of the PDSCH, the UE may assume that a gNB transmission on the PDSCH would be performed with up to 8 transmission layers on antenna ports 1000-1011 as defined in Subclause 7.3.1.4 of [4, TS 38.211], subject to the DMRS reception procedures in Subclause 5.1.6.2. <unchanged text omitted> The UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel scheduled with other RNTIs than C-RNTI. The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, or SI-RNTI and (Qm) > 2. ----- End of Text Proposal ----- Agreements: - Confirm the following working assumption: - P-RNTI PDSCH is not required to be decoded by UE in RRC_CONNECTED mode - Note: P-RNTI PDCCH may not schedule a P-RNTI PDSCH but rather carries the message by itself, which may be used for e.g. systemInfoModification, cmas-Indication, and etws-Indication Working assumption: - For baseline (Capability 1) UE processing times, for PDSCH mapping type B with 2-symbol duration, the minimum UE processing times are defined as the following where N1 is the corresponding value from Table 5.3-1 of TS 38.214: - N1 + 3 + d symbols when there is a time-domain overlap of ‘d’ symbols (d = {0,1,2}) between the scheduling PDCCH and the scheduled PDSCH Agreements: The previous WA is confirmed with updates: - While UE acquires SI upon being triggered by Paging DCI - UE is not required to decode C-RNTI PDSCH if the SI-RNTI PDSCH is overlapped with at least one symbol - In case UE autonomously monitors SI-RNTI PDCCH while monitoring C-RNTI PDCCH, and both SI-RNTI PDSCH and C-RNTI PDSCH are overlapped with at least one symbol, the UE is not required to decode SI-RNTI PDSCH - (Working assumption)The first two bullets apply unless TBS of SI-RNTI PDSCH ≤ 2976 for FR1, then UE decodes both SI-RNTI PDSCH and C-RNTI PDSCH - The first two bullets always apply in FR2 Conclusion: Consider whether further handling on UE processing time may be needed in the following cases - L1 is the length (in symbols) of a first PDSCH allocation - L1-dmrs is location of last DMRS in the allocation - L2 is the length (in symbols) of a second PDSCH allocation - L2-dmrs is location of last DMRS in the allocation - g is the delay (in symbols) between the first and second PDSCH allocation - In case where L1 – L2 – g > 0 - The DMRS locations between the first and second PDSCH allocation are different Conclusion: At least for the following conditions below, the processing times listed in Table 2 have been considered as potential candidates for specification of Capability #2, although there has been no conclusion on the final value. - Single numerology for PDCCH, PDSCH, and PUSCH and no UCI multiplexing - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - PDSCH allocation with at least 7 symbols is supported - One unicast PDSCH received and/or one unicast PUSCH transmitted within the same slot - FFS: More than one PDSCH and/or PUSCH case - Single CC - FFS: CA case with and without cross-carrier scheduling - For C-RNTI only - FFS: also applicable to the cases when C-RNTI and with other broadcast RNTIs are processed simultaneously by the UE - FFS: whether Capability #2 relaxation is applied dynamically depending on conditions - Note: Companies are also encouraged to provide processing times for 60kHz (FR1) - Note: Companies are also encouraged to provide conditions under which more aggressive processing times (within the range) could be enabled Table 2. UE Processing Times for Capability #2 Configuration; HARQ Timing Parameter; Units; 15 KHz SCS; 30 KHz SCS. Front-loaded DMRS only; N1; Symbols; [2.5-4]; [2.5-6]. Frequency-first RE-mapping; N21; Symbols; [2.5-6]; [2.5-6]. - 1If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2 in table. Conclusion: The existing agreements were made before RAN2 defined SUL and non-SUL as a single serving cell - One TB is mapped to one DL/UL carrier. - Re-transmission of a TB cannot take place on different carrier than the initial transmission. So far this implies for some companies that for SUL, the re-transmission of a TB should not take place on a different carrier than the initial transmission. Other companies believe re-transmission may take place on a different carrier in case of SUL and UL. - If intention for previous agreement was to be worded “per cell” then it would be restricted to CA - RAN1 should discuss whether to clarify the existing agreements - Note: in SUL there is only one cell and one HARQ entity Working assumption: - The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #1: - 15 kHz case: 8 + 5 - 30 kHz case: 10 + 5 - 60 kHz case: 17 + 5 - 120 kHz case: 20 + 5 The proposal in R1-1805747 is agreed. The four TPs in R1-1805731 are endorsed in principle, up to editor to refine the wording where necessary Agreements: - Adopt following TP for 38.213 subclause 8.3 Msg3 PUSCH: If the UE applies transform precoding to an Msg3 PUSCH transmission with frequency hopping, the frequency offset for the second hop [6, TS38.214] is given in Table 8.3-1. Number of PRBs in initial active UL BWP; Value of Hopping Bits; Frequency offset for 2nd hop. 0; floor(). 1; floor(). 00; floor(). 01; floor(). 10; －floor(). 11; Reserved. Agreements: - To adopt the following TP for Section 6.3 of 38.214 In case of intra-slot frequency hopping is configured for PUSCH without repetitions, the number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbolPUSCH,s/2 NsymbolPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot. Conclusion: - Confirmation of the activation/deactivation signalling for SP-CSI on PUSCH is not explicitly supported in the specs - Up to gNB implementation Agreements: - For Rel.15 Dec. drop, for both Type 1 and Type 2 PUSCH transmissions with a configured grant, when a UE is configured with data mapping Type B and K> 1, the same symbol allocation is applied across the K consecutive slots. Agreements: - Adopt the following TP regarding the number of layers in LBRM operation. --------------Start of TP, 5.4.2.1 --------------------- The bit sequence after encoding from Subclause 5.3.2 is written into a circular buffer of length for the -th coded block, where is defined in Subclause 5.3.2. For the -th code block, let if and otherwise, where, , is determined according to Subclause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Subclause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following: - maximum number of layers for one TB supported by the UE for the serving cell; - maximum modulation order configured for the serving cell, if configured by higher layers; otherwise a maximum modulation order is assumed for DL-SCH ; Conclusion: - It is RAN1 understanding that UE is not expected to handle cases where the Nref value (in 5.4.2.1) changes across transmissions for the same transport block. - No spec impact. Agreements: - Following change is adopted for TS 38.213 section. ------------------------------------------------start of text proposal------------------------------------------------ If a UE detects a DCI format 2_1 in a PDCCH transmitted in a control resource set in a slot, the set of symbols indicated by a field in DCI format 2_1 includes the last symbols prior to the first symbol of the control resource set in the slot where is the value of higher layer parameter Monitoring-periodicity-PDCCH-slot, is the number of symbols per slot, is the subcarrier spacing configuration for a serving cell with mapping to a respective field in the DCI format 2_1, is the subcarrier spacing configuration of the DL BWP where the UE receives the PDCCH conveying the DCI format 2_1. If the UE is configured with higher layer parameters UL-DL-configuration-common or UL-DL-configuration-common-Set2, symbols indicated as uplink by UL-DL-configuration-common or UL-DL-configuration-common-Set2 are excluded from the last symbols prior to the first symbol of the control resource set in the slot. The resulting set of symbols includes a number of symbols that is denoted as. -------------------------------------------end of text proposal--------------------------------------------------------- Agreements: - Regarding the discrepancy in RAN1 and RAN2 agreements w.r.t. initial active BWP vs. the up to configured 4 BWPs, to adopt the RAN2 agreements and make necessary RAN1 spec update - The corresponding TP is shown below: ------------------------------------- Begin Text Proposal Sec 7.3.1.1.2 of 38.212 -------------------------------------- < Omitted > - Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL bandwidth parts configured by higher layersdefined in Table 7.3.1.1.2-1. The bitwidth for this field is determined as bits, where - and the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id if the higher layer parameter BandwidthPart-Config configures up to 3 bandwidth parts and the initial bandwidth part is not included in higher layer parameter BandwidthPart-Config; - otherwise and the bandwidth part indicator is defined in Table 7.3.1.1.2-1.; - is the number of configured bandwidth parts according to higher layer parameter BandwidthPart-Config. < Omitted > Table 7.3.1.1.2-1: Bandwidth part indicator Value of BWP indicator field; Bandwidth part. 1 bit; 2 bits. 0; 00; First bandwidth part configured by higher layers. 1; 01; Second bandwidth part configured by higher layers. 10; Third bandwidth part configured by higher layers. 11; Fourth bandwidth part configured by higher layers. ------------------------------------- End Text Proposal Sec 7.3.1.1.2 of 38.212 ----------------------------------------- ------------------------------------- Begin Text Proposal Sec 7.3.1.2.2 of 38.212 -------------------------------------- < Omitted > - Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL bandwidth parts configured by higher layersdefined in Table 7.3.1.1.2-1. The bitwidth for this field is determined as bits, where - and the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id if the higher layer parameter BandwidthPart-Config configures up to 3 bandwidth parts and the initial bandwidth part is not included in higher layer parameter BandwidthPart-Config; - otherwise and the bandwidth part indicator is defined in Table 7.3.1.1.2-1.; - is the number of configured bandwidth parts according to higher layer parameter BandwidthPart-Config. < Omitted > ------------------------------------- End Text Proposal Sec 7.3.1.2.2 of 38.212 ----------------------------------------- Agreements: - To adopt the following TP: ------------------------------------- Begin Text Proposal Sec 12 of 38.213 -------------------------------------- < Omitted > For each DL BWP or UL BWP in a set of DL BWPs or UL BWPs, respectively, the UE is configured the following parameters for the serving cell as defined in [4, TS 38.211] or [6, TS 38.214]: - a subcarrier spacing provided by higher layer parameter DL-BWP-mu or UL-BWP-mu; - a cyclic prefix provided by higher layer parameter DL-BWP-CP or UL-BWP-CP; - a PRB offset with respect to the PRB determined by higher layer parameters offset-pointA-low-scs and ref-scs and a number of contiguous PRBs provided by higher layer parameter DL-BWP-BW or UL-BWP-BW; - a starting PRB and a number of contiguous PRBs indicated by higher layer parameter locationAndBandwidth which is interpreted as RIV according to [5.1.2.2.2, TS 38.213], setting =275 and the starting PRB is a PRB offset with respect to the PRB indicated by higher layer parameters offsetToCarrier and subcarrierSpacing. - an index in the set of DL BWPs or UL BWPs by respective higher layer parameters DL-BWP-index or UL-BWP-index; - DCI format 1_0 or DCI format 1_1 detection to a PDSCH reception timing values by higher layer parameter DL-data-time-domain, PDSCH reception to a HARQ-ACK transmission timing values by higher layer parameter DL-data-DL-acknowledgement, and DCI format 0_0 or DCI format 0_1 detection to a PUSCH transmission timing values by higher layer parameter UL-data-time-domain; < Omitted > ------------------------------------- End Text Proposal Sec 12 of 38.213 -------------------------------------- The draft LS is approved. Final LS (accepting all change marks) in R1-1805726 Agreements (RAN2 impact) - The existing higher-layer parameter SCS-SpecificCarrier provided to a UE via a dedicated RRC signal is also applicable to Scell & SUL Agreements: - It’s RAN1’s understanding that the active DL (or UL) BWP switching delay (i.e. T μs) for two types of UE capability will be specified in RAN4 specs. - Send LS to RAN4 – R1-1805727, which is approved, final LS in R1-1805755 - For DCI-based active DL (or UL) BWP switching, a UE is not expected to switch its active DL (or UL) BWP based on an active DL (or UL) BWP switching DCI which is considered as invalid where invalidity is defined as when the signaled K0 (or K2) value in the DCI can’t guarantee the active DL (or UL) BWP switching delay equal to or larger than the one specified in RAN4 specs to the UE based on the UE’s capability. Agreements: Confirm the following working assumption with updates: - Sizes of all DCI bitfields in DCI formats 0-1 and 1-1 in USS determined by current BWP. Data transmitted on the BWP indicated by the BWP index. If the BWP index activates another BWP, transform as follows: - Zero-pad too small bitfields to match the new BWP - Truncate too large bitfields to match the new BWP - The truncation is done from MSB (including the bit indicating the resource allocation type) - Zero-padding is done for MSB Conclusion: - It is confirmed that when a UE is configured with semi-static HARQ-ACK codebook, the HARQ association set is determined based on the semi-static (via RMSI/UE-specific) UL/DL configurations for PDSCH occasions in addition to HARQ-ACK in response to SPS release - This is already reflected in the current specs Agreements: - Confirm WA from RAN1#92 with the following update - When a UE is configured with semi-static HARQ-ACK Codebook, on a per cell basis: - If the UE indicates capability to receive more than one unicast PDSCH per slot, it assumes a max number of non-overlapping candidate unicast PDSCH occasions per slot as determined by the SLIV in the configured pdsch-symbolAllocation table. - Otherwise, the UE is expected to receive only one unicast PDSCH per slot, and HARQ-ACK association set assumes one unicast PDSCH per slot - The UE is not expected to receive SPS PDSCH release and unicast PDSCH in a same slot - Handling HARQ-ACK for PDCCH for SPS release follows the same way as in LTE Agreements: - For unpaired spectrum and DL/UL BWP switching - UE does not transmit HARQ-ACK for PDSCH reception(s) on the DL BWP prior to DL/UL BWP switching Agreements: - When UE is to rate-match a PDSCH around PDCCH scheduling the PDSCH, and PDCCH is received in the CORESET configured with WB RS, the UE shall rate-match around - The union of REs of the PDCCH scheduling the PDSCH and the WB RS REs for the configured CORESET Agreemets: - To adopt the following TP to 5.1.4 of 38.214 When receiving PDSCH not conveying [RAR, OSI, Paging, Msg4, SIB1], Tthe REs corresponding to the union of configured or dynamically indicated resources in Subclauses 5.1.4.1, 5.1.4.2 and resources corresponding to SS/PBCH are declared as not available for PDSCH in Subclause 7.3.1.5 of [4, TS 38.211]. Agreements: - When an PDCCH candidate collides with any RB-level resource (FFS RE-level), where PDSCH is semi-statically not mapped, as described in TS38.214 Section 5.1.4.1 then - The UE is not required to monitor the PDCCH candidate overlapping with the resource even only by one RE Agreements: Clarify in the specification that the periodicity of bitmap-pair, when bitmap-3 is not configured, is equal to the length of the bitmap-2. - For bitmap-pair not included in one or two groups of resource sets (semi-static RMR) and associated with two-slot long bitmap-2, first slot of bitmap-2 corresponds to even slots, second slot corresponds to odd slots. - For bitmap-pair not included in one or two groups of resource sets (semi-static RMR) and associated with one slot long bitmap-2, corresponds to every slot. Agreements: For resources of a bitmap-pair configured with bitmap-3 and of a CORESET included in one or two groups of resource sets (dynamic RMR), the dynamic indication of availability for PDSCH applies to a set of slot(s) where the bitmap-pair/CORESET actually exists among the slots of PDSCH scheduled.

Channel coding

Agreement: The UE is not expected to transmit or receive the bit sequence if K+npc>E, where npc is the number of parity check bits as defined in Section 5.3.1.2; - Editor to capture this in Section 5.3 of TS 38.212 Agreement: Polar code segmentation is applied if ( and ) or A >= 1013 - Editor to capture this in Section 6.3.1.2.1 of TS 38.212 Conclusion: It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB. Note: This does not have any RAN1 specification impact. A note similar to the one in TS36.321, Subclause 5.3.2.2 can be added to 38.321.

NR-LTE co-existence

Conclusion: Placement of the SUL CIF field in DCI needs to be corrected. - Details should be handled in the DCI discussions in scheduling and HARQ AI Agreement: In Rel-15, Case 1 HARQ timing is supported for the LTE serving cells only for the following cases - A single FDD cell - DL CA with FDD-FDD CA and FDD-TDD CA, where the LTE PCell is FDD and there is only one UL carrier - The UE is not configured for cross-carrier scheduling Agreement: For a UE operating in EN-DC when configured with Case 1 HARQ timing on an FDD PCell, the following is supported for the LTE serving cells - For an FDD SCell, the same DL HARQ timing as the PCell is applied for the FDD SCell, - For a TDD SCell, the same DL HARQ timing as the PCell is applied restricted to the set of DL/special subframes on the TDD SCell - The UL HARQ timing for the FDD PCell is the same for the CA and non-CA cases

UL power control

Agreement Definition of PUSCH open-loop parameter j=0 should also include Msg3 transmission in the RRC_CONNECTED state Agreement For PUSCH Msg3 in the RRC_CONNECTED state, UE shall use the SSB or CSI-RS associated with the PRACH for the pathloss measurement Agreement For PUSCH Msg3 and UE in RRC_CONNECTED state, if the UE is configured with more than one closed-loop process for PUSCH, the CL-PC index shall be fixed to l=0. Agreement For PUCCH without high layer parameter PUCCH-Spatial-relation-info, the closed loop index l should also be set to a fixed value, i.e., l=0. Agreement: Single K1 value is supported for PUCCH power control - The value will be decided in RAN1#93 Agreement Single K2 value is supported for PUCCH power control - The value will be decided in RAN1#93 Agreement: UE uses one selected SSB or CSI-RS resource among SSB(s) and CSI-RS resource(s) associated with indicated RACH resource(s) in the handover message for pathloss estimation for PRACH during handover. Agreement: - For group-common TPC command in DCI format 2-2, both closed loops can be adjusted dynamically Working Assumption For group-common TPC command in DCI format 2-2, One bit closedloopindex field is present along with the TPC command in the DCI format 2_2 to indicate which closed loop the TPC command applies to - When a UE is configured with 1 closed loop, the bit closedloopindex field is NOT present. - When a UE is configured with 2 closed loops, the bit location of closedloopindex field follows the 2 bit TPC command. Notes: UE assumes that the start bit of the TPC command is X*tpc_index, where X=3 if configured with 2 closed-loop process, otherwise X=2. Conclusion For further discussion in RAN1#93 - K value for group common TPC is the minimum of the configured K2 values plus one. - Applies for both PUCCH, PUSCH, SRS - Power scaling Agreement Default parameter setting for virtual PHR How to set {j, qd, l} - For j, P0alphasetindex = 0 of p0-pusch-alpha-setconfig - For qd, pusch-pathlossreference-index = 0 of pusch-pathloss-Reference-rs - For l, l =0 - Note: If the UE is configured with multiple UL BWPs, j, qd, l corresponding to lowest BWP ID are used Conclusion: Type 3 PHR is supported only in case PUSCH is not configured Agreement - At least from RAN1 perspective, it is not needed to include Pcmax,c together with PH in case of virtual PHR Agreement Priority rule for SUL - For simultaneous uplink transmissions that are power-limited, when one or multiple serving cell(s) are configured with both UL and SUL carriers - Existing priority rule based on signals/channels content - In case transmissions with the same priority level on the two UL carriers, - the UL carrier which is configured for PUCCH has higher priority

UE feature

Agreement Feature 0-13 “Phase coherence across non-contiguous UL symbols in slot in the transmission of one channel” deleted from the NR UE feature list as this feature is not supported in Rel-15. Note : Creat the following new feature if the following new feature is supported in Rel-15 after other discussion related to R1-1803374. 1-13: Simultaneous Transmission of PRACH and SR (new): Optional with capability signaling Conclusion NR UE feature related to June version of Rel-15 specification will be discussed separately. Conclusion: “Optional” in the UE feature list means it’s optional with UE capaiblity signaling R1-1805728 NR UE feature list NTT DOCOMO, INC., AT&T The LS is agreed and final LS in R1-1805752

NR URLLC

Agreements: - The two BLER targets that are configurable for URLLC for CSI reporting are: - Option B. (10-1, 10-5) - Note: The definition of the test case for the BLER target of 10-5 should take into account channel and interference variations and estimation errors. Agreements: - Highest spectral efficiency for CQI based on 10-5 BLER target for URLLC is no more than 772/1024*6 - Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6 - It doesn’t necessarily mean that the CQI table introduced for eMBB can not be directly reused for URLLC – it’s still a separate discussion - Note that - Whether or not to have two tables or a single table covering both BLER targets is a separate issue Agreements: - In total, there are two CQI tables for URLLC CQI reporting - The first table for URLLC CQI reporting is the same as the existing 64QAM CQI table without any change, which is for BLER target 10-1 for URLLC - Note: this means the agreement on “Highest spectral efficiency for CQI based on 10-1 BLER target for URLLC is no more than 873/1024*6” is overturned - The new table will have entries corresponding to BLER target 10-5 - For CSI reporting, the CQI field is 4-bit. Agreements: - For BLER 10-5, - Companies are encouraged to perform simulations for the new CQI table for URLLC, including - The lowest SE entry - E.g., 30~50/1024*2 - Note that the highest SE entry of no more than 772/1024*6 is already agreed - Consider using approximately equally spaced SNR values - Other options are not precluded - Whether or not some existing CQI entries for BLER 10-1 can be reused - Consider exsiting CQI entires when applicable - In total 15 CQI entries (+1 OOR entry) - In performing the simulations, consider - Fading channel (TDL-A, 30ns) & (TDL-C, 300ns) - Other options are not precluded - Payload of 32 bytes - Other payload sizes can also be considered, up to each company - SNR at 5% geometry for the lowest SE entry - Other options are not precluded - For other simulation assumptions, refer to agreements from RAN1#92 - Similar considerations are also applicable to the MCS table evaluations Conclusion: - There is no consensus in Rel-15 to support: - Defining a new DCI format(s) that has a smaller DCI payload size than DCI format 0-0 and DCI format 1-0 unicast data, and/or - For a given carrier, PDCCH repetitions over same or multiple PDCCH monitoring occasion(s) of the same or multiple CORESET and search space Conclusion: - There is no concensus in Rel-15 to support handling inter-UE UL multiplexing of transmission with different reliability requirements (including the potential need for UL UE pre-emption)

Email discussions after the meeting are listed below.

  - [92b-NR-01] Email discussion/approval of reply LS to R1-1803581

    - Agreed in R1-1705782

  - [92b-NR-02-xxx] xxx: spec #: Editors to prepare CRs capturing the agreements and working assumptions

  - [92b-NR-03] Email discussion come-up with identifying necessary information (e.g., periodicities, offsets, occasions) for updating RRC signaling searchSpace such that it includes the set of periodicities defined for ssb-PeriodicityServingCell. Aim to target providing an LS to RAN2

    - Agreed in R1-1805440

  - [92b-NR-04] Way Forward on NR Peak Data Rate Formula Email discussion/approval including the related RAN2 LS(s)

    - Agreed as follows

1) Motivation for Scaling Factor (response to LS R1-1801324 from RAN2) - Scaling factor is used to reflect the association of capability mismatch between the baseband capability and RF capability for both SA UE and NSA UE. - Scaling factor is used to scale down maximum throughput of NSA UEs operating in EN-DC scenario that share hardware resources between LTE and NR. - RAN1 considers the described scaling factor signalled per band in per band combination per UE capability as an important parameter. RAN1 understands that RAN2 is responsible for the overall design of the UE capabilities and that RAN2 has to ensure a reasonable size of the overall UE capability structure. Hence, RAN1 kindly asks RAN2 to aim to include the scaling factor into the capability signalling if possible from a capability signalling perspective. 2) Updates to the scaling factor (update to the LS R1-1721733 to RAN2) • Scaling factor, f(j) , are {1, 0.8, 0.75, 0.4} • NR supports separate scaling factor indication for DL and UL • f(j) is signalled per band in per band combination per UE capability signalling, where f(j) applies to j-th NR component carrier

Outgoing LS is listed below.

  - R1-1805765	LS on time domain resource allocation

  - R1-1805766	LS on RRC parameters for NR

  - R1-1805712	LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth

  - R1-1805653	LS on Maximum TBS for PDSCH containing RMSI

  - R1-1805759	Reply LS response on handling parallel RA procedure and SR procedure

  - R1-1805657	Reply LS on Message 3 size for NR

  - R1-1805760	LS on Rx beam selection for RRM measurements

  - R1-1805717	LS on SSB frequency location of PScell

  - R1-1805762	Reply LS on RAN2 agreements on RRM

  - R1-1805761	Reply LS on RAN1 on Beam- and Cell- Radio-Link-Monitoring

  - R1-1805621 	LS on time domain behavior of CSI-RS and CSI-IM

  - R1-1805627	LS on UL cross carrier beam indication	Samsung

  - R1-1805622	LS on beamFailureRecoveryTimer

  - R1-1805623	LS on NR BFR

  - R1-1805626	LS on NR CSI-RS

  - R1-1805625	LS on aperiodic SRS triggering

  - R1-1805763	LS for supporting aperiodic TRS

  - R1-1805726	Reply LS on SI Reception in BWP

  - R1-1805755	LS on active BWP switching delay specification

  - R1-1805752	LS on NR UE features

- RAN1 #93 (May 2018)

General

The TPs in R1-1806496 are endorsed.

Initial access and mobility

Agreements: - For connected mode, no explicit EPRE offset between SSS and PDCCH is specified by RAN1 in Rel-15. - Note: the EPRE offset between SSS and PDCCH may be implicitly derived from other parameters, e.g. power offset of SSB to NZP CSI-RS and power offset of NZP CSI-RS to PDCCH. This does not mandate any gNB behaviour. Conclusion: - It is understood that the resource grid of Nsize, ugrid, x is the same as the CarrierBandwidth in 38.331 - Up to editor how to capture Agreements: Update the previous agreements as follows: - In initial cell selection and idle mode, the UE may assume that the EPRE offset between PDCCH DMRS and SSS in the QCL’ed SSB is in the range [X, Y]dB when PDDCH with CRC scrambled by SI-RNTI, P-RNTI and RA-RNTI, where X=-8 & Y=8 - FFS: value of X and Y and dependence on numerology - ¦X¦ & Y are no more than 12 Agreements: - Adopt the following text proposal to 38.211 section 7.4.3.1 below: The quantity is the subcarrier offset from subcarrier 0 in common resource block to subcarrier 0 of the SS/PBCH block, where the 4 least significant bits of are given by the higher-layer parameter ssb-SubcarrierOffset and for SS/PBCH block type A the most significant bit of is given by in the PBCH payload as defined in subclause 7.1.1 of [4, TS 38.212]. When ssb-SubcarrierOffset is not provided, is derived from the frequency difference between the SS/PBCH block and Point A. Agreements: k0μ is computed using k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ/2NscRB -Ngrid,xstart,μ0+Ngrid,xsize,μ0/2NscRB2μ0-μ where μ0 is the largest subcarrier spacing configured. Agreements: - Adopt the following change “…to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters.” to sections 7.4.1.1.2, 7.4.1.3.2 and 7.4.3.1 in 38.211. Agreements: - For TDD, the UE may assume that the SFN is aligned among cells of the same frequency layer Agreements: - Adopt to below TP on 38.213, section 13: If a UE detects a first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for for FR1 or for for FR2, the UE may determine the global synchronization channel number (GSCN) of a second SS/PBCH block having a control resource set for an associated Type0-PDCCH common search space as . is the GSCN of the first SS/PBCH block and is a GSCN offset provided by Table 13-16 for FR1 and Table 13-17 for FR2. If a UE detects the second SS/PBCH block at according to the indication in the first SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present as specified in sub-clause 4.1, UE may ignore the assistance information related to GSCN of SS/PBCH locations in its SS/PBCH block search. If a UE detects a SS/PBCH block and determines that a control resource set for Type0-PDCCH common search space is not present, and for for FR1 or for for FR2, the UE may assume determines that there is no SS/PBCH block having an associated Type0-PDCCH common search space within a GSCN range . and are respectively determined by the four most significant bits and the four least significant bits of RMSI-PDCCH-Config. If UE has not detected within a reasonable time any SS/PBCH block with the associated control resource set for Type0-PDCCH common search space being present, UE may ignore the assistance information related to GSCN of SS/PBCH locations in its SS/PBCH block search. Agreements: - NR supports 2-bits RV field in the DCI with SI-RNTI for CSS type 0 and CSS type 0a. Agreements: - Make the following modification on Table 13-2 in TS 38.213. Index; SS/PBCH block and control resource set multiplexing pattern; Number of RBs; Number of Symbols; Offset (RBs). 0; 1; 24; 2; 5. 1; 1; 24; 2; 6. 2; 1; 24; 2; 7. 3; 1; 24; 2; 8. 4; 1; 24; 3; 5. 5; 1; 24; 3; 6. 6; 1; 24; 3; 7. 7; 1; 24; 3; 8. 8; 1; 48; 1; 18. 9; 1; 48; 1; 20. 10; 1; 48; 2; 18. 11; 1; 48; 2; 20. 12; 1; 48; 3; 18. 13; 1; 48; 3; 20. 14; Reserved. 15; Reserved. Agreements: - To adopt the following TP to Section 10 of 38.213: For monitoring of a PDCCH candidate If the UE has received ssb-PositionsInBurst in SystemInformationBlockType1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if monitoring for PDCCH candidate is not for a Type0-PDCCH common search space and at least one RE for monitoring a PDCCH candidate for a DCI format with CRC not scrambled by SI-RNTI on the serving cell overlaps with respective at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SystemInformationBlockType1, the UE is not required to monitor the PDCCH candidate. - If a UE has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if monitoring for PDCCH is not for a Type0-PDCCH common search space and at least one RE for monitoring a PDCCH candidate for a DCI format with CRC not scrambled by SI-RNTI on the serving cell overlaps with respective at least one RE corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon, the UE is not required to monitor the PDCCH candidate. - If the has not received both ssb-PositionsInBurst in SystemInformationBlockType1 and ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell and if the UE monitors the PDCCH candidate for a Type0-PDCCH common search space on the serving cell according to the procedure described in Subclause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. Agreements: - Adopt Option 2 in LS R1-1805712 for supporting the RMSI CORESET configuration for {SSB SCS, minimum channel bandwidth} = {15kHz, 10MHz}. - Send a reply LS to RAN4 informing RAN1’s decision to adopt Option 2 – R1-1807819, which is approved and final LS in R1-1807831 - Adopt the corresponding TP (Section 13, 38.213) as follows: Table 13-1: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz Table 13-2: Set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space when {SS/PBCH block, PDCCH} subcarrier spacing is {15, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz Conclusion: - It is understood that for the 5-bit MCS indication in PDCCH DCI for broadcast scheduling, only the entires associated with QPSK modulation are valid entries - Up to editor how to capture Agreements: - NR supports in one PDCCH containing an indication in paging DCI for simultaneous short messages and scheduling information, where the short messages (w/o PDSCH) can be used for all UEs while the scheduling information is intended for Idle UEs only - Replace the existing 1-bit indication in TS38.212 with 2 bits with the following indication: - 00: Reserved - 01: Paging scheduling only - 10: Short Message only - 11: Paging scheduling and Short Messages - Note: The above assumes a single DCI format/structure in which the bit field location is determined based on both paging scheduling and short messages being present. This and other remaining issues related to DCI designs will be finalized in the DCI session. - Note: the above support is subject to size-limitations for scheduling information and the actual short messages - To send LS to RAN2 – R1-1807750, which is approved with one update “like to ask”, final LS in R1-1807763 Agreements: - When SS/PBCH block and control resource set multiplexing pattern 2 or 3 is configured in PBCH and initial DL BWP is active, for SS/PBCH block based RRM, RLM and [BM] measurement purposes only, UE is expected to be able to measure the SS/PBCH block associated with control resource set configured in PBCH. - Note: When SS/PBCH block and control resource set multiplexing pattern 2 or 3 is configured in PBCH and initial DL BWP is active, SS/PBCH based RRM, RLM, and [BM] is still possible when the initial active DL BWP does not contain SS/PBCH The draft LS is approved with updates - Remove “Reply” in the title - RAN4 “to” instead of “cc” - To RAN2, RAN4 - Action – add RAN4 as well Final LS in R1-1807893 Agreements: - For the previous working assumption “For the Semi-static UL/DL configuration in RMSI, only PRACH occasions within the UL part and X part are valid as long as it does not precede or collide with an SSB in the RACH slot, and it is at least N symbols after DL part and/or the last symbol of an SSB”, N = 2 for Msg1 SCS = 120kHz Agreements: - Confirm the current WA on starting OFDM symbol number for all formats using short sequences in FR1 and unpaired spectrum Agreements: - For PRACH configuration table for FR2, for the configuration indices - 8 - 22 - 37 - 47 - 75 - 104 - 123 - 152 - 163 - 181 - 191 slot numbers 7 & 15 are supported. Working assumption: For FR1 and unpaired spectrum: - Remove 4 to 6 entries from formats 0-3 - Reduce format B1 with 6 entries - These removed/reduced entries are then used to add entries for format A2 (3 entries), A3(3 entries) and format B4 (4 entries) For FR2 and unpaired spectrum: - Reduce format B1 with 6 entries - The 6 entries are then used to add entries for format A2 (2 entries) and B4(4 entries) The above working assumption is obsolete due to the agreements below Agreements: - The following entries for FR1 and unpaired spectrum are removed: 15; 0; 1; 0; 1; 0; -; -; 0. 22; 0; 1; 0; 3,4,8; 0; -; -; 0. 57; 3; 1; 0; 1; 0; -; -; 0. 68; 3; 1; 0; 6,7,8,9; 0; -; -; 0. Agreements: - The following 8 entries for format B1 for FR1 and unpaired spectrum are deleted 131; B1; 16; 1; 9; 2; 2; 6; 2. 132; B1; 8; 1; 9; 2; 2; 6; 2. 134; B1; 2; 1; 2,3,4,7,8,9; 2; 1; 6; 2. 135; B1; 2; 1; 8,9; 2; 2; 6; 2. 137; B1; 2; 1; 7,9; 8; 1; 3; 2. 147; B1; 1; 0; 3,4,8,9; 2; 1; 6; 2. 148; B1; 1; 0; 3,4,8,9; 2; 2; 6; 2. 149; B1; 1; 0; 0,1,2,3,4,5,6,7,8,9; 8; 1; 3; 2. Agreements: - The following two entries for format A2 for FR1 and unpaired spectrum are updated: 92; A2; 4 2; 1; 2,3,4,7,8, 9; 0; 12; 3; 4. 94; A2; 82; 1; 4 ,7,9; 0; 12; 3; 4. Agreements: - The following entries for format A2, A3 and B4 for FR1 and unpaired spectrum are added: Format A2: A2; 1; 0; 2; 0; 1; 3; 4. A2; 1; 0; 7; 0; 1; 3; 4. A2; 1; 0; 2,7; 0; 1; 3; 4. Format A3: A3; 1; 0; 2; 0; 1; 2; 6. A3; 1; 0; 7; 0; 1; 2; 6. A3; 1; 0; 2,7; 0; 1; 2; 6. Format B4: B4; 2; 1; 9; 0; 1; 1; 12. B4; 1; 0; 9; 0; 1; 1; 12. B4; 1; 0; 2; 0; 1; 1; 12. B4; 1; 0; 1; 0; 1; 1; 12. B4; 1; 0; 7; 0; 1; 1; 12. B4; 1; 0; 4; 0; 1; 1; 12. Agreements: - The following entries are deleted for format B1 in FR2 and unpaired spectrum: 88; B1; 16; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 90; B1; 8; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 92; B1; 4; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. 94; B1; 4; 1; 3,7,11,15,19,23,27,31,35,39; 2; 1; 6; 2. 95; B1; 2; 1; 4,9,14,19,24,29,34,39; 2; 1; 6; 2. Agreements: - The following entries for FR2 and unpaired spectrum are updated: 78; A3; 1; 0; 9,11,13; 2; 1; 2; 6. 127; B4; 1; 0; 19,39; 2; 2; 1; 12. 128; B4; 1; 0; 3,5,7,9,11,13; 02; 1; 1; 12. 133; B4; 1; 0; 7,15,23,31,39; 0; 1; 1; 12. 135; B4; 1; 0; 23,27,31,35,39; 2; 2; 1; 12. 136; B4; 1; 0; 3,5,7,9,11,13,15,17,19,21,23,25; 2; 1; 1; 12. 138; B4; 1; 0; 4,9,14,19,24,29,34,39; 2; 2; 1; 12. 141; B4; 1; 0; 3,7,11,15,19,23,27,31,35,39; 2; 1; 1; 12. 144; C0; 16; 1; 4,9,14,19,24,29,34,39; 0; 2; 7; 2. 146; C0; 8; 1; 4,9,14,19,24,29,34,39; 0; 1; 7; 2. 193; C2; 1; 0; 23,27,31,35,39; 7; 2; 1; 6. 195; C2; 1; 0; 4,9,14,19,24,29,34,39; 7; 2; 1; 6. 198; C2; 1; 0; 3,7,11,15,19,23,27,31,35,39; 7; 1; 1; 6. Agreements: - The following entries are added for format A2, A3 and B4 for FR2 unpaired spectrum Format A2: A2; 1; 0; 3,5,7,9,11,13; 0; 1; 3; 4. Format A3: A3; 1; 0; 3,5,7,9,11,13; 0; 1; 2; 6. Format B4 B4; 1; 0; 9,11,13,15,17,19; 0; 1; 1; 12. B4; 1; 0; 3,5,7,9,11,13,15,17,19,21,23,25; 0; 2; 1; 12. B4; 1; 0; 7,15,23,31,39; 0; 2; 1; 12. Agreements: - Confirm the following working assumption with updates - If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2For the Semi-static UL/DL configuration in RMSI, PRACH occasions in the UL part are always valid, and only a PRACH occasions within the UL part and X part is are valid as long as it does not precede or collide with an SSB in the RACH slot and it is at least N symbols after the DL part and/or the last symbol of an SSB. - N = 0 for Msg1 SCS = 1.25kHz, 5kHz - N = 2 for Msg1 SCS = 15kHz - N = 2 for Msg1 CS = 30kHz - N = 2 for Msg1 SCS = 60kHz - N = 2 or 3 for Msg1 SCS = 120kHz - Note for format B4, N = 0 irrespectively of Msg1 SCS - UE is not expected to receive DL signals during any valid RACH occasion and N symbols before valid RO occasions - The SSB to RO association pattern period shall repeat at most every 160 ms - If there are leftover ROs after an integer number of SSB to RO association period(s) within the SSB to RO association pattern period. Those leftover ROs are invalid. - Note: In the UL/DL semi-static configuration, 0 UL slots and 0 UL symbols can be configured Agreements: - To adopt to the following TPs: 38.213 Section 8.1 <Unchanged parts> If a UE is provided higher layer parameter tdd-UL-DL-ConfigurationCommon, or is also provided higher layer parameter tdd-UL-DL-ConfigurationCommon2, valid PRACH occasions are ones that include uplink symbols or flexible symbols that start at least symbols after a last downlink symbol or a last SS/PBCH block transmission symbol a PRACH occasion is valid if - It is within the UL symbols or, - It does not precede or collide with an SS/PBCH block in the RACH slot, and it is at least symbols after the last downlink symbol right before the PRACH occasion and the last symbol of an SS/PBCH block right before the PRACH occasion, where is provided in Table 8.1-2 as a function of the preamble subcarrier spacing value. For preamble format B4 [4, TS 38.211], . <Unchanged parts> -----------Text Proposal-------- 38.213 Section 11.1 <Unchanged parts> For a the set of symbols of a slot indicated to a UE by higher layer parameter prach-ConfigurationIndex in RACH-ConfigCommon for PRACH transmissions, occupied by valid PRACH occasions as described in Subclause 8.1 and symbols before valid PRACH occasions where is provided in Table 8.1-2 and for preamble format B4 [4, TS 38.211], , the UE does not receive PDSCH or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect theis set of symbols of the slot to be indicated as downlink by higher layer parameters tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationCommon2, or tdd-UL-DL-ConfigDedicated. <Unchanged parts> Agreements: -------------- Text proposal for 38.211, section 6.3.3.2 --------------------------- <omitted > For the purpose of slot numbering in the tables, the following subcarrier spacing shall be assumed: - for PRACH preamble formats 0 – 3 - for PRACH preamble formats A1, A2, A3, B1, B2, B3, B4, C0, C2 where is the configured PRACH subcarrier spacing. - 15kHz for FR1 - 60kHz for FR2 For paired spectrum and within the same frequency range, and L = 4, - the UE may for handover purpose assume an absolute value of the relative time difference between radio frame i in the current cell and the target cell is less than 153600∙TS, if the any of the following conditions of the PRACH configurations are met: - for PRACH configurations in which x is not equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3 - for PRACH configurations in which x is equal to 1 in Tables 6.3.3.2-2 and 6.3.3.2-3 and the association period in Table 8.1-1 of [38.213] is not equal to 1 ================================================================================ Agreements: Confirm the following working assumption. - For PDCCH ordered CFRA, the minimum timing gap between PDCCH order reception and Msg1 transmission is - N2+BWPSwitching+ Delay, - If BWP switching is not required, BWPSwitching=0; otherwise, BWPSwitching is up to RAN4 - N2 refers to the UE processing time value determined in the HARQ/scheduling session with front loaded plus additional DMRS and UE capability #1 - N2 is dependent on the SCS of Msg1 - Working assumption: For Msg1 with 1.25 kHz or 5 kHz SCS, calculation of N2 uses 15 kHz SCS. - Delay includes at least MAC layer delay in initializing PRACH - Delay = 250 us for FR2, Delay = 500 us for FR1 Agreements: Update previous agreements as follows: - PDCCH order for RACH procedure includes the following fields: - Random Access Preamble index – 6 bits. Indicating which Random access preamble to use in case of contention-free random access procedure, or the value 000000 in case of contention-based random access procedure - For CFRA only: - FFS BWP index. Indicating which BWP to transmit the Random access preamble on (no BWP index is necessary in Rel-15) - SUL indicator – 1 bit. Indicating whether to transmit the Random access preamble on SUL or normal uplink carrier - SSB index - 6 bits are used to indicate an SSB index, which, in turn, identifies a group of RACH occasions - PRACH occasion Mask index – 3 4 bits are used to indicate the relative RACH occasion PRACH Mask index that corresponds to the RO(s) in the set of RACH occasions identified by the indicated SSB index the indicated SSB index identified group of RACH occasions - RAN1 supports RAN2’s agreements regarding the PRACH table as in R1-1805803. - The set of RACH occasions identified by the indicated SSB index within one association period are ordered in the following way: - In the order of increasing frequency multiplexed RACH occasions and then - In the order of increasing time multiplexed RACH occasions within a RACH slot and then - In the order of increasing RACH slot indices To draft reply LS to RAN2 based on the above agreements and RAN2 LS – R1-1807889, which is approved and final LS in R1-1807892 Agreements: Confirm the following working assumption with updates in red: For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot n+k where k-1 is equal to ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where For a TA command in MAC-CE received on slot n, the corresponding adjustment of the uplink transmission timing shall apply from the beginning of slot m, where slot m is determined starting from the end of slot n with an additional offset of ceil((duration of N1 + L2 + duration of N2 + TA_max)/slot_duration) and where - N1 refers to the value determined in control session with front loaded plus additional DMRS and UE capability #1 and is determined by the reference SCS - N2 also refers to the value determined in control session with UE capability #1 and is determined by the reference SCS - The reference SCS for N1 & N2 is the minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG, and their corresponding configured downlink carriers/BWPs. - L2 refers to the MAC processing latency and is equal to 0.5 msec. - TA_max is equal to the maximum timing advance value that the 12 bit TA command can provide with respect to the UL SCS minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG and initial active UL BWP - Note: Spec captures the value k offset for different SCS - UE is not expected that UL transmissions in the same slot shall apply different TA values - If this happens, it is up to UE’s implementation. - Slot m and slot_duration are defined based on the minimum SCS among the subcarrier spacings of all the configured BWPs in all the configured uplink carriers in a TAG. Agreements: - To adopt the following TP for section 7.3 of 38.213 (with changes in red). If the PRACH transmission from the UE is not in response to a detection of a PDCCH order by the UE, referenceSignalPower is provided by ss-PBCH-BlockPower. If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers non-contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower or, when the UE is configured resources for a periodic CSI-RS reception, is obtained by higher layer parameter ss-PBCH-BlockPower and powerControlOffsetSS; where powerControlOffsetSS that provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214], depending on the DL RS that the DMRS of the PDCCH order is quasi-collocated with as described in Subclause 10.1. The UE is provided by higher layer parameters CFRA-SSB-Resource and CFRA-CSIRS-Resource an association between PRACH preamble indexes and corresponding indexes for SS/PBCH blocks and periodic CSI-RS resource configurations. If the PRACH transmission from the UE is in response to a detection of a PDCCH order by the UE which triggers contention based random access procedure, referenceSignalPower is provided by ss-PBCH-BlockPower. Agreements: - To adopt the following TP to section 8.2 of 38.213: A UE When receivinges the PDCCH with DCI format 1_0 with the CRC scrambled by the corresponding RA-RNTI and the corresponding PDSCH that includes the DL-SCH transport block, the UE may assume with the same DM-RS antenna port quasi co-location properties, as described in [6, 38.214], as for a detected SS/PBCH block or a received CSI-RS the SS/PBCH block or the CSI-RS resource the UE used for RACH association and transmission. If the UE attempts to detect the PDCCH with the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order which triggers non-contention based random access procedure, the UE may assumes that the PDCCH and the PDCCH order have same DM-RS antenna port quasi co-location properties. Agreements: - To adopt the following to section 5.1 of 38.214: When receiving PDSCH for Random Access Response (RAR) the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE selected used for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a RAR triggered by a PDCCH order which triggers non-contention based random access procedure, the UE may assume that the DM-RS port of the received PDCCH order and the PDSCH of the corresponding RAR are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. Agreements: - Kindly request RAN2 to adopt the following TP for 38.331 and send an LS to RAN2 – R1-1807707, which is approved and final LS in R1-1807764 -- ASN1START -- TAG-RACH-CONFIG-COMMON-START RACH-ConfigCommon ::= SEQUENCE { rach-ConfigGeneric RACH-ConfigGeneric, totalNumberOfRA-Preambles INTEGER (1..63) OPTIONAL, -- Need S ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { oneEighth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneFourth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneHalf ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, two ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32}, four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4) } OPTIONAL, -- Need M groupBconfigured SEQUENCE { ra-Msg3SizeGroupA ENUMERATED { b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, messagePowerOffsetGroupB ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18}, numberOfRA-PreamblesGroupA INTEGER (1..64) ssb-perRACH-OccasionAndnumberOfRA-PreamblesGroupAPerSSBPerRACH-Occasion CHOICE { oneEighth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneFourth ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, oneHalf ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, one ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64}, two ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32}, four INTEGER (1..16), eight INTEGER (1..8), sixteen INTEGER (1..4) } } OPTIONAL, -- Need R ra-ContentionResolutionTimer ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}, rsrp-ThresholdSSB RSRP-Range OPTIONAL, -- Need R rsrp-ThresholdSSB-SUL RSRP-Range OPTIONAL, -- Cond SUL prach-RootSequenceIndex CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) }, msg1-SubcarrierSpacing SubcarrierSpacing, restrictedSetConfig ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}, msg3-transformPrecoding ENUMERATED {enabled} OPTIONAL, -- Need R ... } Conclusion: - From RAN1 perspective, both 56 bits and 72 bits msg 3 payload sizes can be supported in NR. Up to RAN2 to decide whether or not it’s necessary to support both sizes. - Send an LS to RAN2, R1-1807766, which is approved and final LS in R1-1807842 Agreements: - For CSI-RS based Msg1 transmission, from RAN1 perspective, 0dB power offset of CSI-RS resource RE to SS RE is assumed by the UE without any explicit RRC configuration - Up to RAN2 to discuss whether or not a new RRC configuration parameter is necessary Agreements: - From a UE’s perspective, if a Msg2/3/4 search space overlaps in time with UL part and/or valid RACH occasions, the search space in that particular slot is invalid for the UE Working assumption: - In single CC and intra-band CA, if the SSB/CSI-RS that UE selects for RACH association and Msg1 (re)transmission is not quasi-co-located (QCLed) - with respect to type D - with the DM-RS antenna port associated with PDCCH reception in reference search spaces, UE is not expected to decode the PDCCH and or PDSCH in the reference search space scrambled by an RNTI other than those associated with Type1-PDCCH common search space, only if they are it is overlapped with PDCCH or PDSCH scrambled by the RNTIs associated with Type1-PDCCH common search space in at least one symbol. - Reference search spaces denote the Type0-PDCCH common search space, the Type0A-PDCCH common search space, the Type2-PDCCH common search space, or Type3-PDCCH common search space. Agreements: To reply RAN2 LS: - Whether tci-StatesPDCCH is ignored during the RA procedure (for BFR it seems already clear) or the tci-StatePDCCH should have been in the SearchSpace (e.g. if it is meant to be used for USS only and not for CSS)? - While monitoring Type 1 PDCCH common search space during the RA procedure, UE can ignore the tci-StatesPDCCH of the associated CORESET, if any tci-StatesPDCCH is configured with the associated CORESET. - Note: In case of PDCCH ordered CFRA, UE follows the same procedure that got agreed in RAN1 92 RACH procedure session. - Whether it is allowed to configure UEs with CSI-RS based RA and send the RAR on CORESET #0 (associated with SSBs)? - RAN1 is still discussing this. Agreements: - To adopt the following TP, section 10.1 of 38.213 If a UE is not provided higher layer parameter ra-SearchSpace for Type1-PDCCH common search space set, the control resource set and the association between PDCCH monitoring occasions for Type1-PDCCH common search space set and the SS/PBCH block index are same as the control resource set and the association of PDCCH monitoring occasions for Type0-PDCCH common search space set as described in Subclause 13. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level are given in Table 10.1-1. Agreements: - Confirm that only periodic CSI-RS is supported for CSI-RS based RRM in Rel-15 Agreements: - For RRM, UE may assume that in TDD bands, the half radio frame boundaries of the cells in the same frequency layer are aligned. - When useServingCellTimingForSync is set to TRUE for measurements, it means the following: - For intra-frequency measurements, UE may use the serving cell timing to derive the SSB index of neighbor cells in the same frequency layer. - For inter-frequency measurements, UE may use timing of any detected cell in the target frequency layer to derive the SSB index of neighbor cells of the target frequency layer - Note: cells in different frequency layer are not assumed to be half radio frame aligned. - UE may assume that in TDD bands useServingCellTimingForSync is assumed to be set to TRUE - Note: half radio frame alignment is the minimal requirement in RAN1’s understanding that allows the UE to use timing of one cell to derive the SSB index of another cell. However, it is up to RAN4 to determine the exact requirements for the “useServingCellTimingForSync”. In RAN1 understanding, the agreements above should not impact RAN4 discussion on the requirements for “useServingCellTimingForSync”. - RAN 1 believes useServingCellTimingForSync information should be also applicable to UEs in IDLE/INACTIVE mode. - It is up to RAN2 to determine to how to provide/apply useServingCellTimingForSync information in IDLE/INACTIVE mode. - Send an LS in R1-1807741 to RAN2/RAN4 to inform the above agreements, which is approved with the update that the bullet regarding “UE may assume that in TDD bands useServingCellTimingForSync is assumed to be set to TRUE” should be left-indented twice, final LS in R1-1807767 Conclusion: - With respect to agreement made in RAN4 2018 Ad-hoc #1 stating “Note: whether or not to define specific behavior for rate matching around above symbols is up to RAN1”, RAN1 will not define any additional rate matching behavior in Rel-15. Agreements: - No dedicated measurement windows for CSI-RS for L3 mobility is supported in Rel-15 - If CSI-RS for L3 mobility with associated SSB is configured, SMTC configuration (including frequency location and SCS of the SSB) should be also provided regardless whether UE is configured to measure SSBs in the same MO. - UE is expected to detect the associated SSB (to the CSI-RSs) within the configured SMTC window. - Note that when SSB based RRM measurements are configured, UE is always expected to be configured with SMTC information. LS to RAN2 – R1-1807906, which is approved and final LS in R1-1807909 Agreements: Confirm the following working assumptions regarding the values of X/Y/Z: - The maximum number of RLM-RS(s) and BFD-RS(s) should depend on whether same RS(s) is shared between RLM and BFD. The maximum number of unique RS(s), each RS using different set of resources, for both RLM-RS(s) and BFD-RS(s) are: - X RS(s) per BWP for below 3 GHz, - X=2(working assumption) - Y RS(s) per BWP for above 3 GHz and below 6 GHz, - Y=6 (working assumption) - Z RS(s) per BWP for above 6 GHz, - Z=8(working assumption) Agreements: - Remove csi-IM-ResourceElementPattern, csi-IM-ResourceElementPattern, csi-IM-ResourceId, periodicityAndOffset and freqBand from the list of ‘not applicable parameters’ from section 5 in 38.213. - Note: there is no RAN2 impact Agreements: - Confirm WA on RLM based on CSI-RS for tracking: If the TCI-states refer to CSI-RS for tracking, it is up to UE to select a NZP-CSI-RS resource from the configured resources for CSI-RS for tracking for RLM Agreements: - UE is not expected to monitor more than NRLM RadioLinkMonitoringRS for radio link monitoring when UE monitors RSs based on TCI states of PDCCH. Conclusion: - Aperiodic IS and aperiodic OOS indication based on successful/unsuccessful beam failure recovery is not supported in Rel-15. Agreements: - In case of no explicit RLM-RS configuration, when TCI-states indicate a combination of SSB, CSI-RS, and CSI-RS for tracking, UE RLM behavior is as follows: - When an active TCI state contains single RS, the UE is expected to perform RLM measurement using RS configured by the active TCI state - When an active TCI state contains two RSs, the UE is expected to use the RS associated with QCL Type D as RLM-RS. - For RLM, it is assumed that a TCI state that contains two RS should have one and only one RS with TypeD QCL. - UE is not required to perform RLM measurements with aperiodic RS configured in the TCI state. Agreements: - To adopt the following TP to section 4.2 of TS36.213 Upon reception of a timing advance command for a TAG containing the primary cell or PSCell, the UE adjusts uplink transmission timing for PUCCH/PUSCH/SRS of the primary cell or PSCell based on the received timing advance command. The UL transmission timing for PUSCH/SRS/PUCCH(if any) of a secondary cell is the same as the primary cell if the secondary cell and the primary cell belong to the same TAG. If the primary cell in a TAG operates with paired DL/UL spectrum and a secondary cell in the same TAG operates with unpaired DL/UL spectrum, a UE may assume that for FR1 and for FR2 [10, TS 38.133]. If the UE is configured with a SCG, the UL transmission timing for PUSCH/SRS of a secondary cell other than the PSCell is the same as the PSCell if the secondary cell and the PSCell belong to the same TAG. Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if all the serving cells in the TAG have the same duplex mode type, the UE adjusts uplink transmission timing for PUSCH/SRS/PUCCH(if any) of all the secondary cells in the TAG based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS/PUCCH(if any) is the same for all the secondary cells in the TAG. Upon reception of a timing advance command or a timing adjustment indication for a TAG not containing the primary cell or PSCell, if a serving cell in the TAG has a different duplex mode type compared to the duplex mode type of another serving cell in the same TAG, the UE adjusts uplink transmission timing for PUSCH/SRS/PUCCH(if any) of all the secondary cells in the TAG by using for FR1 and for FR2 regardless of the duplex mode type of the serving cells and based on the received timing advance command or a timing adjustment indication where the UL transmission timing for PUSCH /SRS/PUCCH(if any) is the same for all the secondary cells in the TAG. ……………… In other cases, a timing advance command [11, TS 38.321], , for a TAG indicates adjustment of the current value, , to the new value, , by index values of = 0, 1, 2,..., 63, where for a subcarrier spacing of kHz, . If a UE has multiple active UL BWPs in a same TAG, including UL BWPs in two UL carriers of a serving cell, the timing advance command value is relative to the largest larger subcarrier spacing of the multiple active UL BWPs. The applicable value for an UL BWP with lower subcarrier spacing may be rounded to align with the timing advance granularity for the UL BWP with the lower subcarrier spacing while satisfying the timing advance accuracy requirements in [10, TS38.133].

MIMO

Agreement The text proposal in R1-1807242 is agreed with the following revision. Delete the following text: - If the number of DL PT-RS ports associated to a tci-State in DCI is set to 2, - when one codeword is scheduled, the scheduled number of PT-RS ports is one, and the PT-RS port is associated with the lowest indexed DM-RS port. - when two codewords are scheduled, the scheduled number of PT-RS ports is two, and the first PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the first codeword. The second PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the second codeword. Note: Rel-15 agreements of supporting multi-TRPs/panels/beams related to e.g. DMRS/PTRS/TCI states can be a starting point for future discussions. Agreement: Section 7.3.1.2.2 of TS38.212 … - DMRS sequence initialization – 1 bit if transform precoding is enabled If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. In case one of the transport blocks is disabled as specified in Subclause 5.1.3.2 of [6, TS 38.214], the enabled transport block is mapped to codeword 0. Agreement The term “rate matched” is not defined, hence we suggest replacing it with “not available for PDSCH” to align with section 5.1.4 terminology of TS 38.214. - Editor to make appropriate revision Agreement Section 5.1.3.2 of TS38.214 … In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then a transport block is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1for the corresponding transport block, otherwise the transport block is enabled. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword. Agreement For PDSCH/PUSCH scrambling, - For DCI formats 1_1 and 0_1, the scrambling sequence initialization is done using the configured ID if the ID is provided by higher layers, otherwise using the default ID - For DCI formats 0_0 and 1_0 with C-RNTI/CS-RNTI in USS, the scrambling sequence initialization is done using the configured ID if the ID is provided by higher layers, otherwise using the default ID - For DCI 0_0 and 1_0 in CSS, the scrambling sequence initialization is done using the default ID Agreement - Support number of SRS resources for codebook based transmission as a UE capability Agreement Adopt the following TP for 38.214 6.1.1.1 Codebook based UL transmission For codebook based transmission, if PUSCH is scheduled by DCI format 0_1, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank from the DCI, given by DCI fields of SRS resource indicator and Precoding information and number of layers in subclause 7.3.1.1.2 of [TS 38.212], where the TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the antenna ports {0…ν-1} that correspond to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Subclause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n. Agreement Adopt the following TP for 38.214. 6.1.1.1 Codebook based UL transmission A UE shall not expect to be configured with the higher layer parameter codebookSubset set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in SRS-ResourceSet with usage set to ”codeBook” SRS-Config indicates two SRS antenna ports are configured. For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to “codeBook” SRS resource set and only one SRS resource can be indicated based on the SRI from within the SRS resource set. The maximum number of configured SRS resources for codebook based transmission is 2. If AP- aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of AP- aperiodic SRS resources. When multiple SRS resources are configured in SRS-ResourceSet with usage set to ”codeBook”, the UE shall expect that higher layer parameter nrofSRS-Ports in the SRS-ResourceSet SRS-Config shall be configured with the same value in all SRS resources, and the higher layer parameter resourceType in SRS-ResourceSet shall be configured with the same value in all SRS resources. Agreement TP to 38.214 Section 6.1.1.2: -------------------------------------------------------Start of TP for 38.214-------------------------------------------------------------- 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, the UE can calculate the precoder used for the transmission of precoded SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set. - If aperiodic SRS resource set is configured, the [CSI-RS information in the same slot TBD] associated NZP CSI-RS for UL channel measurement is indicated via SRS request field in DCI format 0_1 and 1_1, where AperiodicSRS-ResourceTrigger (indicating the association among aperiodic SRS triggering state), triggered SRS resource(s) srs-ResourceSetId, NZP-CSI-RS-ResourceId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId for UL channel measurement) are higher layer configured by AperiodicSRS-ResourceTrigger in SRS-ResourceSet SRS-Config. A UE may receive the dynamic SRS transmission request for aperiodic SRS transmission in the same slot as the reception of the DL CSI-RS resource A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the AP-NZP CSI-RS resource and the first symbol of the AP-aperiodic SRS transmission is less than 42 OFDM symbols. - If the UE is configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of SRS request field is not “00” as in Table 7.3.1.1.2-24 of [5, TS 38.212]. The CSI-RS is located in the same slot as the SRS request field, while any of the tci-States for aperiodic CSI-RS shall not be configured with ‘QCL-TypeD’. - If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceConfigID for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet SRS-Config per set. -----------------------------------------------------------------End of TP ---------------------------------------------------------------- Agreement -------------------------------------------------------Start of TP for 38.214-------------------------------------------------------------- 6.2.1 UE sounding procedure The 2-bit SRS request field [5 TS38.212] in DCI format 0_1, 1_1 indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Subclause 11.4 of [6, TS 38.213]. -----------------------------------------------------------------End of TP ---------------------------------------------------------------- Agreement TP proposal to 38.214 Section 6.1.1.2: -------------------------------------------------------Start of TP for 38.214-------------------------------------------------------------- 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI given by SRS resource indicator field from the DCI. The UE shall use one or multiple SRS resources for SRS transmission, where the number of SRS resources which can be configured to the UE for simultaneously transmission in the same RBs is a UE capability. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI before slot n. For non-codebook based transmission, the UE can calculate the precoder used for the transmission of precoded SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook'. - If aperiodic SRS resource is configured, the [CSI-RS information in the same slot TBD] UL channel measurement is indicated via DCI, where the association among aperiodic SRS triggering state, triggered SRS resource(s) srs-ResourceSetId, NZP-CSI-RS-ResourceId, csi-RS are higher layer configured by AperiodicSRS-ResourceTrigger in SRS-Config. A UE may receive the dynamic SRS transmission request for aperiodic SRS transmission in the same slot as the reception of the DL CSI-RS resource A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the AP-CSI-RS resource and the first symbol of the AP-SRS transmission is less than 42 OFDM symbols. - If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceConfigID for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-Config per set. The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) in the DCI format 0_1 in increasing order. For non-codebook based transmission, the UE does not expect to be configured with both spatialRelationInfo for SRS resource and associatedCSI-RS in SRS-Config for SRS resource set. For non-codebook based transmission, the UE can be scheduled with DCI format 0_1 when at least one SRS resource is configured in SRS-ResourceSet with usage set to ‘nonCodebook'. -----------------------------------------------------------------End of TP ---------------------------------------------------------------- Conclusion For non-codebook based UL transmission, - gNB may assume that UE applies a single precoder throughout the transmission bandwidth for PUSCH and SRS Working Assumption TP to 38.214, Section 6.1.1.2: -------------------------------------------------------Start of TP for 38.214-------------------------------------------------------------- 6.1.1.2 Non-Codebook based UL transmission For non-codebook based transmission, the UE can determine its PUSCH precoder and transmission rank based on the wideband SRI given by SRS resource indicator field from the DCI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to subclause 7.3.1.1.2 of [TS 38.212], or the SRI is given by srs-ResourceIndicator according to subclause 6.1.2.3. -----------------------------------------------------------------End of TP ---------------------------------------------------------------- Note: Whether non-codebook based UL transmission can be used for configured grant type-1/type-2 PUSCH transmission is determined under the scheduling agenda item Agreement ----Text Proposal for TS 38.214------------------------------------------------------------------ 5.1.2.3 Physical resource block (PRB) bundling < Unchanged parts are omitted > A UE may assume that precoding granularity is consecutive resource blocks in the frequency domain for bandwidth part i. can be equal to one of the values among {2, 4, wideband}. If is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the bandwidth part i with consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one to or more. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that is equal to 2 PRBs. If is determined as "wideband", the UE is not expected to be scheduled with non-contiguous resource allocation and the UE may assume that the same precoding is applied to the allocated resource. If is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partition the bandwidth part i with consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one to or more. When is determined as one of the values among {2, 4}, The first PRG size is given by and the last PRG size given by if , and the last PRG size is if . The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG partitioned above. For PDSCH carrying SIB1 scheduled by PDCCH with CRC scrambled by SI-RNTI, PRG is partitioned from the lowest numbered resource block of the CORESET signalled in PBCH. If a UE is scheduled a PDSCH with DCI format 1_0, then the UE shall assume that is equal to 2 PRBs. When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, for each carrier bandwidth part i is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType given by PDSCH-Config. When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, the UE shall assume that is equal to 2 PRBs. In case UE is scheduled by DCI 1_0, the higher layer parameter prb-BundlingType is set to 'static', and when receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, the value of is fixed as 2 PRBs. If a UE is scheduled a PDSCH with DCI format 1_0 scrambled with C-RNTI and CS-RNTI, then the UE shall assume that is equal to 2 PRBs. When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'dynamicBundling', the higher layer parameters bundleSizeSet1 and bundleSizeSet2 configure two sets of values, the first set include can take one or two values among {2, 4, wideband}, and the second set include can take one value among {2, 4, wideband}. The UE is not expected to be configured with (2, 4) in the first set. If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Subclause 7.3.1.2.2 of [2, TS 38.212] - is set to '0', the UE shall use the value from the second set of values when receiving PDSCH scheduled by the same DCI. - is set to '1' and one value is configured for the first set of values, the UE shall use this value when receiving PDSCH scheduled by the same DCI - is set to '1' and two values are configured for the first set of values as n2-wideband' (corresponding to two values 2 and wideband) or n4-wideband' (corresponding to two values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise is set to the remaining configured value of 2 or 4, respectively. When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'staticBundling', the value is configured with the single value indicated by the higher layer parameter bundleSize. When a UE is configured with RBG = 2 for bandwidth part i according to Section 5.1.2.2.1, or when a UE is configured with interleaving unit of 2 for VRB to PRB mapping provided by the higher layer parameter vrb-ToPRB-Interleaver given by PDSCH-Config for bandwidth part i, the UE is not expected to be configured with = 4. < Unchanged parts are omitted > Agreement If interference measurement is performed on aperiodic NZP CSI-RS, UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement. Agreement: For TS38.214, the following text proposal is agreed: 5.2.1.4.2 Report Quantity Configurations If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to ‘cri-RSRP’, 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', and multiple resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k≥0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RS-Resources in the corresponding nzp-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resources in the corresponding csi-IM-ResourceSet (if configured). Agreement: For TS38.214, the following text proposal is agreed: 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS ---unchanged texts are ommitted----- - For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Subclause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State’s for the aperiodic CSI-RS resources associated with the CSI triggering state. If a TCI- State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP. - A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the CSI triggering state ID in the up to trigger states with codepoint ‘1’ mapped to the triggering state having the smallest triggering state ID. Agreement: For TS38.214, the following text proposal is agreed: 5.1.4.2 PDSCH resource mapping with RE level granularity ---unchanged texts are ommitted----- The UE may be configured with a DCI field for triggering the aperiodic ZP-CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config, is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s) configured (up to 2 bits). Each triggering state none-zero codepoint of ZP CSI-RS trigger in DCI is to triggers one aperiodic ZP-CSI-RS-ResourceSet in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetIds =1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetIds=2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetIds=3. The first state Codepoint ‘00’ is reserved for not triggering aperiodic ZP CSI-RS. Agreement - When DL BWP is switched, the following resources defined in the BWP in activated state stays in its activated state and is suspended until the DL BWP is switched back. - Semi-persistent CSI-RS/CSI-IM resource - semi-persistent ZP CSI-RS resource set - When UL BWP is switched, the following resources defined in the BWP in activated state stays in its activated state and is suspended until the UL BWP is switched back. - Semi-persistent SRS Agreement In DRX mode operation, PUSCH-based SP-CSI is not reported when UE is not in Active Time. Agreement: - Send an LS to RAN2 capturing the following details for non-PMI feedback to correct inaccurate implementation of RAN1 agreements - RI restriction is supported - For rank-R port index, the sequence size shall be R, e.g., rank 2-8 includes 2-entry PortIndex8 LS is endorsed in R1-1807727 Agreement An NZP CSI-RS resource set for interference measurement can contain more than one resource. - The entire resource set is used for interference measurement - For all the resources configured in an NZP CSI-RS resource set for interference measurement, the total number of port shall be no larger than 18 Agreement - To select which reference SCS µ is used to determine Z and Z’ for a CSI report - Let µDL correspond to the subcarrier spacing of the PDCCH with which the DCI was transmitted, µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and - For Type B processing capability, µCSI-RS correspond to the subcarrier spacing of the ap-CSI-RS for the CSI report - For Type A processing capability, µCSI-RS correspond to the minimum subcarrier spacing of the all aperiodic CSI-RS triggered by the DCI - To determine Z, the reference SCS µ corresponds to min(min(µDL, µCSI-RS), µUL) - To determine Z’, the reference SCS µ corresponds to min(min(µDL, µCSI-RS), µUL) Agreement - Make the following corrections in TS 38.214 and 38.212 - Define CSI report mapping order for PUCCH and PUSCH (including report ordering when applying CSI omission rules) according to CSI reporting priority rule in Section 5.2.5 of 38.214 Agreement Align CPU occupancy rules so that the CPU is occupied until the last symbol of the PUSCH/PUCCH carrying the report for all cases (except for Type B capability with P/SP CSI-RS where a CPU is always occupied) Agreement L1-RSRP reporting is treated as a new Latency Class where the Z’ values follow the reported beam report timing values in capability 2-25 Conclusion TRS does not occupy a CPU Agreement - For the purpose of simultaneous CSI-RS reception in UE features 2-33, 2-36, 2-40, 2-41 and 2-43, CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted N times if the CSI-RS resource is referred by N resource Report settings Agreement - For PUSCH-based SP-CSI report setting in activated state, when either UL BWP or DL BWP is switched, the SP-CSI report setting transitions to inactivated state. New activation message is required to transition the SP-CSI report setting back to activated state. Agreement When a carrier is deactivated, the following resources configured in the carrier in activated state would also be deactivated and needs new activation message to transit back to activated state. - Semi-persistent CSI-RS/CSI-IM resource - Semi-persistent CSI reporting on PUCCH - Semi-persistent SRS - Semi-persistent ZP CSI-RS resource set Agreement Adopt the following TP for TS 38.214 /************************ Start of Text Proposal **************************/ For each sub-band index s, a 2-bit sub-band differential CQI is defined as: - Sub-band Offset level (s) = sub-band CQI index (s) – wideband CQI index /************************ Unchanged parts omitted**************************/ /************************ End of Text Proposal **************************/ Agreement Support CSI report based on 1-port CSI-RS, where the number of bits for RI/LI/PMI is 0. Make the corresponding updates in 38.212. Agreement The number of CQI fields in UCI is determined only from rank restriction Agreement The most recent CSI measurement occasion occurs in DRX Active time for CSI to be reported Agreement - Add the following sentences to 38.214 Section 5.2.1.4.2 as the definition of LI: - “The LI indicates which column of the precoder matrix of the reported PMI corresponds to the strongest layer of the codeword corresponding to the largest reported wideband CQI. If two wideband CQIs are reported and have equal value, the LI corresponds to strongest layer of the first codeword.” Agreement - WB CSI report, the UCI parameter ordering for LI reporting is as follows: CRI → RI → LI (if present) → Padding bits (if present) → PMI → CQI. - Note: This is already captured in specification Agreement - Remove UE capability FG 2-57 on low latency CSI - Remove UE capability component 6 in FG 2-35 on Type A/B CSI processing capability - Refine the UE CSI processing capability as follows - Define two tables - Table 1 for the case where only a single CSI report is aperiodically triggered - Table 1 is defined for low-latency CSI only Table 1: A-CSI timing requirement 1 with single capability SCS; Low-latency CSI. CSI only (proposal). Z(1); Z’(1). 15kHz; FFS: 10 or 9; FFS: 8 or 7. 30kHz; 13; 11. 60kHz; 25; 21. 120kHz; 43; 36. - Table 2 for all other cases Table 2: A-CSI timing requirement 2 SCS; Low-latency CSI. CSI-only. Z(1); Z’(1). 15kHz; 22; 16. 30kHz; 33; 30. 60kHz; 44; 42. 120kHz; 97; 85. Table 5: A-CSI timing requirement 2 (assuming CRI occupies Ks CPU) proposal SCS; High-Latency CSI. CSI only (proposal). Z(1); Z’(1). 15kHz; 40; 37. 30kHz; 72; 69. 60kHz; 141; 140. 120kHz; 152; 140. - When X CSI processing units are unoccupied and a single report with low latency CSI class without UL-SCH is triggered - Select one of the following alternatives: - The timing requirements are defined by Z(1) in Table 1 - There is no “advanced UE CSI processing” capability in Rel-15 - This report occupies X CSI processing units - X is the number of CSI processing units supported by a UE - Otherwise, the timing requirements are defined by (Z,Z’) in Table 2 and follows the regular CPU occupancy rules - When a single DCI triggers N >= 1 A-CSI reports RS (each with single CSI-RS resource in the set for channel measurement) ((Zn, Z'n) for report n), but UE only has M<=X unoccupied CSI processing units, UE is only required to update M of the N CSI reports - Additionally, CSI for a report n=0,1,…M-1 is not updated if the PUSCH scheduling offset does not give enough CSI calculation time for that CSI report according to maxk=0,…,min⁡(M,N)-1(Zk') where the reports are indexed according to the priority rule in Section 5.2.5 of TS38.214 - CPU occupancy rules for Table 2 - When single DCI triggers N>=1 A-CSI reports based on A-CSI-RS (each with single CSI-RS resource in the set for channel measurement), - min(M, N) CPUs are occupied from the first symbol after the PDCCH triggering the report until the last symbol of the PUSCH carrying the report - For A/P/SP CSI reporting based on P/SP CSI-RS - The CSI reporting setting associated with P/SP CSI-RS/CSI-IM resource is assigned [one or Ks] CSI processing unit(s) and occupiedy [it or them] from the earliest one of the first symbol of the latest CSI-RS/CSI-IM resource earlier than the corresponding CSI reference resource until the last symbol of the PUSCH/PUCCH carrying the report - When triggered with a CSI report with a set containing Ks > 1 CSI-RS resources for channel measurement - For Ks=2, at most 16 CSI-RS ports per CSI-RS resource can be configured - For 8>=Ks>2, at most 8 CSI-RS ports per CSI-RS resource can be configured - The CSI report occupies Ks CSI processing units and the latency requirement for each unit is calculated based on (Z, Z') according to the High latency CSI class with the Z values in Table 5 - Simultaneous resources in FG 2-36, 2-40, 2-41, 2-43, 2-33 - In any slot, the number of “active” CSI-RS resources/ports should not exceed the numbers reported in FG2-36, 2-40, 2-41, 2-43, 2-33 - An NZP CSI-RS resource is active in a duration of time defined as follows: - For A-CSI-RS, starting from the end of the PDCCH containing the request, and ending at the end of the PUSCH containing the report associated with this A-CSI-RS. - For SP-CSI-RS, starting from the end of the activation command is applied, and ending at the end of the deactivation command is applied. - For P-CSI-RS, the activate duration ends when the P-CSI-RS configuration is released. - Modify UE feature group 2-33 as follows: Features. #; 2-33. Feature group; CSI-RS and CSI-IM reception for CSI feedback. Components; 1. Supported max # of configured NZP-CSI-RS resources per CC 2. Supported max # of ports across all configured NZP-CSI-RS resources per CC 3. Supported max # of configured CSI-IM resources per CC 4. Supported max # simultaneous CSI-RS resources in active BWPs across all CCs 5. Supported max total # of CSI-RS ports in simultaneous CSI-RS resources in active BWPs across all CCs. Prerequisite feature groups; 2-32. Need for gNB to know whether the; Yes. Consequences if the feature; CSI acquisition is not supported. Type (see R2-1712078); Type 3. Need of FDD/TDD differentiation; N.A.. Need of FR1/FR2 differentiation; N.A.. RAN5 implication. Note; Note: all the candidate values are the range of capability signaling which doesn’t determine whether UE is mandatory to support all the signaling values.. Responsible WG. RAN WG recommendation; Component-1 candidate values: {from 1 to 32} Component-2 candidate values: , further down-select between: Alt.1: {from 4 to 64}, Alt.2: {from 4 to 256}, Alt.3: {from 4 to 256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {8, 16, 24, …, 248, 256}. TSG-RAN decision. Agreement: If a UE only has M CPUS available and is triggered with N>M reports, UE shall be required to update the M reports with highest priority according to priority rule in Section 5.2.5 Agreement - If UE supports X simultaneous CSI calculations, a UE is not expected to be configured with an aperiodic CSI trigger state containing more than X CSI reports settings across all CCs Agreement When single CSI report is not multiplexed with UL-SCH and Z’ timing requirement is not met (but Z condition is met), a UE does not expect to be scheduled with a DCI where the Z’ criterion is not met, and may ignore the DCI Agreement - To select which reference SCS µ is used to determine Z and Z’ for a CSI report - Let µDL correspond to the subcarrier spacing of the PDCCH with which the DCI was transmitted, µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS correspond to the minimum subcarrier spacing of the all aperiodic CSI-RS triggered by the DCI - To determine Z and Z’, the reference SCS µ corresponds to min(min(µDL, µCSI-RS), µUL) - Note that subcarrier spacing corresponding to min(min(µDL, µCSI-RS), µUL) should be used for calculation of time interval Conclusion For PUSCH-based SP-CSI, each DCI only activates a single trigger state. One codepoint of the CSI request field triggers a single trigger state. The PUSCH-based SP-CSI trigger state definition is kept as is, each trigger states activates a single CSI report setting. - The same size of CSI request field is used for SP-CSI as for aperiodic CSI and depends on reportTriggerSize. The number of trigger states for SP-CSI and aperiodic CSI does not need to be the same. Agreement For PUSCH-based SP-CSI, multiple SP-CSI trigger states are allowed to be simultaneously active. Deactivation of an SP-CSI trigger state is indicated with DCI format 0_1 - UE is not expected to transmit more than one SP-CSI on PUSCH per slot, for all CCs Agreement - Use the following DCI fields for SP-CSI activation/deactivation: Table A: Special field for SP-CSI Activation PDCCH validation DCI format 0_1. HARQ process number; set to all '0's. Redundancy version; set to '00'. Table B: Special fields for SP-CSI Deactivation PDCCH Validation DCI format 0_1. HARQ process number; set to all '0's. Modulation and coding scheme; set to all '1's. Resource block assignment; If higher layer configures RA type 0 only, set to all ‘0’s; If higher layer configures RA type 1 only, set to all ‘1’s; If higher layer configures dynamic switch between RA type 0 and 1, then if MSB is’0’, set to all ‘0’s; else, set to all ‘1’s. Redundancy version; set to '00'. Agreement How to correct CSI reference resource with respect to definition of downlink slot? - Adopt TP below <TP for Section 5.2.2.1.1 of TS 38.214> A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible symbol is configured as a downlink slot for that UE, and - it does not fall within a configured measurement gap for that UE </TP> Agreement - Add the below sentence to section 5.2.1.5.1 of TS 38.214 - A UE is not expected to receive more than one DCI with CSI request per slot - Note: editor of TS 38.214 may change the exact wording to reflect the above sentence. - Note: This applies to Rel-15. Conclusion WB Type II CSI on PUSCH (comprising both Part 1 and Part 2) reporting is supported Conclusion - 38.212/38.213/38.214 editors to align definition of “CSI report” to mean either a set of PMI/RI/CQI/LI corresponding to one CSI report setting or the physical channel carrying possibly multiple CSIs - 38.212/38.213/38.214 editors to align definition of single/multiple CQI/PMI to wideband/subband CQI/PMI Agreement - When (single or multiple) CSI report(s) is multiplexed with UL-SCH and N2’ criterion is met but Z’’ criterion is not met, the UE drops the CSI, but transmit the UL-SCH without CSI multiplexed on PUSCH Agreement Agree to the following text proposal for TS38.213 and TS38.214. - When the HARQ/ACK corresponding to the PDSCH carrying the MAC CE command is transmitted in the slot n, the command shall be applied starting from slot n + 3Nslotsubframe, + 1. Agreement >>> Text proposal for 38.214 Section 5.2.1.4.3 >>> For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as ‘enabled', the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP uses is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP uses is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133]. >>> End text proposal >>> Agreement Text proposal for Section 9.2.1 in 38.213 If a UE does not have dedicated PUCCH resource configuration, provided by higher layer parameter PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by higher layer parameter pucch-ResourceCommon in SystemInformationBlockType1 through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial active UL BWP provided by SystemInformationBlockType1. The PUCCH resource set is provided by higher layer parameter PUCCH-Resource-Common and includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset, and a cyclic shift index set for a PUCCH transmission. The UE transmits a PUCCH using frequency hopping. If a UE does not have dedicated PUCCH resource configuration, provided by higher layer parameter PUCCH-ResourceSet in PUCCH-Config, the UE transmits the PUCCH using the same spatial domain transmission filter as for the Msg3 PUSCH transmission. LS is endorsed with the following text revisions in R1-1807725 Replace “Concern: NTT DoCoMo (more details on complexity needed), OPPO (complexity from NW/performance perspective should be included)” with “Some companies showed concerns on complexity from NW/performance perspective and additional specification effort.” Agreement - (Working Assumption) If either CORESET associated with SearchSpace-BFR or SearchSpace-BFR is not configured, UE assumes contention-free PRACH BFR is not performed. - In this case, SearchSpace-0 or CORESET-0 is not used for monitoring contention-free PRACH BFR response. - If either CORESET associated with SearchSpace-BFR or SearchSpace-BFR is not configured, UE can still perform contention-based PRACH BFR based on configuration. - Send LS to RAN2 The LS is endorsed in R1-1807883 Working Assumption In case of implicit configuration of BFD set q0, if there are two RS indices in the RS set indicated by TCI states for CORESETs, the RS indices associated with QCL type D in the RS sets indicated by TCI states for CORESETs are used for beam failure detection. Agreement (Impact to RAN2 specifications) Adopt the following text proposal in 38.214 -------------------------------------begin text proposal-------------------------------------- 6.1.4.1 Modulation order and target code rate determination >>> unchanged text omitted <<< - for MCS index 0, 1 and 28, q=1 if UE has reported to support pi/2 BPSK modulation; and q=2 in other cases - for MCS index 0, 1, and 28, if higher-layer parameter PUSCH-tp-pi2BPSK is configured, q = 1; otherwise q= 2 -------------------------------------end text proposal-------------------------------------- Include as part of LS to RAN2 Agreement (Impact to RAN2 specifications) A new RRC parameter, PUSCH-tp-pi2BPSK, needs to be introduced to clarify default capability. The table below provides the parameter values and other details. This parameter is UE optional. Sub-feature Group; RAN1 Spec; RAN2 Parent IE; Parameter Name; Description; Values (default); UE/Cell Specific; Comment. PUSCH; 38.214; PUSCH-Config; PUSCH-tp- pi2PBSK; For a UE capable of supporting pi/2-BPSK for PUSCH with transform precoding, this parameter configures whether pi/2-BPSK modulation is enabled or disabled for PUSCH with transform precoding. For a UE not capable of supporting pi/2-BPSK on PUSCH with transform precoding, this parameter is not applicable.; {enabled, disabled} (disabled); UE specific; Introduced to clarify default capability.. LS to RAN2 to inform them of the above agreements on pi/2 BPSK is approved in R1-1807720. Agreement For the case of CSIRS+PDCCH on the same OFDM symbols where spatial QCL is configured for CSI-RS - NW configuration should ensure spatial QCL - UE is not expected to be configured with CSI-RS with repetition=ON in the symbols UE is configured to monitor the CORESET Above agreement applies to both same BWP as well as intra-band CA if spatial QCL is applicable. Agreement: TP for Section 6.2.1 in TS 38.214 ------Unchanged parts are omitted------ For PUCCH formats 0 and 2, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s) or if aperiodic SRS is configured and PUCCH consists of beam failure request. A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with semi-persistent or periodic PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only. ------Unchanged parts are omitted------ Agreement: If the following change is captured in TS38.213 after Busan meeting, then the red highlighted part can be removed from TS38.214. 6.2.1 UE sounding procedure <……> A UE shall not transmit simultaneously SRS resource(s) and PRACH. If SRS and PRACH would occur on overlapping OFDM symbol(s), SRS is not transmitted on the overlapping symbol(s). Agreement (Impact to RAN2 specifications) Text proposal #3 for Section 5.2.2.3.1 in TS38.214 < Unchanged parts are omitted > The bandwidth and initial CRB index of a CSI-RS resource within a BWP, as defined in Subclause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min24, NBWPsize. < Unchanged parts are omitted > Send an LS to RAN2. Agreement (Impact to RAN2 specifications) Change the value range of nrofRBs in CSI-RS bandwidth configuration from 24 to maxNrofPhysicalResourceBlocks + 1 and include as part of an LS to RAN2. Ask RAN2 to update the relevant description. Send an LS to RAN2. Agreement UE expects to be configured with same BWP_info for all CSI-RS resources within one set. UE expects to be configured with same density for all CSI-RS resources within one set. Agreement - For P/SP-CSI-RS, UE expects to be configured with same periodicity for all CSI-RS resources within one set (slot offset can still be different for different CSI-RS resources). Agreement Text proposal for Section 5.1.6.1.2 in 38.214 If the UE is configured with a CSI-ReportConfig with reportQuantity set to "cri-RSRP", or "none" and if the CSI-ResourceConfig for channel measurement (higher layer parameter resourcesForChannelMeasurement) contains a NZP-CSI-RS-ResourceSet that is configured with the higher layer parameter repetition and configured without the higher layer parameter trs-Info, the UE can only be configured with the same number (1 or 2) of ports with the higher layer parameter nrofPorts for all CSI-RS resources within the set, Agreement UE expects to be configured with same number of ports for all CSI-RS resources within one set. Agreement Text proposal for Section 7.4.1.5.2 in TS38.211 < Unchanged parts are omitted > where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with at the start of each OFDM symbol where is the slot number within a radio frame, is the OFDM symbol number within a slot, and equals the higher-layer parameter scramblingID. < Unchanged parts are omitted > Note: Inside the equation, the number of 14 is replaced as N_symb^slot. Agreement Suggest RAN2 to change the value range of firstOFDMSymbolInTimeDomain2 as from 2 to 12 and include as part of an LS to RAN2. The LS is endorsed in R1-1807887 Agreement Text proposal for Section 7.4.1.5.1 in 38.211 < Unchanged parts are omitted > Zero-power and non-zero-power CSI-RS are defined - for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3 - for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission. UE performs the same measurement/reception on channels/RSs except PDSCH regardless of whether they collide with ZP CSI-RS or not. < Unchanged parts are omitted > Working Assumption Text proposal for TS38.214 If the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that a PDCCH DM-RS transmitted in the all search space sets associated with CORESET CORESET are quasi co-located with ‘QCL-Type D’, if ‘QCL-Type D’ is applicable. Furthermore, the UE shall not expect to be configured with CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s). Agreement: Text proposal for TS38.214: 4.1 Power allocation for downlink ... For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE (1/[dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Subclause 5.1.6.2. The DM-RS scaling factor specified in subclause 7.4.1.1.2 of [4, TS 38.211] is given by . Table 4.1-1: The ratio of PDSCH EPRE to DM-RS EPRE Number of DM-RS CDM groups without data; DM-RS configuration type 1; DM-RS configuration type 2. 1; 0 dB; 0 dB. 2; -3 dB; -3 dB. 3; -; -4.77 dB. ... - 5.1.6.2 DM-RS reception procedure ... The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the number of additional DM-RS, the DM-RS symbol location, and DM-RS configuration type as described in Subclause 7.4.1.1 of [4, TS 38.211]. When receiving PDSCH scheduled by DCI format 1_1, the UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-2-4, 7.3.1.2.2-1-3, 7.3.1.2.2-1-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1-1, 7.3.1.2.2-1-2, 7.3.1.2.2-1-3, 7.3.1.2.2-1-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively. When receiving PDSCH scheduled by DCI format 1_0, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PDSCH with allocation duration of 2 symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases. ... 6.2.2 UE DM-RS transmission procedure ... For PUSCH scheduled by DCI format 0_1, the UE shall assume the DM-RS CDM groups indicated in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of Subclause 7.3.1.1 of [5, TS38.212] are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) correspond to CDM group 0, {0,1}, {0,1,2}, respectively. For PUSCH scheduled by DCI format 0_0, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases. For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ([dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor specified in subclause 6.4.1.1.3 of [4, TS 38.211] is given by . Agreement TP for Section 6.2.2 in 38.214: - 6.2.2 UE DM-RS transmission procedure When transmitted PUSCH is not scheduled by PDCCH format 0_1 with CRC scrambled by C-RNTI or CS-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled: - The UE shall assume dmrs-AdditionalPosition=’pos2’ and up to two additional DM-RS can be transmitted according to PUSCH duration. If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition=’pos1’ and up to one additional DM-RS can be transmitted according to PUSCH duration. … Agreement Text proposals for TS 38.211 Section 6.4.1.1.3 6.4.1.1.3 Precoding and mapping to physical resources ------Unchanged parts are omitted------ The time-domain index and the supported antenna ports are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6: - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used - if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [5, TS 38.212], Table 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is 1 for each hop. ------Unchanged parts are omitted------ Table 6.4.1.1.3-6: PUSCH DM-RS positions for single-symbol DM-RS and frequency hopping enabled. Duration in symbols for each hop; DM-RS positions. PUSCH mapping type A; PUSCH mapping type B. dmrs-AdditionalPosition (=2); dmrs-AdditionalPosition (=3); dmrs-AdditionalPosition. 0; 1; 0; 1; 0; 1. First hop; Second hop; First hop; Second hop; First hop; Second hop; First hop; Second hop; First hop; Second hop; First hop; Second hop. ≤3; -; -; -; -; -; -; -; -; 0; 0. 4; 0; 0; 0; 0; 0; 0. 5, 6; 0; 0, 4; 0; 0, 4; 0; ,; 0, 4. 7; 0; ,; 0, 4; 0; 0, 4; 0; ,; 0, 4. Agreement =========================== Text Proposal for TS38.214 ============================== 5.1.6.2 DM-RS reception procedure When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH scheduling DCI format 1_0 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in subclause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition < Unchanged parts are omitted > ================================= End of Text Proposal =============================== Agreement Text proposals for TS 38.214 Section 5.1.6.2 ------Unchanged parts are omitted------ 5.1.6.2 DM-RS reception procedure When receiving PDSCH scheduled by PDCCH with CRC scrambled by SI-RNTI, RA-RNTI, P-RNTI or TC-RNTI, or receiving PDSCH scheduling DCI format 1_0 by PDCCH with CRC scrambled by C-RNTI or CS-RNTI, or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in subclause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition ------Unchanged parts are omitted------ A UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where “1”, “2” and “3” for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-4 of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively. ------Unchanged parts are omitted------ Agreement /************************* Start of Text Proposal for 38.211 ************************/ 6.4.1.1.3 Precoding and mapping to physical resources The time-domain index and the supported antenna ports are given by Tables 6.4.1.1.3-3 through 6.4.1.1.3-6: - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 1 and frequency hopping is disabled, the tables shall be used according to single-symbol DM-RS - if the higher-layer parameter maxLength in DMRS-UplinkConfig is equal to 2 and frequency hopping is disabled, the associated DCI determines whether single-symbol or double-symbol DM-RS shall be used - if the higher-layer parameter dmrs-AdditionalPosition is equal to or more than 1 and frequency hopping is enabled according to clause 7.3.1.1.2 in [4, TS 38.212], Table 6.4.1.1.3-6 shall be used assuming dmrs-AdditionalPosition is 1 for each hop. If , dmrs-AdditionalPosition is assumed to be 0. /******************************* omitted part *******************************/ Table 6.4.1.1.3-5: PUSCH DM-RS time index . DM-RS duration; Supported antenna ports. Configuration type 1; Configuration type 2. single-symbol DM-RS; 0; 0 – 3; 0 – 5. double-symbol DM-RS; 0, 1; 0 – 7; 0 – 11. /************************** End of Text Proposal for 38.211*************************/ Agreement For PDSCH/PUSCH mapping type A with 8 symbols duration - Support the value in the bracket (additional DMRS in symbol 7 is supported) For PUSCH mapping type B with 5 symbols duration, - Support the value in the bracket (additional DMRS in symbol 4 is supported) Send an LS to RAN4 to inform them of the potential issue on different power / different antenna (Peng) LS is approved in R1-1807726 Conclusion: No consensus in RAN1 for addressing high PAPR in CP-OFDM DMRS for NR Rel-15. Agreement =============================== Text Proposal for TS38.211 =============================== 6.4.1.1 Demodulation reference signal for PUSCH 6.4.1.1.1 Sequence generation 6.4.1.1.1.1 Sequence generation when transform precoding is disabled If transform precoding for PUSCH is not enabled, the sequence shall be generated according to . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within a frame, and - and are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 not a msg3 PUSCH according to clause 8.3 in [5, TS 38.213]; - and is given by the higher-layer parameter scramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI; - and otherwise. is indicated by the DMRS sequence initialization field in the DCI Format 0_1 in 3GPP TS 38.212 [X] associated with the PUSCH transmission. < Unchanged parts are omitted > 7.4.1.1 Demodulation reference signals for PDSCH 7.4.1.1.1 Sequence generation The UE shall assume the sequence is defined by . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with where is the OFDM symbol number within the slot, is the slot number within a frame, and - and are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_1 with the CRC scrambled by C-RNTI or CS-RNTI; - and is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI or CS-RNTI; - and otherwise is indicated by the DMRS sequence initialization field in the DCI Format 1_1 in 3GPP TS 38.212 [X] associated with the PDSCH transmission. ================================= End of Text proposal =============================== Agreement Text proposal for TS 38.214 v15.1.0 Section 5.1.6.3 A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum ModOrder as it reported to support. If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig , - the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, respectively. - if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna ports’ presence and pattern are a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, - if the higher layer parameter timeDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume LPT-RS = 1. - if the higher layer parameter frequencyDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume KPT-RS = 2. - otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2 and the UE shall assume The UE shall assume that PT-RS is not present when, - the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or - the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or - the number of scheduled RBs is smaller than 3, or - the RNTI equals RA-RNTI, SI-RNTI or P-RNTI. - otherwise, if the RNTI equals RA-RNTI, SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present Agreement Text proposal for clause 6.2.3.2 of TS 38.214 The UE shall assume no PT-RS is present when the number of scheduled RBs is less than or equal to NRB0 if NRB0 > 01 or if the RNTI equals TC-RNTI. Agreement Text proposal for 38.214, section 6.2.3.1 <unchanged parts omitted> For PT-RS, the transmit power of PT-RS is derived from , which is the power ratio between power of PUSCH and power of PT-RS per port. For codebook based uplink transmission, wWhen the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is , - If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE is given by , where is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor specified in subclause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the TPMI field in DCI. The UE shall assume ptrs-Power in PTRS-UplinkConfig is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH . - Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE UL-PTRS-power /; The number of PUSCH layers ( ). 1; 2; 3; 4. All cases; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial and non- coherent and non-codebook based; Full coherent; Partial coherent; Non-coherent and non-codebook based. 00; 0; 3; 3Qp-3; 4.77; 3Qp-3; 6; 3Qp; 3Qp-3. 01; 0; 3; 3; 4.77; 4.77; 6; 6; 6. 10; Reserved. 11; Reserved. Agreement Text proposal for Section 6.4.1.2.2.1 and 7.4.1.2.2 of TS 38.211 is the C-RNTI, which can be C-RNTI/SP-CSI C-RNTI/CS-RNTI for DCI-based scheduling, and CS-RNTI for grant-free type 1 transmission. Conclusion The TS38.211, 38.214 editors are encouraged to check editorial corrections in R1-1805990 related to PTRS Agreement Text proposals for TS 38.211 v15.1.0 Section 7.4.1.2.2 < Unchanged parts are omitted > If present, the UE shall assume the PDSCH PT-RS is scaled by a factor to conform with the transmission power specified in clause 4.1 of [6, TS 38.214] and mapped to resource elements according to when all the following conditions are fulfilled - is within the OFDM symbols allocated for the PDSCH transmission - resource element is not used for DM-RS, NZP CSI-RS (except for NZP CSI-RS for mobility), ZP CSI-RS, SS/PBCH block, Agreement The two aperiodic SRS resource sets for 1T4R antenna switching can only be allowed to be triggered by one single DCI, which means that the UE shall expect that the two SRS resource sets for 1T4R antenna switching are both configured with the same value of the higher layer parameter aperiodicSRS-ResourceTrigger and with different values of the higher layer parameters slotoffset. Agreement: RAN1 has identified the following adjacent TDM combinations of SRS resource configured for antenna switching and other physical channels - SRS-PUSCH - PUSCH-SRS - SRS-PUCCH - PUCCH-SRS - RAN1 notes that PUCCH may convey payload with different priorities (e.g., CSI, ACK/NACK), or may have different formats, or PUSCH and PUCCH may use different number of UL symbols - Send an LS to RAN4 request that they address the issue of antenna switching in such adjacent combinations in RAN4 spec - RAN1 does not plan to introduce scheduling restrictions on back to back SRS antenna switching and PUSCH/PUCCH channel unless serious concerns are identified in RAN4 LS is agreed in R1-1807722 Agreement: For a SRS resource set, - a UE is not expected to be configured with multiple SRS resources with different periodicities within a SRS resource set. Agreement New text in Section 6.2.1.3 in 38.214: For n-th (n>=1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided - it is no earlier than the summation of - the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and - the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL, - it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time. otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission Agreement Text proposal for section 6.2.1.3 of 38.214 A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier indicating indicating the switching from carrier c2 which is configured for PUSCH/PUCCH transmission in the case that switching-from carrier is from the serving cell configured with one uplink carrier. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwichFromCarrier indicating the switching from carrier c2 which is configured for PUSCH/PUCCH transmission in the case that switching-from carrier is from the serving cell configured with two uplink carriers. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL), uplink transmission is interrupted the UE temporarily suspends the uplink transmission on carrier c2. Agreement: Text proposal for section 6.2.1.3 of 38.214 (remove brackets) If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by [SystemInformationBlockType0] or [SystemInformationBlockType1] on serving cell c2. - 38.214 editor to check the higher layers parameters SystemInformationBlockType0, SystemInformationBlockType1 Agreement Make the following changes: - Change the title of Section 6.2.1.2 in 38.214 as follows 6.2.1.2 UE antenna switching UE sounding procedure for DL CSI acquisition - Support following change in UE capability 2-55 (SRS Tx Switch) Component-1 is a list of TRx pairs, candidates are {1T2R, 1T4R, 2T4R, 1T4R/2T4R, T=R} Agreement >>> Text proposal for 38.214 Section 6.2.1.2 >>> When UE antenna switching is enabled by the higher layer parameter SRS-SetUse set as 'antenna switching' for a UE that supports transmit antenna switching, a UE may be configured with one of the following configurations depending on the UE capability: - up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has with two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and where the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or - up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each SRS resource set has with two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and where the SRS port pair of the second resource in the set is associated with a different UE antenna port pair than the SRS port pair of the first resource in the same set, or - up to one SRS resource set configured with higher layer parameter resourceType in SRS-ResourceSet set to “periodic” or “semi-persistent”, withwith four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and where the SRS port of each resource in the set is associated with a different UE antenna port, and or zero or two SRS resource sets each configured with higher layer parameter resourceType in SRS-ResourceSet set to “aperiodic” and with a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. The UE shall expect that the two sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet, or, - up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4 and a guard period of Y symbols, where the UE does not transmit any other signal, is used in the case the SRS resource of a set are transmitted in the same slot. In this case, the guard period is in-between the SRS resources of the set. If the UE indicates support of “1T4R/2T4R” antenna switching and is configured with SRS sets with SRS-SetUse set as “antenna switching”, the UE is only expected to be configured with same number of SRS ports, either one or two, for all SRS resources in the sets. The UE does not expect to be configured or triggered more than one SRS resource set with SRS-SetUse set as “antenna switching” in the same slot, if the UE indicates support of “1T2R, 1T4R, 2T4R, or 1T4R/2T4R” antenna switching, otherwise the UE does not expect to be configured or triggered more than one SRS resource set with SRS-SetUse set as “antenna switching” in the same symbol. Agreement -------------------------------------------------------Start text proposal----------------------------------------------------------- 38.214 v15.0.0 6.2.1.3 UE sounding procedure between component carriers … For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters rf-RetuningTimeUL and rf-RetuningTimeDL) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in the [SRS_capability [13, TS 38.306]. For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to ‘typeA’, and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers., where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter SRS-SetUse set to ‘antenna switching’ and higher layer parameter resourceType in SRS-ResourceSet set to ‘aperiodic’. … --------------------------------------------------------End text proposal---------------------------------------------------------- For Type-B SRS switching across CCs, similar revisions to be made by the editor. Agreement Change in Section 6.2.1 of 38.214: “- if the UE is configured with the higher layer parameter SRS-SpatialRelationInfo set to 'SSB/PBCH', the UE shall transmit the SRS resource with the same spatial domain transmission filter used for the reception of the SSB/PBCH, if the higher layer parameter SRS-SpatialRelationInfo is set to 'CSI-RS', the UE shall transmit the SRS resource with the same spatial domain transmission filter used for the reception of the periodic CSI-RS or of the semi-persistent CSI-RS, or of the latest aperiodic CSI-RS. If the higher layer parameter SRS-SpatialRelationInfo is set to 'SRS', the UE shall transmit the SRS resource with the same spatial domain transmission filter used for the transmission of the periodic SRS or of the semi-persistent SRS or of the aperiodic SRS. “ Agreement The following text proposal is agreed. ============= Text Proposal in 6.4.1.4.2 in 38.211 ====================== if SRS-GroupSequenceHopping equals ’groupHopping’, group hopping but not sequence hopping shall be used and where the pseudo-random sequence is defined by clause 5.2.1 and shall be initialized with at the beginning of each radio frame. ============= End of Text Proposals in 38.211 ====================== is replaced with . Agreement -------------------------------------------------------Start text proposal----------------------------------------------------------- 38.211 v15.0.0 6.4.1.4.3 Mapping to physical resources If , frequency hopping is enabled and the frequency position indices are defined by where is given by Table 6.4.1.4.3-1, and where regardless of the value of … --------------------------------------------------------End text proposal---------------------------------------------------------- Agreement For intra-slot SRS antenna switching, during the gap of Y symbol(s), the same priority rules for SRS and other uplink channels are applicable. Agreement (RRC impact) Up to maxNrofSRS-TriggerStates-1 SRS triggering states can be configured for one SRS resource set. - As part of LS sent to RAN2 The LS is endorsed in R1-1807882 LS is endorsed in R1-1807723 Agreement Aperiodic TRS is supported for FR1 as a UE optional feature capability with separate FR1/FR2 indication - There are no additional specification details introduced to the existing FR2 design Note: MediaTek has concerns on the usefulness of this feature for FR1 Agreement A UE is not expected to be configured with CSI-ReportConfig with the higher layer parameter reportQuantity set to other than ‘none’ for aperiodic CSI-RS resource set configured with TRS info Agreement - CSI-RS resource set with TRS info configured with periodic CSI-RS resources has no linkage to any of the reporting settings Agreement Text Proposal for 5.1.6.1.1 38.214: For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot. Agreement - Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being ‘QCL-Type-A’ and ‘QCL-TypeD’, where applicable, with the periodic CSI-RS resources - Note: Huawei has concerns on the UE complexity due to lack of restriction on subcarrier location but for the progress of NR completion, will accept the above compromise Agreement The following working assumption on DM-RS antenna ports is confirmed. ---- Unchanged parts omitted---- 4.4.1 Antenna ports An antenna port is defined such that the channel over which a symbol on the antenna port is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed. - For DM-RS associated with PDSCH the channel over which a symbol on one of these antenna ports is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols are within the same scheduled PDSCH, same slot and in the same PRG as described in 3GPP TS 38.214 Section 5.1.2.3. ---- Unchanged parts omitted---- Agreement The following text proposal is agreed ---- Unchanged parts omitted, section 7.2 of 38.211---- 7.2 Physical resources The frame structure and physical resources the UE shall assume when receiving downlink transmissions are defined in Clause 4. The following antenna ports are defined for the downlink: - Antenna ports starting with 1000 for PDSCH - Antenna ports starting with 2000 for PDCCH - Antenna ports starting with 3000 for channel-state information reference signals - Antenna ports starting with 4000 for SS/PBCH block transmission A UE shall not assume that two antenna ports are quasi co-located with respect to any QCL type unless specified otherwise. The UE may assume that antenna ports within a CSI-RS resource are QCL with QCL Type A, Type D (when applicable) and average gain. The UE may assume that DMRS ports associated with a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain. ---- Unchanged parts omitted---- Agreement The following text proposal is agreed. ---- Unchanged parts omitted, section 10.1 of 38.213---- The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the control resource set configured by pdcch-ConfigSIB1 in MasterInformationBlock and for corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain and ‘QCL-typeA’ and ‘QCL-typeD’, , when applicable. The value for the DM-RS scrambling sequence initialization is the cell ID. A subcarrier spacing is provided by higher layer parameter subCarrierSpacingCommon in MasterInformationBlock. ---- Unchanged parts omitted ---- Agreement The following text proposal is agreed. ---- Unchanged parts omitted ---- The UE receives an activation command [10, TS 38.321] used to map up to 8 TCI states to the codepoints of the DCI field ‘Transmission Configuration Indication’. When the HARQ-ACK corresponding to the PDSCH carrying the activation command is transmitted in slot n, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from slot n+3Nslotsubframe,µ. After a UE receives higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the antenna ports of one DM-RS port group of PDSCH of a serving cell are quasi co-located with the SSB determined in the initial access procedure with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial Rx parameters, where applicable. ---- Unchanged parts omitted ---- Agreement: The QCL signalling in red are additionally agreed. QCL linkage for above 6GHz after RRC; signalling. SSB TRS w.r.t average delay, Doppler shift, spatial RX parameters; QCL type: C + D. TRS CSI-RS for BM w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A + D. TRS CSI-RS for CSI w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A. TRS DMRS for PDCCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A + D. TRS DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread estimation; QCL type: A + D. SSB CSI-RS for BM w.r.t. average delay, Doppler shift, spatial RX parameters; QCL type: C+D. SSB CSI-RS for CSI w.r.t, spatial RX parameters; QCL type: D. SSB DMRS for PDCCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; QCL type: A+D. SSB DMRS for PDSCH (before TRS is configured) w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; QCL type: A+D. CSI-RS for BM DMRS for PDCCH w.r.t. spatial RX parameters; QCL type: D. CSI-RS for BM DMRS for PDSCH w.r.t., spatial RX parameters; QCL type: D. CSI-RS for CSI DMRS for PDSCH w.r.t. average delay, Doppler shift, delay spread, Doppler spread, spatial RX parameters; Note: QCL parameters may not be derived directly from CSI-RS for CSI; QCL type: A+D. CSI-RS for BM CSI-RS for TRS/BM/CSI w.r.t. spatial RX parameters; QCL type: D. Agreement Note: For all the tables below, if a row has the same RS type, same RS ID should be assumed. If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE should only expect the following two possible configurations of the higher layer parameter TCI-State: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1*; SS/PBCH Block; QCL-TypeC; SS/PBCH Block; QCL-TypeD. 2*; SS/PBCH Block; QCL-TypeC; CSI-RS (BM); QCL-TypeD. * If QCL type-D is applicable, DL RS2 and QCL type-2 shall be configured for the UE If a CSI-RS resource is in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without repetition, the UE should only expect the following three possible configurations of the higher layer parameter TCI-State: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1**; TRS; QCL-TypeA; SS/PBCH Block; QCL-TypeD. 2**; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3*; TRS; QCL-TypeB. * If QCL type-D is not applicable ** If QCL type-D is applicable, DL RS2 and QCL type-2 shall be configured for the UE If a CSI-RS resource in a NZP-CSI-RS-ResourceSet is configured with higher layer parameter repetition, the UE should only expect the following three configurations of the higher layer parameter TCI-State: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; TRS; QCL-TypeA; TRS; QCL-TypeD. 2; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3; SS/PBCH Block; QCL-TypeC; SS/PBCH Block; QCL-TypeD. For the next two tables, if QCL type-D is applicable, DL RS2 and QCL type-2 shall be configured for the UE except for the default case (fourth row in the two tables below). If TRS for downlink is used for QCL type-D, the TRS must have an SSB or CSI-RS for BM as source RS for QCL type-D. For the DM-RS of PDCCH, the UE should only expect the following three configurations of the higher layer parameter TCI-State while the fourth configuration is valid as default, before TRS is configured: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; TRS; QCL-TypeA; TRS; QCL-TypeD. 2; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3**; CSI-RS (CSI); QCL-TypeA. 4*; SS/PBCH Block*; QCL-TypeA; SS/PBCH Block*; QCL-TypeD. * Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption **Note: Only when QCL type-D is not applicable For the DM-RS of PDSCH, the UE should only expect the three following configurations of the higher layer parameter TCI-State while the fourth is valid by default, before TRS is configured: Valid TCI state Configuration; DL RS 1; qcl-Type1; DL RS 2 (if configured); qcl-Type2 (if configured). 1; TRS; QCL-TypeA; TRS; QCL-TypeD. 2; TRS; QCL-TypeA; CSI-RS (BM); QCL-TypeD. 3**; CSI-RS (CSI); QCL-TypeA; CSI-RS (CSI); QCL-TypeD. 4*; SS/PBCH Block*; QCL-TypeA; SS/PBCH Block*; QCL-TypeD. * Before TRS configured. Note: this is not a TCI state, rather a valid QCL assumption ** Note: QCL parameters may not be derived directly from CSI-RS (CSI)

Scheduling/HARQ aspects

Agreements: - To adopt the TP to 7.3.2.2 of 38.211 For both interleaved and non-interleaved mapping, the UE may assume - the same precoding being used within a REG bundle if the higher-layer parameter precoderGranularity equals sameAsREG-bundle - the same precoding being used across the all resource-element groups within the set of contiguous resource blocks in the CORESET if the higher-layer parameter precoderGranularity equals allContiguousRBs Agreements: - To adop the TP to 7.4.1.3.2 of 38.211 - subcarrier 0 of the lowest-numbered resource block in the CORESET if the CORESET is configured by the PBCH or SIB1, Agreements: - To adopt the TP to 10.1 of 38.213: an indication for a presence or absence of a transmission configuration indication (TCI) field forDCI format 1_1 transmitted by a PDCCH in control resource set , by higher layer parameter TCI-PresentInDCI Agreements: Confirm the following working assumptions with updates: - At least for Dec. Drop, Case 1-1 and Case 1-2, map all candidates of USS search-space-set with lower SS set ID before candidates of USS with higher ID - If all candidates in a SS set can’t be mapped, any candidates in the SS set and in any subsequent SS sets are dropped (not mapped) - Case 2 FFS Agreements: - For following, regardless of whether one or more numerologies, the limit of BDs/CCEs per CC per slot is equal to the limit of BDs/CCEs for non-CA case. - Self-scheduling with up to 4 DL-CCs - Self-scheduling with up to T DL-CCs where the UE reports BD capability of y >= T - y is integer from (4, …, 16) - T is integer from (1, .., 16) - For cross-carrier scheduling where the scheduling CC and all the CCs schedulable by the scheduling CC have the same numerology and the number of DL-CCs is up to 4 or with up to T DL-CCs where the UE reports BD capability of y >= T, the limit of BDs/CCEs of the scheduling CC per slot is (the number of CCs schedulable by the scheduling CC) x (the limit of BDs/CCEs for non-CA case) - Note: this is inline with the previous agreements at RAN1#92bis meeting Agreements: - For self-scheduling with the same numerology or different numerologies, and the number of DL-CCs is <= 4 or with up to T DL-CCs where the UE reports BD capability of y >= T, network ensures the number of BDs/CCEs on any SCell does not exceed the non-CA limit. Agreements: - For self-scheduling with same numerology, and the number of DL-CCs is more than 4 and with up to T DL-CCs where the UE reports BD capability of y < T, the limit of BDs/CCEs per CC per slot is - The total number of BDs/CCEs across CCs is based on UE BD capability. It can be split across CCs, subject to the non-CA limit on each CC. - For SCell, NW ensures no overbooking based on non-CA case occurs. Agreements: - For UE BD capability reporting y = integer(4, …, 16), at least for self-scheduling, - For UE not supporting CA with different numerologies, the reported value equally applies to all numerologies - (Working assumption) For UE supporting CA with different numerologies, - When the UE is configured with CA with the same numerology, the reported value applies - When the UE is configured with CA with different numerologies, the reported value applies to each set of DL-CCs with the same numerology, i.e.: - If a UE is configured with DL-CCs of X0, X1, X2, X3, where Xi denotes the number of DL-CCs with the numerology i, the limit total number of (BDs or CCEs) for the DL-CCs with the numerology i is given by Floor{Xi / (X0 + X1 + X2 + X3) * (Mi or Ni) * y} per slot of the numerology i - Where Mi and Ni represent the number of BDs and CCEs per slot specified for non-CA case, respectively - Note: some of the values of Xi may be zero depending on the CA configuration - For each CC, the non-CA limit still applies - FFS the impact of BWP, if any Agreements: - TC-RNTI is not monitored on USS Agreements: - To adopt the following TP to 10.1 of 38.213: If, for a search space set , a CCE aggregation level , and a control resource set , any CCE index for a second PDCCH candidate overlaps with any CCE index for a first PDCCH candidate with index , where the PDCCH candidates are indexed in ascending order of a respective first CCE index , the UE is not expected to monitor the second PDCCH candidate with index Agreements: - For a given SS set or across SS sets, the UE is not expected to be configured with PDCCH monitoring occasion/periodicity causing partial time-domain overlapping for the same CORESET. - Partial-overlapping in time-domain for the same CORESET means the case of that the separation between two monitoring occasions is smaller than the CORESET duration but greater than 0. Conclusion: - It is understood that configuration of cross-carrier scheduling for a CC is UE-specific, i.e., applicable to all USS sets containing 0-1/1-1 configured on the active DL BWP of the scheduling cell enabled with cross-carrier scheduling with CIF. Agreements: - To reply LS in R1-1807732 - Regarding RAN2 agreements 1 & 2 - Agree to the answer in red on slide 4 of R1-1807838, with one update - “contains the CORESET#0…” - “as long as the CORESET#0…” - Regarding RAN2 agreements 3a - Answer: this is only for CSS configurations - For other RAN2 agreements, no comments from RAN1 Agreements: - PDCCH with CRC scrambled by C-RNTI and CS-RNTI are not monitored on Type3-CSS on SCell - When a UE is configured with cross-carrier scheduling and CS-RNTI for a CC, the UE monitors DCI format(s) 0_1 and/or 1_1 with CIF with CRC scrambled by CS-RNTI - For CS-RNTI, the CIF field is used to indicate cross-carrier activation of SPS/configured grant Type2 Agreements: Update previous agreements as follows: - NW and UE maintain the same understanding on SSB/CORESET#0/SS#0 in connected_mode at least for non-broadcast PDCCH and for broadcast PDCCH - Solutions FFS - For the broadcast PDCCH, it is up to UE which common search space to monitor based on which SSB in both connected, in-active, and idle modes - Unicast PDSCH can be scheduled by a DCI associated with the CORESET #0 Agreements: - Clarify the agreements at RAN1#91 as follows: - Case 1: PDCCH monitoring of all SS sets that are monitored in a slot occurs within 3 consecutive OFDM symbols that have fixed positions in each slot periodicity of 14 or more symbols - Case 1-1: PDCCH monitoring on up to three OFDM symbols at the beginning of a slot - Case 1-2: PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot - For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot - Case 2: PDCCH monitoring other than Case 1periodicity of less than 14 symbols - Note: this includes the PDCCH monitoring of up to three OFDM symbols at the beginning of a slot Agreements: - For self-scheduling with different numerologies, and the number of DL-CCs is more than 4 and with up to T DL-CCs where the UE reports BD capability of y < T, the limit of BDs/CCEs per CC per slot is - (Working assumption) The total number of BDs/CCEs across CCs per numerology is based on UE BD capability. It can be split across CCs for the given numerology, subject to the non-CA limit on each CC. - If a UE is configured with DL-CCs of X0, X1, X2, X3, where Xi denotes the number of DL-CCs with the numerology i, the limit of (BDs or CCEs) for the DL-CCs with the numerology i is given by Floor{Xi / (X0 + X1 + X2 + X3) * (Mi or Ni) * y} per slot of the numerology i - For SCell, NW ensures no overbooking based on non-CA case occurs Agreements: - For clarification on association b/w CORESET and A0/A1/A2, select one option from the following. - For a CORESET with ControlResourceSetId, the corresponding Ap that is used in the PDCCH hashing is determined by p = ControlResourceSetId modulo 3. Agreements: - The TP in R1-1807901 is endorsed [Draft] Reply LS on subcarrier spacing for BWPs and TDD configurations is endorsed. The final LS is in R1-1807676 Agreements: When a configured SFI monitoring is cancelled under the condition that the symbols are indicated to be UL or flexible by an earlier SFI - (Follow earlier agreement on configure DCI format 2_0 not detected) For the slots till the next configured DCI format 2_0 monitoring occasion with slot format not provided by earlier SFI, PDCCH monitoring is performed but the UE will cancel RRC configured transmission, and assume RRC configured DL transmission is not transmitted, during semi-static configured flexible symbols - Spec change may not be needed - If GC-PDCCH monitoring is skipped due to DRX (treat this the same as if the configured GC-PDCCH monitoring is not detected) - (Follow earlier agreement on configure DCI format 2_0 not detected) For the slots in the DRX ON cycle before the next configured DCI format 2_0 monitoring occasion, PDCCH monitoring is performed but the UE will cancel RRC configured transmission, and assume RRC configured DL transmission is not transmitted, during semi-static configured flexible symbols - Spec change may not be needed Agreements: If the SFI monitored in one cell is used to cancel an UE-specific RRC configured UL transmission in another BWP in another cell, the N2 for the cancellation timeline is with respect to the SCS the SFI is monitored with. Agreements: - Confirm the working assumption on slot format 55. (Already captured in spec) - On slot format indication for ECP, reuse the slot format indication for NCP, and determine the direction of an ECP symbol using the indicated direction of (partially) overlapping NCP symbols - If one of the (partially) overlapping NCP symbol is D, and the other (partially) overlapping NCP symbol is U, the ECP symbol will be X Agreements: When configure the UE-specific SFI table, each entry at least explicitly specifies the slot formats for the slots equals to the configured SFI monitoring periodicity. Agreements: For Rel.15, on semi-static flexible symbol in a TDD cell, UE does not expect simultaneous RRC configurated DL reception and RRC configured UL transmission in the same symbol Conclusion: For the previous agreement “For a slot covered by multiple SFIs transmitted at different slots, the UE does not expect to receive different slot format indicated by different SFIs”, this include the case 255 is used for the slot format as well. Agreements: For the previous agreement on “RRC configured DL reception and RRC configured UL transmission cancellation based on miss-detection of configured DCI format 2_0 monitoring” only applies for slots when their slot formats are not indicated by any other detected DCI format 2_0 The TPs in R1-1807813 are endorsed. Agreements: - For CS-RNTI DCI sizes - For DCI format 1-1 with CS-RNTI, the DCI size are the same as the DCI format 1-1 with C-RNTI in respective search space. - For DCI format 0-1 with CS-RNTI, the DCI size are the same as the DCI format 0-1 with C-RNTI in respective search space. - For DCI format 1-0 with CS-RNTI, the DCI size are the same as the DCI format 1-0 with C-RNTI in respective search space. - For DCI format 0-0 with CS-RNTI, the DCI size are the same as the DCI format 0-0 with C-RNTI in respective search space. Agreements: - Confirm the working assumptions in the previous agreements as shown below: - For PDSCH scheduled by P-RNTI (working assumption), and RA-RNTI, scaling factor is applied in the calculation of Ninfo of TBS determination - (Working assumption) values of scaling factor: 1, ½, ¼ - FFS the possibility of adding another value, e.g., 1/8 or 1/16 - DCI content for paging updated as shown in the table Field; Bits; Comment. Short Messages Indicator; 12; Already captured in 38.212. Need to add interpretation.. In case of short message service. Short message; [8]; Defined by RAN2. The number of bits to be checked with RAN2.. Reserved bits; To meet overall DCI size. In case of paging message. Frequency domain resource assignment. Time domain resource assignment; 4; Note: applicability of the currently agreed default table to paging or whether an additional default table or SIB1 signaled table is FFS.. VRB-to-PRB mapping; [1]; Depends on whether interleaved mapping is mandatory or not. Modulation and coding scheme; 5; Same MCS table as for “normal” transmission without 256QAM. TB scaling; 2; To scale TB size. Reserved bits; (fixed value); To meet overall DCI size. Agreements: - The 1-bit indicator in DCI 2-x(=0,1,2,3) formats is removed Agreements: When the DCI size for DCI formats 0-0/1-0 in USS is derived from the initial BWP but applied to another BWP, apply scaling according to - Derive the resource allocation (start and length) from the RIV field assuming the initial BWP - Scale start and length by a factor of K - Apply the scaled start and length to the target BWP K is the max of - ratio between the size of the active BWP and the initial BWP, rounded down to the nearest power of 2 - 1 Agreements: - Reuse DCI format 1-0 with C-RNTI and with resource allocation field equal to all ones for PDCCH order - Note: need to update the spec such that no HARQ ACK is reported for PDCCH order Agreements: - If the size of DCI format 0-0/1-0 is determined by the initial downlink BWP - Determine the size of the resource allocation field for format 0-0 from respective (UL or SUL) initial uplink BWP - If overall DCI size is larger than what is defined by DCI format 1-0, truncate the resource allocation field (MSBs) to match the overall size of formats 0-0 and 1-0 Agreements: - For DCI format 1-1, the number of bits for the PDSCH-to-HARQ-timing-indicator field is given by ceil(log2(#values configured by RRC)) Agreements: - If both PDSCH/PUSCH mapping type A and B are configured for a UE, the size of the antenna port field is determined by the maximum required for type A and B. - Zero-padding of MSBs is used if necessary to fill the antenna port field. Agreements: - Inform RAN2 to remove the comment in the RRC spec that the size for DCI format 2-1 must be a multiple of 14. RAN1 only uses multiples of 14 bits for preemption indication and discards the remaining bits (if any) in DCI format 2-1. - LS in R1-1807841, which is approved and final LS in R1-1807903 Agreements: - The TP as listed in proposal 3.1 in R1-1807662 is adopted - Note: the part “m=0, 1, …” is the one replacing the deleted text for m Agreements: - On a per PUCCH group basis: - If a UE is configured with overlapping PUCCH resources for CSI-only reporting in a slot, - If a UE is configured with multi-CSI-PUCCH-Config, the UE multiplexes all the CSI reports corresponding to CSI only PUCCH resources (overlapping or not) in the slot on a single multi-CSI-PUCCH resource, following the agreed priority rules for CSI reporting. - If a UE is not configured with multi-CSI-PUCCH-Config, among all the PUCCH resources for CSI reporting, the UE selects maximum two non-overlapping PUCCH resources for CSI reports with the highest priority. - If two non-overlapping PUCCH resources are selected, they include at least one with PUCCH format 2. - For CSI-only reporting in a slot, if a UE is configured with more than two non-overlapping CSI reports in a slot, the UE selects the maximum two PUCCH resources with the highest priority CSI repots. - If two non-overlapping PUCCH resources are selected, they include at least one with PUCCH format 2. Agreements: - For step 1, while there are overlapping PUCCH resources in a slot, - The UE determines the PUCCH resource (resource A) that overlaps with at least another PUCCH resource with the priority order of earliest followed by largest duration (if applicable) - In case of multiple candidates for resource A, the UE can pick one (implementation-specific) - The UE determines a set of PUCCH resources (resource set X) overlapping with PUCCH resource A. - The UE determines a PUCCH resource and corresponding UCI for multiplexing the PUCCH resource A and PUCCH resources in set X in one shot. - The determined PUCCH resource and the corresponding UCI replace resource set X and resource A - For step 1, when there is no more overlapping PUCCH resources in the slot, the UE determines “the timeline requirements for overlapping UL channels”. If the requirement is met, the UE proceeds to Step 2. - Note: the above is per PUCCH group - The above agreements is to replace Step 1 in the agreements under 7.1.3.2.3 Agreements: - When a PUCCH resource carrying SR and other UCI overlaps with PUSCH, if the UCI on PUCCH is going to be multiplexed on PUSCH, the SR bit(s) is dropped from the UCI before multiplexing on PUSCH. Agreements: - When a UE is configured for one PUCCH resource set for UCI transmission including HARQ-ACK, the UE is not expected to be provided with simultanouesARQ-ACK-CSI=TRUE. Agreements: - If a HARQ-ACK transmission from a UE using PUCCH format 1 overlaps with SR transmission occasions corresponding to K SR PUCCH configurations each using PUCCH format 1 in a slot, - In case of negative SR, the HARQ-ACK is transmitted on the HARQ-ACK resource. In case of positive SR, the HARQ-ACK is transmitted on the SR resource corresponding to the positive SR. Agreements: - If a HARQ-ACK transmission from a UE using PUCCH format 0 overlaps with SR transmission occasions corresponding to K SR PUCCH configurations, in a slot, - If any SR is positive, indicate positive SR. Else indicate negative SR Agreements: - To add OACK & OSR in determining the number of CRC bits as part of Section 9.2.5.2 of 38.213 Agreements: - To adopt the following TP to 5.2.2.2 of 38.211: - Note: the TP is not to change the contents of the tables below, but to re-order the sequences <Unchanged parts are omitted> Table 5.2.2.2-2: Definition of for. 0; -3; 1; -3; -3; -3; 3; -3; -1; 1; 1; 1; -3. 1; -3; 3; 1; -3; 1; 3; -1; -1; 1; 3; 3; 3. 2; -3; 3; 3; 1; -3; 3; -1; 1; 3; -3; 3; -3. 3; -3; -3; -1; 3; 3; 3; -3; 3; -3; 1; -1; -3. 4; -3; -1; -1; 1; 3; 1; 1; -1; 1; -1; -3; 1. 5; -3; -3; 3; 1; -3; -3; -3; -1; 3; -1; 1; 3. 6; 1; -1; 3; -1; -1; -1; -3; -1; 1; 1; 1; -3. 7; -1; -3; 3; -1; -3; -3; -3; -1; 1; -1; 1; -3. 8; -3; -1; 3; 1; -3; -1; -3; 3; 1; 3; 3; 1. 9; -3; -1; -1; -3; -3; -1; -3; 3; 1; 3; -1; -3. 10; -3; 3; -3; 3; 3; -3; -1; -1; 3; 3; 1; -3. 11; -3; -1; -3; -1; -1; -3; 3; 3; -1; -1; 1; -3. 12; -3; -1; 3; -3; -3; -1; -3; 1; -1; -3; 3; 3. 13; -3; 1; -1; -1; 3; 3; -3; -1; -1; -3; -1; -3. 14; 1; 3; -3; 1; 3; 3; 3; 1; -1; 1; -1; 3. 15; -3; 1; 3; -1; -1; -3; -3; -1; -1; 3; 1; -3. 16; -1; -1; -1; -1; 1; -3; -1; 3; 3; -1; -3; 1. 17; -1; 1; 1; -1; 1; 3; 3; -1; -1; -3; 1; -3. 18; -3; 1; 3; 3; -1; -1; -3; 3; 3; -3; 3; -3. 19; -3; -3; 3; -3; -1; 3; 3; 3; -1; -3; 1; -3. 20; 3; 1; 3; 1; 3; -3; -1; 1; 3; 1; -1; -3. 21; -3; 3; 1; 3; -3; 1; 1; 1; 1; 3; -3; 3. 22; -3; 3; 3; 3; -1; -3; -3; -1; -3; 1; 3; -3. 23; 3; -1; -3; 3; -3; -1; 3; 3; 3; -3; -1; -3. 24; -3; -1; 1; -3; 1; 3; 3; 3; -1; -3; 3; 3. 25; -3; 3; 1; -1; 3; 3; -3; 1; -1; 1; -1; 1. 26; -1; 1; 3; -3; 1; -1; 1; -1; -1; -3; 1; -1. 27; -3; -3; 3; 3; 3; -3; -1; 1; -3; 3; 1; -3. 28; 1; -1; 3; 1; 1; -1; -1; -1; 1; 3; -3; 1. 29; -3; 3; -3; 3; -3; -3; 3; -1; -1; 1; 3; -3. Table 5.2.2.2-3: Definition of for 0; -1; 3; -1; -3; 3; 1; -3; -1; 3; -3; -1; -1; 1; 1; 1; -1; -1; -1. 1; 3; -3; 3; -1; 1; 3; -3; -1; -3; -3; -1; -3; 3; 1; -1; 3; -3; 3. 2; -3; 3; 1; -1; -1; 3; -3; -1; 1; 1; 1; 1; 1; -1; 3; -1; -3; -1. 3; -3; -3; 3; 3; 3; 1; -3; 1; 3; 3; 1; -3; -3; 3; -1; -3; -1; 1. 4; 1; 1; -1; -1; -3; -1; 1; -3; -3; -3; 1; -3; -1; -1; 1; -1; 3; 1. 5; 3; -3; 1; 1; 3; -1; 1; -1; -1; -3; 1; 1; -1; 3; 3; -3; 3; -1. 6; -3; 3; -1; 1; 3; 1; -3; -1; 1; 1; -3; 1; 3; 3; -1; -3; -3; -3. 7; 1; 1; -3; 3; 3; 1; 3; -3; 3; -1; 1; 1; -1; 1; -3; -3; -1; 3. 8; -3; 1; -3; -3; 1; -3; -3; 3; 1; -3; -1; -3; -3; -3; -1; 1; 1; 3. 9; 3; -1; 3; 1; -3; -3; -1; 1; -3; -3; 3; 3; 3; 1; 3; -3; 3; -3. 10; -3; -3; -3; 1; -3; 3; 1; 1; 3; -3; -3; 1; 3; -1; 3; -3; -3; 3. 11; -3; -3; 3; 3; 3; -1; -1; -3; -1; -1; -1; 3; 1; -3; -3; -1; 3; -1. 12; -3; -1; -3; -3; 1; 1; -1; -3; -1; -3; -1; -1; 3; 3; -1; 3; 1; 3. 13; 1; 1; -3; -3; -3; -3; 1; 3; -3; 3; 3; 1; -3; -1; 3; -1; -3; 1. 14; -3; 3; -1; -3; -1; -3; 1; 1; -3; -3; -1; -1; 3; -3; 1; 3; 1; 1. 15; 3; 1; -3; 1; -3; 3; 3; -1; -3; -3; -1; -3; -3; 3; -3; -1; 1; 3. 16; -3; -1; -3; -1; -3; 1; 3; -3; -1; 3; 3; 3; 1; -1; -3; 3; -1; -3. 17; -3; -1; 3; 3; -1; 3; -1; -3; -1; 1; -1; -3; -1; -1; -1; 3; 3; 1. 18; -3; 1; -3; -1; -1; 3; 1; -3; -3; -3; -1; -3; -3; 1; 1; 1; -1; -1. 19; 3; 3; 3; -3; -1; -3; -1; 3; -1; 1; -1; -3; 1; -3; -3; -1; 3; 3. 20; -3; 1; 1; -3; 1; 1; 3; -3; -1; -3; -1; 3; -3; 3; -1; -1; -1; -3. 21; 1; -3; -1; -3; 3; 3; -1; -3; 1; -3; -3; -1; -3; -1; 1; 3; 3; 3. 22; -3; -3; 1; -1; -1; 1; 1; -3; -1; 3; 3; 3; 3; -1; 3; 1; 3; 1. 23; 3; -1; -3; 1; -3; -3; -3; 3; 3; -1; 1; -3; -1; 3; 1; 1; 3; 3. 24; 3; -1; -1; 1; -3; -1; -3; -1; -3; -3; -1; -3; 1; 1; 1; -3; -3; 3. 25; -3; -3; 1; -3; 3; 3; 3; -1; 3; 1; 1; -3; -3; -3; 3; -3; -1; -1. 26; -3; -1; -1; -3; 1; -3; 3; -1; -1; -3; 3; 3; -3; -1; 3; -1; -1; -1. 27; -3; -3; 3; 3; -3; 1; 3; -1; -3; 1; -1; -3; 3; -3; -1; -1; -1; 3. 28; -1; -3; 1; -3; -3; -3; 1; 1; 3; 3; -3; 3; 3; -3; -1; 3; -3; 1. 29; -3; 3; 1; -1; -1; -1; -1; 1; -1; 3; 3; -3; -1; 1; 3; -1; 3; -1. Table 5.2.2.2-4: Definition of for 0; -1; -3; 3; -1; 3; 1; 3; -1; 1; -3; -1; -3; -1; 1; 3; -3; -1; -3; 3; 3; 3; -3; -3; -3. 1; -1; -3; 3; 1; 1; -3; 1; -3; -3; 1; -3; -1; -1; 3; -3; 3; 3; 3; -3; 1; 3; 3; -3; -3. 2; -1; -3; -3; 1; -1; -1; -3; 1; 3; -1; -3; -1; -1; -3; 1; 1; 3; 1; -3; -1; -1; 3; -3; -3. 3; 1; -3; 3; -1; -3; -1; 3; 3; 1; -1; 1; 1; 3; -3; -1; -3; -3; -3; -1; 3; -3; -1; -3; -3. 4; -1; 3; -3; -3; -1; 3; -1; -1; 1; 3; 1; 3; -1; -1; -3; 1; 3; 1; -1; -3; 1; -1; -3; -3. 5; -3; -1; 1; -3; -3; 1; 1; -3; 3; -1; -1; -3; 1; 3; 1; -1; -3; -1; -3; 1; -3; -3; -3; -3. 6; -3; 3; 1; 3; -1; 1; -3; 1; -3; 1; -1; -3; -1; -3; -3; -3; -3; -1; -1; -1; 1; 1; -3; -3. 7; -3; 1; 3; -1; 1; -1; 3; -3; 3; -1; -3; -1; -3; 3; -1; -1; -1; -3; -1; -1; -3; 3; 3; -3. 8; -3; 1; -3; 3; -1; -1; -1; -3; 3; 1; -1; -3; -1; 1; 3; -1; 1; -1; 1; -3; -3; -3; -3; -3. 9; 1; 1; -1; -3; -1; 1; 1; -3; 1; -1; 1; -3; 3; -3; -3; 3; -1; -3; 1; 3; -3; 1; -3; -3. 10; -3; -3; -3; -1; 3; -3; 3; 1; 3; 1; -3; -1; -1; -3; 1; 1; 3; 1; -1; -3; 3; 1; 3; -3. 11; -3; 3; -1; 3; 1; -1; -1; -1; 3; 3; 1; 1; 1; 3; 3; 1; -3; -3; -1; 1; -3; 1; 3; -3. 12; 3; -3; 3; -1; -3; 1; 3; 1; -1; -1; -3; -1; 3; -3; 3; -1; -1; 3; 3; -3; -3; 3; -3; -3. 13; -3; 3; -1; 3; -1; 3; 3; 1; 1; -3; 1; 3; -3; 3; -3; -3; -1; 1; 3; -3; -1; -1; -3; -3. 14; -3; 1; -3; -1; -1; 3; 1; 3; -3; 1; -1; 3; 3; -1; -3; 3; -3; -1; -1; -3; -3; -3; 3; -3. 15; -3; -1; -1; -3; 1; -3; -3; -1; -1; 3; -1; 1; -1; 3; 1; -3; -1; 3; 1; 1; -1; -1; -3; -3. 16; -3; -3; 1; -1; 3; 3; -3; -1; 1; -1; -1; 1; 1; -1; -1; 3; -3; 1; -3; 1; -1; -1; -1; -3. 17; 3; -1; 3; -1; 1; -3; 1; 1; -3; -3; 3; -3; -1; -1; -1; -1; -1; -3; -3; -1; 1; 1; -3; -3. 18; -3; 1; -3; 1; -3; -3; 1; -3; 1; -3; -3; -3; -3; -3; 1; -3; -3; 1; 1; -3; 1; 1; -3; -3. 19; -3; -3; 3; 3; 1; -1; -1; -1; 1; -3; -1; 1; -1; 3; -3; -1; -3; -1; -1; 1; -3; 3; -1; -3. 20; -3; -3; -1; -1; -1; -3; 1; -1; -3; -1; 3; -3; 1; -3; 3; -3; 3; 3; 1; -1; -1; 1; -3; -3. 21; 3; -1; 1; -1; 3; -3; 1; 1; 3; -1; -3; 3; 1; -3; 3; -1; -1; -1; -1; 1; -3; -3; -3; -3. 22; -3; 1; -3; 3; -3; 1; -3; 3; 1; -1; -3; -1; -3; -3; -3; -3; 1; 3; -1; 1; 3; 3; 3; -3. 23; -3; -1; 1; -3; -1; -1; 1; 1; 1; 3; 3; -1; 1; -1; 1; -1; -1; -3; -3; -3; 3; 1; -1; -3. 24; -3; 3; -1; -3; -1; -1; -1; 3; -1; -1; 3; -3; -1; 3; -3; 3; -3; -1; 3; 1; 1; -1; -3; -3. 25; -3; 1; -1; -3; -3; -1; 1; -3; -1; -3; 1; 1; -1; 1; 1; 3; 3; 3; -1; 1; -1; 1; -1; -3. 26; -1; 3; -1; -1; 3; 3; -1; -1; -1; 3; -1; -3; 1; 3; 1; 1; -3; -3; -3; -1; -3; -1; -3; -3. 27; 3; -3; -3; -1; 3; 3; -3; -1; 3; 1; 1; 1; 3; -1; 3; -3; -1; 3; -1; 3; 1; -1; -3; -3. 28; -3; 1; -3; 1; -3; 1; 1; 3; 1; -3; -3; -1; 1; 3; -1; -3; 3; 1; -1; -3; -3; -3; -3; -3. 29; 3; -3; -1; 1; 3; -1; -1; -3; -1; 3; -1; -3; -1; -3; 3; -1; 3; 1; 1; -3; 3; -3; -3; -3. <Unchanged parts are omitted > Agreements: - When single slot PUCCH overlaps with multi-slot PUSCH repetition in a slot, if the timeline requirement is met for the overlapped slot, multiplexing UCI on PUSCH in the overlapped slot; otherwise, it is considered as an error case for the overlapped slot Agreements: - At least for Dec drop, when multi-slot PUCCH repetition overlap with single/multi-slot PUSCH repetition in time, dropping PUSCH without deferral in overlapping slots if the timeline requirement within overlapping slots is met; otherwise; it is considered as the error case for overlapping slots Agreements: - In 6.3.2.4.1 of TS38.211, - No intra-slot frequency hopping is replaced as higher layer parameter PUCCH-frequency-hopping = “disabled”. - Intra-slot frequency hopping is replaced as higher layer parameter PUCCH-frequency-hopping = “enabled”. - Add the following: - The channel over which a symbol on the antenna port used for UL transmission is conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols correspond to the same frequency hop, when frequency hop is enabled, irrespective of whether the frequency hop distance is zero or not. Agreements: For HARQ-ACK, CSI part 1, and CSI part 2 (if exists) transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1, and CSI part 2 (exists), are determined as follows: If CSI part 2 exists, - SE is the spectrum efficiency which is code rate * modulation order - Note: if HARQ-ACK is up to 2 bits, in the equation for calculation is the amount of reserved REs assuming 2 bits HARQ-ACK Agreements: - For omission of lower priority information bits for CSI part 2 on PUSCH without UL-SCH, the threshold coding rate is given by where R is the signaled code rate in DCI. Agreements: Within a group of overlapping PUCCH/PUSCH channels satisfying the timeline requirements, adopt the following procedure - Step 1: determine a set of non-overlapping (in time)PUCCH resource(s) for UCI multiplexing by considering only the PUCCH resources in the group (irrespective whether or not PUSCH(s) exists), where each PUCCH resource satisfies the timeline requirements - Step 2: - if the resulting PUCCH resource(s) in step 1 overlapping with PUSCH(s), multiplex UCIs on the overlapping PUSCH(s); - FFS: When UCI includes SR - Otherwise, multiplex UCI on the determined PUCCH resource(s) Agreements: - DCI format 0_1 includes an ‘UL-SCH indicator’ field of 1 bit. Modulation and code rate are determined by I_MCS field in DCI. Agreements: In intra-band CA, if PRACH transmission and PUSCH/PUCCH/SRS transmission collide. - how to handle collision is up to UE implementation except when PRACH and SRS collides, PRACH take precedence. - Note: The definition of collision follows the PRACH procedure agreements in RAN1 92bis. Agreements: - In a PUCCH group, within one set of colliding PUCCHs/PUSCHs channels, when the detemined PUCCH resource overlaps with one or multiple PUSCHs which satisfy timeline requirements for overlapped UL channels, - UE first determines CC index for UCI multiplexing based on procedure defined in 38.213 V15.1.0 - If there are multiple PUSCHs on the determined CC, UE multiplex PUCCH on the PUSCH with earliest starting symbol on that CC. Agreements: - The UE is not expected to be configured to transmit on the same OFDM symbol with an SRS resource and a PUCCH/PUSCH across different CCs in intra-band CA - Note: no spec change is needed. - Parallel SRS and PUCCH/PUSCH transmissions across CCs is supported in inter-band CA. - Note: if case parallel SRS and PUCCH/PUSCH is supported, the SRS resource does not belong to a set which is for antenna switching, if the SRS resource set for antenna switching has more than one SRS resource (T < R) - Supporting of this feature is subject to UE capability which is a separate capability Agreements: - Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs is supported in inter-band CA - Supporting of this feature is subject to UE capability which is a separate capability Agreements: - Adopt the following TP in 38.212 Section 7.3.1.1.2 - 1st downlink assignment index – 1 or 2 bits - 1 bit for semi-static HARQ-ACK codebook; - 2 bits for dynamic HARQ-ACK codebook with single HARQ-ACK codebook. Working assumption: - In the WA of timeline requirements on overlapping UL channels made in RAN1 92bis, , where T = 1 - If is not applicable, - If is not applicable, - If is not applicable, - Note: T is based on the as specified in 38.214. - Note: the definition ofs , , and follows the same definitions in TS 38.214. Agreements: Confirm working assumption as: For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, - One RMSI value is used for indicating cell-specific PRB offset = Floor(NBWP/4) for PUCCH duration = 14 symbols. - NBWP is the size of initial UL BWP - For 2 symbols, 4 symbols and 10 symbols, - Floor(NBWP/4) is not specified. - At least one additional offset is specified in number of PRBs for each duration. - Note: Offset = 0 has been agreed. Agreements: For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, adopt the following table for specifying cell-specific PUCCH resources/parameters: - NBWP is the size of initial UL BWP RMSI value; PUCCH format; Starting symbol; Number of symbols; Cell-specific PRB offset; Initial CS index set. 0; 0; 12; 2; 0; {0, 3}. 1; 0; 12; 2; 0; {0, 4, 8}. 2; 0; 12; 2; 3; {0, 4, 8}. 3; 1; 10; 4; 0; {0, 6}. 4; 1; 10; 4; 0; {0, 3, 6, 9}. 5; 1; 10; 4; 2; {0, 3, 6, 9}. 6; 1; 4; 10; 0; {0, 6}. 7; 1; 4; 10; 0; {0, 3, 6, 9}. 8; 1; 4; 10; 2; {0, 3, 6, 9}. 9; 1; 0; 14; 0; {0, 6}. 10; 1; 0; 14; 0; {0, 3, 6, 9}. 11; 1; 0; 14; 2; {0, 3, 6, 9}. 12; 1; 0; 14; 4; {0, 3, 6, 9}. 13; 1; 0; 14; Floor(NBWP/4); {0, 3, 6, 9}. 14; FFS. 15; FFS. Agreements: For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, the PUCCH resource within the is determined as belows. - PRB offset = cell-specific PRB offset + UE-specific PRB offset - It is up to the editor to adopt one option among the two below, where - nCS is the number of CS indices in the CS index set. - m is the PUCCH resource indicator field value, - CCEstart is the starting CCE index of the DCI received - NCCE,pNCCE,p is the number of CCEs in control resource set p. - Option 1: Equation-based expression. - PUCCH resource index rPUCCH = 2m+Floor(2*CCEstart/NCCE,p) - Hopping direction h = Floor(rPUCCH/8) - UE-specific PRB offset p = Floor((rPUCCH-8h)/nCS) - CS iIndex in the initial CS index set c = Mod (rPUCCH-8h, nCS) - Option 2: Table-based expression. - CCE-based implicit bit = Floor(2*CCEstart/NCCE,p) - For RMSI value with initial CS index set {0, 3}, ARI bits; CCE-based implicit bit; Hopping direction; UE specific PRB offset; Initial CS index. 000; 0; 0; 0; 0. 1; 3. 001; 0; 1; 0. 1; 3. 010; 0; 2; 0. 1; 3. 011; 0; 3; 0. 1; 3. 100; 0; 1; 0; 0. 1; 3. 101; 0; 1; 0. 1; 3. 110; 0; 2; 0. 1; 3. 111; 0; 3; 0. 1; 3. - For RMSI values with initial CS index set {0, 4, 8}, ARI; CCE-based implicit bit; Hopping direction; UE-specific PRB offset; Initial CS index. 000; 0; 0; 0; 0. 1; 4. 001; 0; 8. 1; 1; 0. 010; 0; 4. 1; 8. 011; 0; 2; 0. 1; 4. 100; 0; 1; 0; 0. 1; 4. 101; 0; 8. 1; 1; 0. 110; 0; 4. 1; 8. 111; 0; 2; 0. 1; 4. - For RMSI values with initial CS index set {0, 6}, ARI; CCE-based implicit bit; Hopping direction; UE specific PRB offset; Initial CS index. 000; 0; 0; 0; 0. 1; 6. 001; 0; 1; 0. 1; 6. 010; 0; 2; 0. 1; 6. 011; 0; 3; 0. 1; 6. 100; 0; 1; 0; 0. 1; 6. 101; 0; 1; 0. 1; 6. 110; 0; 2; 0. 1; 6. 111; 0; 3; 0. 1; 6. - For RMSI values with initial CS index set {0, 3, 6, 9}, ARI; CCE-based implicit bit; Hopping direction; UE specific PRB offset; Initial CS index. 000; 0; 0; 0; 0. 1; 3. 001; 0; 6. 1; 9. 010; 0; 1; 0. 1; 3. 011; 0; 6. 1; 9. 100; 0; 1; 0; 0. 1; 3. 101; 0; 6. 1; 9. 110; 0; 1; 0. 1; 3. 111; 0; 6. 1; 9. Agreements: For PUCCH resource allocation before a UE has a dedicated PUCCH configuration, adopt the following table for specifying cell-specific PUCCH resources/parameters: - NBWP is the size of initial UL BWP RMSI value; PUCCH format; Starting symbol; Number of symbols; Cell-specific PRB offset; Initial CS index set. 0; 0; 12; 2; 0; {0, 3}. 1; 0; 12; 2; 0; {0, 4, 8}. 2; 0; 12; 2; 3; {0, 4, 8}. 3; 1; 10; 4; 0; {0, 6}. 4; 1; 10; 4; 0; {0, 3, 6, 9}. 5; 1; 10; 4; 2; {0, 3, 6, 9}. 6; 1; 10; 4; 4; {0, 3, 6, 9}. 7; 1; 4; 10; 0; {0, 6}. 8; 1; 4; 10; 0; {0, 3, 6, 9}. 9; 1; 4; 10; 2; {0, 3, 6, 9}. 10; 1; 4; 10; 4; {0, 3, 6, 9}. 11; 1; 0; 14; 0; {0, 6}. 12; 1; 0; 14; 0; {0, 3, 6, 9}. 13; 1; 0; 14; 2; {0, 3, 6, 9}. 14; 1; 0; 14; 4; {0, 3, 6, 9}. 15; 1; 0; 14; Floor(NBWP/4); {0, 3, 6, 9}. Agreements: Update the default tables for time-domain allocation as shown below. - Note: not all the entries are valid for RMSI scheduling - For pattern 2, rows 1-7, 8, 9, 10, 11, and 15 are valid for RMSI scheduling - For pattern 3, at least rows 2-5 & 9-12 are valid for RMSI scheduling Table 5.1.2.1.1-3: Default PDSCH time domain resource allocation B Row index; dmrs-TypeA-Position; PDSCH mapping type; K0; S; L. 1; 2,3; Type B; 0; 2; 2. 2; 2,3; Type B; 0; 4; 2. 3; 2,3; Type B; 0; 6; 2. 4; 2,3; Type B; 0; 8; 2. 5; 2,3; Type B; 0; 10; 2. 6; 2,3; Type B; 1; 2; 2. 7; 2,3; Type B; 1; 4; 2. 8; 2,3; Type B; 0; 2; 4. 9; 2,3; Type B; 0; 4; 4. 10; 2,3; Type B; 0; 6; 4. 11; 2,3; Type B; 0; 8; 4. 12; 2,3; Type B; 0; 10; 4. 13; 2,3; Type B; 0; 2; 7. 14; 2; Type A; 0; 2; 12. 3; Type A; 0; 3; 11. 15; 2,3; Type B; 1; 2; 4. Table 5.1.2.1.1-4: Default PDSCH time domain resource allocation C Row index; dmrs-TypeA-Position; PDSCH mapping type; K0; S; L. 1; 2,3; Type B; 0; 2; 2. 12; 2,3; Type B; 0; 4; 2. 23; 2,3; Type B; 0; 6; 2. 34; 2,3; Type B; 0; 8; 2. 45; 2,3; Type B; 0; 10; 2. 6; Reserved. 7; Reserved. 8; 2,3; Type B; 0; 2; 4. 9; 2,3; Type B; 0; 4; 4. 10; 2,3; Type B; 0; 6; 4. 11; 2,3; Type B; 0; 8; 4. 12; 2,3; Type B; 0; 10; 4. 13; 2,3; Type B; 0; 2; 7. 14; 2; Type A; 0; 2; 12. 3; Type A; 0; 3; 11. 15; 2; Type A; 0; 0; 6. 16; 3; Type A; 0; 2; 6. Agreements: Adopt the following default tables for ECP: - For PUSCH time-domain RA table for ECP, j=2 follows the previous agreements on 60kHz, Delta = 4. Default PDSCH time-domain RA table for ECP (note: this case is not applicable to RMSI since ECP for RMSI is not supported) Row index; DL-DMRS-typeA-pos; PDSCH mapping type; K0; S; L. 1; 2; Type A; 0; 2; 6. 3; Type A; 0; 3; 5. 2; 2; Type A; 0; 2; 10. 3; Type A; 0; 3; 9. 3; 2; Type A; 0; 2; 9. 3; Type A; 0; 3; 8. 4; 2; Type A; 0; 2; 7. 3; Type A; 0; 3; 6. 5; 2; Type A; 0; 2; 5. 3; Type A; 0; 3; 4. 6; 2; Type B; 0; 6; 4. 3; Type B; 0; 8; 2. 7; 2; Type B; 0; 4; 4. 3; Type B; 0; 6; 4. 8; 2,3; Type B; 0; 5; 6. 9; 2,3; Type B; 0; 5; 2. 10; 2,3; Type B; 0; 9; 2. 11; 2,3; Type B; 0; 10; 2. 12; 2,3; Type A; 0; 1; 11. 13; 2,3; Type A; 0; 1; 6. 14; 2,3; Type A; 0; 2; 4. 15; 2,3; Type B; 0; 4; 6. 16; 2,3; Type B; 0; 8; 4. Default PUSCH time domain RA table with ECP Row index; PUSCH mapping type; K2'; S; L. 1; Type A; j; 0; 8. 2; Type A; j; 0; 12. 3; Type A; j; 0; 10. 4; Type B; j; 2; 10. 5; Type B; j; 4; 4. 6; Type B; j; 4; 8. 7; Type B; j; 4; 6. 8; Type A; j+1; 0; 8. 9; Type A; j+1; 0; 12. 10; Type A; j+1; 0; 10. 11; Type B; j+2; 0; 6. 12; Type A; j+2; 0; 12. 13; Type A; j+2; 0; 10. 14; Type B; j; 8; 4. 15; Type A; j+3; 0; 8. 16; Type A; j+3; 0; 10. Agreements: - For slot aggregation, - the RV field in the DCI scheduling the transmission indicates the RV of the first slot of the PDSCH/PUSCH transmission, - slots cycle through the RVs in order 0, 2, 3, 1 starting from the RV indicated in the DCI Agreements: For RAR and msg4: - if PDSCH-ConfigCommon has provided a table - Use this table - else - Use the default PDSCH table A Agreements: - For TB size determination, assume Xoh=0 for all transmission scheduled with broadcast RNTIs (SI-RNTI, RA-RNTI, P-RNTI,) and msg3 regardless of UE state. - Adopt following TP for 38.214: 6.1.4.1 Modulation order and target code rate determination For PUSCH scheduled by RAR UL Grant or the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI 6.1.4.2 Transport block size determination For PUSCH scheduled by RAR UL Grant or the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI. Agreements: For C-RNTI and CS-RNTI in CSS at least in Pcell (FFS Scell Type 3 CSS) - If the CSS is not associated with CORESET#0 and PDSCH-Config has provided a table - Use this table for PDSCH transmission scheduled by DCI in CSS - else if PDSCH-ConfigCommon has provided a table - Use this table for PDSCH transmission scheduled by DCI in CSS - else - Use the default PDSCH table A (mux pattern 1) - Note: Same table is used as in USS - Note: If some entries in the table before RRC reconfiguration and in the table after RRC reconfiguration are the same, the UE can be schedulable during the RRC reconfiguration period - No specification impact - Note: it is assumed that random access response can be received in CORESET #0 For C-RNTI and CS-RNTI in CSS: - if the CSS is not associated with CORESET#0 and PUSCH-Config has provided a table - Use this table for PUSCH transmission scheduled by DCI in CSS - else if PUSCH-ConfigCommon has provided a table for PUSCH - Use this table for PUSCH transmission scheduled by DCI in CSS - else - Use the default PUSCH table - Note: Same table is used as in USS - Note: If some entries in the table before RRC reconfiguration and in the table after RRC reconfiguration are the same, the UE can be schedulable during the RRC reconfiguration period - No specification impact - Note: it is assumed that random access response can be received in CORESET #0 Agreements: - Increase the number of bits for RIV in RAR from 12 bits to 14 bits. Agreements: - Change Delta from 2 to 3 for 30 kHz for msg 3 processing Agreements: - In addition to previously agreed rows, also row 8 in Table C can be used for RMSI scheduling Agreements: - For UE behavior in RRC_IDLE, if PDSCHs among SI-RNTI PDSCH, P-RNTI PDSCH, and RA-RNTI/TC-RNTI PDSCH are overlapped with at least one symbol for a given UE from the primary cell - NR supports UE to decode up to two PDSCH simultaneously in Pcell, but not more than two (if more than two are received, the decoding prioritization is up to UE implementation) - Note: the above assumes that all the PDSCHs to be decoded by the UE have non-overlapped PRBs Agreements: The following working assumption as part of the previous agreement is confirmed with updates: - While UE acquires SI upon being triggered by Paging DCI - UE is not required to decode C-RNTI PDSCH if the SI-RNTI PDSCH is overlapped with at least one symbol - In case UE autonomously monitors SI-RNTI PDCCH while monitoring C-RNTI PDCCH, and both SI-RNTI PDSCH and C-RNTI PDSCH are overlapped with at least one symbol, the UE is not required to decode SI-RNTI PDSCH - (Working assumption) The first two bullets apply unless TBS of SI-RNTI PDSCH ≤ 2976 for FR1, then UE decodes both SI-RNTI PDSCH and C-RNTI PDSCH - Note: the above assumes that the PDSCHs to be decoded by the UE have non-overlapped PRBs - The first two bullets always apply in FR2 Agreements: - To adopt the following TP: ------ Text Proposal for TS 38.214 Subclause 5.3 ------ N1 is based on µ of table 5.3-1 that corresponds to the min (µDL, µUL) where the µDL corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in subclause 4.41 of [4, TS 38.211] If the higher layer parameter dmrs-AdditionalPosition is configured with a value greater than 0 (at least for one of PDSCH mapping Type A or B) or if the high layer parameter is not configured (at least for one of PDSCH mapping Type A or B), then N1 value follows from the second column of table 5.3-1; otherwise, the N1 value follows from the first column of table 5.3-1. ------ End of Text Proposal ------ Agreements: The following working assumption is confirmed with updates: - The processing requirement for HARQ-ACK in response to DL SPS release is N OFDM symbols from last symbol of the corresponding PDCCH for UE Capability #1: - 15 kHz case: 8 + 2 5 - 30 kHz case: 10 + 2 5 - 60 kHz case: 17 + 5 - 120 kHz case: 20 + 5 Agreements: In the case of multiplexing CSI with uplink data on PUSCH, the MIMO session should update agreements related to CSI multiplexing with UL-SCH (i.e., data on PUSCH), and NR should adopt the framework below: - Z’’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the DCI triggering a CSI report on PUSCH to the earliest possible start of the corresponding CSI transmission on PUSCH from UE perspective - Z’’ ≥ Z + d where Z is based on CSI-only processing time from the MIMO session - N2’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant to the earliest possible start of the corresponding PUSCH from UE perspective - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH - d symbols - FFS value of d <= N2 - E.g., d=1 - E.g., d=1 for 15kHz, 2 for 30kHz, 3 for 60kHz, and 4 for 120kHz - E.g., d=N2 - UE is not expected transmit the CSI multiplexed with uplink data if the network set the values of K2 and SLIV without leaving sufficient time for UE processing Agreements: - d in the above agreement is updated to the following - 2 for 15kHz - 2 for 30kHz - 3 for 60kHz - 4 for 120kHz Conclusion: - The following limitation in DL control procedure is beneficial (possible even necessary) from a processing time perspective. - FFS whether to capture in specification or as capability. (If proposed as capability, this would be in addition to feature 3-5, 3-5a); For type 1 with dedicated RRC configuration, type 3, and UE-SS,, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a ocassion gap; For type 1 with dedicated RRC configuration, type 3 and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation between two occasions for any search space, is one or more of the following cases for a given SCS 1OFDM symbol 2OFDM symbols 4OFDM symbols 7OFDM symbols. Agreements: - Cross-carrier scheduling should at least satisfy the causality constraints between scheduling PDCCH and PDSCH as for self-scheduling, also taking into carrier timing difference - Note: in the case of mixed numerology, limitations on the number of symbols to buffer need to be taken into account Agreements: - When HARQ-ACK for the PDSCH with larger SCS is transmitted on a carrier with smaller SCS - K1=0 corresponds to the slot for the smaller SCS which overlaps with the PDSCH - When HARQ-ACK for the PDSCH with smaller SCS is transmitted on a carrier with larger SCS - K1=0 corresponds to the slot for the larger SCS with end boundary aligned to the slot for the corresponding PDSCH Conclusion: - Note that below are already in specification - When larger SCS schedules smaller SCS, overlapping slot is considered as K0=0 or K2=0 - When smaller SCS schedules larger SCS, K0=0 or K2=0 is defined for slot with boundary aligned with that of slot containing the corresponding PDCCH - Slot timing is defined by the start of the slot Agreements: - For cross-carrier scheduling across different numerology - FFS: how to specify additional constraints related to K0 to address the number of symbols which may need to be buffered Agreements: - For any two HARQ process IDs A and B for a given cell, - If the scheduling DCI scrambled by C-RNTI for unicast PUSCH transmission A comes before (in time) the scheduling DCI scrambled by C-RNTI for unicast PUSCH transmission B, then for the Dec. 2017 baseline capability - UE is not expected to be scheduled such that PUSCH for B is before the PUSCH for A - For any two HARQ process IDs A and B for a given cell, - If the scheduling DCI scrambled by C-RNTI for unicast PDSCH transmission A comes before (in time) the scheduling DCI scrambled by C-RNTI for unicast PDSCH transmission B, then for the Dec. 2017 baseline capability - UE is not expected to be scheduled such that PDSCH for B is before the PDSCH for A Agreements: - To adopt the following TP to Section 5.3 of 38.214: If the first uplink symbol of the physical channel which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUSCH or PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1 then the UE shall provide a valid HARQ-ACK message, where L1 is defined as the next uplink symbol with its CP starting after after the end of the last symbol of the PDSCH carrying the TB being acknowledged. - N1 is based on µ of table 5.3-1 that corresponds to the min (µDL,PDCCH, µDL,PDSCH, µUL) where the µDL,PDCCH corresponds to the subcarrier spacing of the downlink with which the PDCCH was transmitted, µDL,PDSCH corresponds to the subcarrier spacing of the downlink with which the PDSCH was transmitted, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in subclause 4.41 of [4, TS 38.211] If the higher layer parameter dmrs-AdditionalPosition is configured with a value greater than 0, or if the high layer parameter is not configured, then N1 value follows from the second column of table 5.3-1; otherwise, the N1 value follows from the first column of table 5.3-1. Agreements: The Capability #2 for (Aggressive) UE processing time in Rel-15 is supported under the following conditions - Non-CA - Note: this does not preclude EN-DC - FFS CA case with Capability #2 supported on only one or more of the carriers, and potential handling of some special cases - Single numerology for PDCCH, PDSCH, and PUSCH for the serving cell - PDSCH/PUSCH allocation with mapping Type A and Type B - For PDSCH mapping type A with last PDSCH symbol ending in symbol ‘i' of a slot, where i < 7 - N1 processing time is increased by (7-i) relative to the case where i=7. - (Working assumption) For PDSCH mapping type B with 4 or 2 symbols - N1 processing time is increased by ‘d’ symbols relative to the case of PDSCH with 7 symbols, where ‘d’ is the amount of time-domain overlap in symbols between the scheduling PDCCH and the scheduled PDSCH - FFS: handling of 3-symbol CORESET where first 2 symbols of CORESET are overlapped with a 2-symbol PDSCH - No UCI multiplexing - FFS: whether similar multiplexing rule as with Capability #1 may be included - For C-RNTI - FFS: simultaneous reception with broadcast PDSCH - Note: The UE signals whether Capability #2 is supported for each SCS, and separately for uplink and downlink Agreements: The Capability #2 UE processing times are given below, where the PDSCH allocation length is assumed to be at least 7 symbols. - N1 - 15kHz: N1 = 3 - 30kHz: N1 = 4.5 - Note: as part of UE capability, - UE may report support with maximum scheduled RB allocation of 136 RBs - If RB allocation by scheduling exceeds the maximum signaled, UE defaults to Capability #1 processing time - OR UE may report support with no restriction on maximum scheduled RB allocation - 60kHz for FR1: 9 - N2 - 15kHz: N2 = 5 for CP-OFDM & For DFT-S-OFDM - 30kHz: N2 = 5.5 - 60kHz for FR1: 11 - If 1st symbol of PUSCH is data-only or FDM data with DMRS, then add 1 symbol to N2. Agreements: In the case of multiplexing CSI, HARQ-ACK, and uplink data on PUSCH - N1’ the number of OFDM symbols required for UE processing from the end of PDSCH to the earliest possible start of the corresponding ACK/NACK transmission on PUSCH from UE perspective - N1’ ≥ N1 + d where N1 is based on the UE capability for ACK-only - Z’’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the DCI triggering a CSI report on PUSCH to the earliest possible start of the corresponding CSI transmission on PUSCH from UE perspective - Z’’ ≥ Z + d where Z is based on CSI-only processing time from the MIMO session - N2’ is the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant to the earliest possible start of the corresponding PUSCH from UE perspective - N2’ ≥ N2 + d where N2 is based on the UE capability for sending data-only on PUSCH - d symbols - 2 for 15kHz - 2 for 30kHz - 3 for 60kHz - 4 for 120kHz - UE is not expected transmit the CSI multiplexed with HARQ-ACK and with uplink data if the network set the values of K1, K2 and SLIV without leaving sufficient time for UE processing Agreements: - The TP in Section 3 of R1-1807902 is endorsed Agreements: - UE is not required to receive on a downlink symbol and then transmit on a uplink symbol if those two symbols are not separated by at least Rx2Tx us on unpaired spectrum for a given serving cell, from the UE perspective - Discuss further whether it’s an error case or to specify a UE behavior - Note that the exact value of Rx2Tx has been specified in RAN4 [R4-1805766] Agreements: - Adopt following TP for 38.213 subclause 10.2. A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or UL Type 2 configured grant PDCCH only if all the following conditions are met: - the CRC parity bits of the DCI format are scrambled with a CS-RNTI provided by higher layer parameter cs-RNTI - the new data indicator field is set to ‘0’. In case of DCI formats 1-1, the new data indicator field refers to the one for the enabled transport block. Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2. If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or UL configured grant Type 2. If validation is not achieved, the UE considers the DCI format as having been detected with a non-matching CRC. Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation DCI format 0_0/0_1; DCI format 1_0; DCI format 1_1. HARQ process number; set to all ‘0’s; set to all ‘0’s; set to all ‘0’s. Redundancy version; set to ‘00’; set to ‘00’; For the enabled transport block: set to ‘00’. Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation DCI format 0_0; DCI format 1_0. HARQ process number; set to all ‘0’s; set to all ‘0’s. Redundancy version; set to ‘00’; set to ‘00’. Modulation and coding scheme; set to all ‘1’s; set to all ‘1’s. Resource block assignment; set to all ‘1’s; set to all ‘1’s. Agreements: - To adopt the following TP (to reflect a previous agreement) to 6.1.2.3 of 38.214: For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or when a UL grant for scheduling the same TB is received within the period P, whichever is reached first. UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. Agreements: - Update the conclusion from RAN1#92bis as follows: Conclusion: - It is RAN1 understanding that UE is not expected to handle cases where the Ncb Nref value (in 5.4.2.1) changes across transmissions for the same transport block. - No spec impact. Conclusion: - Performance is not expected to be optimized for the following special cases (while supported in specification) in which retransmissions of a transport block occur at or near peak rate - the TB is transmitted with significantly lower code rate (considering initial and retransmission) than R_LBRM(=2/3) - the retransmission occurs on a much shorter PDSCH duration than the initial transmission - No spec change Agreements: - To adopt the TP to Section 5.4.2.1 of 38.212 maximum number of layers for one TB supported by the UE for the serving cell, which for UL-SCH is according to higher layer parameter ULmaxRank if the parameter is configured; The draft LS is approved. Final LS in R1-1807873. R1-1807729 Agreements: - When DCI format 1_1 is used for active BWP switching, - A UE ignores the following DCI bitfields after MSB zero-padding when the old active DL BWP is configured with 1 TB and the indicated DL BWP is configured with 2 TBs - Modulation and coding scheme for TB2 - New data indicator for TB2 - Redundancy version for TB2 - FFS For “transmission configuration indication” bitfield, a UE follows the TCI for the CORESET with the lowest CORESE ID of the new active DL BWP in the same slot as the scheduled PDSCH - For “frequency domain resource assignment” bifield with the following cases, no further special handling, in addition to MSB truncation and zero-padding, is needed - When the indicated DL BWP is configured with Type 0 resource allocation only - When the indicated DL BWP is configured with both Type 0 & Type 1 resource allocation - In case of MSB zero-padding, Type 0 resource allocation is always applied - When the indicated DL BWP is configured with Type 1 resource allocation only & bitfield truncation is applied - For “frequency domain resource assignment” bitfield with the following case, continue discussion on whether further special handling, in addition to MSB truncation and zero-padding, is needed - When the indicated DL BWP is configured with Type 1 resource allocation only & MSB zero-padding is applied - For other DCI bitfields, no further special handling, in addition to MSB truncation and zero-padding, is required - When DCI format 0_1 is used for active BWP switching, - FFS For “SRS resource indicator” bitfield, a UE follows the spatial relationship of the PUCCH resource with the lowest PUCCH resource ID of the new active UL BWP - FFS For “PTRS-DMRS association” bitfield, no further special handling, in addition to MSB truncation and zero-padding, is needed - For “frequency domain resource assignment” bitfield with the following cases, no further special handling, in addition to MSB truncation and zero-padding, is required - When the indicated UL BWP is configured with Type 0 resource allocation only - When the indicated UL BWP is configured with both Type 0 & Type 1 resource allocation - In case of MSB zero-padding, Type 0 resource allocation is always applied - When the indicated UL BWP is configured with Type 1 resource allocation only & bitfield truncation is applied - For “frequency domain resource assignment” bitfield with the following case, continue discussion whether further special handling, in addition to MSB truncation and zero-padding, is required - When the indicated UL BWP is configured with Type 1 resource allocation only & bitfield zero-padding is applied - For other DCI bitfields, no further special handling, in addition to MSB truncation and zero-padding, is required Agreements: - If a UE doesn’t support DCI-based active BWP switching, the UE ignores the “BWP indicator” bitfield in DCI format 1_1 and DCI format 0_1 when the bitfield exists - Note: it doesn’t imply that DCI-based active BWP switching is an optional feature Agreements: - It was noticed by RAN1 that there is an agreement regarding that a UE should report UL Tx DC location to the network by a semi-static signalling. However, such signalling wasn’t implemented in RAN2’s RRC spec. yet. RAN1 kindly asks RAN2 to implement such signalling. - FFS details of the signalling - Check offline further on DL Tx DC location related issue - To send an LS to RAN2 – Peikai (MTK), R1-1807905, which is approved and final LS in R1-1807910 Agreements: - When DCI format 1_1 is used for active BWP switching, - For “frequency domain resource assignment” bitfield with the following case, no further special handling, in addition to MSB truncation and zero-padding, is needed - When the indicated DL BWP is configured with Type 1 resource allocation only & MSB zero-padding is applied - For “transmission configuration indication” bitfield, - When there is no TCI bitfield in the BWP switching DCI, a UE follows the TCI indication defined in the spec, assuming the TCI bitfield disabled, based on the old active DL BWP - Otherwise, no further special handling, in addition to MSB truncation and zero-padding, is needed - When DCI format 0_1 is used for active BWP switching, - For “frequency domain resource assignment” bitfield with the following case, no further special handling, in addition to MSB truncation and zero-padding, is required - When the indicated UL BWP is configured with Type 1 resource allocation only & bitfield zero-padding is applied - (working assumption) For “PTRS-DMRS association” bitfield, - When there is no “PTRS-DMRS association” bitfield in the BWP switching DCI, a UE follows the behaviour defined in the spec, assuming “PTRS-DMRS association” bitfield disabled - Otherwise, no further special handling, in addition to MSB truncation and zero-padding, is needed Agreements: - If there is active UL BWP switching between the time of TA command reception and the corresponding TA applying time, a UE applies the TA command based on the numerology of new active UL BWP for the UL transmission in new active UL BWP - Otherwise, A UE assumes the same absolute TA value before & after active UL BWP switching, including the case of numerology change Agreements: - For UL, a 13-bit indicator is used to indicate the Tx DC location corresponding to a UL BWP within a carrier - This information is provided for each UL BWP configuration - It’s also RAN1’s understanding that this information can be provided in RRCReconfigurationComplete from a UE to the network when a UL BWP is configured - 1 bit is used to indicate whether there is 7.5 KHz shift - “0” means no 7.5 KHz shift - “1” means 7.5 KHz shift - 12 bits are used to indicate the Tx DC location with the following table Subcarrier index (12 bits) within the carrier corresponding to the numerology of the UL BWP; Tx DC location. 0; 0. ⁞; ⁞. 3299; 3299. 3300; Outside the carrier. 3301; Undetermined position within the carrier. 3302; Reserved. 3303; Reserved. ⁞; Reserved. 4095; Reserved. - For DL, 12-bit indicator is used to indicate the Tx DC location corresponding to a numerology within a carrier - This information is provided for each numerology potentially used for the carrier - It’s RAN1’s understanding that this information can be optionally provided in SCS-SpecificCarrier from the network to a UE - It’s RAN1’s understanding that the UE can assume that Tx DC location is outside the carrier if this information is not provided – up to RAN4 to handle performance related issues - 12 bits are used to indicate the Tx DC location with the following table Subcarrier index (12 bits) within the carrier corresponding to a numerology; Tx DC location. 0; 0. ⁞; ⁞. 3299; 3299. 3300; Outside the carrier. 3301; Reserved. 3302; Reserved. 3303; Reserved. ⁞; Reserved. 4095; Reserved. Conclusion: - It is understood that only TB-level HARQ-ACK is supported for SPS PDSCH Agreements: - To clarify in 38.213 that “among the DCI formats 1_0 or DCI formats 1_1 indicating the same slot for HARQ-ACK reporting” is intended for the last DCI Agreements: - For semi-static HARQ codebook, for PDSCH with slot aggregation over R slots, keep current specifications and include that number of slots can be >1. Agreements: - UE is not expected to transmit more than one PUCCH with HARQ-ACK in a slot at least for Dec. 2017 drop. Agreements: - UE reports a PDSCH detection outcome (ACK/NACK) for a PDSCH occasion only in HARQ-ACK codebook indicated by PDSCH-to-HARQ timing field in corresponding DL DCI. UE reports NACK/DTX in other HARQ-ACK codebooks where UE reports HARQ-ACK for the PDSCH occasion Agreements: - Semi-static HARQ-ACK codebook size does not depend on the time duration between a last symbol for any PDSCH reception candidate and a first symbol for a PUCCH/PUSCH transmission that includes the HARQ-ACK codebook - UE sets to NACK the HARQ-ACK bits in the HARQ-ACK codebook that correspond to PDSCH reception occasions that do not satisfy the above timing requirement - No additional specification impact Agreements: - Prioritize grant-based PUSCH over grant-free PUSCH for UCI multiplexing Agreements: When an PDCCH candidate collides with any RE of LTE CRS configured for NR PDSCH rate-matching, where PDSCH is semi-statically not mapped, as described in TS38.214 Section 5.1.4.1 then - The UE is not required to monitor the PDCCH candidate overlapping with the resource even only when the overlaps is by one RE - Note: this is the same behavior as agreed for collision with RB-level in RAN1#92b Agreements: - To adop the following TP to 5.1.4 of 38.214: “a pair of reserved resources in numerology of the BWP indicated by an RB level bitmap (higher layer parameter resourceBlocks given by RateMatchPattern) with 1RB granularity and a symbol level bitmap spanning one or two slots (higher layer parameters symbolsInResourceBlock given by RateMatchPattern) for which the reserved RBs apply. The bit equal to 1 in the RB and symbol level bitmaps indicates that corresponding resource is not available for PDSCH. For each pair of RB and symbol level bitmaps, a UE may be configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by RateMatchPattern ) corresponding to a unit equal to a duration of the symbol level bitmap, and bit equal to 1 indicating whether the pair is present in the unit or not.” …. A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of RB symbol level resource set indices forming a union of resource-sets not available for PDSCH dynamically if corresponding bit in the PDCCH with a scheduling DCI. The REs corresponding to the union of configured RB-symbol level resource-sets that are not included in neither of the two groups are not available for PDSCH.” The draft LS is approved (by accepting all changes). Final LS in R1-1807907

Channel coding

Agreement: For transmission on PUCCH, if the number of bits in a CSI part 2 is less than 3 bits, zeros shall be appended to the CSI part 2 sequence until the payload size equals 3. Text proposal 2 in R1-1807765 is agreed. The text proposals in Section 2.4 of R1-1807042 are endorsed. Agreement: In the determination of EUCI, a UE assumes the number of CRC bits to be 11 when the payload size. FFS: Further check if a similar correction is needed to 36.213 to the procedure for selection of PUCCH resource and the dropping of CSI.

NR-LTE co-existence

The text proposal in R1-1806890 is endorsed. The editor to incorporate the TP into TS36.213. Conclusion: - Current RAN1 assumption is that that the switching between LTE and NR on different carrier frequencies for EN-DC operation is negligible for the cases below - Intra-band TDD EN-DC band combination described in TS38.101-3 where ‘single UL allowed’ is applicable - Intra-band FDD EN-DC band combination described in TS38.101-3 where ‘single UL allowed’ is applicable - Inter-band EN-DC band combination described in TS38.101-3 where ‘single UL allowed’ is applicable - RAN1 assumes RAN4 is discussing switching times for the above cases Conclusion: When the UE is configured with multiple UL carriers on different frequencies, but the UE operates only on one of the UL carriers at a given time among a pair of LTE and NR carriers, LTE operation with an LTE TDD PCell using the same HARQ timing as LTE operation without EN-DC is supported by the current RAN1 specifications at least when no LTE UL CA is used at least for the following cases - The UE supports dynamic power sharing - The UE is configured with P_LTE + P_NR not greater than P_cmax

UL power control

Agreement For large payload size for delta_TF,c, K1 = 2.4 Agreement For small payload size for delta_TF,c, K2 = 6.0 [linear] Agreement The following working assumption is confirmed For group-common TPC command in DCI format 2-2, One bit closedloopindex field is present along with the TPC command in the DCI format 2_2 to indicate which closed loop the TPC command applies to - When a UE is configured with 1 closed loop, the bit closedloopindex field is NOT present. - When a UE is configured with 2 closed loops, the bit location of closedloopindex field follows the 2 bit TPC command. Notes: UE assumes that the start bit of the TPC command is X*tpc_index, where X=3 if configured with 2 closed-loop process, otherwise X=2. Conclusion No further RAN1 specification support for additional cases of resetting closed-loop accumulation in Rel-15. Agreement For PUSCH scheduled by DCI format 0_0, if the PUSCH transmission uses the spatial relation for a PUCCH resource (i.e., as agreed in RAN1#92bis). - The UE shall use the PL reference associated with the PUCCH resource whose spatial relation is used for the PUSCH transmission; - P0 and alpha are given by, the values corresponding p0alphasetindex=0 of p0-pusch-alpha-setconfig (i.e., j=2). - PUSCH closed loop index l = 0. Agreement: K value for non-scheduled UL transmission is the minimum of the common configured K2 values of the associated BWP. - Applies for both PUSCH and SRS Working Assumption For PUCCH, K value for non-scheduled UL transmission is the minimum of the common configured K2 values Working Assumption For group common TPC - If group TPC commands for PUSCH are received the K symbols before PUSCH transmission period i, the accumulation is updated according to all the group common TPC commands; - If group TPC commands for PUCCH are received the K symbols before PUCCH transmission period i, the accumulation is updated according to all the group common TPC commands; - If group TPC commands for SRS not tied with PUSCH are received the K symbols before SRS transmission period i, the accumulation is updated according to all the group common TPC commands; Notes: How to capture the above is up to editor, especially for the time unit of i and K. Agreement In case of UL CA with different SCSs, when the slot of the UL transmission carrying the PHR overlaps with the slot on CC with higher SCS, the PH of the first slot with higher SCS that fully overlaps with the slot with the lower SCS is reported. Agreement RAN1 changes the text in TS38.213 to be consistent with TS38.321 on how UE determines whether PH value for an activated Serving Cell is based on real transmission or a reference format. The text from TS38.321 is provided below: “UE determines whether PH value for an activated Serving Cell is based on real transmission or a reference format by considering the downlink control information which has been received until and including the PDCCH occasion in which the first UL grant is received since a PHR has been triggered.” Working Assumption For CA, when calculating the PHR of a cell based on a real transmission, transmissions whose DCI come after the first DCI after PHR trigger are ignored. Conclusion Editor to note the text proposal below which was agreed in RAN1#92. To capture the following text proposal: ---------------- Start TP for 38.213 sub clause 7.6.1 ---------------------------------- If the UE indicates a capability for dynamic power sharing between EUTRA and NR and - if the UE is not configured for operation with shortened TTI and processing time on the MCG [13, TS 36.213], and - if the UE transmission(s) in subframe of the MCG overlap in time with UE transmission(s) in slot of the SCG, and - if in any portion of slot of the SCG, the UE reduces transmission power in any portion of slot of the SCG so that in any portion of slot , where and are the linear values of the total UE transmission powers in subframe of the MCG and in slot of the SCG, respectively. -------------------- End TP ---------------------------------------------------------------- Working Assumption Cross-carrier indication for power control parameters - For PUCCH, if higher layer parameters “cell” and “bwp-Id” are included in the PUCCH-SpatialRelationInfo configuration: - The reference signal corresponding to pathloss index ‘q_d’ included in PUCCH-SpatialRelationInfo is from the serving cell and active BWP indicated by the higher layer parameters “cell” and “bwp-Id”. Note: No RRC specification impact. Agreement Selection of UL carrier for virtual PUSCH PHR - For the PC setting for virtual PUSCH PHR in case of CA/DC, for a cell configured with non-SUL and SUL, a default UL carrier is selected according to the following rules: - Semi-statically configured carrier for PUSCH if present, otherwise - The semi-statically configured carrier for PUCCH if present, otherwise the non-SUL carrier Conclusion - PUCCH-SCell is included in ‘a secondary cell’ and no update of TS38.213 is needed.

UE feature

The draft LS is approved. Final LS in R1-1807896 R1-1807891 Updated NR UE features NTT DOCOMO, AT&T Note: in the approved R1-1807891, the editorial change for entry 2-33a should be reverted. However, this change will be reflected in RAN1#94 Note: Current discussion so far condiers only UE processing capability #1. Agreement: For the candidate set value of FG 2-25: X1={2, 4, 8}, X2= {4, 8, 14}, X4={14, 28, 56} Candidate value set for X3 is {8, 14, 28} Agreement: For 2-28, remove SCS 15 and 30 Agreement For 2-30, change the candidate value set for component-2: Component-2, candidate value set is {2, 4, 8, 16} Agreement For 2-54a, change the component-1 to: 1. Maximum number of simultaneous transmitted SRS resources per CC at one symbol for NCB PUSCH Agreement For 2-14, add a new component: “Supported max number of SRS resource per set (SRS set use is configured as for codebook).” With Candidate value: {1, 2} Agreement 2-4; TCI states for PDSCH; 1. Support number of active TCI states per BWP per CC, including control and data 2. maximum number of configured TCI states per CC for PDSCH; 2-1; Yes; Only one TCI state can be supported; Type 1; N.A.; N.A.; Component-1: Candidate value set: {1, 2, 3, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64} FFS: mandatory value for component-2. 2-4a; Additional active TCI state for PDCCH; Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH; 2-1; Yes; No additional TCI state for control; Type 1; N.A.; N.A.; Note: Only applicable if Component-1 of 2-4 is set to 1; Mandatory with capability signalling. Agreement On 2-1, add a new component as below (for mandatory without capability ) 3. Support one active TCI state Agreement: For each of Type I, Type I multi-panel, Type II, Type II with port selection codebooks, clarify that the number of CSI-RS resources and total number of ports used are counted multiple times (n times) if the same resource or port is used in multiple (n) simultaneous CSI calculations. Add a note to clarify the above agreement in the feature table. Agreement: For FG2-28, candidate value set: Alt-1: {14, 28, 48} Alt-2: {“14-14”, “28-28”, “48-48”, “14-336”, “28-336”, “48-336”}, where the first threshold corresponding to the case when UE doesn’t activate antenna panel (using same panel to receive DCI and CSI-RS), and the second threshold corresponding to the case when UE may activate antenna panel (other than the panel used to receive DCI) Note: Alt-2 requires some spec change to support more than 4 slots offset between DCI and A-CSI-RS transmission. Discuss additional spec requirements for Alt-2 in main session in RAN1#93, if support more than 24 slot offsets between DCI and A-CSI-RS transmission are supported, Alt.2 is agreed, otherwise Alt.1 is agreed. Discuss in RAN plenary Agreement: For FG 2-33, candidate value set for component-2: {2, 4, 8, 12, 16 : 8 : 256} Agreement For 2-15a, 2-53, 2-55, 2-57, keep the signalling granuality as 4) per band per band comb. Agreement: For 2-36, 2-40, 2-41 and 2-43, maximal size of the list is 16 Agreement: For 2-36, Component-2 candidate set is {“Mode-1 only”, “Mode-1 and Mode-2”}. Agreement For 2-36, 2-40, 2-41, and 2-43: add a new component: - Max # of CSI-RS resource in a resource set, FR1/FR2 differentiation: candidate value is {1:8} Remove 2-39 Agreement For 2-41, candidate value set for component-4 is: {“no amplitude subset restriction”, “support amplitude subset restriction”} Agreement For 2-54, - Remove 2-54 from UE feature list. - Specify the minimum time interval, N in unit of symbols, between DCI triggering and A-SRS transmission is N2+42. Agreement Add a new FG 2-51a; Aperiodic TRS; DCI triggering Aperiodic TRS associated with periodic TRS; 2-50; Yes; A-TRS is not supported; Type 1; N.A.; Yes; Optional with capability signaling. Agreement: 2-51a is differentiated by FR1 and FR2. Proposal: Add a new FG as below: 2-33b; SP CSI-RS; 1. Support SP CSI-RS; 2-1; Yes; Type 4; No; Yes; RAN1; FFS on the mandatory or optional. 2-33c; SP CSI-IM; 2. Support SP CSI-IM; 2-1; Yes; Type 4; No; Yes; RAN1; FFS on the mandatory or optional. Make final decision in the main session: Agreements: On 2-50: TRS bandwidth configuration = BWP or min(52, BWP) - TRS bandwidth configuration does not imply UE processing bandwidth

NR URLLC

Agreements: - For Rel-15, capture the functionalities of all the agreements made for URLLC generically in RAN1 specs (i.e. without mentioning “URLLC”). Agreements: - For CQI table for URLLC with target BLER of 10^-5, - At least some entries for CQI table for target BLER of 10^-1 are re-used - The highest SE entry is 772/1024*6 - The lowest SE entry is between (30 ~ 40)/1024*2 Agreements: - The MCS table for CP-OFDM based PUSCH is separate from the MCS table for DFT-s-OFDM based PUSCH - The same MCS table is used for PDSCH and CP-OFDM based PUSCH - At least one new MCS table is introduced for URLLC Agreements: - The CQI table for URLLC with target BLER of 10^-5 is composed of 16 entries, including one entry of “out of range”. Agreements: - In CSI report, RRC parameter cqi-Table implicitly provides the BLER target for CQI reporting. - New 64QAM table implies BLER target =10^-5 only. - Existing 64QAM table and 256QAM table imply BLER target =10^-1 only. - From RAN1 perspective, RRC parameter bler-Target is not to be used by RAN1 specification in Rel-15. Thus, from RAN1 perspective, this parameter can be removed from 38.331 in Rel-15 - Spec update is necessary since the parameter already appears in RAN1 specs - Send an LS to RAN2 to inform the above agreements – R1-1807743 Agreements: - Lowest SE entry for the new CQI table is 30/1024 * 2 Agreements: - The CQI table for URLLC with target BLER of 10^-5 is: CQI index; modulation; code rate x 1024; efficiency. 0; out of range. 1; QPSK; 30; 0.0586. 2; QPSK; 50; 0.0977. 3; QPSK; 78; 0.1523. 4; QPSK; 120; 0.2344. 5; QPSK; 193; 0.3770. 6; QPSK; 308; 0.6016. 7; QPSK; 449; 0.8770. 8; QPSK; 602; 1.1758. 9; 16QAM; 378; 1.4766. 10; 16QAM; 490; 1.9141. 11; 16QAM; 616; 2.4063. 12; 64QAM; 466; 2.7305. 13; 64QAM; 567; 3.3223. 14; 64QAM; 666; 3.9023. 15; 64QAM; 772; 4.5234. Agreements: - For both CP-OFDM and DFT-s-OFDM, the new MCS table for URLLC has a size of 32 entries, and the MCS information field in DCI is 5-bit. Agreements: - For CP-OFDM, the 32-entry MCS table for URLLC includes at least 3 reserved entries for modulation order. - FFS if more reserved entries are needed - For DFT-s-OFDM, the 32-entry MCS table for URLLC includes at least 4 reserved entries for modulation order (pi/2-BPSK, QPSK, 16QAM, 64QAM). - FFS if more reserved entries are needed - Regarding pi/2-BPSK support for PUSCH, the same signalling and procedure are used for URLLC and eMBB. Agreements: - The lowest SE entry in the new MCS table is the same as the lowest SE entry of the CQI table for BLER target of 10^-5. Agreements: - For PDSCH and PUSCH with CP-OFDM, one new MCS table is introduced for URLLC: MCS Index; Modulation Order; Code rate R × 1024; Spectral efficiency. 0; 2; 30; 0.0586. 1; 2; 40; 0.0781. 2; 2; 50; 0.0977. 3; 2; 64; 0.1250. 4; 2; 78; 0.1523. 5; 2; 99; 0.1934. 6; 2; 120; 0.2344. 7; 2; 157; 0.3066. 8; 2; 193; 0.3770. 9; 2; 251; 0.4902. 10; 2; 308; 0.6016. 11; 2; 379; 0.7402. 12; 2; 449; 0.8770. 13; 2; 526; 1.0273. 14; 2; 602; 1.1758. 15; 4; 340; 1.3281. 16; 4; 378; 1.4766. 17; 4; 434; 1.6953. 18; 4; 490; 1.9141. 19; 4; 553; 2.1602. 20; 4; 616; 2.4063. 21; 6; 438; 2.5664. 22; 6; 466; 2.7305. 23; 6; 517; 3.0293. 24; 6; 567; 3.3223. 25; 6; 616; 3.6094. 26; 6; 666; 3.9023. 27; 6; 719; 4.2129. 28; 6; 772; 4.5234. 29; 2; reserved. 30; 4. 31; 6. Agreements: - For PUSCH with transform precoding, one new MCS table is introduced for URLLC: MCS Index IMCS; Modulation Order Qm; Code rate R × 1024; Spectral efficiency. 0; q; 60/q; 0.0586. 1; q; 80/q; 0.0781. 2; q; 100/q; 0.0977. 3; q; 128/q; 0.1250. 4; q; 156/q; 0.1523. 5; q; 198/q; 0.1934. 6; 2; 120; 0.2344. 7; 2; 157; 0.3066. 8; 2; 193; 0.3770. 9; 2; 251; 0.4902. 10; 2; 308; 0.6016. 11; 2; 379; 0.7402. 12; 2; 449; 0.8770. 13; 2; 526; 1.0273. 14; 2; 602; 1.1758. 15; 2; 679; 1.3262. 16; 4; 378; 1.4766. 17; 4; 434; 1.6953. 18; 4; 490; 1.9141. 19; 4; 553; 2.1602. 20; 4; 616; 2.4063. 21; 4; 658; 2.5703. 22; 4; 699; 2.7305. 23; 4; 772; 3.0156. 24; 6; 567; 3.3223. 25; 6; 616; 3.6094. 26; 6; 666; 3.9023. 27; 6; 772; 4.5234. 28; q; reserved. 29; 2. 30; 4. 31; 6. Agreements: - Regarding configuration of CQI reporting and MCS table: - The new MCS table can be used even when the new CQI table is NOT configured Agreements: - For URLLC, for grant-based transmissions, introduce one RRC parameter for configuring a new RNTI. - When the new RNTI is not configured, existing RRC parameter mcs-table is extended to select from 3 MCS tables (existing 64QAM MCS table, existing 256QAM MCS table, new 64QAM MCS table). - When mcs-table indicates the new 64QAM MCS table: - For DCI format 0_0/1_0 in CSS, existing 64QAM MCS table is used. - For DCI formats 0_0/1_0/0_1/1_1 in USS, new 64QAM MCS table is used. - Otherwise, follow existing behaviour. - Note: the configuration for DL and UL is separate - When the new RNTI (via RRC) is configured, RNTI scrambling of DCI CRC is used to choose MCS table: - If the DCI CRC is scrambled with the new RNTI, the new 64QAM MCS table is used; otherwise, follow existing behaviour. Intel: concerns about impact of introducing new RNTI on false alarm probability increase compared with eMBB vivo: share the same concern as above Agreements: For both initial transmission and re-transmissions for GF scheduling for URLLC, - For UL configured grant, the MCS table is configured by the existing parameter associated with the RRC configured grant configuration, which is extended to include the new 64QAM MCS table. - For DL SPS, RRC indicates whether or not the new 64QAM table is configured. The indication for the new MCS table for DL SPS is separate from the one for grant-based DL scheduling.

Email discussions after the meeting are listed below.

  - [93-NR-01] Email discussion on whether any response is needed to the RAN4 LS (R1-1807805) and, if needed, the response LS details until June 29, 2018

  - [93-NR-02-spec#] NR maintenance Editor to prepare CRs (capturing agreements and working assumptions): drafts to be distributed by Thursday May 31, to be agreed by Tuesday June 5.

  - [93-NR-03] Send an LS to RAN2 to inform the above agreements and removal of parameter k_0 for email approval 5/30

  - [93-NR-04] Email discussion to conclude Type 3 UE capabilities and to approve reply LS by 5/31

  - [93-NR-05-spec#] NR URLLC Editor to prepare CRs: drafts to be distributed by Thursday May 31, to be agreed by Tuesday June 5.

  - [93-NR-06] R1-1807874 Email approval till 5/31

Outgoing LS is listed below.

  - R1-1807831	Reply LS on RMSI CORESET configuration for a band with 15kHz SSB SCS and 10MHz minimum channel bandwidth

  - R1-1807763	LS on Simultaneous PDSCH Reception with Paging

  - R1-1807893	Reply LS on Initial Active BWP for Pattern 2 and Pattern 3

  - R1-1807892	Reply LS on the design of PRACH table

  - R1-1807764	LS on numberOfRA-PreamblesGroupA

  - R1-1807842	Reply LS on MSG3 size for NR

  - R1-1807767	LS on useServingCellTimingForSync definition

  - R1-1807909	LS on CSI-RS for L3 mobility

  - R1-1807727	LS on configuration of non-PMI port index

  - R1-1807725	Reply LS on beam failure recovery

  - R1-1807883	LS on BFR search space configuration

  - R1-1807720	Signalling Aspects Related to Pi/2 BPSK

  - R1-1807887	LS on NR CSI-RS

  - R1-1807726	LS on DMRS positions of PUSCH

  - R1-1807722	LS on adjacent TDM combinations of SRS for antenna switching and other UL physical channels

  - R1-1807882	LS on aperiodic SRS

  - R1-1807723	Reply LS on removal of CSI-RS-for-tracking from TCI-State

  - R1-1807676	Reply LS on subcarrier spacing for BWPs and TDD configurations

  - R1-1807903	LS on the size of DCI format 2-1

  - R1-1807873	LS on bandwidth configuration for initial BWP

  - R1-1807910	LS on signalling of transmitter-side DC location

  - R1-1807907	Reply to LS on Defining reserved resources in NR-SIB1

  - R1-1807896	LS on updated NR UE features

## ---