3GPP TSG RAN meeting #81								 RP-181738

Gold Coast, Australia, September 10-13, 2018

## Status Report to TSG

Agenda item:			10.2.1

| WI / SI Name | Shortened TTI and processing time for LTE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| included in this status report | Core part: | Yes | Perf. part: | Yes |  | Testing part: | No |
| Acronym | LTE_sTTIandPT |  |  |  |  |  |  |
| Unique ID | 720091 |  |  |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) |  |  |  |  |  |  |  |
| Target Completion Date | Core part: Sept 18 |  |  |  | Performance part: Dec 18 |  |  |
| Is the WI/SI complete? | Core part: Yes |  |  |  | Performance Part: No |  |  |

Source:

| Leading WG |  | TSG RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Florent Munier |
|  | Company | Ericsson |
|  | Email | florent.munier@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.2.1	Agreements

Agreement

UE capability on the maximum number of CSI processes to be updated is defined separately for

- {slot,slot}

- {subslot,slot}

- {subslot,subslot} processimg timeline set 1

- {subslot,subslot} processimg timeline set 2

Note: This replaces the existing UE capability on the maximum number of CSI processes to be updated for sTTI. RAN2 should be informed about this.

Agreement:

The sPDCCH rate matching for dynamic retransmissions of SPS sPDSCH applies the same mode as for scheduled sPDSCH scrambled with C-RNTI.

Agreement

The sRBG size and starting point granularity for 2/3os and 7os sPUSCH is for 1.4 MHz and 3 MHz as per table below (the same as legacy).

|  | 1.4 MHz | 3 MHz |
| --- | --- | --- |
| sRBG size | 1 | 1 |
| Starting point granularity | 1 | 1 |

Agreement:

For PUSCH without UL-SCH, define a UE capability per UL sTTI length whether to support BPRE derivation based on the actual derived O_CQI. Define an RRC parameter uplinkPower-CSIPayload to configure a UE to derive BPRE based on either the actual value of O_CQI or the largest value of O_CQI across all RI values. In case the UE does not support the capability, the UE will derive BPRE based on the largest value of O_CQI across all RI values.

Agreement:

In case of sPUSCH in UpPTS, the following TBS scaling factors apply depending on the number of symbols, X:

- X ≤ 3: 0.125

- X > 3: 0.375

Agreement:

Define a pseudo HARQ-ID to subframe mapping for synchronous HARQ (HARQ process pool sharing) for improved asynchronous and synchronous HARQ coexistence.

#### 2.2.2	Remaining Open issues

## 2.2	RAN2

#### 2.1.1	Agreements

Correction of HART RTT Timers (CR R2-1813111on 36.321)

Enhancement for reduction of UE capability signaling for sTTI was approved:

R2-1811849    Rel-15 LTE UE capability size reduction methods   Intel Corporation           discussion

-     Nokia thinks that we can agree for sTTI for now.  Intel thinks that we can maybe use for EN-DC

=>  From rel-15, the UE reports the full set of sTTI/sPT capabilities only when the eNB explicitly requests in the UECapabilityEnquiry.

=>  Noted

R2-1811850    CR for LTE UE capability size reduction methods   Intel Corporation           CR        Rel-15   36.331   15.2.2   3507     -           C          TEI15

=>  The CR will be merged in the main sTTI CR

R2-1811851    CR for LTE UE capability size reduction methods   Intel Corporation           CR        Rel-15   36.306   15.1.0   1630     -           C          TEI15

=>  The CR will be merged in the main sTTI CR

#### 2.3.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

BS RF performance part

Agreed with the CR for BS RF conformance test (TS36.141)

#### 2.4.2	Remaining Open issues

BS demodulation part

- Alignment of PUSCH demodulation performance

- Alignment of SPUCCH demodulation performance

- CR for TS36.104 for BS demodulation requirements (PUSCH and SPUCCH)

- CR for TS36.141 for BS demodulation requirements (PUSCH and SPUCCH

UE demodulation part

- Alignment of Slot/Subslot-PDSCH demodulation performance

- Alignment of SPDCCH demodulation performance

- Set the test points for CQI reporting test for CRS based and CSI-RS based CQI reporting

- Set the requirements for CQI reporting test for CRS based and CSI-RS based CQI reporting

- CR for TS 36.101 for UE demodulation requirements

- CR for TS 36.101 for CQI reporting test requirements

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

#### 2.5.4	Estimated Level of Completion

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT. Based on the joint RAN-SA#80 session the items relevant for this section are shown in http://www.3gpp.org/ftp/tsg_sa/TSG_SA/TSGS_80/Docs/SP-180561.zip and summarized below:

| Work Area | SA WIDs | Rapporteurs | RAN WIDs | Rapporteurs |
| --- | --- | --- | --- | --- |
| URLLC for 5G | FS_5G_URLLC FS_Vertical_LAN cyberCAV | SA2: Hui.ni@huawei.com SA2: Devaki.chandramouli@nokia.com SA1: joachim.walewski@siemens.com | FS_NR_L1enh_URLLC FS_NR_unlic FS_NR_IIOT | RAN1: chengyan.cheng@huawei.com RAN1: jingsun@qti.qualcomm.com RAN2: dawid.koziol@nokia.com |
| V2X for 5G | FS_eV2XARC | SA2: laeyoung.kim@lge.com SA1: sungduck.chun@lge.com | FS_NR_V2X | RAN1: Hanbyul.seo@lge.com Matthew.webb@huawei.com |
| Positioning | FS_eLCS 5G_HYPOS | SA2: aiming@catt.cn SA1: lionel.ries@esa.int | FS_NR_POS | RAN1:alexey.khoryaev@INTEL.COM asbjorn.grovlen@ERICSSON.COM |
| UE Capabilities | FS_RACS | SA2: harisz@qti.qualcomm.com | FS_RACS_RAN | RAN2: alex.hsu@mediatek.com |
| 5G Satellite Aspects | FS_5GSAT_ARCH 5GSAT | SA2: cyril.michel@thalesaleniaspace.com SA1: cyril.michel@thalesaleniaspace.com | FS_NR_NTN_solutions | RAN3: nicolas.chuberre@thalesaleniaspace.com |

RAN and SA rapporteurs are requested to fill in these clauses jointly.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#88

R4-1810093		CR to TS 36.141 - sTTI introduction	Ericsson

R4-1811571		CR to TS 36.141 - sTTI introduction	Ericsson

R4-1809947		Simulation results for sPUCCH Format4	Samsung

R4-1811117		Simulation results for SPUCCH format 4	Nokia, Nokia Shanghai Bell

R4-1811161		Simulation results for sTTI BS SPUSCH demodulation requirements	Huawei

R4-1811162		CR: SPUSCH conformance test for 36.141	Huawei

R4-1811163		Simulation results for sTTI BS SPUCCH demodulation requirements	Huawei

R4-1811164		CR: SPUCCH conformance test for 36.141	Huawei

R4-1811237		Simulation results for sPUCCH	Ericsson

R4-1811238		Performance requirements for SPUCCH	Ericsson

R4-1811239		Performance requirements for SPUSCH	Ericsson

R4-1811240		results summary for sPUCCH and sPUSCH	Ericsson

R4-1811354		Simulation results for sPUCCH Format4	Samsung

R4-1811379		CR: SPUSCH conformance test for 36.141	Huawei

R4-1811380		CR: SPUCCH conformance test for 36.141	Huawei

R4-1809775		Simulation result  for FDD sPDSCH demodulation	Qualcomm Incorporated

R4-1809776		Simulation result for CRS-based FDD sPDCCH demodulation	Qualcomm Incorporated

R4-1810148		Simulation results of PDSCH for sTTI UE demodulation requirements	Ericsson

R4-1810149		Summary of simulation results for sTTI UE demodulation requirements	Ericsson

R4-1810152		Introduction of SPDCCH demodulation requirements	Ericsson

R4-1810153		Introduction of Slot/Subslot-PDSCH demodulation requirements	Ericsson

R4-1811165		Simulation results for sTTI PDSCH demodulation requirements	Huawei, HiSilicon

R4-1811166		Simulation results for sTTI UE SPDCCH demodulation requirements	Huawei, HiSilicon

R4-1811381		Introduction of Slot/Subslot-PDSCH demodulation requirements	Ericsson

R4-1811382		Introduction of SPDCCH demodulation requirements	Ericsson

R4-1809777		Discussion on sTTI CSI reporting test	Qualcomm Incorporated

R4-1810150		CQI reporting tests for CRS-based PDSCH	Ericsson

R4-1810151		CQI reporting tests for DMRS-based PDSCH	Ericsson

R4-1810154		Introduction of CQI tests for sTTI	Ericsson

R4-1811167		Discuss on sTTI UE CSI test	Huawei, HiSilicon

R4-1811383		Introduction of CQI tests for sTTI	Ericsson

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template