3GPP TSG RAN meeting #87e										RP-200217

Electronic Meeting, March 16-19, 2020

## Status Report to TSG

Agenda item:			9.3.7

| WI / SI Name | Status report of WI: NR positioning support |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_POS |  |  |  |  |
| Unique ID | 830077 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-192581 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 06/2020 (updated from 03/2020) | Performance part: 12/2020 (updated from 09/2020) | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 90% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  |  |
| --- | --- | --- |
| Rapporteur | Name | Alexey Khoryaev |
|  | Company | Intel Corporation |
|  | Email | alexey.khoryaev@intel.com |
| Rapporteur | Name | Florent Munier |
|  | Company | Ericsson |
|  | Email | florent.munier@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

There is a need for additional TUs allocated for RAN3 and RAN4 WGs that require extension of WI for completion of core part till June 2020.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 started maintenance work on NR Positioning. The following e-mail discussions were organized to address remaining open issues for all agenda items based on contributions [1]-[64]:

#### DL Reference Signals for NR Positioning

This sub-section provides outcome of RAN1 WG e-mail discussions, that were organized to address raised issues on NR DL reference signals for NR Positioning:

- [100e-NR-Pos-DL-PRS-01] E-mail discussion/approval to resolve issue #1 in R1-2001176

- The summary of e-mail discussion was uploaded in R1-2001233. The text proposal for TS 38.211 were endorsed based on TPs in R1-2001234, i.e. TP#1 and TP#2(Option B) from R1-2001234 were agreed.

- [100e-NR-Pos-DL-PRS-02] E-mail discussion/approval to resolve issue #2 in R1-2001176

- The summary of e-mail discussion was uploaded in R1-2001235. RAN1 made additional agreements (captures in Conclusion Section #4 of R1-2001235) to clarify assumptions of the previous RAN1 WG agreement on UE DL PRS processing capabilities.

#### UL Reference Signals for NR Positioning

This sub-section provides outcome of RAN1 WG e-mail discussions, that were organized to address raised issues on NR UL reference signals for NR Positioning:

- [100e-NR-Pos-ULRS-01] E-mail discussion/approval to resolve the issue of simultaneous SRS transmission (proposal: UE is not expected to transmit SRS for positioning and SRS for other usages on the same symbol)

- Outcome of this e-mail discussion is the TP for TS38.214 in Section 3 of R1-2001286 that was agreed by RAN1

- [100e-NR-Pos-ULRS-02] E-mail discussion/approval to resolve SRS priorities/collision with PUSCH

- Outcome of this e-mail discussion is captured in R1-2001287. The TP in Section 3.1 of R1-2001287 was agreed by RAN1

- [100e-NR-Pos-ULRS-03] E-mail discussion/approval to resolve the issue of periodic SRS for positioning configuration (proposal: Add the parameter slot offset for an SRS resource when the higher layer parameter resource Type in SRS-Resource is set to 'aperiodic'.)

- Outcome of this e-mail discussion is captured in R1-2001288.

  - RAN1 agreed LS (R1-2001386) where RAN1 respectfully asks RAN2 to take into account the addition of a slot offset parameter in the aperiodic SRS resource configuration  for the [SRS-for-positioning] SRS and the removal of slot offset configuration in SRS resource set for the  case of the [SRS-for-positioning] SRS.

- [100e-NR-Pos-ULRS-04] E-mail discussion/approval to resolve the issue of spatial relation information for SRS for positioning (default, MAC CE update)

- Outcome of this e-mail discussion is captured in R1-2001289. RAN1 made the following agreements:

  - The feature of default spatial relation info is not applicable to SRS for positioning

  - The feature of using one MAC CE to activate one same spatial relation info for multiple SRS resources in multiple CCs is not applicable to SRS for positioning.

  - The text proposal of Section 3.1 in R1-2001289 is endorsed.

#### UE and gNB measurements for NR Positioning

This sub-section provides outcome of RAN1 WG e-mail discussions, that were organized to address raised issues on UE and gNB measurements for NR Positioning:

- [100e-NR-Pos-Measurements-01] Email approval of a TP on essential corrections to UE/gNB measurements (TS 38.215, section 5.1.6.5) (reference: R1-2001156, issues 2.1/2.2/2.5/2.6/2.7/2.10).

- The summary of e-mail discussion was uploaded in R1-2001243. The agreed by RAN1 text proposals to update section 5.1.6.5 of the TS 38.215 were endorsed in R1-2001244.

#### Physical-layer procedures to support UE/gNB measurements

This sub-section provides outcome of RAN1 WG e-mail discussions, that were organized to address raised issues on On physical layer procedures for NR Positioning:

- [100e-NR-Pos-PHY procedures-01] Email approval of a TP on essential corrections to PRS reception procedure (TS 38.214, sub-clause 5.1.6.5) (reference: R1-2001168, issues #1, 2, 4-11; R1-2001176, issue #3; R1-2001156, part of the issues 2.1 and issue 2.10 related to Section 5.1.6.5, 38.214)

- The summary of e-mail discussion was uploaded in R1-2001364 where multiple text proposals to correct section “5.1.6.5 PRS reception procedure” in the TS 38.214 were discussed. Finally, proposal provided in the summary section was agreed by RAN1.

- [100e-NR-Pos-PHY procedures-02]: Email discussion/approval on UE procedure for determining slot format (TS 38.213, TS 38.214) (reference: R1-2001168, issue #12)

- The summary of e-mail discussion was uploaded in R1-2001365, where several text proposals aiming to clarify UE DL PRS reception behavior in case of dynamic slot format indication. Finally, proposal provided in the summary section was agreed by RAN1.

- [100e-NR-Pos-PHY procedures-03]: Email discussion/approval of a TP on essential corrections to UL power control for SRS-for-positioning (TS 38.213, sub-clause 7.3.1) (reference: R1-2001168, issues#15/16/17) by 2/28 – Sven (Qualcomm)

- The summary of e-mail discussion was uploaded in R1-2001366, where several text proposals were agreed clarifying UE behavior for transmission of SRS for positioning. Finally, proposal provided in the summary section was agreed by RAN1.

#### 2.1.2	Remaining Open issues

RAN1 is to continue maintenance of NR Positioning work aiming to resolve technical issues if any are identified at the subsequent meetings.

## 2.2	RAN2

RAN2 continued the work on NR Positioning. The following e-mail discussions were organized to address remaining open issues for all agenda items based on contributions [65]-[142]:

#### 2.2.1	Agreements

RAN2 agreed LTE Stage 2 CR R2-2002111 and RRC CR R2-2001216 on PPP RTK;

RAN2 agreed NR stage 2 (TS38.305) CR R2-2002241, stage 2 (TS38.300) CR R2-2001960, RRC CR R2-2002247, LPP CR R2-2002243 and MAC CR R2-2002246

For NR RAT dependent, RAN2 agreed:

-	Support semi-persistent SRS for positioning.

-	There is no support of Rel-16 NR RAT-dependent positioning methods with LTE signals.

-	SUPL will be marked as not supported (can be revisited if OMA take some action).

-	Beam level measurement results as in Rel-15 are added in NR ECID method.

-	The ProvideAssistanceData can be upgraded as below.

	The required physical resources are put in:

	nr-DL-PRS-ProvideAssistanceData-r16 (nr-DL-PRS-AssistanceDataList-r16,

nr-SSB-Config-r16)

	The selected physical resources index for some positioning method are put in:

	nr-Multi-RTT-ProvideAssistanceData-r16

	nr-DL-AoD-ProvideAssistanceData-r16

	nr-DL-TDOA-ProvideAssistanceData-r16

-	Non-critical extension is used in message body to capture Rel-16 NR dependent positioning methods, and prefix “nr” is used to distinguish LTE and NR. The EN is removed;

-	Common NR positioning IEs are captured in section 6 as new sub-clause. NR-PhysCellId is moved to section 6.4.1.

-	Do not group report configuration, indicate requested measurement per positioning method.

-	UL SRS/DL PRS RSRP measurements is optional for multi-RTT.

-	For RAT dependent positioning method, introduce following RAN2 related NR positioning capabilities as a baseline:

-		DL-TDOA, DL-AoD: positioning mode (UE based or UE assisted);

-		DL-TDOA, DL-AoD, NR E-CID, Multi-RTT: Support of periodical Reporting;

-		NR ECID: Support of triggered Reporting;

-	Add support for additional path reporting to LPP for timing-based measurements, similar to LTE.

-	Recommend RAN3 to support timing-based additional path reporting in NRPPa.

-	The name DL-AoD is kept.

-	Confirm (same as current running CR) when a UE is configured to report  multiple DL PRS RSTD, PRS RSRP, RxTX measurements with each measurement between a different pair of DL PRS resources or DL PRS resource sets, and those multiple measurements being performed on the same pair of TRPs, the UE reports one full measurement results and additional delta measurement(s).

-	Confirm that same as stage 2/3 CR and follow RAN3 preference, AoD is calculated in LMF.

-	PRS resource set ID and PRS resource ID should be indicated within dl PRS QCL.

-	Harmonise posSIB numbering across CRs in R2-2001951

-	RAN2 confirm for posSIBs, UE should make an on-demand request for the SIB based on the feature requirement and not based on SIB update mechanism.

On broadcast AD, RAN2 agreed:

-	For positioning On-demand SI request message sent by the UE in RRC_CONNECTED is per SIB.

-	For positioning, On-demand SI request message sent by the UE in RRC IDLE/INACTIVE is per SI.

-	Subscription based mechanism is not pursued in Rel-16.

-	Use SIB1 for scheduling as a baseline in the running CR.

On UE based positioning, RAN2 agreed:

-	Provide the LCS-to-GCS translation parameter in the assistance data as part of the spatial direction information of the DL-PRS Resources (IE NR-DL-PRS-BeamInfo).

-	Do not include the RTD Drift Rate in the assistance data in Rel-16.

-	RTD is not provided per DL-PRS Resource in the assistance data in Rel-16.

-	For the baseline CR, the angular information is provided using a single field with 0.1-degrees resolution.  FFS if we migrate in the future to a more compact representation.

Following LSs have been agreed:

-	R2-2992244 LS on uplink additional path reporting;

-	R2-2002245 LS on positioning measurement range and granularity

There are two agreed email discussions to be organized after the meeting:

[109e#xx][NR/Pos] Non-periodic SRS for positioning

Discuss the proposals from R2-2001935.

Intended outcome: Summary for next meeting

Deadline:  Long

[109e#xx][NR/Pos] Details of spatial relation for positioning

Continue the discussion from R2-2001936 and resolve open issues.

Intended outcome: Summary for next meeting

Deadline:  Long

#### 2.2.2	Remaining Open issues

RAN2 has completed objectives of the WI, although confirmation is needed on spatial relationship. RAN2 is to continue maintenance of NR Positioning work aiming to resolve technical issues if any are identified at the subsequent meetings.

## 2.3	RAN3

Some progress was made on NRPPa and broadcast assistance data delivery within the limitations of the e-meeting.

#### 2.3.1	Agreements

RAN3 endorsed the following BL CRs:  R3-200060, R3-200061, R3-200062, and R3-200777 to TS 38.305, TS 38.473, TS 38.470, and TS 38.455, respectively. Furthermore, a clarification TP resolving an ambiguity in stage-2 (TS 38.305) figure depicting positioning architecture was agreed in R3-201373.

A TP for TS 38.455 was agreed in R3-201290, which adds most of the NRPPa messages required to support new positioning methods and their respective information elements, specifically:

- Positioning information request/response/failure/update

- TRP information request/response/failure

- Measurement request/response/failure/report/abort

Additionally, a TP with further details of broadcast assistance data delivery over F1 was agreed in R3-201021.

#### 2.3.2	Remaining Open issues

Due to limitations of the e-meeting, the following email discussions could not conclude and will have to continue in the next meeting:

- Exchange of DL PRS configuration over Xn

- Remaining aspects of Broadcast Assistance Data Delivery

- Positioning Support in split gNB Architecture

Furthermore, additional work remains to be done on finalizing the NRPPa CR and stage-2, in particular to align between RAN3 and RAN3.

## 2.4	RAN4

#### 2.4.1	Agreements

The summary of the RAN4 WG e-mail discussion on RRM relates aspects for NR positioning was provided in two parts submitted in R4-2002310 (Part 1 – relate to UE) and R4-2002311 (Part 2 – related to gNB) respectively. These documents were noted and served as a basis for further RAN4 discussion on RRM related aspects.

RRM Part 1

The following agreements were made by RAN4 WG on selected RRM related topics (R4-2002310 - Part 1) based on the first round of e-mail discussion:

- Topic# 1: PRS-RSTD Side conditions

- 1-1: Reference cell side condition in FR1

- Agreements:

  - Side condition for FR1 PRS-RTD reference cell to be PRS Es/Iot = -6 dB.

- Topic# 3: Inter-frequency vs. Intra-frequency PRS-RSTD

- 3-1: Whether definition of intra-frequency and inter-frequency RSTD measurement is needed

- Agreements:

  - RAN4 to define intra-frequency and inter-frequency RSTD measurements and the corresponding requirements.

- Topic #10: PRS-RSRP report mapping tables

- Agreements:

  - RAN4 to use absolute SS-RSRP report mapping table in TS 38.133 for absolute PRS-RSRP report mapping table.

- Topic #10: Non-DRX requirements for PRS-RSRP

- Non-DRX only requirements

- Agreements:

  - PRS-RSRP measurement requirements will be defined for only non-DRX regardless of whether and which DRX configuration is configured for the UE.

- Topic #19: Non-DRX requirements for Rx-Tx time difference

- Non-DRX only requirements

- Agreements:

  - Rx-Tx time difference measurement requirements will be defined for only non-DRX regardless of whether and which DRX configuration is configured for the UE.

- Topic #25: E-CID positioning method

- Agreements:

  - R15 core measurement requirements, measurement report mapping, and measurement accuracy requirements for SS-RSRP, SS-RSRQ, CSI-RS-RSRP and CSI-RS-RSRQ are applicable to E-CID positioning technique without any modification.

  - All existing measurement gap configurations in R15 applicable to RRM measurements to be applicable for E-CID measurements.

After the 2nd round of e-mail discussion the following documents were approved:

- R4-2002328 - WF on NR Positioning UE requirements (RAN4#94-e email thread #56).

- R4-2002288 – CR 0569 to TS 38.133 on NR E-CID measurement requirements

RAN4 WG also discussed and agreed to update link level simulation assumptions for RSTD and RSRP measurements. The details were endorsed in R4-2002287.

RRM Part 2

The following agreements were made by RAN4 WG on selected RRM related topics (R4-2002311 - Part 2) after the 1st round email discussion:

- Topic #2: gNB measurement report mapping (AI 8.8.2)

- Issue 2-1: Report mapping for SRS-RSRP measurement

- Agreements:

  - SRS-RSRP reporting granularity = 1dB. Maximum and minimum values are FFS until next meeting.

  - SRS-RSRP minimum value:

  - Option 1: SRS-RSRP min value = SS-RSRP min value.

  - Option 2: SRS-RSRP min value  SS-RSRP min value.

  - SRS-RSRP maximum value:

  - Option 1: SRS-RSRP max value = SS-RSRP max value.

  - Option 2: SRS-RSRP max value  SS-RSRP max value.

- Issue 2-4: Report mapping for AoA and ZoA

- Agreements:

  - AoA and ZoA report mappings:

  - AoA: from -180 to +180 degrees with granularity = 0.1 degree.

  - ZoA: from 0 to +180 degrees with granularity = 0.1 degree.

After the 2nd round of RAN4 WG email discussion including discussions on NR Positioning requirements for UE and gNB, the following documents were approved:

- R4-2002275 - WF on NR Positioning gNB measurement requirements and report mapping. Capture conclusions and candidate options on Topic #1 and Topic #2 in WF.

- R4-2002276 - WF on NR Positioning measurement impact on RRM requirements

- R4-2002280 - LS on gNB measurements report mapping for NR Positioning.

- Captures RAN4 #94e conclusions on gNB measurements report mapping

#### 2.4.2	Remaining Open issues

Specify RRM Core and Performance requirements

- UE and gNB measurements report mapping

- UE measurements requirements

- gNB measurements requirements

- NR Positioning measurement impact on RRM requirements

- Corresponding test cases

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2000190	Maintenance of DL PRS for NR positioning	Huawei, HiSilicon

- R1-2000243	Maintenance on DL reference signals for NR positioning	ZTE

- R1-2000287	Discussion on remaining issues on DL RS for NR positioning	vivo

- R1-2000341	Discussion on remaining issues on DL RS for NR positioning	vivo

- R1-2000462	Remaining Issues on DL Positioning Reference Signal	OPPO

- R1-2000539	Remaining issues on DL PRS for NR Positioning	CATT

- R1-2000575	TP on DL Reference Signals	Futurewei

- R1-2000639	DL reference signals for NR Positioning	Samsung

- R1-2000686	Remaining issues of DL reference signals for NR Positioning	LG Electronics

- R1-2000760	Corrections on DL PRS resource set parameters	CMCC

- R1-2000804	Correction of DL PRS sequence generation	Nokia, Nokia Shanghai Bell

- R1-2001002	Maintenance of rel16 DL reference signals for NR positioning	Ericsson

- R1-2001142	Feature lead summary for AI 7.2.8.1 – DL Reference Signals for NR Positioning	Intel Corporation

- R1-2001176	Feature lead summary#2 for AI 7.2.8.1 – DL Reference Signals for NR Positioning	Intel Corporation

- R1-2001233	Output of email thread [100e-NR-Pos-DL-PRS-01]	Intel Corporation

- R1-2001234	Text proposal to TS 38.211 based on outcome of [100e-NR-Pos-DL-PRS-01]	Intel Corporation

- R1-2001235	Output of email thread [100e-NR-Pos-DL-PRS-02]	Intel Corporation

- R1-2000191	Maintenance of SRS for NR positioning	Huawei, HiSilicon

- R1-2000288	Discussion on remaining issues on UL RS for NR positioning	vivo

- R1-2000342	Discussion on remaining issues on UL RS for NR positioning	vivo

- R1-2000463	Remaining Issues on UL Positioning Reference Signal	OPPO

- R1-2000540	Remaining issues on UL SRS for NR Positioning	CATT

- R1-2000576	TP on UL Reference Signals	Futurewei

- R1-2000640	UL reference signals for NR Positioning	Samsung

- R1-2000687	Remaining issues of UL reference signals for NR Positioning	LG Electronics

- R1-2000761	Discussions on UL SRS for positioning collision rules	CMCC

- R1-2001003	Maintenance of rel16 UL reference signals for NR positioning	Ericsson

- R1-2001128	FL Summary for UL Reference Signals for NR Positioning	Ericsson

- R1-2001194	Revised FL Summary for UL Reference Signals for NR Positioning	Ericsson

- R1-2001286	Outcome of email thread [100e-NR-Pos-ULRS-01]	Ericsson

- R1-2001287	Outcome of email thread [100e-NR-Pos-ULRS-02]	Ericsson

- R1-2001288	Outcome of email thread [100e-NR-Pos-ULRS-03]	Ericsson

- R1-2001289	Outcome of email thread [100e-NR-Pos-ULRS-04]	Ericsson

- R1-2000192	Maintenance of NR positioning measurements	Huawei, HiSilicon

- R1-2000289	Discussion on remaining issues on UE and gNB measurements for NR positioning	vivo

- R1-2000343	Discussion on remaining issues on UE and gNB measurements for NR positioning	vivo

- R1-2000464	Remaining Issues on Measurements for NR Positioning	OPPO

- R1-2000541	Remaining issues on NR Positioning Measurements	CATT

- R1-2000577	TP on Measurements	Futurewei

- R1-2000641	UE and gNB measurements for NR Positioning	Samsung

- R1-2000688	Remaining issues of UE and gNB measurements for NR Positioning	LG Electronics

- R1-2000805	Maintenance of Measurement Definitions	Nokia, Nokia Shanghai Bell

- R1-2000932	FL Summary of Remaining issues on NR Positioning Measurements	CATT

- R1-2000978	Maintenance on UE and gNB measurements for NR Positioning	Qualcomm Incorporated

- R1-2001004	Maintenance of rel16 Physical-layer procedures to support UE - gNB measurements	Ericsson

- R1-2001156	FL Summary#2 of Remaining issues on NR Positioning Measurements	CATT

- R1-2001243	Output of email thread [100e-NR-Pos-Measurements-01]	CATT

- R1-2001244	Text proposal to TS 38.215 based on outcome of [100e-NR-Pos-Measurements-01]	CATT

- R1-2000193	Maintenance of physical layer procedures to support positioning measurements	Huawei, HiSilicon

- R1-2000244	Maintenance on physical layer procedures for NR positioning	ZTE

- R1-2000290	Discussion on remaining issues on physical-layer procedures for NR positioning	vivo

- R1-2000344	Discussion on remaining issues on physical-layer procedures for NR positioning	vivo

- R1-2000465	Remaining Issues on Physical Layer Procedures for NR Positioning	OPPO

- R1-2000578	TP on Measurement Procedures	Futurewei

- R1-2000642	Physical-layer procedures to support UE/gNB measurements	Samsung

- R1-2000689	Remaining issues of Physical-layer procedures to support UE/gNB measurements	LG Electronics

- R1-2000762	Discussions on TA adjustment on timing measurements	CMCC

- R1-2001005	Maintenance of rel16 UE and gNB measurements for NR Positioning	Ericsson

- R1-2001077	Remaining opens on NR Positioning physical layer procedures	Intel Corporation

- R1-2001143	Summary of remaining issues on PHY procedures for NR positioning measurements	Qualcomm

- R1-2001168	Summary#2 of remaining issues on PHY procedures for NR positioning measurements	Qualcomm

- R1-2001364	Summary of Email Discussion [100e-NR-Pos-PHY procedures-01]: PRS reception procedure	Qualcomm Incorporated

- R1-2001365	Summary of Email Discussion [100e-NR-Pos-PHY procedures-02]: UE procedure for determining slot format	Qualcomm Incorporated

- R1-2001366	Summary of Email Discussion [100e-NR-Pos-PHY procedures-03]: UE procedures for SRS for positioning	Qualcomm Incorporated

- R2-2000010	LS on agreements related to NR Positioning (R1-1913522; contact: Nokia)	RAN1

- R2-2001933	Draft reply LS on agreements related to NR Positioning	Nokia

- R2-2000033	LS on DL-AOD procedure (R3-197794; contact: Huawei)	RAN1

- R2-2000038	Response LS on Reference Point for Timing Related Measurements in FR2 (R4-1915801; contact: CATT, 2x3Ericsson)	RAN4

- R2-2001080	Stage 2 CR for the introduction of SSR positioning support into LTE 	Intel Corporation, ESA

- R2-2002111	Stage 2 CR for the introduction of SSR positioning support into LTE 	Intel Corporation, ESA

- R2-2000473	Running stage 2 CR on NR positioning ([108#84][NR Pos])	Intel Corporation, ESA

- R2-2001934	Running stage 2 CR on NR positioning ([108#84][NR Pos])	Intel Corporation, ESA

- R2-2002241	Running stage 2 CR on NR positioning ([108#84][NR Pos])	Intel Corporation, ESA

- R2-2001931	Summary on the Stage-2 Aspects of R16 Positioning	Huawei, HiSilicon, Intel Corporation

- R2-2001935	Summary of offline discussion #611 on non-periodic SRS for positioning	Huawei, HiSilicon

- R2-2002330	Running CR for the introduction of NR positioning on MAC spec	Huawei, HiSilicon

- R2-2002246	Running CR for the introduction of NR positioning on MAC spec	Huawei, HiSilicon

- R2-2001936	Summary of offline discussion #612 on spatial relationship for positioning Huawei, HiSilicon

- R2-2000513	Discussion on non-periodic SRS resource for positioning	ZTE Corporation

- R2-2000967	Remaining issues on SRS configuration	Huawei, HiSilicon

- R2-2001214	Semi-persistent and aperiodic SRS-for-positioning	Qualcomm Incorporated

- R2-2001237	Spatial Relations and MAC CE	Ericsson

- R2-2001216	Introduction of PPP-RTK (SSR) 	Qualcomm Incorporated

- R2-2001937	Introduction of PPP-RTK (SSR) 	Qualcomm Incorporated	CR

- R2-2001333	 Running CR for the introduction of NR positioning 	Ericsson

- R2-2002112	 Running CR for the introduction of NR positioning 	Ericsson

- R2-2001938	 Running CR for the introduction of NR positioning 	Ericsson

- R2-2002242	 Running CR for the introduction of NR positioning 	Ericsson

- R2-2001255	Running CR on 38.331 for on-demand SI procedure in RRC_CONNECTED for Positioning Ericsson

- R2-2002052	Summary for Positioning RRC running CR	Ericsson

- R2-2000243	UL SRS UE capabilities captured by RRC in TS 38.331.	CATT

- R2-2001228	Introduction of NR positioning 	Qualcomm Incorporated

- R2-2000968	Discussion on GAP request for RSTD measurement	Huawei, HiSilicon

- R2-2001168	Introduction of Rel-16 NR positioning	Intel Corporation

- R2-2000474	LPP CR Capturing RAN1 parameters for positioning ([108#85][NR Pos])	Intel Corporation

- R2-2001941	LPP CR Capturing RAN1 parameters for positioning ([108#85][NR Pos])	Intel Corporation

- R2-2001959	Report of [AT109e][615][POS] Update RAN1 parameters part of 37.355 CR (Intel)	Intel Corporation

- R2-2001942	Introduction of Rel-16 NR positioning	Intel Corporation

- R2-2002243	Introduction of Rel-16 NR positioning	Intel Corporation

- R2-2001230	Introduction of PPP-RTK (SSR) 	Qualcomm Incorporated

- R2-2001279	Summary of [108#86][NR/Pos] Single positioning method approach in LPP	Ericsson

- R2-2002115	Summary of [108#86][NR/Pos] Single positioning method approach in LPP	Ericsson

- R2-2001943	Summary of [AT109e][617][POS] Single positioning method	Ericsson

- R2-2000475	UE capability on positioning ([108#85][NR Pos])	Intel Corporation

- R2-2001944	Summary of [AT109e][618][POS] Supported frequency/band list	Intel

- R2-2001243	Summary of [108#87][NR/Rel-16] Additional path reporting	Ericsson

- R2-2001659	Summary of [108#87][NR/Rel-16] Additional path reporting	Ericsson

- R2-2001945	Draft LS on uplink additional path reporting	Ericsson

- R2-2001946	Text proposal on additional path reporting	Ericsson

- R2-2001173	Summary on LPP for aganda 6.8.2.3	Intel Corporation

- R2-2001948	Draft LS on positioning measurement range and granularity	Intel

- R2-2002245 LS on positioning measurement range and granularity	Intel

- R2-2001947	Summary of [AT109e][622][POS] Uplink capability for positioning	Huawei	discussion

- R2-2001951	Report of [AT109e][601][POS] Harmonise posSIB numbering across CRs (Intel)	Intel Corporation

- R2-2002331	Way forward on the AoD calculation	Huawei, HiSilicon

- R2-2000241	Design of ProvideAssistantData for RAT-Dependent positioning methods	CATT

- R2-2000289	Reduce overhead of RSTD measurement report

- R2-2000290	Remaining issues on support of NR RAT-dependent positioning

- R2-2000476	Open issues in LPP CR	Intel Corporation

- R2-2000966	Remaining issues on DL positioning procedure	Huawei, HiSilicon, MediaTek

- R2-2000969	Discussion on DL-AoD positioning procedure	Huawei, HiSilicon

- R2-2000970	Discussion on SRS capability transfer	Huawei, HiSilicon

- R2-2000991	SSB Configuration for UL-PRS and DL-PRS	LG Electronics Inc.

- R2-2001232	posSIBs for NR positioning 	Qualcomm Incorporated

- R2-2001278	Single positioning method approach in LPP	Ericsson

- R2-2001353	Strongest first path indication with RSTD and UE RxTx measurements 	Ericsson

- R2-2001241	Summary of [108#88][NR/Rel-16] Remaining issues on broadcast assistance data	Ericsson

- R2-2001950	Summary of [AT109e][621][POS] Open issues on broadcast assistance data	CATT

- R2-2002053	Summary for Broadcast of Assistance Data	Ericsson

- R2-2000242	Further Considerations on Broadcast Assistance Data	CATT

- R2-2000971	Discussion on on-demand SI in connected for NR positioning	Huawei, HiSilicon

- R2-2001236	Segmentation info in gNB	Ericsson

- R2-2001239	Overhead in current structure	Ericsson

- R2-2001268	Restructuring of LPP Broadcast solution to remove overheads	Ericsson

- R2-2001269	Restructuring of RRC Broadcast solution to remove overheads	Ericsson

- R2-2001636	On supporting of SIB for positioning 	Samsung R&D Institute UK

- R2-2001234	Summary of [108#89][NR/Pos] UE-based downlink positioning assistance data 	Qualcomm Incorportaed

- R2-2001949	Summary of [AT109e][624][POS] Open issues on UE-based downlink positioning assistance data	Qualcomm Incorporated

- R2-2001245	Summary of UE-based positioning Agenda Item 6.8.2.5	Qualcomm Incorporated

- R2-2000837	On supporting UE-based positioning	Sony

- R2-2001240	UE-based configuration options	Ericsson

- R2-2001244	Remaining details for UE-based downlink positioning assistance data	Qualcomm Incorporated

- R3-200060	Transmission Measurement Function in NG-RAN	Ericsson

- R3-200061	Introduction of positioning support over F1AP	Qualcomm Incorporated

- R3-200062	Positioning support over F1AP	Huawei

- R3-200063	Introduction of NR Positioning	Ericsson

- R3-200424	"(TP for NR_POS BL CR for TS 38.455) 	NRPPa open issues for 	Broadcast Assistance Data Delivery"	Qualcomm Incorporated

- R3-200425	"(TP for NR_POS BL CR for TS 38.473) 	F1AP issues for Broadcast 	Assistance Data Delivery"	Qualcomm Incorporated

- R3-200426	(TP for NR_POS BL CR for TS 38.455)  NRPPa changes to support Exchange of TRP Information and Measurements by specific TRPs	Qualcomm Incorporated

- R3-200457	Discussion on SRS related UL positioning procedure	Huawei

- R3-200458	Multiple discussions on NRPPa (FFS & TRP & TP)	Huawei

- R3-200459	(TP for BL CR for TS 38.455) NR Assistance Data Delivery	Huawei

- R3-200460	(TP for BL CR for TS 38.473): F1 support for NR positioning	Huawei

- R3-200461	(TP for BL CR for TS 38.470): F1 support for NR positioning	Huawei

- R3-200462	F1 support for positioning	Huawei

- R3-200479	(TP for NR_POS BL CR for TS 38.455) Additional details for NRPPa Measurement procedures	Nokia, Nokia Shanghai Bell

- R3-200480	TRP configuration information exchange	Nokia, Nokia Shanghai Bell

- R3-200546	Introduce Positioning SI Area ID	CATT

- R3-200547	(TP for [NR_POS-Core] BL CR for TS 38.455) Introduce Positioning SI Area ID	CATT

- R3-200776	(TP for NR_POS BL CR for TS 38.473) F1AP changes to support Exchange of TRP Information	Qualcomm Incorporated

- R3-200777	Introduction of NR Positioning	Ericsson

- R3-200778	Positioning measurements support in NRPPa	Intel Corporation

- R3-200779	(TP for BL CR 38.455) Positioning measurements	Intel Corporation

- R3-200780	Positioning measurements support in F1-AP	Intel Corporation

- R3-200781	(TP for BL CR 38.473) Positioning measurements	Intel Corporation

- R3-200784	Introduction of NR Positioning	Ericsson

- R3-201016	[TP to BL CR for TS 38.305] correction on NG-RAN architecture	Ericsson, Intel

- R3-201017	Exchange of DL PRS configuration over Xn	Ericsson

- R3-201018	Introduction of DL PRS configuration exchange over Xn	Ericsson

- R3-201019	Reporting of UL AoA beam info to LMF	Ericsson

- R3-201020	Discussion on remaining FFS of delivery of Positioning Assistance Data over F1AP	Ericsson

- R3-201021	[TP to BL CR for TS 38.470] Positioning support over F1AP	Ericsson

- R3-201187	Summary of offline discussion for CB: # 56_Email056-Pos_BLs	Intel

- R3-201188	Summary of offline discussion for CB: # 57_Email057-Pos_NRPPa_ext	Nokia

- R3-201189	Summary of offline discussion for CB: # 58_Email058-Pos_PRSexchange_Xn	Ericsson

- R3-201190	Summary of offline discussion for CB: # 59_Email059-Pos_Broadcast_Assist	Huawei

- R3-201191	Summary of offline discussion for CB: # 60_Email060-Pos_split_gNB_arch	Qualcomm

- R3-201290	(TP for NR_POS BL CR for 38.455) NRPPa extensions for RAT-dependent positioning	Nokia, Nokia Shanghai Bell

- R3-201373	[TP to BL CR for TS 38.305] correction on NG-RAN architecture	Ericsson, Intel

- R3-201392	Summary of offline discussion for CB: # 56_Email056-Pos_BLs	Intel

- R4-2002179	Email discussion summary for RAN4#94e_#56_NR_pos_RRM_Part_1, Qualcomm

- R4-2002310	Email discussion summary for RAN4#94e_#56_NR_pos_RRM_Part_1, Qualcomm

- R4-2002277 WF on NR Positioning UE requirements, Qualcomm

- R4-2002328 WF on NR Positioning UE requirements, Qualcomm

- R4-2002289 CR 0568 to TS 38.133 on Measurement report mapping for PRS RSRP, Ericsson

- R4-2002288 NR E-CID measurement requirements, Ericsson

- R4-2001950 LS on NR E-CID measurements, Ericsson

- R4-2002287 Updated link level simulation assumptions for RSTD and RSRP measurements, Huawei

- R4-2002180	Email discussion summary for RAN4#94e_#57_NR_pos_RRM_Part_2, Ericsson

- R4-2002311	Email discussion summary for RAN4#94e_#57_NR_pos_RRM_Part_2, Ericsson

- R4-2002275 WF on NR Positioning gNB measurement requirements and report mapping, Ericsson

- R4-2002280 LS on gNB measurements report mapping for NR Positioning, Ericsson

- R4-2002276 WF on NR Positioning measurement impact on RRM requirements, Qualcomm

- R4-2002275 WF on NR Positioning gNB measurement requirements and report mapping, Ericsson

- R4-2002276 WF on NR Positioning measurement impact on RRM requirements, Qualcomm

- R4-2002280 LS on gNB measurements report mapping for NR Positioning, Ericsson

- R4-2002277	WF on NR Positioning UE requirements, Qualcomm

- R4-2002328	WF on NR Positioning UE requirements, Qualcomm

- R4-2002275	WF on NR Positioning gNB measurement requirements and report mapping, Ericsson

- R4-2002276	WF on NR Positioning measurement impact on RRM requirements, Qualcomm

- R4-2002280	LS on gNB measurements report mapping for NR Positioning, Ericsson

- R4-2001947	LS on UE measurement report mapping for UE positioning measurements in NR, Ericsson

- R4-2001632	On report mapping for UE/gNB positioning measurement, Huawei, HiSilicon

- R4-2001633	[draft] reply LS on agreements related to NR Positioning, Huawei, HiSilicon

- R4-2000388	Discussion on UE PRS processing behavior, Intel Corporation

- R4-2001918	UE behaviour for processing DL PRS without measurement gap, Nokia, Nokia Shanghai Bell

- R4-2000389	Further Discussion on NR PRS RSTD Requirements for UE, Intel Corporation

- R4-2000589	Discussion on RSTD measurement requirements, CATT

- R4-2000731	On PRS-RSTD measurements for NR positioning, Qualcomm Incorporated

- R4-2000783	On RSTD measurement report mapping for NR positioning, Apple

- R4-2000784	LS on RSTD measurement report mapping for NR positioning, Apple

- R4-2000998	Discussion on PRS-RSTD measurement, MediaTek inc.

- R4-2001637	Discussion on RSTD measurement, Huawei, HiSilicon

- R4-2001941	On PRS RSTD measurements, Ericsson

- R4-2001942	On PRS RSTD measurement report mapping, Ericsson

- R4-2001943	Measurement report mapping for PRS RSTD (CR), Ericsson

- R4-2000590	Discussion on PRS-RSRP measurement requirements, CATT

- R4-2000732	On PRS-RSRP measurements for NR positioning, Qualcomm Incorporated

- R4-2000999	Discussion on PRS-RSRP measurement, MediaTek inc.

- R4-2001638	Discussion on PRS-RSRP measurement, Huawei, HiSilicon

- R4-2001944	On PRS-RSRP measurements Source, Ericsson

- R4-2001945	On PRS-RSRP measurement report mapping, Ericsson

- R4-2001946	Measurement report mapping for PRS-RSRP, Ericsson

- R4-2002289	Measurement report mapping for PRS-RSRP, Ericsson

- R4-2000603	Discussion on Rx-Tx timing difference measurement, CATT

- R4-2000604	Discussion on definition of Rx-Tx timing difference measurement, CATT

- R4-2000733	On UE Rx-Tx time difference measurement for NR positioning, Qualcomm Incorporated

- R4-2001000	Discussion on UE Rx-Tx time difference measurements, MediaTek inc.

- R4-2001639	Discussion on Rx-Tx time difference measurement, Huawei, HiSilicon

- R4-2001859	UE Rx-Tx Measurement Report Mapping in NR, Ericsson

- R4-2001940	On UE Rx-Tx measurements, Ericsson

- R4-2000734	On positioning requirements for E-CID, Qualcomm Incorporated

- R4-2001948	NR E-CID measurement requirements, Ericsson

- R4-2002288	NR E-CID measurement requirements, Ericsson

- R4-2001949	NR E-CID reporting criteria requirements, Ericsson

- R4-2001950	LS on NR E-CID measurements, Ericsson

- R4-2000391	Link-level simulation results for RSTD measurement, Intel Corporation

- R4-2000591	Link-level simulation results of PRS RSRP, CATT

- R4-2000592	Link-level simulation results of PRS RSTD, CATT

- R4-2000735	Link-level PRS-RSTD simulation results, Qualcomm Incorporated

- R4-2001001	Link level evaluation on PRS-RSRP and PRS-RSTD, MediaTek inc.

- R4-2001635	Link level simulation results for PRS measurement, Huawei, HiSilicon

- R4-2001636	Updated link level simulation assumption for RSTD and RSRP, Huawei, HiSilicon

- R4-2002287	Updated link level simulation assumption for RSTD and RSRP, Huawei, HiSilicon

- R4-2001939	Link simulation results for NR RSTD, Ericsson

- R4-2000054	Discussion on gNB requirements and report mapping, ZTE Corporation

- R4-2000390	Considerations on gNB measurement requirements for NR positioning, Intel Corporation

- R4-2000736	on gNB requirements for NR positioning, Qualcomm Incorporated

- R4-2001196	Views on gNB measurement for NR positioning, NTT DOCOMO, INC.

- R4-2001496	On gNB measurement requirements, Ericsson

- R4-2001634	Discussion on gNB requirements for NR positioning, Huawei, HiSilicon, CMCC

- R4-2001919	On gNB measurement requirements for NR positioning, Nokia, Nokia Shanghai Bell

- R4-2000605	Further discussion on impact of positioning measurement on RRM requirements, CATT

- R4-2000737	On Impact of NR positioning on existing RRM requirements, Qualcomm Incorporated

- R4-2001640	Impact of PRS measurement on existing RRM requirements, Huawei, HiSilicon

- R4-2000593	Discussion on frequency layer and measurement gap, CATT

- R4-2000738	On UE-based positioning requirements, Qualcomm Incorporated

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template