3GPP TSG-RAN WG4 Meeting #94-e	R4-2002470

# Electronic Meeting, 24 Feb. – 6 Mar., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0925 | rev | 1 | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.104 on channel spacing correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-2-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The channel spacing currently captured in TS 37.141 for carriers between E-UTRA and NR is incorrect. There is no 30kHz channel raster cases but 30kHz channel raster granularity. Hence it is proposed to correct current version |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete the 30kHz channel raster and change to 30kHz channel raster granularity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Channel spacing not calculated correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

--------------Start of text change-------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception".

[3]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD) ".

[4]	3GPP TS 25.105: "Base Station (BS) radio transmission and reception (TDD) ".

[5]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[6]	3GPP TS 45.005: "Radio transmission and reception".

[7]	ITU-R Recommendation M.1545, "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[8]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[9]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing".

[10]	3GPP TS 25.141: "Base Station (BS) conformance testing (FDD) ".

[11]	3GPP TS 51.021: "Base Station System (BSS) equipment specification; Radio aspects".

[12]	3GPP TS 25.142: "Base Station (BS) conformance testing (TDD) ".

[13]	Recommendation ITU-R SM.329-10, "Unwanted emissions in the spurious domain".

[14]	3GPP TR 25.942: "Radio Frequency (RF) system scenarios".

[15]	ITU-R recommendation SM.328: "Spectra and bandwidth of emissions".

[16]	IEC 60721: "Classification of environmental conditions".

[17]	IEC 60721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations".

[18]	IEC 60721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Section 4: Stationary use at non-weather protected locations".

[19]	ETSI EN 300 019-1-3, European Standard (Telecommunications series), “Environmental Engineering (EE); Environmental conditions and environmental tests for telecommunications equipment; Part 1-3: Classification of environmental conditions; Stationary use at weather protected locations”

[20]	ETSI EN 300 019-1-4, European Standard (Telecommunications series), “Environmental Engineering (EE); Environmental conditions and environmental tests for telecommunications equipment; Part 1-4: Classification of environmental conditions; Stationary use at non-weather protected locations”.

[21]	IEC 60068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold".

[22]	IEC 60068-2-2 (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat".

[23]	IEC 60068-2-6 (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)".

[24]	CEPT ECC Decision (13)03, "The harmonised use of the frequency band 1452-1492 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)".

[25]	CEPT ECC Decision (17)06, “The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)”.

[26]	3GPP TS 38.141-1: "NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing".

[27]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[28]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

--------------Start of next change-------------

4.5	Channel arrangement

4.5.1	Channel spacing

The GSM/EDGE carrier spacing is 200 kHz [6].

The nominal UTRA FDD channel spacing is 5 MHz. The nominal channel spacing is 1.6 MHz for the 1.28 Mcps UTRA TDD Option. These can be adjusted to optimise performance in a particular deployment scenario [3,4].

In E-UTRA the spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent E-UTRA carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA carriers. The channel spacing can be adjusted to optimize performance in a particular deployment scenario [5].

The standalone NB-IoT carrier spacing is 200 kHz.

In NR the spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the BS channel bandwidths. The nominal channel spacing between two adjacent NR carriers is defined as following:

-	For NR FR1 operating bands with 100 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

-	For NR FR1 operating bands with 15 kHz channel raster,

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2 + {-5kHz, 0kHz, 5kHz}

where BWChannel(1) and BWChannel(2) are the BS channel bandwidths of the two respective NR carriers. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario [27].

The spacing between E-UTRA and NR carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between and E-UTRA carrier and an adjacent NR carrier  is defined as following:

-	For NR operating bands with 100 kHz channel raster,

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2

-	For NR operating bands with 15 kHz channel raster,

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz} for ∆FRaster equals to 15 kHz

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals to 30 kHz

where BWE-UTRA_Channel and BWNR_Channel are the channel bandwidths of the E-UTRA and NR carriers, ∆FRaster is the  band dependent channel raster granularity defined in TS38.101-1[28]. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

-------------End of change-------------