3GPP TSG RAN Meeting #87e​		RP-200504

Electronic Meeting, March 16-19, 2020​ 	(revision of RP-19xxxx)

Source:		Moderator (SoftBank)

Title:	Email discussion summary of [FWA_23dbm]

Agenda Item:	9.1.1

Document for:	Information

Introduction

This is a document summarizing email discussion during RAN#87-e meeting. The following contributions

- RP-200135	New WID on Introduction of FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258	SoftBank Corp.

- RP-200136	Motivation for Introduction of FR2 FWA NR UE with maximum TRP of 23dBm for band n257	SoftBank Corp., Rakuten Mobile, KDDI, NTT DOCOMO

- RP-200277	Motivation for introduction of new FR2 FWA UE power class	Huawei, HiSilicon

Goal of the email discussion is to develop a stable WID aiming at the approval at this e-meeting.

Outcome and proposal of this email discussion

Moderator’s proposal

- Approve the WID with the latest version

  - https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_87e/Inbox/Drafts/FWA_23dbm/revision-RP-200135_NR_New_FR2_FWA-WID_r12_rm.doc

  - https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_87e/Inbox/Drafts/FWA_23dbm/revision-RP-200135_Time_budget_request_r12.xls

- Note: the latest WID above will be uploaded as RP-200503

- As for beam corresponding aspect and testability aspect, interested companies are encouraged to continue discussion until RAN#88.

The details of the discussions and the background of this proposal can be found in Section 3 and 4.

Proposed Schedule

1. Collection of companies' view (deadline: 12:00(pm) 17th March of UTC)

Please input your views on Tdoc RP-200135, which is the latest version after the email discussion. I have no plans to prepare any word document for the 1st stage discussion for now, so please use plain text to input your comment.

2. Moderator's way forward (deadline: 3:00(am) 18th March of UTC)

The moderator will summarize all the input from interested companies, and prepare a way forward (and/or modified WID) to address the concerns. The intermediate summary document may be prepared then.

3. Final review and addition of supporting IMs (deadline: 8:00(am) 19th March of UTC)

Deadline may be changed considering the number of input. Basically I would like to set an earlier deadline than others to avoid the collision with other topics. Official summary document (with tdoc No) may be submitted if requested by Mr. Chairman/MCC.

Email discussion

Collection of companies' view

3 companies have input their views, and they are captured in the table below:

| Company | Comment |
| --- | --- |
| Intel | We support the approach to list 2 candidate options on how to introduce new UE types and agree to keep final decision on this up to WG-level discussion. Same time, since the latest WID includes these 2 candidate options (new power class or new UE capability), we suggest to remove references to power class 5 from the objectives as follows: - Operating band for power class 5 UE is band n257 and n258. Define requirements for operating bands n257 and n258 In addition, “new power class” term is used in the Table of Impacted existing TS/TR. We suggest to replace all relevant text with the “new FR2 FWA UE with maximum TRP of 23dBm” term. |
| Qualcomm | Thank you for managing this discussion and WID objectives look good but as Intel points out, the WID terminology should be consistent for all parts. Regarding the two options, could proponent enlighten us what was the motivation to consider a new type of capability as an option for the function that is currently fulfilled by the power class designator? If we define a new capability, distinguishing this in RAN4 specifications is difficult or at least makes the readability of the specification quite bad. In addition to complicating capability signaling interface, we have to consider requirements for UE with that capability separately anyway so there is very little re-use regardless of the new capability or new power class approach so it is difficult to see the benefit. Having said this, we are fine to discuss this aspect in WG level if this is what proponents want. It is just a little surprising to see this potentially contentious issue being added this late without any motivation provided. It also works against the urgency aspect of this work item. Discussion how to differentiate UE types in Ran4 originally was highly contentious and took long time. I can easily see us in RAN4 spending majority of this work item discussing about this issue. |
| Apple | RAN4 had in the past held extensive discussion whether Pmax signaling can be adopted in FR2 for the purpose of reducing the UE emissions from regulatory perspective, such as for fixed wireless deployments where the max EIRP or max TRP regulation is more restrictive than the existing UE power class capability. RAN4 was unable to come to a consensus on this, since compliance with emission regulation is a UE design requirement which impacts more than just chipset capability (for example, antenna panel integration into form factor, beam calibration, etc.). We view “Option 1” in the objective (i.e. “Existing power class with any modifications/additions (e.g. new UE capability) is reused”) as a variant of this proposal, where some signaling solution other than power class is used to differentiate very distinct UE types/designs. Based on this analysis, we don’t see a clear feasibility for the approach according to Option 1. Our recommendation is to remove this option and to focus the work on introducing a new power class according to the WID motivation. As we also mentioned in the past on this topic, the RF specifications for fixed wireless access devices are not complete in Rel-15 and Rel-16 NR (namely, beam correspondence requirements are missing, and RAN4 decisions of FR2 RF test grids from the testability perspective have not been made). We would like to request that this objective be added into the WID not only for this new PC but also for PC1. |

Summary of companies’ veiw and moderator’s way forward

Three points were raised during the email discussion.

- Point 1: Choice of options (i.e. UE capability v.s. power class)

- Technical discussions on the pros and cons took place

- Companies are interested in when the choice is made (now or during WI phase) ?

- Point 2: Improvement of the text

- Terminology should be aligned, i.e. use of “power class” should be avoided until the choice of option is made

- Point 3: Addition of objective (late comment)

- Completion of RF specifications for FWA UEs should be included in this WI.

Point 1: It is always good to limit the scope of WI. However considering the comment from companies as well as the history of this discussion, it wouldn’t be so easy to make the decision within a short period. Therefore, the moderator would like to propose to keep the current objective, i.e. the choice of options is made during the WI phase and to encourage companies to continue the offline discussion until then.

Point 2: Given the recommendation for point 1, the proposal should be valid. So, the moderator would like to propose to apply the corresponding change to the WID.

Point 3: The proposal may be valid from a technical point of view. However, inclusion of this scope would not be appropriate because of the following reasons:

- The proponents of this WI think this WI is urgent from the business/regulatory point of view. The extension of the scope should be considered separately with a different timeline.

- More importantly, this is not non-spectrum item but spectrum item. Since the specific band numbers are clearly shown in the WI title, any impact to other band (i.e. PC1) shall be avoided..

Therefore, the moderator would like to ask the proponent of point 3 to bring a separate new WI if any impact to other band(s) is foreseen.

Comments from two companies are input in response to the proposals above. The issues pointed out by these comments ware addressed in the next round of discussion.

| Company | Comment |
| --- | --- |
| T-Mobile USA | T-Mobile USA shares Qualcomm’s concerns expressed below, the changes proposed in option 1 effect more than n257 and n258 which is why T-Mobile USA would have concerns with option 1 in the final WID. The proposed language changes to the WID are as follows: To address the issue described in section 3, one option from the following is chosen: – Option 1. Existing power class with any modifications/additions (e.g. new UE capability) is reused. – Option 2. A new power class is defined. Depending on the decision above, Introduce a new power class and the following requirements for the existing power class with the new capability or the new power class. RF part: Define the RF requirements for the new UE capability or new UE power class aligned with regional (e.g. Japanese) regulation for a FWA type of device: – Define requirements for operating bands n257 and n258 – UE RF Tx requirements  Maximum TRP equal to 23dBm  Maximum peak EIRP 43 dBm  Min EIRP higher than current PC3  Spherical coverage requirement sufficient for FWA type device (e.g. 85%-ile same as PC1)  MPR/AMPR requirements based on PC3 (max TRP of 23dBm)  Beam correspondence requirements – UE RF Rx requirements  REFSENs requirement including min peak EIS, spherical coverage EIS – Define general and band dedicated requirements based on band, CA and EN-DC configurations requests RRM part: – Define RRM core requirements which are band class dependent for this new UE capability or power class  RRM requirements for PC1 should be taken as baseline RAN2 Introduce any signaling necessary to support the addition of UE capability or power class based on the choice of option. |
| Apple | We also agree with John’s suggested revisions. Furthermore, to follow up on the comment I shared earlier regarding the unfinished specifications for PC1, I would like to further clarify the comment. The intention is not to overload the WID with objectives for a different power class but, rather, to keep the eventual agreements on beam correspondence and measurement grid for fixed wireless access devices (e.g. PC1 and this new power class) harmonized. Toward that end, my suggestion is to capture in the objective that these requirements can be reused for both PCs. Suggested markup (on top of the T-Mobile suggestion) is below: To address the issue described in section 3, one option from the following is chosen: – Option 1. Existing power class with any modifications/additions (e.g. new UE capability) is reused. – Option 2. A new power class is defined. Depending on the decision above, Introduce a new power class and the following requirements for the existing power class with the new capability or the new power class. RF part: Define the RF requirements for the new UE capability or new UE power class aligned with regional (e.g. Japanese) regulation for a FWA type of device: – Define requirements for operating bands n257 and n258 – UE RF Tx requirements  Maximum TRP equal to 23dBm  Maximum peak EIRP 43 dBm  Min EIRP higher than current PC3  Spherical coverage requirement sufficient for FWA type device (e.g. 85%-ile same as PC1)  MPR/AMPR requirements based on PC3 (max TRP of 23dBm)  Beam correspondence requirements for a FWA type of device with applicability to the new power class (outcome can be reused for PC1) – UE RF Rx requirements  REFSENs requirement including min peak EIS, spherical coverage EIS – Define general and band dedicated requirements based on band, CA and EN-DC configurations requests - Assess the testability aspects * Define measurement grid for a FWA type of device with applicability to the new power class (outcome can be reused for PC1) RRM part: – Define RRM core requirements which are band class dependent for this new UE capability or power class  RRM requirements for PC1 should be taken as baseline RAN2 Introduce any signaling necessary to support the addition of UE capability or power class based on the choice of option. |

Review of WID

The discussion was held based on the updated WID, which is available to the FTP server.

| WID | https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_87e/Inbox/Drafts/FWA_23dbm/revision-RP-200135_NR_New_FR2_FWA-WID_r11_rm.doc |
| --- | --- |
| TU request | https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_87e/Inbox/Drafts/FWA_23dbm/revision-RP-200135_Time_budget_request_r11.xls |

Based this revision, the following points were discussed, and the companies’ view is summarized in the table below:

(Q1) TU request

Intel has proposed to add 0.5TU for RAN4#95, and the rapporteurs think it is reasonable. So, this proposal will be accepted unless further comments are received.

(Q2) choice of options

Even though the rapporteur proposes to keep both options at this moment (leave it to WG discussion), some companies still strongly suggest to choose one  (i.e. Option 2: define new power class) now. Rapporteur kindly asked interested companies to input their preference to move forward.

(Alt 1) - support option 1: UE capability

(Alt 2) - support option 2: New power class:

(Alt 3) - keep both options until the sufficient WG discussion is made:

(Q3) modification for beam correspondence part

One company has suggested adding some more description for this part, i.e. "Beam correspondence requirements for a FWA type of device with applicability to the new power class (outcome can be reused for PC1)" to keep the commonality with other FWA UE, (i.e. PC1). Companies view on this proposal were requested.

(Q4) addition of testability aspect

One company has suggested adding a new objective "Assess the testability aspects" to achieve common measurement grid for a FWA type UEs.  Currently, companies view on this aspect is not clear. Companies were suggested to input their view (esp. supporting company) for this proposal.

| Company | Question | Comment |
| --- | --- | --- |
| Samsung | (2) | Regarding the choice of UE capability and power class, we also think UE power class is more straight-forward since power class definition in FR2 is not the maximum transmitting power capability but also related to UE implementation design including form factor, antenna and so. However, as compromise, we think we can further discuss in WI phase. For (2) choice of options, as I said in previous e-mail, we also support Alt 2 but can accept the current WID for the sake of progress. Therefore, could you please add [Samsung] under Alt 2? We can continue discuss other aspects after seeing other companies view |
| Samsung | (3) | For PC1, I actually would like to follow the same argument above, i.e., it is NOT straightforward that BC requirements defined for PC5 can re-used for PC1. I agreed separated WI is better solution if proponent would like to introduce the BC requirements for PC1. <additional comment was input later> It seems there is some misunderstanding. For modification for beam correspondence, I did not see any comments on removing BC objective for PC5. What has been discussed in whether BC requirements for PC5 can be applied for PC1. My comment is actually about how to treat the BC for PC1, i.e., either treated together in this WI or in separated WI. Given the difference between this new FWA type (PC5) from existing PC1 is not clear, I think it is premature to decide to reused the same BC for PC1 and PC5. To further address the concerns of missing BC for existing PC1, I suggest BC requirements for PC1 can be discussed in separated WI, of course also pending the discussion on BC for PC5 in next quarter. To avoid further confusion, it is better to modify my comments precisely (3) modification for beam correspondence part Apple has suggested to add some more description for this part, i.e. "Beam correspondence requirements for a FWA type of device with applicability to the new power class (outcome can be reused for PC1)" to keep the commonality with other FWA UE, (i.e. PC1). On the other hand Samsung suggest to have a separate WI to address this beam correspondence for power class 1 because they think it is not straightforward to reuse the beam correspondence requirements for PC5 before RAN4 conclude the BC requirements for PC5. I would suggest companies to input their view (esp. supporting company) for this. If no sufficient number of support is shown, I would propose to keep the current sentence as it is. Hope above clarifies the current situation |
| Oppo | (2) | Support Alt 2, New power class |
| Oppo | (3) | We understand the intention of keeping commonality with other FWA UE and also think it is a good idea, but we prefer to introduce that for the PC1 in a separate NWI to speed up the discussion of PC5. After the beam correspondence requirement defined for PC5, other FWA power classes can evaluate whether it can be reused or adjusted. Currently we are not quite sure of the difference between PC5 and PC1 FWA UEs in reality. |
| LG | (2) | LG support Alt.2 i.e. new power class definition would benefit by reducing the discussion topic considering only 6 month of core completion time. |
| NTT DOCOMO | (2) | Regarding 2), we support option1 and agree with the compromise idea as Yosuke-san provided. i.e., Alt 3. We would like to discuss this issue at WG level. At this moment, both options should be kept. |
| Huawei, HiSilicon | (2) | we understood that there were extensive discussions for signaling. In our understanding, some companies including us proposed to define the new UE category, while some company proposed to reuse the existing one with additional capability signaling. Those proposals were discussed in previous RAN plenary meetings. We view the current Yosuke-san’ version as a compromise solution, i.e., taking the proposals from both side and making decision within the limited number of meetings. We approaches the deadline of this RAN plenary. I think we can live with two options in the SID and have further discussion in WG meetings. In order to address your concern and make progress, we wonder and would like to check if the proponents for capability signaling approach do have strong preference to keep it. If so, it might be fair to keep both options in SID. |
| Huawei, HiSilicon | (4) | This testability issue seems a general issue, as you said the testability can also apply to PC1. Could we have separate SI/WI for it? We may need consider the general solution for all the related FR2 power classes. We are afraid if we only consider “power class 5” we would miss some aspects. We prefer a general solution. |
| Intel | (2) | Our preference is to keep Alt 3 such that companies may have more technical discussion on both solutions at the WG-level to assess the pros/cons of both approaches. Per discussion below I have not seen too much technical reasons for focusing on new PC solution only rather than specification convenience. |
| TIM | (2) | Concerning (2) I do not have a strong preference regarding alternatives 1, 2 and 3. However, I think (2) is not one of the first topic to be discussed, and can be decided towards the end of the Work item. My suggestion is to keep it open for now, don’t “waste” time at WG level and come back on this issue at RAN#88. |
| TIM | (3, 4) | Finally, as a general comment, since there is an urgent market demand for this power class, I suggest for (3) and (4) to keep focused as much as possible the objectives of the Work Item. |
| KDDI | (2) | Though KDDI has been supporting the new power class option, the compromise option of Alt 3 is very much agreeable for smooth proceeding. Market demand will be taking off in the course of time, the same way as rest of the world where Power Class 1 is consistent with their regulations. |
| Nokia | (3) | The version in https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_87e/Inbox/Drafts/FWA_23dbm/revision-RP-200135_NR_New_FR2_FWA-WID_r11_rm.doc was acceptable from the Nokia perspective. However, I noticed some discussion there were some revision proposals for the beam correspondence bullet, which I don’t entirely understand and we don’t agree on. (3) modification for beam correspondence part Apple has suggested to add some more description for this part, i.e. "Beam correspondence requirements for a FWA type of device with applicability to the new power class (outcome can be reused for PC1)" to keep the commonality with other FWA UE, (i.e. PC1). On the other hand Samsung suggest to have a separate WI to address this because they think it is not straightforward. I would suggest companies to input their view (esp. supporting company) for this. If no sufficient number of support is shown, I would propose to keep the current sentence as it is. As discussed in the earlier RAN plenaries, the beam correspondence is important functionality for FR2 and it should be specified for this and other power classes. Unfortunately in Rel-15 we haven’t been able to complete the other power classes but PC3 and Rel-16 progress for BC enhancements has been slow as well. We would not like to see that lack of BC progress in RAN4 would be used as a reason why BC is not defined for this FR2 FWA UE power class. Therefore, in our view the original beam correspondence bullet in the WID should be kept unchanged. |
| MediaTek | (3) | we support this description proposed by Apple. It’s fine to have better requirement alignment between different power classes based on similar UE type (i.e. FWA here). Moreover, in our understanding, RAN4 actually didn’t have a thorough discussion on FWA beam correspondence requirement, we think it’s good to discuss it deeply. |
| MediaTek | (4) | we support this objective of “assess the testability aspects”. We paid almost attention on phone type/size DUT; however, for testability aspect, UE type/size is one of the key factors. We think further studies for FWA type UE is fine. |

Note: For (Q2), some companies have already shown their preference during the 1st/2nd round discussion.

Qualcomm: Prefer Alt 2, but OK with Alt 3
T-mobile USA: Support Alt 2

Rapporteur’s proposal after 2nd round of email discussion

For (Q1)

  - Add 0.5TU request for RAN4#95 RF

For (Q2)

  - Majority companies support option 2 (new power class), while there are some supporting companies who want to study more on option 1 (UE capability)

  - Meanwhile, most of the companies are OK to Alt.3 (keep both options) as compromise

  - Based on the companies’ preference above, it is propsoed to go with Alt.3 (keep both options) with the understanding that some companies strongly prefer to option 2

For (Q3, Q4)

  - There is no consensus to include these proposals to this WI because both positive comments and concerns are received. More discussion is necessary whether and/or how to address the issues. Therefore, it is proposed to postpone the decision on these proposals, and interested companies are encouraged to continue offline discussions.