3GPP TSG RAN meeting #90e										RP-202758

Electronic Meeting, December 7-11, 2020

## Status Report to TSG

Agenda item:			9.7.6

| WI / SI Name | Study on XR (Extended Reality) evaluations for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_XR_eval |  |  |  |  |
| Unique ID | 860062 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201145 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 06/2021 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 20 % | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Juan Montojo |
|  | Company | Qualcomm |
|  | Email | juanm@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

While a new time budget is not submitted at this time, the efficiency of e-meetings is not as good as that of regular physical meetings and the initial plan to complete the study in 3 Quarters may not be realistically feasible. Note that the RAN1 discussions during Q4-2020 have been over email only, i.e., no GTW sessions scheduled for this item yet.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

TR 38.838 skeleton

[103-e-NR-XR-01] Email discussion/approval for XR TR skeleton till 11/5 – Eddy (Qualcomm)

Through the above email discussion, the skeleton of TR 38.838 was endorsed in R1-2009818.

Agreements on applications, traffic model and evaluation methodology

[103-e-NR-XR-02] Email discussion/approval for applications, traffic model and evaluation methodology – Eddy (Qualcomm) and Xiaohang (vivo)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

Through the above email discussions, the following agreements were made for RAN1 #103-e.

Agreement 1. XR applications

RAN1 confirms that diverse applications of VR1/2, AR1/2, CG are of interest for study. Potential prioritization/down selection of these applications for evaluation is to be discussed after detailed traffic models and relevant evaluation assumptions are stable.

- FFS: other applications, e.g., XR conferencing

Agreement 2. Traffic model

Traffic model for DL and UL should reflect various aspects, e.g., various bit rates, variable frame/packet (definition of frame/packet to be clarified with traffic model as necessary) size, and periodicity (how to model jitter is FFS).  RAN1 will strive to conclude on detailed traffic models in the next RAN1 meeting (104-e) where SA4 outcome on traffic model is expected to be available.

- Statistical model is preferred.

- It is preferred traffic model for both UL and DL have a certain degree of variability so thatand the total number of traffic models can be reduced.

- Note: Taking into account the fact that the decision on traffic models may hold many other crucial decisions, discussion on traffic model in the next RAN1 meeting is prioritized from the beginning.

Agreement 3: Adopt the following deployment for XR/CG evaluations

- Indoor hotspot: FR1 and FR2

  - Detailed definition of Indoor hotspot refers to TR 38.913.

  - Channel model: InH. Detailed definition of InH refers to TR 38.901.

- Dense urban: FR1 and FR2

  - Detailed deployment refers to TR 38.913, where single layer with Marco layer is assumed.

  - Channel model: UMi. Detailed definition of UMi refers to TR 38.901.

FFS: Whether to prioritize FR1 for evaluation.

Note 1: When selecting the deployment and evaluation assumptions for XR/CG evaluations, it is up to company to evaluate FR1 or FR2 or both for the frequency range.

Note 2: It does not mean that all applications are evaluated for all the deployment scenarios.

Agreement 4: Urban Macro can be reported for XR/CG evaluations only for FR1.

- FFS: whether Uma is optional or not

- Following parameters can be assumed.

| Parameter | Proposed value |
| --- | --- |
|  | Urban Macro (FR1) |
| Layout | 21cells with wraparound ISD = 500 m |
| BS Tx power | FR1: 49 dBm/20 MHz |

Agreement 5: It is to be further discussed how to prioritize the combinations of deployment scenarios and applications after traffic models for each application are stable.

Agreement 6: System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

FFS: how to calculate the percentage of satisfied users across multiple drops of simulations

Agreement 7: Adopt the simulation assumptions in table 1 as below

Table 1: Simulation assumptions for XR evaluation (Part 1)

| Parameter | Proposed value |  |
| --- | --- | --- |
|  | Indoor hotspot FR1/FR2 | Dense urban FR1/FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21cells with wraparound ISD: 200m |
| Carrier frequency | FR1: 4 GHz FR2: 30 GHz |  |
| Subcarrier spacing | FR1: 30 kHz FR2: 120 kHz |  |
| BS height | 3m | 25m |
| UE height | hUT=1.5 m |  |
| BS noise figure | FR1: 5 dB FR2: 7 dB |  |
| UE noise figure | FR1: 9 dB FR2: 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic FFS:Ideal(optional) |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8 dBi |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, FR2: UE antenna radiation pattern model 1, 5dBi |  |

Agreement 8: Adopt the following UE distribution for XR/CG evaluation for outdoor scenario

- For outdoor scenario:

  - FR1: 80% indoor, 20% outdoor

  - FR2: 100% outdoor

Other UE distribution can be evaluated optionally.

Agreement 9: Adopt the following TDD configuration for XR/CG evaluation

- FR1:

  - Option 1: DDDSU

  - Option 2: DDDUU

- FR2:

  - Option 1: DDDSU

FFS detailed S slot format

Note: Other TDD configuration or FDD can be optionally evaluated.

Agreement 10:  Adopt the following BS antenna parameters for indoor scenario for XR/CG evaluation

- FR1:

  - 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4)

  - (dH, dV) = (0.5, 0.5)λ

- FR2:

  - Option 2: 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1)

  - (dH, dV) = (0.5, 0.5)λ

Other BS antenna parameters can be optionally evaluated

Agreement 11:  For XR/CG evaluation, adopt the following assumptions for downtilt

·         For XR/CG evaluation, adopt the following assumptions for downtilt

- Dense Urban

  - FFS: 6 or 12 degree

- Indoor hotspot

  - 90° (pointing to the ground)

Other downtilt can be optionally evaluated

Agreement 12:  Adopt the simulation assumptions in table 3 as below

Table 3: Simulation assumptions for XR evaluation (Part 3)

| Power control parameter | Companies should report |
| --- | --- |
| Transmission scheme | Companies should report |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc • and etc. |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |

Agreement 13: The following aspects are to be discussed after traffic model is stable.

- For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available.  Additional metrics to be collected will be further discussed after traffic model is stable.

- Various options for traffic arrival offset among UEs per cell were proposed by companies, e.g., even offset, random offset, no offset. It will be discussed after traffic model is determined.

Agreement 14: System bandwidth for XR/CG evaluations are as follows.

- For FR1,

  - Baseline: 100 MHz

  - Optional: 20/40 MHz (FFS: 200 MHz)

- FFS FR2

Agreement 15: For outdoor scenarios, the BS antenna parameters are as follows.

- FFS FR1,

  - Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8)

  - Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2)

  - Option 3: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1,4,4)

(dH, dV) = (0.5λ, 0.5λ)

- FR2:

  - 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1)

(dH, dV) = (0.5λ, 0.5λ)

Other configurations can be optionally evaluated.

Agreement 16: UE antenna parameters for XR/CG evaluations are as follows

- FR1:

  - Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ

  - Optional: 4T/4R, 1T/2R, 2T2R

- FFS FR2: down-selection between the next two options. Please indicate if you have preference.

  - Option 1 (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151)

    - (M, N, P)=(1, 4, 2), 3 panels (left, right, top)

    - (Mp, Np) is up to company. Need to be reported with simulation result.

  - Option 2 (from TR 38.802 – developed in Rel-14)

    - 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90°

Agreement 17: BS Tx power for XR/CG evaluations are as follows

- For Indoor hotspot:

  - FR1:

    - 24 dBm per 20 MHz

  - FR2:

    - 23 dBm per 80 MHz. EIRP should not exceed 58 dBm

- For Dense urban:

  - FR1:

    - 44 dBm per 20 MHz

  - FR2:

    - 40 dBm per 80 MHz. EIRP should not exceed 73 dBm

For system BW larger than above, Tx power scales up accordingly.

Agreement 18: UE max Tx power for XR/CG evaluations are as follows

- FR1: 23 dBm

- FR2: 23 dBm, maximum EIRP 43 dBm

Agreement 19. Baseline power evaluation methodology

TR38.840 is the baseline methodology potentially with some modifications if necessary.  RAN1 aim to minimize modeling effort.

Agreement 20.

- RAN1 continues to discuss evaluation methodologies for UE power consumption and system capacity.

- RAN1 is to discuss whether/how to study/evaluate mobility and coverage at a later stage, e.g., starting from Q1 2021.

#### 2.1.2	Remaining Open issues

In accordance to the SID:

- Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc.

- Complete evaluation methodology to assess XR and CG performance along with the identification of KPIs of interest for relevant deployment scenarios

- Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template