3GPP TSG RAN meeting #90e 																RP-20xxxx

Electronic Meeting, 7-11 December 2020

Agenda item:			TBA

Source:	Email discussion moderator (Intel)

Title:	Report from Email Discussion [90E][43][60GHz_OTA]

Document for:	Discussion and decision

# Introduction

This document provides as summary of the following email discussion during RAN#90-e:

[90E][43][60GHz_OTA]

Goal: Generate an agreeable way forward.

Input contributions covered: 2661

# Discussion

## Background

The following summarizes the key observations and proposals listed in RP-202661:

Observation #1: OTA test methods are used as a baseline approach for NR mmWave test methodology for RF, RRM, and Demodulation testing. Observation #2: The existing mmWave OTA UE test methods are applicable to FR2 frequency bands and their extension to carrier frequencies above 52.6 GHz should be further studied Proposal #1: Further study and define NR 52.6-71GHz OTA test methods within the Rel-17 timeframe - Option 1: Initiate a separate SI in parallel with NR 52.6 – 71 GHz WI - Option 2: Extend the scope of the NR 52.6 – 71 GHz WI to cover the testability aspects Proposal #2: Consider the following SI/WI objectives to enable NR 52.6-71GHz OTA test methods - Study and define the over the air (OTA) test methods for UE RF, RRM, and demodulation requirements for the 52.6GHz-71GHz frequency range [RAN4] - Extend the applicability of the FR2 OTA test methods in TR 38.810 wherever possible - Identify any changes needed, including general testing and calibration, permitted test methods, multi-path fading propagation conditions, measurement applicability criteria. - Target device types: Handheld UE, laptop, tablet, FWA, vehicular mounted device; other UE types not precluded.

## Initial round

### Open issues

The following questions are proposed to be discussed in the initial round:

- Question #1: Whether to further study and define NR 52.6-71GHz OTA test methods within the Rel-17 timeframe

- Question #2: How to organize the NR 52.6-71GHz OTA test methods studies within the Rel-17 timeframe

  - Option 1: Initiate a separate SI to study NR 52.6-71GHz OTA test methods

  - Option 2: Extend the scope of the NR 52.6 – 71 GHz WI to cover the testability aspects

  - Option 3: other options?

- Question #3: Candidate study objectives

  - Option 1: Study and define the over the air (OTA) test methods for UE RF, RRM, and demodulation requirements for the 52.6GHz-71GHz frequency range [RAN4]

    - Extend the applicability of the FR2 OTA test methods in TR 38.810 wherever possible

    - Identify any changes needed, including general testing and calibration, permitted test methods, multi-path fading propagation conditions, measurement applicability criteria.

    - Target device types: Handheld UE, laptop, tablet, FWA, vehicular mounted device; other UE types not precluded.

    - Utilize free space testing configuration for test methods definition.

### Companies views’ collection

Question #1: Whether to further study and define NR 52.6-71GHz OTA test methods within the Rel-17 timeframe?

| Company | Comments |
| --- | --- |
| Company A |  |

Question #2: How to organize the NR 52.6-71GHz OTA test methods studies within the Rel-17 timeframe

- Option 1: Initiate a separate SI to study NR 52.6-71GHz OTA test methods

- Option 2: Extend the scope of the NR 52.6 – 71 GHz WI to cover the testability aspects

- Option 3: other options?

| Company | Comments |
| --- | --- |
| Company A |  |

Question 3: Candidate study objectives

- Option 1: Study and define the over the air (OTA) test methods for UE RF, RRM, and demodulation requirements for the 52.6GHz-71GHz frequency range [RAN4]

  - Extend the applicability of the FR2 OTA test methods in TR 38.810 wherever possible

  - Identify any changes needed, including general testing and calibration, permitted test methods, multi-path fading propagation conditions, measurement applicability criteria.

  - Target device types: Handheld UE, laptop, tablet, FWA, vehicular mounted device; other UE types not precluded.

  - Utilize free space testing configuration for test methods definition.

Companies are encouraged to share views on the candidate objectives of the studies and whether proposed Option 1 objectives are acceptable.

| Company | Comments |
| --- | --- |
| Company A |  |

### Summary and recommendation for further discussion

## Intermediate round

## Fine-tuning round

# Conclusion

Based on the email discussion, the following are proposed:

- TBD