3GPP TSG RAN meeting #91e										RP-210643

Electronic Meeting, March 16-26, 2021

## Status Report to TSG

Agenda item:			9.7.23

| WI / SI Name | NR and MR-DC measurement gap enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MG_enh |  |  |  |  |
| Unique ID | 890161 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202658 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: 09/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 10% | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ato (Tsang-Wei) YU, Rui HUANG |
|  | Company | MediaTek Inc., Intel Corporation |
|  | Email | ato.yu@mediatek.com, rui.huang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #98e (January 2021, Electronic)

- A WF on mmultiple concurrent and independent MG patterns was approved in [7]

  - Work plan

    - Before RAN4#100b (Q4’21), RAN4 focuses on the functionality and principles needed to support parallel MG patterns without considering pre-configured gap and NCSG.

  - Definition

    - Concurrent MG definition

      - UE behavior for non-overlapping, partially or fully overlapped cases is irrelevant to the definition and will be discussed separately.

      - Note 1: current definition does not address pre-configured MG patterns and NCSG. FFS how to address pre-configured MG patterns and NCSG.

    - Concurrent MG patterns

      - MG patterns are selected from Rel-16 gap patterns #0 to #25.

  - Applicability

    - The measurement purposes of concurrent gaps include:

      - Different SMTC configurations

      - Different RSs, e.g., SSB, CSI-RS, PRS, RSSI

      - Different RATs

      - FFS whether to extend to NCSG or pre-configured MG in the 2nd phase of the WI

      - Other purposes not precluded

    - RAN4 to ensure both UE and NW have the same understanding on the usage of each measurement gap.

  - Overlapping

    - RAN4 to work on at least non-overlapping concurrent gap as a start point.

  - Measurement Requirement

    - Measurement capability

      - Keep Rel-16 UE measurement capability of number of layers to be monitored by gap(Section 9.1.3 of TS38.133) for UE configured with concurrent gaps

  - Others

    - No new gap pattern will be introduced in the scope of concurrent gaps objective in this WI.

    - Do not introduce the concurrent gap in LTE SA mode.

- A WF on Pre-configured MG patterns approved in [4]

  - General procedures of pre-configured MGs:

    - 1. (Re)Configuration of the pre-configured MG

    - 2. Activation the pre-configured MG following a DCI or timer-based BWP switch

    - 3. Deactivation the pre-configured MG following a DCI or timer-based BWP switch

  - Using scenarios

    - Deprioritize MR-DC scenario.

    - Whether the pre-configured gaps shall be considered as a part of multiple concurrent gap patterns framework is considered this in 2nd stage.

  - RRM requirements with Pre-configured MGs

    - No any new interruption requirements introduced by pre-configured MGs

- A WF on NCSG was approved in [4]

  - UE behavior within NCSG

    - Same as UE within LTE NCSG, that is

      - During VIL1/VIL2 UE is not expected to transmit or receive any date on corresponding serving cell(s).

      - During ML UE is expected to transmit and receive data on the corresponding serving cell(s).

  - NCSG pattern

    - ML: If legacy gap pattern can be reused for NCSG, we can assume that MGL= VIL1+ML+VIL2

    - VIRP: If legacy gap pattern can be reused for NCSG, VIRP = MGRP of legacy MG

  - Impacts on RRM requirements due to NCSG

    - Requirements related to MGTA and impact to UL transmission follow Rel-15.

#### 2.4.2	Remaining Open issues

- Multiple concurrent and independent MG patterns

  - Work plan

    - Rapporteurs are encouraged to reflect extended Rel-17 timelines in the WID in the next plenary. The work plan can be updated afterwards.

  - Definition

    - Concurrent MGs are multiple MGs that are configured for measurements during a common period of time

    - Exact definition of common period of time is FFS

    - FFS definition of independent MG

    - FFS whether to merge the definition of independent gap and concurrent gap.

  - Applicability

    - FFS on whether to define the framework of configuring gaps dedicated to specific purpose(s).

  - Relation to per-UE gap and per-FR gap

    - FFS the relation to per-UE gap and per-FR gap

    - FFS the max number of concurrent gap for per-UE gap FR1-gap and FR2-gap.

    - FFS whether a per FR gap and concurrent gap capable UE shall support multiple concurrent gaps on at least one FR

  - Overlapping

    - FFS whether to work on partially and fully-overlapped cases.

  - Overhead

    - FFS Overhead for configuring multiple concurrent MG patterns.

  - Measurement Requirement

    - FFS how to define CSSF for concurrent gaps.

    - FFS: RAN4 to reuse the following existing MG related requirements for concurrent gaps: MG reference timing, effective MGRP, MG interruption and UE UL behaviour after MG.

    - Other requirements can be further discussed in future meetings

  - Others

    - FFS whether to trigger LS to get RAN2 feedback for network configuration under DC mode

- Pre-configured MG patterns

  - FFS on the following procedures for pre-configured MGs:

    - 1. (Re)Configuration of the pre-configured MG

    - FFS if specific procedure for activation after the RRC configuration is needed

    - 2. Activation the pre-configured MG following a DCI or timer-based BWP switch

    - 3. Deactivation the pre-configured MG following a DCI or timer-based BWP switch

    - Note 1: The conditions and details of each procedure are FFS

    - Note 2: MG activation in this context means that both NW and UE assume that the pre-configured MG will be used for measurements.

    - Note 3: MG deactivation in this context means that both NW and UE assume that the pre-configured MG will not be used for measurements and UE should be able to receive scheduled data.

  - Configuration procedure

    - FFS on how pre-configured MGs can be configured (per BWP or per UE/FR):

    - FFS on relation of pre-configured MG pattern and with the current RRC configured MG

  - Activation/Deactivation procedures

    - FFS on in which cases the pre-configured MGs shall be activated

    - FFS on how pre-configured MGs can be activated/deactivated:

    - FFS on evaluation on MG activation/deactivation mechanism

    - FFS on Signaling to activate (enable) the pre-configured MGs

  - Using scenarios

    - FFS on whether the pre-configured MG when BWP switching on the multiple CCs be discussed?

  - RRM requirements with Pre-configured MGs

    - FFS on activation/deactivation delay

    - FFS on measurement period for the measurements with the pre-configured MGs

    - FFS on transitions between gapless and gap-based measurement procedures during ongoing measurements

    - FFS on scheduling restriction during pre-configured MGs when not used

  - Gap patterns for pre-configured MGs

    - FFS on number of pre-configured MG patterns:

    - FFS on Per-UE/Per-FR pre-configured MG pattern applicability

- NCSG

  - Scenarios and use cases

    - The use cases of NR NCSG can be similar as these of LTE

      - FFS specific use cases

      - FFS whether the usage of per-CC gap can be FFS

    - FFS on whether to define separate NCSG patterns for sync and async needs more discussion

  - NCSG pattern

    - FFS on which legacy patterns in Rel16 can be reused for NCSG

    - FFS on whether VIL shall be defined as the equivalent time of the interrupted slots or, equals to absolute RF retuning time defined in Rel-15

      - FFS: depending on numerology

    - FFS on Per-UE/Per-FR NCSG applicability

  - NCSG configuration

    - FFS on implicit or explicit configuration of NCSG

    - FFS on number of NSCG patterns configured

  - Impacts on RRM requirements due to NCSG

    - FFS on interruption requirements

    - FFS on Per-UE or Per-FR capability support

  - Measurement applicability

    - FFS: whether NCSG can be configured simultaneously with legacy gap pattern

    - FFS on RF combination limitation

    - FFS on Rx beam limitation

    - FFS on searcher limitation

    - FFS the scheduling and measurement restriction between serving cell L1 measurement, intra-frequency measurements and inter-frequency measurements for NCSG

  - Specification impacts

    - FFS on which clause can be used to include NCSG pattern in 38.133

  - Signaling

    - FFS on how to consider the relation between NCSG and ‘NeedForGap’

    - FFS on NW configuration signaling e.g gp-ncsg0/1…/n like LTE.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN4 #98e (January 2021, Electronic)

- R4-2103472	Moderator (MediaTek)	Email discussion summary: [98e][233] NR_MG_enh_1

- R4-2103473	Moderator (Intel Corporation)	Email discussion summary: [98e][234] NR_MG_enh_2

- R4-2103676	MediaTek	WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns

- R4-2103677	Intel Corporation	WF on R17 NR MG enhancements

- R4-2103714	Moderator (MediaTek)	Email discussion summary: [98e][233] NR_MG_enh_1

- R4-2103715	Moderator (Intel Corporation)	Email discussion summary: [98e][234] NR_MG_enh_2

- R4-2104096	MediaTek	WF on R17 NR MG enhancements - Multiple concurrent and independent MG patterns

- R4-2101061	MediaTek Inc., Intel Corporation	Work plan of R17 NR and MR-DC measurement gap enhancements WI

- R4-2100221	Apple	Consideration on preconfigured measurement gap patterns

- R4-2100454	CATT	Initial discussion on pre-configured MG pattern

- R4-2100712	Xiaomi	Discussion on pre-configured MG pattern for NR

- R4-2100871	CMCC	Discussion on pre-configured MG pattern(s)

- R4-2101062	MediaTek inc.	Pre-configured MG pattern(s) per configured BWP

- R4-2101269	Intel Corporation	Discussion on pre-configured measurement gap

- R4-2101381	vivo	Considerations on pre-configured MG patterns

- R4-2101537	OPPO	Views on pre-configured MG pattern(s) for NR_MG_enh

- R4-2102268	Nokia, Nokia Shanghai Bell	Discussion on Pre-configured MG pattern(s)

- R4-2102622	Qualcomm CDMA Technologies	Views on pre-configured MG patterns

- R4-2102655	Ericsson	Overview of requirements for pre-configured measurement gaps

- R4-2102810	Huawei, HiSilicon	Initial discussion on (de)activation of pre-configured MGs

- R4-2100113	ZTE Corporation	Discussion on independent and concurrent MGs

- R4-2100222	Apple	Discussion on multiple concurrent and independent MG patterns

- R4-2100455	CATT	Initial discussion on multiple concurrent and independent MG patterns

- R4-2100641	LG Electronics	Discussion on multiple concurrent and independent MG patterns

- R4-2100713	Xiaomi	Discussion on multiple concurrent and independent MG patterns for NR

- R4-2100870	CMCC	Discussion on multiple concurrent and independent MG patterns

- R4-2101063	MediaTek inc.	Multiple concurrent and independent gap patterns

- R4-2101081	NEC	Discussion on NR measurement gap enhancements requirements

- R4-2101270	Intel Corporation	Discussion on multiple and independent concurrent measurement gaps in NR

- R4-2101538	OPPO	Views on Multiple concurrent and independent MG patterns for NR_MG_enh

- R4-2102269	Nokia, Nokia Shanghai Bell	Discussion on multiple concurrent and independent MG patterns

- R4-2102297	Qualcomm Incorporated	On requirements for mulitple concurrent and independent MG patterns

- R4-2102535	Ericsson	On parallel measurement gap patterns

- R4-2102811	Huawei, HiSilicon	Initial discussion on multiple concurrent MGs

- R4-2100223	Apple	On network controlled small gap

- R4-2100456	CATT	Initial discussion on Network Controlled Small Gap (NCSG)

- R4-2100460	CATT	CR on NCSG in 38.133

- R4-2101064	MediaTek inc.	Network Controlled Small Gap

- R4-2101271	Intel Corporation	Discussion on NCSG in NR

- R4-2101382	vivo	Considerations on network controlled small gap

- R4-2101539	OPPO	Views on pre-configured MG pattern(s) for NR_MG_enh

- R4-2102611	Qualcomm CDMA Technologies	Discussion on network controlled small gap

- R4-2102656	Ericsson	Overview of requirements for network controlled small gap

- R4-2102689	Nokia, Nokia Shanghai Bell	On Introduction of Network Controlled Small Gaps for NR

- R4-2102812	Huawei, HiSilicon	Initial discussion on NCSG

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template