3GPP TSG-WG4 Meeting #99-e	R4-2111433

# Electronic Meeting, May 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0864 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1: update configured transmitted power for V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-05-29 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The requirements are revised to include the scenarios of both mode 1 and mode 2, which cover cases of with and without serving cells. 2. SAR as well as dutycycle based solution is not considered for NR V2X, ΔPPowerClass should not be considered in the Pcmax fomula 3. No SRS antenna switching is supported for NR V2X, ∆TRxSRS does not need to be considered 4. maxTxPower is mapped to PMAX,CBR according to 38.213, however, for S-SSB and PSFCH, condition of channel busy ratio is not considered for these two channels. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. remove ΔPPowerClass in the Pcmax formula 2. remove ∆TRxSRS in the Pcmax formula 3. remove PCMAX,S-SSB and PCMAX,PSFCH for explanation for maxTxPower 4. sl-maxTxPower’ instead of ‘maxTxPower’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement of V2X Pcmax is ambiguous and not aligned with RAN1 and RAN2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

### 6.2E.4	Configured transmitted power for V2X

#### 6.2E.4.1	General

The NR V2X UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f, c = MIN {PEMAX,c – TC,c,  PPowerClass –– MAX(MAX(MPRc , A-MPRc)+ ΔTIB,c + TC,c , P-MPRc), PRegulatory,c }

PCMAX_H,f, c = MIN {PEMAX,c, PPowerClass,  PRegulatory }

where

-	PCMAX,f,c is configured for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	For the total transmitted power PCMAX,PSSCH/PSCCH , PEMAX,c is the value given by IEsl-maxTxPower, defined by TS 38.331, when the UE is not associated with a serving cell on the NR V2X carrier .

-	PPowerClass is the maximum UE power specified in Table 6.2.1-1 without taking into account the tolerance specified in the Table 6.2.1-1;

-	MPRc and A-MPRc for serving cell c are specified in clause 6.2E.2 and clause 6.2E.3 for PSSCH\PSCCH, S-SSB and PSFCH, respectively;

-	TIB,c, TC,c,  and P-MPRc are specified in clause 6.2.4

-	PRegulatory,c= 10 - Gpost connector dBm the V2X UE is within the protected zone [12] of CEN DSRC tolling system and operating in Band n47; PRegulatory,c= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on 0dB PSD offset between PSSCH and PSCCH.

For the measured configured maximum output power PUMAX,c for NR V2X sidelink transmissions non-concurrent with NR uplink transmissions, the same requirement as in clause 6.2.4 shall be applied.

For NR V2X UE supporting SL MIMO, the transmitted power is configured per each UE.

For NR V2X UE with two transmit antenna connectors, the tolerance is specified in Table 6.2E.4.1-1. The requirements shall be met with SL MIMO configurations specified in Table 6.2D.1-2.

If the UE transmits on two antenna connectors at the same time, the tolerance is specified in Table 6.2E.4.1-1.

Table 6.2E.4.1-1: PCMAX,c tolerance schemes for MIMO

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c = 26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

## <Start of Change>