3GPP RAN TSG Meeting #92-e		RP-211539

Electronic meeting, June 14 – 18, 2021

Agenda item:			9.7.4.8

Source:	Moderator (Nokia)

Title:	Email discussion summary of [92-e-22-RF-FR2-WI]

Document for:	Discussion

# Introduction

In RAN#92-e, an email thread [92-e-22-RF-FR2-WI] is assigned to discuss the following tdocs: RP-211174, 1175, 1394, 1395, 1460.

The plan is to discuss on the proposed changes to the WID first. Then the rapporteur can update the WID, if needed, based on the outcome of this email thread.

# Topic #1: RP-211174 and RP-211175

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| RP-211174 and RP-211175 | Nokia, Nokia Shanghai Bell | This WID revision proposes to - To put this objective on hold until there is a operator request for band combination. Study and if feasible define UE RF requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for (IBM) based on explicitly requested band combinations - Remove these objectives from UL gaps for self-calibration and monitoring. - PA efficiency and power consumption - Transceiver calibration due to temperature variation - Add a new objective - Enhancement of beam correspondence during initial access and RRC_INACTIVE state [RAN4 RF] - SSB-based without UL beam sweeping - For initial access, verification of beam correspondence based on msg1 spherical coverage (at least) |

## Company views

Is WID revision acceptable?

| Company | Comments |
| --- | --- |
| Ericsson | We are fine with the proposed updates/revisions. |
| Apple | We are fine with the revisions as described in the first two bullet points but have concern on the third bullet point to add a new objective for “Enhancement of beam correspondence during initial access and RRC_INACTIVE state”. We think UE beam correspondence can be well verified in the connected mode. There is no need and not practical to define new requirements for beam correspondence based on msg1 spherical coverage during initial access. If a UE can successfully enter the connected mode, that already implies the UE can pass the requirement for initial access. |
| Qualcomm | We support the amendment, especially considering the SDT motivation |
| MTK | We have concern on the new objective to be added. If the intention is to check UE’s beam correspondence for rough beam, we think it is already covered by existing RRM test cases, e.g., - A.7.3.2.1.2 Inter-frequency RRC Re-establishment in FR2 - Beam type: rough - AoA setup #3: 2 AoAs which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. Since RRC re-establishment is a mandatory test for FR2 standalone UE, we wonder what additional goal is to be achieved via this new objective. |
| Intel | We are ok with the first two updates (putting inter-band objective on hold and removing PA efficiency and Transceiver calibration from UL gap objectives). Regarding the new objective for beam correspondence, the LS R1-2106309 asks RAN4 whether there is a need to define the beam correspondence requirements for Small Data Transmission (Configured Grant SDT and/or Random Access SDT) in RRC_INACTIVE state. Further discussion in RAN4 is needed before considering this objective. Also, the discussion has already taken place in the previous meetings and there was no consensus to define initial access BC. |
| Nokia | Yes, we support this WID revision. |
| Samsung | New objective for beam correspondence enhancement was once proposed in previous RAN4 meeting as well as new objective for intra-band CA BW class extension to 1600MHz. We expressed our concern then that maybe only one new objective can be adopted due to RAN4 TU consideration. Now that CA BW class extension was already adopted so we think there is no much room left for beam correspondence enhancement in Rel-17 FR2 RF enhancement WI. |
| LG Electronics | We’re fine with 1st and 2nd bullet in revised WID. However, we have concern on 3rd bullet. The beam correspondence on initial access has been discussed in RAN many times and removed in WI scope. Also we are not sure whether beam correspondence in the initial access is really necessary. |
| Huawei, HiSilicon | For beam correspondence objective, we believe RAN4 requirement already include beam correspondence ability for initial access (idle state) or inactive state, it is verified under connected state and such measurement result can represent the UE’s BC ability under different state. Additionally, the LS R1-2106309 was sent to RAN4 and asks whether there is a need to define the beam correspondence requirements for inactive state. It still need RAN4 experts to discuss on the question in next WG meeting. |
| Sony | We support the revision in general. For the IBM within the same frequency group, an alternative to wait for the operator’s input could be that we wait until the CBM requirement is finalized. |
| vivo | We are fine with the updates in the first two bullets (inter-band CA and UL gaps). Regarding new requirements for initial access BC, we share similar views with Apple that UE beam correspondence has been well verified in the connected mode. |
| ZTE | We are fine with the proposed changes. |
| AT&T | We support the proposed WID revision. |
| OPPO | Ok with first and second changes, not ok with beam correspondence and this has been discussed for many times without new contents. |

## Initial Summary

| Topic | Status summary |
| --- | --- |
| Issue 1-1: IBM requirement To put this objective on hold until there is a operator request for band combination. - Study and if feasible define UE RF requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for (IBM) based on explicitly requested band combinations | Summary: Generally, this change is acceptable. Sony had a proposal to add that the IBM work continues IF the CBM requirement is finalized. Tentative Agreement. Modify WID as below - Study and if feasible define UE RF requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for (IBM) based on explicitly requested band combinations. (on hold until there is operator request or CBM requirements are finalized for one band combination) |
| Issue 1-2: PA calibration - Remove these objectives from UL gaps for self-calibration and monitoring. - PA efficiency and power consumption - Transceiver calibration due to temperature variation | Summary: Generally, this change is acceptable. Tentative Agreement. Modify WID and remove following objectives - PA efficiency and power consumption - Transceiver calibration due to temperature variation |
| Issue 1-3: Initial access BC Add a new objective - Enhancement of beam correspondence during initial access and RRC_INACTIVE state [RAN4 RF] - SSB-based without UL beam sweeping - For initial access, verification of beam correspondence based on msg1 spherical coverage (at least) | Summary: No clear majority view Recommendation for intermediate round: Continue discussion |

## Intermediate round

Please comment tentative agreements and further discuss initial acces BC in below table

| Topic | Comment |
| --- | --- |
| Issue 1-1: IBM requirement | Huawei, HiSilicon: we genenrally agree to keep the original objective on hold, but the new proposal ” or CBM requirements are finalized” is not acceptable. Because whether RF requirements is defined for such combinations are dependent of request while CBM/IBM is UE capability. Sony: We support the revised proposal. We think the revised proposal aligns with the moderator’s recommendation in the last RAN4 meeting. In addition, IBM/CBM is a UE capability which is up to UE implementation, it is unclear to us why it has to be dependent of request. Nokia: we support the tentative agreement and revised proposal. |
| Issue 1-2: PA calibration | Ericsson: we are fine to remove these objectives AT&T: We are OK with the tentative agreement. Verizon: We are fine to remove these objectives too! Intel: we are fine to remove Sony: We are fine with the revision. ZTE: We are fine with the tentative agreement. Nokia: We support the tentative agreement and removal of these objectives |
| Issue 1-3: Initial access BC | Ericsson: We support to include this objective. We do not agree that the RRM RRC re-establishment tests in anyway equivalent to BC for initial access. Such requirements are anyway needed for small data in inactive state (RAN1 LS to RAN4: R1-2106309). So it does not increase RAN4 work load. AT&T: We continue to support the new objective. Verizon: We support to include this objective Apple: We have concern to include this objective. It would simply increase the compliance test burden without gaining further test coverage for UE beam correspondence. Intel: We disagree to include this objective. RAN1 LS R1-2106309 asks RAN4 whether there is a need to define the beam correspondence requirements for SDT and not forces us to do it. RAN4 has not discussed this LS yet. Also, the expected workload is non-trivial and it will require TU extension. Huawei, HiSilicon: as comments provided in 1st round, we cannot agree to include ‘initial access BC’ into the WID. Beam correspondence requirement is used to specify UE’s RF ability on UL and DL correspondence, it can be well verified under connected state which is to save test complexity and the measurement result can well represent the ability for other RRC states. MTK: We disagree on introducing this objective. Besides what we mentioned in the initial round, we also have a question for clarification. If the intention is to test UE’s beam correspondence for rough beam, it can be done in many other procedures, e.g., IDLE mode re-selection, handover or PSCell addition. We are not 100% clear why this has only to do with initial access, which involve a lots of other UE actions (raster search, synchronization, SIB decoding) rather than just beam correspondence itself. Samsung: we share similar view as Intel. Besides RAN4 work load in Rel-17, RAN1 LS is asking for RAN4 question for SDT which does not mean new objectives to be added directly without RAN4 discussion on whether enhanced BC is needed. LG Electronics : We cannot support to include this new objective as mentioned in the initial round. Sony: We support to include this objective. ZTE: We support to include this objective. Nokia: We support to include this BC objective. We don’t believe that requirements and test cases in RRC Connected are sufficient to verify RRC_Inactive and Idle mode performance. |

| Topic | Status summary |
| --- | --- |
| Issue 1-1: IBM requirement Study and if feasible define UE RF requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for (IBM) based on explicitly requested band combinations. (on hold until there is operator request or CBM requirements are finalized for one band combination) | Summary: Generally companies agree to out his objective on hold. Sony proposed to add the yellow highlight criteria to re-enter the work. One company is against Sony’s addition. Moderator recommendation: Put objective on hold with Sony’s proposal taken into account. |
| Issue 1-2: PA calibration - Remove these objectives from UL gaps for self-calibration and monitoring. - PA efficiency and power consumption - Transceiver calibration due to temperature variation | Summary: Generally, this change is acceptable. Moderator recommendation: Modify WID and remove following objectives - PA efficiency and power consumption - Transceiver calibration due to temperature variation |
| Issue 1-3: Initial access BC Add a new objective - Enhancement of beam correspondence during initial access and RRC_INACTIVE state [RAN4 RF] - SSB-based without UL beam sweeping - For initial access, verification of beam correspondence based on msg1 spherical coverage (at least) | Summary: No clear majority view Moderator recommendation: Do not add Initial access BC objective. Further discuss the RAN1 LS in next RAN4 meeting. |

Final round

# Topic #2: RP-211394 and RP-211395

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| RP-211394 and RP-211395 | Huawei, HiSilicon | Add a new objective under UL gaps for self-calibration and monitoring. - Coherent uplink MIMO |

## Company views

Is the proposed new objective agreeable?

| Company | Comments |
| --- | --- |
| Apple | We are okay with the objective. |
| Intel | The objective is fine |
| Nokia | No Like discussed in the last RAN4 meeting this objective should not be added to the WID. Instead the study for phase I should continue and companies proposing UL gaps for coherent UL MIMO need to first show that this proposal will provide testable performance gains compared to the current requirements without UL gaps. RAN4 has already agreed that these studies can be done under the current WID without revisions. |
| Huawei, HiSilicon | As the proponent of this proposal, we support to add the objective. in the agreed WF in R4-2107857, it clearly states that • UL gap for coherent UL MIMO is within the scope of WI for FR2 enhancement. Response to Nokia: we add this objective into the WID which includes phase 1 study. |
| Sony | To our understanding, RAN4 has agreed this objective can be further discussed without explicitly adding a new objective in the WI in the last meeting. Therefore, it is not clear to us the necessity of this revision. |
| vivo | We are OK with the new objective. |
| OPPO | This has already been agreed in RAN4, and support to make it clear in the WID. |

## Initial Summary

| Topic | Status summary |
| --- | --- |
| Add a new objective under UL gaps for self-calibration and monitoring. - Coherent uplink MIMO | Summary: Generally, this change is acceptable. Tentative Agreement. Modify WID as proposed in RP-211394 and RP-211395 with an understanding that this objective is still in Phase 1. |

## Intermediate round

Please comment tentative Agreement. Modify WID as proposed in RP-211394 and RP-211395

| Topic | Comment |
| --- | --- |
| Add a new objective under UL gaps for self-calibration and monitoring. Coherent uplink MIMO | Apple: OK with the proposal in RP-211394 and RP-211395 and also the tentative agreement. Intel: Agree with proposal Huawei, HiSilicon: we support the tentative agreement. Samsung: support the tentative agreement Nokia: We can accept the tentative agreement although it was explicitly discussed in the last RAN4 meeting that no WID revisions are needed. |

| Topic | Status summary |
| --- | --- |
| Add a new objective under UL gaps for self-calibration and monitoring. - Coherent uplink MIMO | Summary: Generally, this change is acceptable. Moderator recommendation: Modify WID as proposed in RP-211394 and RP-211395 with an understanding that this objective is still in Phase 1. |

# Topic #3: RP-211460

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| RP-211460 | MediaTek Inc. | - Proposal 1: Plenary intervention is needed to resolve current situation on MRTD and MTTD for FR2 inter band CA with CBM in RAN4. - Proposal 2: If MRTD 260ns is not agreeable, remove CBM related objectives in the WID. |

## Company views

Is MRTD 260ns for CBM UE agreeable? If not is CBM related objectives removed from WID?

| Company | Comments |
| --- | --- |
| Ericsson | We do NOT agree with MRTD of 260 ns for CBM. We suggest to keep the current objective on CBM in the WID. In R4-2108037, “WF on RRM requirements for FR2 Inter-band DL CA and UL CA” was approved. It has 3 options and option 2 (3 us but with degradation after certain value) was new. Companies are investigating different options until August meeting. |
| Apple | We agree that MRTD should be less than half of the CP length for 120kHz SCS for CBM and 260ns defined for intra-band DL CA can also be specified for inter-band DL CA from the same frequency group. If there is no demand for inter-band CA from the same frequency group, we think the CBM objective can be removed from the WID. |
| Qualcomm | We are ok with restricting the MRTD for CBM UEs to 260 ns. Given the current deadlock on MRTD value, we can consider a value greater than 260 ns for CBM UEs only if an agreeable requirement framework to verify MRTD capability is developed. |
| MTK | This issue has been discussed for roughly 2 years. If compromise is possible, it should not have remained open at this moment. Also, since companies have been insisting their positions for 2 years, we do not see the difficulty to keep the same position for 1 or 2 more meetings. This issue has consumed non-trivial GTW online time for almost every meeting. As RAN4 workload remains high in this year, we suggest to serious consider whether to keep this objective in the WI scope. |
| Intel | We would like to further look at the possibility of convergence on any of the options in the next WG meeting. Our observation is that there are potential deployments which guarantee the TD within 260ns so that a CBM UE can work properly under. We also understand the typical MRTD value is always 3us though. The network deployment is transparent to UE. We believe it is fair to introduce a UE capability to indicate its MRTD handling ability so that the network understands it and configure CBM inter-band CA accordingly. Let’s try to converge in RAN4 for one more meeting if there is still no consensus, maybe the objective needs to be removed. |
| Nokia | Not acceptable We could agree to remove the CBM objective from interband CA and leave it only for intraband CA. We would also like to agree an example band combination for IBM UEs within the same frequency group to allow progress of the work before an operator request for band combination. |
| Samsung | MRTD is under discussion in RRM session. Even 260ns MRTD for CBM UE is not agreed, that does not mean CBM related objectives need to be removed since some performance degradation is allowed as indicated in the WID objectives. |
| LG Electronics | Our preference is 260ns. However, based on the agreed WF(R4-2108037), further study on 3 candidate options is necessary in RAN4 working group. |
| Huawei, HiSilicon | We do not agree to restrict the MRTD for CBM type to 260ns. From the latest RAN4 discussion status, options on MRTD for CBM are still open and allow further discussion in next WG meeting. To solve the issue, there are several solutions we used to propose in RAN4. One solution is to define ≥ 3us MRTD and allow performance degradation for CBM, or UE can report capability on MRTD requirement and gNB configure CA based on UE capability. |
| Sony | We do not agree to remove the CBM related objectives. We think it is a important for a UE implementation to support inter-band CA, which may support both the same frequency group and different frequency groups; therefore, CBM needs to be specified. |
| vivo | We are OK with 260ns for CBM UE. Regarding CBM inter-band CA issue, several proposals are under discussion in RAN4 which could potentially resolve this issue. We think further discussion in WG level would be good. If no consensus in Q3, removing this objective may be acceptable for us. |
| ZTE | We cannot accept a 260ns MRTD for CBM. Other solutions could be further investigated and more discussions are needed under the current objectives in RAN4 before concluding this issue. |
| AT&T | We do not agree with a restriction of 260ns for MRTD for CBM. There are options being pursued in RAN4 for convergence and we should allow RAN4 to further analyze these options. |
| NTT DOCOMO, INC. | Since we had a good progress in last RAN4 meeting about this problem, we would like to continue discussion at least until next WG meeting. If we cannot reach the agreement in August, we can agree with removing CBM related objective from WID. |
| OPPO | Ok with 260ns restriction of CBM, but we are not ok to remove CBM from the WID. Group discussion is needed. |

## Initial Summary

| Topic | Status summary |
| --- | --- |
| Proposal 1: Plenary intervention is needed to resolve current situation on MRTD and MTTD for FR2 inter band CA with CBM in RAN4. Proposal 2: If MRTD 260ns is not agreeable, remove CBM related objectives in the WID. | Summary: No clear majority view. Many companies felt that it is worth pursuing discussion in RAN4 at least one more meeting as some progress was seen in previous RAN4 meeting. Tentative agreement: Keep WID as it is and continue discussion in RAN4. |

## Intermediate round

Please comment tentative Agreement.

| Topic | Comment |
| --- | --- |
| CBM MRTD | Ericsson: we support the tentative agreement to keep the current objective on CBM in the WID. AT&T: We also support the tentative agreement. Verizon: We support tentative agreement and continue discussion in RAN4. Apple: Though we support 260ns as the MRTD requirement for CBM like for intra-band CA, we can also accept MRTD within the full CP length for inter-band CA based on CBM implementation. We are also fine with the tentative agreement to continue discussions in RAN4. Intel: Agree with proposal Huawei, HiSilicon: we support the tentative agreement. MTK: After checking the views from companies, we are fine to compromise to the tentative agreement. But we would suggest to add a deadline for this discussion, e.g., RAN4#100e. We want to avoid keeping wasting online GTW time every meeting. Samsung: support the tentative agreement. LG Electronics : We support the tentative agreement. Sony: We support this proposal. ZTE: Fine with the tentative agreement. Nokia. We support the tentative agreement. NTT DOCOMO, INC.: We support tentative agreement. |

| Topic | Status summary |
| --- | --- |
| Proposal 1: Plenary intervention is needed to resolve current situation on MRTD and MTTD for FR2 inter band CA with CBM in RAN4. Proposal 2: If MRTD 260ns is not agreeable, remove CBM related objectives in the WID. | Summary: No clear majority view. Many companies felt that it is worth pursuing discussion in RAN4 at least one more meeting as some progress was seen in previous RAN4 meeting. Moderator recommendation: Keep WID as it is and continue discussion in RAN4. |

# Final proposals/recommendations