3GPP TSG-RAN Meeting #93-e	RP-21xxxx

Electronic Meeting, September 13-17, 2021

Agenda item:			9.3.4.3

Source:	Moderator (China Telecom)

Title:	Moderator's summary for email discussion [93e-25-CRSIntfHandling]

Document for:	Discussion

Introduction

This document is the summary of email discussion [93e-25-CRSIntfHandling] on CRS interference handling for NR PDSCH in scenarios with overlapping spectrum for LTE and NR, and the discussion outcome (if any) will be reflected in the revised WID on “Rel-17 Further enhancement on NR demodulation performance”.

All the following 5 tdocs recommend to define NR PDSCH demodulation requirements for neighbouring cell LTE CRS-IM in Rel-17, and the main discussion point is whether network assistance signalling for CRS-IM is needed or not.

| Tdoc | Title | Source |
| --- | --- | --- |
| RP-211835 | Revised WID: Further enhancement on NR demodulation performance | China Telecom |
| RP-211950 | CRS interference handling in NR | Apple, MediaTek |
| RP-212199 | Views on LTE CRS interference handling for NR UE | Nokia, Nokia Shanghai Bell |
| RP-212226 | Views on Rel-17 CRS-IM requirements in scenarios with overlapping spectrum for LTE and NR | Intel Corporation |
| RP-212490 | LS on RAN4 evaluation for LTE CRS interference handling for NR UE (R4-2115741; to: RAN; cc: -; contact: China Telecom) | RAN4 |

Initial round

Open issues and companies views’ collection

Issue #1: Except the network assistance signalling part (which is discussed separately in Issue #2), any comments on the other parts of RAN4 recommendations in LS RP-212490?

RAN4 recommends to define NR PDSCH demodulation requirements for neighbouring cell LTE CRS-IM in scenarios with overlapping spectrum for LTE and NR in Rel-17:

•	Use LLR weighting as baseline reference receiver, and further discuss the feasibility of CRS-IC receiver taking into account the UE complexity and PDSCH processing time.

•	Synchronous network scenario is prioritized. The asynchronous network scenario will be discussed after RAN #93e meeting.

•	15 kHz SCS for NR is prioritized. The 30 kHz SCS scenario will be discussed after RAN #93e meeting.

•	RAN4 will further discuss the necessity of network assistance signaling and UE capability signaling during requirements definition phase.

| Company | Comment |
| --- | --- |

Issue #2: Whether to assume network assistance information for PDSCH CRS-IM?

- Option 1: Yes (Apple, MediaTek)

- Option 2: No (Nokia - for Rel-17)

- Option 3: Task RAN4 to further discuss the necessity of network assistance signaling during requirements definition phase. (Intel, China Telecom)

| Company | Comment |
| --- | --- |

Initial round summary

Intermediate round

Open issues and companies views’ collection

Intermediate round summary

Final round

Open issues and companies views’ collection

Final round summary

Final conclusions

- Annex: Contacts

Please provide a company contact that the email discussion moderator can contact if required.

| Company | Contact name and email |
| --- | --- |