3GPP TSG-RAN5 Meeting #92-e	R5-216076

# Electronic Meeting, 16 – 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0337 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FR2 DL 256QAM demodulation test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL256QAM_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the WP, new test case for FR2 DL 256QAM will be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add applicability of test requirements due to maximum achievable SNR in 7.1.1_1. Add new test case for FR2 DL 256QAM in 7.2.2.2.1. Add MU and TT for FR2 DL 256QAM test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This WI will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1_1 7.2.2.2.1 7.2.2.2.1_3(New) F.1.1.2 F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR 38.522 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-214532: r1: set applicability due to maximum achivable SNR to ‘TBD’ based on the up-to-date analysis add message exception ‘PDSCH-Config’ modify title for maessage exception tables |  |  |  |  |  |  |  |  |

<Start of change 1>

7.1.1_1	Applicability of test requirements due to maximum achievable SNR

Table 7.1.1_1-1 specifies the current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 7.1.1_1-1: Current assumption of maximum testable SNRBB under fading conditions

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | [24.0] | [20.8] | [17.8] |
| n258 mid | [24.0] | [20.8] | [17.8] |
| n259 mid | TBD | TBD | TBD |
| n260 mid | TBD | [7.3] | TBD |
| n261 mid | [24.0] | [20.8] | [17.8] |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2 for indirect farfield (IFF), PC3, Quiet Zone size ≤ 30 cm under fading conditions.

Table 7.1.1_1-2: Current assumption of maximum testable SNRBB

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | TBD | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | TBD | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | TBD | - | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | TBD | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | TBD | - | x |
|  | 2-3 | 100 | Yes | 15.7 | x | x | TBD | - | x |
|  | 2-4 | 100 | Yes | 15.8 | x | x | TBD | - | x |
|  | 2-5 | 100 | Yes | 16 | x | x | TBD | - | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | TBD | - | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | TBD | - | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22.0 | TBD | TBD | TBD | TBD | TBD |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | TBD | - | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | TBD | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | TBD | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | TBD | x | x |

<End of change 1>

<Start of change 2>

7.2.2.2.1	2Rx TDD FR2 PDSCH mapping Type A performance

7.2.2.2.1_0	Minimum conformance requirements

For PDSCH Type-A scheduling, the requirements are specified in Table 7.2.2.2.1_0-3, 7.2.2.2.1_0-4 and 7.2.2.2.1_0-5, with the addition of the parameters in Table 7.2.2.2.1_0-2 and the downlink physical channel setup according to Annex C.5.1. The purpose is to verify the performance of PDSCH Type-A scheduling.

The test purposes are specified in Table 7.2.2.2.1_0-1.

Table 7.2.2.2.1_0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-3, 1-4, 2-1, 2-2, 2-3, 2-4, 2-5, 2-6 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-2 |
| Verify the PDSCH mapping Type A enhanced performance requirement Type 1 under 2 receive antenna conditions and with 2 MIMO layers. | 3-1 |

Table 7.2.2.2.1_0-2: Test Parameters

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD |  |
| Active DL BWP index |  |  | 1 |  |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS (l0) |  | For Test 1-1 and 1-2: 3 for CSI-RS resource 1 and 3 7 for CSI-RS resource 2 and 4 |  |  |
|  | CSI-RS offset | Slots | For Test 1-2: 82 for CSI-RS resource 1 and 2 83 for CSI-RS resource 3 and 4 |  |  |  |
| PDCCH configuration | Number of PDCCH candidates and aggregation levels |  | 1/AL4 for Test 1-4 and 2-3 1/AL8 for other tests |  |  |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |  |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | WB for 1-1, 2 for other tests |
|  | Resource allocation type |  | Test 2-1: Type 1 with start RB = 30, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 2-1: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 for Test 1-1, 1-3, 1-4, 2-2, 2-4 10 for Test 2-1, 2-3, 2-5, 2-6, 3-1 16 for Test 1-2 |  |  |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | As defined in Annex A.1.3 |  |  |

Table 7.2.2.2.1_0-3: Minimum performance for Rank 1 (FRC)

| Test num. | Reference channel | Bandwidth (MHz)/Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-1.1TDD | 100/120 | QPSK, 0.30 | FR2.120-1. A | TDLC60-300 | 2x2 ULA Low | 70 | -0.4 |
| 1-2 | R.PDSCH.5-2.1 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2 ULA Low | 30 | 1.7 |
| 1-3 | R.PDSCH.5-3.1TDD | 100/120 | 64QAM, 0.46 | FR2.120-1 | TDLA30-300 | 2x2 XPL Med | 70 | 12.4 |
| 1-4 | R.PDSCH.5-10.1 TDD | 50/120 | 256QAM 0.67 | FR2.120-1 | TDLD30-75 | 2x2 ULA Low | 70 | 20.2 |

<Unchanged part skipped>

7.2.2.2.1_3	2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with 256QAM for SA and NSA (Rel-16 and foward)

7.2.2.2.1_3.1	Test Purpose

Verify the PDSCH mapping Type A normal performance with different channel models, MCSs and number of MIMO layers.

7.2.2.2.1_3.2	Test Applicability

This test applies to all types of NR UE release 16 and forward supporting 256QAM for PDSCH.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and NR 256QAM for PDSCH.

7.2.2.2.1_3.3	Test Description

Same test description as in clause 7.2.2.2.1_1.3 with following exception:

-	Table 7.2.2.2.1_3.4-1 instead of Table 7.2.2.2.1_1.4-1

7.2.2.2.1_3.3.1	Message contents

Message contents are according to TS 38.508-1 [6] clause 5.4.2 with the following exceptions:

7.2.2.2.1_3.3.1_1	Message exceptions for SA

Same as 7.2.2.2.1_1.3.3_1 with following exceptions:

Table 7.2.2.2.1_3.3.1_1-1: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| Sl80 | 7 |  | Test 1-4 |
| } |  |  |  |
| } |  |  |  |

Table 7.2.2.2.1_3.3.1_1-2: SearchSpace

| Derivation Path: TS 38.508-1 Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n1 | AL4 | Test 1-4 |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.2.2.2.1_3.3.1_1-3: PDSCH-Config

| Derivation Path: TS 38.508-1 Table 5.4.2.0-26 with condition DEMOD_FR2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table | qam256 |  | Test 1-4 |
| } |  |  |  |

Table 7.2.2.2.1_3.3.1_1-4: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 Table 5.4.2.0-25 with condition DEMOD_FR2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | 8 |  | Test 1-4 |
| } |  |  |  |

7.2.2.2.1_3.3.1_2	Message exceptions for NSA

Same as 7.2.2.2.1_3.3.1_1.

7.2.2.2.1_3.4	Test Requirements

Table 7.2.2.2.1.0-2 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A.3.2 for each throughput test shall meet or exceed the specified value in Table 7.2.2.2.1_3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 7.2.2.2.1_3.4-1: Test Requirement for Rank 1 (FRC) for 256QAM

| Test num. | Reference channel | Bandwidth (MHz)/Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 1-4 | R.PDSCH.5-10.1 TDD | 50/120 | 256QAM 0.67 | FR2.120-1 | TDLD30-75 | 2x2 ULA Low | 70 | 22.0 |

<End of change 2>

<Start of change 3>

F.2.1.2	Measurement of Demod Performance requirements

<Unchanged part skipped>

Table F.2.1.2-2: Maximum test system uncertainty for FR2 demodulation performance test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.2.2.2.1_1 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA | 2Tx, Rank 1: ± 1.82 dB for Doppler < 100 Hz ± 1.78 dB for Doppler ≥100 Hz 2Tx, Rank 2: ± 1.67 dB for Doppler < 100Hz ± 1.63 dB for Doppler ≥100 Hz | Overall system uncertainty for fading conditions comprises four quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time 5. Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator SNR Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2 ) + Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB AWGN flatness and signal flatness ±3.6 dB SNR uncertainty due to finite test time ±0.3 dB for doppler < 100Hz, otherwise 0 dB Impact on non-ideal isolation between branches for the wireless cable mode 0.60 dB for Rank1, 0.45 dB for Rank2 |
| 7.2.2.2.1_2 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with enhanced type 1 receiver for SA and NSA | 2Tx, Rank 2: ± 1.67 dB for Doppler < 100Hz ± 1.63 dB for Doppler ≥ 100Hz | Same as 7.2.2.2.1_1 |
| 7.2.2.2.1_3 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with 256QAM for SA and NSA (Rel-16 and foward) | 2Tx, Rank 1: ± 1.82 dB for Doppler < 100 Hz | Same as 7.2.2.2.1_1 |
| 7.3.2.2.1 2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA | 1Tx, rank1: ± 1.74 dB | Overall system uncertainty for fading conditions comprises four quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time 5. Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator SNR Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2 ) + Impact on non-ideal isolation between branches for the wireless cable mode gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for 1Tx, ±0.7 dB for 2Tx AWGN flatness and signal flatness ±3.6 dB SNR uncertainty due to finite test time ±0.4 dB Impact on non-ideal isolation between branches for the wireless cable mode 0.6 for Rank1 and 0.45 for rank2 |
| 7.3.2.2.2 2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA | 2Tx, rank1: ± 1.84 dB | Same as 7.3.2.2.1 |

<End of change 3>

<Start of change 4>

F.2.3.2	Measurement of Demod Performance requirements

The derivation of the test requirements for the test cases in section 7 is defined in Table F.2.3.2-1.

Table F.2.3.2-1: Derivation of Test Requirements (FR2 demodulation performance tests)

| Test | Minimum Requirement in TS 38.101-4 | Test Tolerance (TT) | Test Requirement in TS 38.521-4 |
| --- | --- | --- | --- |
| 7.2.2.2.1_1 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA | SNRs as specified | 2Tx, Rank 1: 1.8 dB 2Tx, Rank 2: 1.7 dB for doppler < 100Hz 1.6 dB otherwise | Formula: SNR + TT T-put limit unchanged |
| 7.2.2.2.1_2 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with enhanced type 1 receiver for SA and NSA | SNRs as specified | 2Tx, Rank 2: 1.7 dB for doppler < 100Hz 1.6 dB otherwise | Formula: SNR + TT T-put limit unchanged |
| 7.2.2.2.1_3 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with 256QAM for SA and NSA (Rel-16 and foward) | SNRs as specified | 2Tx, Rank 1: 1.8 dB | Formula: SNR + TT T-put limit unchanged |
| 7.3.2.2.1 2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA | SNRs as specified | 1Tx, rank1: 1.7 dB | Formula: SNR + TT T-put limit unchanged |
| 7.3.2.2.2 2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA | SNRs as specified | 2Tx, rank1: 1.8 dB | Formula: SNR + TT T-put limit unchanged |

<End of change 4>