3GPP TSG RAN Meeting #93-e									RP-211771

Electronic Meeting, Sep 13 - 17, 2021

Status Report to TSG

Agenda item:			9.3.3.4

| WI / SI Name | NR QoE management and optimizations for diverse services |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_QoE-Core |  |  |  |  |
| Unique ID | 860061 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211406 |  |  |  |  |
| Target Completion Date (indicate if changed) | Work Item: N/A | Core part: 03/2022 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 40% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Gen Cao |
|  | Company | China Unicom |
|  | Email | caogen@chinauincom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#115-e (Aug. 2021)

- Configuration architecture general aspect

- It is the RAN2 understanding the RRC ID, MeasConfigAppLayerId, is sufficient to identify the QoE configuration between UE and gNB.

	- RAN2 assumes that gNB keeps the mapping between MeasConfigAppLayerId and QoE Reference. The mapping is sent to the target gNB as part of QoE configuration and information at handover.

	- Confirm that RAN2 deprioritizes QoE measurement in RRC_IDLE/RRC_INACTIVE in Rel-17.

- Start and Stop

No contributions were treated in this RAN2 meeting.

- Support for Mobility

	- RAN2 assumes that all QoE mobility related agreements made by RAN2 are applicable at least to signalling based QoE.

	- Area scope parameter is not introduced in RRC procedures supporting QoE.

	- When the UE resumes the connection in a gNB supporting QoE, the target gNB should explicitly indicate which QoE measurement configurations should be kept by the UE during RRC resume procedure, e.g. in RRCResume message. The UE shall release all QoE measurement configurations not indicated by the gNB for restoration.

	- During the handover to target gNB which supports QoE, the target gNB decides which QoE configurations to keep and which to release during a handover, e.g. based on QoE configuration information received from the source gNB in Xn/Ng signalling (exact information is up to RAN3) including the RRC container.

	- The UE discards the reports received from application layer in case it has no associated QoE configuration configured.

	- In case the UE resumes the connection in a gNB not supporting QoE, the UE should release all QoE measurement configurations.

- Radio-related measurements and information for QoE

None

- RAN visible QoE

None

- Handling of incoming LSs

R2-2106938	LS on the mapping between service types and slice at application (R3-212904; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:SA4, CT1, SA5	Cc:RAN2, SA2

Noted

R2-2106945	LS on requirement for configuration changes of ongoing QMC sessions (R3-212953; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:SA4	Cc:SA5, RAN2

Noted

R2-2106949	LS on the area handling for QoE during mobility (R3-212976; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:RAN2, SA4	Cc:SA5

Noted

- Handling of running CRs

R2-2108108	Running RRC CR for QoE measurements	Ericsson	draftCR	Rel-17	38.331	16.5.0	B	NR_QoE-Core

[Post115-e][078][QoE] RRC running CR (Ericsson)

Scope: Progress the 38331 running CR. Update with agreements. Use Editors notes where appropriate.

Intended outcome: Endorsed draft CR.

Deadline: Short 2 (not for RP)

R2-2108209	38.300 running CR for introduction of QoE measurements in NR	Huawei, China Unicom, HiSilicon	draftCR	Rel-17	38.300	16.6.0	NR_QoE-Core

[Post115-e][079][QoE] Stage-2 running CR (Huawei, China Unicom)

Scope: Progress the 38300 running CR. Update with agreements. Use Editors notes where appropriate.

Intended outcome: Endorsed draft CR.

Deadline: Short 2 (not for RP)

- LSs out

R2-2109200	QoE Reference and maximum number of QoE configurations in RRC	RAN2 	LSout

[045] The LS out is approved

RAN2 send the following agreements on supporting QoE mobility to SA5, RAN3, and cc to SA4

RAN2 has discussed the configuration of QoE measurements in RRC_CONNECTED and have agreed to define a short RRC identifier, MeasConfigAppLayerId, to identify each QoE measurement configuration. The MeasConfigAppLayerId is sent together with each QoE report from the UE to the network. According to RAN2 understanding the short MeasConfigAppLayerId can be mapped to the QoE Reference in the gNB and forwarded to OAM together with the report. The mapping can be transferred to the target gNB as part of QoE measurement configuration at handover.

RAN2 is discussing the maximum number of simultaneous QoE measurement configurations for a UE. RAN2 is considering values 8, 16, 32 or 64 for maximum number of simultaneous QoE configurations in the UE.

Then RAN2 respectfully asks SA5 for feedback on:

- Not sending the QoE Reference outside the container in RRC signalling to a UE in RRC_CONNECTED

- The maximum number of simultaneous QoE configurations in the UE

2.2.2	Remaining Open issues

For RAN visible QoE:

- FFS for RAN-visible QoE configuration and report (depend on RAN3).

For Configuration:

- FFS whether pause resume will affect all configurations or whether pause resume can act selectively per configuration.

	- FFS if the RRC layer forwards the MeasConfigAppLayerId together with the QoE configuration to the application layer.

	- FFS how the indication that gNB indicate which QoE measurement configurations should be kept by the UE during RRC resume procedure looks like, e.g. granularity per QoE configuration or common for all QoE configurations.

For Mobility:

	- FFS whether the gNB needs to know the QoE configurations for which there are ongoing QoE sessions, e.g. to enable QoE configuration handling upon mobility (pending SA4 reply on the ongoing QoE measurement session continuity requirement).

For Start and stop:

- FFS for handling of QoE reports during RAN overload

2.3	RAN3

2.3.1	Agreements

RAN3#112-e (May 2021)

- QoE measurement Configuration and Reporting

	- Liaise SA5 on the support of (de)activation of NR QoE, including concerns on whether current Trace Function could support QoE mechanism, decoupling of deactivation, failure handling and QoE Reference

	- Introduce a new IE "QoE Reference" explicitly over interfaces at least for s-based, whether it can be applied to m-based and whether it is per service type or per slice depends on feedback from SA5

	- Introduce the following additional new IEs:

- a list of UE Application layer measurement configuration IE for each service type.

- inside each UE Application layer measurement configuration IE:

- Container.

- a numerated IE indicating service type (e.g., Streaming services, MTSI services, VR, MBMS, XR).

- Area scope (a list of cells/TA/TAI/PLMN).

- Support for Mobility

	- Include signaling based QoE measurement configuration in handover preparation messages i.e. in XnAP: HANDOVER REQUEST, NGAP: HANDOVER REQUEST.

	- Signaling based QoE measurement configuration is stored in NG-RAN when UE enters RRC_INACTIVE and is propagated to new serving NG-RAN using Retrieve UE context procedure when UE resumes RRC connection in another NG-RAN i.e. include signaling based QoE configuration in RETRIEVE UE CONTEXT RESPONSE in XnAP.

	- Include multiple sets of signaling-based QoE measurements configuration in Xn/NG: HANDOVER REQUEST and Xn: RETRIEVE UE CONTEXT RESPONSE.

	- Management based QoE should not override an existing signaling based QoE configuration.

	- Option 1 is agreed by RAN3 on area handling for QoE i.e. the network is responsible for keeping track of whether the UE is inside or outside the area and the network configures/releases configuration accordingly. Send LS to RAN2 and SA4 informing RAN3 agreements.

	- Upon the reception of QoE configuration on a non-supporting node, the target node should not set up any QoE session with MCE and should not initiate any QoE measurement collection.

	- Liaise SA4 to check if QoE requirement for ongoing session continuity is also applicable for NR QMC and in case QoE configuration release is received during an ongoing session.

- Per slice QoE measurement

None

- RAN visible QoE

	- The service types supported in the Rel17 RAN-visible QoE framework are DASH streaming and VR.

	- WA: The following metrics, pertaining to DASH streaming and VR services, should be supported in the Rel17 RVQOE framework:

- Buffer Level

- Average Throughput

- Playout Delay

	- LS to other WGs, based on the resolution of the WA above, is expected at the next RAN3 meeting.

	- The following is supported within the RVQOE framework:

- RAN-visible QoE metrics: a subset of legacy QoE metrics data collected from UE, which are useful for RAN.

- RAN-visible QoE values: a set of values derived from QoE metrics data through a model/function defined in collaboration with SA4 (pending SA4).

	- WA: The RAN generates the RVQOE measurement configuration

	- The UE is assumed to indicate to the RAN its capability with respect to providing RVQOE metrics (LS to RAN2 seems needed).

	- WA: RVQOE collection can be configured only if QoE measurements are configured for the same service type.

	- Together with the QoE measurements, the RVQOE is supported in the following aspects:

- Activation, and deactivation procedures

- WA: Multiple simultaneous QoE measurements

	- WA: the ID used to identify QoE measurements is reused for identifying the RVQOE measurements.

	- WA: the RVQOE report is provided inside a dedicated IE, outside the QoE report container.

	- Whether transfer of RVQOE configuration to the target be supported will be discussed after the basic solution for mobility has been defined.

	- Whether the RVQOE report can be signaled from the target to the source at handover will be discussed after the basic solution for mobility has been defined.

	- WA: gNB-CU may signal RVQoE report to gNB-DU over F1

- Alignment of Radio-Related Measurement and QoE Measurements

	- Immediate MDT is taken as baseline for the collection of Radio-related Measurements to assist QoE analysis.

	- Existing measurements specified for immediate MDT can be used for Radio-related measurements for QoE analysis.

	- New radio-related measurements, if any, should be defined in the SON/MDT WI.

	- Radio-related measurement and QoE measurement can be configured simultaneously by OAM for the alignment.

	- OAM (e.g. TCE or MCE) is responsible for correlation.

- LSs out

	- LS on how to support the (de)activation and failure handling of NR QMC (to: SA5; cc: SA2)

	- LS on the mapping between service types and slice at application (to: SA4,CT1,SA5; cc: RAN2, SA2)

	- LS on the area handling for QoE during mobility (to: RAN2,SA4; cc: SA5)

	- LS on the QoE requirement for ongoing session continuity (to: SA4; cc: SA5,RAN2)

RAN3#113-e (Aug 2021)

- QoE measurement Configuration and Reporting

	- Wait the reply LS from SA5, before we make decision on whether to reuse Trace or not.

	- In NGAP, at least INITIAL CONTEXT SETUP REQUEST, along with HANDOVER REQUEST should be enhanced for NR QoE.

	- Agree on supported service types for NR QoE management in Rel-17: Streaming services, MTSI service, VR.

	- Wait for SA5’s feedback: 1) introduction of QoE Reference for each service type of QoE measurement (i.e. support multi service QoE measurements in one message); 2) a separate and single MCE address is used for the QoE measurements of all service type in one message.

	- Slice scope is a list of S-NSSAI.

	- To include slice scope outside the configuration container over NG.

	- Slice related identifier should be included in the QoE measurement report from UE.

	- No additional requirements on QoE measurement to support roaming UEs.

- Support for Mobility

	- Include signaling based QoE measurement configuration in handover preparation messages i.e. in XnAP: HANDOVER REQUEST, NGAP: at least HANDOVER REQUEST.

	- Signalling based QoE can override an existing management based QoE configuration.

- Per slice QoE measurement

- RAN visible QoE

	- RAN visible QoE measurement activation, UE AS indicates to UE APP that RAN visible QoE measurement has been triggered, potentially with RAN visible QoE metrics needed to be collected at UE APP as requested by RAN.

	- RAN visible QoE measurement deactivation, UE AS indicates to UE APP that RAN visible QoE measurement has been terminated, and then UE APP stops to provide RVQoE measurement results to UE AS.

	- Turn into an agreement the WA that the RAN generates the RVQoE measurement configuration.

	- Turn into an agreement the WA that the ID used to identify QoE measurements is reused for identifying the RVQoE measurements.

	- Turn into an agreement the WA stating that RVQoE collection can be configured only if QoE measurements are configured for the same service type.

	- Turn into an agreement the WA stating that multiple simultaneous RVQoE measurements are supported.

	- The RVQoE configuration can be configured flexibly (i.e., it is not fixed).

	- The RVQoE configuration sent to UE should contain:

•	Metrics to be reported, as a mandatory IE.

	- The decision about the final list is expected at the next meeting.

	- Turn into an agreement the WA stating that the RVQoE report is provided inside a dedicated IE, outside the QoE report container.

	- The RAN decides whether RVQOE measurement collection and reporting is activated.

	- The gNB-CU may signal RVQoE report to gNB-DU over F1.

- Alignment of Radio-Related Measurement and QoE Measurements

	- Postpone the discussion on alignment for the case that MDT is configured before QoE configuration till clarification is received from SA5 on QoE activation/deactivation procedure (i.e., whether to reuse trace function for QoE and if multiple trace sessions can be supported).

	- An indicator is required in the QoE configuration to NG-RAN to inform whether it should perform MDT and QoE measurements in a time-aligned manner. FFS whether an explicit or implicit indicator.

	- WA: NG-RAN should include Trace Reference and Trace Recording Session Reference in the QoE report sent to MCE

	- WA: NG-RAN should NOT include the Trace Reference and Trace Recording Session Reference in the QoE configuration sent to UE

	- NG-RAN can include session start and session end time stamp information related to MDT and QoE reports autonomously (e.g., using the same clock for MDT and QoE )to assist the correlation entity.

	- QoE and related MDT report can be sent to the same collection entity.

- LSs out

	- LS on RAN3 agreements for NR QoE (to: RAN2, SA4, SA5)

2.3.2	Remaining Open issues

	- FFS whether "Measurement Collection Entity IP Address" is per service type or per "QoE Reference" depends on feedback from SA5

	- FFS whether slice info should be signalled as an explicit IE in the configuration message and in the report message over radio interface.

	- FFS on whether include signaling based QoE measurement configuration in NGAP HANDOVER REQUIRED.

	- FFS on whether include multiple sets of signaling-based QoE measurements configuration in NGAP HANDOVER REQUIRED.

	- FFS on the definition of Slice related identifier, e.g., Slice ID, PDU session ID or DRB ID.

	- FFS whether to configure the prioritization mechanism for the UE to report the pending QoE reports

	- FFS: if there is a need to send the pause/resume indication to the MCE.

	- FFS on source NG-RAN node to include s-based QMC configuration in NGAP HANDOVER REQUIRED.

	- For DASH streaming and VR services, RVQOE parameters: Play List (FFS) and additional metrics are FFS; detailed descriptions are FFS.

	- FFS on the support for RVQOE in other aspects (e.g. mobility, alignment with radio-related measurements).

	- The RVQoE configuration sent to UE should contain: Sample percentage (FFS), Start Time (FFS), Duration (FFS), Reporting Interval for periodic case (FFS), Triggering Event (FFS), DRB information (or QoS flow information), to be reported (FFS).

	- FFS: RVQoE and legacy QOE can be reported separately.

	- FFS on the RVQoE report can be signalled from the target to the source node after a successful handover.

	- FFS whether PDU session information should and can be included in the RVQoE report.

	- FFS on the RVQoE configuration is propagated from the source to target node upon mobility in RRC_CONNECTED and during context retrieval upon resumption from RRC_INACTIVE. The target/new RAN node may assemble a different RVQoE configuration.

	- QoE reference and Trace reference should be considered for correlation, how to correlate and whether other information is needed are FFS.

	- FFS whether to support the scenario that the MDT measurements are used only for QoE analysis.

	- FFS on the approach for aligning the MDT and QoE measurements i.e., whether to use a network based solution (e.g. OAM should activate/deactivate appropriately) or a UE assisted solution (e.g. UE indicates start/stop time of QoE, UE keeps MDT config pending at RRC till session starts)

	- FFS whether radio related information should be included.

	- FFS whether to include QoE reference in MDT configuration sent to NG-RAN

	- FFS whether to include the QoE reference in MDT configuration sent to UE

	- FFS whether to include QoE reference in MDT report sent to TCE

	- FFS whether and how to achieve alignment in case QoE reporting is paused

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SA4

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#115-e

- R2-2106938	LS on the mapping between service types and slice at application (R3-212904; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:SA4, CT1, SA5	Cc:RAN2, SA2

- R2-2106945	LS on requirement for configuration changes of ongoing QMC sessions (R3-212953; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:SA4	Cc:SA5, RAN2

- R2-2106949	LS on the area handling for QoE during mobility (R3-212976; contact: Qualcomm)	RAN3	LS in	Rel-17	NR_QoE	To:RAN2, SA4	Cc:SA5

- R2-2108108	Running RRC CR for QoE measurements	Ericsson	draftCR	Rel-17	38.331	16.5.0	B	NR_QoE-Core

- R2-2108209	38.300 running CR for introduction of QoE measurements in NR	Huawei, China Unicom, HiSilicon	draftCR	Rel-17	38.300	16.6.0	NR_QoE-Core

- R2-2107099	General aspects in QoE	Samsung	discussion	Rel-17

- R2-2107380	Discussion on NR QoE configuration	CATT	discussion	NR_QoE-Core

- R2-2107396	Further discussion on QoE measurement collection in NR	OPPO	discussion	Rel-17	NR_QoE-Core

- R2-2107513	QoE handling in RAN	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core	R2-2105479

- R2-2107816	Left issues for QoE configuration and reporting	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2108109	Configuration and reporting of QoE measurements	Ericsson	discussion	Rel-17	NR_QoE-Core

- R2-2108197	Discussion on QoE measurement and configuration	China Unicom, China Southern Power Grid	discussion	Rel-17	NR_QoE-Core

- R2-2108206	Discussion on QoE measurement configuration and reporting	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2108207	QoE handling during UE mobility	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2108227	Discussion on NR QoE configuration	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2108514	More considerations on configuration and reporting	CMCC	discussion	Rel-17

- R2-2108594	Discussion on QoE measurement configuration	vivo	discussion	Rel-17	NR_QoE-Core

- R2-2109036	[Pre115-e][008][QoE] Summary Support for Mobility	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2107100	Pause and resume in QoE	Samsung	discussion	Rel-17

- R2-2107101	Storing QoE reports in AS at pause	Samsung	discussion	Rel-17

- R2-2107381	Activation and deactivation for QoE collection	CATT	discussion	NR_QoE-Core

- R2-2107382	Discussion on QoE collection start and stop	CATT	discussion	NR_QoE-Core

- R2-2107397	Discussion on QoE measurement pausing and resuming	OPPO	discussion	Rel-17	NR_QoE-Core

- R2-2107514	RAN control on QoE reporting	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core

- R2-2107515	QoE pausing	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_QoE-Core	R2-2105920

- R2-2107615	Pause/Resume functionality	Apple	discussion	Rel-17	DUMMY

- R2-2107817	Left issues for QoE pause and resume procedure	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2107852	Discussion on the partial QoE reporting and buffering at RAN overload	ITRI	discussion	NR_QoE-Core

- R2-2107882	Stop and start for QoE measurement reporting	LG Electronics Inc.	discussion	Rel-17

- R2-2108213	Discussion on pause and resume mechanism	China Unicom, China Southern Power Grid	discussion	Rel-17	NR_QoE-Core

- R2-2108226	Discussion on NR QoE start and stop	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2108515	More considerations on start and stop	CMCC	discussion	Rel-17

- R2-2107818	Support of RAN visible QoE	Qualcomm Incorporated	discussion	NR_QoE-Core

- R2-2108110	Mobility Support for NR QoE Management	Ericsson	discussion	Rel-17	NR_QoE-Core

- R2-2108111	[Draft] Support for Session Start and Session End Indication	Ericsson	LS out	Rel-17	NR_QoE-Core	To:CT1

- R2-2108208	Initial thoughts on non-RAN2 led objectives	Huawei, HiSilicon	discussion	Rel-17	NR_QoE-Core

- R2-2108228	Discussion on NR QoEcontinuity in handover	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2108595	Discussion on QoE continuity during mobility	vivo	discussion	Rel-17	NR_QoE-Core

- R2-2109038	[Pre115-e][007][QoE] Summary 8.14.2.1 Excluding Mobility	Ericsson 	discussion	Rel-17	NR_QoE-Core

- R2-2109200	QoE Reference and maximum number of QoE configurations in RRC	RAN2 	LSout

- R2-2109105	Report of offline: [AT115-e][046][QoE] Mobility (Huawei)		Huawei

RAN3#112-e

- R3-211732	NR QoE configuration and reporting

- R3-211733	(TP for 38.413 and 38.423) NR QoE configuration and reporting

- R3-211734	Per slice QoE measurements

- R3-211735	QoE measurement collection and reporting continuity in mobility scenarios

- R3-211736	RAN visible QoE

- R3-211737	Alignment of Radio-Related Measurement and QoE Measurements

- R3-211835	Discussion on NR QoE configuration procedures

- R3-211836	Discussion on NR QoE configuration details

- R3-211837	Discussion on per-slice QoE measurement

- R3-211838	Discussion on Measurement Collection and Continuity in Mobility

- R3-211839	Discussion on relevant set of RAN-visible QoE parameters

- R3-211840	Discussion on RAN visible QoE configuration and reporting

- R3-211841	Discussion on Alignment of MDT and QoE Measurements

- R3-211978	RAN3 impacts for supporting QoE Measurement Collection in NR

- R3-211979	Support of NR QoE Measurement Collection

- R3-211980	Support of NR QoE Measurement Collection

- R3-211981	Discussion on RAN visible QoE

- R3-211982	Discussion on the alignment of Radio-Related Measurement and QoE Measurement

- R3-211985	Procedures for Configuration, Activation and Deactivation of QMC

- R3-211986	QoE Configuration and Reporting

- R3-211987	paper 3

- R3-211988	QoE Mobility Support

- R3-211989	RAN-visible QoE Services and Metrics

- R3-211990	RAN-visible QoE - Configuration and Reporting

- R3-211991	The Alignment of Radio-Related Measurements and QoE Measurements

- R3-212322	Baseline and stage 2 aspects for NR QoE

- R3-212323	Introduction of QoE Measurement Collection for NR

- R3-212324	Principles for configuration of NR QoE measurements

- R3-212325	Analysis of QoE metrics for use by the NG-RAN

- R3-212326	On the alignment of QoE measurements and MDT measurements

- R3-212380	paper 33

- R3-212381	CR TS 38.413 QoE Configuration and Reporting - Signalling Design

- R3-212394	Workplan for Rel-17 NR QoE in RAN3

- R3-212434	Consideration on NR QoE activation procedure

- R3-212435	Discussion on NR QoE Configuration

- R3-212440	 (TP for TS 38.300) Introduce NR QoE

- R3-212441	Discussion on configuration details in NR standalone mode

- R3-212442	 (TP for TS 38.413) NR QOE configuration

- R3-212443	 (TP for TS 38.423) NR QOE configuration

- R3-212445	Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-212448	Further consideration on RAN visible QoE

- R3-212449	Alignment of MDT and QoE Measurements

- R3-212452	(TP for 38.401) Alignment of MDT and QoE Measurements

- R3-212453	(TP for 38.473) Alignment of MDT and QoE Measurements

- R3-212455	(TP for 38.463) Alignment of MDT and QoE Measurements

- R3-212496	Alignment of radio related measurement and QoE measurement

- R3-212497	RAN visible QoE metrics

- R3-212498	Per-slice QoE measurement

- R3-212515	Further analysis on spec impacts of the potential solutions to QoE visibility

- R3-212516	[Draft] LS on QoE visibility at RAN

- R3-212517	Further analysis on spec impacts of the potential solutions to RAN assitsted measurement

- R3-212518	Further analysis on spec impacts of the potential solutions to QoE measurement per slice

- R3-212519	Further analysis on spec impacts on configuration and reporting

- R3-212520	CR to 38.413 on Introduction of QoE measurement

- R3-212521	Further analysis on spec impacts on RAN intervention of QoE measurement

- R3-212591	On per-slice QoE measurement

- R3-212970     LS on how to support the (de)activation and failure handling of NR QMC

- R3-212904     LS on the mapping between service types and slice at application

- R3-212976     LS on the area handling for QoE during mobility

- R3-212953     LS on the QoE requirement for ongoing session continuity

RAN3#113-e

- R3-212953     LS on the QoE requirement for ongoing session continuity

- R3-213316	CR TS 38.423 Mobility Support for NR QoE Measurement Collection

- R3-213317	QoE Configuration and Reporting

- R3-213318	CR TS 38.413 QoE Configuration and Reporting - Signalling Design

- R3-213319	QoE Mobility Support

- R3-213320	RAN-visible QoE Metrics

- R3-213321	RAN-visible QoE Configuration and Reporting

- R3-213322	The Alignment of Radio-Related Measurements and QoE Measurements

- R3-213489	QoE Configuration and Reporting

- R3-213490	QoE measurement in mobility scenarios

- R3-213491	RAN visible QoE configuration and reporting

- R3-213492	Updated Workplan for Rel-17 NR QoE

- R3-213493	38300 running CR for NR QoE_

- R3-213654	QoE configuration and reporting

- R3-213655	QoE measurement collection and reporting continuity in mobility scenarios

- R3-213656	Support for RAN Visible QoE

- R3-213657	Alignment of Radio-Related Measurement and QoE Measurements

- R3-213681	Discussion on stage 2 description for NR QMC

- R3-213682	Introduction of QoE Measurement Collection for NR

- R3-213683	Choices for configuration of NR QoE measurements

- R3-213684	Open issues on mobility

- R3-213685	Analysis of metrics for RAN visible QoE

- R3-213686	Remaining open issues for alignment of radio-related measurements and QoE measurements

- R3-213791	Discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-213944	Discussion on NR QoE configuration procedures

- R3-213945	Introduction of NR QoE measurements on Xn interface

- R3-213946	Discussion on NR QoE configuration details

- R3-213947	Discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-213948	Discussion on RAN visible QoE configuration and reporting

- R3-213949	[Draft]LS on the configuration and report of the RAN-visible QoE

- R3-213950	Discussion on Alignment of MDT and QoE Measurements

- R3-213964	Further discussion on QoE Measurement Collection in NR

- R3-213965	(CR for TS 38.413) Support of NR QoE

- R3-213966	(CR for TS38.423) Support of NR QoE

- R3-213967	Discussion on RAN visible QoE

- R3-213968	(CR for TS38.473) Support of QoE information transfer

- R3-213969	Discussion on the alignment of Radio-Related Measurement and QoE Measurement

- R3-214040	Further consideration on NR QoE (de)activation procedure

- R3-214041	(TP for TS 38.300) Introduce NR QoE

- R3-214042	(TP for TS 38.413) NR QOE configuration

- R3-214043	(TP for TS 38.423) NR QOE configuration

- R3-214044	Discussion on NR QOE configuration

- R3-214045	Discussion on Measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-214046	Further consideration on RAN visible QoE

- R3-214047	[draft] LS on RAN visible QoE

- R3-214048	Discussion on alignment of MDT and QoE Measurements

- R3-214049	(TP for 38.401) Alignment of MDT and QoE Measurements

- R3-214050	(TP for E1/F1) Alignment of MDT and QoE Measurements

- R3-214070	Further discussions on configuration and reporting of QoE measurements

- R3-214071	CR to 38.413 on Introduction of QoE measurement

- R3-214072	Further discussions on overland handling

- R3-214073	Further discussions on per-slice QoE measurement

- R3-214074	Further discussions on measurement Collection and Continuity in Intra-System Intra-RAT Mobility

- R3-214075	Further discussions on RAN-visible QoE

- R3-214076	Draft LS on RAN-visible QoE conclusions

- R3-214077	Further discussions on Radio-Related Measurement and QoE Measurements

- R3-214108	Further discussions on alignment of radio related measurement and QoE measurement

- R3-214109	Further discussions on RAN visible QoE

- R3-214110	Further discussions on per-slice QoE measurement

- R3-214131	On the reception of QoE configuration in a non-supporting node

- R3-214194	CB: # QoE1_Workplan - Summary of email discussion

- R3-214195	CB: # QoE2_ Stage2 - Summary of email discussion

- R3-214196	CB: # QoE3_Configuration_Report - Summary of email discussion

- R3-214197	CB: # QoE4_Mobility - Summary of email discussion

- R3-214198	CB: # QoE5_RANVisible - Summary of email discussion

- R3-214199	CB: # QoE6_MDTAlignment - Summary of email discussion