3GPP TSG RAN#93e																				RP-212250

Electronic Meeting, 13-17 September, 2021

Source:	CATT

Title:	On Rel-18 Expanded and improved Positioning

Agenda item:	9.0.2

Document for:	Discussion and Decision

# Introduction

3GPP has specified multiple NR location technologies in Rel-16 and Rel-17 to support positioning use cases and applications. To support higher accuracy location requirements for 5G location service applications, Rel-17 has defined the target positioning requirements of <1m (90%) for commercial use cases and < 0.2m (90%) for IIoT use cases with end-to-end latency <100ms [2], which is much better positioning performance than Rel-16. However, there is still a need for support expanded and improved positioning in Rel-18.

In the pre-RAN#93 email discussion [RAN93e-R18Prep-10], the following topics areas are identified in the final summary for Rel-18 Expanded and improved Positioning [1]:

- Study/specify solutions (including reference signals, measurements, procedures, etc) for sidelink positioning considering the following: [RAN1, RAN2]

  - To include ranging (i.e. relative positioning) and absolute positioning

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

Note: The next phase of discussion will include selection of use cases to be used for evaluation and design. This does not restrict use of the solutions for other use cases.

  - Spectrum: ITS, licensed

  - Coordination with SA2 as required (e.g. architecture aspects)

- Improved accuracy, integrity, and power efficiency:

  - Study/Specify solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1, Coordination with SA2 as required]

  - Study/Specify solutions for accuracy improvement based on PRS/SRS bandwidth aggregation and NR carrier phase measurements [RAN1, RAN4, RAN2]

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, identify potential enhancements to help address any limitations [RAN1, RAN2]

- Define positioning support for RedCap UEs, considering the following:

  - Evaluate performance of existing positioning procedures and measurements with RedCap UEs[RAN1, RAN4]

  - Based on the evaluation, identify potential enhancements to help address possible limitations associated with RedCap UEs [RAN1, RAN2]

  - Define performance requirements for positioning by RedCap UEs [RAN4]

In this contribution, we further provide our views on the above topics areas and Rel-18 expanded and improved positioning.

# Sidelink positioning

For sidelink positioning, the following moderator’s proposal is included in the final summary[1]:

- Study/specify solutions (including reference signals, measurements, procedures, etc) for sidelink positioning considering the following: [RAN1, RAN2]

  - To include ranging (i.e. relative positioning) and absolute positioning

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

Note: The next phase of discussion will include the selection of use cases to be used for evaluation and design. This does not restrict the use of the solutions for other use cases.

  - Spectrum: ITS, licensed

  - Coordination with SA2 as required (e.g. architecture aspects)

We share a similar view as the majority of companies that there is a need to include the ranging. However, in our view, ranging is not the same as relative positioning. Clearly, when the relative position of two entities is known, the ranging information (both distance and angle) between them can also be known. However, if only having one of the ranging information (distance or angle) between two entities does not necessarily mean the relative positioning between them is known. In our view, each of the ranging information (distance or angle) can be obtained more directly from the positioning measurements without going through the step for positioning calculation. For example, the relative angle between a UE and a TRP may be obtained from UL-AoA measurement. The distance between a UE and a TRP can be obtained by the UE/gNB Rx-Tx time difference measurements between them. Thus, we may want to distinguish the ranging and relative positioning in the WID.

Proposal 1: Suggest making the following change to the moderator’s proposal for the SL positioning:

  - To include ranging, (i.e. relative positioning) and absolute positioning

In addition to reference signals, measurements and procedures, which are mentioned in the main bullet of the above moderator’s proposal for sidelink positioning, we may also need to consider, more specifically, the techniques for the SL positioning. For example,  we may want to specifically include both UE-assisted sidelink positioning, where the sidelink signals and measurements from UE are used for positioning, but the positioning solution is still calculated by the network; and UE-based slidelink positioning, where the positioning solution is calculated by the UE. Also, the hybrid positioning based on sidelink measurements and Un interface measurements should also be supported in Rel-18. Furthermore, sidelink-based cooperative positioning method may be used to improve the positioning accuracy and reliability through the information exchange between the gNBs, UE and LMF. The method may be especially useful for the situation where a UE is not able to receive the DL signals from any TRP, but need to determine its absolute position based on the SL signals from neighbouring UEs that can receive the DL signals from TRPs.

Proposal 2: Suggest the following change to the moderator’s proposal for the SL positioning:

- Study and specify the solutions for UE-based and UE-assisted cooperative sidelink positioning by integrating SL measurements with existing DL, UL, DL+UL measurements [RAN1, RAN2, RAN3]

# Improved accuracy, integrity, and power efficiency

## Solutions for accuracy improvement

For accuracy improvement, the following moderator’s proposal is included in the final summary:

- Study/Specify solutions for accuracy improvement based on PRS/SRS bandwidth aggregation and NR carrier phase measurements [RAN1, RAN4, RAN2]

We are supportive of the above proposal, especially the use of the NR carrier phase measurements for accuracy improvement.  GNSS carrier phase measurements have been very successfully used for supporting cm-level accuracy positioning for outdoor environments. In recent years, carrier phase measurements obtained from wireless OFDM communication systems are also proposed (e.g.,[2]). In comparison with GNSS carrier-phase positioning, the implementation of NR carrier phase positioning would have many salient advantages, including [3][4]:

- NR carrier phase measurements can be obtained directly from Rel-16 NR reference signals (e.g., PRS/SRS)

- The power of NR reference signals (e.g., PRS/SRS) in a subcarrier can be much stronger than the received power of GNSS signal in whole signal bandwidth

- The transmission of DL PRS/UL SRS signals can be flexibly configured

- Much lower implementation complexity since TRPs are stationary

- No need to consider the ionospheric and tropospheric errors

Similar to GNSS carrier phase positioning, some technical challenges need to be carefully considered for NR carrier phase positioning, such as the impact of the network time/frequency synchronization errors and multipath errors. In Rel-17 ePOS WI,  RAN1 has reached an agreement to introduce the positioning reference units (PRUs) for NR positioning enhancement. PRUs can be further extended to support double differential positioning techniques, which are commonly used to eliminate the impact of the time/frequency errors. How to mitigate the impact of the multipath on NR carrier phase positioning may also need to be investigated in Rel-18.

Proposal 3: Suggest the following change to the moderator’s proposal for the accuracy improvement with carrier phase measurements:

- Study and specify the solutions for the mitigation of the multipath and the network time/frequency synchronization errors for NR carrier phase positioning [RAN1, RAN2, RAN3]

Improving positioning accuracy based on PRS/SRS bandwidth aggregation was discussed in Rel-17 ePOS SI. Whether it is beneficial to use PRS/SRS bandwidth aggregation for improving positioning accuracy is tightly related to the timing alignment error and phase continuity between different carrier components. For example, we may only consider the PRS/SRS bandwidth aggregation for intra-band contiguous aggregation. Thus, we propose to limit the scope to intra-band carrier aggregation, and include the feasibility study of carrier aggregation positioning under the impact of timing alignment error, phase continuity, etc.

Proposal 3: Suggest the following change to the moderator’s proposal for the accuracy improvement with PRS/SRS bandwidth aggregation:

- Study/Specify solutions for accuracy improvement based on intra-band PRS/SRS bandwidth aggregation [RAN1, RAN4, RAN2]

  - including the feasibility study of PRS/SRS bandwidth aggregation under the impact of timing alignment error, phase continuity, etc.

## Integrity for RAT dependent positioning

RAT-dependent positioning integrity was down scoped from Rel-17 SI. In our view, it is important to further work on the RAT-dependent positioning integrity in Rel-18 based on the work of the GNSS positioning integrity completed in Rel-17. In our view, most of the accomplishments made in Rel-17 for GNSS positioning integrity can be used for RAT-dependent positioning integrity, including the signalling and procedures, the KPI definitions. The main work in Rel-18 for RAT-dependent positioning integrity may be the identification of the error sources, threat models, occurrence rates and failure modes requiring RAT-dependent positioning integrity.

Proposal 4: Suggest the following change to the moderator’s proposal for RAT-dependent positioning integrity:

- Including the identification of the error sources, threat models, occurrence rates and failure modes for RAT-dependent positioning integrity

## Low power high accuracy positioning (LPHAP)

Low power high accuracy positioning is identified as an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power high accuracy positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/synchronization, and a difficult to predict factor to take additional losses through e.g. security, power management, microcontroller, and self-discharge of batteries into account in TS 22.104 [6]. Examples of target applications for low power high accuracy positioning are asset tracking in process automation, tracking of vehicles, and tool tracking.

We are supportive of the moderator’s proposal for LPHAP in the final summary in [1].

Proposal 5: Support the moderator’s proposal for LPHAP.

# RedCap positioning

For supporting RedCap UE positioning, we envision there are no significant changes in existing positioning architecture and solutions. There may have some impacts on signalling, protocol, message, and procedures. The main issues that may need to be considered include the impacts of the reduced complexity on NR positioning coverage and performance and whether there is a need to have a further enhancement in the physical layer to support RedCap positioning, including the configuration of the DL PRS and UL SRS for positioning, the physical layer procedures, the UE capability, etc.

We are supportive of the objective related to RedCap positioning in the final summary in [1]. For the evaluation of the performance of existing positioning procedures and measurements, we think it can be done by RAN4.

Proposal 6: Suggest the following change to the moderator’s proposal for RedCap Positioning:

- Evaluate the performance of existing positioning procedures and measurements with RedCap UEs[RAN1, RAN4]

# Summary

In this contribution, we provided our views on Rel-18 Expanded and improved Positioning. Based on the discussion, we propose the following modifications to the moderators’ proposals in the final summary [1]:

- Study/specify solutions (including reference signals, measurements, procedures, etc) for sidelink positioning considering the following: [RAN1, RAN2]

  - To include ranging, (i.e. relative positioning), and absolute positioning

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

Note: The next phase of discussion will include the selection of use cases to be used for evaluation and design. This does not restrict the use of the solutions for other use cases.

  - Spectrum: ITS, licensed

  - Coordination with SA2 as required (e.g. architecture aspects)

  - Study and specify the solutions for UE-based and UE-assisted cooperative sidelink positioning by integrating SL measurements with existing DL, UL, DL+UL measurements [RAN1, RAN2, RAN3]

- Improved accuracy, integrity, and power efficiency:

  - Study/Specify solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1, Coordination with SA2 as required]

Including the identification of the error sources, threat models, occurrence rates and failure modes for RAT-dependent positioning integrity

  - Study/Specify solutions for accuracy improvement based on PRS/SRS bandwidth aggregation and NR carrier phase measurements [RAN1, RAN4, RAN2]

including the solutions for the mitigation of the multipath and the network time/frequency synchronization errors for NR carrier phase positioning

  - Study/Specify solutions for accuracy improvement based on intra-band PRS/SRS bandwidth aggregation [RAN1, RAN4, RAN2]

including the feasibility study of PRS/SRS bandwidth aggregation under the impact of timing alignment error, phase continuity, etc.

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, identify potential enhancements to help address any limitations [RAN1, RAN2]

- Define positioning support for RedCap UEs, considering the following:

  - Evaluate the performance of existing positioning procedures and measurements with RedCap UEs[RAN1, RAN4]

  - Based on the evaluation, identify potential enhancements to help address possible limitations associated with RedCap UEs [RAN1, RAN2]

  - Define performance requirements for positioning by RedCap UEs [RAN4]

# References

- RP-210660, “Moderator’s summary for discussion [RAN93e-R18Prep-10] Expanded and improved Positioning”, Moderator (Intel).

- H. Dun, C. C. J. M. Tiberius and G. J. M. Janssen, "Positioning in a Multipath Channel Using OFDM Signals With Carrier Phase Tracking," in IEEE Access, vol. 8, pp. 13011-13028, 2020.

- RWS-210414, “On further positioning enhancements in Rel-18”, CATT

- R1-2007860, “Discussion of NR positioning enhancements”, CATT.

- TR 38.855, “Study on NR positioning support”.

- TR 38.857, “Study on NR Positioning Enhancements”.

- TS 22.261, "Service requirements for the 5G system; Stage 1".

- R1-2106326, “LS on Positioning Reference Units (PRUs) for enhancing positioning performance”, RAN1 (CATT).

- TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”.

- TS 22.186, “Enhancement of 3GPP support for V2X scenarios”.

- TS 22.261: "Service requirements for the 5G system".