3GPP TSG RAN meeting #93-e                                                                          RP-212358

Online, September 13-17, 2021

# Source:	Apple

# Title:	New WID Proposal: Sub-DRB Differentiation for Enhanced Performance and Special Packet Prioritization

# Document for:	Approval

Agenda Item:	9.0.3

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title: 	Sub-DRB Differentiation for Enhanced Performance and Special Packet Prioritization

## Acronym: WI_NR-L2_Optimizations

## Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty but you may make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part |  |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access | X |
|  | Core Network | X |
|  | Services | X |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  | PDCP, RLC, MAC nodes: Yes |
| No | X |  |  |  |  |
| Don't know |  |  |  | X |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Classical examples: List a preceding SI or a preceding WI (e.g. if you further enhance a topic). Also related or dependent WIs in other TSGs should be indicated.

## 3	Justification

This work item will identify and evaluate solutions for high throughput low latency scenarios where overhead reductions, intra-flow packet prioritization and flow synchronization can be tricky due to the latency requirements on the UE and the network. This WI deals with the following problem statements

P1. Overhead reduction at L2 in terms of header optimizations and processing

P2. Ability to prioritize special packets within a flow

all within the context of high throughput and low latency.

Overhead on the UE can stem from packet processing especially in scenarios such as TCP ACKs where processing of a high number of small packets most of which carry information that is unnecessary to the other end in order for the protocol to function properly. These protocols have a cumulative nature in terms of the data they send i.e. a more recent packet provides all the information that is provided by all the earlier packets. Other protocols which exhibit such similar behaviours are RTCP, time synchronization in URLLC etc.

An additional problem noticed in such high throughput low latency applications is the head of line scenarios where important packets within a particular flow cannot be processed and prioritized ahead of the others. This can lead to missing QoS timelines or scenarios of frequent re-adjustment of grant timelines between the UE and networks when there are services needing multiple QoS flows. The overhead to push these “important” packets ahead of the existing queues to be transmitted effectively and efficiently is very high in using existing 3GPP mechanisms. Networks vendors also invest a lot into deep packet inspections to ensure that packets within flows needing prioritization can be aptly treated.

This WI proposal aims to tackle these problems and any additional related scenarios in an effective, secure and efficient manner.   A detailed description of the problem statements is also provided in RP-212357 – (Sub-DRB Differentiation for Enhanced Performance and Special Packet Prioritization).

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The following goals should be taken into account for the solutions considered:

- Achieving a backward compatible solution to reduce overhead on small packets such as TCP ACKs at high data rates in terms of bytes transmitted

- Achieving a backward compatible solution to reduce the overhead of packet processing due to the addition of headers

- Achieving a capability through which in-flow packet prioritization can be made without impacts to security and privacy

- Achieving a capability to exchange additional data without impacting the overall QoS requirements of the service

The objective of the study is to identify and evaluate potential solutions for improving overall implementation scenarios in 3GPP NR for high throughput low latency scenarios. The detailed objectives are as follows:

- Design and implement a framework with the ability to send packets with reduced overhead of packetization and processing and in-flow packet prioritization without impacting security and privacy [RAN2, RAN3]

- Ensure proper capability definitions with minimum parameter set for such a separator flow at RAN [RAN2]

- Recommend other working groups of the feasibility of a 5QI definition of such in flow packet prioritization rules [RAN2]

  - Consulting SA2 might be needed

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each of them in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Title | For info at TSG# | For approval at TSG# | Remarks |
| 38.321/38.322/38.323 | TR | Sub-DRB Differentiation for Enhanced Performance and Special Packet Prioritization |  |  |  |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

## 7	Work item leadership

RAN2 (Primary), RAN3 (Secondary)

## 8	Aspects that involve other WGs

None identified yet

NOTE:	For RAN WIDs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

Apple