3GPP TSG-RAN5 Meeting #93-e	R5-216636

# Electronic Meeting, 8th Nov – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2052 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on simultaneous co-existence of NR cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Table 4.4.2-1, Note 2 clarifies that NR cell 2 and Cell 11 shall not co-exist due to the same timing defined in TS 38.523-3. But that is also related to the SSB index specified in Table 4.4.2-2 and it is applied to any intra-frequency cells using the same SSB index. It is proposed to remove Note 2 from Table 4.4.2-1 and add a new note in Table 4.4.2-2. The same clarification is required in clause 6.3.2 for NGC cells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Table 4.4.2-1: Note 2 updated to void. - Table 4.4.2-2: added Note 3 with recommendations. - Similar updates done for NAS in clauses 6.3.2.1 & 6.3.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cells that should not co-exist in signalling test cases will remain incomplete and unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2, 6.3.2.1, 6.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section >

### 4.4.2	Simulated cells

NOTE 1: For NAS test cases see clause 6.3.2.

NOTE 2: Test frequency and range defined in table 4.4.2-1 do not apply to TS 38.521-1, TS 38.521-2 and TS 38.521-3 test cases.

Test frequencies and simulated NR cells are defined in table 4.4.2-1. Test frequencies and simulated E-UTRA and UTRA cells are defined in TS 36.508 [2] table 4.4.2-1.

For NR cells, NRf1 is the default test frequency. For UTRA cells, f8 as specified in TS 36.508 [2] table 4.2.2-1 is the default test frequency. For E-UTRA cells, f1 as specified in TS 36.508 [2] table 4.2.2-1 is the default test frequency.

Default parameters for simulated NR cells are specified in table 4.4.2-2 and table 4.4.2-3.

Default parameters for simulated E-UTRA cells are specified in TS 36.508 [2] table 4.4.2-1A and table 4.4.2-2.

Default parameters for simulated UTRA cells are specified in TS 36.508 [2] clause 4.4.4.

Common parameters for NR simulated cells are specified in clauses 4.4.3.

Other cell specific parameters are specified in TS 36.508 [2] clause 4.4.3 to 4.4.7.

Table 4.4.2-1: Definition of test frequencies and simulated NR cells

| Test frequency | RAT | Operating band | Range | Simulated NR cells |
| --- | --- | --- | --- | --- |
| NRf1 | NR | Operating band under test | Mid (Note 1, Note 3 | NR Cell 1, NR Cell 2, NR Cell 4, NR Cell 11, NR Cell 489 |
| NRf2 | NR | Operating band under test | High (Note 1, Note 3) | NR Cell 3, NR Cell 12, NR Cell 23 |
| NRf3 | NR | Operating band under test | Low (Note 1, Note 3) | NR Cell 6, NR Cell 13 |
| NRf4 | NR | Operating band under test | (Note 1) | NR Cell 14 |
| NRf5 | NR | Operating band for inter-band cells | Mid (Note 1) | NR Cell 10, NR Cell 30, NR Cell 31 |
| NRf6 | NR | Operating band for inter-band cells | High (Note 1) | NR Cell 28, NR Cell 29 |
| NRf7 | NR | Operating band for inter-band cells | Low (Note 1) |  |
| NRf8 | NR | Operating band for SDL cell | Mid (Note 1) | NR Cell 32 |
| NRf9 | NR | Operating band for SUL cell | Mid (Note 1) | NR Cell 33 |
| Note 1: For signalling test, see clause 6.2.3. Note 2: Void. Note 3: For RRM test with NR intra-band non-contiguous CA, the test frequencies for the set of non-contiguous component carriers are specified in clauses 4.3.1.1.4 for FR1 and in clause 4.3.1.2.4 for FR2 without any regard to range. Thus “Low”, “Mid” and “High” information in this table does not apply. Unless otherwise stated, test point with maximum Wgap is chosen. |  |  |  |  |

Table 4.4.2-2: Default NR parameters for simulated NR cells

| cell ID |  | NR Cell Identifier |  |  | Physical layer cell identity |  | PRACH-rootSequenceIndex FDD |  | PRACH-rootSequenceIndex TDD | SSB-Index2,3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | gNB Identifier |  | Cell Identity |  |  | Note 1 |  | Note 1 |  |  |
| NR Cell 1 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0000'B | 0 |  | 0 |  | 0 |  | 1 |
| NR Cell 2 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0010'B | 2 |  | 32 |  | 32 |  | 0 |
| NR Cell 3 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 0011'B | 3 |  | 0 |  | 0 |  | 1 |
| NR Cell 4 |  | '00 0000 0000 0000 0000 0000 0011'B |  | '00 0000 0100'B | 4 |  | 64 |  | 64 |  | 1 |
| NR Cell 6 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 0110'B | 6 |  | 0 |  | 0 |  | 1 |
| NR Cell 10 |  | '00 0000 0000 0000 0000 0000 0101'B |  | '00 0000 1010'B | 10 |  | 0 |  | 0 |  | 1 |
| NR Cell 11 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0000 1011'B | 11 |  | 96 |  | 96 |  | 0 |
| NR Cell 12 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 1100'B | 12 |  | 32 |  | 32 |  | 0 |
| NR Cell 13 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 1101'B | 13 |  | 32 |  | 32 |  | 0 |
| NR Cell 14 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0000 1110'B | 14 |  | 0 |  | 0 |  | 1 |
| NR Cell 23 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0001 0111'B | 23 |  | 64 |  | 64 |  | 1 |
| NR Cell 28 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0001 1100'B | 28 |  | 0 |  | 0 |  | 1 |
| NR Cell 29 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0001 1101'B | 29 |  | 32 |  | 32 |  | 0 |
| NR Cell 30 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0001 1110'B | 30 |  | 32 |  | 32 |  | 0 |
| NR Cell 31 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0001 1111'B | 31 |  | 64 |  | 64 |  | 1 |
| NR Cell 32 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0000'B | 32 |  | - |  | - |  | 1 |
| NR Cell 33 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0001'B | 33 |  | - |  | - |  | - |
| NR Cell 489 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '01 1110 1001'B | 489 |  | 128 |  | 128 |  | 0 |
| Note 1: To avoid collision of the preambles between intra-frequency cells, with the default zeroCorrelationZoneConfig value set to 15, the PRACH-rootSequenceIndex values have been separated by 32 root sequences per intra-frequency cell. Note 2: This SSB-Index does not apply for RRM test cases in TS 38.533 [18]. RRM test cases shall use the SSB index defined in A.3 of TS 38.533 [18]. Note 3: For signalling test, simultaneous co-existence of active NR cells defined on the same frequency and same SSB-Index is not recommended due to the same timing used for intra-frequency NR cells in TS 38.523-3 [23]. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.4.2-3: Default NAS parameters for simulated NR cells

| cell ID | Tracking Area |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC | MNC |  |  | AMF region ID | AMF Set ID | AMF Pointer |  |
| NR Cell 1 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 [26] clause 2.10 |
| NR Cell 2 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 3 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 4 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 6 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 10 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 11 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 23 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 12, NR Cell 28 | TAI-3 | 002 | 11 | 1 | TAI-3 | 253 | 1 | 1 |  |
| NR Cell 13, NR Cell 29 | TAI-4 | 003 | 21 | 1 | TAI-4 | 252 | 1 | 1 |  |
| NR Cell 14, NR Cell 30 | TAI-5 | 004 | 31 | 1 | TAI-5 | 251 | 1 | 1 |  |
| NR Cell 31 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 489 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates 5G-GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |  |  |

Table 4.4.2-4: Default parameters for simulated SNPN cells

| cell ID | Network Identifier (NID) |  |
| --- | --- | --- |
|  | Assignment mode (Note 1) | NID value |
| NR Cell 1 | 1 | 1 |
| NR Cell 2 | 1 | 2 |
| NR Cell 3 | 1 | 3 |
| NR Cell 4 | 1 | 4 |
| NR Cell 6 | 1 | 6 |
| NR Cell 10 | 1 | 10 |
| NR Cell 11 | 1 | 11 |
| NR Cell 12 | 1 | 12 |
| NR Cell 13 | 1 | 13 |
| NR Cell 14 | 1 | 14 |
| NR Cell 23 | 1 | 23 |
| NR Cell 28 | 1 | 28 |
| NR Cell 29 | 1 | 29 |
| NR Cell 30 | 1 | 30 |
| NR Cell 31 | 1 | 32 |
| NR Cell 32 | 1 | 32 |
| NR Cell 33 | 1 | 33 |
| NR Cell 489 | 1 | 489 |
| Note 1: As per TS 23.003 [26] clause 12.7.1, Network Identifier comprises of Assignment mode (one hexadecimal digit) and NID value (10 hexadecimal digits). Self-assignment NIDs are chosen by setting the assignment mode to value 1. |  |  |

<End of modified section >

<Start of modified section >

### 6.3.2	Default configurations for NAS test cases

The default configurations specified in this subclause apply only to NAS test cases. They apply to all NAS test cases unless otherwise specified.

#### 6.3.2.1	Simulated network scenarios for NAS test cases

Simulated network scenarios for NAS test cases to be tested are specified in the pre-test conditions of each individual test case.

Any combination is allowed with the following restrictions:

-	a maximum 3 cells on the same frequency can be used, i.e. only 3 cells out of NGC Cell A, NGC Cell B, NGC Cell C and NGC Cell D may be used simultaneously in each individual test case when cells in the test case are in different PLMNs (refer to Table 6.3.2.2-3).

#### 6.3.2.2	Simulated NAS cells

Simulated NAS cells and default NAS parameters are specified in Table 6.3.2.2-1. Unless otherwise specified in a test case, default radio parameters of the NAS cells are specified as per Table 6.3.2.2-2.

Unless otherwise specified, the default parameters specified in clause 4.4.2 will also apply to all NAS cells.

Table 6.3.2.2-1: Default NAS parameters for simulated NAS cells

| NAS cell ID | Tracking Area |  |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC |  | MNC |  |  | AMF Region ID | AMF Set ID | AMF Pointer |  |
| NGC Cell A | TAI-1 | (Note 3) |  |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.10.1 [26]. |
| NGC Cell B | TAI-2 | (Note 3) |  |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NGC Cell C | TAI-3 | (Note 3) |  |  | 3 | TAI-3 | 252 | 1 | 1 |  |
| NGC Cell D | TAI-4 | (Note 3) |  |  | 4 | TAI-4 | 252 | 1 | 1 |  |
| NGC Cell E | TAI-12 | 002 |  | 101 | 3 | TAI-12 | 244 | 1 | 1 |  |
| NGC Cell F | TAI-11 | 003 | 101 |  | 2 | TAI-11 | 239 | 1 | 1 |  |
| NGC Cell G | TAI-7 | (Note 4) |  | 02 | 1 | TAI-7 | 238 | 1 | 1 |  |
| NGC Cell H | TAI-8 | (Note 4) |  | 02 | 2 | TAI-8 | 237 | 1 | 1 |  |
| NGC Cell I | TAI-9 | 002 |  | 101 | 1 | TAI-9 | 244 | 1 | 1 |  |
| NGC Cell J | TAI-10 | 003 |  | 101 | 1 | TAI-10 | 236 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure for initial access or mobility (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.8.3). Note 4: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.8.3). |  |  |  |  |  |  |  |  |  |  |

Table 6.3.2.2-2: Default radio parameters for simulated NAS cells when cells are in same PLMN and access stratum is NR

| NAS cell ID | Frequency | NR cell ID (Note 1) |
| --- | --- | --- |
| NGC Cell A | NRf1 | NR Cell 1 |
| NGC Cell B | NRf1 | NR Cell 2 |
| NGC Cell C | NRf1 | NR Cell 4 |
| NGC Cell D | NRf1 | NR Cell 11 |
| NGC Cell E | NA | NA |
| NGC Cell F | NRf2 | NR Cell 3 |
| NGC Cell G | NA | NA |
| NGC Cell H | NA | NA |
| NGC Cell I | NA | NA |
| NGC Cell J | NRf2 | NR Cell 12 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2. Note 2: Simultaneous co-existence of active NGC cells defined on the same frequency and same SSB-Index is not recommended (in line with Table 4.4.2-2 Note 3). |  |  |

Table 6.3.2.2-3: Default PLMN and radio parameters for simulated NAS cells when cells are in different PLMNs and access stratum is NR

| NAS cell ID | PLMN | Frequency | NR cell ID (Note 1) |
| --- | --- | --- | --- |
| NGC Cell A | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 1 |
| NGC Cell B | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 2 |
| NGC Cell C | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 4 |
| NGC Cell D | MCC/MNC=MCC/MNC in USIM | NRf1 | NR Cell 11 |
| NGC Cell E | MCC=002 MNC=101 | NRf2 | NR Cell 3 |
| NGC Cell F | MCC=003 MNC=101 | NRf4 | NR Cell 14 |
| NGC Cell G | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 12 |
| NGC Cell H | MCC = MCC in USIM MNC=02 | NRf2 | NR Cell 23 |
| NGC Cell I | MCC=002 MNC=101 | NRf3 | NR Cell 6 |
| NGC Cell J | MCC=002 MNC=101 | NRf3 | NR Cell 13 |
| Note 1: Default NR parameters for simulated NR cells are as specified in Table 4.4.2-2 Note 2: Simultaneous co-existence of active NGC cells defined on the same frequency and same SSB-Index is not recommended (in line with Table 4.4.2-2 Note 3). |  |  |  |

<End of modified section >