3GPP TSG-RAN5 Meeting #93-e	R5-217118
Electronic Meeting, 8 November – 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2128 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Meas exception for TRS config in RF test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TRS are required for RF testing, as defined in 38.521-1 and 38.521-2, annex A. Corresponding messages exceptions are defined in 38.508-1, section 5.4.1. However, one exception is needed to allow that configuration take effect. (Editorial) table number is discontinuous for last 2 tables in the section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added table 5.4.1-12 to activate csi-MeasConfig in case of TRS. Renumbered tables 5.4.1-16 and 5.4.1-17 as 5.4.1-13 and 5.4.1-14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message exception will remain misaligned with required RF tests conditions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

### 5.4.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

For Tx test cases in which Power Class 3 UEs verifying Power Class 3 requirements, refer to Table 5.4.1-1; For Tx test cases in which Power Class 2 UEs verifying Power Class 2 requirements, refer to Table 5.4.1-2.

Table 5.4.1-1: P-Max-PC3

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA |
|  | 23 |  | EN-DC |

Table 5.4.1-2: P-Max-PC2

| Derivation Path: Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | Not present |  | SA |
|  | 26 |  | EN-DC |

| Condition | Explanation |
| --- | --- |
| SA | NR standalone RF tests cases |
| EN-DC | EN-DC RF tests cases specified in 38.521-3 |

Table 5.4.1-3: Void

Table 5.4.1-4: PUSCH-Config

| Derivation Path: Clause 4.6.3 Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| maxRank | 2 |  | 2TX_UL_MIMO |
|  | 1 |  | ULFPTx_Mode1 or ULFPTx_Mode2 or ULFPTx_ModeFull |
| ul-FullPowerTransmission-r16 | Not present |  | 2TX_UL_MIMO |
|  | fullpowerMode1 |  | ULFPTx_Mode1 |
|  | fullpowerMode2 |  | ULFPTx_Mode2 |
|  | fullpower |  | ULFPTx_ModeFull |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

#### CSI-RS for Tracking

Table 5.4.1-5: CSI-RS-ResourceMapping for TRS

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  | TRS |
| } |  |  |  |
| firstOFDMSymbolInTimeDomain | 6 | l0 = 6 for CSI-RS resource 1 and 3 | TRS AND FR1 |
|  | 10 | l0 = 10 for CSI-RS resource 2 and 4 | TRS AND FR1 |
|  | 8 | l0 = 8 for CSI-RS resource 1 | TRS AND FR2 |
|  | 12 | l0 = 12 for CSI-RS resource 2 | TRS AND FR2 |
| nrofPorts | p1 |  | TRS |
| Cdm-Type | noCDM |  | TRS |
| Density CHOICE{ |  |  |  |
| three | Null |  | TRS |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation for TRS |  | TRS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TRS | Tracking-Reference Signal |

Table 5.4.1-6: CSI-ResourcePeriodicityAndOffset for TRS

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 0 | SCS 15kHz, CSI-RS resource 1 and 2 | TRS AND FR1_15kHz |
|  | 1 | SCS 15kHz, CSI-RS resource 3 and 4 | TRS AND FR1_15kHz |
| Slots20 | 1 | SCS 30kHz, CSI-RS resource 1 and 2 | TRS AND FR1_30kHz |
|  | 2 | SCS 30kHz, CSI-RS resource 3 and 4 | TRS AND FR1_30kHz |
| Slots40 | 2 | SCS 60kHz, FR1, CSI-RS resource 1 and 2 | TRS AND FR1_60kHz |
|  | 3 | SCS 60kHz, FR1, CSI-RS resource 3 and 4 | TRS AND FR1_60kHz |
| Slots80 | 40 | SCS 60kHz, FR2, CSI-RS resource 1 and 2 | TRS AND FR2_60kHz |
| Slots160 | 80 | SCS 120kHz, CSI-RS resource 1 and 2 | TRS AND FR2_120kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |
| FR2_60kHz | FR2 is used under the test. SCS is set to 60kHz. |
| FR2_120kHz | FR2 is used under the test. SCS is set to 120kHz. |
| TRS | Tracking-Reference Signal |

Table 5.4.1-7: CSI-MeasConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 2 entries in case of FR2 4 entries in case of FR1 | 2 entries in case of FR2 4 entries in case of FR1 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 | FR1 |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 | FR1 |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1 |  |
| } |  |  |  |
| csi-IM-ResourceToAddModList | Not present |  |  |
| csi-IM-ResourceSetToAddModList | Not present |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ResourceConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig { | 1 entry |  |  |
| CSI-ResourceConfig[1] | CSI-ResourceConfig for TRS | entry 1 |  |
| } |  |  |  |
| reportTriggerSize | Not present |  |  |
| aperiodicTriggerStateList SetupRelease | Not present |  |  |
| } |  |  |  |

Table 5.4.1-8: NZP-CSI-RS-Resource for TRS

| Derivation Path: 38.508-1 Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| NZP-CSI-RS-ResourceId | 0 | CSI-RS resource 1 |  |
|  | 1 | CSI-RS resource 2 |  |
|  | 2 | CSI-RS resource 3 |  |
|  | 3 | CSI-RS resource 4 |  |
| CSI-RS-ResourceMapping | CSI-RS-ResourceMapping for TRS |  | Content conditioned by the CSI-RS resource # |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | ScramblingId |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for TRS |  | Content conditioned by the CSI-RS resource # |
| qcl-InfoPeriodicCSI-RS | TCI-StateId |  |  |
| } |  |  |  |

Table 5.4.1-9: NZP-CSI-RS-ResourceSet for TRS

| Derivation Path: Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 0 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries in case of FR2 4 entries in case of FR1 |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-ResourceId[3] | 2 | entry 3 CSI-RS resource 3 | FR1 |
| NZP-CSI-RS-ResourceId[4] | 3 | entry 4 CSI-RS resource 4 | FR1 |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | Not present |  |  |
| trs_Info | true |  |  |
| } |  |  |  |

Table 5.4.1-10: CSI-ResourceConfig for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | 0 | entry 1 |  |
| } |  |  |  |
| csi-SSB-ResourceSetList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id |  |  |
| resourceType | periodic |  |  |
| } |  |  |  |

Table 5.4.1-11: CSI-FrequencyOccupation for TRS

| Derivation Path: TS 38.508-1 Table 4.6.3-33 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-FrequencyOccupation ::= SEQUENCE { |  |  |  |
| startingRB | 0 |  |  |
| nrofRBs | max(4*ceil(m/4),24) | m is the bandwidth of active BWP |  |
| } |  |  |  |

Table 5.4.1-12: ServingCellConfig

| Derivation Path: Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig CHOICE { |  |  |  |
| setup | csi-MeasConfig for TRS |  | TRS |
| } |  |  |  |

#### PDCCH Configuration

##### PDCCH-config

Table 5.4.1-13: PDCCH ControlResourceSet

| Derivation Path: Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 48 RBs of the BWP | RFACS AND AL8 |
|  | 11111111 11111111 00000000 00000000 00000000 00000 | CORESET to use the least significant 96 RBs of the BWP | RFACS AND AL16 |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |
| AL8 | PDCCH aggregationLevel 8 is configured |
| AL16 | PDCCH aggregationLevel 16 is configured |

Table 5.4.1-14: PDCCH Search Space

| Derivation Path: Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n2 |  | RFACS |
| aggregationLevel8 | n2 |  | RFACS |
| aggregationLevel16 | n2 |  | RFACS |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RFACS | Used in RF Adjacent Channel Selectivity testing |

## <<< END OF CHANGES >>>