3GPP TSG RAN Meeting # 94e 	RP-212808

Electronic Meeting, December 6th - December 17th , 2021

Agenda Item:	9.1

Source:		IITH, CEWiT, IITM, Tejas Networks, Reliance Jio

Title:		Way forward on pi/2 BPSK

Document for:	Discussion and decision

# Introduction

# In order to address the LMLC (Low-Mobility-Large-Cell) requirements laid down by ITU, TSDSI has submitted the 5Gi standard as a candidate RIT to ITU-R WP5D that is approved as an IMT 2020 standard. A key component of 5Gi standard that improves rural coverage is the low PAPR “pi/2 BPSK with spectrum shaping” waveform that boosts the uplink power and consequently the cell range.

Note that 5Gi standard uses a different form of pi/2 BPSK waveform when compared to the version specified in 3GPP 5G NR specifications. Specifically, 5Gi mandates the use of pi/2 BPSK modulation with 1+D spectrum shaping. For the same, associated signaling changes are also supported. This contribution describes a proposal that aims at harmonizing the 5Gi pi/2 BPSK specification with that of 3GPP 5G NR specifications. This proposal also facilitates the harmonization of 5Gi and 5G and helps in providing enhanced rural coverage.

# Proposed Changes

The changes are in 38.211 and 38.306

38.211

#### 6.3.1.4	Transform precoding

If transform precoding is not enabled according to 6.1.3 of [6, TS38.214],  for each layer .

If transform precoding is enabled according to 6.1.3 of [6, TS38.214], υ=1 and  depends on the configuration of phase-tracking reference signals.

If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are not being used, the block of complex-valued symbols x00,…,x0Msymblayer-1 for the single layer λ=0 shall be divided into MsymblayerMscPUSCH sets, each corresponding to one OFDM symbol and .

If the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used, the block of complex-valued symbols x00,…,x0Msymblayer-1 shall be divided into sets, each set corresponding to one OFDM symbol, and where set  contains  symbols and is mapped to the complex-valued symbols x0(lMscPUSCH+i') corresponding to OFDM symbol  prior to transform precoding, with i'∈0,1,…,MscPUSCH-1 and . The index m of PT-RS samples in set , the number of samples per PT-RS group Nsampgroup, and the number of PT-RS groups NgroupPT-RS are defined in clause 6.4.1.2.2.2. The quantity  when OFDM symbol  contains one or more PT-RS samples, otherwise .

Transform precoding shall be applied according to

y0l.MscPUSCH+k=1MscPUSCHhfki=0MscPUSCH-1xl.MscPUSCH+ie-j2πikMscPUSCH 
k=0,…,MscPUSCH-1
l=0,…,MsymblayerMscPUSCH-1

resulting in a block of complex-valued symbols y00,…,y0Msymblayer-1, where a spectrum shaping filter hfk is applied when π/2-BPSK is configured for PUSCH and the filter values are unspecified. When π/2-BPSK is not configured for PUSCH hfk=1,k=0,…,MscPUSCH-1 . The variable, where  represents the bandwidth of the PUSCH in terms of resource blocks, and shall fulfil

where  is a set of non-negative integers.

##### 6.3.2.6.4	Transform precoding

The block of complex-valued symbols y0,…,y(NSFPUCCH,sMsymb-1) shall be transform precoded according to

zl.MscPUCCH,s+k=1MscPUCCH,shfkm=0MscPUCCH-1yl.MscPUCCH+me-j2πmkMscPUCCH,s 
k=0,…,MscPUCCH,s-1
l=0,…,NSFPUCCH,sMsymb MscPUCCH,s-1

resulting in a block of complex-valued symbols z0,…,z(NSFPUCCH,sMsymb-1). When π/2-BPSK is configured for PUCCH, a spectrum shaping filter hfk is applied and the filter values are unspecified. When π/2-BPSK is not configured for PUCCH, hfk=1,k=0,…,MscPUCCH,s-1 .

##### 6.4.1.1.3	Precoding and mapping to physical resources

The sequence  shall be mapped to the intermediate quantity ak,l(pj,μ) according to

-	if transform precoding is not enabled,

-	if transform precoding is enabled

ak,l(p0,μ)=wfk'wtl'hfkr2n+k'
k=4n+2k'+Δ
k'=0,1
l=l+l'
n=0,1,….

where , , and Δ are given by Tables 6.4.1.1.3-1 and 6.4.1.1.3-2 and the configuration type is given by the higher-layer parameter DMRS-UplinkConfig, and both k' and Δ correspond to p0, …, pν-1. When π/2-BPSK is configured for PUSCH, a spectrum shaping filter hfk is applied and the filter values are unspecified. When π/2-BPSK is not configured for PUSCH, hfk=1,k=0,…,MscPUSCH-1 .

The intermediate quantity ak,l(pj,μ)=0 if Δ corresponds to any other antenna ports than pj.

###### 6.4.1.3.3.2	Mapping to physical resources

The sequence shall be multiplied with the amplitude scaling factor , , in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with  to resource elements k,lp,μ on antenna port  according to

ak,lp,μ=βPUCCH,shf(k).rlm.
m=0,1,….,MscPUCCH,s-1

where

-	 is defined relative to subcarrier 0 of the lowest-numbered resource block assigned for PUCCH transmission,

-	 is given by Table 6.4.1.3.3.2-1 for the case with and without intra-slot frequency hopping and with and without additional DM-RS as described in clause 9.2.1 of [TS 38.213], where  corresponds to the first OFDM symbol of the PUCCH transmission.

The resource elements k,lp,μ shall be within the resource blocks assigned for PUCCH transmission according to clause 9.2.1 of [5, TS 38.213]. When π/2-BPSK is configured for PUCCH, a spectrum shaping filter hfk is applied and the filter values are unspecified. When π/2-BPSK is not configured for PUCCH, hfk=1,k=0,…,MscPUCCH,s-1 .

38.306

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| lowPAPR-DMRS-PUCCH-r16 Indicates whether the UE supports low PAPR DMRS for PUCCH format 3 and format 4 with transform precoding and with pi/2 BPSK modulation. UE indicates support of this feature shall indicate support of pucch-F3-4-HalfPi-BPSK and any combination of support of pucch-F3-WithFH, pucch-F4-WithFH and pucch-F1-3-4WithoutFH. | Band | Yes | N/A | N/A |
| lowPAPR-DMRS-PUSCHwithPrecoding-r16 Indicates whether the UE supports low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK modulation. UE indicates support of this feature shall indicate support of pusch-HalfPi-BPSK. | Band | Yes | N/A | N/A |
| powerBoosting-pi2BPSK Indicates whether UE supports power boosting for pi/2 BPSK, when applicable as defined in 6.2 of TS 38.101-1 [2]. This capability is not applicable to IAB-MT. | Band | Yes | TDD only | No |
| pucch-F3-4-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK for PUCCH format 3/4 as defined in 6.3.2.6 of TS 38.211 [6]. It is optional for FR1 and mandatory with capability signalling for FR2. This capability is not applicable to IAB-MT. | UE | Yes | No | No |
| pusch-HalfPi-BPSK Indicates whether the UE supports pi/2-BPSK modulation scheme for PUSCH as defined in 6.3.1.2 of TS 38.211 [6]. It is optional for FR1 and mandatory with capability signalling for FR2. This capability is not applicable to IAB-MT. | UE | Yes | No | No |