3GPP TSG-RAN4 Meeting #101-e		R4-2119450

# Electronic Meeting, November 1-12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2225 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing RRM performance requirements for n259 FWA in Rel-17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_FWA_Bn259-Perf |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To introduce missing RRM performance requirements for UE power class 5 for band n259 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following requirements are missing for some measurements and are specified: - Minimum SSB_RP at RX beam peak and spherical coverage - Minimum CSI-RS_RP at RX beam peak and spherical coverage |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE supporting band n259 for UE PC5 may not fulfil all the performance requirements and its RRM performance cannot be guanranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.2, B.2.2, B.2.3, B.2.4.1, B.2.4.2, B.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 |  |  |
|  |  | n259 |  |  | -105.5 |  | -117.5+Y5 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |  |
|  |  | n262 | -120.3+Y1 | -105.6 | -103.6 | -118.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n259 |  |  | -92.7 |  |  |  |  |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |  |
|  |  | n262 | -112.1+Z1 | -93.7 | -90.5 | -106.7+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.1.2-2:

- The value of Y for Power classes 1, 4 and 5 is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1, 4 and 5 respectively

- The value of Z for Power classes 1, 4 and 5 is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1, 4 and 5 respectively

----------------------NEXT CHANGE----------------------------

## B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.6+Y5 |  |  |
|  |  | n259 |  |  | -108.5 |  | -120.5+Y5 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |  |
|  |  | n262 | -123.3+Y1 | -108,6 | -106.6 | -121.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.6+Z5 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |  |
|  |  | n262 | -115.1+Z1 | -96.7 | -93.5 | -109.7+Z4 |  |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.2-2:

- The value of Y for power classes 1, 4 and 5 is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and 5 respectively

- The value of Z for power classes 1, 4 and 5 is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and 5 respectively

## B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.6+Y5 |  |  |
|  |  | n259 |  |  | -106.5 |  | -118.5+Y5 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |  |
|  |  | n262 | -121.3+Y1 | -106.6 | -104.6 | -119.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.6+Z5 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |  |
|  |  | n262 | -113.1+Z1 | -94.7 | -91.5 | -107.7+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.3-2:

- The value of Y for power classes 1, 4 and 5 is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and 5 respectively

- The value of Z for power classes 1, 4 and 5 is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and 5 respectively

## B.2.4	Conditions for NR L1-RSRP reporting

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 |  |  |
|  |  | n259 |  |  | -105.5 |  | -117.5+Y5 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |  |
|  |  | n262 | -120.3+Y1 | -105.6 | -103.6 | -118.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |  |
|  |  | n262 | -112.3+Z1 | -93.7 | -90.5 | -106.7+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.1-2:

- The value of Y for power classes 1, 4 and 5 is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and 5 respectively

- The value of Z for power classes 1, and 4 and 5 is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and 5 respectively

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 | -115.5 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.4+Y5 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | -123.6+Y5 |  |  |
|  |  | n259 |  |  | -108.5 |  | -120.5+Y5 |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |  |
|  |  | n262 | -123.3+Y1 | -108.6 | -106.6 | -121.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.4+Z5 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | -115.6+Z5 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |  |
|  |  | n262 | -115.1+Z1 | -96.7 | -93.5 | -109.7+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.4.2-2:

- The value of Y for power classes 1, 4 and 5 is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and 5 respectively

- The value of Z for power classes 1, 4 and 5 is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and 5 respectively

## B.2.5	Conditions for RRC connection release with redirection to NR

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | -121.6+Y5 |  |  |
|  |  | n259 |  |  | -106.5 |  | -118.5+Y5 |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |  |
|  |  | n262 | -121.3+Y1 | -106.6 | -104.6 | -119.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | -113.6+Z5 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |  |
|  |  | n262 | -113.1+Z1 | -94.7 | -91.5 | -107.7+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Editor’s notes for Table B.2.5.2-2:

- The value of Y for power classes 1, 4 and 5 is FFS, where Y1, Y4 and Y5 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1, 4 and 5 respectively

- The value of Z for power classes 1, 4 and 5 is FFS, where Z1, Z4 and Z5 are the rough/fine beam gain differences in spherical coverage directions for power classes 1, 4 and 5 respectively

----------------------NEXT CHANGES----------------------------

----------------------END OF CHANGES----------------------------