3GPP TSG RAN #94-e	RP-213121

e-Meeting, December 6th – 17th, 2021

Source:	Sharp

Title:	Views on Rel-18 UL MIMO

Agenda Item:	8A.1

Document for:	Discussion and Decision

Background

The topic of MIMO evolution for Downlink and Uplink was set as one of the email threads as a candidate of Rel-18 work items based on the guidance after RAN#93-e, and the email discussion was conducted during October 20th to 29th. The moderator’s summary and the draft WID were prepared as RP-212661 and RP-212701, respectively. In this contribution, we show our views on uplink MIMO evolution in Rel-18.

Discussion

In the current proposed WID, the following objectives for the UL MIMO enhancements are captured:

- Enhancements on UL MIMO: - Study, and if justified, specify at least UL DMRS, SRS to enable 6 and 8 Tx UL operation to support [up to 4 layers per UE] in the UL targeting CPE/ FWA/vehicle/Industrial devices - Study, and if needed, specify features to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability - Study, and if justified, specify panel-specific[/beam-specific] timing/power control for UL multi-TRP/panel scenario - Study, and if justified, specify frequency-selective precoding targeting devices with equal or more than 4 Tx - Study, and if justified, specify 2 CW for equal or more than 2 layers in uplink

  - The number of layers per UE

As discussed in the email discussion, one of the discussion points are whether more than 4 layers are supported or not. In our view, handheld devices like smartphones may not be required to implement more than 4 layers. However, non-handheld devices such as CPE, FWA, and vehicle can implement more than 4 antenna ports compared to handheld devices. In addition, the current NR specifications support up to 8 layers in the DL and up to 4 layers in the UL. This means the UL performance is limited and may be congested. Therefore, more than 4 layers should be supported because higher throughput may be required for CPE/ FWA/vehicle/Industrial devices, and it should not be limited to up to 4 layers.

Proposal 1: More than 4 layers per UE should be studied for CPE/ FWA/vehicle/Industrial devices

  - multi-panel UL transmission

In our view, simultaneous multi-panel UL transmission is one of the important enhancements to achieve higher throughput, and panel-specific timing/power control are also specified

Proposal 2: simultaneous multi-panel UL transmission should be specified

Proposal 3: panel-specific timing/power control should be specified

  - Others

We are supportive of the study of frequency selective precoding targeting more than 4 Tx and we are also supportive of more than 2 CW.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: More than 4 layers per UE should be studied for CPE/ FWA/vehicle/Industrial devices

Proposal 2: simultaneous multi-panel UL transmission should be specified

Proposal 3: panel-specific timing/power control should be specified

References

- RP-212657, “Additional Guidance on RAN Rel-18 Email Discussion during October 20th to 29th”, RAN#94-e

- RP-212661, “Moderator’s summary of discussion for [RAN94e-R18Prep-01] MIMO Evolution for Downlink and Uplink” , RAN#94-e

- RP-212701, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e