3GPP TSG RAN #94-e	RP-213122

e-Meeting, December 6th – 17th, 2021

Source:	Sharp

Title:	Views on Rel-18 IAB/VMR

Agenda Item:	8A.3

Document for:	Discussion and Decision

# Background

The topic of Additional topological improvements – IAB (Integrated Access and Backhaul)/VMR (Vehicle Mounted Relay) was set as one of the email threads as a candidate of Rel-18 work items based on the guidance after RAN#93-e, and the email discussion was conducted during October 20th to 29th. The moderator’s summary and the draft WID were prepared as RP-212678 and RP-212718, respectively. In this contribution, we show our views on Mobile IAB in Rel-18.

# Discussion

In the current proposed WID, the following objectives for the Mobile IAB are captured:

The detailed objectives of the WI are listed as follows: - Define Procedures for migration/topology adaptation to enable IAB-node mobility [RAN3, RAN2] - Enhancements for mobility of an IAB-node together with its served UEs. It is FFS whether surrounding UEs shall also be served [RAN3, RAN2] Note: Solutions should avoid to touch upon topics where Rel17 discussions already occurred and where the topic was excluded from Rel17. - Mitigation of interference due to IAB-node mobility, including the avoidance of reference and control signal collisions (e.g. PCI, RACH). [If is FFS whether RAN2 or RAN3 should be leading the activity] The following principles should be respected: - Mobile IAB-nodes should be able to serve legacy UEs. - Solutions providing optimisation for Mobile IAB may entail Rel18 UE enhancements, provided that such enhancements are backwards compatible

The mobile IAB is one of the most important use cases to improve the coverage and capacity enhancements to onboard and/or surrounding UEs and support the use cases in TS 22.839.

Proposal 1: This WI should start in Rel-18.

In TS 22.839, many non-terrestrial and HAPS use cases are shown as use cases of vehicle mounted relay. In our view, non-terrestrial use cases and HAPS should not be included in Rel-18.

Proposal 2: NTN use cases should not be considered in Rel-18.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: This WI should start in Rel-18.

Proposal 2: NTN use cases should not be considered in Rel-18.

# References

- RP-212657, “Additional Guidance on RAN Rel-18 Email Discussion during October 20th to 29th”, RAN#94-e

- RP-212678, “Moderator’s summary for discussion [RAN94e-R18Prep-18] Additional Topological Improvements – IAM/VMR”, RAN#94-e

- RP-212719, “New WID draft Mobile IAB”, RAN#94-e