3GPP TSG RAN Meeting #94e				        	                  RP-213300

Electronic meeting, December 6 - 17, 2021

Source:	CATT

Title:	Motivation on Further RRM enhancement for NR and MR-DC in Rel-18

Agenda Item:	8A.4

Document for:	Discussion

Introduction

In Pre-RAN#94e email discussion [1], the potential RRM enhancement in R18 was discussed and the following working areas were endorsed:

- FR2 RRM enhancements (RAN4, RAN2)

- General RRM requirement enhancement and leftovers (RAN4, RAN2)

- Measurement gap related enhancement and leftover (RAN4, RAN2)

During the discussion, most companies support to continue the RRM enhancement in R18 and many topics are raised. But considering the RAN4 workload and TU arrangement, some down scoping is needed. So only high level working areas were endorsed in the email discussion and more detailed objectives need to be handled in this RAN plenary meeting. And based on the discussion and the potential topic lists,  the dedicated WI for the three working areas may be helpful.

In this paper, we provide the justification of some potential RRM enhancement in Rel-18 based on the working areas above.

Discussion

General RRM requirement enhancement and leftovers

Enhancement for CSI-RS based L3 measurement

In R16, CSI-RS based L3 measurement requirements were defined under some conditions e.g. single FFT assumption, time domain restriction of 5ms time window etc. And in R17 RRM enhancement, due to the tight timeline and limited scope, the CSI-RS enhancement was not included in R17 even though there are extensive support on some aspects. So we think it can be considered in R18 RRM enhancement at least for the following aspects.

- Due to lack of RAN2 involvement in R16, the time domain restriction for CSI-RS based L3 measurement is defined as applicability of requirements in RAN4 i.e. the requirements are applied when the CSI-RS resources in the same frequency layer are configured within up to two separate windows for intra-frequency measurement and within the a same up to 5ms time window for inter-frequency measurement. But it is not a systematic solution for network to configure the CSI-RS resources. It is better to introduce the signaling indication for a clear configuration window (e.g. CMTC).

- Due to the tight timeline in R16, the collision case between CSI-RS based L3 measurement of neighbor cell and the SSB/CSI-RS based RLM/BFD or other SSB/CSI-RS based L1measurements of serving cell is not considered in CSI-RS based L3 measurement requirements. The cases should be clarified in R18 RRM enhancement.

- In the current specification, the CSI-RS based L3 measurement requirements are defined only for intra-frequency without gap and inter-frequency with gap. However, the network configuration is not always so restrictive, there are more possibilities existing in the network. So more practical cases shall be considered for CSI-RS based L3 measurement in R18 which has also been included in SSB and PRS measurement, e.g.

  - intra-frequency measurement with gap（e.g. The BW of the target CSI-RS resources is not within the active BWP of the UE for intra-frequency measurement）

  - inter-frequency measurement without gap （e.g. The BW of the target CSI-RS resources is within the active BWP of the UE for inter-frequency measurement）

In summary, the CSI-RS based L3 measurement enhancement need to be further discussed in Rel-18 at lease for the following aspects:

  - RRM requirements for the case when CSI-RS based L3 measurement have collision with SSB or CSI-RS based L1 measurement

  - Further study the time domain restriction of the CSI-RS resources configuration and identify solutions to address the limitation e.g.

    - Introduce CMTC for CSI-RS based L3 measurement (including RRM requirements and signaling).

  - Further study some practical cases for CSI-RS based L3 measurement.

Proposal 1: Further study and specify the RRM requirements for CSI-RS based L3 measurement in Rel-18, including but not limited to:

  - RRM requirements for the case when CSI-RS based L3 measurement have collision with SSB or CSI-RS based L1 measurement.

  - Further study the time domain restriction of the CSI-RS resources configuration and identify solutions to address the limitation e.g.

    - Introduce CMTC for CSI-RS based L3 measurement (including RRM requirements and signaling).

  - Further study some practical cases for CSI-RS based L3 measurement.

RRM requirements for FR1+FR1 NR-DC

When discussing the R17 RRM enhancement, it was found that even though in TS 38.101-1/3, FR1+FR1 and FR1+FR2 band combinations are both defined for inter-band NR-DC, in current RRM specification, some requirements for FR1+FR1 are missing (e.g. for PSCell addition delay requirement, only FR2 PSCell is defined). Since the FR1+FR1 requirements are supposed to be the baseline in some scenarios, complete RRM requirements for FR1+FR1 NR-DC are expected. So the RRM requirements for FR1+FR1 shall be studied and specified in R18.

Proposal 2: Further study the RRM requirements for FR1+FR1 NR-DC in Rel-18.

HO with PSCell for new scenarios

For HO with PSCell requirements in R17 RRM enhancement, following the WID, only the following scenarios are defined:

- from NR SA to EN-DC

- from EN-DC to EN-DC

- from NE-DC to NE-DC

- from NR-DC to NR-DC

But from the discussing, there can be other scenarios that operators are interested in (e.g. from NR SA to NE-DC, from NR SA to NR-DC and from LTE SA to EN-DC etc.). Considering the market demand and commercial deployment, these scenarios can be considered in R18.

Proposal 3: Further study the new scenarios for HO with PSCell requirements in Rel-18.

TCI switching enhancement

In Rel-16 TCI switching requirement, there was a time gap from “slot n+ THARQ +(3 ms) / NR slot length” to “slot n+ THARQ +(3 ms+TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length”. And UE is not required to receive DL data during this time gap which probably impact the throughput performance in case the first SSB arrives 160ms later. Due to limited scope of RRM requirements in R17, this enhancement was not included in R17. In our understanding, RAN4 can further discuss if UE behavior can be enhanced to receive DL data during the time gap of Tfirst-SSB in R18.

Proposal 4: Further study and specify the RRM requirements for TCI witching enhancement in Rel-18.

RLM requirement enhancement

The function of RLM is to monitor the DL signal quality of the primary cell for the purpose of determining UE out-of-sync or in-sync status. The RLM design has been extensively discussed in RAN1 during many meetings. Reference [4-20] shows the broad interest of this topic in RAN1 from different companies. One of the most important aspects discussed for RLM is whether to support multiple IS/OOS PDCCH BLER pairs for supporting RLM optimization of VoNR and URLLC. Finally RAN1 agreed to support configurability between two pairs of IS/OOS BLER values for RLM purposes for VoNR and a LS was also sent to RAN4 to let RAN4 to decide the second BLER pair.

From the LS in R1-1809853, very clear background information on the purpose of the second IS/OOS BLER pair was included. It has been observed in LTE, when semi-persistent scheduling is used, the service quality can be adequate also when the SINR is below the level corresponding to a PDCCH BLER of 10%. The most prominent example is for VoLTE. In these cases, it has been observed that the UE triggers RLF also when the voice quality is quite adequate. In other words, RLF may be triggered prematurely. We think RLM is a very important feature which impact operators’ network performance and deployment cost. We propose to have some discussion on this feature in Rel-18.

Proposal 5: Study on how to introduce the second IS/OOS BLER pair for VoNR in Rel-18.

Measurement gap related enhancement and leftover

RRM enhancement for gap-less measurement

In the current specification, some requirements are always gap based, e.g. CSI-RS based inter-frequency measurement and inter-RAT NR measurement on non-serving cell in EN-DC. But similar as the inter-frequency SSB measurement, when the target resources are within the active BWP, it is possible to perform the CSI-RS based measurement without gap. To increase the UE throughput and reduce the measurement latency, it is proposed to define RRM requirements without gap for inter-RAT NR measurement and inter-frequency CSI-RS based L3 measurement if the SSB/CSI-RS is fully confined within the UE active BWP to save gaps and align with inter-frequency SSB based measurement.

Proposal 6: Further study the RRM enhancement for gap-less measurement in Rel-18 including but not limited to:

  - CSI-RS based inter-frequency measurement without gaps

  - Inter-RAT NR measurement without gaps

RRM enhancement for “NeedForGap”

In R16 TEI, the gap indication ‘NeedForGap’ was introduced by RAN2 which can indicate whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured. The indication can avoid the unnecessary gap configuration and reduce the gap overhead. But the possible interruption or RRM requirements impact need further study based on the indication. In pre-RAN#94e email discussion, some companies mentioned this requirement can be included in R17 NCSG discussion, but we think this is independent topic and cannot be covered by NCSG discussion. The first reason is that the indication of NCSG has not been discussed in RAN2 and there is no agreement that it is based on “NeedForGap”. The second one is that we think in NCSG discussion, the target is to define the RRM requirements (e.g. dedicated CSSF and measurement period) with NCSG while this topic is about the requirements with no gap and no NCSG. So we suggest to specify this enhancement in R18.

Proposal 7: Further study the RRM enhancement with “NeedForGap”.

Enhanced indication of UE per-FR gap capabilities

Per-FR gap capability was introduced in Rel-15 and is per-UE reported. When UE support per-FR gap, the interruption can only apply to the same frequency range. But with more FR1+FR2 band combinations introduced, the processing complexity to support per-FR gap is increased as it is not purely the issue of RF architecture. If UE have problems to support this per-FR measurement gap only in certain FR1+FR2 band combinations, then UE can only report the capability of per-UE gap which will definitely impact the UE performance. In previous meeting, one possible solution raised for this issue is to have an additional per-BC indication. Based on this solution, RAN4 discussed the feasibility and the possible impact on the existing RRM requirements but no conclusions are reached. In our understanding, this issue can be further studied in R18 on the top of the extensive discussion in R16.

Proposal 8: Further study the enhanced indication of per-FR gap capability in Rel-18.

Joint requirements for Pre-MG, concurrent gaps and NCSG

In R17 NR and MR-DC measurement gap enhancements WI, three objectives pre-configured MG (pre-MG), concurrent gaps and Network Controlled Small Gap (NCSG) were defined. And it was agreed in WID [23] that the joint requirements for the three objectives can be discussed after stabilizing individual requirements. Until RAN4#101e meeting, there are many open issues to be discussed for each individual objective [24-26]. The target completion plenary for core part of this WI is RAN#95 meeting and only one quarter left for core requirements discussion. Considering the timeline there may be not enough time to discuss the three objectives jointly. We propose to define joint requirements for pre-configured MG (pre-MG), concurrent gaps and NCSG in R18.

Proposal 9: Study and specify joint requirements for pre-configured MG (pre-MG), concurrent gaps and Network Controlled Small Gap (NCSG) in R18.

Support of per-FR gap for PRS measurement

In R16 positioning support, two new patterns were introduced for PRS measurement which is per-UE capability. And for the legacy gap used for PRS measurement, it was agreed in maintenance stage in RAN4#100e meeting that Rel-16 PRS-based measurements are supported with per-UE measurement gaps only [22]. In our understanding, this definitely restricts the NW configuration and UE data throughput. For example, even for a per-FR capable of UE and positioning measurement is configured in only one FR, only per-UE gap can be configured which will cause unnecessary data loss in the other FR.

In another aspect, the concern on supporting per-FR gap for PRS measurement is the UE implementation when RRM measurement and PRS measurement are configured simultaneously in different FRs. But we think there is no big difference with the case that RRM measurements are configured simultaneously in different FRs which has been supported in the current specification. So we propose to study the per-FR gap support for PRS measurement in R18.

Proposal 10: Study and specify per-FR gap support for PRS measurement in R18.

Conclusion

In this contribution, we provide the justification to specify some enhanced RRM requirements in Rel-18 based on the endorsed high level working areas and the following proposals are given:

Proposal 1: Further study and specify the RRM requirements for CSI-RS based L3 measurement in Rel-18, including but not limited to:

  - RRM requirements for the case when CSI-RS based L3 measurement have collision with SSB or CSI-RS based L1 measurement.

  - Further study the time domain restriction of the CSI-RS resources configuration and identify solutions to address the limitation e.g.

    - Introduce CMTC for CSI-RS based L3 measurement (including RRM requirements and signaling).

  - Further study some practical cases for CSI-RS based L3 measurement.

Proposal 2: Further study the RRM requirements for FR1+FR1 NR-DC in Rel-18.

Proposal 3: Further study the new scenarios for HO with PSCell requirements in Rel-18.

Proposal 4: Further study and specify the RRM requirements for TCI witching enhancement in Rel-18.

Proposal 5: Study on how to introduce the second IS/OOS BLER pair for VoNR in Rel-18.

Proposal 6: Further study the RRM enhancement for gap-less measurement in Rel-18 including but not limited to:

  - CSI-RS based inter-frequency measurement without gaps

  - Inter-RAT NR measurement without gaps

Proposal 7: Further study the RRM enhancement with “NeedForGap”.

Proposal 8: Further study the enhanced indication of per-FR gap capability in Rel-18.

Proposal 9: Study and specify joint requirements for pre-configured MG (pre-MG), concurrent gaps and Network Controlled Small Gap (NCSG) in R18.

Proposal 10: Study and specify per-FR gap support for PRS measurement in R18.

References

RP-212682, Moderator’s summary for discussion [RAN94e-R18Prep-22], RAN4 Chair

RP-191027,  WF for Rel-16 RAN4 RRM, Demod, HST and URLLC scope, RAN4 chairman, RAN#84

RAN4_96e_RRM_chairman_report_12_EOM

R1-1717038, “RLM in NR”, ZTE, Sanechips

R1-1717055, “Remaining details on Radio link monitoring in NR”, Huawei, HiSilicon

R1-1717360, “Radio Link Monitoring for NR”, Intel Corporation

R1-1717464, “IMR configuration for Radio link monitoring in NR”, Vivo

R1-1717584, “Radio link monitoring”, Samsung

R1-1717742, “Discussions on the IS and OOS counting procedure”, Spreadtrum Communications

R1-1717805, “NR Radio Link Monitoring”, CATT

R1-1717933, “Discussion on Radio Link Monitoring in NR”, LG Electronics

R1-1718061, “Remaining details on NR radio link monitoring	Guangdong”, OPPO Mobile Telecom

R1-1718186, “Discussion on remaining details on radio link monitoring for mobility management”, NTT DOCOMO

R1-1718321, “Discussion on Radio Link Monitoring,”	MediaTek Inc.

R1-1718383, “Radio link monitoring procedure and thresholds	,”AT&T

R1-1718534, “Radio link monitoring consideration”, Qualcomm Incorporated

R1-1718803, “Radio Link Monitoring in NR”, Nokia, Nokia Shanghai Bell

R1-1718718, “Radio link monitoring for mobility management”, Ericsson

R1-1718794, summary of discussion for NR Radio link monitoring, Intel

R1-1809853, LS on second BLER pair for RLM

RP-211579, Moderator's summary for email discussion [92-e-23-RRM-Enh]

R4-2115301, WF on UE PRS measurement requirements, Huawei

RP-211591, Revised WID on NR and MR-DC measurement gap enhancements, Intel, MTK

R4-2120301, WF on R17 NR MG enhancements – Pre-configured MG, Intel

R4-2120414, WF on NR MG enhancements – General issues and Multiple concurrent MGs, MTK

R4-2120415, WF on R17 NR MG enhancement – NCSG, Apple