3GPP TSG RAN#94-e																				RP-213305

Electronic Meeting, December 6 - 17, 2021

Source:	CATT

Title:	On Rel-18 Expanded and improved Positioning

Agenda item:	8A.1

Document for:	Discussion and Decision

# Introduction

3GPP has specified multiple NR location technologies in Rel-16 and Rel-17 to support positioning use cases and applications. To support higher accuracy location requirements for 5G location service applications, Rel-17 has defined the target positioning requirements of <1m (90%) for commercial use cases and < 0.2m (90%) for IIoT use cases with end-to-end latency <100ms [2], which is much better positioning performance than Rel-16. However, there is still a need for support expanded and improved positioning in Rel-18.

In the pre-RAN#94 email discussion [RAN94e-R18Prep-06], the revised objectives, as a result of the final round discussion for further NR positioning enhancements in [1], are provided with the proposed WID in RP-212706[2].

In this contribution, we provide our views on the objectives of Rel-18 further NR positioning enhancements and propose some further changes.

# Discussions

## Sidelink positioning

For sidelink positioning, the following objectives are included in the proposed WID [2]:

- Study (for study phase of 9 months) solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements [Moderator comment: These could be moved to the justification section in the final WID]: - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the identified uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering: [RAN1, RAN2] - Relative positioning, ranging and absolute positioning - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes, including signal design, phy layer control signalling, resource allocation, physical layer measurements, associated physical layer procedures, etc [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with SA2 as required] - Recommend solutions, if any, to be specified in the normative phase [RAN1, RAN2]

From the above objectives, it seems the work scope for sidelink positioning may be too large, especially for the simulation evaluation. For sidelink positioning in Rel-18, our suggestion is to focus on the basic functionalities of the sidelink positioning, and then in Rel-19 to consider the enhancement of the sidelink positioning, similar to the development of basic NR positioning in Rel-16, and the enhancements in NR positioning in next releases. To reduce the work scope with the consideration of developing the basic functionalities, our suggestions are:

- For the coverage scenarios: we may first focus on in-coverage and out-of-coverage. The investigation of the partial-coverage can be considered in Rel-19.

- For the use cases, if we want to consider all of the use cases for V2X, public safety, commercial and IIOT, the workload for simulation evaluation may be too much for one release. Thus, our suggestion is to focus on V2X, public safety and commercial use cases. The IIOT use cases can be considered in Rel-19, similar to NR positioning, where IIOT use cases are not considered in the Rel-16 but in Rel-17.

Proposal 1: Suggest making the following changes to the proposed objectives for the SL positioning:

- Scenario/requirements [Moderator comment: These could be moved to the justification section in the final WID]:

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

## Improved accuracy, integrity, and power efficiency

For “Improved accuracy, integrity, and power efficiency”, the following objectives are included in the proposed WID [2]:

- Improved accuracy, integrity, and power efficiency: - Study (for study phase of 9 months) solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, failure modes, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2, RAN1] - Study (for study phase of 9 months) and, if found beneficial, specify solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers[RAN1, RAN4, RAN2, RAN3]: - RAN4 to consider implications of PRS/SRS bandwidth aggregation (e.g. timing errors, phase coherency, frequency errors, power imbalance, etc). - Study (for study phase of 9 months and, if found beneficial, specify solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4, RAN2, RAN3] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary - Signalling for configuration and measurement reporting [RAN2, RAN3] - Study (for study phase of 9 months) the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, identify potential enhancements to help address any limitations [RAN2, RAN1]

In our view, it is essential for Rel-18 to support further accuracy improvements, especially the use of the NR carrier phase measurements for accuracy improvement.  GNSS carrier phase measurements have been very successfully used for supporting cm-level accuracy positioning for outdoor environments. In recent years, carrier phase measurements obtained from wireless OFDM communication systems are also proposed (e.g.,[2]). In comparison with GNSS carrier-phase positioning, the implementation of NR carrier phase positioning would have many salient advantages, as pointed out by many companies in the previous email discussion [1], and also in our previous paper [3].

Although “Improved accuracy, integrity, and power efficiency” includes 4 separate objectives, the workload is relatively much smaller than the SL positioning, especially for the workload for RAN1. In addition, considering that RAN1 has studied PRS/SRS bandwidth aggregation in Rel-17, and has identified that there is a need for RAN4 to study the feasibility of PRS/SRS bandwidth aggregation, thus if RAN4 decides it is feasible to support PRS/SRS bandwidth aggregation for intra-band carriers, it seems no need for RAN1 to study it again. Then, RAN1 can be involved directly in the normative work after the study phase, once RAN4 decides it is feasible to support PRS/SRS bandwidth aggregation for intra-band carriers.

Observation 1: RAN1 has studied PRS/SRS bandwidth aggregation in Rel-17 and identified that there is a need for RAN4 to study the feasibility of PRS/SRS bandwidth aggregation. There is no need for RAN1 to study it again. Thus, the study of accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers can be led by RAN4.

Observation 2: If RAN4 decides that it is feasible to support PRS/SRS bandwidth aggregation for intra-band carriers, RAN1 can be involved directly in the normative work.

Proposal 2: Suggest making the following changes to the Improved accuracy, integrity, and power efficiency:

  - Study (for study phase of 9 months) and, if found beneficial, specify solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers[RAN1, RAN4, RAN1, RAN2, RAN3]:

    - RAN4 to consider implications of PRS/SRS bandwidth aggregation (e.g. timing errors, phase coherency, frequency errors, power imbalance, etc).

## RedCap positioning

For RedCap positioning, the following objectives are included in the proposed WID [2]:

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1, RAN4] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2] - Define core and performance requirements for positioning measurements performed by RedCap UEs [RAN4]

For supporting RedCap UE positioning, we envision there are no significant changes in existing positioning architecture and solutions. There may have some impacts on signalling, protocol, message, and procedures. The main issues that may need to be considered include the impacts of the reduced complexity on NR positioning coverage and performance and whether there is a need to have a further enhancement in the physical layer to support RedCap positioning, including the configuration of the DL PRS and UL SRS for positioning, the physical layer procedures, the UE capability, etc.

In the proposed WID objective, both RAN1 and RAN4 works on the evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs.The  duplication seems to be unnecessary. In our understanding, RAN4 needs always to go through the simulation evaluation procedure when defining the performance requirements for positioning measurements, thus it seems there is no need for RAN1 to be involved in the simulation evaluation. In addition, it seems there is no clear and/or urgent commercial demand for the changes of supporting very high-positioning performance for RedCap UEs. Thus, in our view, in Rel-18 the focus of RedCap positioning is to support the positioning functionalities based on R16 and R17 NR positioning solutions without the change of the physical layer design. Further enhancements of RedCap positioning for higher layer signalling and procedure can be considered if needed, e.g., the enhancement of the eDRX for the reduction of the power consumption. Thus, we suggest that ReCap positioning can be handled by RAN2 and RAN4. RAN1 does not need to be involved.

Proposal 3: Suggest making the following changes to Positioning support for RedCap UEs:

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1, RAN4]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with RedCap UEs [RAN1, RAN2]

  - Define core and performance requirements for positioning measurements performed by RedCap UEs [RAN4]

## SI/WI structure

During the previous email discussion, the following options for the overall structures were discussed:

- Option 1: Single WI in which the items requiring study start with a study phase. The WID will be updated at the end of the study phase with new objectives coming from the conclusions of the studies. Those objectives not requiring study can start immediately on the normative work.

- Option 2: Single SI for 9 months to study all the items that require it. At the end of the study phase, a WI will be created that includes objectives for these items and also objectives for those items that did not need study.

- Option 3: Both SI and WI are approved: the SI is for the objectives with the study phase; and the WI for the objectives that do not need the study phase.

Among these options, our preference is Option 2 for multiple reasons:

- Option 2 is used for both Rel-16 and Rel-17 NR positioning. In Rel-17, some of the objectives were included in the WI w/o the study phase. In past experience, it is highly desirable that all of the objectives start the normative work at the same time.

- Based on the proposed SI/WI objectives, if Option 1 were adopted, only RedCap positioning could start immediately on the normative work. However, the work for RedCap positioning depends mainly on RAN4. The problem is that there is a 6-month time lag between the time schedule of RAN4 and RAN1. Thus, when Rel-18 starts in other WGs, RAN4 is still working on Rel-17 performance requirement, and cannot start the normative work for RedCap positioning when Option 1 or Option 3 is adopted.

Proposal 4: For the SI/WI structures, we suggest adopting Option 2, i.e., Single SI for 9 months to study all the items that require it. At the end of the study phase, a WI will be created that includes objectives for these items and also objectives for those items that did not need study.

# Summary

In this contribution, we provided our views on Rel-18 Further Positioning Enhancements. Based on the discussion, we propose the following modifications to the proposed WID [2]:

Proposal 1: Suggest making the following changes to the proposed objectives for the SL positioning:

- Scenario/requirements [Moderator comment: These could be moved to the justification section in the final WID]:

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

Observation 1: RAN1 has studied PRS/SRS bandwidth aggregation in Rel-17 and identified that there is a need for RAN4 to study the feasibility of PRS/SRS bandwidth aggregation. There is no need for RAN1 to study it again. Thus, the study of accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers can be led by RAN4.

Observation 2: If RAN4 decides that it is feasible to support PRS/SRS bandwidth aggregation for intra-band carriers, RAN1 can be involved directly in the normative work.

Proposal 2: Suggest making the following changes to the Improved accuracy, integrity, and power efficiency:

  - Study (for study phase of 9 months) and, if found beneficial, specify solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers[RAN1, RAN4, RAN1, RAN2, RAN3]:

    - RAN4 to consider implications of PRS/SRS bandwidth aggregation (e.g. timing errors, phase coherency, frequency errors, power imbalance, etc).

Proposal 3: Suggest making the following changes to Positioning support for RedCap UEs:

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1, RAN4]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with RedCap UEs [RAN1, RAN2]

  - Define core and performance requirements for positioning measurements performed by RedCap UEs [RAN4]

Proposal 4: For the SI/WI structures, we suggest adopting Option 2, i.e., Single SI for 9 months to study all the items that require it. At the end of the study phase, a WI will be created that includes objectives for these items and also objectives for those items that did not need study.

# References

- RP-212666, Moderator’s summary for [RAN94e-R18Prep-06] on Expanded and improved positioning, Moderator (Intel)

- RP-212706, New WID on NR Positioning Enhancements, Intel (Email discussion moderator)

- RP-212250 On Rel-18 Expanded and improved Positioning CATT