3GPP TSG RAN Meeting #94-e			RP-213379

Electronic Meeting, December 6 – 17, 2021

Agenda Item:	8A.5

Source:	Apple

Title:	User Plane Enhancements

Document for:	Discussion/Decision

# Introduction

With the growth of 5G, applications and services needing and utilizing low latency attributes have increased quite dramatically. This has paved the way for disruption in both traditional and non-traditional sectors while higher coverage and multi-gigabit throughputs are important as well. Applications such as 8K video, AR/VR/XR and those with URLLC kind of requirements can result in a multi-fold increase in complexity while having to retain the same form factors due to marketing demands. However, Rel-16 and Rel-17 though providing major enhancements to new services, operate with under the fundamental structures and protocols of the Rel-15 user plane. Some enhancements have also put in extra burden through full user plane integrity protection with L2 overheads. The overhead is not only in terms of bytes but also in terms of additional processing cycles needed for framing all packets irrespective of their importance while still ensuring low latency as an important criterion even in multi gigabit throughput ranges.

In this document, we present our views on why and how modifications can be made to L2 protocol stacks in line with other enhancements made in Rel-16 and Rel-17 for ensuring a successful assurance of high throughput low latency scenarios of 5G. Moreover, a smooth transition to 5G-Advanced can be achieved without impacting older UEs or networks.

# Discussion

In this section, our intent is to show why modifications to the existing user plane architectures are needed to support the high throughput low latency use cases of 5G-Advanced. For this purpose, we have identified two representative problem statements. However, we encourage the reader to look beyond the obvious and identify that each of these problems can be generalized to fit in more generic needs with an effective framework structure.

Problem 1: Overhead due to small data processing of unimportant packets

Problem 2: Overhead due to prioritizing packets within flows

## Small packet processing overhead

TCP [5] is the most proficient protocol used by over 90% of all mobile internet traffic. Its popularity is due to its assurance of reliable data transfer using its cumulative ack based scheme. With throughputs in 5G reaching multi-gigabits per second, there is constant pressure on the UE in terms of processing acknowledgements for received data in the UL. Despite being discussed during Rel-15 [1], due to the lack of sufficient time during the beginning of a new 5G release, this item was not pursued. Back then, the peak throughputs were 1Gbps but have subsequently increased to 20Gbps. At 20Gbps with TCP, the UEs need to process ~1.6M TCP ACKs per second (assuming no additional enhancements are present). This implies 1.6M L2 headers need to be processed, which is creating challenges in implementation. Additionally, there is also considerable overhead in terms of bytes as added headers for small TCP ACK packets. Delaying TCP ACKs in UL limited scenarios through concatenation schemes can impact and limit DL throughput. Head-of-line blocking issues, where older TCP ACKs hold up newer ones, can further degrade DL throughputs. These issues arise due to the inability of RAN AS protocols to utilize to their benefit the inherent cumulative nature of the TCP protocol (i.e., the most recent cumulative ACK is sufficient to move the TCP engine forward if received within a TCP RTT at the other end). Given this, we have the following observation:

Observation 1: A TCP ACK sender needs to only transmit the most recent cumulative ACK to the receiver for moving the window forward provided this is received within a TCP RTT.

Observation 2: RAN2 L2 protocols do not utilize the cumulative nature of the most popular Internet transport mechanism.

## High priority packets within flows

In applications such as 8K video or those using video flows such as AR/VR, some packets (e.g., I-frames) within the flow are of a higher importance than others (e.g., P-frames, B-frames) due to the kind of information they carry. Video traffic accounts of over 60% [2] of all mobile data traffic. Most of this traffic is non-QoS and uses best effort DRBs. At high data rates, maintaining consistent user experiences over best effort flows isn’t always easy. In such instances, ensuring that the most important packets are sent as needed and can be prioritized over the air would ensure some end-to-end quality maintenances. With video traffic, the important data is carried by I-frames (intra-coded picture key frames) while the P-frames (predicted picture frames) and B-frames (bi-directional predicted picture frames) ensure application layer compressed information can be sent efficiently instead of sending full images every time. These frames can only be identified by the application since they cannot be distinguished using existing NAS packet filters and are part of the flow which is traversed through a specific DRB. We, therefore, have the following observation.

Observation 3: In cases where high priority packets within a particular flow need to be transmitted, prioritization of packets belonging to the same DRB cannot be performed with existing RAN2 protocols.

An additional issue with both Problems 1 and 2 is that these are not limited to UL and savings resulting from enhancements to these problems can also help DL (and therefore gNB) in both reduction of overhead of processing unnecessary packets and prioritizing important packets of a flow that are tagged as important within a flow. We, therefore, have the following observation and proposal.

Observation 4: Implementation of overhead reduction schemes at gNB for specific packets of certain traffic types can lead to a high efficiency in terms of DL throughput utilization.

Proposal 1: Study overhead reduction and prioritization techniques to enhance QoS in high-throughput and low-latency scenarios.

Currently the only way these packets can be identified is at the end points – either on the UE application or on the server generating the data. Therefore, existing mechanisms based on IP 5-tuple filtering might not always work. Given the plethora of potential applications possible with 5G, such in-flow differentiation will need to be taken more seriously.

## Separate DRBs

One solution to achieve a differentiated treatment of certain packets in a data flow is to create a separate DRB with new 5QI restrictions for all such traffic, encompassing average rates, delay budget, packet loss rates etc. Such special DRB can map to a QoS slightly better than the QoS of the original data flow itself. However, packets from different services need to all be placed on this one special DRB and might continue to face the same issues of head-of-line blocking. An alternate solution to create a separate doppelganger DRB to prioritize certain packets over others within a flow seems not practical due to the current limitation on the 29 DRBs [3] available (though only 16 are mandatory minimum). Increasing the # of DRBs though possible, is not the best solution due to the additional demands placed on the UEs and thus counterproductive to keeping the UE costs low.

Observation 5: A separate DRB for all differentiated packets though being the simplest solution isn’t the best nor is it future proof.

## Overhead reduction and in-flow packet prioritization

During Rel-15 timeframe, PDCP concatenation was proposed as an alternate option to reduce overhead especially due to small packets. In this method, several PDCP SDUs are merged to form a single large PDCP PDU, thus, reducing the overhead in terms of processing as well as number of bytes. Though this solution can ensure reduced overheads, it is unable to take care of prioritization scenarios. Also, since the entire problem statement discussion is related to high throughput low latency scenarios, waiting for aggregation/concatenation can induce additional latency that might be unacceptable.

Observation 6: PDCP concatenation, though effective for overhead reduction, does not allow for in-flow packet prioritization and causes additional delay.

Alternate solutions include separate RLC bearers or MAC based separators allowing both in-flow packet prioritization and queue management techniques through which overhead reduction can also be achieved. This would need multiple changes, however, at the corresponding user plane layer. One change is an implementation of a LIFO queuing mechanism at RLC through which the most recent TCP ACK can be sent by the UE or the server. Similarly, important packets can be separated out within a DRB and sent through the higher priority RLC flow. Similar approaches are possible at MAC as well. The layers do not perform concatenation and therefore will need additional tags originating from the higher layers. Though, the tagging can be left up to UE implementation, the network can ensure control through rate limiting the RLC flows to ensure proper implementation.

Below are some results in terms of number of TCP ACK packets to be processed using the various schemes. The results were obtained for the following parameters: TCP Window Size = 4M, TCP Delayed ACK of 4 [6], PDCP concatenation with max PDCP PDU size of 9000 bytes resulting in ~ 6 TCP Packets of 1500 bytes each. Results for a modified implementation of RLC Mode (called RLC CM or SM) where the most recently received TCP ACK is sent while following a timed discarding of the those received earlier are also shown in Table 1. As can be seen, the new RLC mode can provide processing reduction by reducing the number of packets by over 99% thus providing a long-term solution even when throughputs reach 100s of gigabits per second. A corresponding concatenation scheme is 50 times more inefficient than the modified RLC mode.

| TCP Rate/#Packets Processed in UL | Packets Received/second | Delayed Acks/Second | Delayed ACKs + Concatenation Schemes | Modified RLC using CM/SM mode |
| --- | --- | --- | --- | --- |
| 4 Gbps | 370370 | 92593 | 15432 | 375 |
| 12 Gbps | 1111111 | 277778 | 46296 | 1125 |
| 20 Gbps | 1851852 | 462963 | 77160 | 1875 |
| 100 Gbps | 9259259 | 2314814 | 385802 | 9375 |

Table 1: Number of TCP ACKs to be processed in various enhanced schemes

Proposal 2: Study modifications at RLC or MAC layers to achieve both overhead reduction (in terms of bytes and processing) and in-flow packet prioritization.

## Application layer operation, privacy, and user experience

Special packet prioritization is an additional problem which arises with multi-flow QoS applications. When applications utilize multiple flows and a combination of latency sensitive and non-latency sensitive flows, synchronization of these flows at the application layers can help ensure appropriate user experience. UE implementations today, in such situations, request BSR due to, for instance, misalignment of respective configured grant (CG) configurations. This is an additional burden as the misalignment can happen multiple times within a session due to various issues including those related to application layer processing, memory management, bursty traffic, overload, etc. From a network perspective, deep packet inspection may help identify corresponding flows and any misalignment created from end to end, such that the network may re-adjust grants to meet updated traffic demands. However, revealing the details of traffic flows typically is a privacy and security issue. Solutions proposed in this regard are related to UE assistance information messages or MAC CEs which request the network to re-align the CGs with the UE delays as offsets.

With some of the mechanisms mentioned in this paper, control can be facilitated between either the communication end points or through gNB per application instance with appropriately assigned priority for these flows. Additional control may originate directly from applications incurring a delay to be passed with priority to the network so that adjustments can be made faster by the gNB. This aligns with intra-flow packet (in this case metric) prioritization. We, therefore, have the following proposals.

Observation 7: Overhead optimization proposals can be utilized to also achieve inter-flow synchronization for applications with tight latency limits.

Proposal 3: Study configurations to achieve flow synchronization under latency bounds using overhead optimization principles.

Proposal 4: Study overhead optimization protocols without compromising user privacy and security needs.

The issues and problem areas identified in this paper should be evaluated, and a set of solutions understood. We therefore propose to have a short study item in Rel-18.

Proposal 5: Discuss and confirm overhead reduction and user plane optimizations at L2 and higher layers using the SID description in RP-213380 [4].

# Conclusions

In this contribution, we provided our views on overhead reduction optimizations using L2 and other higher layer protocols. We have indicated the problem statements and highlighted potential solution areas. We ask RAN to consider this contribution and the corresponding proposal SID as defined in RP-213380. For ease of readability, we list our observations followed by the proposals.

Observation 1: A TCP ACK sender needs to only transmit the most recent cumulative ACK to the receiver for moving the window forward provided this is received within a TCP RTT.

Observation 2: RAN2 L2 protocols do not utilize the cumulative nature of the most popular Internet transport mechanism.

Observation 3: In cases where high priority packets within a particular flow need to be transmitted, prioritization of packets belonging to the same DRB cannot be performed with existing RAN2 protocols.

Observation 4: Implementation of overhead reduction schemes at gNB for specific packets of certain traffic types can lead to a high efficiency in terms of DL throughput utilization.

Observation 5: A separate DRB for all differentiated packets though being the simplest solution isn’t the best nor is it future proof.

Observation 6: PDCP concatenation, though effective for overhead reduction, does not allow for in-flow packet prioritization and causes additional delay.

Observation 7: Overhead optimization proposals can be utilized to also achieve inter-flow synchronization for applications with tight latency limits.

Proposal 1: Study overhead reduction and prioritization techniques to enhance QoS in high-throughput and low-latency scenarios.

Proposal 2: Study modifications at RLC or MAC layers to achieve both overhead reduction (in terms of bytes and processing) and in-flow packet prioritization.

Proposal 3: Study configurations to achieve flow synchronization under latency bounds using overhead optimization principles.

Proposal 4: Study overhead optimization protocols without compromising user privacy and security needs.

Proposal 5: Discuss and confirm overhead reduction and user plane optimizations at L2 and higher layers using the SID description in RP-213380 [4].

# References

- R2-166475, “Concatenation for NR”, RAN2#95bis, Samsung, Oct 2016

- “Mobile Marketing Statistics Compilation 2021, “https://www.smartinsights.com/mobile-marketing/mobile-marketing-analytics/mobile-marketing-statistics/,” Smart Insights.com, Mar 2021

- “Packet Data Convergence Protocol (PDCP) Specification”, 3GPP TS 38.323 V16.4.0, Jun 2021

- RP-213380, “New SID on user plane enhancements”, Apple, Dec 2021

- "A Roadmap for Transmission Control Protocol (TCP) Specification Documents”, RFC 7414, Duke et. al., Feb. 2015

- “Requirements for Internet hosts – Communication Layers”, RFC 1122, R. Braden, Oct 1989

- RP-212357, Sub-DRB Differentiation for Enhanced Performance and Special Packet Prioritization, Apple, Sep 2021