3GPP TSG-RAN Meeting #95-e	RP-220xxx

# Online, March 17-23, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2924 | rev | 2 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of ePowSav in TS 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CATT |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | R17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of R17 UE Power Saving for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: RAN2#116 e-meeting: - Add a new SIB-X for TRS/CSI-RS configuration for idle/inactive-mode UEs. - Add parameters according to RAN1’s LS (R2-2111246). - Add parameters related to paging subgrouping RAN2#116bis e-meeting: - Add parameters according to RAN1’s LS(R2-2200095). - Updated subgroup related configurations according to the e-meeting agreements RAN2#117 e-meeting: - Add parameters according to RAN1’s LS(R2-2202111). - Add RLM/BFD relaxation criteria and configuration - Added further details on TRS configuration - Added further details on PEI/subgrouping configuration RAN#95e - Add the configuration and reporting of the state of RLM relaxation and BFD relaxation separately. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R17 UE Power Saving for NR is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.2.2.4.x, 5.3.5.5.7, 5.3.5.5.9, 5.3.5.9, 5.3.5.10, 5.3.7.2, 5.3.7.3, 5.3.13.2, 5.7, 6.2.2, 6.3.1, 6.3.2, 6.4, 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR0417 TS 38.304 CR0227 TS 38.306 CRxxxx TS 38.133 CRxxxx TS 38.213 CRxxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

ACK	Acknowledgement

AM	Acknowledged Mode

ARQ	Automatic Repeat Request

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

BAP	Backhaul Adaptation Protocol

BCD	Binary Coded Decimal

BFD	Beam Failure Detection

BH	Backhaul

BLER	Block Error Rate

BWP	Bandwidth Part

CA	Carrier Aggregation

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

CAPC	Channel Access Priority Class

CBR	Channel Busy Ratio

CCCH	Common Control Channel

CG	Cell Group

CHO	Conditional Handover

CLI	Cross Link Interference

CMAS	Commercial Mobile Alert Service

CP	Control Plane

CPC	Conditional PSCell Change

C-RNTI	Cell RNTI

CSI	Channel State Information

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DFN	Direct Frame Number

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

DL-SCH	Downlink Shared Channel

DM-RS	Demodulation Reference Signal

DRB	(user) Data Radio Bearer

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

EN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to EPC

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRA/EPC	E-UTRA connected to EPC

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDD	Frequency Division Duplex

FFS	For Further Study

GERAN	GSM/EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GSM	Global System for Mobile Communications

HARQ	Hybrid Automatic Repeat Request

HRNN	Human Readable Network Name

IAB	Integrated Access and Backhaul

IAB-DU	IAB-node DU

IAB-MT	IAB Mobile Termination

IDC	In-Device Coexistence

IE	Information element

IMSI	International Mobile Subscriber Identity

kB	Kilobyte (1000 bytes)

L1	Layer 1

L2	Layer 2

L3	Layer 3

LBT	Listen Before Talk

MAC	Medium Access Control

MCG	Master Cell Group

MDT	Minimization of Drive Tests

MIB	Master Information Block

MPE	Maximum Permissible Exposure

MR-DC	Multi-Radio Dual Connectivity

N/A	Not Applicable

NE-DC	NR E-UTRA Dual Connectivity

(NG)EN-DC	E-UTRA NR Dual Connectivity (covering E-UTRA connected to EPC or 5GC)

NGEN-DC	E-UTRA NR Dual Connectivity with E-UTRA connected to 5GC

NID	Network Identifier

NPN	Non-Public Network

NR-DC	NR-NR Dual Connectivity

NR/5GC	NR connected to 5GC

PCell	Primary Cell

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

PEI	Paging Early Indication

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated Non-Public Network

posSIB	Positioning SIB

PRS	Positioning Reference Signal

PSCell	Primary SCG Cell

PWS	Public Warning System

QoS	Quality of Service

RAN	Radio Access Network

RAT	Radio Access Technology

RLC	Radio Link Control

RLM	Radio Link Monitoring

RMTC	RSSI Measurement Timing Configuration

RNA	RAN-based Notification Area

RNTI	Radio Network Temporary Identifier

ROHC	Robust Header Compression

RPLMN	Registered Public Land Mobile Network

RRC	Radio Resource Control

RS	Reference Signal

SBAS	Satellite Based Augmentation System

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SI	System Information

SIB	System Information Block

SL	Sidelink

SLSS	Sidelink Synchronisation Signal

SNPN	Stand-alone Non-Public Network

SpCell	Special Cell

SRB	Signalling Radio Bearer

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

TAG	Timing Advance Group

TDD	Time Division Duplex

TM	Transparent Mode

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UP	User Plane

In the ASN.1, lower case may be used for some (parts) of the above abbreviations e.g. c-RNTI.

<Next modification>

##### 5.2.2.4.x	Actions upon reception of SIBx

Upon receiving SIBx, the UE shall:

1>	if the UE has stored at least one segment of SIBx and the value tag of SIBx has changed since a previous segment was stored:

2>	discard all stored segments;

1>	store the segment;

1>	if all segments have been received:

2>	assemble SIBx-IEs from the received segments.

The UE should discard any stored segments for SIBx if the complete SIBx has not been assembled within a period of 3 hours. The UE shall discard any stored segments for SIBx upon cell (re-) selection.

<Next modification>

##### 5.3.5.5.7	SpCell Configuration

The UE shall:

1>	if the SpCellConfig contains the rlf-TimersAndConstants:

2>	configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6;

1>	else if rlf-TimersAndConstants is not configured for this cell group:

2>	if any DAPS bearer is configured:

3>	use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1;

2>	else

3>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	if the SpCellConfig contains spCellConfigDedicated:

2>	configure the SpCell in accordance with the spCellConfigDedicated;

2>	consider the bandwidth part indicated in firstActiveUplinkBWP-Id if configured to be the active uplink bandwidth part;

2>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id if configured to be the active downlink bandwidth part;

2>	if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated:

3>	stop timer T310 for the corresponding SpCell, if running;

3>	stop timer T312 for the corresponding SpCell, if running;

3>	reset the counters N310 and N311.

1>	if the SpCellConfig contains the lowMobilityEvaluationConnected:

2>	the UE may perform the evaluation of the low mobility criterion for this cell group as specified in 5.7.X.1;

1>	if the SpCellConfig contains the goodServingCellEvaluationRLM:

2>	the UE may perform the evaluation of the good serving cell quality criterion for this cell group as specified in 5.7.X.2;

1>	if the SpCellConfig contains the goodServingCellEvaluationBFD:

2>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.X.2;

<Next modification>

##### 5.3.5.5.9	SCell Addition/Modification

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	if the sCellState is included:

3>	configure lower layers to consider the SCell to be in activated state;

2>	else:

3>	configure lower layers to consider the SCell to be in deactivated state;

2>	for each measId included in the measIdList within VarMeasConfig:

3>	if SCells are not applicable for the associated measurement; and

3>	if the concerned SCell is included in cellsTriggeredList defined within the VarMeasReportList for this measId:

4>	remove the concerned SCell from cellsTriggeredList defined within the VarMeasReportList for this measId;

2>	if the SCellConfig contains the goodServingCellEvaluationBFD:

3>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.X.2.

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated;

2>	if the sCellToAddModList was received in an RRCReconfiguration message including reconfigurationWithSync, or received in an RRCResume message, or received in an RRCReconfiguration message including reconfigurationWithSync embedded in an RRCResume message or embedded in an RRCReconfiguration message or embedded in an E-UTRA RRCConnectionReconfiguration message or embedded in an E-UTRA RRCConnectionResume message:

3>	if the sCellState is included:

4>	configure lower layers to consider the SCell to be in activated state;

3>	else:

4>	configure lower layers to consider the SCell to be in deactivated state.

2>	if the SCellConfig contains the goodServingCellEvaluationBFD:

3>	the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.X.2.

<Next modification>

#### 5.3.5.9	Other configuration

The UE shall:

1>	if the received otherConfig includes the delayBudgetReportingConfig:

2>	if delayBudgetReportingConfig is set to setup:

3>	consider itself to be configured to send delay budget reports in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to send delay budget reports and stop timer T342, if running.

1>	if the received otherConfig includes the overheatingAssistanceConfig:

2>	if overheatingAssistanceConfig is set to setup:

3>	consider itself to be configured to provide overheating assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide overheating assistance information and stop timer T345, if running;

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

1>	if the received otherConfig includes the drx-PreferenceConfig:

2>	if drx-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on DRX parameters for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on DRX parameters for power saving for the cell group and stop timer T346a associated with the cell group, if running;

1>	if the received otherConfig includes the maxBW-PreferenceConfig:

2>	if maxBW-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum aggregated bandwidth for power saving for the cell group and stop timer T346b associated with the cell group, if running;

1>	if the received otherConfig includes the maxCC-PreferenceConfig:

2>	if maxCC-PreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of secondary component carriers for power saving for the cell group and stop timer T346c associated with the cell group, if running;

1>	if the received otherConfig includes the maxMIMO-LayerPreferenceConfig:

2>	if maxMIMO-LayerPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the maximum number of MIMO layers for power saving for the cell group and stop timer T346d associated with the cell group, if running;

1>	if the received otherConfig includes the minSchedulingOffsetPreferenceConfig:

2>	if minSchedulingOffsetPreferenceConfig is set to setup:

3>	consider itself to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide its preference on the minimum scheduling offset for cross-slot scheduling for power saving for the cell group and stop timer T346e associated with the cell group, if running;

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 1:	The UE is requested to attempt to have valid detailed location information available whenever sending a measurement report for which it is configured to include available detailed location information. The UE may not succeed e.g. because the user manually disabled the GPS hardware, due to no/poor satellite coverage. Further details, e.g. regarding when to activate GNSS, are up to UE implementation.

1>	if the received otherConfig includes the btNameList:

2>	if btNameList is set to setup, include available Bluetooth measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

1>	if the received otherConfig includes the sl-AssistanceConfigNR:

2>	consider itself to be configured to provide configured grant assistance information for NR sidelink communication in accordance with 5.7.4;

1>	if the received otherConfig includes the referenceTimePreferenceReporting:

2>	consider itself to be configured to provide UE reference time assistance information in accordance with 5.7.4;

1>	else:

2>	consider itself not to be configured to provide UE reference time assistance information;

1>	if the received otherConfig includes the rlm-RelaxationReportingConfig:

2>	consider itself to be configured to report the relaxation state of RLM measurements with 5.7.4;

1>	else:

2>	consider itself not to be configured to report the relaxation state of RLM measurements;

1>	if the received otherConfig includes the bfd-RelaxationReportingConfig:

2>	consider itself to be configured to report the relaxation state of BFD measurements with 5.7.4;

1>	else:

2>	consider itself not to be configured to report the relaxation state of BFD measurements;

#### 5.3.5.10	MR-DC release

The UE shall:

1>	as a result of MR-DC release triggered by E-UTRA or NR:

2>	release SRB3, if established, as specified in 5.3.5.6.2;

2>	release measConfig associated with SCG;

2>	if the UE is configured with NR SCG:

3>	release the SCG configuration as specified in clause 5.3.5.4;

3>	release otherConfig associated with the SCG, if configured;

3>	stop timers T346a, T346b, T346c, T346d, T346e, T34x and T34y associated with the SCG, if running;

3>	release bap-Config associated with the SCG, if configured;

3>	release iab-IP-AddressConfigurationList associated with the SCG, if configured;

2>	else if the UE is configured with E-UTRA SCG:

3>	release the SCG configuration as specified in TS 36.331 [10], clause 5.3.10.19 to release the E-UTRA SCG;

<Next modification>

### 5.3.7	RRC connection re-establishment

#### 5.3.7.1	General

Figure 5.3.7.1-1: RRC connection re-establishment, successful

Figure 5.3.7.1-2: RRC re-establishment, fallback to RRC establishment, successful

The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB setup or, for IAB, SRB2, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to clause 5.3.3.4.

The network applies the procedure e.g as follows:

-	When AS security has been activated and the network retrieves or verifies the UE context:

-	to re-activate AS security without changing algorithms;

-	to re-establish and resume the SRB1;

-	When UE is re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context:

-	to discard the stored AS Context and release all RBs and BH RLC channels;

-	to fallback to establish a new RRC connection.

If AS security has not been activated, the UE shall not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'other'. If AS security has been activated, but SRB2 and at least one DRB or, for IAB, SRB2, are not setup, the UE does not initiate the procedure but instead moves to RRC_IDLE directly, with release cause 'RRC connection failure'.

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or

1>	upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2; or

1>	upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with subclause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] subclause 5.3.11.3 in NE-DC; or

1>	upon reconfiguration with sync failure of the SCG while MCG transmission is suspended in accordance with subclause 5.3.5.8.3; or

1>	upon SCG change failure while MCG transmission is suspended in accordance with TS 36.331 [10] subclause 5.3.5.7a; or

1>	upon SCG configuration failure while MCG transmission is suspended in accordance with subclause 5.3.5.8.2 in NR-DC or in accordance with TS 36.331 [10] subclause 5.3.5.5 in NE-DC; or

1>	upon integrity check failure indication from SCG lower layers concerning SRB3 while MCG is suspended; or

1>	upon T316 expiry, in accordance with sub-clause 5.7.3b.5.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	if UE is not configured with conditionalReconfiguration:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, except SRB0;

2>	release the MCG SCell(s), if configured;

2>	if MR-DC is configured:

3>	perform MR-DC release, as specified in clause 5.3.5.10;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

2>	release idc-AssistanceConfig, if configured;

2>	release btNameList, if configured;

2>	release wlanNameList, if configured;

2>	release sensorNameList, if configured;

2>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

2>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

2>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

2>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

2>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured stop timer T346e associated with the MCG, if running;

2>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T34x associated with the MCG, if running;

2>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T34y associated with the MCG, if running;

2>	release releasePreferenceConfig, if configured stop timer T346f, if running;

2>	release onDemandSIB-Request if configured, and stop timer T350, if running;

2>	release referenceTimePreferenceReporting, if configured;

2>	release sl-AssistanceConfigNR, if configured;

2>	release obtainCommonLocation, if configured;

1>	if any DAPS bearer is configured:

2>	reset the source MAC and release the source MAC configuration;

2>	for each DAPS bearer:

3>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

3>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

2>	for each SRB:

3>	release the PDCP entity for the source SpCell;

3>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the source SpCell;

2>	release the physical channel configuration for the source SpCell;

2>	discard the keys used in the source SpCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20].

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in VarConditionalReconfig:

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:	It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

1>	else:

2>	if UE is configured with conditionalReconfiguration:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T34x associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T34y associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	suspend all RBs, except SRB0;

2>	remove all the entries within VarConditionalReconfig, if any;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 2:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

<Next modification>

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, or for NR sidelink communication/V2X sidelink communication as specified in sub-clause 5.3.13.1a) requests the resume of a suspended RRC connection.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	if the resumption occurs after release with redirect with mpsPriorityIndication:

3>	set the resumeCause to mps-PriorityAccess;

2>	else:

3>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNA update as specified in 5.3.13.8:

2>	if an emergency service is ongoing:

NOTE:	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

3>	set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [23];

3>	if the access attempt is barred:

4>	set the variable pendingRNA-Update to true;

4>	the procedure ends;

1>	if the UE is in NE-DC or NR-DC:

2>	if the UE does not support maintaining SCG configuration upon connection resumption:

3>	release the MR-DC related configurations (i.e., as specified in 5.3.5.10) from the UE Inactive AS context, if stored;

1>	if the UE does not support maintaining the MCG SCell configurations upon connection resumption:

2>	release the MCG SCell(s) from the UE Inactive AS context, if stored;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

1>	apply the default SRB1 configuration as specified in 9.2.1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	release delayBudgetReportingConfig from the UE Inactive AS context, if stored;

1>	stop timer T342, if running;

1>	release overheatingAssistanceConfig from the UE Inactive AS context, if stored;

1>	stop timer T345, if running;

1>	release idc-AssistanceConfig from the UE Inactive AS context, if stored;

1>	release drx-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346a, if running;

1>	release maxBW-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346b, if running;

1>	release maxCC-PreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346c, if running;

1>	release maxMIMO-LayerPreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346d, if running;

1>	release minSchedulingOffsetPreferenceConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T346e, if running;

1>	release rlm-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T34x, if running;

1>	release bfd-RelaxationReportingConfig for all configured cell groups from the UE Inactive AS context, if stored;

1>	stop all instances of timer T34y, if running;

1>	release releasePreferenceConfig from the UE Inactive AS context, if stored;

1>	release wlanNameList from the UE Inactive AS context, if stored;

1>	release btNameList from the UE Inactive AS context, if stored;

1>	release sensorNameList from the UE Inactive AS context, if stored;

1>	release obtainCommonLocation from the UE Inactive AS context, if stored;

1>	stop timer T346f, if running;

1>	release referenceTimePreferenceReporting from the UE Inactive AS context, if stored;

1>	release sl-AssistanceConfigNR from the UE Inactive AS context, if stored;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T319;

1>	set the variable pendingRNA-Update to false;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

<Next modification>

## 5.7	Other

<Partially omitted>

### 5.7.4	UE Assistance Information

#### 5.7.4.1	General

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is for the UE to inform the network of:

-	its delay budget report carrying desired increment/decrement in the connected mode DRX cycle length, or;

-	its overheating assistance information, or;

-	its IDC assistance information, or;

-	its preference on DRX parameters for power saving, or;

-	its preference on the maximum aggregated bandwidth for power saving, or;

-	its preference on the maximum number of secondary component carriers for power saving, or;

-	its preference on the maximum number of MIMO layers for power saving, or;

-	its preference on the minimum scheduling offset for cross-slot scheduling for power saving, or;

-	its preference on the RRC state, or;

-	configured grant assistance information for NR sidelink communication, or;

-	its preference in being provisioned with reference time information, or;

-	its relaxation state for RLM measurements, or;

-	its relaxation state for BFD measurements.

#### 5.7.4.2	Initiation

A UE capable of providing delay budget report in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide delay budget report and upon change of delay budget preference.

A UE capable of providing overheating assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting internal overheating, or upon detecting that it is no longer experiencing an overheating condition.

A UE capable of providing IDC assistance information in RRC_CONNECTED may initiate the procedure if it was configured to do so, upon detecting IDC problem if the UE did not transmit an IDC assistance information since it was configured to provide IDC indications, or upon change of IDC problem information.

A UE capable of providing its preference on DRX parameters of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a preference on DRX parameters and upon change of its preference on DRX parameters.

A UE capable of providing its preference on the maximum aggregated bandwidth of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum aggregated bandwidth preference and upon change of its maximum aggregated bandwidth preference.

A UE capable of providing its preference on the maximum number of secondary component carriers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of secondary component carriers preference and upon change of its maximum number of secondary component carriers preference.

A UE capable of providing its preference on the maximum number of MIMO layers of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a maximum number of MIMO layers preference and upon change of its maximum number of MIMO layers preference.

A UE capable of providing its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving in RRC_CONNECTED may initiate the procedure in several cases, if it was configured to do so, including upon having a minimum scheduling offset preference and upon change of its minimum scheduling offset preference.

A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state.

A UE capable of providing configured grant assistance information for NR sidelink communication in RRC_CONNECTED may initiate the procedure in several cases, including upon being configured to provide traffic pattern information and upon change of traffic patterns.

A UE capable of providing an indication of its preference in being provisioned with reference time information may initiate the procedure upon being configured to provide this indication, or if it was configured to provide this indication and upon change of its preference.

A UE capable of relaxing its RLM measurements of a cell group in RRC_CONNECTED state shall initiate the procedure upon being configured to do so, and upon change of its relaxation state for RLM measurements in RRC_CONNECTED state.

A UE capable of relaxing its BFD measurements in serving cells of a cell group in RRC_CONNECTED shall initiate the procedure upon being configured to do so, and upon change of its relaxation state for BFD measurements in RRC_CONNECTED state.

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

2>	if the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report; or

2>	if the current delay budget is different from the one indicated in the last transmission of the UEAssistanceInformation message including delayBudgetReport and timer T342 is not running:

3>	start or restart timer T342 with the timer value set to the delayBudgetReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide a delay budget report;

1>	if configured to provide overheating assistance information:

2>	if the overheating condition has been detected and T345 is not running; or

2>	if the current overheating assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message including overheatingAssistance and timer T345 is not running:

3>	start timer T345 with the timer value set to the overheatingIndicationProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide overheating assistance information;

1>	if configured to provide IDC assistance information:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current IDC assistance information is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

NOTE 1:	The term "IDC problems" refers to interference issues applicable across several subframes/slots where not necessarily all the subframes/slots are affected.

NOTE 2:	For the frequencies on which a serving cell or serving cells is configured that is activated, IDC problems consist of interference issues that the UE cannot solve by itself, during either active data exchange or upcoming data activity which is expected in up to a few hundred milliseconds.
For frequencies on which a SCell or SCells is configured that is deactivated, reporting IDC problems indicates an anticipation that the activation of the SCell or SCells would result in interference issues that the UE would not be able to solve by itself.
For a non-serving frequency, reporting IDC problems indicates an anticipation that if the non-serving frequency or frequencies became a serving frequency or serving frequencies then this would result in interference issues that the UE would not be able to solve by itself.

1>	if configured to provide its preference on DRX parameters of a cell group for power saving:

2>	if the UE has a preference on DRX parameters of the cell group and the UE did not transmit a UEAssistanceInformation message with drx-Preference for the cell group since it was configured to provide its preference on DRX parameters of the cell group for power saving; or

2>	if the current drx-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including drx-Preference for the cell group and timer T346a associated with the cell group is not running:

3>	start the timer T346a with the timer value set to the drx-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current drx-Preference;

1>	if configured to provide its preference on the maximum aggregated bandwidth of a cell group for power saving:

2>	if the UE has a preference on the maximum aggregated bandwidth of the cell group and the UE did not transmit a UEAssistanceInformation message with maxBW-Preference for the cell group since it was configured to provide its preference on the maximum aggregated bandwidth of the cell group for power saving; or

2>	if the current maxBW-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxBW-Preference for the cell group and timer T346b associated with the cell group is not running:

3>	start the timer T346b with the timer value set to the maxBW-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxBW-Preference;

1>	if configured to provide its preference on the maximum number of secondary component carriers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of secondary component carriers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxCC-Preference for the cell group since it was configured to provide its preference on the maximum number of secondary component carriers of the cell group for power saving; or

2>	if the current maxCC-Preference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxCC-Preference for the cell group and timer T346c associated with the cell group is not running:

3>	start the timer T346c with the timer value set to the maxCC-PreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxCC-Preference;

1>	if configured to provide its preference on the maximum number of MIMO layers of a cell group for power saving:

2>	if the UE has a preference on the maximum number of MIMO layers of the cell group and the UE did not transmit a UEAssistanceInformation message with maxMIMO-LayerPreference for the cell group since it was configured to provide its preference on the maximum number of MIMO layers of the cell group for power saving; or

2>	if the current maxMIMO-LayerPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including maxMIMO-LayerPreference for the cell group and timer T346d associated with the cell group is not running:

3>	start the timer T346d with the timer value set to the maxMIMO-LayerPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current maxMIMO-LayerPreference;

1>	if configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of a cell group for power saving:

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling of the cell group and the UE did not transmit a UEAssistanceInformation message with minSchedulingOffsetPreference for the cell group since it was configured to provide its preference on the minimum scheduling offset for cross-slot scheduling of the cell group for power saving; or

2>	if the current minSchedulingOffsetPreference information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message including minSchedulingOffsetPreference for the cell group and timer T346e associated with the cell group is not running:

3>	start the timer T346e with the timer value set to the minSchedulingOffsetPreferenceProhibitTimer of the cell group;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current minSchedulingOffsetPreference;

1>	if configured to provide its release preference and timer T346f is not running:

2>	if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or

2>	if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state:

3>	start timer T346f with the timer value set to the releasePreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference;

1>	if configured to provide configured grant assistance information for NR sidelink communication:

2>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide configured grant assistance information for NR sidelink communication;

1>	if configured to provide preference in being provisioned with reference time information:

2>	if the UE did not transmit a UEAssistanceInformation message with referenceTimeInfoPreference since it was configured to provide preference; or

2>	if the UE's preference changed from the last time UE initiated transmission of the UEAssistanceInformation message including referenceTimeInfoPreference:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide preference in being provisioned with reference time information;

1>	if configured to provide the relaxation state of RLM measurements of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with rlm-MeasRelaxationState since it was configured to provide the relaxation state of RLM measurements for the cell group; or

2>	if the relaxation state of RLM measurements for the cell group has changed since the last transmission of the UEAssistanceInformation message including rlm-MeasRelaxationState of the cell group and timer T34x associated with the cell group is not running:

3>	start timer T34x with the timer value set to the rlm-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of RLM measurements of the cell group;

1>	if configured to provide the relaxation state of BFD measurements of serving cells of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with bfd-MeasRelaxationState since it was configured to provide the relaxation state of BFD measurements for the cell group; or

2>	if the relaxation state of BFD measurements in any serving cell of the cell group has changed since the last transmission of the UEAssistanceInformation message including bfd-MeasRelaxationState of the cell group and timer T34y associated with the cell group is not running:

3>	start timer T34y with the timer value set to the bfd-RelaxtionReportingProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the relaxation state of BFD measurements of serving cells of the cell group.

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2 or 5.3.5.3;

2>	set delayBudgetReport to type1 according to a desired value;

1>	if transmission of the UEAssistanceInformation message is initiated to provide overheating assistance information according to 5.7.4.2 or 5.3.5.3;

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR1;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR1;

3>	if the UE prefers to temporarily reduce maximum aggregated bandwidth of FR2:

4>	include reducedMaxBW-FR2 in the OverheatingAssistance IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all downlink carriers of FR2;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE prefers to be temporarily configured across all uplink carriers of FR2;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR1-DL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR1-UL to the number of maximum MIMO layers of each serving cell operating on FR1 the UE prefers to be temporarily configured in uplink;

3>	if the UE prefers to temporarily reduce the number of maximum MIMO layers of each serving cell operating on FR2:

4>	include reducedMaxMIMO-LayersFR2 in the OverheatingAssistance IE;

4>	set reducedMIMO-LayersFR2-DL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in downlink;

4>	set reducedMIMO-LayersFR2-UL to the number of maximum MIMO layers of each serving cell operating on FR2 the UE prefers to be temporarily configured in uplink;

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedMaxCCs, reducedMaxBW-FR1, reducedMaxBW-FR2, reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in OverheatingAssistance IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR;

3>	for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2>	if there is at least one supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include victimSystemType for each UL CA combination included in affectedCarrierFreqCombList;

3>	if the UE sets victimSystemType to wlan or bluetooth:

4>	include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

3>	else:

4>	optionally include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

NOTE 1:	When sending an UEAssistanceInformation message to inform the IDC problems, the UE includes all IDC assistance information (rather than providing e.g. the changed part(s) of the IDC assistance information).

NOTE 2:	Upon not anymore experiencing a particular IDC problem that the UE previously reported, the UE provides an IDC indication with the modified contents of the UEAssistanceInformation message (e.g. by not including the IDC assistance information in the idc-Assistance field).

1>	if transmission of the UEAssistanceInformation message is initiated to provide drx-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include drx-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on DRX parameters for the cell group:

3>	if the UE has a preference for the long DRX cycle:

4>	include preferredDRX-LongCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the DRX inactivity timer:

4>	include preferredDRX-InactivityTimer in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX cycle:

4>	include preferredDRX-ShortCycle in the DRX-Preference IE and set it to the preferred value;

3>	if the UE has a preference for the short DRX timer:

4>	include preferredDRX-ShortCycleTimer in the DRX-Preference IE and set it to the preferred value;

2>	else (if the UE has no preference on DRX parameters for the cell group):

3>	do not include preferredDRX-LongCycle, preferredDRX-InactivityTimer, preferredDRX-ShortCycle and preferredDRX-ShortCycleTimer in the DRX-Preference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxBW-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxBW-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum aggregated bandwidth for the cell group:

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR1:

4>	include reducedMaxBW-FR1 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR1 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR1 in the cell group;

3>	if the UE prefers to reduce the maximum aggregated bandwidth of FR2:

4>	include reducedMaxBW-FR2 in the MaxBW-Preference IE;

4>	set reducedBW-DL to the maximum aggregated bandwidth the UE desires to have configured across all downlink carriers of FR2 in the cell group;

4>	set reducedBW-UL to the maximum aggregated bandwidth the UE desires to have configured across all uplink carriers of FR2 in the cell group;

2>	else (if the UE has no preference on the maximum aggregated bandwidth for the cell group):

3>	do not include reducedMaxBW-FR1 and reducedMaxBW-FR2 in the MaxBW-Preference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxCC-Preference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxCC-Preference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of secondary component carriers for the cell group:

3>	include reducedMaxCCs in the MaxCC-Preference IE;

3>	set reducedCCsDL to the number of maximum SCells the UE desires to have configured in downlink in the cell group;

3>	set reducedCCsUL to the number of maximum SCells the UE desires to have configured in uplink in the cell group;

2>	else (if the UE has no preference on the maximum number of secondary component carriers for the cell group):

3>	do not include reducedMaxCCs in the MaxCC-Preference IE;

NOTE 3:	The UE can implicitly indicate a preference for NR SCG release by reporting the maximum aggregated bandwidth preference for power saving of the cell group, if configured, as zero for both FR1 and FR2, and by reporting the maximum number of secondary component carriers for power saving of the cell group, if configured, as zero for both uplink and downlink.

1>	if transmission of the UEAssistanceInformation message is initiated to provide maxMIMO-LayerPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include maxMIMO-LayerPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the maximum number of MIMO layers for the cell group:

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR1:

4>	include reducedMaxMIMO-LayersFR1 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR1-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR1 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR1-UL to the preferred maximum number of uplink MIMO layers of each FR1 serving cell that the UE operates on in the cell group;

3>	if the UE prefers to reduce the number of maximum MIMO layers of each serving cell operating on FR2:

4>	include reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

4>	set reducedMIMO-LayersFR2-DL to the preferred maximum number of downlink MIMO layers of each BWP of each FR2 serving cell that the UE operates on in the cell group;

4>	set reducedMIMO-LayersFR2-UL to the preferred maximum number of uplink MIMO layers of each FR2 serving cell that the UE operates on in the cell group;

2>	else (if the UE has no preference on the maximum number of MIMO layers for the cell group):

3>	do not include reducedMaxMIMO-LayersFR1 and reducedMaxMIMO-LayersFR2 in the MaxMIMO-LayerPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide minSchedulingOffsetPreference of a cell group for power saving according to 5.7.4.2 or 5.3.5.3:

2>	include minSchedulingOffsetPreference in the UEAssistanceInformation message;

2>	if the UE has a preference on the minimum scheduling offset for cross-slot scheduling for the cell group:

3>	if the UE has a preference for the value of K0 (TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK0-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK0-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK0-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K0 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK0-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K0;

3>	if the UE has a preference for the value of K2 (TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling with 15 kHz SCS:

4>	include preferredK2-SCS-15kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 30 kHz SCS:

4>	include preferredK2-SCS-30kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 60 kHz SCS:

4>	include preferredK2-SCS-60kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

3>	if the UE has a preference for the value of K2 for cross-slot scheduling with 120 kHz SCS:

4>	include preferredK2-SCS-120kHz in the MinSchedulingOffsetPreference IE and set it to the desired value of K2;

2>	else (if the UE has no preference on the minimum scheduling offset for cross-slot scheduling for the cell group):

3>	do not include preferredK0 and preferredK2 in the MinSchedulingOffsetPreference IE;

1>	if transmission of the UEAssistanceInformation message is initiated to provide a release preference according to 5.7.4.2 or 5.3.5.3:

2>	include releasePreference in the UEAssistanceInformation message;

2>	set preferredRRC-State to the desired RRC state on transmission of the UEAssistanceInformation message;

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication of preference in being provisioned with reference time information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference in being provisioned with reference time information:

3>	set referenceTimeInfoPreference to true;

2>	else:

3>	set referenceTimeInfoPreference to false;

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of RLM measurements of a cell group according to 5.7.4.2:

2>	if the UE performs RLM measurement relaxation on the cell group:

3>	set the rlm-MeasRelaxationState to true;

2> else:

3>	set the rlm-MeasRelaxationState to false;

1>	if transmission of the UEAssistanceInformation message is initiated to provide the relaxation state of BFD measurements of a cell group:

2>	for each serving cell of the cell group:

3> if the UE performs BFD measurement relaxation on this serving cell according to TS 38.133 [14]:

4>	set the n-th bit of bfd-MeasRelaxationState to ‘1’, where n is equal to the servCellIndex value + 1 of the serving cell;

3> else:

4>	set the n-th bit of bfd-MeasRelaxationState to ‘1’, where n is equal to the servCellIndex value + 1 of the serving cell

The UE shall set the contents of the UEAssistanceInformation message for configured grant assistance information for NR sidelink communication:

1>	if configured to provide configured grant assistance information for NR sidelink communication:

2>	include the sl-UE-AssistanceInformationNR;

NOTE 4:	It is up to UE implementation when and how to trigger configured grant assistance information for NR sidelink communication.

The UE shall:

1>	if the procedure was triggered to provide configured grant assistance information for NR sidelink communication by an NR RRCReconfiguration message that was embedded within an E-UTRA RRCConnectionReconfiguration:

2>	submit the UEAssistanceInformation to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured:

3>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

2>	else:

3>	submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

1>	else if the UE is in NR-DC:

2>	if the UE assistance configuration that triggered this UE assistance information is associated with the SCG:

3>	if SRB3 is configured:

4>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

3>	else:

4>	submit the UEAssistanceInformation message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3;

2>	else:

3>	submit the UEAssistanceInformation message via SRB1 to lower layers for transmission;

1>	else:

2>	submit the UEAssistanceInformation message to lower layers for transmission.

<Next modification>

### 5.7.x	RLM/BFD relaxation

#### 5.7.X.1	Relaxed measurement criterion for low mobility

The relaxed measurement criterion for UE with low mobility in RRC_CONNECTED is fulfilled when:

-	(SS-RSRPRef – SS-RSRP) < SSearchDeltaP-Connected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the SpCell based on SSB (dB).

-	SS-RSRPRef = reference L3 RSRP measurement of the SpCell based on SSB (dB), set as follows:

-	After receiving low mobility criterion configuration, or

-  After MAC of the CG successfully completes a Random Access procedure after applying a reconfigurationWithSync in spCellConfig of the CG while low mobility criterion is configured, or

-	If (SS-RSRP - SS-RSRPRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-Connected:

-	The UE shall set the value of SS-RSRPRef to the current SS-RSRP value of the SpCell.

#### 5.7.X.2	Relaxed measurement criterion for good serving cell quality

The relaxed measurement criterion of good serving cell quality for RLM starts to be evaluated after receiving the good serving cell quality criterion configuration and is fulfilled when the downlink radio link quality on the configured RLM-RS resource is evaluated to be better than the threshold Qin+XdB,, wherein

- Qin is specified in section 8.1 of TS 38.133 [14].

- X is the parameter offset in goodServingCellEvaluationRLM.

The relaxed measurement criterion of good serving cell quality for BFD starts to be evaluated after receiving the good serving cell quality criterion configuration and is fulfilled when the downlink radio link quality on the configured BFD-RS resource is evaluated to be better than the threshold Qin+XdB,, wherein

- Qin is specified in section 8.1 of TS 38.133 [14].

- X is the parameter offset in goodServingCellEvaluationBFD.

<Next modification>

### 6.2.2	Message definitions

<Partially omitted>

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-IEs ::=              SEQUENCE {

radioBearerConfig                       RadioBearerConfig                                                      OPTIONAL, -- Need M

secondaryCellGroup                      OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Cond SCG

measConfig                              MeasConfig                                                             OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                           OPTIONAL,

nonCriticalExtension                    RRCReconfiguration-v1530-IEs                                           OPTIONAL

}

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

fullConfig                              ENUMERATED {true}                                                      OPTIONAL, -- Cond FullConfig

dedicatedNAS-MessageList                SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message                     OPTIONAL, -- Cond nonHO

masterKeyUpdate                         MasterKeyUpdate                                                        OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery                  OCTET STRING (CONTAINING SIB1)                                         OPTIONAL, -- Need N

dedicatedSystemInformationDelivery      OCTET STRING (CONTAINING SystemInformation)                            OPTIONAL, -- Need N

otherConfig                             OtherConfig                                                            OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1540-IEs                                           OPTIONAL

}

RRCReconfiguration-v1540-IEs ::=        SEQUENCE {

otherConfig-v1540                       OtherConfig-v1540                                                      OPTIONAL, -- Need M

nonCriticalExtension                    RRCReconfiguration-v1560-IEs                                           OPTIONAL

}

RRCReconfiguration-v1560-IEs ::=         SEQUENCE {

mrdc-SecondaryCellGroupConfig            SetupRelease { MRDC-SecondaryCellGroupConfig }                        OPTIONAL,   -- Need M

radioBearerConfig2                       OCTET STRING (CONTAINING RadioBearerConfig)                           OPTIONAL,   -- Need M

sk-Counter                               SK-Counter                                                            OPTIONAL,   -- Need N

nonCriticalExtension                     RRCReconfiguration-v1610-IEs                                          OPTIONAL

}

RRCReconfiguration-v1610-IEs ::=        SEQUENCE {

otherConfig-v1610                       OtherConfig-v1610                                                    OPTIONAL, -- Need M

bap-Config-r16                          SetupRelease { BAP-Config-r16 }                                      OPTIONAL, -- Need M

iab-IP-AddressConfigurationList-r16     IAB-IP-AddressConfigurationList-r16                                  OPTIONAL, -- Need M

conditionalReconfiguration-r16          ConditionalReconfiguration-r16                                       OPTIONAL, -- Need M

daps-SourceRelease-r16                  ENUMERATED{true}                                                     OPTIONAL, -- Need N

t316-r16                                SetupRelease {T316-r16}                                              OPTIONAL, -- Need M

needForGapsConfigNR-r16                 SetupRelease {NeedForGapsConfigNR-r16}                               OPTIONAL, -- Need M

onDemandSIB-Request-r16                 SetupRelease { OnDemandSIB-Request-r16 }                             OPTIONAL, -- Need M

dedicatedPosSysInfoDelivery-r16         OCTET STRING (CONTAINING PosSystemInformation-r16-IEs)               OPTIONAL, -- Need N

sl-ConfigDedicatedNR-r16                SetupRelease {SL-ConfigDedicatedNR-r16}                              OPTIONAL, -- Need M

sl-ConfigDedicatedEUTRA-Info-r16        SetupRelease {SL-ConfigDedicatedEUTRA-Info-r16}                      OPTIONAL, -- Need M

targetCellSMTC-SCG-r16                  SSB-MTC                                                              OPTIONAL, -- Need S

nonCriticalExtension                    RRCReconfiguration-v17xy-IEs                                          OPTIONAL

}

RRCReconfiguration-v17xy-IEs ::=        SEQUENCE {

otherConfig-v17xy                       OtherConfig-v17xy                                                    OPTIONAL, -- Need M

nonCriticalExtension                    SEQUENCE {}                                                          OPTIONAL

}

MRDC-SecondaryCellGroupConfig ::=       SEQUENCE {

mrdc-ReleaseAndAdd                      ENUMERATED {true}                                                     OPTIONAL,   -- Need N

mrdc-SecondaryCellGroup                 CHOICE {

nr-SCG                                  OCTET STRING  (CONTAINING RRCReconfiguration),

eutra-SCG                               OCTET STRING

}

}

BAP-Config-r16 ::=                      SEQUENCE {

bap-Address-r16                         BIT STRING (SIZE (10))                                    OPTIONAL, -- Need M

defaultUL-BAP-RoutingID-r16             BAP-RoutingID-r16                                         OPTIONAL, -- Need M

defaultUL-BH-RLC-Channel-r16            BH-RLC-ChannelID-r16                                      OPTIONAL, -- Need M

flowControlFeedbackType-r16             ENUMERATED {perBH-RLC-Channel, perRoutingID, both}        OPTIONAL, -- Need R

...

}

MasterKeyUpdate ::=                 SEQUENCE {

keySetChangeIndicator           BOOLEAN,

nextHopChainingCount            NextHopChainingCount,

nas-Container                   OCTET STRING                                                     OPTIONAL,    -- Cond securityNASC

...

}

OnDemandSIB-Request-r16 ::=                  SEQUENCE {

onDemandSIB-RequestProhibitTimer-r16         ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30}

}

T316-r16 ::=         ENUMERATED {ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms1000, ms1500, ms2000}

IAB-IP-AddressConfigurationList-r16 ::= SEQUENCE {

iab-IP-AddressToAddModList-r16      SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressConfiguration-r16 OPTIONAL, -- Need N

iab-IP-AddressToReleaseList-r16     SEQUENCE (SIZE(1..maxIAB-IP-Address-r16)) OF IAB-IP-AddressIndex-r16         OPTIONAL, -- Need N

...

}

IAB-IP-AddressConfiguration-r16 ::=     SEQUENCE {

iab-IP-AddressIndex-r16                 IAB-IP-AddressIndex-r16,

iab-IP-Address-r16                      IAB-IP-Address-r16                                                OPTIONAL,  -- Need M

iab-IP-Usage-r16                        IAB-IP-Usage-r16                                                  OPTIONAL,  -- Need M

iab-donor-DU-BAP-Address-r16            BIT STRING (SIZE(10))                                             OPTIONAL,  -- Need M

...

}

SL-ConfigDedicatedEUTRA-Info-r16 ::=            SEQUENCE {

sl-ConfigDedicatedEUTRA-r16                    OCTET STRING                                              OPTIONAL,  -- Need M

sl-TimeOffsetEUTRA-List-r16                    SEQUENCE (SIZE (8)) OF SL-TimeOffsetEUTRA-r16             OPTIONAL    -- Need M

}

SL-TimeOffsetEUTRA-r16 ::=        ENUMERATED {ms0, ms0dot25, ms0dot5, ms0dot625, ms0dot75, ms1, ms1dot25, ms1dot5, ms1dot75,

ms2, ms2dot5, ms3, ms4, ms5, ms6, ms8, ms10, ms20}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

bap-Config This field is used to configure the BAP entity for IAB nodes.

bap-Address Indicates the BAP address of an IAB-node. The BAP address of an IAB-node cannot be changed once configured to the BAP entity.

conditionalReconfiguration Configuration of candidate target SpCell(s) and execution condition(s) for conditional handover or conditional PSCell change. For conditional PSCell change, this field may only be present in an RRCReconfiguration message for intra-SN PSCell change. The network does not configure a UE with both conditional PCell change and conditional PSCell change simultaneously. The field is absent if any DAPS bearer is configured or if the masterCellGroup includes ReconfigurationWithSync. For conditional PSCell change, the field is absent if the secondaryCellGroup includes ReconfigurationWithSync. The RRCReconfiguration message contained in DLInformationTransferMRDC cannot contain the field conditionalReconfiguration for conditional PSCell change.

daps-SourceRelease Indicates to UE that the source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released.

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

dedicatedPosSysInfoDelivery This field is used to transfer SIBPos to the UE in RRC_CONNECTED.

dedicatedSIB1-Delivery This field is used to transfer SIB1 to the UE. The field has the same values as the corresponding configuration in servingCellConfigCommon.

dedicatedSystemInformationDelivery This field is used to transfer SIB6, SIB7, SIB8 to the UE with an active BWP with no common serach space configured. For UEs in RRC_CONNECTED, this field is used to transfer the SIBs requested on-demand.

defaultUL-BAP-RoutingID This field is used for IAB-node to configure the default uplink Routing ID, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BAP-RoutingID can be (re-)configured when IAB-node IP address for F1-C related traffic changes. This field is mandatory only for IAB-node bootstrapping.

defaultUL-BH-RLC-Channel This field is used for IAB-nodes to configure the default uplink BH RLC channel, which is used by IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. The defaultUL-BH-RLC-Channel can be (re-)configured when IAB-node IP address for F1-C related traffic changes, and the new IP address is anchored at a different IAB-donor-DU. This field is mandatory for IAB-node bootstrapping. If the IAB-MT is operating in EN-DC, the default uplink BH RLC channel is referring to an RLC channel on the SCG; Otherwise, it is referring to an RLC channel on the MCG.

flowControlFeedbackType This field is only used for IAB-node that support hop-by-hop flow control to configure the type of flow control feedback. Value perBH-RLC-Channel indicates that the IAB-node shall provide flow control feedback per BH RLC channel, value perRoutingID indicates that the IAB-node shall provide flow control feedback per routing ID, and value both indicates that the IAB-node shall provide flow control feedback both per BH RLC channel and per routing ID.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1.

iab-IP-Address This field is used to provide the IP address information for IAB-node.

iab-IP-AddressIndex This field is used to identify a configuration of an IP address.

iab-IP-AddressToAddModList List of IP addresses allocated for IAB-node to be added and modified.

iab-IP-AddressToReleaseList List of IP address allocated for IAB-node to be released.

iab-IP-Usage This field is used to indicate the usage of the assigned IP address. If this field is not configured, the assigned IP address is used for all traffic.

iab-donor-DU-BAP-Address This field is used to indicate the BAP address of the IAB-donor-DU where the IP address is anchored.

keySetChangeIndicator Indicates whether UE shall derive a new KgNB. If reconfigurationWithSync is included, value true indicates that a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. Value false indicates that the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

mrdc-ReleaseAndAdd This field indicates that the current SCG configuration is released and a new SCG is added at the same time.

mrdc-SecondaryCellGroup Includes an RRC message for SCG configuration in NR-DC or NE-DC. For NR-DC (nr-SCG), mrdc-SecondaryCellGroup contains the RRCReconfiguration message as generated (entirely) by SN gNB. In this version of the specification, the RRC message can only include fields secondaryCellGroup, otherConfig, conditionalReconfiguration and measConfig. For NE-DC (eutra-SCG), mrdc-SecondaryCellGroup includes the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRC message can only include the field scg-Configuration.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS security after inter-system handover to NR. The content is defined in TS 24.501 [23].

needForGapsConfigNR Configuration for the UE to report measurement gap requirement information of NR target bands in the RRCReconfigurationComplete and RRCResumeComplete message.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

onDemandSIB-Request If the field is present, the UE is allowed to request SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5.

onDemandSIB-RequestProhibitTimer Prohibit timer for requesting SIB(s) on-demand while in RRC_CONNECTED according to clause 5.2.2.3.5. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

otherConfig Contains configuration related to other configurations. When configured for the SCG, only fields drx-PreferenceConfig, maxBW-PreferenceConfig, maxCC-PreferenceConfig, maxMIMO-LayerPreferenceConfig, minSchedulingOffsetPreferenceConfig, btNameList, wlanNameList, sensorNameList and obtainCommonLocation can be included.

radioBearerConfig Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. This field can only be used if the UE supports NR-DC or NE-DC.

secondaryCellGroup Configuration of secondary cell group ((NG)EN-DC or NR-DC).

sk-Counter A counter used upon initial configuration of S-KgNB or S-KeNB, as well as upon refresh of S-KgNB or S-KeNB. This field is always included either upon initial configuration of an NR SCG or upon configuration of the first RB with keyToUse set to secondary, whichever happens first. This field is absent if there is neither any NR SCG nor any RB with keyToUse set to secondary.

sl-ConfigDedicatedNR This field is used to provide the dedicated configurations for NR sidelink communication.

sl-ConfigDedicatedEUTRA-Info This field includes the E-UTRA RRCConnectionReconfiguration as specified in TS 36.331 [10]. In this version of the specification, the E-UTRA RRCConnectionReconfiguration can only includes sidelink related fields for V2X sidelink communication, i.e. sl-V2X-ConfigDedicated, sl-V2X-SPS-Config, measConfig and/or otherConfig.

sl-TimeOffsetEUTRA This field indicates the possible time offset to (de)activation of V2X sidelink transmission after receiving DCI format 3_1 used for scheduling V2X sidelink communication. Value ms0dpt75 corresponds to 0.75ms, ms1 corresponds to 1ms and so on. The network includes this field only when sl-ConfigDedicatedEUTRA is configured.

targetCellSMTC-SCG The SSB periodicity/offset/duration configuration of target cell for NR PSCell addition and SN change. When UE receives this field, UE applies the configuration based on the timing reference of NR PCell for PSCell addition and PSCell change for the case of no reconfiguration with sync of MCG, and UE applies the configuration based on the timing reference of target NR PCell for the case of reconfiguration with sync of MCG. If both this field and the smtc in secondaryCellGroup -> SpCellConfig -> reconfigurationWithSync are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

t316 Indicates the value for timer T316 as described in clause 7.1. Value ms50 corresponds to 50 ms, value ms100 corresponds to 100 ms and so on. This field can be configured only if the UE is configured with split SRB1 or SRB3.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is absent in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case masterCellGroup includes ReconfigurationWithSync and RadioBearerConfig includes SecurityConfig with SecurityAlgorithmConfig, indicating a change of the AS security algorithms associated to the master key. If ReconfigurationWithSync is included for other cases, this field is optionally present, need N. Otherwise the field is absent. |
| FullConfig | The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise. |
| SCG | The field is mandatory present in: - an RRCReconfiguration message contained in an RRCResume message (or in an RRCConnectionResume message, see TS 36.331 [10]), - an RRCReconfiguration message contained in an RRCConnectionReconfiguration message, see TS 36.331 [10], which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation). The field is optional present, Need M, in: - an RRCReconfiguration message transmitted on SRB3, - an RRCReconfiguration message contained in another RRCReconfiguration message (or in an RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1 - an RRCReconfiguration message contained in another RRCReconfiguration message which is contained in DLInformationTransferMRDC transmitted on SRB3 (as a response to ULInformationTransferMRDC including an MCGFailureInformation) Otherwise, the field is absent |

<Partially omitted>

#### –	SystemInformation

The SystemInformation message is used to convey one or more System Information Blocks or Positioning System Information Blocks. All the SIBs or posSIBs included are transmitted with the same periodicity.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channels: BCCH

Direction: Network to UE

SystemInformation message

-- ASN1START

-- TAG-SYSTEMINFORMATION-START

SystemInformation ::=               SEQUENCE {

criticalExtensions                  CHOICE {

systemInformation                   SystemInformation-IEs,

criticalExtensionsFuture-r16    CHOICE {

posSystemInformation-r16        PosSystemInformation-r16-IEs,

criticalExtensionsFuture        SEQUENCE {}

}

}

}

SystemInformation-IEs ::=           SEQUENCE {

sib-TypeAndInfo                     SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {

sib2                                SIB2,

sib3                                SIB3,

sib4                                SIB4,

sib5                                SIB5,

sib6                                SIB6,

sib7                                SIB7,

sib8                                SIB8,

sib9                                SIB9,

...,

sib10-v1610                         SIB10-r16,

sib11-v1610                         SIB11-r16,

sib12-v1610                         SIB12-r16,

sib13-v1610                         SIB13-r16,

sib14-v1610                         SIB14-r16,

sibx-v17xy							SIBx-r17

},

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

-- TAG-SYSTEMINFORMATION-STOP

-- ASN1STOP

<Next modification>

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

UEAssistanceInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

ueAssistanceInformation             UEAssistanceInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

UEAssistanceInformation-IEs ::=     SEQUENCE {

delayBudgetReport                   DelayBudgetReport                   OPTIONAL,

lateNonCriticalExtension            OCTET STRING                        OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1540-IEs   OPTIONAL

}

DelayBudgetReport::=                CHOICE {

type1                               ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,msMinus80, msMinus60, msMinus40,

msMinus20, ms0, ms20,ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

...

}

UEAssistanceInformation-v1540-IEs ::= SEQUENCE {

overheatingAssistance               OverheatingAssistance               OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v1610-IEs   OPTIONAL

}

OverheatingAssistance ::=           SEQUENCE {

reducedMaxCCs                       ReducedMaxCCs-r16                   OPTIONAL,

reducedMaxBW-FR1                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxBW-FR2                    ReducedMaxBW-FRx-r16                OPTIONAL,

reducedMaxMIMO-LayersFR1            SEQUENCE {

reducedMIMO-LayersFR1-DL            MIMO-LayersDL,

reducedMIMO-LayersFR1-UL            MIMO-LayersUL

} OPTIONAL,

reducedMaxMIMO-LayersFR2            SEQUENCE {

reducedMIMO-LayersFR2-DL            MIMO-LayersDL,

reducedMIMO-LayersFR2-UL            MIMO-LayersUL

} OPTIONAL

}

ReducedAggregatedBandwidth ::= ENUMERATED {mhz0, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400}

UEAssistanceInformation-v1610-IEs ::= SEQUENCE {

idc-Assistance-r16                  IDC-Assistance-r16                  OPTIONAL,

drx-Preference-r16                  DRX-Preference-r16                  OPTIONAL,

maxBW-Preference-r16                MaxBW-Preference-r16                OPTIONAL,

maxCC-Preference-r16                MaxCC-Preference-r16                OPTIONAL,

maxMIMO-LayerPreference-r16         MaxMIMO-LayerPreference-r16         OPTIONAL,

minSchedulingOffsetPreference-r16   MinSchedulingOffsetPreference-r16   OPTIONAL,

releasePreference-r16               ReleasePreference-r16               OPTIONAL,

sl-UE-AssistanceInformationNR-r16   SL-UE-AssistanceInformationNR-r16   OPTIONAL,

referenceTimeInfoPreference-r16     BOOLEAN                             OPTIONAL,

nonCriticalExtension                UEAssistanceInformation-v17xy-IEs	OPTIONAL

}

UEAssistanceInformation-v17xy-IEs ::= SEQUENCE {

rlm-MeasRelaxationState-r17      BOOLEAN                           OPTIONAL,

bfd-MeasRelaxationState-r17      BIT STRING (SIZE (32))            OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                       OPTIONAL

}

IDC-Assistance-r16 ::=                  SEQUENCE {

affectedCarrierFreqList-r16             AffectedCarrierFreqList-r16               OPTIONAL,

affectedCarrierFreqCombList-r16         AffectedCarrierFreqCombList-r16           OPTIONAL,

...

}

AffectedCarrierFreqList-r16 ::= SEQUENCE (SIZE (1.. maxFreqIDC-r16)) OF AffectedCarrierFreq-r16

AffectedCarrierFreq-r16 ::=     SEQUENCE {

carrierFreq-r16                 ARFCN-ValueNR,

interferenceDirection-r16       ENUMERATED {nr, other, both, spare}

}

AffectedCarrierFreqCombList-r16 ::= SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqComb-r16

AffectedCarrierFreqComb-r16 ::=     SEQUENCE {

affectedCarrierFreqComb-r16         SEQUENCE (SIZE (2..maxNrofServingCells)) OF  ARFCN-ValueNR    OPTIONAL,

victimSystemType-r16                VictimSystemType-r16

}

VictimSystemType-r16 ::=    SEQUENCE {

gps-r16                     ENUMERATED {true}        OPTIONAL,

glonass-r16                 ENUMERATED {true}        OPTIONAL,

bds-r16                     ENUMERATED {true}        OPTIONAL,

galileo-r16                 ENUMERATED {true}        OPTIONAL,

navIC-r16                   ENUMERATED {true}        OPTIONAL,

wlan-r16                    ENUMERATED {true}        OPTIONAL,

bluetooth-r16               ENUMERATED {true}        OPTIONAL,

...

}

DRX-Preference-r16 ::=              SEQUENCE {

preferredDRX-InactivityTimer-r16    ENUMERATED {

ms0, ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60, ms80,

ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare9, spare8,

spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL,

preferredDRX-LongCycle-r16          ENUMERATED {

ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512,

ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, spare12, spare11, spare10,

spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycle-r16         ENUMERATED {

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,

preferredDRX-ShortCycleTimer-r16    INTEGER (1..16)    OPTIONAL

}

MaxBW-Preference-r16 ::=            SEQUENCE {

reducedMaxBW-FR1-r16                ReducedMaxBW-FRx-r16                     OPTIONAL,

reducedMaxBW-FR2-r16                ReducedMaxBW-FRx-r16                     OPTIONAL

}

MaxCC-Preference-r16 ::=            SEQUENCE {

reducedMaxCCs-r16                   ReducedMaxCCs-r16                        OPTIONAL

}

MaxMIMO-LayerPreference-r16 ::=     SEQUENCE {

reducedMaxMIMO-LayersFR1-r16        SEQUENCE {

reducedMIMO-LayersFR1-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR1-UL-r16        INTEGER (1..4)

} OPTIONAL,

reducedMaxMIMO-LayersFR2-r16        SEQUENCE {

reducedMIMO-LayersFR2-DL-r16        INTEGER (1..8),

reducedMIMO-LayersFR2-UL-r16        INTEGER (1..4)

} OPTIONAL

}

MinSchedulingOffsetPreference-r16 ::= SEQUENCE {

preferredK0-r16                       SEQUENCE {

preferredK0-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}              OPTIONAL,

preferredK0-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL,

preferredK0-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}             OPTIONAL

}                                                                                  OPTIONAL,

preferredK2-r16                       SEQUENCE {

preferredK2-SCS-15kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-30kHz-r16             ENUMERATED {sl1, sl2, sl4, sl6}             OPTIONAL,

preferredK2-SCS-60kHz-r16             ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL,

preferredK2-SCS-120kHz-r16            ENUMERATED {sl2, sl4, sl8, sl12}            OPTIONAL

}                                                                                 OPTIONAL

}

ReleasePreference-r16 ::=           SEQUENCE {

preferredRRC-State-r16              ENUMERATED {idle, inactive, connected, outOfConnected}

}

ReducedMaxBW-FRx-r16 ::=            SEQUENCE {

reducedBW-DL-r16                    ReducedAggregatedBandwidth,

reducedBW-UL-r16                    ReducedAggregatedBandwidth

}

ReducedMaxCCs-r16 ::=               SEQUENCE {

reducedCCsDL-r16                    INTEGER (0..31),

reducedCCsUL-r16                    INTEGER (0..31)

}

SL-UE-AssistanceInformationNR-r16 ::= SEQUENCE (SIZE (1..maxNrofTrafficPattern-r16)) OF SL-TrafficPatternInfo-r16

SL-TrafficPatternInfo-r16::=          SEQUENCE {

trafficPeriodicity-r16                ENUMERATED {ms20, ms50, ms100, ms200, ms300, ms400, ms500, ms600, ms700, ms800, ms900, ms1000},

timingOffset-r16                      INTEGER (0..10239),

messageSize-r16                       BIT STRING (SIZE (8)),

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16

}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

affectedCarrierFreqList Indicates a list of NR carrier frequencies that are affected by IDC problem.

affectedCarrierFreqCombList Indicates a list of NR carrier frequencie combinations that are affected by IDC problems due to Inter-Modulation Distortion and harmonics from NR when configured with UL CA.

bfd-MeasRelaxationState Indicates the relaxation state of BFD measurements. Each bit corresponds to a serving cell of the cell group. A serving cell is mapped to the (servCellIndex+1)-th bit. A bit that is set to 1 indicates that the UE performs BFD measurements relaxation on the serving cell mapped on this bit. Otherwise it is set to 0.

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX.

interferenceDirection Indicates the direction of IDC interference. Value nr indicates that only NR is victim of IDC interference, value other indicates that only another radio is victim of IDC interference and value both indicates that both NR and another radio are victims of IDC interference. The other radio refers to either the ISM radio or GNSS (see TR 36.816 [44]).

minSchedulingOffsetPreference Indicates the UE's preferences on minimumSchedulingOffset of cross-slot scheduling for power saving.

preferredDRX-InactivityTimer Indicates the UE's preferred DRX inactivity timer length for power saving. Value in ms (milliSecond). ms0 corresponds to 0, ms1 corresponds to 1 ms, ms2 corresponds to 2 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the DRX inactivity timer. If secondary DRX group is configured, the preferredDRX-InactivityTimer only applies to the default DRX group.

preferredDRX-LongCycle Indicates the UE's preferred long DRX cycle length for power saving. Value in ms. ms10 corresponds to 10ms, ms20 corresponds to 20 ms, ms32 corresponds to 32 ms, and so on. If preferredDRX-ShortCycle is provided, the value of preferredDRX-LongCycle shall be a multiple of the preferredDRX-ShortCycle value. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the long DRX cycle.

preferredDRX-ShortCycle Indicates the UE's preferred short DRX cycle length for power saving. Value in ms. ms2 corresponds to 2ms, ms3 corresponds to 3 ms, ms4 corresponds to 4 ms, and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle.

preferredDRX-ShortCycleTimer Indicates the UE's preferred short DRX cycle timer for power saving. Value in multiples of preferredDRX-ShortCycle. A value of 1 corresponds to preferredDRX-ShortCycle, a value of 2 corresponds to 2 * preferredDRX-ShortCycle and so on. If the field is absent from the DRX-Preference IE, it is interpreted as the UE having no preference for the short DRX cycle timer. A preference for the short DRX cycle is indicated when a preference for the short DRX cycle timer is indicated.

preferredK0 Indicates the UE's preferred value of k0 (slot offset between DCI and its scheduled PDSCH - see TS 38.214 [19], clause 5.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k0 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k0 for cross-slot scheduling.

preferredK2 Indicates the UE's preferred value of k2 (slot offset between DCI and its scheduled PUSCH - see TS 38.214 [19], clause 6.1.2.1) for cross-slot scheduling for power saving. Value is defined for each subcarrier spacing (numerology) in units of slots. sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, sl4 corresponds to 4 slots, and so on. If a value for a subcarrier spacing is absent, it is interpreted as the UE having no preference on k2 for cross-slot scheduling for that subcarrier spacing. If the field is absent from the MinSchedulingOffsetPreference IE, it is interpreted as the UE having no preference on k2 for cross-slot scheduling.

preferredRRC-State Indicates the UE's preferred RRC state. The value idle is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_IDLE. The value inactive is indicated if the UE prefers to be released from RRC_CONNECTED and transition to RRC_INACTIVE. The value connected is indicated if the UE prefers to revert an earlier indication to leave RRC_CONNECTED state. The value outOfConnected is indicated if the UE prefers to be released from RRC_CONNECTED and has no preferred RRC state to transition to. The value connected can only be indicated if the UE is configured with connectedReporting.

reducedBW-FR1 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR1, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR1. The aggregated bandwidth across all downlink carrier(s) of FR1 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR1. The aggregated bandwidth across all uplink carrier(s) of FR1 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR1. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR1. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR1 of both the NR MCG and the SCG. This maximum aggregated bandwidth only includes carriers of FR1 of the SCG in (NG)EN-DC. Value mhz0 is not used when indicated to address overheating. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR1 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedBW-FR2 Indicates the UE's preference on reduced configuration corresponding to the maximum aggregated bandwidth across all downlink carrier(s) and across all uplink carrier(s) of FR2, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cell(s) operating on FR2. The aggregated bandwidth across all downlink carrier(s) of FR2 is the sum of bandwidth of active downlink BWP(s) across all activated downlink carrier(s) of FR2. The aggregated bandwidth across all uplink carrier(s) of FR2 is the sum of bandwidth of active uplink BWP(s) across all activated uplink carrier(s) of FR2. If the field is absent from the MaxBW-Preference IE or the OverheatingAssistance IE, it is interpreted as the UE having no preference on the maximum aggregated bandwidth of FR2. When indicated to address overheating, this maximum aggregated bandwidth includes carrier(s) of FR2 of both the NR MCG and the NR SCG. This maximum aggregated bandwidth only includes carriers of FR2 of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum aggregated bandwidth includes carrier(s) of FR2 of the cell group that this UE assistance information is associated with. The aggregated bandwidth can only range up to the current active configuration when indicated to address power savings.

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes both SCells of the NR MCG and PSCell/SCells of the SCG. This maximum number only includes PSCell/SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell/SCells of the cell group that this UE assistance information is associated with. The maximum number of downlink SCells can only range up to the current active configuration when indicated to address power savings.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating or power saving. When indicated to address overheating, this maximum number includes both SCells of the NR MCG and PSCell/SCells of the SCG. This maximum number only includes PSCell/SCells of the SCG in (NG)EN-DC. When indicated to address power saving, this maximum number includes PSCell/SCells of the cell group that this UE assistance information is associated with. The maximum number of uplink SCells can only range up to the current active configuration when indicated to address power savings.

reducedMIMO-LayersFR1-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR1-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR1 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR1. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR1 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-DL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink MIMO layers of each serving cell operating on FR2 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2. The maximum number of downlink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated downlink carrier(s) of FR2 in the cell group when indicated to address power savings.

reducedMIMO-LayersFR2-UL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink MIMO layers of each serving cell operating on FR2 indicated by the field, to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2. The maximum number of uplink MIMO layers can only range up to the maximum number of MIMO layers configured across all activated uplink carrier(s) of FR2 in the cell group when indicated to address power savings.

referenceTimeInfoPreference Indicates whether the UE prefers being provisioned with the timing information specified in the IE ReferenceTimeInfo.

rlm-MeasRelaxationState Indicates the relaxation state of RLM measurements. Value true indicates that the UE performs relaxation of RLM measurements, and value false indicates that the UE does not perform relaxation of RLM measurements.

sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, which is unique for different destination and cast type.

sl-UE-AssistanceInformationNR Indicates the traffic characteristic of sidelink logical channel(s), specified in the IE SL-TrafficPatternInfo, that are setup for NR sidelink communication.

type1 Indicates the preferred amount of increment/decrement to the long DRX cycle length with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on.

victimSystemType Indicate the list of victim system types to which IDC interference is caused from NR when configured with UL CA. Value gps, glonass, bds, galileo and navIC indicates the type of GNSS. Value wlan indicates WLAN and value bluetooth indicates Bluetooth.

SL-TrafficPatternInfo field descriptions

messageSize Indicates the maximum TB size based on the observed traffic pattern. The value refers to the index of TS 38.321 [3], table 6.1.3.1-2.

timingOffset This field indicates the estimated timing for a packet arrival in a sidelink logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

trafficPeriodicity This field indicates the estimated data arrival periodicity in a sidelink logical channel. Value ms20 corresponds to 20 ms, ms50 corresponds to 50 ms and so on.

<Next modification>

### 6.3.1	System information blocks

<Partially omitted>

#### –	SIBx

SIBx contains configurations of TRS resources for idle/inactive UEs.

SIBx information element

-- ASN1START

-- TAG-SIBx-START

SIBx-r17 ::=                SEQUENCE {

segmentNumber-r17             INTEGER (0..1),

segmentType-r17               ENUMERATED {notLastSegment, lastSegment},

segmentContainer-r17          OCTET STRING

}

SIBx-IEs-r17 ::=                      SEQUENCE {

trs-ResouceSetConfig-r17            SEQUENCE (SIZE (1..maxNrofTRS-ResourceSets-r17)) OF TRS-ResourceSet-r17        OPTIONAL,         -- Need R

validityDuration-r17                ENUMERATED {t1, t2, t4, t8, t16, t32, t64, t128, t256, t512, spare6, spare5, spare4, spare3, spare2, spare1}                              OPTIONAL,        -- Need S

lateNonCriticalExtension            OCTET STRING                                                                   OPTIONAL,

...

}

TRS-ResourceSet-r17 ::=             SEQUENCE {

powerControlOffsetSS-r17                      ENUMERATED{db-3, db0, db3, db6},

scramblingID-Info-r17                       CHOICE {

scramblingIDforCommon-r17                 ScramblingId,

scramblingIDperResourceListWith2-r17           SEQUENCE (SIZE (2)) OF ScramblingId,

scramblingIDperResourceListWith4-r17           SEQUENCE (SIZE (4)) OF ScramblingId,

...

},

firstOFDMSymbolInTimeDomain-r17           INTEGER (0..9),

startingRB-r17                            INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs-r17                               INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

ssb-Index-r17                             SSB-Index,

periodicityAndOffset-r17                  CHOICE {

slots10                                   INTEGER (0..9),

slots20                                   INTEGER (0..19),

slots40                                   INTEGER (0..39),

slots80                                   INTEGER (0..79)

},

frequencyDomainAllocation-r17             BIT STRING (SIZE (4)),

indBitID-r17                              INTEGER (0..5),

nrofResources-r17                          ENUMERATED{n2,n4},

...

}

-- TAG-SIBx-STOP

-- ASN1STOP

SIBx field descriptions

trs-ResouceSetConfig RS configuration of TRS occasion(s) for idle/inactive UE(s), in terms of a list of N>=1 NZP TRS resource set(s). The maximum number of TRS resource sets configured by higher layer is 64. If a TRS resource is configured, the L1 based availability indication is always enabled based on that configuration. A UE which acquired SIB-X with a TRS configuration but did not yet receive an associated L1-based availability indication considers the configured TRS as unavailable.

TRS-ResourceSet Common configuration parameters for the TRS resource set.

validityDuration The valid time duration at least for a paging PDCCH based L1 availability indication, time unit is one default paging cycle. When the validity duration is not configured, UE assumes a default time duration to be 2 default paging cycle(s).

TRS-ResourceSet field descriptions

firstOFDMSymbolInTimeDomain The index of the first OFDM symbol in the PRB used for TRS in a slot. The field indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4.

frequencyDomainAllocation Indicates the offset of the first RE to RE#0 in a RB in row1.

indBitID The index of the associated bit in TRS availability indication field in DCI. Each TRS resource set is configured with an ID i for the association with (i+1)-th indication bit in TRS availability indication field in DCI.

nrofRBs Number of PRBs across which corresponding TRS resource spans.

nrofResources The number of TRS resources for a TRS resource set.

periodicityAndOffset The periodicity and slot offset (slot) for periodicTRS. It is used to determine the location of the first slot of TRS resource set. The periodicity value slots10 corresponds to 10 slots, value slots20 corresponds to 20 slots, and so on.

powerControlOffsetSS Power offset (dB) of NZP CSI-RS RE to SSS RE.

scramblingID-Info One or more scrambling IDs are configured for a TRS resource set. If a common scrambling ID is configured, it applies to all the TRS resources within the TRS resource set. Otherwise, each TRS resource within the TRS resource set is provided with a scrambling ID. If the number of TRS resources for the TRS resource set is 2, scramblingIDperResourceListWith2-r17 is configured, while scramblingIDperResourceListWith4-r17 is configured for the case that the number of TRS resources for the TRS resource set is 4.

ssb-Index The index of reference SSB with which quasi-collocation information is provided as specified in TS 38.214 [19] subclause 5.1.5.

startingRB The PRB index where corresponding TRS resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

<Next modification>

### 6.3.2	Radio resource control information elements

<Partially omitted>

#### –	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells).

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                        SEQUENCE {

cellGroupId                                CellGroupId,

rlc-BearerToAddModList                     SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig                        OPTIONAL,   -- Need N

rlc-BearerToReleaseList                    SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity                  OPTIONAL,   -- Need N

mac-CellGroupConfig                        MAC-CellGroupConfig                                                     OPTIONAL,   -- Need M

physicalCellGroupConfig                    PhysicalCellGroupConfig                                                 OPTIONAL,   -- Need M

spCellConfig                               SpCellConfig                                                            OPTIONAL,   -- Need M

sCellToAddModList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig                       OPTIONAL,   -- Need N

sCellToReleaseList                         SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex                        OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent                ENUMERATED {true}                                                   OPTIONAL    -- Cond BWP-Reconfig

]],

[[

bap-Address-r16                            BIT STRING (SIZE (10))                                                  OPTIONAL,   -- Need M

bh-RLC-ChannelToAddModList-r16             SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelConfig-r16 OPTIONAL,   -- Need N

bh-RLC-ChannelToReleaseList-r16            SEQUENCE (SIZE(1..maxBH-RLC-ChannelID-r16)) OF BH-RLC-ChannelID-r16     OPTIONAL,   -- Need N

f1c-TransferPath-r16                       ENUMERATED {lte, nr, both}                                              OPTIONAL,   -- Need M

simultaneousTCI-UpdateList1-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousTCI-UpdateList2-r16            SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList1-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

simultaneousSpatial-UpdatedList2-r16       SEQUENCE (SIZE (1..maxNrofServingCellsTCI-r16)) OF ServCellIndex        OPTIONAL,   -- Need R

uplinkTxSwitchingOption-r16                ENUMERATED {switchedUL, dualUL}                                         OPTIONAL,   -- Need R

uplinkTxSwitchingPowerBoosting-r16         ENUMERATED {enabled}                                                    OPTIONAL    -- Need R

]],

[[

reportUplinkTxDirectCurrentTwoCarrier-r16  ENUMERATED {true}                                                       OPTIONAL    -- Need N

}

-- Serving cell specific MAC and PHY parameters for a SpCell:

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

... ,

[[

lowMobilityEvaluationConnected-r17         SEQUENCE {

s-SearchDeltaP-Connected-r17            ENUMERATED {FFS},

t-SearchDeltaP-Connected-r17            ENUMERATED {FFS}

}                                                                                            OPTIONAL,   -- Need R

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,    -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL    -- Need R

]]

}

GoodServingCellEvaluation-r17 ::=      SEQUENCE {

offset-r17                                   CHOICE {

offsetFR1-r17                                 ENUMERATED {db2, db4, db6, db8},

offsetFR2-r17                                 ENUMERATED {db2, db4, db6, db8}

}                                                                                                    OPTIONAL   -- Need S

}

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]]

}

DAPS-UplinkPowerConfig-r16 ::=      SEQUENCE {

p-DAPS-Source-r16                   P-Max,

p-DAPS-Target-r16                   P-Max,

uplinkPowerSharingDAPS-Mode-r16     ENUMERATED {semi-static-mode1, semi-static-mode2, dynamic }

}

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Cond DRX-Config2

]] ,

[[

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                                OPTIONAL    -- Need R

]]}

-- TAG-CELLGROUPCONFIG-STOP

-- ASN1STOP

Editor’s NOTE: Whether serving cell quality criterion is configured per Scell for BFD needs RAN4 confirmation.

Editor’s NOTE: Current text assumes the low mobility criterion is configured commonly for RLM and BFD. It is FFS whether the low mobility criterion can be configured independently for RLM and BFD.

Editor’s NOTE: Values and range of SearchDeltaP-Connected and t-SearchDeltaP-Connected are still FFS in RAN4.

CellGroupConfig field descriptions

bap-Address BAP address of the parent node in cell group.

bh-RLC-ChannelToAddModList Configuration of the backhaul RLC entities and the corresponding MAC Logical Channels to be added and modified.

bh-RLC-ChannelToReleaseList List of the backhaul RLC entities and the corresponding MAC Logical Channels to be released.

f1c-TransferPath The F1-C transfer path that an EN-DC IAB-MT should use for transferring F1-C packets to the IAB-donor-CU. If IAB-MT is configured with lte, IAB-MT can only use LTE leg for F1-C transfer. If IAB-MT is configured with nr, IAB-MT can only use NR leg for F1-C transfer. If IAB-MT is configured with both, it is up to IAB-MT to select an LTE leg or a NR leg for F1-C transfer. If the field is not configured, the IAB node uses the NR leg as the default one.

mac-CellGroupConfig MAC parameters applicable for the entire cell group.

rlc-BearerToAddModList Configuration of the MAC Logical Channel, the corresponding RLC entities and association with radio bearers.

reportUplinkTxDirectCurrent Enables reporting of uplink and supplementary uplink Direct Current location information upon BWP configuration and reconfiguration. This field is only present when the BWP configuration is modified or any serving cell is added or removed. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message. If UE is configured with SUL carrier, UE reports both UL and SUL Direct Current locations.

reportUplinkTxDirectCurrentTwoCarrier Enables reporting of uplink Direct Current location information when the UE is configured with uplink intra-band CA with two carriers. This field is absent in the IE CellGroupConfig when provided as part of RRCSetup message.

rlmInSyncOutOfSyncThreshold BLER threshold pair index for IS/OOS indication generation, see TS 38.133 [14], table 8.1.1-1. n1 corresponds to the value 1. When the field is absent, the UE applies the value 0. Whenever this is reconfigured, UE resets N310 and N311, and stops T310, if running. Network does not include this field.

sCellState Indicates whether the SCell shall be considered to be in activated state upon SCell configuration.

sCellToAddModList List of secondary serving cells (SCells) to be added or modified.

sCellToReleaseList List of secondary serving cells (SCells) to be released.

secondaryDRX-GroupConfig The field is used to indicate whether the SCell belongs to the secondary DRX group. All serving cells in the secondary DRX group shall belong to one Frequency Range and all serving cells in the legacy DRX group shall belong to another Frequency Range.

simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 List of serving cells which can be updated simultaneously for TCI relation with a MAC CE. The simultaneousTCI-UpdateList1 and simultaneousTCI-UpdateList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

simultaneousSpatial-UpdatedList1, simultaneousSpatial-UpdatedList2 List of serving cells which can be updated simultaneously for spatial relation with a MAC CE. The simultaneousSpatial-UpdatedList1 and simultaneousSpatial-UpdatedList2 shall not contain same serving cells. Network should not configure serving cells that are configured with a BWP with two different values for the coresetPoolIndex in these lists.

spCellConfig Parameters for the SpCell of this cell group (PCell of MCG or PSCell of SCG).

uplinkTxSwitchingOption Indicates which option is configured for dynamic UL Tx switching for inter-band UL CA or (NG)EN-DC. The field is set to switchedUL if network configures option 1 as specified in TS 38.214 [19], or dualUL if network configures option 2 as specified in TS 38.214 [19]. Network always configures UE with a value for this field in inter-band UL CA case and (NG)EN-DC case where UE supports dynamic UL Tx switching.

uplinkTxSwitchingPowerBoosting Indicates whether the UE is allowed to enable 3dB boosting on the maximum output power for transmission on carrier2 under the operation state in which 2-port transmission can be supported on carrier2 for inter-band UL CA case with dynamic UL Tx switching as defined in TS 38.101-1 [15]. Network can only configure this field for dynamic UL Tx switching in inter-band UL CA case with power Class 3 as defined in TS 38.101-1 [15].

DAPS-UplinkPowerConfig field descriptions

p-DAPS-Source The maximum total transmit power to be used by the UE in the source cell group during DAPS handover.

p-DAPS-Target The maximum total transmit power to be used by the UE in the target cell group during DAPS handover.

uplinkPowerSharingDAPS-Mode Indicates the uplink power sharing mode that the UE uses in DAPS handover (see TS 38.213 [13]).

GoodServingCellEvaluation field descriptions

offset The parameter “X” (dB) for the good serving cell quality criterion in RRC_CONNECTED, for a cell operating in FR1 and FR2, respectively. If this field is absent, the UE applies the (default) value of [0] dB for “X”.

ReconfigurationWithSync field descriptions

rach-ConfigDedicated Random access configuration to be used for the reconfiguration with sync (e.g. handover). The UE performs the RA according to these parameters in the firstActiveUplinkBWP (see UplinkConfig).

smtc The SSB periodicity/offset/duration configuration of target cell for NR PSCell change and NR PCell change. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in spCellConfigCommon. For case of NR PCell change, the smtc is based on the timing reference of (source) PCell. For case of NR PSCell change, it is based on the timing reference of source PSCell. If both this field and targetCellSMTC-SCG are absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SCellConfig field descriptions

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in an SCell in RRC_CONNECTED.

smtc The SSB periodicity/offset/duration configuration of target cell for NR SCell addition. The network sets the periodicityAndOffset to indicate the same periodicity as ssb-periodicityServingCell in sCellConfigCommon. The smtc is based on the timing of the SpCell of associated cell group. In case of inter-RAT handover to NR, the timing reference is the NR PCell. In case of intra-NR PCell change (standalone NR) or NR PSCell change (EN-DC), the timing reference is the target SpCell. If the field is absent, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message.

SpCellConfig field descriptions

goodServingCellEvaluationBFD Indicates the criterion for a UE to detect the good serving cell quality for BFD relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables BFD relaxation for the UE.

goodServingCellEvaluationRLM Indicates the criterion for a UE to detect the good serving cell quality for RLM relaxation in the SpCell in RRC_CONNECTED. The field is always configured when the network enables RLM relaxation for the UE.

lowMobilityEvaluationConnected Indicates the criterion for a UE to detect low mobility in RRC_CONNECTED in an SpCell. The s-SearchDeltaP-Connected is the parameter "SSearchDeltaP-connected". And the t-SearchDeltaP-Connected is the parameter " TSearchDeltaP-Connected". Low mobility criterion is configured in NR Pcell for the case of NR SA/ NR CA/ NE-DC/NR-DC, and in the NR PSCell for the case of EN-DC.

reconfigurationWithSync Parameters for the synchronous reconfiguration to the target SpCell.

rlf-TimersAndConstants Timers and constants for detecting and triggering cell-level radio link failure. For the SCG, rlf-TimersAndConstants can only be set to setup and is always included at SCG addition.

servCellIndex Serving cell ID of a PSCell. The PCell of the Master Cell Group uses ID = 0.

| Conditional Presence | Explanation |
| --- | --- |
| BWP-Reconfig | The field is optionally present, Need N, if the BWPs are reconfigured or if serving cells are added or removed. Otherwise it is absent. |
| DRX-Config2 | The field is optionally present, Need N, if drx-ConfigSecondaryGroup is configured. It is absent otherwise. |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup: - at change of AS security key derived from KgNB, - in an RRCReconfiguration message contained in a DLInformationTransferMRDC message, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell, - change of AS security key derived from S-KgNB in NR-DC while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, - MN handover in (NG)EN-DC. Otherwise, it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages and is absent in the masterCellGroup in RRCReconfiguration messages if source configuration is not released during DAPS handover. |
| SCellAdd | The field is mandatory present upon SCell addition; otherwise it is absent, Need M. |
| SCellAddMod | The field is mandatory present upon SCell addition; otherwise it is optionally present, need M. |
| SCellAddSync | The field is optionally present, Need N, in case of SCell addition, reconfiguration with sync, and resuming an RRC connection. It is absent otherwise. |
| SCG | The field is mandatory present in an SpCellConfig for the PSCell. It is absent otherwise. |

NOTE:	In case of change of AS security key derived from S-KgNB/S-KeNB, if reconfigurationWithSync is not included in the masterCellGroup, the network releases all existing MCG RLC bearers associated with a radio bearer with keyToUse set to secondary. In case of change of AS security key derived from KgNB/KeNB, if reconfigurationWithSync is not included in the secondaryCellGroup, the network releases all existing SCG RLC bearers associated with a radio bearer with keyToUse set to primary.

<Next modification>

#### –	DownlinkConfigCommonSIB

The IE DownlinkConfigCommonSIB provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINKCONFIGCOMMONSIB-START

DownlinkConfigCommonSIB ::=     SEQUENCE {

frequencyInfoDL                 FrequencyInfoDL-SIB,

initialDownlinkBWP              BWP-DownlinkCommon,

bcch-Config                         BCCH-Config,

pcch-Config                         PCCH-Config,

... ,

[[

pei-Config-r17                      PEI-Config-r17                         OPTIONAL               -- Need R

]]

}

BCCH-Config ::=                 SEQUENCE {

modificationPeriodCoeff         ENUMERATED {n2, n4, n8, n16},

...

}

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle,

nAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15)

},

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT                                   SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT                                                      SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

...,

[[

nrofPDCCH-MonitoringOccasionPerSSB-InPO-r16                                  INTEGER (2..4)             OPTIONAL  -- Cond SharedSpectrum2

]]

}

PEI-Config-r17 ::=             SEQUENCE {

pei-SearchSpace-r17               SearchSpaceId,

po-NumPerPEI-r17                   ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17            INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                INTEGER (0..16),

firstPDCCH-MonitoringOccasionOfPEI-O-r17      CHOICE {

sCS15KHZoneT-r17                                                                SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT-r17                                                  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT-r17                                 SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT-r17             SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT-r17    SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT-r17                  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT-r17                                   SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT-r17                                                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919)

},

subgroupConfig-r17                SubgroupConfig-r17,

lastUsedCellOnly-r17              ENUMERATED {true}                                                              OPTIONAL,              -- Need R

...

}

SubgroupConfig-r17 ::=         SEQUENCE {

subgroupsNumPerPO-r17                       INTEGER (1.. maxNrofPagingSubgroups-r17),

subgroupsNumForUEID-r17                     INTEGER (1.. maxNrofPagingSubgroups-r17)                    OPTIONAL,              -- Need R

...

}

-- TAG-DOWNLINKCONFIGCOMMONSIB-STOP

-- ASN1STOP

DownlinkConfigCommonSIB field descriptions

bcch-Config The modification period related configuration.

frequencyInfoDL-SIB Basic parameters of a downlink carrier and transmission thereon.

initialDownlinkBWP The initial downlink BWP configuration for a PCell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain. The UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment.

lastUsedCellOnly When present, the field indicates that the UE monitors PEI only if its last connection was released by this cell. A PEI-capable UE stores its last used cell information.

nrofPDCCH-MonitoringOccasionPerSSB-InPO The number of PDCCH monitoring occasions corresponding to an SSB within a Paging Occasion, see TS 38.304 [20], clause 7.1.

pcch-Config The paging related configuration.

pei-Config The PEI related configuration.

subgroupConfig The paging subgroup related configuration.

BCCH-Config field descriptions

modificationPeriodCoeff Actual modification period, expressed in number of radio frames m = modificationPeriodCoeff * defaultPagingCycle, see clause 5.2.2.2.2. n2 corresponds to value 2, n4 corresponds to value 4, and so on.

PCCH-Config field descriptions

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

firstPDCCH-MonitoringOccasionOfPO Points out the first PDCCH monitoring occasion for paging of each PO of the PF, see TS 38.304 [20].

NAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on. If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 2 or 3 (as specified in TS 38.213 [13]): - for ssb-periodicityServingCell of 5 or 10 ms, N can be set to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 20 ms, N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 40 ms, N can be set to one of {quarterT, oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 80 ms, N can be set to one of {oneEighthT, oneSixteenthT} - for ssb-periodicityServingCell of 160 ms, N can be set to oneSixteenthT If pagingSearchSpace is set to zero and if SS/PBCH block and CORESET multiplexing pattern is 1 (as specified in TS 38.213 [13]), N can be set to one of {halfT, quarterT, oneEighthT, oneSixteenthT} If pagingSearchSpace is not set to zero, N can be configured to one of {oneT, halfT, quarterT, oneEighthT, oneSixteenthT}

Ns Number of paging occasions per paging frame.

PEI-Config field descriptions

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/poNumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or mutliple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

payloadSizeDCI-2-7 Payload size of PEI DCI, i.e., DCI format 2_7. The size is no larger than the payload size of paging DCI which has maximum of 41 bits and 43 bits for licensed and unlicensed spectrums, respectively.

pei-FrameOffset Offset, in number of frames from the start of a first paging frame of the paging frames associated with the PEI-O to the start of a reference frame for PEI-O, see TS 38.213 [13], clause 10.4A.

pei-SearchSpace ID of dedicated search space for PEI. It can be configured to one of up to 4 common SS sets configured by commonSearchSpaceList with SearchSpaceId > 0. The CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level follows Table 10.1-1 of TS38.213 [13]. SearchSpaceId = 0 can be configured for the case of SS/PBCH block and CORESET multiplexing pattern 2 or 3.

po-NumPerPEI The number of PO(s) associated with one PEI monitoring occation. It is a factor of N x Ns (total PO number in a paging cycle). The Maximum number of PF associated with one PEI monitoring occation is up to 2. The number of PO mapping to one PEI should be multiple of Ns when po-NumPerPEI is larger than Ns.

SubgroupConfig field descriptions

subgroupsNumPerPO Total number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from physical-layer signaling The field represents the sum of CN-assigned and UEID-based subgroups supported by the network.

subgroupsNumForUEID Number of subgroups per Paging Occasion (PO) for UE to read subgroups indication from physical-layer signaling, for UEID-based subgrouping method. When present, the field is set to an integer smaller than or equal to subgroupsNumPerPOs. subgroupsNumPerPO equals to subgroupsNumForUEID when the network does not support CN-assigned subgrouping. The field is absent when the network does not support UEID-based subgrouping. Both this field and subgroupsNumPerPO are equal to 1 when the network does not support subgrouping.

| Conditional Presence | Explanation |
| --- | --- |
| SharedSpectrum2 | The field is optional present, Need R, if this cell operates with shared spectrum channel access. Otherwise, it is absent, Need R. |

<Next modification>

#### –	PDCCH-Config

The IE PDCCH-Config is used to configure UE specific PDCCH parameters such as control resource sets (CORESET), search spaces and additional parameters for acquiring the PDCCH. If this IE is used for the scheduled cell in case of cross carrier scheduling, the fields other than searchSpacesToAddModList and searchSpacesToReleaseList are absent. If the IE is used for a dormant BWP, the fields other than controlResourceSetToAddModList and controlResourceSetToReleaseList are absent.

PDCCH-Config information element

-- ASN1START

-- TAG-PDCCH-CONFIG-START

PDCCH-Config ::=                    SEQUENCE {

controlResourceSetToAddModList      SEQUENCE(SIZE (1..3)) OF ControlResourceSet                      OPTIONAL,   -- Need N

controlResourceSetToReleaseList     SEQUENCE(SIZE (1..3)) OF ControlResourceSetId                    OPTIONAL,   -- Need N

searchSpacesToAddModList            SEQUENCE(SIZE (1..10)) OF SearchSpace                            OPTIONAL,   -- Need N

searchSpacesToReleaseList           SEQUENCE(SIZE (1..10)) OF SearchSpaceId                          OPTIONAL,   -- Need N

downlinkPreemption                  SetupRelease { DownlinkPreemption }                              OPTIONAL,   -- Need M

tpc-PUSCH                           SetupRelease { PUSCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-PUCCH                           SetupRelease { PUCCH-TPC-CommandConfig }                         OPTIONAL,   -- Need M

tpc-SRS                             SetupRelease { SRS-TPC-CommandConfig}                            OPTIONAL,   -- Need M

...,

[[

controlResourceSetToAddModListSizeExt-v1610 SEQUENCE (SIZE (1..2)) OF ControlResourceSet             OPTIONAL,   -- Need N

controlResourceSetToReleaseListSizeExt-r16 SEQUENCE (SIZE (1..5)) OF ControlResourceSetId-r16        OPTIONAL,   -- Need N

searchSpacesToAddModListExt-r16     SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-r16                     OPTIONAL,   -- Need N

uplinkCancellation-r16              SetupRelease { UplinkCancellation-r16 }                          OPTIONAL,   -- Need M

monitoringCapabilityConfig-r16      ENUMERATED { r15monitoringcapability,r16monitoringcapability }   OPTIONAL,   -- Need M

searchSpaceSwitchConfig-r16         SearchSpaceSwitchConfig-r16                                      OPTIONAL    -- Need R

]] ,

[[

searchSpacesToAddModListExt-v17xy   SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v17xy                   OPTIONAL,   -- Need N

searchSpaceSwitchTimer-r17          INTEGER (1..800)                                                 OPTIONAL,   -- Need R

pdcch-SkippingDurationList-r17      SEQUENCE(SIZE (1..3)) OF PDCCH-SkippingDuration-r17              OPTIONAL   -- Need R

]]

}

SearchSpaceSwitchConfig-r16 ::=     SEQUENCE {

cellGroupsForSwitchList-r16         SEQUENCE(SIZE (1..4)) OF CellGroupForSwitch-r16                  OPTIONAL,   -- Need R

searchSpaceSwitchDelay-r16          INTEGER (10..52)                                                 OPTIONAL    -- Need R

}

CellGroupForSwitch-r16 ::=          SEQUENCE(SIZE (1..16)) OF ServCellIndex

PDCCH-SkippingDuration-r17 ::=      INTEGER (1..800)

-- TAG-PDCCH-CONFIG-STOP

-- ASN1STOP

Editor’s NOTE: It is FFS whether SSSG switching or PDCCH skipping is only applicable when C-DRX is configured. Wait for further RAN1 clarification.

PDCCH-Config field descriptions

controlResourceSetToAddModList, controlResourceSetToAddModListSizeExt List of UE specifically configured Control Resource Sets (CORESETs) to be used by the UE. The network restrictions on configuration of CORESETs per DL BWP are specified in TS 38.213 [13], clause 10.1 and TS 38.306 [26]. The UE shall consider entries in controlResourceSetToAddModList and in controlResourceSetToAddModListSizeExt as a single list, i.e. an entry created using controlResourceSetToAddModList can be modifed using controlResourceSetToAddModListSizeExt (or deleted using controlResourceSetToReleaseListSizeExt) and vice-versa. In case network reconfigures control resource set with the same ControlResourceSetId as used for commonControlResourceSet configured via PDCCH-ConfigCommon, the configuration from PDCCH-Config always takes precedence and should not be updated by the UE based on servingCellConfigCommon.

controlResourceSetToReleaseList, controlResourceSetToReleaseListSizeExt List of UE specifically configured Control Resource Sets (CORESETs) to be released by the UE. This field only applies to CORESETs configured by controlResourceSetToAddModList or controlResourceSetToAddModListSizeExt and does not release the field commonControlResourceSet configured by PDCCH-ConfigCommon.

downlinkPreemption Configuration of downlink preemption indications to be monitored in this cell (see TS 38.213 [13], clause 11.2).

monitoringCapabilityConfig Configures either Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability for PDCCH monitoring on a serving cell. Value r15monitoringcapablity enables the Rel-15 monitoring capability, and value r16monitoringcapablity enables the Rel-16 PDCCH monitoring capability (see TS 38.213 [13], clause 10.1).

pdcch-SkippingDurationList The UE can be configured to be indicated by DCI a value of X (i.e., skipping duration), in units of slots, among at most 3 multiple RRC configured values by scheduling DCIs indicating PDCCH schedules data. For each skipping duration (i.e. the value range of IE PDCCH-SkippingDuration-r17), {1,2,3,…,20,30, 40, 50, 60, 80, 100} are valid for the 15 kHz SCS, {1,2,3,…,40, 60, 80, 100, 120,160,200} are valid for 30 kHz SCS, {1,2,3,…,80, 120, 160, 200, 240, 320,400} are valid for 60kHz SCS, and {1,2,3,…,160, 240, 320,400, 480, 640,800} are valid for 120kHz SCS , {4,8,12,…,640, 960, 1280,1600, 1920, 2560,3200} are valid for 480kHz SCS, and {8,16,24,…,1280, 1920, 2560,3200, 3840, 5120,6400} are valid for 960kHz SCS.

searchSpacesToAddModList, searchSpacesToAddModListExt-r16, searchSpacesToAddModListExt-v17xy List of UE specifically configured Search Spaces. The network configures at most 10 Search Spaces per BWP per cell (including UE-specific and common Search Spaces). If the network includes searchSpaceToAddModListExt-r16, it includes the same number of entries, and listed in the same order, as in searchSpacesToAddModList.

searchSpaceSwitchTimer Timer (in unit of slots) to control the UE behavior to switch from search space group X back to search space group 0, as specified in clause 10 of TS 38.213. A UE does not expect to be configured with Rel-16 SSSG switching parameters and Rel-17 SSSG switching parameters per cell simultaneously. For 15 kHz SCS, {1,2,3,…,20,30, 40, 50, 60, 80, 100} are valid. For 30 kHz SCS, {1,2,3,…,40, 60, 80, 100, 120,160,200} are valid. For 60kHz SCS, {1,2,3,…,80, 120, 160, 200, 240, 320,400} are valid. For 120kHz SCS, {1,2,3,…,160, 240, 320,400, 480, 640,800} are valid. For 480kHz SCS, {4,8,12,…,640, 960, 1280,1600, 1920, 2560,3200} are valid. For 960kHz SCS, {8,16,24,…,1280, 1920, 2560,3200, 3840, 5120,6400} are valid.

tpc-PUCCH Enable and configure reception of group TPC commands for PUCCH.

tpc-PUSCH Enable and configure reception of group TPC commands for PUSCH.

tpc-SRS Enable and configure reception of group TPC commands for SRS.

uplinkCancellation Configuration of uplink cancellation indications to be monitored in this cell (see TS 38.213 [13], clause 11.2A).

SearchSpaceSwitchConfig field descriptions

cellGroupsForSwitchList The list of serving cells which are bundled for the search space group switching purpose (see TS 38.213 [13], clause 10.4). A serving cell can belong to only one CellGroupForSwitch. The network configures the same list for all BWPs of serving cells in the same CellGroupForSwitch.

searchSpaceSwitchDelay Indicates the value to be applied by a UE for Search Space Set Group switching; corresponds to the P value in TS 38.213 [13], clause 10.4. The network configures the same value for all BWPs of serving cells in the same CellGroupForSwitch.

<Next modification>

#### –	SearchSpace

The IE SearchSpace defines how/where to search for PDCCH candidates. Each search space is associated with one ControlResourceSet. For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent (regardless of their presence conditions).

SearchSpace information element

-- ASN1START

-- TAG-SEARCHSPACE-START

SearchSpace ::=                         SEQUENCE {

searchSpaceId                           SearchSpaceId,

controlResourceSetId                    ControlResourceSetId                                        OPTIONAL,   -- Cond SetupOnly

monitoringSlotPeriodicityAndOffset      CHOICE {

sl1                                     NULL,

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639),

sl1280                                  INTEGER (0..1279),

sl2560                                  INTEGER (0..2559)

}                                                                                                   OPTIONAL,   -- Cond Setup

duration                                INTEGER (2..2559)                                           OPTIONAL,   -- Need R

monitoringSymbolsWithinSlot             BIT STRING (SIZE (14))                                      OPTIONAL,   -- Cond Setup

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

searchSpaceType                         CHOICE {

common                                  SEQUENCE {

dci-Format0-0-AndFormat1-0              SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-0                           SEQUENCE {

nrofCandidates-SFI                      SEQUENCE {

aggregationLevel1                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8                       ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16                      ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-1                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-2                           SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-3                           SEQUENCE {

dummy1                                  ENUMERATED {sl1, sl2, sl4, sl5, sl8, sl10, sl16, sl20}  OPTIONAL,   -- Cond Setup

dummy2                                  ENUMERATED {n1, n2},

...

}                                                                                           OPTIONAL    -- Need R

},

ue-Specific                                 SEQUENCE {

dci-Formats                                 ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1},

...,

[[

dci-Formats-MT-r16                   ENUMERATED {formats2-5}                                OPTIONAL,    -- Need R

dci-FormatsSL-r16                    ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1, formats3-0, formats3-1,

formats3-0-And-3-1}                        OPTIONAL,    -- Need R

dci-FormatsExt-r16                   ENUMERATED {formats0-2-And-1-2, formats0-1-And-1-1And-0-2-And-1-2}

OPTIONAL     -- Need R

]]

}

}                                                                                                   OPTIONAL    -- Cond Setup2

}

SearchSpaceExt-r16 ::=                   SEQUENCE {

controlResourceSetId-r16                ControlResourceSetId-r16                                    OPTIONAL,   -- Cond SetupOnly2

searchSpaceType-r16                     SEQUENCE {

common-r16                              SEQUENCE {

dci-Format2-4-r16                       SEQUENCE {

nrofCandidates-CI-r16                   SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-5-r16                      SEQUENCE {

nrofCandidates-IAB-r16                  SEQUENCE {

aggregationLevel1-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel2-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel4-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel8-r16                   ENUMERATED {n1, n2}                         OPTIONAL,   -- Need R

aggregationLevel16-r16                  ENUMERATED {n1, n2}                         OPTIONAL    -- Need R

},

...

}                                                                                           OPTIONAL,   -- Need R

dci-Format2-6-r16                       SEQUENCE {

...

}                                                                                           OPTIONAL,   -- Need R

...,

[[

dci-Format2-7-r17                       SEQUENCE {

nrofCandidates-PEI-r17                   SEQUENCE {

aggregationLevel4-r17                   ENUMERATED {n0, n1, n2, n3, n4}             OPTIONAL,   -- Need R

aggregationLevel8-r17                   ENUMERATED {n0,n1, n2}                      OPTIONAL,   -- Need R

aggregationLevel16-r17                  ENUMERATED {n0, n1}                         OPTIONAL   -- Need R

},

...

}                                                                                           OPTIONAL   -- Need R

]]

}

}                                                                                                   OPTIONAL,    -- Cond Setup3

searchSpaceGroupIdList-r16                      SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1)           OPTIONAL,    -- Need R

freqMonitorLocations-r16                        BIT STRING (SIZE (5))                               OPTIONAL     -- Need R

}

SearchSpaceExt-v17xy ::=                   			SEQUENCE {

searchSpaceId-r17                                    SearchSpaceId,

searchSpaceGroupIdList-r17                          SEQUENCE (SIZE (1.. 3)) OF INTEGER (0.. maxNrofSearchSpaceGroups-1-r17)  OPTIONAL,  -- Need R

...

}

-- TAG-SEARCHSPACE-STOP

-- ASN1STOP

SearchSpace field descriptions

common Configures this search space as common search space (CSS) and DCI formats to monitor.

controlResourceSetId The CORESET applicable for this SearchSpace. Value 0 identifies the common CORESET#0 configured in MIB and in ServingCellConfigCommon. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured in System Information or by dedicated signalling. The CORESETs with non-zero controlResourceSetId are configured in the same BWP as this SearchSpace. If the field controlResourceSetId-r16 is present, UE shall ignore the controlResourceSetId (without suffix).

dummy1, dummy2 This field is not used in the specification. If received it shall be ignored by the UE.

dci-Format0-0-AndFormat1-0 If configured, the UE monitors the DCI formats 0_0 and 1_0 according to TS 38.213 [13], clause 10.1.

dci-Format2-0 If configured, UE monitors the DCI format 2_0 according to TS 38.213 [13], clause 10.1, 11.1.1.

dci-Format2-1 If configured, UE monitors the DCI format 2_1 according to TS 38.213 [13], clause 10.1, 11.2.

dci-Format2-2 If configured, UE monitors the DCI format 2_2 according to TS 38.213 [13], clause 10.1, 11.3.

dci-Format2-3 If configured, UE monitors the DCI format 2_3 according to TS 38.213 [13], clause 10.1, 11.4

dci-Format2-4 If configured, UE monitors the DCI format 2_4 according to TS 38.213 [13], clause 11.2A.

dci-Format2-5 If configured, IAB-MT monitors the DCI format 2_5 according to TS 38.213 [13], clause 14.

dci-Format2-6 If configured, UE monitors the DCI format 2_6 according to TS 38.213 [13], clause 10.1, 11.5. DCI format 2_6 can only be configured on the SpCell.

dci-Formats Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1.

dci-FormatsExt If this field is present, the field dci-Formats is ignored and dci-FormatsExt is used instead to indicate whether the UE monitors in this USS for DCI format 0_2 and 1_2 or formats 0_1 and 1_1 and 0_2 and 1_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 10.1). This field is not configured for operation with shared spectrum channel access in this release.

dci-Formats-MT Indicates whether the IAB-MT monitors the DCI formats 2-5 according to TS 38.213 [13], clause 14.

dci-FormatsSL Indicates whether the UE monitors in this USS for DCI formats 0-0 and 1-0 or for formats 0-1 and 1-1 or for format 3-0 or for format 3-1 or for formats 3-0 and 3-1. If this field is present, the field dci-Formats is ignored and dci-FormatsSL is used.

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset). For IAB-MT, duration indicates number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_5. The IAB-MT ignores this field for DCI format 2_0 and DCI format 2_5. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).

freqMonitorLocations Defines an association of the search space to multiple monitoring locations in the frequency domain and indicates whether the pattern configured in the associated CORESET is replicated to a specific RB set, see TS 38.213, clause 10.1. Each bit in the bitmap corresponds to one RB set, and the leftmost (most significant) bit corresponds to RB set 0 in the BWP. A bit set to 1 indicates that a frequency domain resource allocation replicated from the pattern configured in the associated CORESET is mapped to the RB set.

monitoringSlotPeriodicityAndOffset Slots for PDCCH Monitoring configured as periodicity and offset. If the UE is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the UE is configured to monitor DCI format 2_0, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213 [13], clause 10). If the UE is configured to monitor DCI format 2_4, only the values 'sl1', 'sl2', 'sl4', 'sl5', 'sl8' and 'sl10' are applicable. For IAB-MT, If the IAB-MT is configured to monitor DCI format 2_1, only the values 'sl1', 'sl2' or 'sl4' are applicable. If the IAB-MT is configured to monitor DCI format 2_0 or DCI format 2_5, only the values ′sl1′, ′sl2′, ′sl4′, ′sl5′, ′sl8′, ′sl10′, ′sl16′, and ′sl20′ are applicable (see TS 38.213, clause 10).

monitoringSymbolsWithinSlot The first symbol(s) for PDCCH monitoring in the slots configured for PDCCH monitoring (see monitoringSlotPeriodicityAndOffset and duration). The most significant (left) bit represents the first OFDM in a slot, and the second most significant (left) bit represents the second OFDM symbol in a slot and so on. The bit(s) set to one identify the first OFDM symbol(s) of the control resource set within a slot. If the cyclic prefix of the BWP is set to extended CP, the last two bits within the bit string shall be ignored by the UE or IAB-MT. For DCI format 2_0, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10. For IAB-MT: For DCI format 2_0 or DCI format 2_5, the first one symbol applies if the duration of CORESET (in the IE ControlResourceSet) identified by controlResourceSetId indicates 3 symbols, the first two symbols apply if the duration of CORESET identified by controlResourceSetId indicates 2 symbols, and the first three symbols apply if the duration of CORESET identified by controlResourceSetId indicates 1 symbol. See TS 38.213 [13], clause 10.

nrofCandidates-CI The number of PDCCH candidates specifically for format 2-4 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 10.1).

nrofCandidates-SFI The number of PDCCH candidates specifically for format 2-0 for the configured aggregation level. If an aggregation level is absent, the UE does not search for any candidates with that aggregation level. The network configures only one aggregationLevel and the corresponding number of candidates (see TS 38.213 [13], clause 11.1.1). For a search space configured with freqMonitorLocations-r16, only value ′n1′ is valid.

nrofCandidates-PEI The number of PDCCH candidates specifically for format 2-7 for the configured aggregation level.

nrofCandidates Number of PDCCH candidates per aggregation level. The number of candidates and aggregation levels configured here applies to all formats unless a particular value is specified or a format-specific value is provided (see inside searchSpaceType). If configured in the SearchSpace of a cross carrier scheduled cell, this field determines the number of candidates and aggregation levels to be used on the linked scheduling cell (see TS 38.213 [13], clause 10).

searchSpaceGroupIdList List of search space group IDs which the search space is associated with. The network configures at most 2 search space groups per BWP where the group ID is either 0 or 1. If searchSpaceGroupIdList (i.e. without suffix) is included. The network configures at most 3 search space groups per BWP where the group ID is either 0, 1 or 2 if searchSpaceGroupIdList-r17 is included. And if searchSpaceGroupIdList-r17 is included, searchSpaceGroupIdList (i.e. without suffix) is ignored.

searchSpaceId Identity of the search space. SearchSpaceId = 0 identifies the searchSpaceZero configured via PBCH (MIB) or ServingCellConfigCommon and may hence not be used in the SearchSpace IE. The searchSpaceId is unique among the BWPs of a Serving Cell. In case of cross carrier scheduling, search spaces with the same searchSpaceId in scheduled cell and scheduling cell are linked to each other. The UE applies the search space for the scheduled cell only if the DL BWPs in which the linked search spaces are configured in scheduling cell and scheduled cell are both active. For an IAB-MT, the search space defines how/where to search for PDCCH candidates for an IAB-MT. Each search space is associated with one ControlResearchSet. For a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates, all the optional fields are absent.

searchSpaceType Indicates whether this is a common search space (present) or a UE specific search space as well as DCI formats to monitor for.

ue-Specific Configures this search space as UE specific search space (USS). The UE monitors the DCI format with CRC scrambled by C-RNTI, CS-RNTI (if configured), and SP-CSI-RNTI (if configured)

| Conditional Presence | Explanation |
| --- | --- |
| Setup | This field is mandatory present upon creation of a new SearchSpace. It is optionally present, Need M, otherwise. |
| Setup2 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt-r16 of the parent IE with the field searchSpaceType-r16 included. Otherwise it is optionally present, Need M. |
| Setup3 | This field is mandatory present when a new SearchSpace is set up, if the same SearchSpace ID is not included in searchSpacesToAddModListExt (without suffix) of the parent IE with the field searchSpaceType (without suffix) included. Otherwise it is optionally present, Need M. |
| SetupOnly | This field is mandatory present upon creation of a new SearchSpace. It is absent, Need M, otherwise. |
| SetupOnly2 | In PDCCH-Config, the field is optionally present upon creation of a new SearchSpace and absent, Need M upon reconfiguration of an existing SearchSpace. In PDCCH-ConfigCommon, the field is absent. |

<Next modification>

#### –	SI-SchedulingInfo

The IE SI-SchedulingInfo contains information needed for acquisition of SI messages.

SI-SchedulingInfo information element

-- ASN1START

-- TAG–SI-SCHEDULINGINFO-START

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280},

si-RequestConfig                    SI-RequestConfig                                                OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                                OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                                          OPTIONAL,   -- Need R

...

}

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

SIB-Mapping ::=                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo

SIB-TypeInfo ::=                    SEQUENCE {

type                                ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610, sibType14-v1610,

sibTypex-v17xy, spare2, spare1,... },

valueTag                            INTEGER (0..31)                                                  OPTIONAL, -- Cond SIB-TYPE

areaScope                           ENUMERATED {true}                                                OPTIONAL -- Need S

}

-- TAG-SI-SCHEDULINGINFO-STOP

-- ASN1STOP

SchedulingInfo field descriptions

areaScope Indicates that a SIB is area specific. If the field is absent, the SIB is cell specific.

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting.

si-Periodicity Periodicity of the SI-message in radio frames. Value rf8 corresponds to 8 radio frames, value rf16 corresponds to 16 radio frames, and so on.

SI-SchedulingInfo field descriptions

si-RequestConfig Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-RequestConfigSUL Configuration of Msg1 resources that the UE uses for requesting SI-messages for which si-BroadcastStatus is set to notBroadcasting.

si-WindowLength The length of the SI scheduling window. Value s5 corresponds to 5 slots, value s10 corresponds to 10 slots and so on. The network always configures si-WindowLength to be shorter than or equal to the si-Periodicity.

systemInformationAreaID Indicates the system information area that the cell belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this systemInformationAreaID. The systemInformationAreaID is unique within a PLMN/SNPN.

| Conditional presence | Explanation |
| --- | --- |
| MSG-1 | The field is optionally present, Need R, if si-BroadcastStatus is set to notBroadcasting for any SI-message included in SchedulingInfo. It is absent otherwise. |
| SIB-TYPE | The field is mandatory present if the SIB type is different from SIB6, SIB7 or SIB8. For SIB6, SIB7 and SIB8 it is absent. |
| SUL-MSG-1 | The field is optionally present, Need R, if supplementaryUplink is configured in ServingCellConfigCommonSIB and if si-BroadcastStatus is set to notBroadcasting for any SI-message included in SchedulingInfo. It is absent otherwise. |

<Next modification>

### 6.3.4	Other information elements

<Partially omitted>

#### –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

OtherConfig ::=                 SEQUENCE {

delayBudgetReportingConfig  CHOICE{

release                 NULL,

setup                   SEQUENCE{

delayBudgetReportingProhibitTimer   ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30}

}

}                                                                                                     OPTIONAL        -- Need M

}

OtherConfig-v1540 ::=           SEQUENCE {

overheatingAssistanceConfig     SetupRelease {OverheatingAssistanceConfig}                            OPTIONAL, -- Need M

...

}

CandidateServingFreqListNR-r16 ::= SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR

OtherConfig-v1610 ::=                   SEQUENCE {

idc-AssistanceConfig-r16                SetupRelease {IDC-AssistanceConfig-r16}                       OPTIONAL, -- Need M

drx-PreferenceConfig-r16                SetupRelease {DRX-PreferenceConfig-r16}                       OPTIONAL, -- Need M

maxBW-PreferenceConfig-r16              SetupRelease {MaxBW-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxCC-PreferenceConfig-r16              SetupRelease {MaxCC-PreferenceConfig-r16}                     OPTIONAL, -- Need M

maxMIMO-LayerPreferenceConfig-r16       SetupRelease {MaxMIMO-LayerPreferenceConfig-r16}              OPTIONAL, -- Need M

minSchedulingOffsetPreferenceConfig-r16 SetupRelease {MinSchedulingOffsetPreferenceConfig-r16}        OPTIONAL, -- Need M

releasePreferenceConfig-r16             SetupRelease {ReleasePreferenceConfig-r16}                    OPTIONAL, -- Need M

referenceTimePreferenceReporting-r16    ENUMERATED {true}                                             OPTIONAL,  -- Need R

btNameList-r16                          SetupRelease {BT-NameList-r16}                                OPTIONAL, -- Need M

wlanNameList-r16                        SetupRelease {WLAN-NameList-r16}                              OPTIONAL, -- Need M

sensorNameList-r16                      SetupRelease {Sensor-NameList-r16}                            OPTIONAL, -- Need M

obtainCommonLocation-r16                ENUMERATED {true}                                             OPTIONAL,  -- Need R

sl-AssistanceConfigNR-r16               ENUMERATED{true}                                              OPTIONAL -- Need R

}

OtherConfig-v17xy ::=                   SEQUENCE {

rlm-RelaxationReportingConfig-r17                SetupRelease {RLM-RelaxationReportingConfig-r17}     OPTIONAL, -- Need M

bfd-RelaxationReportingConfig-r17                SetupRelease {BFD-RelaxationReportingConfig-r17}     OPTIONAL -- Need M

}

OverheatingAssistanceConfig ::= SEQUENCE {

overheatingIndicationProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, spare3, spare2, spare1}

}

IDC-AssistanceConfig-r16 ::=    SEQUENCE {

candidateServingFreqListNR-r16  CandidateServingFreqListNR-r16                     OPTIONAL, -- Need R

...

}

DRX-PreferenceConfig-r16 ::=          SEQUENCE {

drx-PreferenceProhibitTimer-r16       ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxBW-PreferenceConfig-r16 ::=        SEQUENCE {

maxBW-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxCC-PreferenceConfig-r16 ::=        SEQUENCE {

maxCC-PreferenceProhibitTimer-r16     ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MaxMIMO-LayerPreferenceConfig-r16 ::= SEQUENCE {

maxMIMO-LayerPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

MinSchedulingOffsetPreferenceConfig-r16 ::=   SEQUENCE {

minSchedulingOffsetPreferenceProhibitTimer-r16 ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, spare2, spare1}

}

ReleasePreferenceConfig-r16 ::=       SEQUENCE {

releasePreferenceProhibitTimer-r16    ENUMERATED {

s0, s0dot5, s1, s2, s3, s4, s5, s6, s7,

s8, s9, s10, s20, s30, infinity, spare1},

connectedReporting                    ENUMERATED {true}                                               OPTIONAL  -- Need R

}

RLM-RelaxationReportingConfig-r17 SEQUENCE {

rlm-RelaxtionReportingProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

BFD-RelaxationReportingConfig-r17 SEQUENCE {

bfd-RelaxtionReportingProhibitTimer    ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30,

s60, s90, s120, s300, s600, infinity, spare2, spare1}

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

OtherConfig field descriptions

bfd-RelaxationReportingConfig Configuration for the UE to report the relaxation state of BDF measurements.

candidateServingFreqListNR Indicates for each candidate NR serving cells, the center frequency around which UE is requested to report IDC issues.

connectedReporting Indicates that the UE can report a preference to remain in RRC_CONNECTED state following a report to leave RRC_CONNECTED state. If absent, the UE cannot report a preference to stay in RRC_CONNECTED state.

delayBudgetReportingProhibitTimer Prohibit timer for delay budget reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot4 means prohibit timer is set to 0.4 seconds, and so on.

drx-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's DRX preferences for power saving.

drx-PreferenceProhibitTimer Prohibit timer for DRX preferences assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

idc-AssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected IDC problem.

maxBW-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred bandwidth for power saving.

maxBW-PreferenceProhibitTimer Prohibit timer for preferred bandwidth assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

maxCC-PreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of carriers for power saving.

maxCC-PreferenceProhibitTimer Prohibit timer for preferred number of carriers assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

maxMIMO-LayerPreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred number of MIMO layers for power saving.

maxMIMO-LayerPreferenceProhibitTimer Prohibit timer for preferred number of number of MIMO layers assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

minSchedulingOffsetPreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preferred minimumSchedulingOffset value for cross-slot scheduling for power saving.

minSchedulingOffsetPreferenceProhibitTimer Prohibit timer for preferred minimumSchedulingOffset assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

obtainCommonLocation Requests the UE to attempt to have detailed location information available using GNSS. NR configures the field if includeCommonLocationInfo is configured for one or more measurements.

overheatingAssistanceConfig Configuration for the UE to report assistance information to inform the gNB about UE detected internal overheating.

overheatingIndicationProhibitTimer Prohibit timer for overheating assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on.

referenceTimePreferenceReporting If present, the field indicates the UE is configured to provide reference time assistance information.

releasePreferenceConfig Configuration for the UE to report assistance information to inform the gNB about the UE's preference to leave RRC_CONNECTED state.

rlm-RelaxationReportingConfig Configuration for the UE to report the relaxation state of RLM measurements.

releasePreferenceProhibitTimer Prohibit timer for release preference assistance information reporting. Value in seconds. Value s0 means prohibit timer is set to 0 seconds, value s0dot5 means prohibit timer is set to 0.5 seconds, value s1 means prohibit timer is set to 1 second and so on. Value infinity means that once a UE has reported a release preference, the UE cannot report a release preference again during the RRC connection.

sensorNameList Configuration for the UE to report measurements from specific sensors.

sl-AssistanceConfigNR Indicate whether UE is configured to provide configured grant assistance information for NR sidelink communication.

<Next modification>

## 6.4	RRC multiplicity and type constraint values

### –	Multiplicity and type constraint definitions

-- ASN1START

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-START

maxAI-DCI-PayloadSize-r16               INTEGER ::= 128      --Maximum size of the DCI payload scrambled with ai-RNTI

maxAI-DCI-PayloadSize-1-r16             INTEGER ::= 127      --Maximum size of the DCI payload scrambled with ai-RNTI minus 1

maxBandComb                             INTEGER ::= 65536   -- Maximum number of DL band combinations

maxBandsUTRA-FDD-r16                    INTEGER ::= 64      -- Maximum number of bands listed in UTRA-FDD UE caps

maxBH-RLC-ChannelID-r16                 INTEGER ::= 65536   -- Maximum value of BH RLC Channel ID

maxBT-IdReport-r16                      INTEGER ::= 32      -- Maximum number of Bluetooth IDs to report

maxBT-Name-r16                          INTEGER ::= 4       -- Maximum number of Bluetooth name

maxCAG-Cell-r16                         INTEGER ::= 16      -- Maximum number of NR CAG cell ranges in SIB3, SIB4

maxTwoPUCCH-Grp-ConfigList-r16          INTEGER ::= 32      -- Maximum number of supported configuration(s) of {primary PUCCH group

-- config, secondary PUCCH group config}

maxCBR-Config-r16                       INTEGER ::= 8       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control

maxCBR-Config-1-r16                     INTEGER ::= 7       -- Maximum number of CBR range configurations for sidelink communication

-- congestion control minus 1

maxCBR-Level-r16                        INTEGER ::= 16      -- Maximum nuber of CBR levels

maxCBR-Level-1-r16                      INTEGER ::= 15      -- Maximum number of CBR levels minus 1

maxCellBlack                            INTEGER ::= 16      -- Maximum number of NR blacklisted cell ranges in SIB3, SIB4

maxCellGroupings-r16                    INTEGER ::= 32      -- Maximum number of cell groupings for NR-DC

maxCellHistory-r16                      INTEGER ::= 16      -- Maximum number of visited cells reported

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxCellWhite                            INTEGER ::= 16      -- Maximum number of NR whitelisted cell ranges in SIB3, SIB4

maxEARFCN                               INTEGER ::= 262143  -- Maximum value of E-UTRA carrier frequency

maxEUTRA-CellBlack                      INTEGER ::= 16      -- Maximum number of E-UTRA blacklisted physical cell identity ranges

-- in SIB5

maxEUTRA-NS-Pmax                        INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxLogMeasReport-r16                    INTEGER ::= 520     -- Maximum number of entries for logged measurements

maxMultiBands                           INTEGER ::= 8       -- Maximum number of additional frequency bands that a cell belongs to

maxNARFCN                               INTEGER ::= 3279165 -- Maximum value of NR carrier frequency

maxNR-NS-Pmax                           INTEGER ::= 8       -- Maximum number of NS and P-Max values per band

maxFreqIdle-r16                         INTEGER ::= 8       -- Maximum number of carrier frequencies for idle/inactive measurements

maxNrofServingCells                     INTEGER ::= 32      -- Max number of serving cells (SpCells + SCells)

maxNrofServingCells-1                   INTEGER ::= 31      -- Max number of serving cells (SpCell + SCells) per cell group

maxNrofAggregatedCellsPerCellGroup      INTEGER ::= 16

maxNrofAggregatedCellsPerCellGroupMinus4-r16   INTEGER ::= 12

maxNrofDUCells-r16                      INTEGER ::= 512     -- Max number of cells configured on the collocated IAB-DU

maxNrofAvailabilityCombinationsPerSet-r16   INTEGER ::= 512 -- Max number of AvailabilityCombinationId used in the DCI format 2_5

maxNrofAvailabilityCombinationsPerSet-1-r16 INTEGER ::= 511 -- Max number of AvailabilityCombinationId used in the DCI format 2_5 minus 1

maxNrofSCells                           INTEGER ::= 31      -- Max number of secondary serving cells per cell group

maxNrofCellMeas                         INTEGER ::= 32      -- Maximum number of entries in each of the cell lists in a measurement object

maxNrofCG-SL-r16                        INTEGER ::= 8       -- Max number of sidelink configured grant

maxNrofCG-SL-1-r16                      INTEGER ::= 7       -- Max number of sidelink configured grant minus 1

maxNrofSS-BlocksToAverage               INTEGER ::= 16      -- Max number for the (max) number of SS blocks to average to determine cell measurement

maxNrofCondCells-r16                    INTEGER ::= 8       -- Max number of conditional candidate SpCells

maxNrofCSI-RS-ResourcesToAverage        INTEGER ::= 16      -- Max number for the (max) number of CSI-RS to average to determine cell measurement

maxNrofDL-Allocations                   INTEGER ::= 16      -- Maximum number of PDSCH time domain resource allocations

maxNrofSR-ConfigPerCellGroup            INTEGER ::= 8       -- Maximum number of SR configurations per cell group

maxLCG-ID                               INTEGER ::= 7       -- Maximum value of LCG ID

maxLC-ID                                INTEGER ::= 32      -- Maximum value of Logical Channel ID

maxLC-ID-Iab-r16                        INTEGER ::= 65855   -- Maximum value of BH Logical Channel ID extension

maxLTE-CRS-Patterns-r16                 INTEGER ::= 3       -- Maximum number of additional LTE CRS rate matching patterns

maxNrofTAGs                             INTEGER ::= 4       -- Maximum number of Timing Advance Groups

maxNrofTAGs-1                           INTEGER ::= 3       -- Maximum number of Timing Advance Groups minus 1

maxNrofBWPs                             INTEGER ::= 4       -- Maximum number of BWPs per serving cell

maxNrofCombIDC                          INTEGER ::= 128     -- Maximum number of reported MR-DC combinations for IDC

maxNrofSymbols-1                        INTEGER ::= 13      -- Maximum index identifying a symbol within a slot (14 symbols, indexed from 0..13)

maxNrofSlots                            INTEGER ::= 320     -- Maximum number of slots in a 10 ms period

maxNrofSlots-1                          INTEGER ::= 319     -- Maximum number of slots in a 10 ms period minus 1

maxNrofPhysicalResourceBlocks           INTEGER ::= 275     -- Maximum number of PRBs

maxNrofPhysicalResourceBlocks-1         INTEGER ::= 274     -- Maximum number of PRBs minus 1

maxNrofPhysicalResourceBlocksPlus1      INTEGER ::= 276     -- Maximum number of PRBs plus 1

maxNrofControlResourceSets              INTEGER ::= 12      -- Max number of CoReSets configurable on a serving cell

maxNrofControlResourceSets-1            INTEGER ::= 11      -- Max number of CoReSets configurable on a serving cell minus 1

maxNrofControlResourceSets-1-r16        INTEGER ::= 15      -- Max number of CoReSets configurable on a serving cell extended in minus 1

maxNrofCoresetPools-r16                 INTEGER ::= 2       -- Maximum number of CORESET pools

maxCoReSetDuration                      INTEGER ::= 3       -- Max number of OFDM symbols in a control resource set

maxNrofSearchSpaces-1                   INTEGER ::= 39      -- Max number of Search Spaces minus 1

maxSFI-DCI-PayloadSize                  INTEGER ::= 128     -- Max number payload of a DCI scrambled with SFI-RNTI

maxSFI-DCI-PayloadSize-1                INTEGER ::= 127     -- Max number payload of a DCI scrambled with SFI-RNTI minus 1

maxIAB-IP-Address-r16                   INTEGER ::= 32      -- Max number of assigned IP addresses

maxINT-DCI-PayloadSize                  INTEGER ::= 126     -- Max number payload of a DCI scrambled with INT-RNTI

maxINT-DCI-PayloadSize-1                INTEGER ::= 125     -- Max number payload of a DCI scrambled with INT-RNTI minus 1

maxNrofRateMatchPatterns                INTEGER ::= 4       -- Max number of rate matching patterns that may be configured

maxNrofRateMatchPatterns-1              INTEGER ::= 3       -- Max number of rate matching patterns that may be configured minus 1

maxNrofRateMatchPatternsPerGroup        INTEGER ::= 8       -- Max number of rate matching patterns that may be configured in one group

maxNrofCSI-ReportConfigurations         INTEGER ::= 48      -- Maximum number of report configurations

maxNrofCSI-ReportConfigurations-1       INTEGER ::= 47      -- Maximum number of report configurations minus 1

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

maxNrofCSI-ResourceConfigurations-1     INTEGER ::= 111     -- Maximum number of resource configurations minus 1

maxNrofAP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers            INTEGER ::= 128     -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger  INTEGER ::= 16      -- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources             INTEGER ::= 192     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1           INTEGER ::= 191     -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet       INTEGER ::= 64      -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourceSets          INTEGER ::= 64      -- Maximum number of NZP CSI-RS resource sets per cell

maxNrofNZP-CSI-RS-ResourceSets-1        INTEGER ::= 63      -- Maximum number of NZP CSI-RS resource sets per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig    INTEGER ::= 128     -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources              INTEGER ::= 32      -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1            INTEGER ::= 31      -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1         INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet        INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets           INTEGER ::= 16

maxNrofCSI-IM-Resources                 INTEGER ::= 32      -- Maximum number of CSI-IM resources

maxNrofCSI-IM-Resources-1               INTEGER ::= 31      -- Maximum number of CSI-IM resources minus 1

maxNrofCSI-IM-ResourcesPerSet           INTEGER ::= 8       -- Maximum number of CSI-IM resources per set

maxNrofCSI-IM-ResourceSets              INTEGER ::= 64      -- Maximum number of NZP CSI-IM resource sets per cell

maxNrofCSI-IM-ResourceSets-1            INTEGER ::= 63      -- Maximum number of NZP CSI-IM resource sets per cell minus 1

maxNrofCSI-IM-ResourceSetsPerConfig     INTEGER ::= 16      -- Maximum number of CSI IM resource sets per resource configuration

maxNrofCSI-SSB-ResourcePerSet           INTEGER ::= 64      -- Maximum number of SSB resources in a resource set

maxNrofCSI-SSB-ResourceSets             INTEGER ::= 64      -- Maximum number of CSI SSB resource sets per cell

maxNrofCSI-SSB-ResourceSets-1           INTEGER ::= 63      -- Maximum number of CSI SSB resource sets per cell minus 1

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

maxNrofFailureDetectionResources        INTEGER ::= 10      -- Maximum number of failure detection resources

maxNrofFailureDetectionResources-1      INTEGER ::= 9       -- Maximum number of failure detection resources minus 1

maxNrofFreqSL-r16                       INTEGER ::= 8       -- Maximum number of carrier frequncy for for NR sidelink communication

maxNrofSL-BWPs-r16                      INTEGER ::= 4       -- Maximum number of BWP for for NR sidelink communication

maxFreqSL-EUTRA-r16                     INTEGER ::= 8       -- Maximum number of EUTRA anchor carrier frequncy for NR sidelink communication

maxNrofSL-MeasId-r16                    INTEGER ::= 64      -- Maximum number of sidelink measurement identity (RSRP) per destination

maxNrofSL-ObjectId-r16                  INTEGER ::= 64      -- Maximum number of sidelink measurement objects (RSRP) per destination

maxNrofSL-ReportConfigId-r16            INTEGER ::= 64      -- Maximum number of sidelink measurement reporting configuration(RSRP) per destination

maxNrofSL-PoolToMeasureNR-r16           INTEGER ::= 8       -- Maximum number of resoure pool for NR sidelink measurement to measure for

-- each measurement object (for CBR)

maxFreqSL-NR-r16                        INTEGER ::= 8       -- Maximum number of NR anchor carrier frequncy for NR sidelink communication

maxNrofSL-QFIs-r16                      INTEGER ::= 2048    -- Maximum number of QoS flow for NR sidelink communication per UE

maxNrofSL-QFIsPerDest-r16               INTEGER ::= 64      -- Maximum number of QoS flow per destination for NR sidelink communication

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofPageRec                          INTEGER ::= 32      -- Maximum number of page records

maxNrofPCI-Ranges                       INTEGER ::= 8       -- Maximum number of PCI ranges

maxPLMN                                 INTEGER ::= 12      -- Maximum number of PLMNs broadcast and reported by UE at establisghment

maxNrofCSI-RS-ResourcesRRM              INTEGER ::= 96      -- Maximum number of CSI-RS resources per cell for an RRM measurement object

maxNrofCSI-RS-ResourcesRRM-1            INTEGER ::= 95      -- Maximum number of CSI-RS resources per cell for an RRM measurement object minus 1

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxNrofQuantityConfig                   INTEGER ::= 2       -- Maximum number of quantity configurations

maxNrofCSI-RS-CellsRRM                  INTEGER ::= 96      -- Maximum number of cells with CSI-RS resources for an RRM measurement object

maxNrofSL-Dest-r16                      INTEGER ::= 32      -- Maximum number of destination for NR sidelink communication

maxNrofSL-Dest-1-r16                    INTEGER ::= 31      -- Highest index of destination for NR sidelink communication

maxNrofSLRB-r16                         INTEGER ::= 512     -- Maximum number of radio bearer for NR sidelink communication per UE

maxSL-LCID-r16                          INTEGER ::= 512     -- Maximum number of RLC bearer for NR sidelink communication per UE

maxSL-SyncConfig-r16                    INTEGER ::= 16      -- Maximum number of sidelink Sync configurations

maxNrofRXPool-r16                       INTEGER ::= 16      -- Maximum number of Rx resource poolfor NR sidelink communication

maxNrofTXPool-r16                       INTEGER ::= 8       -- Maximum number of Tx resourcepoolfor NR sidelink communication

maxNrofPoolID-r16                       INTEGER ::= 16      -- Maximum index of resource pool for NR sidelink communication

maxNrofSRS-PathlossReferenceRS-r16      INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for SRS power control.

maxNrofSRS-PathlossReferenceRS-1-r16    INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for SRS power control-1.

maxNrofSRS-ResourceSets                 INTEGER ::= 16      -- Maximum number of SRS resource sets in a BWP.

maxNrofSRS-ResourceSets-1               INTEGER ::= 15      -- Maximum number of SRS resource sets in a BWP minus 1.

maxNrofSRS-PosResourceSets-r16          INTEGER ::= 16      -- Maximum number of SRS Positioning resource sets in a BWP.

maxNrofSRS-PosResourceSets-1-r16        INTEGER ::= 15      -- Maximum number of SRS Positioning resource sets in a BWP minus 1.

maxNrofSRS-Resources                    INTEGER ::= 64      -- Maximum number of SRS resources.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources minus 1.

maxNrofSRS-PosResources-r16             INTEGER ::= 64      -- Maximum number of SRS Positioning resources.

maxNrofSRS-PosResources-1-r16           INTEGER ::= 63      -- Maximum number of SRS Positioning resources in an SRS Positioning

-- resource set minus 1.

maxNrofSRS-ResourcesPerSet              INTEGER ::= 16      -- Maximum number of SRS resources in an SRS resource set

maxNrofSRS-TriggerStates-1              INTEGER ::= 3       -- Maximum number of SRS trigger states minus 1, i.e., the largest code point.

maxNrofSRS-TriggerStates-2              INTEGER ::= 2       -- Maximum number of SRS trigger states minus 2.

maxRAT-CapabilityContainers             INTEGER ::= 8       -- Maximum number of interworking RAT containers (incl NR and MRDC)

maxSimultaneousBands                    INTEGER ::= 32      -- Maximum number of simultaneously aggregated bands

maxULTxSwitchingBandPairs               INTEGER ::= 32      -- Maximum number of band pairs supporting dynamic UL Tx switching in a band combination

maxNrofSlotFormatCombinationsPerSet     INTEGER ::= 512     -- Maximum number of Slot Format Combinations in a SF-Set.

maxNrofSlotFormatCombinationsPerSet-1   INTEGER ::= 511     -- Maximum number of Slot Format Combinations in a SF-Set minus 1.

maxNrofTrafficPattern-r16               INTEGER ::= 8       -- Maximum number of Traffic Pattern for NR sidelink communication.

maxNrofPUCCH-Resources                  INTEGER ::= 128

maxNrofPUCCH-Resources-1                INTEGER ::= 127

maxNrofPUCCH-ResourceSets               INTEGER ::= 4       -- Maximum number of PUCCH Resource Sets

maxNrofPUCCH-ResourceSets-1             INTEGER ::= 3       -- Maximum number of PUCCH Resource Sets minus 1.

maxNrofPUCCH-ResourcesPerSet            INTEGER ::= 32      -- Maximum number of PUCCH Resources per PUCCH-ResourceSet

maxNrofPUCCH-P0-PerSet                  INTEGER ::= 8       -- Maximum number of P0-pucch present in a p0-pucch set

maxNrofPUCCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUCCH power control.

maxNrofPUCCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUCCH power control minus 1.

maxNrofPUCCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUCCH power control extended.

maxNrofPUCCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUCCH power control

-- minus 1 extended.

maxNrofPUCCH-PathlossReferenceRSsDiff-r16 INTEGER ::= 60    -- Difference between the extended maximum and the non-extended maximum

maxNrofPUCCH-ResourceGroups-r16         INTEGER ::= 4       -- Maximum number of PUCCH resources groups.

maxNrofPUCCH-ResourcesPerGroup-r16      INTEGER ::= 128     -- Maximum number of PUCCH resources in a PUCCH group.

maxNrofMultiplePUSCHs-r16               INTEGER ::= 8       -- Maximum number of multiple PUSCHs in PUSCH TDRA list

maxNrofP0-PUSCH-AlphaSets               INTEGER ::= 30      -- Maximum number of P0-pusch-alpha-sets (see 38,213, clause 7.1)

maxNrofP0-PUSCH-AlphaSets-1             INTEGER ::= 29      -- Maximum number of P0-pusch-alpha-sets minus 1 (see 38,213, clause 7.1)

maxNrofPUSCH-PathlossReferenceRSs       INTEGER ::= 4       -- Maximum number of RSs used as pathloss reference for PUSCH power control.

maxNrofPUSCH-PathlossReferenceRSs-1     INTEGER ::= 3       -- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1.

maxNrofPUSCH-PathlossReferenceRSs-r16   INTEGER ::= 64      -- Maximum number of RSs used as pathloss reference for PUSCH power control extended

maxNrofPUSCH-PathlossReferenceRSs-1-r16 INTEGER ::= 63      -- Maximum number of RSs used as pathloss reference for PUSCH power control minus 1

maxNrofPUSCH-PathlossReferenceRSsDiff-r16  INTEGER ::= 60   -- Difference between maxNrofPUSCH-PathlossReferenceRSs-r16 and

-- maxNrofPUSCH-PathlossReferenceRSs

maxNrofNAICS-Entries                    INTEGER ::= 8       -- Maximum number of supported NAICS capability set

maxBands                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

maxBandsMRDC                            INTEGER ::= 1280

maxBandsEUTRA                           INTEGER ::= 256

maxCellReport                           INTEGER ::= 8

maxDRB                                  INTEGER ::= 29      -- Maximum number of DRBs (that can be added in DRB-ToAddModLIst).

maxFreq                                 INTEGER ::= 8       -- Max number of frequencies.

maxFreqLayers                           INTEGER ::= 4       -- Max number of frequency layers.

maxFreqIDC-r16                          INTEGER ::= 128     -- Max number of frequencies for IDC indication.

maxCombIDC-r16                          INTEGER ::= 128     -- Max number of reported UL CA for IDC indication.

maxFreqIDC-MRDC                         INTEGER ::= 32      -- Maximum number of candidate NR frequencies for MR-DC IDC indication

maxNrofCandidateBeams                   INTEGER ::= 16      -- Max number of PRACH-ResourceDedicatedBFR that in BFR config.

maxNrofCandidateBeams-r16               INTEGER ::= 64      -- Max number of candidate beam resources in BFR config.

maxNrofCandidateBeamsExt-r16            INTEGER ::= 48      -- Max number of PRACH-ResourceDedicatedBFR in the CandidateBeamRSListExt

maxNrofPCIsPerSMTC                      INTEGER ::= 64      -- Maximun number of PCIs per SMTC.

maxNrofQFIs                             INTEGER ::= 64

maxNrofResourceAvailabilityPerCombination-r16 INTEGER ::= 256

maxNrOfSemiPersistentPUSCH-Triggers     INTEGER ::= 64      -- Maximum number of triggers for semi persistent reporting on PUSCH

maxNrofSR-Resources                     INTEGER ::= 8       -- Maximum number of SR resources per BWP in a cell.

maxNrofSlotFormatsPerCombination        INTEGER ::= 256

maxNrofSpatialRelationInfos             INTEGER ::= 8

maxNrofSpatialRelationInfos-plus-1      INTEGER ::= 9

maxNrofSpatialRelationInfos-r16         INTEGER ::= 64

maxNrofSpatialRelationInfosDiff-r16     INTEGER ::= 56      -- Difference between maxNrofSpatialRelationInfos-r16 and maxNrofSpatialRelationInfos

maxNrofIndexesToReport                  INTEGER ::= 32

maxNrofIndexesToReport2                 INTEGER ::= 64

maxNrofSSBs-r16                         INTEGER ::= 64      -- Maximum number of SSB resources in a resource set.

maxNrofSSBs-1                           INTEGER ::= 63      -- Maximum number of SSB resources in a resource set minus 1.

maxNrofS-NSSAI                          INTEGER ::= 8       -- Maximum number of S-NSSAI.

maxNrofTCI-StatesPDCCH                  INTEGER ::= 64

maxNrofTCI-States                       INTEGER ::= 128     -- Maximum number of TCI states.

maxNrofTCI-States-1                     INTEGER ::= 127     -- Maximum number of TCI states minus 1.

maxNrofUL-Allocations                   INTEGER ::= 16      -- Maximum number of PUSCH time domain resource allocations.

maxQFI                                  INTEGER ::= 63

maxRA-CSIRS-Resources                   INTEGER ::= 96

maxRA-OccasionsPerCSIRS                 INTEGER ::= 64      -- Maximum number of RA occasions for one CSI-RS

maxRA-Occasions-1                       INTEGER ::= 511     -- Maximum number of RA occasions in the system

maxRA-SSB-Resources                     INTEGER ::= 64

maxSCSs                                 INTEGER ::= 5

maxSecondaryCellGroups                  INTEGER ::= 3

maxNrofServingCellsEUTRA                INTEGER ::= 32

maxMBSFN-Allocations                    INTEGER ::= 8

maxNrofMultiBands                       INTEGER ::= 8

maxCellSFTD                             INTEGER ::= 3       -- Maximum number of cells for SFTD reporting

maxReportConfigId                       INTEGER ::= 64

maxNrofCodebooks                        INTEGER ::= 16      -- Maximum number of codebooks suppoted by the UE

maxNrofCSI-RS-ResourcesExt-r16          INTEGER ::= 16      -- Maximum number of codebook resources supported by the UE for eType2/Codebook combo

maxNrofCSI-RS-Resources                 INTEGER ::= 7       -- Maximum number of codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-r16          INTEGER ::= 512     -- Maximum number of alternative codebook resources supported by the UE

maxNrofCSI-RS-ResourcesAlt-1-r16        INTEGER ::= 511     -- Maximum number of alternative codebook resources supported by the UE minus 1

maxNrofSRI-PUSCH-Mappings               INTEGER ::= 16

maxNrofSRI-PUSCH-Mappings-1             INTEGER ::= 15

maxSIB                                  INTEGER::= 32       -- Maximum number of SIBs

maxSI-Message                           INTEGER::= 32       -- Maximum number of SI messages

maxPO-perPF                             INTEGER ::= 4       -- Maximum number of paging occasion per paging frame

maxPEI-perPF-r17                            INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

maxAccessCat-1                          INTEGER ::= 63      -- Maximum number of Access Categories minus 1

maxBarringInfoSet                       INTEGER ::= 8       -- Maximum number of access control parameter sets

maxCellEUTRA                            INTEGER ::= 8       -- Maximum number of E-UTRA cells in SIB list

maxEUTRA-Carrier                        INTEGER ::= 8       -- Maximum number of E-UTRA carriers in SIB list

maxPLMNIdentities                       INTEGER ::= 8       -- Maximum number of PLMN identites in RAN area configurations

maxDownlinkFeatureSets                  INTEGER ::= 1024    -- (for NR DL) Total number of FeatureSets (size of the pool)

maxUplinkFeatureSets                    INTEGER ::= 1024    -- (for NR UL) Total number of FeatureSets (size of the pool)

maxEUTRA-DL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxEUTRA-UL-FeatureSets                 INTEGER ::= 256     -- (for E-UTRA) Total number of FeatureSets (size of the pool)

maxFeatureSetsPerBand                   INTEGER ::= 128     -- (for NR) The number of feature sets associated with one band.

maxPerCC-FeatureSets                    INTEGER ::= 1024    -- (for NR) Total number of CC-specific FeatureSets (size of the pool)

maxFeatureSetCombinations               INTEGER ::= 1024    -- (for MR-DC/NR)Total number of Feature set combinations (size of the pool)

maxInterRAT-RSTD-Freq                   INTEGER ::= 3

maxHRNN-Len-r16                         INTEGER ::= 48      -- Maximum length of HRNNs

maxNPN-r16                              INTEGER ::= 12      -- Maximum number of NPNs broadcast and reported by UE at establishment

maxNrOfMinSchedulingOffsetValues-r16    INTEGER ::= 2       -- Maximum number of min. scheduling offset (K0/K2) configurations

maxK0-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K0)

maxK2-SchedulingOffset-r16              INTEGER ::= 16      -- Maximum number of slots configured as min. scheduling offset (K2)

maxDCI-2-6-Size-r16                     INTEGER ::= 140     -- Maximum size of DCI format 2-6

maxDCI-2-7-Size-r17                     INTEGER ::= 43      -- Maximum size of DCI format 2-7

maxDCI-2-6-Size-1-r16                   INTEGER ::= 139     -- Maximum DCI format 2-6 size minus 1

maxNrofUL-Allocations-r16               INTEGER ::= 64      -- Maximum number of PUSCH time domain resource allocations

maxNrofP0-PUSCH-Set-r16                 INTEGER ::= 2       -- Maximum number of P0 PUSCH set(s)

maxOnDemandSIB-r16                      INTEGER ::= 8       -- Maximum number of SIB(s) that can be requested on-demand

maxOnDemandPosSIB-r16                   INTEGER ::= 32      -- Maximum number of posSIB(s) that can be requested on-demand

maxCI-DCI-PayloadSize-r16               INTEGER ::= 126     -- Maximum number of the DCI size for CI

maxCI-DCI-PayloadSize-1-r16             INTEGER ::= 125     -- Maximum number of the DCI size for CI minus 1

maxWLAN-Id-Report-r16                   INTEGER ::= 32      -- Maximum number of WLAN IDs to report

maxWLAN-Name-r16                        INTEGER ::= 4       -- Maximum number of WLAN name

maxRAReport-r16                         INTEGER ::= 8       -- Maximum number of RA procedures information to be included in the RA report

maxTxConfig-r16                         INTEGER ::= 64      -- Maximum number of sidelink transmission parameters configurations

maxTxConfig-1-r16                       INTEGER ::= 63      -- Maximum number of sidelink transmission parameters configurations minus 1

maxPSSCH-TxConfig-r16                   INTEGER ::= 16      -- Maximum number of PSSCH TX configurations

maxNrofCLI-RSSI-Resources-r16           INTEGER ::= 64      -- Maximum number of CLI-RSSI resources for UE

maxNrofCLI-RSSI-Resources-1-r16         INTEGER ::= 63      -- Maximum number of CLI-RSSI resources for UE minus 1

maxNrofCLI-SRS-Resources-r16            INTEGER ::= 32      -- Maximum number of SRS resources for CLI measurement for UE

maxCLI-Report-r16                       INTEGER ::= 8

maxNrofConfiguredGrantConfig-r16        INTEGER ::= 12      -- Maximum number of configured grant configurations per BWP

maxNrofConfiguredGrantConfig-1-r16      INTEGER ::= 11      -- Maximum number of configured grant configurations per BWP minus 1

maxNrofCG-Type2DeactivationState        INTEGER ::= 16      -- Maximum number of deactivation state for type 2 configured grants per BWP

maxNrofConfiguredGrantConfigMAC-1-r16   INTEGER ::= 31      -- Maximum number of configured grant configurations per MAC entity minus 1

maxNrofSPS-Config-r16                   INTEGER ::= 8       -- Maximum number of SPS configurations per BWP

maxNrofSPS-Config-1-r16                 INTEGER ::= 7       -- Maximum number of SPS configurations per BWP minus 1

maxNrofSPS-DeactivationState            INTEGER ::= 16      -- Maximum number of deactivation state for SPS per BWP

maxNrofDormancyGroups                   INTEGER ::= 5       --

maxNrofPagingSubgroups-r17                 INTEGER ::= 8       -- Maximum number of paging subgroups per paging occasion

maxNrofPUCCH-ResourceGroups-1-r16       INTEGER ::= 3       --

maxNrofServingCellsTCI-r16              INTEGER ::= 32      -- Maximum number of serving cells in simultaneousTCI-UpdateList

maxNrofTxDC-TwoCarrier-r16              INTEGER ::= 64      -- Maximum number of UL Tx DC locations reported by the UE for 2CC uplink CA

maxNrofTRS-ResourceSets-r17             INTEGER ::= 64      -- Maximum number of TRS resource sets

maxNrofSearchSpaceGroups-1-r17            INTEGER ::= 3       -- Maximum number of search space groups

-- TAG-MULTIPLICITY-AND-TYPE-CONSTRAINT-DEFINITIONS-STOP

-- ASN1STOP

<Next modification>

### 7.1.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 | Upon transmission of RRCSetupRequest. | Upon reception of RRCSetup or RRCReject message, cell re-selection and upon abortion of connection establishment by upper layers. | Perform the actions as specified in 5.3.3.7. |
| T301 | Upon transmission of RRCReestabilshmentRequest | Upon reception of RRCReestablishment or RRCSetup message as well as when the selected cell becomes unsuitable | Go to RRC_IDLE |
| T302 | Upon reception of RRCReject while performing RRC connection establishment or resume, upon reception of RRCRelease with waitTime. | Upon entering RRC_CONNECTED or RRC_IDLE, upon cell re-selection and upon reception of RRCReject message. | Inform upper layers about barring alleviation as specified in 5.3.14.4 |
| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync. | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of MCG, in case of the handover from NR or intra-NR handover, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T310 | Upon detecting physical layer problems for the SpCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, upon receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, upon the reconfiguration of rlf-TimersAndConstant, upon initiating the connection re-establishment procedure, upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync for that cell group, and upon initiating the MCG failure information procedure. Upon SCG release, if the T310 is kept in SCG. | If the T310 is kept in MCG: If AS security is not activated: go to RRC_IDLE else: initiate the MCG failure information procedure as specified in 5.7.3b or the connection re-establishment procedure as specified in 5.3.7 or the procedure as specified in 5.3.10.3 if any DAPS bearer is configured. If the T310 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.7.3. |
| T311 | Upon initiating the RRC connection re-establishment procedure | Upon selection of a suitable NR cell or a cell using another RAT. | Enter RRC_IDLE |
| T312 | If T312 is configured in MCG: Upon triggering a measurement report for a measurement identity for which T312 has been configured and useT312 has been set to true, while T310 in PCell is running. If T312 is configured in SCG and useT312 has been set to true: Upon triggering a measurement report for a measurement identity for which T312 has been configured, while T310 in PSCell is running. | Upon receiving N311 consecutive in-sync indications from lower layers for the SpCell, receiving RRCReconfiguration with reconfigurationWithSync for that cell group, upon reception of MobilityFromNRCommand, upon initiating the connection re-establishment procedure, upon the reconfiguration of rlf-TimersAndConstant, upon initiating the MCG failure information procedure, upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync for that cell group, and upon the expiry of T310 in corresponding SpCell. Upon SCG release, if the T312 is kept in SCG | If the T312 is kept in MCG initiate the MCG failure information procedure as specified in 5.7.3b or the connection re-establishment procedure. If the T312 is kept in SCG, Inform E-UTRAN/NR about the SCG radio link failure by initiating the SCG failure information procedure.as specified in 5.7.3. |
| T316 | Upon transmission of the MCGFailureInformation message | Upon receiving RRCRelease, RRCReconfiguration with reconfigurationwithSync for the PCell, MobilityFromNRCommand, or upon initiating the re-establishment procedure | Perform the actions as specified in 5.7.3b.5. |
| T319 | Upon transmission of RRCResumeRequest or RRCResumeRequest1. | Upon reception of RRCResume, RRCSetup, RRCRelease, RRCRelease with suspendConfig or RRCReject message and upon cell re-selection. | Perform the actions as specified in 5.3.13.5. |
| T320 | Upon reception of t320 or upon cell (re)selection to NR from another RAT with validity time configured for dedicated priorities (in which case the remaining validity time is applied). | Upon entering RRC_CONNECTED, upon reception of RRCRelease, when PLMN selection or SNPN selection is performed on request by NAS, when the UE enters RRC_IDLE from RRC_INACTIVE, or upon cell (re)selection to another RAT (in which case the timer is carried on to the other RAT). | Discard the cell reselection priority information provided by dedicated signalling. |
| T321 | Upon receiving measConfig including a reportConfig with the purpose set to reportCGI | Upon acquiring the information needed to set all fields of cgi-info, upon receiving measConfig that includes removal of the reportConfig with the purpose set to reportCGI and upon detecting that a cell is not broadcasting SIB1. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T322 | Upon receiving measConfig including reportConfigNR with the purpose set to reportSFTD and drx-SFTD-NeighMeas is set to true. | Upon acquiring the SFTD measurement results, upon receiving measConfig that includes removal of the reportConfig with the purpose set to reportSFTD. | Initiate the measurement reporting procedure, stop performing the related measurements. |
| T325 | Upon reception of RRCRelease message with deprioritisationTimer. |  | Stop deprioritisation of all frequencies or NR signalled by RRCRelease. |
| T330 | Upon receiving LoggedMeasurementConfiguration message | Upon log volume exceeding the suitable UE memory, upon initiating the release of LoggedMeasurementConfiguration procedure | Perform the actions specified in 5.5a.1.4 |
| T331 | Upon receiving RRCRelease message with measIdleDuration | Upon receiving RRCSetup, RRCResume, RRCRelease with idle/inactive measurement configuration, upon cell selection/reselection to a cell that does not belong to the validityArea (if configured), or upon cell re-selection to another RAT. | Perform the actions as specified in 5.7.8.3. |
| T342 | Upon transmitting UEAssistanceInformation message with DelayBudgetReport. | Upon releasing delayBudgetReportingConfig during the connection re-establishment/resume procedures, and upon receiving delayBudgetReportingConfig set to release. | No action. |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon releasing overheatingAssistance during the connection re-establishment procedure, upon initiating the connection resumption procedure, and upon receiving overheatingAssistanceConfig set to release. | No action. |
| T346a (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with drx-Preference. | Upon releasing drx-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving drx-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346b (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxBW-Preference. | Upon releasing maxBW-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxBW-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346c (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxCC-Preference. | Upon releasing maxCC-PreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxCC-PreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346d (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with maxMIMO-LayerPreference. | Upon releasing maxMIMO-LayerPreferenceConfig during the connection re-establishment/resume procedures, upon receiving maxMIMO-LayerPreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346e (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with minSchedulingOffsetPreference. | Upon releasing minSchedulingOffsetPreferenceConfig during the connection re-establishment/resume procedures, upon receiving minSchedulingOffsetPreferenceConfig set to release, or upon performing MR-DC release. | No action. |
| T346f | Upon transmitting UEAssistanceInformation message with releasePreference. | Upon releasing releasePreferenceConfig during the connection re-establishment/resume procedures, or upon receiving releasePreferenceConfig set to release. | No action. |
| T34x (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with rlm-RelaxationReportingConfig. | Upon releasing rlm-RelaxationReportingConfig during the connection re-establishment/resume procedures, upon receiving rlm-RelaxationReportingConfig set to release, or upon performing MR-DC release. | No action. |
| T346 (The UE maintains one instance of this timer per cell group) | Upon transmitting UEAssistanceInformation message with bfd-RelaxationReportingConfig. | Upon releasing bfd-RelaxationReportingConfig during the connection re-establishment/resume procedures, upon receiving bfd-RelaxationReportingConfig set to release, or upon performing MR-DC release. | No action. |
| T350 | Upon transmitting DedicatedSIBRequest message with requestedSIB-List and/or requestedPosSIB-List. | Upon acquiring the requested SIB(s) or posSIB(s), upon releasing onDemandSIB-Request during the connection re-establishment procedures, upon receiving onDemandSIB-Request set to release, upon reception of RRCRelease or upon successful change of PCell while in RRC_CONNECTED. | No action |
| T380 | Upon reception of t380 in RRCRelease. | Upon reception of RRCResume, RRCSetup or RRCRelease. | Perform the actions as specified in 5.3.13. |
| T390 | When access attempt is barred at access barring check for an Access Category. The UE maintains one instance of this timer per Access Category. | Upon cell (re)selection, upon entering RRC_CONNECTED, upon reception of RRCReconfiguration including reconfigurationWithSync, upon change of PCell while in RRC_CONNECTED, upon reception of MobilityFromNRCommand, or upon reception of RRCRelease. | Perform the actions as specified in 5.3.14.4. |
| T400 | Upon transmission of RRCReconfigurationSidelink | Upon reception of RRCReconfigurationFailureSidelink or RRCReconfigurationCompleteSidelink | Perform the Sidelink radio link failure related actions as specified in 5.8.9.3. |

End of Changes