3GPP TSG RAN Meeting #96										     RP-221403

Budapest, Hungary, June 6-9, 2022

Status Report to TSG

Agenda item:			9.4.1.1

| WI / SI Name | Further Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FeMIMO |  |  |  |  |
| Unique ID | 860040 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-212535 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 09/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 60% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

Core Requirement

- RF Core Requirement Maintenance

Some remaining details were discussed and the following Tdocs were approved/Endorsed in RAN4#103-e:

- WF on FeMIMO maintenance was approved in R4-2211227

- LS on GP positioning between two UL SRS resource sets approved in R4-2211226

- Draft CR on clarification of PMPR in FR2 was endorsed in R4-2211192 (R16) and R4-2209379 (R17)

- RRM Core Requirement Maintenance

Some remaining details were discussed and the following Tdocs were approved/Endorsed in RAN4#103-e:

- WF on FeMIMO RRM impact for unified TCI was approved in R4-2211203

- WF on FeMIMO RRM requirements for inter-cell beam management and TRP-specific link recovery was approved in R4-2211148

- CR on L1-RSRP measurement requirements for inter-cell BM in R17 was agreed in R4-2211114

- DraftCR on maintaining R17 TRP specific BFR requirements was agreed in R4-2211115

- CR on TRP specific CBD and BFR requirements was agreed in R4-2211116

- CR on measurement restriction and scheduling availability for inter cell L1-RSRP measurement was agreed in R4-2211188

- DraftCR on maintaining L1-RSRP measurement requirements for R17 inter-cell beam managements was agreed in R4-2211160

- CR on unified TCI in R17 feMIMO was agreed in R4-2211144

- CR on known condition of unified TCI for UL was agreed in R4-2208466

Performance Requirement

- RRM Performance Requirement

The following Tdocs were approved/Endorsed in RAN4#103-e:

- WF on FeMIMO RRM performance requirements was approved in R4-2210617.

- Draft CR to TS38.133 Accuracy Requirement for R17 L1-SINR Measurement on NSC was endorsed in R4-2208277.

Specifically, the following agreements were achieved in RAN4 Chairman Notes and/or approved way forward for each topic:

Unified TCI state switching

Agreement

- No test case will be introduced for DCI-based unified TCI state switching

- MAC-CE-based unified TCI state switching test cases will be defined in FR2 EN-DC and NR SA operation mode.

- Define the MAC-CE based TCI state switching test cases:

  - For serving cells, define the test cases for Joint TCI and UL TCI state

  - For cells with different PCI, define the test case for DL TCI state

- Configurations for TCI state switching test case

  - Specify test cases for known TCI state only

    - For the known DL or Joint TCI test cases, the TCI is not in the active TCI list

    - For the known UL TCI test cases, the PL-RS is not maintained.

L1-RSRP measurement on Cells with different PCI

Agreement

- No new section for accuracy requirement of L1-RSRP Measurement on Cells with different PCI, revise the applicability rule of the current accuracy requirement to support the cells with different PCI.

- No need to introduce separated test cases for measurement accuracy.

- Scenarios for the test case of L1-RSRP measurement on Cells with different PCI

  - Define test cases for

    - FR1 NR SA

    - FR1-FR2 CA operation where FR2 SCell is to be tested

  - For CA scenario, FR2 NR Cell is to be tested in the test cases.

- DRX configuration for the L1-RSRP measurement procedure test cases

  - For each test case, UE is configured either with DRX or without DRX

TRP specific BFD and LR

Agreement

- Define the TRP specific BFR and LR test case for both FR1 and FR2 and both EN-DC and NR SA.

- Scenarios and configurations for defining TRP specific BFD and LR test case

  - RAN4 aims to agree a reasonable test case list to minimize test effort

  - The table below can be taking starting point of scenarios and configurations of the test for further discussion

| 1 | EN-DC | FR1 | PSCell | SSB based | Non-DRX |
| --- | --- | --- | --- | --- | --- |
| 2 | EN-DC | FR2 | PSCell | CSI-RS based | DRX |
| 3 | EN-DC | FR1 | SCell | CSI-RS-based BFD and SSB-based LR | Non-DRX |
| 4 | NR SA | FR1 | PCell | CSI-RS based | DRX |
| 5 | NR SA | FR2 | PCell | SSB based | Non-DRX |
| 6 | NR SA | FR2 | SCell | CSI-RS based | DRX |

  - Other options not precluded

- DRX configuration for the TRP specific BFD and LR test cases

  - For each test case, UE is configured either with DRX or without DRX

- Demodulation and CSI requirement

The following Tdocs were approved in RAN4#103-bis-e meeting

- WF on general and CSI requirement for Rel-17 FeMIMO was approved in R4-2210669

- WF on demodulation requirement for Enhancement on HST-SFN deployment was approved in R4-2210670

- WF on demodulation requirement for Enhancement on Multi-TRP was approved in R4-2210671

Specifically, the following agreements were achieved in RAN4 Chairman Notes and/or approved way forward for each topic:

WF on demodulation requirement for Enhancement on Multi-TRP

Agreement

- Test Scope

  - Introduce PDCCH requirements for multi-TRP repetition transmission

    - Option 1: With UE soft-combining

    - Option 2: without UE soft-combining

  - RAN4 further evaluate the performance, the detailed UE processing pending on UE implementation and if companies’ results can be aligned, then no need to restrict/align UE receiver assumption i.e. how to combine/select PDCCH from different TRP

- Test setup for PDCCH requirement

  - Aggregation level

    - Option 1: AL=2

    - Option 2: AL=4

    - Decide aggregation level base on simulation results in next meeting.

  - Operating SNR condition for selection AL

    - Option 1: SNR > [-4] dB

    - Other options are not precluded

  - SNR setting for each TRP

    - Option 1: Balance SNR (no blocking considered)

      - The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP

    - Option 2: Blocked TRP testing

      - Blocked TRP testing for Option 2 in Issue 1-1-1 (without UE soft-combining): Probability of blocking p=10% with X=10 dB blocking as a starting point

    - Other options are not precluded

  - Time offset/Frequency offset for each TRP

    - Same value as single-DCI PDSCH with FDM scheme as starting point

      - 15KHz SCS: timing offset =-0.5us, frequency offset=200Hz

      - 30KHz SCS: timing offset =-0.25us, frequency offset=300Hz

WF on demodulation requirement for Enhancement on HST-SFN deployment

Agreement

- Test Scope

  - Whether to define PDSCH requirement with HST-SFN scheme B

    - Option 1: Introduced the PDSCH requirement with HST-SFN B under test applicability rule

      - FFS on the pre-compensation modeling

    - Option 2: Not to introduce the PDSCH requirement with HST-SFN B

    - Further discuss the difference compared other transmission schemes from UE processing, deployment scenario and the receiver performance aspect and RAN4 will make decision on RA4#104-e meeting whether HST SFN scheme B test cases will be specified or not

    - FFS on the pre-compensation modelling

- Test setup for PDSCH requirement for SFN scheme A with Single Carrier

  - Maximum Doppler shift

    - RAN4 will make final decision about the value of Maximum Doppler shift in RAN4#104-e meeting based on the simulation results, e.g. compare the required SNR to achieve 70% of peak rate with MCS17 (or MCS13) with rank 2.

      - Option 1: 870Hz (baseline)

      - Option 2: 972Hz

    - Encourage interested companies provide the simulation results for both Options in RAN4#104-e meeting.

  - MCS and Rank

    - MCS17 rank 2 as the baseline.

    - RAN4 may consider MCS13 rank 2 according to the simulation results

  - UE capability

    - The PDSCH demodulation requirements for HST-SFN Scheme A should be applicable for UE capable of “23-6-1 SFN scheme A (scheme 1) for PDSCH and PDCCH”

- Test setup for PDSCH requirement for SFN scheme B

  - Modelling of TRP pre-compensation

    - Assume perfect modelling of TRP pre-compensation as baseline.

    - Interested companies can evaluate the following model:

      - The frequency shift FD,k (Hz) from kth RRH is given by:

FD,k=fc×real-v×y-xky-xk×C for k*Ds≤a<k*Ds+Ds

and

FD,k+1=FD,k+F∆,k+1 for k*Ds≤a<k*Ds+Ds

where

F∆,k=+0.1 ppm*fc ,  k mod 2=0-0.1 ppm*fc ,  k mod 2=1

WF on CSI requirement for Rel-17 FeMIMO

Agreement

- CSI reporting requirement for multi-TRP

  - Test cases for CSI reporting enhancement for m-TRP transmission

    - Only PMI requirement introduced for CSI reporting enhancement for m-TRP transmission with Single-DCI based scheme.

  - MIMO correlation: XP high

  - Test Metric: TP ratio between following PMI and Random PMI

  - Test point: 90%

  - Number of CSI-RS Ports

    - 8 ports per TRP as baseline

    - Encourage interesting companies to provide simulation results for both 4 and 8 ports in RAN4#104-e meeting, and make the final decision in RAN4#104-e meeting

- PMI reporting requirement for Rel-17 enhanced type II PS codebook

  - Further discuss test case design especially for BF modelling in BS side, RAN4 will not introduce requirements for Rel-17 FeType II PS codebook if RAN4 can’t identify proper test case set-up by end of Aug RAN4 meeting.

  - BF Modelling

    - Wide beam W1 Modeling

      - Option 1a: MIMO fading channel as Rel-13 LTE Class B K=1 PMI test cases

      - Option 2: LTE power scaling method similar as Rel-13 LTE Class B K>1 CRI test case

      - Option 2a: fixed, i.e., non-phase rotating, beamformer based on LTE power scaling used to define the specific beamformers for CSI-RS and data

    - Frequency selective precoding Wf Modeling

      - Option 1: explicitly derived from chosen TDL model

      - Option 2: fixed Wf for each selected TDL channel model

      - Option 3: Not modeling

    - FFS on TE implementation with Wf to UE under test operation

      - Option 1: specific implementation to UE under test operation

      - Option 2: transparent to UE under test operation

  - Test set up

    - Option 1

  - FFS on the TE implementation for W1 and Wf

    - Option 1: hardcoded or algorithm defined on TE, and use the same W1 for all TDL models and test variants for FeType II PS CB performance and adjust the Wf per TDL model

    - Option 2: Use the same method as type-I/II, i,e., gNB follow the PMI reported from UE. No specific algorithm in gNB/TE side

  - FFS on the feasibility of TE implementation for W1 and Wf

  - Test Metric

    - Option 1: following PMI with random PMI

    - Option 2: Following FeType II CB over following eType II CB

    - Option 3: Following FeType II CB PMI over Type 1 single panel random PMI

  - Encourage interesting companies including TE vendors bring more analysis about the test design especially for BF modelling in BS side and test feasibility in RAN4#104-e meeting.

2.4.2	Remaining Open issues

Performance Requirement

- RRM Performance Requirement

- Work split and introduced RRM test case drafting in TS 38.133

- Demodulation and CSI requirement

- FFS to define PDSCH demodulation requirement for SFN scheme B

- FFS to define PMI requirement with FeType II port selection

- FFS on detail test setup for PDCCH requirement for Multi-TRP enhancement

- FFS on detail test setup for PDSCH demodulation requirement for SFN scheme A

- FFS on detail test setup for PMI requirement for single-DCI based on Multi-TRP scheme

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #103-e (May. 2022, Electronic)

| R4-2207663 | Transmissions on a gap between SRSs | Qualcomm Incorporated |
| --- | --- | --- |
| R4-2207778 | On per-beam based PMPR | Apple |
| R4-2207779 | On SRS switching in FeMIMO | Apple |
| R4-2207806 | Discussion on RRM requirements for Unified TCI | Apple |
| R4-2207807 | Discussion on RRM requirements for inter-cell beam management | Apple |
| R4-2207809 | Discussion on other RRM requirements for FeMIMO | Apple |
| R4-2208058 | Discussion on remaining issue about Unified TCI state in FeMIMO | Intel Corporation |
| R4-2208059 | Discussion on remaining issue about inter-cell beam management in FeMIMO | Intel Corporation |
| R4-2208060 | Discussion on FeMIMO test case | Intel Corporation |
| R4-2208265 | On General for FeMIMO | Nokia, Nokia Shanghai Bell |
| R4-2208276 | Discussion on RRM Performance part for Rel-17 NR FeMIMO | Samsung |
| R4-2208277 | Draft CR to TS38.133 Accuracy Requirement for R17 L1-SINR Measurement on NSC | Samsung |
| R4-2208278 | Draft CR to TS38.133 Corrections on R17 L1-RSRP requirement on NSC | Samsung |
| R4-2208279 | Draft CR to TS38.133 Corrections on R17 unified TCI requirement | Samsung |
| R4-2208381 | GP position handling in gap between two UL SRS resource sets | Nokia, Nokia Shanghai Bell |
| R4-2208383 | [Draft] LS on GP positioning between two UL SRS resource sets | Nokia, Nokia Shanghai Bell |
| R4-2208465 | Discussion on unified TCI for DL and UL | MediaTek Inc. |
| R4-2208466 | CR on known condition of unified TCI for UL | MediaTek Inc. |
| R4-2208467 | Discussion on inter cell beam management | MediaTek Inc. |
| R4-2208468 | CR on measurement restriction and scheduling availability for inter cell L1-RSRP measurement | MediaTek Inc. |
| R4-2208477 | Views on Rel-17 HST-SFN scheme | NTT DOCOMO, INC. |
| R4-2208494 | Views on FeMIMO Demodulation requirements | Samsung |
| R4-2208495 | Discussion and simulation results for Rel-17 CSI reporting under FeMIMO WI | Samsung |
| R4-2208509 | Discussion on demodulation requirements for enhancement to support HST-SFN | CMCC |
| R4-2208510 | Discussion on test cases for further enhancement on MIMO for NR | CMCC |
| R4-2208601 | Discussion on clarification of P-MPR for Rel-17 eMIMO | vivo |
| R4-2208602 | CR for clarification of P-MPR for Rel-17 eMIMO | vivo |
| R4-2208775 | Discussion on Impact of MPE enhancements | ZTE Corporation |
| R4-2208776 | Discussion on Unified TCI for DL and UL | ZTE Corporation |
| R4-2208840 | Discussion on demodulation performance requirements definition for Rel17 multi-TRP | MediaTek Inc. |
| R4-2209005 | Discussion on RRM remaining issues for R17 unified TCI framework | Huawei, Hisilicon |
| R4-2209006 | DraftCR on maintaining TCI state switching requirements for R17 unified TCI | Huawei, Hisilicon |
| R4-2209007 | Discussion on RRM remaining issues for R17 inter-cell beam managements | Huawei, Hisilicon |
| R4-2209009 | Discussion on remaining issues for R17 TRP specific BFR | Huawei, Hisilicon |
| R4-2209011 | Discussion on RRM test cases for R17 NR FeMIMO | Huawei, Hisilicon |
| R4-2209134 | DraftCR on maintaining L1-RSRP measurement requirements for R17 inter-cell beam managements | Huawei, Hisilicon |
| R4-2209135 | DraftCR on maintaining R17 TRP specific BFR requirements | Huawei, Hisilicon |
| R4-2209167 | Remaining issues for SRS | Huawei, HiSilicon |
| R4-2209500 | Discussion on remaining issues in RRM requirements for unified TCI in R17 feMIMO | vivo |
| R4-2209501 | CR on unified TCI in R17 feMIMO | vivo |
| R4-2209502 | Discussion on remaining issues in RRM requirements for inter-cell L1 beam measurements in R17 feMIMO | vivo |
| R4-2209503 | CR on L1-RSRP measurement requirements for inter-cell BM in R17 | vivo |
| R4-2209504 | Discussion on remaining issues in other RRM impacts in R17 feMIMO | vivo |
| R4-2209505 | Discussion on perf requirements and test cases in R17 feMIMO | vivo |
| R4-2209696 | Discussion on the PDSCH requirement for HST-SFN scenario | Ericsson |
| R4-2209697 | Discussion on the enhancement on Multi-TRP | Ericsson |
| R4-2209698 | Discussion on the CSI reporting for Multi-TRP transmission | Ericsson |
| R4-2209699 | Discussion on the Rel-17 eType II port selection codebook | Ericsson |
| R4-2209735 | On CSI reporting for Multi-TRP transmission for FeMIMO | Nokia, Nokia Shanghai Bell |
| R4-2209736 | On Rel-17 eType II port selection codebook for FeMIMO | Nokia, Nokia Shanghai Bell |
| R4-2209779 | Discussion on RRM performance requirements for FeMIMO | Apple |
| R4-2209786 | DraftCR on maintenance of Inter-cell L1-RSRP measurement requirements | Apple |
| R4-2209887 | Discussion on UE FeMIMO demod HST-SFN | Huawei, HiSilicon |
| R4-2209888 | Simulation results on UE FeMIMO demod HST-SFN | Huawei, HiSilicon |
| R4-2209889 | Discussion on UE FeMIMO demod mTRP | Huawei, HiSilicon |
| R4-2209890 | Simulation results on UE FeMIMO demod mTRP | Huawei, HiSilicon |
| R4-2209891 | Discussion on UE FeMIMO CSI mTRP | Huawei, HiSilicon |
| R4-2209892 | Discussion on UE FeMIMO CSI FeTypeII PS codebook | Huawei, HiSilicon |
| R4-2210002 | Draft CR on P-MPR reporting and configured transmit power | Apple AB |
| R4-2210050 | DraftCR on maintenance of TRP specific BFD requirements | Apple |
| R4-2210052 | Remaining issues on unified TCI switching delay requirement | Nokia, Nokia Shanghai Bell |
| R4-2210053 | DraftCR on DCI based DL and UL TCI switching delay requirements | Nokia, Nokia Shanghai Bell |
| R4-2210054 | Remaining issues on inter-cell beam management | Nokia, Nokia Shanghai Bell |
| R4-2210055 | Remaining issues on other items of Rel-17 feMIMO RRM | Nokia, Nokia Shanghai Bell |
| R4-2210138 | RRM requirements of unified TCI state for FeMIMO | Ericsson |
| R4-2210139 | CR on unified TCI state switching requirements | Ericsson |
| R4-2210140 | RRM requirements for inter-cell BM in FeMIMO | Ericsson |
| R4-2210141 | CR on L1-RSRP measurements for a cell with different PCI from serving cell | Ericsson |
| R4-2210142 | Discussion on TRP specific link recovery procedures | Ericsson |
| R4-2210143 | CR on TRP specific CBD and BFR requirements | Ericsson |
| R4-2210144 | Scope of RRM test cases for FeMIMO | Ericsson |
| R4-2210149 | Views on m-TRP CSI Performance Requirements | Qualcomm Incorporated |
| R4-2210150 | Views on Performance Requirements for Further Enhanced TypeII Port Selection Codebook | Qualcomm Incorporated |
| R4-2210151 | Views on FeMIMO HST Performance Requirements | Qualcomm Incorporated |
| R4-2210152 | Views on Performance Requirements for Enhanced Multi-TRP | Qualcomm Incorporated |
| R4-2210248 | Email discussion summary for [103-e][113] NR_feMIMO_maintenance | Moderator (Samsung) |
| R4-2210300 | Email discussion summary for [103-e][228] NR_feMIMO_RRM_1 | Moderator (Intel) |
| R4-2210301 | Email discussion summary for [103-e][229] NR_feMIMO_RRM_2 | Moderator (Samsung) |
| R4-2210334 | Email discussion summary for [103-e][328] NR_FeMIMO_Demod | Moderator (Samsung) |
| R4-2210448 | Email discussion summary for [103-e][113] NR_feMIMO_maintenance | Moderator (Samsung) |
| R4-2210497 | Email discussion summary for [103-e][228] NR_feMIMO_RRM_1 | Moderator (Intel) |
| R4-2210498 | Email discussion summary for [103-e][229] NR_feMIMO_RRM_2 | Moderator (Samsung) |
| R4-2210531 | Email discussion summary for [103-e][328] NR_FeMIMO_Demod | Moderator (Samsung) |
| R4-2210552 | WF on FeMIMO maintenance | Samsung |
| R4-2210616 | WF on FeMIMO RRM impact for unified TCI | Intel |
| R4-2210617 | WF on FeMIMO RRM performance requirements | Samsung |
| R4-2210669 | WF on CSI requirement for Rel-17 FeMIMO | Samsung |
| R4-2210670 | WF on demodulation requirement for Enhancement on HST-SFN deployment | Ericsson |
| R4-2210671 | WF on demodulation requirement for Enhancement on Multi-TRP | Huawei |
| R4-2211113 | Draft CR to TS38.133 Corrections on R17 L1-RSRP requirement on NSC | Samsung |
| R4-2211114 | CR on L1-RSRP measurement requirements for inter-cell BM in R17 | vivo |
| R4-2211115 | DraftCR on maintaining R17 TRP specific BFR requirements | Huawei, Hisilicon |
| R4-2211116 | CR on TRP specific CBD and BFR requirements | Ericsson |
| R4-2211144 | CR on unified TCI in R17 feMIMO | vivo |
| R4-2211148 | WF on FeMIMO RRM requirements for inter-cell beam management and TRP-specific link recovery | Huawei |
| R4-2211149 | LS on active TCI state list for UL TCI | Nokia |
| R4-2211160 | DraftCR on maintaining L1-RSRP measurement requirements for R17 inter-cell beam managements | Huawei, Hisilicon |
| R4-2211174 | LS on GP positioning between two UL SRS resource sets | Nokia, Nokia Shanghai Bell |
| R4-2211188 | CR on measurement restriction and scheduling availability for inter cell L1-RSRP measurement | MediaTek Inc. |
| R4-2211203 | WF on FeMIMO RRM impact for unified TCI | Intel |
| R4-2211226 | LS on GP positioning between two UL SRS resource sets | Nokia, Nokia Shanghai Bell |
| R4-2211227 | WF on FeMIMO maintenance | Samsung |

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template