3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220813

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2623 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_NR5GC_RRC_ProcedureDelayReestablishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of function : f_NR5GC_RRC_ProcedureDelayReestablishment When UE initiates RACH procedure to send the Reestablishment Complete , “repeat” is missing inside the alt block which results in the failure : Unexpected Rrc Pdu received |  |  |  |  |  |  |  |  |
| Summary of change: |  | Repeat has to be called on the reception of Preamble Ind , so that reestablishment complete message gets received inside alt block |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for NR testcases 8.1.5.8.1	2

2.1	Correction to the function f_NR5GC_RRC_ProcedureDelayReestablishment	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcases 8.1.5.8.1

## Correction to the function f_NR5GC_RRC_ProcedureDelayReestablishment

| Function name | f_NR5GC_RRC_ProcedureDelayReestablishment |
| --- | --- |
| Reason for change | In the current TTCN implementation of function : f_NR5GC_RRC_ProcedureDelayReestablishment When UE initiates RACH procedure to send the Reestablishment Complete , “repeat” is missing inside the alt block which results in the failure : Unexpected Rrc Pdu received |
| Summary of change | Repeat has to be called on the reception of Preamble Ind , so that reestablishment complete message gets received inside alt block |
| TTCN module | NR5GC_Handover.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_ProcedureDelayReestablishment(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE >> /* UE initiates RACH procedure */ [] SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)) { f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable), -, tsc_NoCnfReq); // @sic R5s220683 sic@ v_RACH_ProcedureStarted := true; } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } << SKIPPED CODE >> }

After Change:

function f_NR5GC_RRC_ProcedureDelayReestablishment(NR_CellId_Type p_NR_CellId, template (value) NR_SRB_COMMON_REQ p_SRB_COMMON_REQ, template (present) NR_SRB_COMMON_IND p_SRB_COMMON_IND, integer p_Nms, charstring p_TestcaseStep ) runs on NR_BASE_PTC return NR_SRB_COMMON_IND { << SKIPPED CODE >> /* UE initiates RACH procedure */ [] SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)) { f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable), -, tsc_NoCnfReq); // @sic R5s220683 sic@ v_RACH_ProcedureStarted := true; repeat; //WA#WI=977601 } [] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND { v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame; v_T4Flag := true; t_Watchdog.stop; } [] t_Watchdog.timeout { /* timeout => after T5 */ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout"); } << SKIPPED CODE >> }