3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220837

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2642 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the template cs_NR_PDCP_Config_SplitSRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Refer 38.331 RRC pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. Refer 38523-1 Table 8.2.2.3.1.3.3-1D: PDCP-Config (Table 8.2.2.3.1.3.3-1C) pdcp-Duplication false pdcp-Duplication Not present one UL path Split_SRB For Split SRB , if Pdcp duplication is not intended , then it is proposed to set the value as Omit |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 8.2.2.3.1 Pdcp Duplication is set to Omit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for NR testcase 8.2.2.3.1	3

2.1	Correction to the template cs_NR_PDCP_Config_SplitSRB	3

2.2	New Implementation : template cs_NR_PDCP_MoreThanOneRLC_SRB	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 8.2.2.3.1

## Correction to the template cs_NR_PDCP_Config_SplitSRB

| Template name | cs_NR_PDCP_Config_SplitSRB |
| --- | --- |
| Reason for change | Refer 38.331 RRC pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. Refer 38523-1 Table 8.2.2.3.1.3.3-1D: PDCP-Config (Table 8.2.2.3.1.3.3-1C) pdcp-Duplication false pdcp-Duplication Not present one UL path Split_SRB For Split SRB , if Pdcp duplication is not intended , then it is proposed to set the value as Omit |
| Summary of change | For 8.2.2.3.1 Pdcp Duplication is set to Omit |
| TTCN module | NR_RRC_Templates |
| MCC160 Comment |  |

Before Change:

template (value) PDCP_Config cs_NR_PDCP_Config_SplitSRB(template (omit) PDCP_Config.drb p_PDCP_Config_DRB := omit, template (omit) LogicalChannelIdentity p_LogicalChannelId := omit, // @sic R5s190161 Ch. 5 sic@ CellGroupId p_CellGroup := tsc_NR_CellGroupId_MCG, boolean p_Pdcp_Duplication := false ) := //@sic R5-202576 sic@ /* @status APPROVED (ENDC) */ cs_NR_PDCP_Config_Split(p_PDCP_Config_DRB, p_LogicalChannelId, p_CellGroup, p_Pdcp_Duplication, omit);

After Change:

template (value) PDCP_Config cs_NR_PDCP_Config_SplitSRB(template (omit) PDCP_Config.drb p_PDCP_Config_DRB := omit, template (omit) LogicalChannelIdentity p_LogicalChannelId := omit, // @sic R5s190161 Ch. 5 sic@ CellGroupId p_CellGroup := tsc_NR_CellGroupId_MCG, template (omit) boolean p_Pdcp_Duplication := omit, #WA#WI=979376 template (omit) UL_DataSplitThreshold p_UL_DataSplitThreshold := infinity_ #WA#WI=979376 ) := //@sic R5-202576 sic@ /* @status APPROVED (ENDC) */ //cs_NR_PDCP_Config_Split(p_PDCP_Config_DRB, p_LogicalChannelId, p_CellGroup, p_Pdcp_Duplication, omit); #WA#WI=979376 cs_PDCP_Config_Def (p_PDCP_Config_DRB, #WA#WI=979376 cs_NR_PDCP_MoreThanOneRLC_SRB(cs_NR_PDCP_PrimaryPath(p_CellGroup, p_LogicalChannelId), p_UL_DataSplitThreshold, p_Pdcp_Duplication ));

## New Implementation : template cs_NR_PDCP_MoreThanOneRLC_SRB

| Template name | cs_NR_PDCP_MoreThanOneRLC_SRB |
| --- | --- |
| Reason for change | Refer 38.331 RRC pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the state of the duplication at the time of receiving this IE. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. Refer 38523-1 Table 8.2.2.3.1.3.3-1D: PDCP-Config (Table 8.2.2.3.1.3.3-1C) pdcp-Duplication false pdcp-Duplication Not present one UL path Split_SRB For Split SRB , if Pdcp duplication is not intended, then it is proposed to set the value as Omit |
| Summary of change | For 8.2.2.3.1 Pdcp Duplication is set to Omit |
| TTCN module | NR_RRC_Templates |
| MCC160 Comment |  |

Implemented Function

#WA#WI=979376 Start template (value) NR_MoreThanOneRLC_Type cs_NR_PDCP_MoreThanOneRLC_SRB(template (value) NR_PrimaryPath_Type p_PrimaryPath, template (omit) UL_DataSplitThreshold p_UlDataSplitThreshold := infinity_, template (omit) boolean p_Pdcp_Duplication:= omit) := { /* @status APPROVED (ENDC, NR5GC) */ /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ primaryPath := p_PrimaryPath, ul_DataSplitThreshold := p_UlDataSplitThreshold, pdcp_Duplication := p_Pdcp_Duplication }; #WA#WI=979376 End