3GPP TSG RAN ad hoc									                               RPa160026

Barcelona, Spain, January 28 - 29, 2016

Agenda Item:		5

Source:			ZTE

Title:				text proposal for eMBB related deployment scenarios in Chapter 6

Document for:		approval

Introduction

In 3GPP RAN# 70 meeting, the “SI Scenarios and Requirements for Next Generation Access Technologies” was approved with the aim to identify typical usage scenarios for next generation access technologies and the required capabilities in each corresponding usage scenarios, and to provide guidance to the technical work to be performed in RAN WGs. In order to achieve this, the study item should fulfill the following objectives [1]:

- Identify the typical deployment scenarios associated with attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, etc.

This contribution presents our views on the indoor hotspot and Dense Urban deployment scenarios in eMBB.

Text proposal

Based on the draft of TR skeleton of Scenarios and Requirements for Next Generation Access Technologies [3] and discussion proposal [2], the text proposal can be found below:

Note:  listed deployment scenarios and their attributes in the following TP are for preliminary discussion, which would be further updated according to study in future.

------------------------------------------------<TEXT START>-----------------------------------------------------

- 6	Scenarios

[This subsection first briefly introduces the three usage scenarios defined by ITU-R, and then describes several deployment scenarios for the three usage scenarios. The mapping between usage scenarios and deployment scenarios needs to be clarified]

- 6.1	Deployment scenarios for eMBB

[This subsection describes the deployment scenarios for eMBB and their attributes such as carrier frequency, inter-site distance, user density, maximum mobility speed, etc. The detailed evaluation related aspects, e.g., channel model, are not included in this TR]

- 6.1.1	Indoor Hotspot (InH)

The Indoor hotspot, one of the typical eMBB deployment scenarios [6], can be considered as a scenario with isolated cells in a building/hotspot serving stationary and pedestrian users. Moreover, Indoor hotspot scenario focuses on achieving higher user throughput by deploying denser sites and hence the smallest cells. Indoor hotspot scenario was considered when 4G scenarios were defined [4] and evaluated. However, in order to meet new challenges in designing new radio access system, the scenario needs to be re-considered with some new elements added.

Its attributes are listed in Table 6.1.1.1.

Table 6.1.1.1 Attributes of deployment scenario of Indoor Hotspot

| Attribute | Value |
| --- | --- |
| Deployment scenarios | Indoor Hotspot |
| Carrier Frequency | When carrier frequency is lower than 6GHz, carrier frequency is 3.5GHz; |
|  | When carrier frequency is higher than 6GHz, carrier frequency is is FFS |
| Total system bandwidth | 100MHz for LF |
|  | FFS for HF |
| Network layout | Small cell only Indoor floor |
| Inter-Site Distance (ISD) | 10-20m |
| Antenna configuration | up to 32 antenna elements with XPOL for LF |
|  | FFS for HF |
| Maximum mobility speed | 3km/h |
| Backhaul/Fronthaul | Ideal or very fast |
| Device distribution | 100% indoor |

- 6.1.2	Dense Urban

The dense urban, one of some typical eMBB deployment scenarios, can be considered denser network deployment scheme, higher user densities, higher traffic loads and outdoor and outdoor-to-indoor coverage than before. More specifically, The Dense Urban deployment scenario focuses on Heterogeneous network including Macro cells (i.e. Radio access points shall be above rooftop level) and/or small cells (i.e. Radio acess points shall be below rooftop level) and high user densities and traffic loads in city centres and dense urban areas.Moreover, dense urban focuses on achieving higher user throughput by deploying denser sites and hence the smallest cells. In order to meet these challenges, therefore, dense urban scenario was considered when 4G scenarios were defined [4] and evaluated. In order to meet new challenges in designing new radio access system, the scenario needs to be re-considered with some new elements added.

Its attributes are listed in Table 6.1.1.1.

Table 6.1.2.1 Attributes of deployment scenario of Dense Urban

| Attribute | Value |  |
| --- | --- | --- |
|  | Macro/micro cells | small cells |
| Network layout | macro cells with outdoor small cells |  |
| Inter-Site Distance (ISD) | 200-500m | More than 10 small cells in a cluster i.e. around 10-50m |
| Carrier Frequency | 2G / 3.5GHz | When carrier frequency is lower than 6GHz, carrier frequency is 3.5GHz; |
|  | When carrier frequency is higher than 6GHz, carrier frequency is is FFS |  |
| The total system Bandwidth | 100MHz for LF | 100MHz for LF |
|  | FFS for HF | HF is FFS |
| Antenna Configuration | Up to 128 antenna elements for LF | Up to 64 antenna elements for LF |
|  | FFS for HF |  |
| Backhaul | Non-Ideal backhaul/ Self backhaul |  |
| Maximum mobility speed | 3km/h for indoor UEs 30km/h for outdoor UEs |  |
| Device distribution | 50% indoor, 50% outdoor More Option is FFS |  |
| Maximum mobility speed | 3km/h indoor UE 30km/h outdoor UE |  |

- 6.1.3	Deployment scenario N

(Additional definition of deployment scenario will be studied further till next meeting)

--------------------------------------------------<TEXT END>------------------------------------------------------

Conclusion

In this contribution, test environments, deployment scenarios and their attributes for usage scenario of eMBB are discussed and proposed. Proposals are:

- Proposal 1. Consider contents including deployment scenarios and their attributes proposed in TP

References

  - RP-152257, Study on Scenarios and Requirements for Next Generation Access Technologies, CMCC, RAN#70, Stiges, Spain, Dec.7-10, 2015

  - RP-16xxxx, Deployment scenarios for eMBB, ZTE, RAN Ad hoc, Barcelona, Spain, January 28 - 29, 2016

  - RP-16xxxx, TR skeleton of Scenarios and Requirements for Next Generation Access Technologies, CMCC, RAN Ad hoc, Barcelona, Spain, January 28 - 29, 2016

  - ITU-R Recommendation M.2083,"IMT Vision - Framework and overall objectives of the future development of IMT for 2020 and beyond, " 2015